3GPP TSG RAN Meeting #106 			          RP-242488

Madrid, Spain, December 9-12, 2024

Agenda Item:		9.3.1.1

Source:				InterDigital, Inc.

Title:						On the Scope of Rel-19 NR MIMO Phase 5

Document for:		Discussion and Decision

# Background

RANP #102 approved the WID for NR MIMO Phase 5 [1]. The WID covers five main objectives, where one of the described objectives is to specify 3-antenna-port codebook-based transmissions. In RANP #105, the WID was revised to including some additional related enhancements, as highlighted below [2],

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource - Note: UL full power transmission mode 1 and 2 are not supported. - Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

# Discussion

In RAN1 #119 meeting, RAN1 completed the discussion on recently approved enhancements related to support of non-codebook-based uplink transmission, and details related to 3T6R antenna switching operation [3]. Further, per WID, UE capability discussion for support of 3T3R was discussed and agreed.

Agreement (#119) The following capabilities are introduced for Rel-19 - 't3r3' for 3T3R. - 't3r6' for 3T6R.

From the perspective of some companies, details related to 3T3R antenna switching, such as SRS resource/set configuration, should not be discussed under the agenda item (9.2.3), as the WID states that,

“Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.”

However, it has been argued by the other group of companies that discussing SRS configuration details for support of 3T3R antenna switching, including SRS resource/set definition, is not considered as an enhancement. According to the opinion, SRS configuration details is inherent to the agreed capability, and it should be discussed, as without it, UE capability reporting of ‘3T3R’ SRS antenna switching will not be meaningful.

Observation 1: UE behaviour, when reporting a UE capability of ‘3T3R’ antenna switching, is not clear.

Proposal 1: RANP clarifies that SRS configuration details, including SRS resource/set definition for ‘3T3R’ antenna switching, is required to enable ‘3T3R’ antenna switching capability.

# Conclusion

In this contribution, we discussed our views with respect to ‘3T3R’ antenna switching. Based on the discussion, following observation and proposal are made,

Observation 1: UE behaviour, when reporting a UE capability of ‘3T3R’ antenna switching, is not clear.

Proposal 1: RANP clarifies that SRS configuration details, including SRS resource/set definition for ‘3T3R’ antenna switching, is required to enable ‘3T3R’ antenna switching capability.

# References

- RP-234007, “New WID: NR MIMO Phase 5”, 3GPP TSG RANP Meeting #102, Samsung (Moderator), December 2023

- RP-242394, “WID revision: NR MIMO Phase 5”, 3GPP RANP Meeting #105, Samsung (Moderator), September 2024

- R1-2409465, “Summary of Discussion on 3TX CB-based Uplink in RAN1#119”, InterDigital, Inc. (Moderator), November 2024