3GPP TSG-RAN Meeting #106 		                                    		     RP-242489

Madrid, Spain, Dec. 9th – 12th

Agenda Item:	9.3.1.3

Source:	InterDigital, Inc.

Title:	On the Scope of Rel-19 AI/ML Positioning

Document for:	Discussion and Decision

# Introduction

In RAN#105 [1], for AIML positioning, RAN recommended that “RAN1 will not discuss any proposals targeting second priority issues in Q4, 2024.” In this contribution, scope of Rel 19 AIML positioning for Q1 and Q2 for 2025 is discussed and we make a proposal to focus on 1st priority issues only for the remaining meetings until sufficient progress is made.

# Progress in RAN WGs

As indicated in the WID, Case 2a and 2b are in the 2nd priority. In this contribution, we discuss why we support to avoid discussion on Case 2a and 2b until details related to 1st priority cases are clarified.

In RAN1#119, RAN1 made an agreement on enhancement for existing path-based measurement for Case 3b [2]. RAN1 now needs to discuss and agree on parameter values for enhanced measurement. The following key issues need to be discussed in RAN1 in the remaining 3 meetings to complete the WI for Rel-19;

- Case 1, details related to consistency between training phase and inference phase

- Case 1, details related performance monitoring in LCM (Life Cycle Management)

- Details related to output in Case 3a

The following agreement, related to the LCM procedure, was agreed in RAN2#127b. It is clear from the agreement that RAN1 inputs related to details in the procedure are needed to make progress in RAN2 discussion. For example, RAN2 needs to know whether how the LMF can signal network side additional conditions to the UE.

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

In RAN1#119, RAN1 agreed to study the following options and down-select one of the options for determination of consistency for Case 1. It should be noted that the aforementioned options for a study in RAN1 only focus on explicit or implicit indication for TRP location and it is not clear whether other network side additional conditions need to be signalled explicitly or implicitly to the UE.

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs).

In addition, RAN2 needs feedback from RAN1 related to details of inference configurations and when they, in the agreed procedure, can be provided to the UE. Finally, a definition of a “functionality” also needs to be discussed in RAN1 such that RAN2 can work on activation or deactivation mechanism for the function. Overall, RAN1 and RAN2 need a tight coordination to finalize the details of the LCM procedure and achieve successful completion of the work item. We make the following observation

Observation 1: For Case 1, RAN1 needs to discuss details related to determination of consistency, inference configurations and definition of a “functionality” and coordinate with RAN2 to finalize the details of LCM

In RAN1#116b, RAN1 agreed on the following options for entities which can perform performance monitoring [3]. In Option A, the UE performs performance monitoring. In Option B, the LMF performs performance monitoring. RAN1 is currently evaluating which options to support. Depending on the options, new signaling between UE and LMF may need to be introduced, impacting RAN2 work.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Details related to Case 3a also require discussion in RAN1. For example, the definition of the AIML-generated LOS indicator and details related to associated measurements require discussions.

Observation 2: For Case 3a, RAN1 needs to finalize details of the output of an AIML model

There are also discussion topics that are not mentioned here, namely details related to collected data. For example, RAN1 needs to discuss details of pairing between measurements and associated ground truth and assistance information (e.g., timestamp, quality indicator) related to collected data. Considering RAN1 should use the last 2 meetings to finalize the details of RRC signalling, RAN1 should finish technical discussions related to the core design in RAN1#120, giving enough time for RAN2 to finalize details of signalling. Based on the observations, we have a concern on starting discussion on Case 2a and Case 2b which may potentially take away time for finalizing details related to Case 1, Case 3a and Case 3b. For AIML based positioning, whether an associated ID will be used or not for indicating consistency in network side additional conditions will have an impact on signalling. In addition, the structure of assistance data will also depend on whether an associated ID is present or not. Discussions related to performance monitoring may also have an impact on signalling.  Therefore, we make the following proposal.

Proposal 1: No RAN1 discussion on proposals for 2nd priority issues in Q1, 2025 or remove 2nd priority issues from the WID

# Conclusion

Observation 1: For Case 1, RAN1 needs to discuss details related to determination of consistency, inference configurations and definition of a “functionality” and coordinate with RAN2 to finalize the details of LCM

Observation 2: For Case 3a RAN1 needs to finalize details of the output of an AIML model

Proposal 1: No RAN1 discussion on proposals for 2nd priority issues in Q1, 2025 or remove 2nd priority issues from the WID.

# Reference

[1] RAN chair notes, RAN#105, Melbourne, Australia, Sept., 2024.

[2] RAN1 chair notes, RAN1#119, Orland, USA, Nov., 2024.

[3] RAN1 chair notes, RAN1#116b, Changsha, China, Apr., 2024.