3GPP TSG RAN Meeting #106 			          RP-242491

Madrid, Spain, December 9-12, 2024

Agenda Item:		15.2

Source:				InterDigital, Inc.

Title:						Scope for Rel-20 MIMO for 5G-Advanced

Document for:		Discussion and Decision

# Background

3GPP has paved the way for deployment of advanced MIMO technology by providing related specifications and procedures for UE vendors, network equipment suppliers and operators. MIMO is the backbone of a cellular network and plays a key role for achieving high spectrum efficiency, coverage and reliability. Therefore, in each release, one or more of the above objectives for uplink or downlink transmission have been the focus of the developed specifications [1]. In this contribution, we provide our views regarding the general scope of NR MIMO in Rel-20 5G-Advanced.

# Discussion

Since Rel-15, 3gpp has discussed and specified various enhancements for NR MIMO [1]. In Rel-15 and -16, the focus of the NR MIMO was primarily on developing a robust and versatile framework for downlink transmissions and beam management; including design of various Codebook Types, design of reference signals for demodulation and channel sounding, beam related procedures for indication/selection, failure detection and recovery, etc. Then, in Rel-17 and -18, uplink-oriented enhancements got more traction, by inclusion of features such as PUCCH/PUSCH repetition, SRS capacity/interference enhancements, STxMP, 8TX UEs, etc. In the ongoing Rel-19 NR MIMO discussion, the scope of the effort is on UE-driven beam management, UE-assisted calibration for CJT, 3-port UL transmission and asymmetric DL/UL multi-TRP transmission.

Through the development of the last 5 releases of NR MIMO, due to the limited time, the scope of each objective was restricted, and not always all sub-topics were discussed.  Hence, its scope should be primarily focused on important remaining sub-topics and key enhancements in Rel-20 to fully enable the features introduced in earlier releases and ensure the performance of 5G-Advanced network.

Observation 1:  Through the development of last 5 releases of NR MIMO, due to time constraints, the scope of each objective was limited.

Proposal 1: The scope of Rel-20 should be primarily focused on important remaining sub-topics from earlier releases.

Remaining Important Sub-topics from Earlier Releases

In the following, we review some of the remaining important left-over sub-topics from earlier releases of NR MIMO that should be considered for further discussion and decision in Rel-20,

- SRS Beam Sweeping for Asymmetric UL mTRP: In the absence of a downlink reference signal from the uplink-only-TRP, the initial focus of Rel-19 uplink-only-TRP transmission has been to enable configuration of a pathloss power offset to support a meaningful power setting for uplink. Later, the possibility of configuration of 2 TA information was also supported to allow a better alignment of the uplink transmission with respective of the two TRPs. However, to support asymmetric UL mTRP in FR2, the important missing part is the uplink beam selection as there is no downlink reference signal as the target for uplink beam determination. Hence, to fully exploit potential benefits of asymmetric UL TRP in FR2, an SRS beam-sweeping procedure should be specified.

- Frequency-Selective Uplink Transmission: Due to non-negligible delay spread of the channel, the wireless channel exhibits a frequency-selective behavior that requires adaptation of the transmission strategy. For downlink, subband based precoding has been supported, however it has not been considered for uplink transmission. The main issue related to support of subband-base precoding for uplink is the required overhead for precoder indication. One simple way to support subband precoding without any impact on overhead is through use of N random precoders over a scheduled bandwidth, where N can be configured or indicated to a UE.

- Support of other Modes of Transmission for STxMP: Rel-18 sDCI STxMP supports single codeword SDM PUSCH, SFN PUSCH and SFN PUCCH. Further, the max number of panels is restricted to two. However, considering Rel-18 8TX UE capability, further potential enhancements can be considered by increasing the number of panels to 4 and augmenting the number of codewords to 2. Hence, beside the existing modes, more transmission modes such as 2-codeword PUSCH+PUSCH, PUSCH+PUCCH, etc., modes can be considered to enhance the flexibility and spectrum efficiency of the overall system.

- Partial- and Fully-Coherent Codebook-based for 3TX UE: In Rel-19, codebook-based transmission was specified for a non-coherent UE. However, several companies have expressed their interest in support of codebook-based transmission for partially- and fully-coherent UEs. While implementations based on full-coherency is certainly possible, the partial-coherent assumption has received the most interest, as it would be the most natural implementation of a 3TX UE.

- DFT-S-OFDM Multi-layer MIMO: In Rel-15, DFT-S-OFDM was specified for rank1 uplink transmission. However, for higher ranks, it was decided to use OFDM as the waveform for uplink MIMO transmissions. To improve uplink spectrum efficiency, some companies have shown interest to use DFTS-OFDM for MIMO transmission for uplink.

Observation 2: There are several important subtopics from earlier releases of NR MIMO that were left out due to limited time.

Proposal 2: Consider the following important remaining sub-topics for discussion and decision in Rel-20 NR MIMO, where topic order is based on our priority:

- SRS Beam Sweeping for Asymmetric UL mTRP

- Frequency-Selective Uplink Transmission

- Support of other Modes of Transmission for STxMP

- Partial- and Fully-Coherent Codebook-based for 3TX UE

- DFT-S-OFDM Multi-layer MIMO for uplink

Table 1- Summary of potential topics for Rel-20 NR MIMO

|  | Topic | Motivation |
| --- | --- | --- |
| First Priority | SRS Beam Sweeping for Asymmetric UL mTRP | Exploit potential benefits of asymmetric UL TRP in FR2 |
|  | Frequency-Selective Uplink Precoding | Enable adaptation of uplink precoding per subband |
|  | Support of other Modes of STxMP Transmission | Aim to match capacity of 8TX UE, e.g., 2CW, 4 panels |
| Second Priority | Partial- and Fully-Coherent CB-based for 3TX UE | Improve performance by targeting more capable UEs |
|  | DFT-S-OFDM Multi-layer MIMO for UL only | Enhance uplink capacity |

# Conclusion

In this contribution, as captured in Table 1, we discussed our views with respect to the scope of Rel-20 NR MIMO. Based on the presented discussion, following observations and proposals are made,

Observation 1:  Through the development of last 5 releases of NR MIMO, due to the limited time, the scope of the objectives was restricted.

Proposal 1: The scope of Rel-20 should be primarily focused on important remaining sub-topics from earlier releases, however other key enhancements may also be considered to insure providing a good baseline for development of 6G specifications.

Observation 2: There are several important subtopics from earlier releases of NR MIMO that were left out due to limited time.

Proposal 2: Consider the following important remaining sub-topics for discussion and decision in Rel-20 NR MIMO, where topic order is based on our priority:

- SRS Beam Sweeping for Asymmetric UL mTRP

- Frequency-Selective Uplink Transmission

- Support of other Modes of Transmission for STxMP

- Partial- and Fully-Coherent Codebook-based for 3TX UE

- DFT-S-OFDM Multi-layer MIMO for uplink and downlink

# References

- RP-242394, “WID revision: NR MIMO Phase 5”, 3GPP RANP Meeting #105, Samsung (Moderator), September 2024