3GPP TSG RAN Meeting #106	RP-242530

Madrid, Spain, December 9-12, 2024

Agenda item:	16

Source:	Nokia

Title:	Nokia views on IMT-2030 Technical Performance Requirements

Document for:	Discussion

# 1	Introduction

ITU-R Working Party 5D (WP 5D) has developed Report ITU-R M.2516 titled “Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond”, and Recommendation ITU-R M.2160 titled “Framework and overall objectives of the future development of IMT for 2030 and beyond” in the previous WRC cycle. In their LS in [1], ITU-R WP 5D request input from external organizations such as 3GPP on aspects including but not limited to:

- Candidate items for minimum technical performance requirements based on M.2160 Usage Scenarios and Capabilities, including necessary background information and justification, which are requested to be provided preferably by WP 5D #48 (Feb 2025 TBC).

- Associated target values for the above candidate items for the minimum technical performance requirements which are requested to be provided preferably by WP 5D #49, (June/July 2025 TBC)

The Framework includes definition of capabilities of IMT-2030, where some are currently listed with a range of research target values. The IMT-2030 Capabilities are illustrated in Figure 1. WP 5D is now expected to define Technical Performance Requirements (TPRs) for the capabilities, which then need to be mapped to usage scenarios and test environments, if applicable (refer to Section 4 of Rec. ITU-R M.2160). It should be recognized that some of these TPRs can be expressed and evaluated quantitatively, whereas others can only be expressed and evaluated qualitatively, i.e. as in IMT-2020, a target value may not always be defined for a TPR. Some capabilities that will not be evaluated may be left out of the ITU-R Report (without having associated TPRs); instead, IMT-2030 systems support for those Capabilities may be described verbally using the “Submission Template”, which is expected to be similar to ITU-R Report M.2411 defined for IMT-2020.

Figure 1. Capabilities of IMT-2030.

In this contribution we provide our view on the IMT-2030 Capabilities, including items for which Technical Performance Requirements should be developed, and how compliance for such TPRs is to be evaluated.

# 2	Discussion

The IMT-2030 capabilities can be roughly split into two groups:

- “Legacy” Capabilities that were already present in IMT-2020 (Green colour in Figure 1), and

- Capabilities that are new

In the following we discuss these two groups of capabilities. The proposals related to the TPRs as well as High-level evaluation assumptions are provided in Section 3.

## 2.1	Legacy Capabilities Derived from IMT-2020

As IMT-2030 is based on evolution IMT-2020, the Capabilities defined for IMT-2020 should naturally carry over to IMT-2030. The following TPRs were defined in IMT 2020 in relation to the Capabilities:

- Peak data rate

- Peak Spectral Efficiency

- User experienced data rate

- Average Spectral efficiency

- 5th percentile user Spectral efficiency

- Area traffic capacity

- Connection Density

- Mobility

- Mobility interruption time

- User plane latency

- Control plane latency

- Reliability

- Energy Efficiency / Sustainability

- Bandwidth

- Support of wide range of services

- Supported spectrum band(s)/range(s)

We see that IMT-2030 should support the same TPRs as IMT-2020, while the associated performance numbers are expected to be more ambitious. The same evaluation methodology can and should be largely re-used as well, to allow for performance comparisons between different generations of radio standards, and to avoid unnecessary work in both ITU-R and 3GPP. However, in some cases there is a need to consider extension of the IMT-2020 TPRs to better reflect the target Usage Scenarios of IMT-2030. In some other cases, IMT-2020 already provides more than satisfactory performance, and the requirements will not need to be stretched further. Below we discuss further some of the legacy TPRs. The related proposals are summarized in Table 1 in Section 3.

Reliability:

Reliability TPR in IMT-2020 was targeting in particular the URLLC Usage Scenario, with very low User Plane latency (down to 1ms) supported for small application data (e.g. 20 bytes). Given that radio air interface latency is only one of the components impacting the end-to-end latency, and other delay components such as propagation times to servers and related processing times are of the same or larger order, the IMR-2020 requirements for URLLC are sufficient also to meet the needs of IMT-2030.

While URLLC remains to be an important use case for IMT-2030, there are also new usage scenarios, where low latency (in the order of a few ms) is combined with larger data packet sizes (e.g. a few MBs). Use cases for such data rates and latencies include e.g. XR, video conferencing, and support for emerging AI based services. Therefore IMT-2030 TPRs should account for also such types of services, and defining new performance requirements for moderately low latency in combination with larger packet sizes should be considered.

Energy Efficiency / Sustainability:

Sustainability is a key design target for the forthcoming 6G deployments and devices. Sustainability is a very broad field, including aspects related to manufacturing, deployment and operation during the life-time of the 6G networks and devices. Most of the aspects affecting overall sustainability of 6G are not directly related to standards, but rather on how the equipment is manufactured and operated and are hence not within the scope of IMT-2030 TPRs.

Energy efficiency is a key aspect for operation of IMT-2030 networks and UEs, affecting overall sustainability as well as operational cost and user experience. It is also possible to define quantifiable technical performance requirements for energy efficiency. Therefore, it makes sense to focus on EE, and in particular network EE when considering IMT-2030 related TPRs for sustainability.

When considering evaluation of EE, one should focus on defining requirements in a way that is agnostic to the exact type of implementation, which is not dictated by the standards. Instead, the requirements should verify the fundamental properties of radio standards that facilitate energy efficient operation. In IMT-2020 there were no numerical requirements set for EE, but instead network and UE energy efficiency were verified by inspection of support for high sleep ratio and duration, i.e. the network’s transmission rate of signals in case of an unloaded cell. We see that similar approach can be the starting point for IMT-2030 as well.

While IMT-2020 did not set specific requirements for sleep ratio and sleep duration, considering the constantly increasing importance of energy efficiency, IMT-2030 should adopt a more advanced approach and set quantifiable TPRs for EE. There are two main alternatives considered at ITU-R WP 5D:

Approach A: An approach similar to IMT-2020, but with specific requirements set for sleep ratio (e.g. > 95 %) and sleep duration (e.g. > 20 ms). Analytical approach can be used to verify that and RIT/SRIT meets the set requirements.

Approach B: Setting limit for the relative energy consumption in an unloaded case when compared to a fully loaded case. The requirement may be defined e.g. in terms of a percentage, such as 5%. This approach involves use of base station power models, that model the relative power consumption of different types of transmissions as well as transitions between them. The TPR can be evaluated analytically or with a relatively simple simulation, where the power model is applied on top a minimum set of transmissions in a case where there is no data traffic in a cell.

The two approaches described above are both ultimately analyzing the same thing, i.e. the minimum amount of transmissions occurring in a cell with no data traffic. Approach A is simpler in terms of definition in ITU-R, as well as analysis. Approach B provides more insight on how the actual network energy consumption varies between the fully loaded and unloaded cases; the downside is more complicated definition (especially the power model) and evaluation. Regarding the power model, 3GPP has developed and applied power models in past studies, but such models are not universally applicable nor technology agnostic, since they assume certain specific types of signals and protocols to be applied, that may not be present in other standards. Therefore, for Approach B to work, a simple and technology agnostic power model should be agreed upon at ITU-R WP 5D, or the power model should be left for the technology proponent to provide. This is unlikely to be a trivial discussion at ITU-R.  Given that approach A avoids this extra hurdle while capturing the same amount of information about the energy efficiency of the different systems, our preference is to adopt approach A in ITU-R.

Security and Resilience:

Security and Resilience are key targets for the design of both 5G s well as 6G. Both Security and Resilience are broad concepts, including aspects going beyond radio standards and scope of ITU-R WP 5D. It is clearly impractical to define numerical requirements for security and resilience, and hence these two capabilities were not having specific TPRs in IMT-2020; instead, the related aspects were considered in the Submission Template, where technology proponents were tasked to verbally provide information on how the RAT in question supports these aspects.

As in 5G, it remains impossible to define numerical targets for Security and Resilience. However, verbal description on the supported security mechanisms and means to guarantee resilient operation should be provided, either using the Submission template as in IMT-2020, or by defining a TPR that is evaluated by “Inspection”. The latter approach may be considered especially for Resilience.

Bandwidth:

The minimum requirement for bandwidth in IMT-2020 was 100 MHz for frequencies corresponding to (approximately) FR1. In IMT-2030 the requirement for bandwidth should be larger.   We propose to set the requirement for bandwidth to be at least 400 MHz. This can be achieved by e.g. a single carrier with 400 MHz channel bandwidth, or with carrier aggregation of two 200 MHz carriers.

## 2.2	New Capabilities of IMT-2030

In addition to the Capabilities that were already present in IMT-2020, IMT-2030 also defines a set of new Capabilities, extending the use of 6G to new use cases. These are discussed in the following.

Positioning:

While Positioning is a new capability to ITU-R, present 3GPP standards for NR already provide support for multiple positioning techniques. Technical performance requirements should be defined for selected use cases (e.g. indoor and outdoor), considering also that practically achievable positioning accuracy is largely deployment dependent. The current IMT-2030 research target for positioning accuracy (sub-meter level) is only feasible with certain specific scenarios, and IMT-2030 TPRs should not be limited to the most demanding use cases only.

Sensing:

Sensing is one of the fundamentally new capabilities in IMT-2030. Unlike e.g. positioning, sensing is not presently supported in the existing 3GPP NR standards. Sensing encompasses a large variety of techniques and use cases, and it is hard to foresee which ones will be the most important applications in the future.

Some of the mentioned potential use cases for sensing include e.g. intrusion detection​, environment monitoring​, detection and tracking in factories​, UAV flight trajectory tracking, traffic sensing, automotive use, health checking and environment monitoring​. These potential use cases may further have different targets in terms of what is the sensing object (e.g. human, automobile, drone, factory goods…). It is apparent that technical performance requirements cannot be set to optimally cater for all envisioned use cases; instead, it is necessary to focus on key aspects that are common for most of the anticipated sensing applications.

Given that sensing is still largely in a research phase, and anticipated applications are rather diverse, it is preferred to focus in IMT-2030 on requirements that are not tied to a specific type of sensing object or use case. Most of the foreseen sensing applications assume at least localization of the sensing object to be performed. Therefore, it is proposed that IMT-2030 focuses on this aspect. By setting requirements for localization accuracy or resolution, evaluated analytically or by simulations, it is possible to verify the main boundaries for the sensing performance of an IMT-2030 standard, while avoiding complex prioritization of largely varying use cases.

AI-related Capabilities:

Use of AI in radio networks can be roughly categorized as follows:

- AI for network: using AI in operating the network, to e.g. improve performance

- Network for AI: providing network connectivity for AI services

AI for network:  AI will likely be widely used in IMT-2030 systems to improve the performance of radio interface functions such as symbol detection/decoding, channel estimation, scheduling, network optimization etc. Corresponding improvements in network performance are expected to be reflected in TPRs such as Spectrum Efficiency, User Experienced Data Rate, and Reliability. There is no need to define separate TPRs for “AI for Network”.

Network for AI: AI services are one of the major use cases for IMT-2030. IMT-2030 should therefore provide sufficient performance to support AI related tasks. Many of the existing TPRs are already good metrics for determining how well the network can support AI services. Moreover, an extension of the “Reliability” TPR as discussed in Section 2.1 can also ensure that necessary support for AI is present in IMT-2030.

Coverage:

Coverage is listed as one of the capabilities of IMT-2030. Network coverage is a complex aspect to measure, and it depends on multiple factors including network deployment (inter-site distance, TX power, antenna configuration, BS antenna height, type of geographical environment, etc.). Coverage is also not a binary measure (yes/no), but instead it varies depending on the assumption on the required minimum bit rate and maximum allowed latency. Therefore, statistical properties, such as spectrum efficiency CDF, are the best way of characterizing network’s coverage in various types of deployment scenarios. The legacy TPRs discussed in Section 2.1. meet this purpose already, and there is no need to define additional TPRs for coverage.

Another aspect directly impacting coverage is the link budget. Provisioning of a link budget along with Maximum Coupling Loss (MCL) may be considered, as in IMT-2020.

Interoperability:

Interoperability refers to the radio interface being based on member-inclusivity and transparency, so as to enable functionality(ies) between different entities of the system. It is clearly hard to define technical performance requirements for interoperability. Related aspects of an IMT-2030 radio standard can be covered in the submission template for IMT-2030.

The proposals related to the items for IMT-2030 Technical Performance requirements are summarised in Table 1 in Section 3.

Proposal: 1: Technical Performance Requirements are developed for the items listed in Table 1. It can be further discussed if some of the aspects that are not directly measurable (e.g. Resilience) should be evaluated by inspection, of be included into the submission template.

## 2.3	Carrier Frequency and Bandwidth

The Technical Performance Requirements are tightly coupled with the related evaluation scenarios. While the exact evaluation assumptions require more discussion, it is important to set some of the elementary aspects early, such that work on detailed TPRs and the associated values can proceed.

The single most important evaluation assumption is carrier frequency. Carrier frequency impacts the propagation characteristics (pathloss, penetration, and channel model in general) as well as the practically feasible antenna configurations and will have a significant impact on achievable performance, affecting aspects including spectral efficiency, coverage, and others. As many of the new frequency bands considered for IMT-2030 are located around 7 GHz (e.g. frequency ranges 6.425-7.125 and 7.125-8.4 GHz), we propose to add 7 GHz carrier frequency in the set of evaluation assumptions for IMT-2030.

Proposal 2: 7 GHz carrier frequency is included in the set of evaluation parameters of IMT-2030, to reflect the frequency ranges 6.425-7.125 and 7.125-8.4 GHz.

Regarding the bandwidth assumed in evaluations, to align with the proposed minimum bandwidth of 400 MHz, 400 MHz and 200 MHz channel bandwidths should be considered at least in some evaluation scenarios.

Proposal 3: 400 MHz and 200 MHz channel bandwidths are included in the set of evaluation parameters of IMT-2030.

# 3	Conclusions

In this contribution we have provided our views on Technical Performance requirements for IMT-2030, as well as related high-level evaluation assumptions. The following Table 1 summarizes the proposed items for which TPRs should be developed, as well as the basic evaluation methodology for each TPR.

Proposal: 1: Technical Performance Requirements are developed for the items listed in Table 1. It can be further discussed if some of the aspects that are not directly measurable (e.g. Resilience) should be evaluated by inspection, of be included into the submission template.

Table 1. The proposed items for Technical Performance requirements (TPRs) of IMT-2030. The TPRs in green are carried over from IMT-2020 with some updates, while the TPRs in blue are completely new.

| TPR | Evaluation methodology | Comments |
| --- | --- | --- |
| Peak Data Rate | Analytical |  |
| Peak Spectral Efficiency | Analytical |  |
| User Experienced Data Rate | Simulation |  |
| Spectral Efficiency | Simulation | Both average and 5th percentile |
| Area Traffic Capacity | Analytical |  |
| User Plane Latency | Analytical | No need to reduce UP latency below 1 ms. |
| Control Plane Latency | Analytical |  |
| Connection Density | Analytical |  |
| Energy Efficiency | Analytical | Requirements for sleep ratio and sleep duration. |
| Reliability | Simulation | The evaluation scenarios can be extended beyond URLLC to cover new use cases such as AI, XR, etc with moderately low latency (a few ms) and medium to high data rates. |
| Mobility | Simulation | Same mobility classes as in IMT-2020 are sufficient. |
| Mobility Interruption Time | Analytical |  |
| Bandwidth | Inspection | 400 MHz minimum BW (e.g. 1 x 400 MHz or 2 x 200 MHz) |
| Support of wide range of services | Inspection |  |
| Supported spectrum band(s)/range(s) | Inspection |  |
| Positioning (New) | Simulation | Define requirements for vertical and horizontal positioning accuracy. The requirements need to take into account the deployment scenarios, and same accuracy cannot be assumed in all cases. |
| Sensing (New) | [Analytical or Simulation] | Focus on TPRs that are use-case agnostic, e.g. localization accuracy and/or resolution, while considering detection probability and false alarm rate as side conditions. |

Furthermore, technical performance requirements are tightly coupled with the evaluation assumptions. While the details of the evaluation parameters will only be discussed later in ITU-R WP 5D, it is beneficial to agree the assumption on carrier frequency at an early phase. We propose to indicate to ITU-R WP 5D that 7 GHz carrier frequency is included into evaluation parameters of IMT-2030, to reflect the frequency ranges 6.425-7.125 and 7.125-8.4 GHz.

Proposal 2: 7 GHz carrier frequency is included in the set of evaluation parameters of IMT-2030, to reflect the frequency ranges 6.425-7.125 and 7.125-8.4 GHz.

Regarding the bandwidth assumed in the evaluations, to align with the proposed minimum bandwidth of 400 MHz, 400 MHz and 200 MHz channel bandwidths should be considered at least in some evaluation scenarios.

Proposal 3: 400 MHz and 200 MHz channel bandwidths are included in the set of evaluation parameters of IMT-2030.

# References

- SP-241439 , “LIAISON STATEMENT TO EXTERNAL ORGANIZATIONS: Minimum requirements related to technical  performance for IMT-2030 radio interface(s)”, ITU-R WP 5D