3GPP TSG RAN Meeting #106								                    RP-242554

Madrid, Spain, December 9-12, 2024

Agenda item:	9.3.2.2

Source: 	Qualcomm Incorporated

Title: 	Views on DL coverage enhancements for Rel-19 NR NTN

Document for:	Discussion/Decision

# Introduction

In this contribution, we express Qualcomm’s view on system-level DL coverage enhancements and proposed way forward for the progress on DL coverage enhancements in the Rel-19 NR NTN WI.

# Discussions

One of the objectives of Rel-19 NR NTN work item is to study and if needed specify solutions, including link level enhancements and/or system level enhancements, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint [1]. One solution that has been discussed extensively is the extension of SSB periodicity.  Two other system level solutions were also proposed but were given less discussion due to limited meeting time, they are [2][3]:

- transmitting DL common channels using a beam footprint wider than the beam footprint for UE DL/UL data,

- enhanced cell DTX.

On the extension of SSB periodicity, RAN1 has agreed to support the maximal periodicity up to 160 ms and is still discussing if 320 ms periodicity is supported [4]:

Agreement

For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

- The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms.

  - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

The impacts on the backward compatibility due to the extension of SSB periodicity were also discussed. Specifically, two important impacts are listed below:

- The extension of SSB periodicity reduces the number of SSBs that a UE can combine and hence the SSB detection rate. Depending on UE implementation, an SSB search window may be shorter than the SSB periodicity and hence multiple scans of a given frequency may be needed even for very high SNR. The above can lead to significant increase of latency or even failure of initial frequency scan for legacy UEs assuming 20 ms SSB periodicity.

- Since the maximal supported value of SSB periodicity (ssb-PeriodicityServingCell, signalled in SIB1) is 160 ms, a legacy UE will not be able to camp on a cell with SSB periodicity greater than 160 ms.

The above impacts on backward compatibility were summarized in the following RAN1 observation [5]

Observation

Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan.

Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

While the backward compatibility issue during initial frequency scan can be alleviated by reducing the synchronization raster frequency points, there is no solution to the backward compatibility issue due to the SIB1 signalling for SSB periodicity larger than 160 ms. Additionally, since existing RAN4 requirements assume the maximal SSB periodicity is 160 ms, extending the SSB periodicity beyond 160 ms will require significant RAN4 work to examine and redefine the requirements on receiver sensitivity, transmit timing, RRM, etc. This will greatly increase the RAN4 workload and may significantly delay the Rel-19 specification for NR NTN.

It is also observed that discussions on SSB periodicity deprived the due attention to other system-level solutions. RAN1 is still discussing these these other system-level solutions, as can be seen in the latest Feature Lead Summary [6 - Sections 5, 6,  12]. Based on the above discussions, we have the following proposal:

Proposal 1: 320 ms or larger SSB periodicity is not supported for Rel-19 NR NTN.

Proposal 2: Update the WID with the following potential target for system level enhancements:

- solutions that enable efficient transmission of DL common channels using a wide beam footprint and transmission of DL/UL dedicated channels (including PRACH) using a narrower beam footprint

- enhancements to cell DTX.

# Conclusions

In this contribution, we gave our views on system-level DL coverage enhancements for Rel-19 NR NTN with the following proposals:

Proposal 1: 320 ms or larger SSB periodicity is not supported for Rel-19 NR NTN.

Proposal 2: Update the WID with the following potential target for system level enhancements:

- solutions that enable efficient transmission of DL common channels using a wide beam footprint and transmission of DL/UL dedicated channels (including PRACH)  using a narrower beam footprint

- enhancements to cell DTX.

# References

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT, RAN#104, June 17-20, 2024.

- R1-2406436, FL Summary #2: NR-NTN downlink coverage enhancements, Thales, RAN1 #119, 	November 18th – 22nd, 2024.

- R1-2407049, DL coverage enhancement for NR NTN, Qualcomm, August 19th – 23rd, 2024.

- Chairman’s Notes, RAN1#118 bis, October 14th – 18th, 2024.

- Chairman’s Notes, RAN1#119, November 18th – 22nd, 2024.

- R1-2409438, FL Summary #4: NR-NTN downlink coverage enhancements, Moderator (Thales), RAN1 #119	, November 18th – 22nd, 2024