3GPP TSG RAN meeting #106										RP-242559

Madrid, Spain, December 9-12, 2024

## Status Report to TSG

Agenda item:			9.3.1.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_AIML_air |  |  |  |  |
| Unique ID | 1020093 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242399 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: Sept ‘25 | Performance part: March ‘26 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 55% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Juan Montojo (RAN1); Xiaofeng Liu (RAN4); Marco Belleschi (RAN2/RAN3) |
|  | Company | Qualcomm; CAICT; Ericsson |
|  | Email | juanm@qti.qualcomm.com; liuxiaofeng1@caict.ac.cn; marco.belleschi@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

The study objectives in this project are expected to be spun off into a new SI (for approval at RAN#106).

While keeping the total number of TUs the same as currently approved, in the attached spreadsheet we present the proposed TUs for the normative objectives.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.1.1	RAN1#118bis

// Specification support for beam management

Agreement

Answer to Q2 in R1-2407604 as below:

RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Agreement

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring),

- At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring

  - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation

  - FFS: on other details including how to configure the resource set/resources for monitoring, including

    - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric

- FFS other alternatives

Agreement

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection:

- Option 1: Based on the uplink slot for the report

- Option 2: Based on the CSI reference resource corresponding to the report

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

Agreement

For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing.

- FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities.

- FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

Agreement

For UE-side AI/ML model, for BM-Case1, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR:

- Periodic (P) CSI-RS

- Semi-persistent (SP) CSI-RS

- Aperiodic (AP) CSI-RS

For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR:

- Periodic (P) CSI-RS

- Semi-persistent (SP) CSI-RS

- FFS: Aperiodic (AP) CSI-RS

Note: above CSI-RS resource refers to that used for beam management.

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring:

- Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference

  - FFS: the resource set(s) for monitoring

  - UE measures the resource set(s) for monitoring.

  - FFS how/when to report the monitoring results.

- Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The dedicated report configuration used for monitoring links to an inference report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - UE measures the resource set(s) for monitoring.

  - FFS when to report the monitoring results.

Agreement

RAN 1 further study the following options for applicability for inference for UE-side model:

Option 1:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied.

  - FFS on whether some IEs in the CSI report configuration can be removed or modified

  - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

- In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig

  - FFS on one or more of the above CSI-ReportConfig to be reported

- FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4

- FFS: whether Step 5 is needed,

Option 2:

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - NW configures one set or multiple sets of inference related parameters

    - Note: the set of inference related parameters is not configured by CSI-ReportConfig

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

  - The associated ID(s) may be configured

    - wherein the associated ID(s) may be

      - FFS: a) part of one set of the inference related parameters, or

      - FFS: b) independently from the one set of the inference related parameters.

- In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated.

- In Step 5, NW configures configuration(s) for CSI report for inference

Option 3:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter.

- In Step 4, UE reports by UAI

  - the applicable one or multiple sets of inference related parameters may be included.

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

    - Note: not applicable may also be replied by UE

    - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4.

  - the associated ID(s) may be included

    - FFS: a) as part of the inference related parameters, or

    - FFS: b) independently from the set of the inference related parameters.

- In Step 5, NW configures configuration(s) for CSI report for inference.

Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

Agreement

For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report.

Agreement

For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Agreement

Incorporating below text into the general part of the LS

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs

- The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing

Agreement

Answer to Q1 in R1-2407604 as below,

In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

// Specification support for positioning accuracy enhancement

Agreement

For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is:

- When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

  - FFS: details on how to associate quality indicator to timing information

Conclusion

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Agreement

From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) Channel measurement;

- (Optional) Quality of the channel measurement;

  - FFS: details of the quality

- (Mandatory) Time stamp of the channel measurement.

Agreement

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

Agreement

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference,

- for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other;

- Companies can provide inputs on further enhancements of existing assistance data, including new information

- Note: this does not mean that training and inference phases are mentioned in assistance data.

Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

|  | Information | UE-based DL-TdoA | UE-based DL-AoD |
| --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |  |  |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 3 | DL-PRS configuration of candidate NR TRPs |  |  |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |  |  |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |  |  |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |  |  |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |  |  |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 9 | PRS-only TP indication |  |  |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID |  |  |
| 11 | LOS/NLOS indicators |  |  |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |  |  |
| 13 | Validity Area of the Assistance Data |  |  |
| 14 | PRU measurements together with the location information of the PRU |  |  |
| 15 | Data facilitating the integrity results determination of the calculated location |  |  |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |  |  |
| 17 | Expected Angle Assistance information |  |  |
| 18 | PRS priority list |  |  |

[1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

[2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

// Specification support for CSI prediction

Agreement

For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance.

Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered.

Agreement

For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated:

- [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees)

- [Optional evaluation] Various TXRU mapping

  - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements)

  - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively)

- Other configurations are not precluded

Note: for other evaluation assumptions, reuse baseline of TR 38.843

Note: report how to map antenna port to antenna elements

Note: report the backbone/structure of AI/ML model used

Agreement

The TP1, TP2 and TP3 to TR38.843 in Section 3 of R1-2409236 are endorsed in principle.

// Additional study on AI/ML for NR air interface: CSI compression

Conclusion

For issues listed for inter-vendor collaboration Direction A, conclude the following

- Answer to Issue 2: From RAN1 perspective, at least,

  - For option 4-1, there is no proprietary information concern of disclosing encoder input/output from NW side to UE side

  - For option 3a-1, there is no proprietary information concern of disclosing encoder parameters from NW side to UE side

  - Note: If some additional information is disclosed from network side to UE side, proprietary concerns for option 4-1 and 3a-1 can not be precluded by default.

Agreement

For issues listed for inter-vendor collaboration direction B, further study the overhead in Direction B based on the size of the encoder (i.e., number of parameters and quantization level), the number of encoders, and how often the parameters need to be transferred.

Agreement

For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression,

- For Direction A:

  - Further study the following options with potential down-selection

    - 4-1: dataset is {target CSI, CSI feedback},

    - 3a-1: encoder parameter exchange, with {target CSI}

    - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

Agreement

- For Direction C:

  - It is clarified that, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side.

  - Study whether the fully specified reference models or their compatible models may provide reasonable performance for deployment at NW and UE

    - Including performance evaluation of training/finetuning the models by modeling data distribution mismatch between synthetic and field data, and/or by using actual field data

    - Including performance evaluation of training/finetuning the models by modelling data  distribution mismatch between NW-side field data and UE-side field data, and/or by using actual field data from both NW-side and UE-side

    - Note: Training/finetuning is done at either or both sides without inter-vendor collaboration.

  - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3)

  - FFS whether the reference model is the same as RAN4 specified model

  - FFS whether a single or multiple reference (and how many) models need to be specified

Agreement

For the evaluation studies of Direction A,

- For encoder parameter exchange from NW-side to UE-side of 3a-1,

  - Alt. 1: UE-side first trains a nominal decoder

    - Step 1: UE-side develops a nominal decoder against the exchanged encoder.

    - Step 2: UE-side develops actual encoder against the nominal decoder

  - Alt. 2: UE-side directly develops and trains the actual encoder.

  - Note: Target CSI is available at UE side

- For dataset exchange from NW-side to UE-side {target CSI, CSI feedback} of 4-1,

  - Alt. 1: UE-side first trains a nominal decoder

    - Step 1: UE-side develops a nominal decoder using the exchanged dataset.

    - Step 2: UE-side develops an actual encoder against the nominal decoder.

  - Alt. 2: UE-side directly develops and trains the actual encoder.

Agreement

For discussion on performance degradation due to UE-side / NW-side data distribution mismatch with respect to UE side additional condition (issue 4 and 6), consider

- NW-side data (dataset A) and UE-side data (dataset B) are mismatched in terms of UE-side additional condition (e.g., SVD phase normalization method, UE antenna virtualization, antenna imbalance, antenna spacing / layout, etc)

  - Note: dataset A may or may not contain dataset B

  - Note: if dataset A includes dataset B, this means NW timely collect data from the UE

  - Note: Dataset A and dataset B are aligned in terms of NW-side additional conditions.

- Case 1: An encoder-decoder pair is trained on dataset B. This serves as an upper-bound.

- Case 2 (for direction A):

  - For direction A: NW-side trains a decoder on dataset A, and UE-side trains an encoder based on either dataset A, dataset B, or both (where the choice depends on the inter-vendor collaboration Options (4-1 or 3a-1), the training alternatives (Alt 1 or Alt 2) within the Options, and the choice of target CSI for training)

- Case 3 (for direction B): NW-side train an encoder-decoder on dataset A and transfers the encoder parameters to UE.

- Study issue 4 and 6 by evaluating the inference performance for Case 2 / 3 on test dataset B, comparing to the inference performance for case 1 on test dataset B.

- Note: Evaluations are assumed to performed without common reference model structure among companies.

Agreement

- For the evaluation study of offline engineering for Direction C for Issue 9:

  - Case 1: The AI/ML model is trained based on training Dataset B, and then the AI/ML model performs inference/test on a dataset from test Dataset B.

    - Note: This serves as the upper-bound performance.

  - Case 2: The AI/ML model is trained based on training Dataset S, and then the AI/ML model performs inference/test on test Dataset B.

    - Note: This represents the performance of Direction C when not using field data.

  - Case 2A:

    - Step 1: The AI/ML model is trained based on training Dataset S

    - Step 2:

      - NW-side and/or UE-side may separately train new CSI reconstruction part and/or CSI generation part compatible to the AI/ML model from Step 1 based on training Dataset B, as agreed in Note 1-1, 1-2, and 1-3 as agreed in RAN1#118.

      - Dataset from Step 1 may be additionally used.

    - Step3: The CSI generation part and the CSI reconstruction part from Step 2 performs inference/test on test Dataset B

    - Note: This represents the performance of Direction C when the AI/ML models are updated based on field data.

  - Note: Dataset S is meant to represent the synthetic data used for fully specified model development, and Dataset B is meant to represent the field data.

    - Examples for Dataset S and B:

      - Modeling via TxRU mapping, UMi vs. UMa, UMa vs. InH, etc.

        - It is acknowledged that the scenario/configuration modeling between S and B may not be complete representative of the difference between synthetic and field data distribution mismatch.

      - Synthetic vs. actual field data (if available)

        - Companies are encouraged to provide details on the scenarios/configurations underlying the field data

  - Note: Case 1 and Case 2 have been studied during Rel-18 generalization study and conclusions have been captured in the TR.

  - Note: For Case 2A, companies may additionally evaluate UE-side / NW-side data distribution mismatch with respect to UE side additional condition. This can be done, in Step 2, by treating the training Dataset B as representing UE-side data distribution and considering a training Dataset A representing NW-side data distribution.

  - Note: Evaluations are assumed to performed without common reference model among companies.

// Additional study on AI/ML for NR air interface: Other aspects of AI/ML model and data

Agreement

Regarding model transfer/delivery Case z4 for inference, further study the following aspects:

- Whether/when UE needs the transfer of new parameters for a known model structure.

- Whether/when the AI model with the transferred parameters is ready for inference.

- The transfer of partial of the parameters for a known model structure.

Agreement

Regarding the study of model transfer/delivery Case z4, for a given known model structure, network can transmit the following information along with the parameters:

- a first indication referring to the known model structure at least for the case there are multiple known model structures

- a second indication referring to the transmitted parameters

- FFS: relationship of model ID, first indication, and second indication

- FFS: transfer partial parameters

Conclusion

From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized for two-sided model in Rel-19.

Agreement

Regarding the study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, ID (denoted as ID-X) can be transmitted from network/network-side to UE/UE-side for the dataset.

- Note: The notation “ID-X” is used for discussion purpose

#### 2.1.1.2	RAN1#119

// Specification support for beam management

Agreement

- For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A.

Conclusion

For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report.

Agreement

For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration,

- Two CSI-ResourceConfigId s are configured for Set A and Set B separately

Agreement

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

  - FFS on whether/what other information along with the applicability is needed

  - If A) is configured in Step 3,

    - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

    - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

  - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

Conclusion

For the CSI-ReportConfig for inference configuration provided in Step 5,

- aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- periodic CSI Report is considered as activated after RRCReconfigurationComplete.

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

  - Any specification impact is a separate discussion

Agreement

Send LS to RAN2 with below information.

RAN1 thanks RAN2 for the LS on applicable functionality reporting for beam management UE-sided model.

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs

- The concept/terminology “ functionality”  of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

RAN 1 made the following agreements related to the Questions from RAN 2:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

RAN1 would like to provide replies on the following questions from RAN2 in R2-2407848:

Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others?

Answer to Q1: In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID?

Answer to Q2: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Q3: Is NW-side additional condition functionality specific?

Answer to Q3: Please also refer to the answer to Q2 to understand the ongoing discussion about the associated ID for NW-side additional condition. And please refer to the agreements related to the Questions from RAN 2.

Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5):

Answer to Q4: And please refer to the agreements related to the Questions from RAN 2.

Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

Answer to Q4-1: There is no consensus yet on whether it is mandatory or optional. There is no conclusion yet on whether it is feasible or not for UE to decide the applicability without NW-side additional condition, and RAN 1 is discussing the related issues.

Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities?

Answer to Q4-2: Please refer to the agreements related to the Questions from RAN 2.

Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc?

Answer to Q4-3: Please refer to the agreements related to the Questions from RAN 2.

Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities?

Answer to Q4-4: Please refer to the agreements related to the Questions from RAN 2.

Q5: What is the content of applicable functionality reporting in Step 4?

Answer to Q5: Please refer to the agreements related to the Questions from RAN 2.

Q6: What is the content of inference configuration in Step 5?

Answer to Q6: Please refer to the agreements related to the Questions from RAN 2. The content of inference configuration as CSI-ReportConfig is to be designed later in RAN1.

Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3?

Answer to Q7: Please refer to the agreements related to the Questions from RAN 2.

Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5?

Answer to Q8: Please refer to the agreements/conclusion related to the Questions from RAN 2.

Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated?

Answer to Q9: Please refer to the agreements related to the Questions from RAN 2.

Q10: Is L1/L2 signaling for functionality activation/deactivation needed?

Answer to Q10:  Please refer to the agreements related to the Questions from RAN 2. With that, RAN1 understands that L1 and MAC signalling can be used for aperiodic CSI Report and semi-persistent CSI report.

// Specification support for positioning accuracy enhancement

Conclusion

For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator,

- LOS/NLOS indicator can’t be reported independently from other measurements

Agreement

For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows.

  - starting time = first detected path rounded down with timing granularity T.

Note: UE-side measurement is a separate discussion

Agreement

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE may signal the monitoring outcome to the LMF.

  - FFS: content of monitoring outcome

- FFS: Option B

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement,

- The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time, same as the path-based measurement.

- The Nt’ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values:

  - Nt'<=24. FFS: Nt' values.

  - Nt = {32, 64, 128}

  - FFS: k

  - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

- FFS: whether transmit offset from gNB to LMF

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Agreement

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following:

- Alternative 1. Info #7 is provided implicitly via associated ID.

  - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

  - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 3.  Info #7 is not be provided from LMF to UE.

  - If info #7 is not provided, UE may assume info #7 is consistent between training and inference.

- Alternative 4. Info #7 is provided explicitly from LMF to UE.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

// Additional study on AI/ML for NR air interface: CSI compression

Definitions:

- Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4)

  - Potential down-selection into one or more among sub-options 3a/5a-1, 3a/5a-2, 3a/5a-3, 4-1, 4-2, and 4-3 considering their feasibility and performance, including at least the following issues

- Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues

Conclusion

- For Direction A 4-1 and 3a-1, send LS to RAN2 for the study of feasibility of parameter / dataset exchange

Conclusion

Above LS, additionally CC SA2, SA3, SA5

Agreement

- For Direction A Option 3a-1 and Direction C, study the feasibility of scalable model structure specification over numbers of Tx ports, CSI feedback payload sizes, and bandwidths, number of slots.

  - Feasibility of Direction A Option 3a-1 is contingent on the feasibility of the scalable model structure specification.

  - Feasibility of Direction C is contingent on the feasibility of the scalable model structure specification and model parameters specification.

  - Note: Angular/Delay/Doppler domain conversion and quantization are considered to be part of feasibility study

Agreement

For studying the standardized model structure,

- For temporal domain Case 0,

  - In case of spatial-frequency domain input, adopt Transformer as the backbone structure.

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 2,

  - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation with additional layers

      - Conv-LSTM

      - LSTM

    - Option 2: Latent adaptation on top of Case 0 model structure

      - Reusing Case 0 structure and changing quantization operation and/or feedback payload over different slots

    - Note: other options are not precluded

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 3,

  - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation

    - Option 2: Latent adaptation

  - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input.

- Take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output, for the study of standardized model structure.

  - Note: Processing steps at the UE to derive angular, delay, and/or Doppler domain basis, and corresponding reconstruction steps at the gNB are considered.

  - Note: scalar/vector quantization and dequantization are considered. Quantization-aware training with jointly updated quantization method/parameters (Case 2-2) is used.

Agreement

For NW to collect data for training, study following spec impacts

- Data format: codebook-based Rel-16 eType2 or Rel-18 eType2 for PMI prediction.

  - FFS if enhancement is needed

  - FFS number of samples in the report.

  - FFS whether channel or precoder is needed for temporal Cases 3

- Configuration of rank/layer, number of subbands

- Mechanism for ground-truth reporting

- FFS: Report additional information regarding the samples, e.g., data quality, FFS the definition of data quality and corresponding parameters.

- FFS if enhancements in CSI-RS and SRS configuration is needed.

- FFS: Report associated information that captures UE side additional condition

- FFS: Configuration / reporting of temporal aspects for temporal Case 2 and Case 3, e.g., association between input and output CSI

- FFS: details of CSI measurement

For UE to collect data for training, study following spec impacts

- NW configuration or UE request, e.g., RS configuration/transmission for data collection

- Whether enhancements in CSI-RS configuration is needed.

- Configuration of temporal aspects for temporal case 2/3, e.g., association between input and output CSI

- FFS: Need of configuration of ID, and configuration of ID

Agreement

To capture all observations in section 2 of R1-2410725 to TR38.843.

Agreement

Draft LS R1-2410915 with the following updates:

Revising “The size becomes 600K * (2000 bits) / (8bits/Byte) = 150 MB+ 11.6 MB = 161.6MB if we assume N1=N2.” To “The size becomes N2 * (2000 bits) / (8bits/Byte) + 11.6 MB” , and

Revising

Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for

- Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Encoder parameter sharing

- Encoder parameter sharing + dataset sharing consisting of {target CSI}

to

Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options

- Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Encoder parameter sharing

- Encoder parameter sharing + dataset sharing consisting of {target CSI}

3.

Deleting

“RAN1 respectfully requests RAN2’s feedback on the feasibility of standardized signaling, considering the size of dataset / encoder parameters provided above.”

4.

Deleting

“The size is based on Case 0 of two-sided CSI compression; the size may or may not be larger for Case 2 and Case 3.” from “For Option 4-1: sharing {target CSI, CSI feedback} dataset:”

Agreement

Final LS R1-2410922 is endorsed.

Agreement

For temporal domain aspects Case 3, study LCM aspects and specification impacts,

consider the following options for training data collection

- Option 1: The target CSI for training is derived based on the predicted CSI of the future slot(s).

- Option 2: The target CSI for training is derived based on the measured CSI of the future slot(s).

- Note: During inference, the input to the CSI generation part is derived based on the predicted CSI.

consider following options for the monitoring labels

- Option 1: The monitoring label is derived based on the predicted CSI of the future slot(s).

  - CSI prediction output is used as input to CSI generation part.

  - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately.

- Option 2: The monitoring label is derived based on the measured CSI of the future slot(s)

  - Option 2a: CSI prediction output is used as input to CSI generation part.

    - Note: This corresponds to end-to-end monitoring of CSI prediction and compression.

  - Option 2b: Measured CSI of the future slot(s) is used as input to CSI generation part for monitoring purpose.

    - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately.

Study how the functionality/model control (activation, deactivation, switching, and fallback) for CSI prediction and CSI compression interacts.

// Additional study on AI/ML for NR air interface: Other aspects of AI/ML model and data

Agreement

For study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model,

- ID-X can be used for pairing the UE-part and the NW-part of a two-sided model

- FFS: other information needed for pairing

Agreement

Regarding the relationship of model ID, first indication, and second indication for model transfer/delivery Case z4, further study the following options:

- Opt.1: model ID consists of the information of the first indication and the second indication

  - E.g., model ID is a combination of the first and second indications

- Opt.2: The second indication is assumed as the model ID

- Opt.3: Model ID is assigned by network and is separated from the first indication and the second indication

Conclusion

Regarding MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, from RAN1 perspective, how to construct the dataset, including whether a dataset constructed from one cell or from multiple cells is up to network implementation.

Conclusion

For the study of model delivery/transfer Case z4, if the model delivery/transfer is directly used for inference, the following options are identified as the candidate solutions to determine the readiness of AI model with the transferred parameters for inference (either or combination of the following options)

- Option 1: UE sends signaling to network to notify that the AI model with the transferred parameters is ready for activation

- Option 2: The AI model with the transferred parameters can be assumed ready for activation from a minimum applicable time after the completion of model delivery/transfer

#### 2.1.2	Remaining Open issues

- Finalize specification support for beam management

- Finalize specification support for positioning accuracy enhancements

- Finalize specification support for CSI prediction

- Finalize study on CSI compression

- Finalize study on other aspects of AI/ML model and data with RAN1 impact

## 2.2	RAN2

#### 2.2.1	Agreements

##### 2.2.1.1	RAN2#127-bis

RAN2#127-bis discussed LCM for UE-sided model for Beam Management use case and made the following agreements:

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

RAN2#127-bis discussed NW-side data collection for the beam management use case, and made the following agreements:

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported

- Periodic reporting of logged data is not supported

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress

RAN2#127-bis discussed LCM for UE-sided model for Positioning use case and made the following agreements:

- The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline:

  - Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message.

  - Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side.

  - Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).

  - Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message.

  - Step 5: The LMF requests the inferred location information using the LPP request location information message.

  - Step 6: UE reports the inferred location using LPP provide location information message.

- Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress).

- Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress).

- FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively).  FFS the signalling details.

- RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

RAN2#127-bis discussed the topic of model transfer/delivery, mainly focusing on the need to define any 3GPP requirement related to the procedure of transferring/delivering a model to the UE. However, no agreement was reached.

The following topics were not discussed in RAN2#127-bis:

- LCM for NW-sided model for Beam Management use case

- UE-side data collection

Further, the following post-meeting email discussion was agreed:

[POST127bis][020][AI PHY] Reply LS to SA2/SA5 (InterDigital/Nokia)

Intended outcome:  Address/discuss SA2 questions from SA2/SA5 LS (if it is sent to RAN2) and possible answers.  The discussion is based on RAN2 understanding and previously made agreements.   No Tdocs should be submitted to the meeting

Deadline:  Nov. 9th, 10 UTC

##### 2.2.1.2	RAN2#128

RAN2#128 discussed LCM for UE-sided model for Beam Management use case and made the following agreements:

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design.

- Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

- Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

  - The network can decide when to start/stop the data collection and send configuration.

  - The network can configure whether UE is allowed to initiate request for data collection.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration.

RAN2#128 discussed NW-side data collection for the beam management use case, and made the following agreements:

- Focus on the following three radio condition event based logging:

  - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2)

  - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value)

  - L1 beam level measurement

- Measurements on aperiodic CSI resources are not reported for NW sided data collection.

- Data collection is controlled by the network.   The UE will not autonomously stop when low power state is detected.

- The UE reports to the network when the power state is low.  We will not specify how the UE determines low power state.   The network should de-configure the data collection (this can be captured in stage 2).

- The UE reports to the network when buffer is or may become full.  FFS when it reports (before and/or after).

- The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory).  FFS how this is signalled and if the reporting can be part of availability indication.

Related to this topic, RAN2 has also agreed to have an email discussion with the intention to discuss the motivation and specification complexity of the three radio conditions listed above, related to the event-based logging.

RAN2#128 discussed LCM for UE-sided model for Positioning use case and made the following agreements:

- For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline.

  - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF.

  - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF.  Configuration details are FFS.

- As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause.  FFS if other fallback options are considered.

Related to this topic, RAN2 has also agreed to have an email discussion with the intention to discuss the FFS from the existing agreements, focusing in particular on the FFSs that do not need RAN1 inputs, and identifying questions to ask RAN1 for resolving those FFSs which are dependent on RAN1 progress.  Further, the email discussion will also attempt to collect general guidelines on the criteria for deciding whether to enhance legacy method(s) or introduce new method.

RAN2#128 discussed the topic of UE-side data collection, focusing in particular on how to reply to the SA2 LS in S2-2411191 and to the SA5 LS in S5-246299.  The content of the reply LSs was first discussed in a email discussion (R2-2410504) and finally agreed in R2-2411152 (reply LS to SA2), and R2-2411114 (reply LS to SA5).

The following topics were not discussed in RAN2#128:

- LCM for NW-sided model for Beam Management use case

- Model transfer/delivery

Further the following post-meeting email discussions were agreed:

[POST128][019][AI PHY] NW side data collection (Nokia)

Intended outcome: Discuss the motivation and specification complexity for the three radio conditions.

Deadline:  Long (Jan.  31st , 10:00 UTC)

[POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson)

Intended outcome:

•		Discuss the FFS from the existing RAN2 Agreements

- Attempt to resolve the FFS which does not need RAN1 input

- Identify questions to ask to RAN1 for resolving FFS waiting on RAN1 progress

•		Collect general guidelines on the criteria for deciding whether to enhance legacy method(s) or introduce new method

Deadline: long (Jan.  31st , 10:00 UTC)

#### 2.2.3	Remaining Open issues

Related to the topic of LCM for UE-sided model for Beam Management use case, RAN2 can continue analyzing the details of LCM-related signalling procedures, focusing on the FFS agreed during RAN2#128. Additionally, RAN2 should discuss the RAN1 reply to the RAN2 LS (R2-2407848) about the applicability functionality reporting, and evaluate the impacts in RAN2 protocols.

Related to the topic of LCM for NW-sided model for Beam Management use case, RAN2 can monitor the progress in RAN1 in order to assess impacts, if any, in RAN2 specifications.

Related to the topic of NW-side data collection, RAN2 can continue the discussion on configuration and reporting mechanisms to enable NW-side data collection, focusing on the FFS agreed during RAN2#128. Further, RAN2 should start discussing how to address in Rel.19 the impact of UE mobility in the NW-side data collection procedures, e.g. whether the UE should discard or retain the logged data upon executing a mobility procedure.

Related to the topic of LCM for Positioning use case, RAN2 can continue the discussion on how to handle dynamic capabilities considering the FFFs from RAN2#128, such as the configurations details and the fallback options when the AIML based positioning method becomes non-applicable. Further, RAN2 should identify issues (if any) from which RAN1 feedback is needed.

Related to the topic of model transfer/delivery, RAN2 can continue the discussion trying to identify requirements (if any).

Related to the topic of two-sided AI/ML CSI compression, RAN2 can start addressing the RAN1 LS (R1-2410922) on signalling feasibility of dataset and parameter sharing, for which RAN1 expects a RAN2 feedback.

Related to the topic of UE-side data collection, whether further discussions are needed in RAN2 depends on RAN#106 discussion/progress.

## 2.3	RAN3

#### 2.3.1	Agreements

##### 2.3.1.1	RAN3#125-bis

RAN3#125-bis discussed the topic of Positioning Accuracy Enhancements, and made the following agreements:

- The existing LCS framework is re-used for the work on AI/ML based positioning.

- For case 3b:

  - Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b.

  - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

  - The details of the related measurements are still pending and subject to further discussion in RAN1.

- For case 3a:

  - The AI/ML model (inference and training functions) is at the gNB.

  - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

  - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope

  - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1

  - Working assumption is that RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model

##### 2.3.1.2	RAN3#126

RAN3#126 discussed the topic of Positioning Accuracy Enhancements, and made the following agreements:

- RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model

- Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

- For the case 3a, the working assumption is that the LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

RAN3 has also agreed that the common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used.

Further for model training in the case 3a, RAN3 has discussed the following options related to how the RAN node requests the training data towards LMF:

- Solution 1a: gNB-triggered Data Collection Request via a new NGAP message

- Solution 1b: gNB-triggered Data Collection Request via a new NRPPa message

- Solution1c: gNB-triggered Data Collection Request via the existing NRPPa message

- Solution 2: “Xn-like Data Collection”over NRPPa

RAN3 has also discussed the LS reply to the SA2 LS on AIML data collection. The LS reply was agreed in R3-247801

#### 2.3.2	Remaining Open issues

Related to the topic of Positioning Accuracy Enhancements, for model training in the case 3a, RAN3 should continue the discussion on how the RAN node requests the training data towards LMF. Further, the following FFSs discussed during RAN3#126 should be addressed:

- FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1).

- FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

- FFS if LMF should be aware that the reported measurements are AI/ML generated.

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	RAN4#112bis

Issue 1-2: Post deployment testing

Agreement:

When operating in the field, two aspects of UE operation may impact performance:

- Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance

- Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance.

For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive:

- Option 1: Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs

  - Validation takes into account the UE hardware in which the model is to be deployed/activated.

  - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model)

  - FFS whether validation can be done at the device along with inference for another active functionality(ies)

  - FFS on the feasibility

  - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models.

  - Other aspects not precluded.

- Option 2: Using performance monitoring and LCM procedures

  - Performance monitoring will be designed in other groups

  - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.

  - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable

FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device.

For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed.

  - Performance monitoring will be designed in other groups

  - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.

  - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

Issue 1-4: AI/ML Performance

Agreement:

- For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists.

  - FFS which legacy requirement/feature are relevant

  - FFS whether AI/ML based performance should be better than the legacy

- FFS on target for AI/ML based beam management

Issue 2-3: RSRP Absolute accuracy

Agreement:

- If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

Issue 2-5: Requirements for beam predictions

Agreement:

- Requirements to be defined:

  - Measurement period for prediction (how long will set B be measured before making a prediction)

    - FFS on whether the legacy measurement period can be reused or not.

  - Prediction accuracy

    - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc)

  - TCI state known/unknown

- FFS on whether the other requirements are needed

Issue 2-9: Test setup requirements

Agreement:

- For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes.

  - Set A and B should be generated in the chamber for ground-truth extraction.

- For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes.

  - Set A and B should be generated in the chamber for ground-truth extraction.

    - For single AoA, FSS how to derive Set A

Issue 2-6:	Measurement error impact evaluation

Agreement:

RAN4 to perform study to assess the impact of measurement errors on prediction accuracy

Perform LLS to evaluate the RSRP prediction accuracy and other KPIs

- UE Model trained using ideal measurements

Simulation assumptions in R4-2417211 can be used as starting point.

Issue 3-1: Requirements for case 1

Agreement:

Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible

Issue 3-3: Report applicability for existing reported metrics

Agreement:

Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b.

Issue 4-1: Simulation results discussion for Option 3

Agreement:

Next step:

Interested companies to share decoder and encoder ML models and dataset in next meeting (RAN4#113)

- Model and dataset used to generate the data submitted to RAN4#112-bis

Additional data to be submitted:

- CDF of SGCS per subband

- CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format)

Other agreements for testability and interoperability issues for CSI compression and CSI prediction

Agreement:

Interested companies are invited to bring further proposals for the next steps for the feasibility study of Option 4(for any of the sub-options of Option 4)

- Analysis of commonality with option 3(whether any steps or data generated for Option 3 can be reused) would be useful

AIML two-sided model logistical issues

Agreement:

All agreements for data sharing are only for the CSI compression use case feasibility study. These agreements can be revisited in an eventual WI phase.

Agreement

For AI/ML model formats:

- Use at least ONNX format for sharing.

- Companies can also share PyTorch model

- The models in different formats from one company shall be the same

- Use version v1.16 or later

- ONNX version and opset version number to be included in the file

Agreement:

For dataset format

- Use NumPy for dataset sharing

- Do not use pickled data(compression mechanism)

- Use npy – single array in each file

Agreement:

Dataset/model input file format:

    - N (samples) X 2 (IQ) X nSB (number of subbands) X nPorts (number of CSI-RS ports) X nLayers (number of layers)

        - In other words, dataset should content following info:

        - 1st dimension: Number of samples

        - 2nd dimension: Real and imaginary

        - 3rd dimension: Number of sub-bands

        - 4th dimension: Number of antenna ports

        - 5th dimension: Number of layers – present only if >1 layer

        - Note: Each element of the dataset will be a float32 real number

The dataset includes eigenvectors calculated based on average covariance channel matrix for the given MIMO layer for each sub-band based on pre-processing assumptions in WF R4-2414447

Single file for fixed scalar quantization(only dataset), TBD whether additional file is needed for variable/trainable quantization

Dataset files can be split into multiple files to enable easier upload.

- Use the 2 digits for split files starting from 00, increment for each additional file

Split files and then archive each file

Agreement:

For file naming scheme

- Folders for AI/ML data sharing and current WI/use case to be created under “RAN4 folder”

- Subfolder created for each meeting

File naming scheme( ML model file and dataset file)

- a unique identifier for the company (4 characters – list to be maintained by RAN4 secretary)

- meeting number

- differentiate model and dataset by identifier (2 characters ml and ds)

- additional identifier with 2 characters (TBD)

  - ec for encoder

  - dc for decoder

  - ei for encoder input

  - do for decoder output

  - lt for latent (decoder input or encoder output)

  - others to be added as needed

- dataset can be split in multiple files – 2 digits

- files to be compressed to zips and uploaded

Agreement:

Files to be shared, to be decided based on option under study

- 	Dataset containing encoder input per subband

- 	Encoder and/or decoder model

##### 2.4.1.2	RAN4#113

Scope of LCM discussion at the current stage

Agreement:

	The discussion should consider use-cases specific aspects

Agreement:

- Whether transition from AI to non-AI would be UE initiated or network initiated

- This will be a decision for other groups

- RAN4 to assess what RRM requirements are needed for each case.

Issue 1-2: Option 3 next step - decoder(s) selection

Agreement:

Choose the decoders with Low, medium, high SGCS among the decoders submitted by companies.

Continue the feasibility study on the two tracks below:

Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset)

- The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders

- At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s).

- At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

Agreement:

- Deadline for the interested companies to share at least trained decoders for Option 3 into the 3GPP server is 6th December, 2024.

  - The trained decoder/encoder/dataset can also be shared

Issue 1-3: Option 4a(Dataset based) for 2-sided model

Agreement:

    - Step 1-3: Reuse results of Option 3

    - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3

      - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1

      - Or mixed dataset from track 2

    - Step 5: Label selected dataset(s) (mapping of encoder input to latent message) [using encoder] corresponding to the selected test decoder(s) from option 3

    - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s)

      - Performance alignment to be checked/discussed

    - Step 7: Conclude on overall feasibility of Option 4a

      - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ).

Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders(using different structures)

Selected test decoder and matching encoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated/mixed dataset

Issue 1-4: Option 4b (encoder based) for 2-sided model

Agreement:

- Step 1-3: Reuse results of Option 3

- Step 4: Select one or more encoder(s) for further analysis based on option 3

  - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1

  - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2

- Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s)

  - Performance alignment to be checked/discussed

- Step 6: Conclude on overall feasibility of Option 4b

  - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ).

Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

Issue 2-1: Channel model for CSI prediction tests

Agreement:

- For channel model for CSI prediction tests, TDL channel models will be used.

  - FFS on whether CDL channel model is needed for generalization test.

Issue 2-2: Reference throughput

Agreement:

- reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook

Issue 2-3: Testing conditions

Agreement:

- use static conditions as baseline.

  - FFS on whether to have non-static condition

Test setup for Beam prediction testing

Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs.

Issue 3-2: UE Rx beam knowledge

Agreement:

- Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS

- RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not.

  - Other way to know UE Rx beams is not precluded

    - UE capability is one of the solutions

Measurement error impact evaluation for beam prediction

Agreement:

Updated simulation assumptions for measurement error impact evaluation for beam prediction is approved in R4-2420480

Issue 3-5: Prediction delay requirements

Agreement:

- Prediction delay includes:

  - Measurement for prediction

    - FFS on whether the legacy measurement delay can be reused

  - Inference delay

  - Other components are FFS

#### 2.4.2	Remaining Open issues

- General aspects

  - Post deployment handling

  - Test coverage and generalization

  - LCM handling

  - Requirement goals

- Testability and interoperability issues for beam management

  - Metrics/KPIs for BM

  - RSRP Prediction accuracy

  - Reported measurements and ground truth related issues

  - Channel models

  - Test setup needs/requirements

  - OTA Test setup for beam management

  - Measurement error impact

  - UE reporting for network side models

  - Beam prediction and TCI state related issues

  - UE side beam prediction requirements impact

  - UE Rx beam knowledge

  - Prediction delay requirements

  - Support for network side model

- Testability and interoperability issues for positioning accuracy enhancement

  - Requirements for case 1/2a/2b

  - Conditions for case 1 position requirement

  - How to test a position requirement for case 1

  - Requirements relating to training data

  - LCM considerations

  - Prioritization of case 2a/2b

  - Need for requirements for case 3a/3b

  - Requirements for measurements and reported metrics/values

  - KPIs

- Testability and interoperability issues for CSI compression

  - Option 3 simulation results for 2nd round

  - Option 3 next step for 2-sided model

  - Option 4a next steps

  - Option 4b next steps

  - Option 4c next steps

  - Reference encoder and/or dataset for Option 3, test decoder verification

- Testability and interoperability issues for CSI prediction

  - Channel model for CSI prediction tests

  - Reference throughput

  - Testing conditions

- AI/ML two-sided model logistical issues

  - Training/test/inference data

  - Scalar quantization

  - Data retention policy

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

### 4.1	RAN1

#### 4.1.1 	RAN1#118bis

// Specification support for beam management

R1-2409118	FL summary #4 for AI/ML in beam management	Samsung (Moderator)

R1-2409117	FL summary #3 for AI/ML in beam management	Samsung (Moderator)

R1-2409116	FL summary #2 for AI/ML in beam management	Samsung (Moderator)

R1-2409115	FL summary #1 for AI/ML in beam management	Samsung (Moderator)

R1-2409114	FL summary #0 for AI/ML in beam management	Samsung (Moderator)

R1-2407616	Discussion on specification support for AI/ML-based beam management	FUTUREWEI

R1-2407653	Discussion on AI/ML for beam management	Huawei, HiSilicon

R1-2407690	Specification Support for AI/ML for Beam Management	Kyocera

R1-2407694	Discussion on AIML for beam management	Spreadtrum Communications

R1-2407728	Discussion on AI/ML for beam management	China Telecom

R1-2407746	Specification support for beam management	Tejas Network Limited

R1-2407796	Discussion on AI/ML-based beam management	ZTE Corporation, Sanechips

R1-2407848	Specification support for beam management	vivo

R1-2407892	Discussion on specification support for beam management	CMCC

R1-2407938	Discussion on AIML beam management	TCL

R1-2407950	Specification support for beam management	Xiaomi

R1-2407988	AI/ML based Beam Management	Google

R1-2408027	Discussion on AI/ML-based beam management	CATT

R1-2408103	Discussion on specification support for beam management	Fujitsu

R1-2408116	Discussion on specification support for AI/ML beam management	Transsion Holdings

R1-2408158	On specification for AI/ML-based beam management	OPPO

R1-2408221	Discussion on specification support for beam management	NEC

R1-2408268	AI/ML for beam management	Ericsson

R1-2408279	Discussion on AI/ML for beam management	InterDigital, Inc.

R1-2408289	Specification support for AI/ML for beam management	Intel Corporation

R1-2408332	Discussions on AI/ML for beam management	LG Electronics

R1-2408365	Specification support for beam management	Fraunhofer HHI, Fraunhofer IIS

R1-2408380	Discussion on specification support for beam management	Ruijie Networks Co. Ltd

R1-2408390	Specification support for AI-enabled beam management	NVIDIA

R1-2408401	Discussions on AI/ML for beam management	Sony

R1-2408428	AI/ML specification support for beam management	Lenovo

R1-2408452	Discussion on AI/ML beam management	Apple

R1-2408533	Discussion on specification support for beam management	Panasonic

R1-2408544	AI/ML for Beam Management	Nokia

R1-2408558	Discussion on specification support for beam management	ETRI

R1-2408608	Discussions on specification support for beam management	Sharp

R1-2408630	Discussion for supporting AI/ML based beam management	Samsung

R1-2408689	Discussion on Beam management	Rakuten Mobile, Inc

R1-2408690	Specification Support for AI/ML for Beam Management	Kyocera Corporation

R1-2408704	Discussion on specification support for AIML-based beam management	MediaTek Inc.

R1-2408749	AI/ML for Beam Management	Meta

R1-2408753	Discussion on AI/ML based beam management	KT Corp.

R1-2408773	Discussion on AI/ML for beam management	NTT DOCOMO, INC.

R1-2408806	Discussions on AI/ML for beam management	CAICT

R1-2408823	Specification support for beam management	KDDI Corporation

R1-2408837	Specification support for AI-ML-based beam management	Qualcomm Incorporated

R1-2408887	On Associated ID for Beam Management Use Case	NTU

R1-2408922	Discussion on Incoming LS on applicable functionality reporting for beam management with UE-sided model	Indian Institute of Tech (M), IIT Kanpur

R1-2408959	Specification support for AI/ML beam management	ITL

// Specification support for positioning accuracy enhancement

R1-2409032	Summary #5 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2409031	Summary #4 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2409030	Summary #3 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2409029	Summary #2 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2409028	Summary #1 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2407649	AI/ML for Positioning Accuracy Enhancement	Ericsson

R1-2407654	Discussion on  AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2407747	Specification support for positioning accuracy enhancement	Tejas Network Limited

R1-2407797	Discussion on AI/ML-based positioning enhancement	ZTE Corporation, Pengcheng Laboratory

R1-2407849	Specification support for positioning accuracy enhancement	vivo

R1-2407893	Discussion on specification support for positioning accuracy enhancement	CMCC

R1-2407951	Discussion on AI/ML-based positioning accuracy enhancement	Xiaomi

R1-2407989	AI/ML based Positioning	Google

R1-2408028	Discussion on AI/ML-based positioning	CATT, CICTCI

R1-2408104	Discussion on specification support for AI/ML-based positioning accuracy enhancement	Fujitsu

R1-2408159	On specification for AI/ML-based positioning accuracy enhancements	OPPO

R1-2408214	Discussion on specification support for AIML based positioning accuracy enhancement	NEC

R1-2408267	Discussion on specification support for positioning accuracy enhancement	TCL

R1-2408290	Specification support for AI/ML for positioning accuracy enhancement	Intel Corporation

R1-2408309	Specification support for positioning accuracy enhancement	Baicells

R1-2408381	Discussion on specification support for positioning accuracy enhancement	Ruijie Networks Co. Ltd

R1-2408385	AI/ML positioning accuracy enhancement	Fraunhofer IIS, Fraunhofer HHI

R1-2408391	Specification support for AI-enabled positioning	NVIDIA

R1-2408402	Support for AI/ML for positioning accuracy enhancement	Sony

R1-2408429	Specification impacts for AI/ML positioning	Lenovo

R1-2408453	Discussion on Specification Support for AI/ML-based positioning	Apple

R1-2408522	Discussion on support for AIML positioning	InterDigital, Inc.

R1-2408545	AI/ML for Positioning Accuracy Enhancement	Nokia

R1-2408559	Discussion on specification support for positioning accuracy enhancement	ETRI

R1-2408609	Discussion on specification support for AI/ML based positioning accuracy enhancements	Sharp

R1-2408631	Discussion for supporting AI/ML based positioning accuracy enhancement	Samsung

R1-2408774	Discussion on AI/ML for positioning accuracy enhancement	NTT DOCOMO, INC.

R1-2408838   Specification support for AI-ML-based positioning accuracy enhancement	Qualcomm Incorporated

R1-2408903	Design for AI/ML based positioning	MediaTek Korea Inc.

R1-2408908	Discussions on specification support for positioning accuracy enhancement for AI/ML	ITL

R1-2408923	Discussion on specification support for AI/ML Positioning Accuracy enhancement	CEWiT

// Specification support for CSI prediction

R1-2409236	Summary #3 of CSI prediction	Moderator (LG Electronics)

R1-2409145	Summary #2 of CSI prediction	Moderator (LG Electronics)

R1-2409144	Summary #1 of CSI prediction	Moderator (LG Electronics)

R1-2407655	Discussion on AI/ML for CSI prediction	Huawei, HiSilicon

R1-2407695	Discussion on AIML for CSI prediction	Spreadtrum Communications

R1-2407798	Discussion on specification support for AI CSI prediction	ZTE Corporation, Sanechips

R1-2407850	Study on consistency issue for csi prediction	vivo

R1-2407894	Discussion on AI/ML for CSI prediction	CMCC

R1-2407939	Discussions on AIML CSI prediction	TCL

R1-2407952	Discussion on UE-side AI/ML model based CSI prediction	Xiaomi

R1-2407990	AI/ML based CSI Prediction	Google

R1-2408029	Discussion on AI/ML-based CSI prediction	CATT

R1-2408080	AI/ML for CSI prediction	Ericsson

R1-2408105	Discussion on specification support for CSI prediction	Fujitsu

R1-2408160	On specification for AI/ML-based CSI prediction	OPPO

R1-2408212	Discussion on specification support for CSI prediction	NEC

R1-2408247	AI/ML for CSI prediction	Mavenir

R1-2408333	Discussions on CSI prediction	LG Electronics

R1-2408362	Discussion on consistency of training / inference for AI/ML-based CSI prediction	Panasonic

R1-2408392	Specification support for AI-enabled CSI prediction	NVIDIA

R1-2408403	Input quantities for CSI prediction model training, inference and monitoring	Sony

R1-2408430	On AI/ML for CSI prediction	Lenovo

R1-2408436	On AI/ML-based CSI prediction	InterDigital, Inc.

R1-2408454	Discussion on AI based CSI prediction	Apple

R1-2408546	AI/ML for CSI Prediction	Nokia

R1-2408597	Discussion on Specification support for CSI prediction	Tejas Network Limited

R1-2408632	Views on AI/ML based CSI prediction	Samsung

R1-2408688	Discussion on CSI feedback enhancement	Rakuten Mobile, Inc

R1-2408694	Specification support for CSI prediction	MediaTek Inc.

R1-2408759	Discussion on AI/ML for CSI prediction	AT&T

R1-2408775	Discussion on AI/ML for CSI prediction	NTT DOCOMO, INC.

R1-2408839	Specification support for CSI prediction	Qualcomm Incorporated

R1-2408924	Discussion on AI/ML for CSI compression	CEWiT

// Additional study on AI/ML for NR air interface: CSI compression

R1-2409195	Draft summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2409160	Draft summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2409159	Draft summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2409158	Draft summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2409157	Draft summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2409156	Draft summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2407617	Discussion of additional study on AI/ML for NR air interface for CSI compression				FUTUREWEI

R1-2407656	Discussion on AI/ML for CSI compression	Huawei, HiSilicon

R1-2407696	Discussion on AIML for CSI compression	Spreadtrum Communications, BUPT

R1-2407748	CSI compression	Tejas Network Limited

R1-2407799	Discussion on study for AI/ML CSI compression	ZTE Corporation, Sanechips

R1-2407851	Discussion on CSI compression	vivo

R1-2407895	Discussion on AI/ML for CSI compression	CMCC

R1-2407940	Discussion on AIML CSI compression	TCL

R1-2407953	Views on two-sided AI/ML model based CSI compression	Xiaomi

R1-2407991	AI/ML based CSI Compression	Google

R1-2408030	Study on AI/ML-based CSI compression	CATT

R1-2408079	AI/ML for CSI compression	Ericsson

R1-2408084	Discussion on AI/ML for CSI compression	BJTU

R1-2408106	Discussion on CSI compression	Fujitsu

R1-2408161	Additional study on AI/ML-based CSI compression	OPPO

R1-2408213	Discussion on CSI compression	NEC

R1-2408291	AI/ML for CSI compression	Intel Corporation

R1-2408334	Study on CSI compression	LG Electronics

R1-2408363	Discussion on AI/ML for CSI compression	Panasonic

R1-2408393	Addtional study on AI-enabled CSI compression	NVIDIA

R1-2408431	On AI/ML for CSI compression	Lenovo

R1-2408437	On AI/ML-based CSI compression	InterDigital, Inc.

R1-2408455	Discussion on AI based CSI compression	Apple

R1-2408547	AI/ML for CSI Compression	Nokia

R1-2408560	Discussion on AI/ML for CSI compression	ETRI

R1-2408633	Views on additional study for AI/ML based CSI compression	Samsung

R1-2408695	Additional study on AI/ML for NR air interface - CSI compression	MediaTek Inc.

R1-2408776	Discussion on AI/ML for CSI compression	NTT DOCOMO, INC.

R1-2408807	Discussions on AI/ML for CSI compression	CAICT

R1-2408840	Additional study on CSI compression	Qualcomm Incorporated

R1-2408952	Discussion on AI/ML for CSI compression	IIT Kanpur

// Additional study on AI/ML for NR air interface: Other aspects of AI/ML model and data

R1-2409170 	Summary#3 for other aspects of AI/ML model and data	Moderator (OPPO)

R1-2409169 	Summary#2 for other aspects of AI/ML model and data	Moderator (OPPO)

R1-2409168	Summary#1 for other aspects of AI/ML model and data	Moderator (OPPO)

R1-2407618	Discussion on other aspects of AI/ML model and data on AI/ML for NR air-interface				FUTUREWEI

R1-2407657	Discussion on other aspects of the additional study for AI/ML	Huawei, HiSilicon

R1-2407697	Discussion on other aspects of AI/ML model and data	Spreadtrum Communications

R1-2407749	Other aspects of AI/ML model and data	Tejas Network Limited

R1-2407800	Discussion on other aspects of AI/ML model and data	ZTE Corporation, Sanechips

R1-2407852	Other aspects of AI/ML model and data	vivo

R1-2407896	Discussion on other aspects of AI/ML model and data	CMCC

R1-2407941	Discussions on other aspects of AlML In NR air interface	TCL

R1-2407954	Further study on AI/ML model and data	Xiaomi

R1-2407992	AI/ML Model and Data	Google

R1-2408031	Study on AI/ML for other aspects	CATT, CICTCI

R1-2408107	Discussion on other aspects of AI/ML model and data	Fujitsu

R1-2408162	Additional study on other aspects of AI/ML model and data	OPPO

R1-2408222	Discussion on other aspects of AI/ML model and data	NEC

R1-2408269	Discussion on other aspects of AI/ML	Ericsson

R1-2408292	Other aspects of AI/ML model and data	Intel Corporation

R1-2408335	Discussion on other aspects of AI/ML model and data	LG Electronics

R1-2408394	Additional study on other aspects of AI model and data	NVIDIA

R1-2408432	Discussion on other aspects of AI/ML model and data	Lenovo

R1-2408438	On other aspects of AI/ML model and data	InterDigital, Inc.

R1-2408456	Discussion on other aspects of AI/ML model and data	Apple

R1-2408541	Discussion on other aspects for AI/ML for air interface	Panasonic

R1-2408548	Other aspects of AI/ML for two-sided model use case	Nokia

R1-2408561	Discussion on other aspects of AI/ML model and data	ETRI

R1-2408634	Views on additional study for other aspects of AI/ML model and data	Samsung

R1-2408750	Other Aspects of AI/ML Model and Data	Meta

R1-2408751	Discussion on other aspects of AI/ML model and data	Sharp

R1-2408758	Other Aspects of AI/ML framework	AT&T

R1-2408777	Discussion on other aspects of AI/ML model and data	NTT DOCOMO, INC.

R1-2408841	Other aspects of AI/ML model and data	Qualcomm Incorporated

R1-2408885	Other aspects of AI/ML model and data	Continental Automotive

#### 4.1.2 	RAN1#119

// Specification support for beam management

R1-2410898  Reply LS on applicable functionality reporting for beam management UE-sided model	TSG RAN WG1

R1-2410893	Draft Reply LS on applicable functionality reporting for beam management UE-sided model			Moderator (Samsung)

R1-2410736	FL summary #3 for AI/ML in beam management	Moderator (Samsung)

R1-2410735	FL summary #2 for AI/ML in beam management	Moderator (Samsung)

R1-2410734	FL summary #1 for AI/ML in beam management	Moderator (Samsung)

R1-2410733	FL summary #0 for AI/ML in beam management	Moderator (Samsung)

R1-2409395	Discussion on AI/ML for beam management	Huawei, HiSilicon

R1-2409447	Discussion on AI/ML for Beam Management	Quectel

R1-2409455	Discussion on AI/ML for beam management	InterDigital, Inc.

R1-2409479	Discussion on AI/ML-based beam management	ZTE Corporation, Sanechips

R1-2409499	Discussion on specification support for beam management	CMCC

R1-2409569	Specification Support for AI/ML for Beam Management	Kyocera

R1-2409581	Discussion for supporting AI/ML based beam management	Samsung

R1-2409625	Discussion on AIML for beam management	Spreadtrum, UNISOC

R1-2409668	Specification support for beam management	vivo

R1-2409741	Specification support for beam management	Intel Corporation

R1-2409749	Specification support for beam management	Tejas Networks Limited

R1-2409780	Specification support for AI-enabled beam management	NVIDIA

R1-2409787	Discussion on AI/ML beam management	Apple

R1-2409840	Discussion on specification support for beam management	Ruijie Networks Co. Ltd

R1-2409855	Discussion on specification support for beam management	NEC

R1-2409877	Discussion on AI/ML for beam management	Xiaomi

R1-2409925	Specification support for AI/ML-based beam management	CATT, CBN

R1-2409957	Discussion on specification support for beam management	Panasonic

R1-2409960	Discussion on specification support for AI/ML beam management	Transsion Holdings

R1-2409985	AI/ML for Beam Management	Nokia

R1-2409994	Discussion on AI/ML for beam management	China Telecom

R1-2410018	AI/ML specification support for beam management	Lenovo

R1-2410029	Discussion on specification support for AI/ML-based beam management	FUTUREWEI

R1-2410048	Discussion on specification support on AI/ML for beam management	Fujitsu

R1-2410101	On specification for AI/ML-based beam management	OPPO

R1-2410149	AI/ML based Beam Management	Google

R1-2410174	Disscussion on AI/ML for beam management	HONOR

R1-2410185	FL plan for mobility enhancements in RAN1#119	Moderator (Fujitsu)

R1-2410193	Discussions on AI/ML for beam management	LG Electronics

R1-2410204	Discussion on AIML beam management	TCL

R1-2410216	Discussion on specification support for beam management	Sony

R1-2410255	Specification support for beam management	Fraunhofer HHI, Fraunhofer IIS

R1-2410257	Discussion on specification support for beam management	ETRI

R1-2410344	Discussion on AI/ML based beam management	KT Corp.

R1-2410347	AI/ML for Beam Management	Meta

R1-2410354	AI/ML for beam management	Ericsson

R1-2410359	Discussions on specification support for beam management	Sharp

R1-2410367	Discussions on AI/ML for beam management	CAICT

R1-2410373	Discussion on Specification Support of AI/ML for Beam Management	Indian Institute of Tech (M), IIT Kanpur

R1-2410376	Discussion on AI/ML for beam management	NTT DOCOMO, INC.

R1-2410466	Specification support for AI-ML-based beam management	Qualcomm Incorporated

R1-2410504	Specification support for beam management	KDDI Corporation

R1-2410519	Discussion on specification support for AIML-based beam management	MediaTek Inc.

R1-2410547	Specification support for AI/ML beam management	ITL

R1-2410587	On Associated ID for Beam Management Use Case	NTU

// Specification support for positioning accuracy enhancement

R1-2410718	Summary #5 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2410717	Summary #4 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2410716	Summary #3 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2410715	Summary #2 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2410714	Summary #1 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2409396	Discussion on AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2409443	AI/ML for Positioning Accuracy Enhancement	Ericsson

R1-2409480	Discussion on AI/ML-based positioning enhancement	ZTE Corporation, Pengcheng Laboratory

R1-2409500	Discussion on specification support for positioning accuracy enhancement	CMCC

R1-2409543	Discussion on AI/ML for positioning accuracy enhancement	New H3C Technologies Co., Ltd.

R1-2409582	Discussion for supporting AI/ML based positioning accuracy enhancement	Samsung

R1-2409669	Specification support for positioning accuracy enhancement	vivo

R1-2409742	Specification support for positioning accuracy enhancement	Intel Corporation

R1-2409750	Specification support for positioning accuracy enhancement	Tejas Networks Limited

R1-2409781	Specification support for AI-enabled positioning	NVIDIA

R1-2409788	Discussion on Specification Support for AI/ML-based positioning	Apple

R1-2409841	Discussion on specification support for positioning accuracy enhancement	Ruijie Networks Co. Ltd

R1-2409845	Discussion on support for AIML positioning	InterDigital, Inc.

R1-2409852	Discussion on specification support for AIML based positioning accuracy enhancement	NEC

R1-2409878	Discussion on AI/ML-based positioning accuracy enhancement	Xiaomi

R1-2409926	Specification support for AI/ML-based positioning	CATT, CICTCI

R1-2409986	AI/ML for Positioning Accuracy Enhancement	Nokia

R1-2410019	Specification impacts for AI/ML positioning	Lenovo

R1-2410049	Discussion on specification support for AIML-based positioning accuracy enhancement	Fujitsu

R1-2410102	On specification for AI/ML-based positioning accuracy enhancements	OPPO

R1-2410150	AI/ML based Positioning	Google

R1-2410205	AI/ML positioning accuracy enhancement	Fraunhofer IIS, Fraunhofer HHI

R1-2410215	Discussion on specification support for positioning accuracy enhancement	TCL

R1-2410217	Support for AI/ML for positioning accuracy enhancement	Sony

R1-2410258	Discussion on specification support for positioning accuracy enhancement	ETRI

R1-2410360	Discussion on specification support for AI/ML based positioning accuracy enhancements	Sharp

R1-2410377	Discussion on AI/ML for positioning accuracy enhancement	NTT DOCOMO, INC.

R1-2410414	Discussion on specification support for AI-ML based positioning accuracy enhancement	Baicells

R1-2410424	Specification Support of AI/ML for Positioning Accuracy Enhancement	Indian Institute of Tech (M), IIT Kanpur

R1-2410467	Specification support for AI-ML-based positioning accuracy enhancement	Qualcomm Incorporated

R1-2410531	Design for AI/ML based positioning	MediaTek Korea Inc.

R1-2410571	Discussion on specification support for AI/ML Positioning Accuracy enhancement	CEWiT

R1-2410588	Discussions on specification support for positioning accuracy enhancement for AI/ML	ITL

// Specification support for CSI prediction

R1-2410818	Summary #2 of CSI prediction	Moderator (LG Electronics)

R1-2410817	Summary #1 of CSI prediction	Moderator (LG Electronics)

R1-2409397	Discussion on AI/ML for CSI prediction	Huawei, HiSilicon

R1-2409449	AI/ML for CSI prediction	Ericsson

R1-2409481	Discussion on specification support for AI CSI prediction	ZTE Corporation, Sanechips

R1-2409501	Discussion on AI/ML for CSI prediction	CMCC

R1-2409583	Views on AI/ML based CSI prediction	Samsung

R1-2409626	Discussion on AIML for CSI prediction	Spreadtrum, UNISOC

R1-2409670	Study on consistency issue for CSI prediction	vivo

R1-2409751	Discussion on study for AI/ML CSI prediction	Tejas Networks Limited

R1-2409782	Specification support for AI-enabled CSI prediction	NVIDIA

R1-2409789	Discussion on AI/ML-based CSI prediciton	Apple

R1-2409853	Discussion on specification support for CSI prediction	NEC

R1-2409879	Discussion on AI/ML model based CSI prediction	Xiaomi

R1-2409927	Specification support for AI/ML-based CSI prediction	CATT

R1-2409987	AI/ML for CSI Prediction	Nokia

R1-2409995	Discussion on AI/ML for CSI prediction	China Telecom

R1-2410020	On AI/ML for CSI prediction	Lenovo

R1-2410042	On AI/ML-based CSI prediction	InterDigital, Inc.

R1-2410050	Discussion on specification support for CSI prediction	Fujitsu

R1-2410103	On specification for AI/ML-based CSI prediction	OPPO

R1-2410151	AI/ML based CSI Prediction	Google

R1-2410194	Discussions on CSI prediction	LG Electronics

R1-2410218	Further views on consistency issues in CSI prediction	Sony

R1-2410248	Discussion on consistency of training / inference for AI/ML-based CSI prediction	Panasonic

R1-2410259	Discussion on specification support for CSI prediction	ETRI

R1-2410336	Discussion on AI/ML for CSI prediction	AT&T

R1-2410378	Discussion on AI/ML for CSI prediction	NTT DOCOMO, INC.

R1-2410468	Specification support for CSI prediction	Qualcomm Incorporated

R1-2410537	AI/ML - Specification support for CSI Prediction	MediaTek Inc.

R1-2410564	AI/ML for CSI prediction	Mavenir

// Additional study on AI/ML for NR air interface: CSI compression

R1-2410922  Final LS on signalling feasibility of dataset and parameter sharing	Moderator (Qualcomm)

R1-2410915  Draft LS on signalling feasibility of dataset and parameter sharing	Moderator (Qualcomm)

R1-2410725  Evaluation summary of Additional study on AI/ML for NR air interface: CSI compression Moderator (Qualcomm)

R1-2410723  Draft summary of Additional study on AI/ML for NR air interface: CSI compression Moderator (Qualcomm)

R1-2410722  Draft summary of Additional study on AI/ML for NR air interface: CSI compression Moderator (Qualcomm)

R1-2410721  Draft summary of Additional study on AI/ML for NR air interface: CSI compression Moderator (Qualcomm)

R1-2410720  Draft summary of Additional study on AI/ML for NR air interface: CSI compression Moderator (Qualcomm)

R1-2410719  Draft summary of Additional study on AI/ML for NR air interface: CSI compression Moderator (Qualcomm)

R1-2409398	Discussion on AI/ML for CSI compression	Huawei, HiSilicon

R1-2409450	AI/ML for CSI compression	Ericsson

R1-2409482	Discussion on study for AI/ML CSI compression	ZTE Corporation, Sanechips

R1-2409502	Discussion on AI/ML for CSI compression	CMCC

R1-2409584	Views on additional study for AI/ML based CSI compression	Samsung

R1-2409627	Discussion on AIML for CSI compression	Spreadtrum, UNISOC

R1-2409671	Discussion on CSI compression	vivo

R1-2409743	AI/ML for CSI compression	Intel Corporation

R1-2409752	Discussion on AI/ML for CSI Compression	Tejas Networks Limited

R1-2409783	Addtional study on AI-enabled CSI compression	NVIDIA

R1-2409790	Discussion on AI based CSI compression	Apple

R1-2409854	Discussion on CSI compression	NEC

R1-2409880	Views on AI/ML model based CSI compression	Xiaomi

R1-2409928	Additional study on AI/ML-based CSI compression	CATT

R1-2409988	AI/ML for CSI Compression	Nokia

R1-2410021	On AI/ML for CSI compression	Lenovo

R1-2410030	Discussion of CSI compression on AI/ML for NR air interface	FUTUREWEI

R1-2410043	On AI/ML-based CSI compression	InterDigital, Inc.

R1-2410051	Discussion on CSI compression with AI/ML	Fujitsu

R1-2410104	Additional study on AI/ML-based CSI compression	OPPO

R1-2410152	AI/ML based CSI Compression	Google

R1-2410195	Study on CSI compression	LG Electronics

R1-2410201	Discussion on AI/ML for CSI compression	KAIST

R1-2410202	Discussion on AIML CSI compression	TCL

R1-2410249	Discussion on AI/ML for CSI compression	Panasonic

R1-2410260	Discussion on AI/ML for CSI compression	ETRI

R1-2410379	Discussion on AI/ML for CSI compression	NTT DOCOMO, INC.

R1-2410425	Discussion and evaluation results on AI/ML for CSI Compression	Indian Institute of Tech (M), IIT Kanpur

R1-2410469	Additional study on CSI compression	Qualcomm Incorporated

R1-2410508	Additional study on AI/ML for NR air interface - CSI compression	MediaTek Inc.

R1-2410572	Discussion on AI/ML for CSI compression	CEWiT

R1-2410589	Discussion an AI/ML based CSI Compression	IIT Kanpur, Indian Institute of Tech (M)

// Additional study on AI/ML for NR air interface: Other aspects of AI/ML model and data

R1-2410776	Summary#2 for other aspects of AI/ML model and data	Moderator (OPPO)

R1-2410775	Summary#1 for other aspects of AI/ML model and data	Moderator (OPPO)

R1-2409399	Discussion on other aspects of the additional study for AI/ML	Huawei, HiSilicon

R1-2409483	Discussion on other aspects of AI/ML model and data	ZTE Corporation, Sanechips

R1-2409503	Discussion on other aspects of AI/ML model and data	CMCC

R1-2409585	Views on additional study for other aspects of AI/ML model and data	Samsung

R1-2409628	Discussion on other aspects of AI/ML model and data	Spreadtrum, UNISOC

R1-2409672	Other aspects of AI/ML model and data	vivo

R1-2409731	Discussion on other aspects of AI/ML	Ericsson

R1-2409744	Other aspects of AI/ML model and data	Intel Corporation

R1-2409753	Discussion on Other aspects of  AI/ML model and data	Tejas Networks Limited

R1-2409784	Additional study on other aspects of AI model and data	NVIDIA

R1-2409791	Discussion on other aspects of AI/ML model and data	Apple

R1-2409856	Discussion on other aspects of AI/ML model and data	NEC

R1-2409881	Further study on AI/ML model and data	Xiaomi

R1-2409929	Additional study on AI/ML for other aspects	CATT, CICTCI

R1-2409989	Other aspects of AI/ML for two-sided model	Nokia

R1-2409996	Discussion on other aspects of AI ML model and data	China Telecom

R1-2410022	Discussion on other aspects of AI/ML model and data	Lenovo

R1-2410031	Discussion on other aspects of AI/ML model and data on AI/ML for NR air-interface				FUTUREWEI

R1-2410044	On other aspects of AI/ML model and data	InterDigital, Inc.

R1-2410052	Discussion on other aspects of AI/ML model and data	Fujitsu

R1-2410105	Additional study on other aspects of AI/ML model and data	OPPO

R1-2410153	AI/ML Model and Data	Google

R1-2410173	Discussion on other aspects for AI/ML for air interface	Panasonic

R1-2410192	Other aspects of AI/ML model and data	Continental Automotive

R1-2410196	Discussion on other aspects of AI/ML model and data	LG Electronics

R1-2410203	Discussions on other aspects of AlML In NR air interface	TCL

Late submission

R1-2410261	Discussion on other aspects of AI/ML model and data	ETRI

R1-2410335	Other Aspects of AI/ML framework	AT&T

R1-2410380	Discussion on other aspects of AI/ML model and data	NTT DOCOMO, INC.

R1-2410426	Discussion on other aspects of AI/ML model and data	Sharp

R1-2410470	Other aspects of AI/ML model and data	Qualcomm Incorporated

### 4.2	RAN2

#### 4.2.1	RAN2#127-bis

100 contributions (for details see agenda item 8.1 in Tdoc list)

#### 4.2.2	RAN2#128

84 contributions (for details see agenda item 8.1 in Tdoc list)

### 4.3	RAN3

#### 4.3.1	RAN3#125-bis

23 contributions (for details see agenda item 20 in Tdoc list)

#### 4.3.2	RAN3#126

26 contributions (for details see agenda item 20 in Tdoc list)

### 4.4	RAN4

// General aspects

R4-2414958	Discussion on general aspects of AIML for NR air interface

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2415169	Discussion on general aspects for AIML for NR air

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2415186	Discussion on general aspects for AIML

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2415253	Discussion on general aspects

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2415271	Discussion on general aspects for AI/ML air

Type: discussion		For: Discussion
					Source: Korea Testing Laboratory

Decision:		Noted.

R4-2415495	Discussion on General Aspects on AI/ML for NR Air Interface

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2415620	Discussion on general aspects on AIML for NR air interface

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2415687	Views on general aspects of AI/ML testability and interoperability

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2415930	Discussions on general aspects of AIML for NR Air Interface

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision:		Noted.

R4-2415964	On General Aspects of AI/ML for NR Air Interface

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2416004	Views on general aspect of AI/ML for NR air interface

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2416202	On general AI/ML aspects

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On general AI/ML aspects

Decision:		Noted.

R4-2416366	Discussion on the AI/ML general aspects

Type: other		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

// Study for CSI compression

// Testability and interoperability issues

R4-2415028	Testability and interoperability issues for CSI compression

Type: other		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2415072	Discussion on testability and interoperability issues for CSI compression

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2415174	Discussion on testability and interoperability issues for CSI compression

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2415187	Discussion on testability and interoperability issues for CSI compression

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2415251	Discussion on testability and interoperability issues for  CSI compression

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2415376	On AI/ML Interoperability and Standardization of Testing and Verification for CSI Use Case

Type: discussion		For: Discussion
					Source: NTU

Abstract:

MCC: This contribution from National Taiwan University provide views on partially specified decoder and reference decoder derivation procedure. Proposal 1: Revise option 4a and 4b to ensure test repeatability. Option 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors. Option 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors. Proposal 2: Study the feasibility of the partial specified test decoder with partial mapping alignment. Proposal 3: Additional margin to account for the mismatch. Consider the test decoder selection criterion based on the agreed step.

Decision:		Noted.

R4-2415384	Discussion on CSI compression Testability and interoperability

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2415498	Study for CSI Compression: Testability and Interoperability issues

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2415621	Discussion on testability and interoperability issues in AIML CSI compression

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2415688	Views on AI/ML testability and interoperability for CSI compression use case

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2415889	CSI Compression: Testability and Interoperability

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2415965	On AI/ML Based CSI Compression

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2416314	CSI compression considerations and simulation results

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Further considerations for 2-sided CSI compression and results for model alignment simulations

Decision:		Noted.

R4-2416349	Discussion on testability and interoperability issues for CSI compression

Type: other		For: Discussion
					Source: ZTE Corporation, Sanechips

Decision:		Noted.

R4-2416395	Discussion on test decoder for CSI compression

Type: discussion		For: Discussion
					Source: Rohde & Schwarz

Decision:		Noted.

R4-2416495	Further study on testability and interoperability issues for AI-CSI compression

Type: other		For: Discussion
					Source: Samsung

Decision:		Noted.

// AI/ML two-sided model logistical issues

R4-2415029	AIML two-sided model logistical issues

Type: other		For: Discussion
					Source: OPPO

Decision:		Noted.

R4-2415188	Discussion on AI/ML two-sided model logistical issues for CSI compression

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2415252	Discussion on AI/ML two-sided model logistical issues

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2415385	Discussion on CSI compression AI/ML two-sided model logistics

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2415500	Discussion on CSI Compression: AI/ML two-sided model logistical issues

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2415622	Discussion on AIML two-sided model logistical issues

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2415689	Views on logistics of AI/ML model and dataset sharing for CSI compression use case

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2415891	Logistical Issues of Two-Sided Model for AI/ML CSF

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2415895	Summary of e-mail discussion on data sharing for AI/ML

Type: other		For: Information
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2415966	On AI/ML CSI Compression Two-Sided Model Logistical Issues

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2416315	On AI/ML model sharing and logistics

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Observations on model sharing

Decision:		Noted.

R4-2416350	Discussion on two-side model logistical issues

Type: other		For: Discussion
					Source: ZTE Corporation, Sanechips

Decision:		Noted.

R4-2416496	Further discussion on AI/ML two-sided model logistical issues

Type: other		For: Discussion
					Source: Samsung

Decision:		Noted.

// CSI prediction Testability and interoperability issues

R4-2414959	Discussion on testability and interoperability issues for CSI prediction

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2415175	Discussion on testability and interoperability issues for CSI prediction

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2415189	Discussion on testability and interoperability issues for CSI prediction

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2415250	Discussion on testability and interoperability issues for  CSI prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2415358	Discussion on testability and interoperability of AI for CSI feedback enhancement

Type: discussion		For: Discussion
					Source: SEU

Decision:		Noted.

R4-2415386	Discussion on CSI prediction testability and interoperability

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2415499	Discussion on CSI Prediction: Testability and Interoperability issues

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2415623	Discussion on testability and interoperability issues in AIML CSI prediction

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2415871	Discussion on CSI prediction

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution shows our initial view on the AI/ML-based CSI prediction.

Decision:		Noted.

R4-2415890	CSI Prediction: Testability

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2415967	On AI/ML Based CSI Prediction

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2416351	Discussion on testability and interoperability issues for CSI prediction

Type: other		For: Discussion
					Source: ZTE Corporation, Sanechips

Decision:		Noted.

R4-2416494	Testability and interoperability issues for AI-CSI prediction

Type: other		For: Approval
					Source: Samsung

Decision:		Noted.

// RRM core requirements for beam management

R4-2414929	Discussion on core requirements of AI/ML BM use case

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision:		Noted.

R4-2415030	RRM core requirements for beam management

Type: other		For: Discussion
					Source: OPPO

Decision:		Noted.

R4-2415071	Discussion on core part requirement for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2415170	Discussion on RRM core requirements for BM

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2415190	Discussion on RRM core requirements for beam management

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2415249	Discussion on RRM requirements for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2415496	Discussion on Beam Management: RRM Core Requirements

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2415624	Discussion on RRM core requirements in AIML Beam management

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2415893	RRM Core Requirements for AI/ML Beam Management

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2416080	Discussion on RRM core requirements for beam management

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on RRM core requirements for beam management

Decision:		Noted.

R4-2416363	Discussion on the RRM requirements of AI/ML Beam management

Type: other		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2416439	RRM core requirements for AI/ML Based Beam Management

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2416492	Further discussion on beam management core requirements

Type: other		For: Discussion
					Source: Samsung

Decision:		Noted.

// Testability and interoperability issues for beam management

R4-2414930	Discussion on testability issues of AI/ML BM use case

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision:		Noted.

R4-2414960	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2415031	Testability and interoperability issues for beam management

Type: other		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2415070	Discussion on testability and interoperability issues for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2415171	Discussion on testability and interoperability issues for BM

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2415191	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2415248	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2415312	Discussion on testability issues for AI/ML beam management

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2415497	Discussion on Beam Management: Testability and Interoperability Issues

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2415625	Discussion on testability and interoperability issues in AIML Beam management

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2416081	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on testability and interoperability issues for beam management

Decision:		Noted.

R4-2416362	Discussion on the Interoperability and testability aspects of AI/ML Beam management

Type: other		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2416391	Discussion on test setup for AI/ML based beam management

Type: discussion		For: Discussion
					Source: ROHDE & SCHWARZ

Decision:		Noted.

R4-2416440	On Testability and Interoperability aspects for AI/ML Based Beam Management

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2416477	On Testability Aspects for FR2 AI/ML Beam Management

Type: other		For: Discussion
					Source: Keysight Technologies UK Ltd

Decision:		Noted.

R4-2416493	Further discussion on testability and interoperability issues for beam management

Type: other		For: Discussion
					Source: Samsung

Decision:		Noted.

// RRM core requirements for positioning accuracy enhancement

R4-2415172	Discussion on RRM core requirements for positioning

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2415192	Discussion on RRM core requirements for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2415255	Discussion on RRM requirements for positioning

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2415501	Discussion on RRM Core Requirements for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2415626	Discussion on RRM core requirements in AIML Positioning

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2415892	RRM Core Requirements for Positioning Accuracy

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2416330	On issues related to AI/ML for positioning

Type: other		For: Discussion
					Source: Ericsson

Abstract:

Discussion on requirements for AI/ML based positioning.

Decision:		Noted.

R4-2416365	Discussion on the RRM requirements of AI/ML positioning

Type: other		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2416447	RRM core requirements for AI/ML positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

// Testability and interoperability issues for positioning accuracy enhancement

R4-2415032	Testability and interoperability issues for positioning

Type: other		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2415173	Discussion on testability and interoperability issues for positioning

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2415193	Discussion on testability and interoperability issues for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2415254	Discussion on testability and interoperability issues for positioning

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2415502	Discussion on Testability and interoperability issues for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2415627	Discussion on testability and interoperability issues in AIML Positioning

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2416331	On testability of AI/ML for positioning

Type: other		For: Discussion
					Source: Ericsson

Abstract:

Discussion on testability aspects related to AI/ML based positioning.

Decision:		Noted.

R4-2416364	Discussion on the Interoperability and testability aspects of AI/ML positioning

Type: other		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2416448	Testability and interoperability issues for AI/ML positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

// Moderator summary and conclusions

R4-2415231	Topic summary for [112bis][127] NR_AIML_air

Type: other		For: Information
					Source: Moderator(Qualcomm)

Abstract:

Summary for AI 6.17

Decision:		Noted.

Newly allocated tdocs in the first round

R4-2417113	WF on requirements for AI/ML air interface

Type: other		For: Approval
					Source: Qualcomm

Decision:		Revised to R4-2417210 (from R4-2417113).

R4-2417210	WF on requirements for AI/ML air interface

Type: other		For: Approval
					Source: Qualcomm

Decision:		Revised to R4-2417212 (from R4-2417210).

R4-2417212	WF on requirements for AI/ML air interface

Type: other		For: Approval
					Source: Qualcomm

Decision:		Approved.

R4-2417187	WF on data sharing for AI/ML air interface

Type: other		For: Approval
					Source: Qualcomm

Decision:		Revised to R4-2417207 (from R4-2417187).

R4-2417207	WF on data sharing for AI/ML air interface

Type: other		For: Approval
					Source: Qualcomm

Decision:		Approved.

R4-2417188	Ad hoc minutes on AI/ML air interface

Type: other		For: Approval
					Source: Qualcomm

Decision:		Noted.

R4-2417189	Simulation assumptions for beam prediction

Type: other		For: Approval
					Source: Vivo

Decision:		Revised to R4-2417208 (from R4-2417189).

R4-2417208	Simulation assumptions for beam prediction

Type: other		For: Approval
					Source: vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

Decision:		Revised to R4-2417211 (from R4-2417208).

R4-2417211	Simulation assumptions for beam prediction

Type: other		For: Approval
					Source: vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

Decision:		Approved.

#### 4.4.2	RAN4#113

// General aspects

R4-2417612	Discussion on general aspects of AIML for NR air interface

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2417700	Discussion on general aspects for AIML for NR air

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2417785	Proposed update for TR 38.843 with RAN4 part

Type: discussion		For: Discussion
					Source: CAICT, Qualcomm, Ericsson

Abstract:

Initial proposed update for TR based on agreements till RAN4 #112bis.

Decision:		Noted.

R4-2417947	Discussion on general aspect for AI

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2418295	Discussions on general aspects of AIML for NR Air Interface

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision:		Noted.

R4-2418457	Discussion on general aspects

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2418478	Discussion on the AI/ML general aspects

Type: other		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2418520	Discussion on General Aspects on AI/ML for NR Air Interface

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2418747	Discussion on general aspects for AIML

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2418897	Views on general aspects of AI/ML testability and interoperability

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2418968	Discussion on general aspects on AIML for NR air interface

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2419177	On General Aspects of AI/ML for NR Air Interface

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2419433	Discussion on general aspects for AI/ML air

Type: discussion		For: Discussion
					Source: Korea Testing Laboratory

Decision:		Noted.

R4-2419705	On general AI/ML aspects

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On general AI/ML aspects

Decision:		Noted.

R4-2419754	Post Deployment Testing of AI-ML

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

Withdrawn

R4-2418392	Discussion on general aspects for AI/ML air

Type: discussion		For: Discussion
					Source: Korea Testing Laboratory

Decision: 		The document was withdrawn.

R4-2419655	On general AI/ML aspects

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On general AI/ML aspects

Decision: 		The document was withdrawn.

// Testability and interoperability issues for CSI compression

R4-2417615	Discussion on testability and interoperability issues for CSI compression

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2417618	Testability and interoperability issues for CSI compression

Type: other		For: Discussion
					Source: OPPO

Decision:		Noted.

R4-2417701	Discussion on testability and interoperability issues for CSI compression

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2417803	Discussion on CSI compression Testability and interoperability

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2417950	Discussion on testability and interoperability issues for CSI compression

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2418095	Further study on testability and interoperability issues for AI-CSI compression

Type: other		For: Approval
					Source: Samsung

Decision:		Noted.

R4-2418455	Discussion on testability and interoperability issues for  CSI compression

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2418523	Study for CSI Compression: Testability and Interoperability issues

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2418748	Discussion on testability and interoperability issues for CSI compression

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2418898	Views on AI/ML testability and interoperability for CSI compression use case

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2418969	Discussion on testability and interoperability issues in AIML CSI compression

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2419171	Discussion on testability and interoperability issues for CSI compression

Type: other		For: Discussion
					Source: ZTE Corporation, Sanechips

Decision:		Noted.

R4-2419178	On AI/ML Based CSI Compression

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2419179	Additional Data for Alignemnt for CSI Compression Use-Case Study

Type: other		For: Discussion
					Source: Nokia

Abstract:

MCC: This was not made available by submission deadline.

Decision:		Noted.

R4-2419345	CSI compression evaluation of option 3 and option 4

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Proposals for the evaluation of option 3 and for option 4

Decision:		Noted.

R4-2419365	On AI/ML Interoperability and Standardization of Testing and Verification for CSI Use Case – A PCA based approach

Type: discussion		For: Discussion
					Source: NTU

Decision:		Noted.

R4-2419749	CSI Compression: Testability and Interoperability

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

Withdrawn

R4-2419093	On AI/ML Interoperability and Standardization of Testing and Verification for CSI Use Case – A PCA based approach

Type: discussion		For: Approval
					Source: NTU

Decision: 		The document was withdrawn.

// AI/ML two-sided model logistical issues

R4-2417804	Discussion on CSI compression AI/ML two-sided model logistics

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2418096	Further discussion on AI/ML two-sided model logistical issues

Type: other		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2418456	Discussion on AI/ML two-sided model logistical issues

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2418525	Discussion on CSI Compression: AI/ML two-sided model logistical issues

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2418749	Discussion on AI/ML two-sided model logistical issues for CSI compression

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2418970	Discussion on AIML two-sided model logistical issues

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2419180	On AI/ML Two-Sided Model Logistical Issues

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2419751	Logistical Issues of Two-Sided Model for AI/ML CSF

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// Testability and interoperability issues for CSI prediction

R4-2417616	Discussion on testability and interoperability issues for CSI prediction

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2417702	Discussion on testability and interoperability issues for CSI prediction

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2417805	Discussion on CSI prediction testability and interoperability

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2418454	Discussion on testability and interoperability issues for  CSI prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2418524	Discussion on CSI Prediction: Testability and Interoperability issues

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2418750	Discussion on testability and interoperability issues for CSI prediction

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2418971	Discussion on testability and interoperability issues in AIML CSI prediction

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2418987	Discussion on CSI prediction

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the AI/ML-based CSI prediction.

Decision:		Noted.

R4-2419172	Discussion on testability and interoperability issues for CSI prediction

Type: other		For: Discussion
					Source: ZTE Corporation, Sanechips

Decision:		Noted.

R4-2419181	On AI/ML Based CSI Prediction

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2419750	CSI Prediction: Testability

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// RRM core requirements for Beam management

R4-2417613	Discussion on RRM core requirements for beam management

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2417619	RRM core requirements for beam management

Type: other		For: Discussion
					Source: OPPO

Decision:		Noted.

R4-2417703	Discussion on RRM core requirements for BM

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2417780	RRM core requirements for AI/ML Based Beam Management

Type: discussion		For: Discussion
					Source: Tejas Network Limited

Decision:		Noted.

R4-2417893	Discussion on core requirements of AI/ML BM use case

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision:		Noted.

R4-2417949	Discussion on core part requirement for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2418093	Further discussion on beam management core requirements

Type: other		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2418453	Discussion on RRM requirements for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2418475	Discussion on the RRM requirements of AI/ML Beam management

Type: other		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2418521	Discussion on Beam Management: RRM Core Requirements

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2418751	Discussion on RRM core requirements for beam management

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2418899	Discussion on RRM core requirements for beam management

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on RRM core requirements for beam management

Decision:		Noted.

R4-2418901	Simulation results of measurement error impact on beam prediction

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Simulation results for AIML air beam management

Decision:		Noted.

R4-2418972	Discussion on RRM core requirements in AIML Beam management

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2419115	On Base Station Tx Beamforming Codebook Design and the Prediction Impact

Type: discussion		For: Discussion
					Source: NTU

Decision:		Noted.

R4-2419184	RRM core requirements for AI/ML Based Beam Management

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2419753	RRM Core Requirements for AI/ML Beam Management

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// Testability and interoperability issues for Beam management

R4-2417614	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2417620	Testability and interoperability issues for beam management

Type: other		For: Discussion
					Source: OPPO

Decision:		Noted.

R4-2417673	FR2 test setup for AI/ML beam management evaluation

Type: other		For: Approval
					Source: Anritsu Corporation

Abstract:

In this contribution we share our proposals on a practical test setup for the AI/ML beam management evaluation.

Decision:		Noted.

R4-2417704	Discussion on testability and interoperability issues for BM

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2417781	Testability and Interoperability aspects for AI/ML Based Beam Management

Type: discussion		For: Discussion
					Source: Tejas Network Limited

Decision:		Noted.

R4-2417894	Discussion on testability issues of AI/ML BM use case

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision:		Noted.

R4-2417948	Discussion on testability and interoperability issues for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2418094	Further discussion on testability and interoperability issues for beam management

Type: other		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2418452	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2418474	Discussion on the Interoperability and testability aspects of AI/ML Beam management

Type: other		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2418522	Discussion on Beam Management: Testability and Interoperability Issues

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2418752	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2418848	On testability issues for AI/ML beam management

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2418900	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on testability and interoperability issues for beam management

Decision:		Noted.

R4-2418941	Discussion on test setup for AI/ML based beam management

Type: discussion		For: Discussion
					Source: Rohde & Schwarz

Decision:		Noted.

R4-2418973	Discussion on testability and interoperability issues in AIML Beam management

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2419185	On Testability and Interoperability aspects for AI/ML Based Beam Management

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2419391	On Test Setup Requirements FR2 AI/ML Beam Management

Type: other		For: Approval
					Source: Keysight Technologies UK Ltd

Decision:		Noted.

// RRM core requirements for Positioning accuracy enhancement

R4-2417705	Discussion on RRM core requirements for positioning

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2418459	Discussion on RRM requirements for positioning

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2418477	Discussion on the RRM requirements of AI/ML positioning

Type: other		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2418526	Discussion on RRM Core Requirements for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2418753	Discussion on RRM core requirements for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2418974	Discussion on RRM core requirements in AIML Positioning

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2419360	On requirements for AI/ML for positioning

Type: other		For: Discussion
					Source: Ericsson

Abstract:

This paper discusses feasibility of defining requirements for AI/ML positioning case 1.

Decision:		Noted.

R4-2419666	RRM core requirements for AI/ML positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2419752	RRM Core Requirements for Positioning Accuracy

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// Testability and interoperability issues for Positioning accuracy enhancement

R4-2417617	Discussion on testability and interoperability issues for positioning

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2417621	Testability and interoperability issues for positioning

Type: other		For: Discussion
					Source: OPPO

Decision:		Noted.

R4-2417706	Discussion on testability and interoperability issues for positioning

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2418458	Discussion on testability and interoperability issues for positioning

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2418476	Discussion on the Interoperability and testability aspects of AI/ML positioning

Type: other		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2418527	Discussion on Testability and interoperability issues for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2418754	Discussion on testability and interoperability issues for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2418975	Discussion on testability and interoperability issues in AIML Positioning

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2419325	Testability and interoperability issues for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2419361	On testability of AI/ML for positioning

Type: other		For: Discussion
					Source: Ericsson

Abstract:

This paper discusses testability of AI/ML based positioning use cases.

Decision:		Noted.

// Moderator summary and conclusions

R4-2418151	Topic summary for [113][129] NR_AIML_air_part1

Type: other		For: Information
					Source: Moderator (Qualcomm)

Abstract:

Topic summary for AI 7.17, 7.17.2, 7,17,3, 7.17.4

Decision:		Noted.

Newly allocated tdocs in the first round

R4-2420335	Ad hoc minutes on AI/ML air interface

Type: other		For: Approval
					Source: Qualcomm

Decision:		Noted.

R4-2420334	WF on AI/ML air interface part 1

Type: other		For: Approval
					Source: Qualcomm

Decision:		Revised to R4-2420513 (from R4-2420334).

R4-2420513	WF on AI/ML air interface part 1

Type: other		For: Approval
					Source: Qualcomm

Decision:		Approved.

R4-2420480	Updated simulation assumption for beam prediction

Type: other		For: Approval
					Source: vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

Decision:		Approved.

Minutes and agreements in the online session and ad hoc

Please refer to the hyperlink below for the detailed minutes.

https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_113/Inbox/Drafts/%5B113%5D%5B100%5D%20Main_Session/3.Wednesday/1.%5B129%5D_R4-2418151.docx

R4-2418152	Topic summary for [113][130] NR_AIML_air_part2

Type: other		For: Information
					Source: Moderator (Ericsson)

Abstract:

Topic summary for AI 7.17, 7.17.1, 7,17,5

Decision:		Noted.

Newly allocated tdocs in the first round

R4-2420336	WF on AI/ML air interface part 2

Type: other		For: Approval
					Source: Ericsson

Decision:		Approved.

Minutes and agreements in the online session and ad hoc

Please refer to the following hyperlink for the detailed minutes

https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_113/Inbox/Drafts/%5B113%5D%5B100%5D%20Main_Session/4.Thursday/A.%5B130%5D_R4-2418152%20Moderator%20summary%20for%20%5B113%5D%5B130%5D%20NR_AIML_air_part2.docx

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template