3GPP TSG RAN Meeting #106									RP-242569

Madrid, Spain, December 9-12, 2024

## Status Report to TSG

Agenda item:			9.3.4.3

| WI / SI Name | NR Radio Resource Management (RRM) Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_Ph5 |  |  |  |  |
| Unique ID | 1031079 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242380 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall:60% RAN4: 60% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Aijun Cao |
|  | Company | CATT |
|  | Email | ajcao@catt.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#113 meeting

The following tdocs have been approved/endorsed in RAN4#112bis meeting for core part:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2420082 | Ad-hoc minutes for NR_RRM_Ph5 WI | Apple |
| R4-2420093 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2420094 | WF on RRM requirements for NR_RRM_Ph5_Part2 | CATT |
| R4-2420132 | draftCR on Fast SCell activation for UE supporting Rel-18 eEMR | CATT, Apple, Nokia, Ericsson, Huawei, HiSilicon, Qualcomm, Samsung, ZTE Corporation, Sanechips, MediaTek, CMCC, China Telecom, LG Electronics, OPPO, vivo |

Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF (including activation condition and de-activation conditions):

Agreement:

- RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement.

Agreement:

- Not support UE mobility status based triggering condition in Rel-19.

Issue 1-1-3: searcher number assumption for RRM measurement with two panels activated on single carrier

Agreement:

Directly discuss the SSB processing time.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC

Agreement:

- For scenario 1/2/3/4

  - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement.

    - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms

    - where TSSB_processing = 2 ms

Agreement:

- For scenario 7

  - to consider an additional processing delay (2ms) after the last SMTC periodicity

Issue 1-1-5: FFS: L3 measurement delay reduction by optimizing Rx BSF with/without DRX

Agreement:

L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case.

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Agreement:

For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability:

- FFS: details of the capability

Issue 1-3: feature capability of L3 measurement delay reduction by optimizing Rx BSF

Agreement:

- Discuss this capability detail by end of the core part

Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Issue 2-1: Clarification on the bullets in WID for this CSSF optimization

Agreement:

•	RAN4 to follow the revised WID (WID RP-242380) in RANP#105 meeting, and no need to have further discussion on this issue.

Issue 2-4-1: Tentative solution 1: measure one serving CC per band

[Wayforwad]:

FFS the following solution:

  - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured,

    - Reuse the legacy principle for neighbour cell measurement

      - Measure PCC if PCC in band or

      - Measure PSCC if PSCC in band or

      - Measure SCC with neighbor cell MO or

      - UE implementation

    - FFS whether indication is per-UE or per-band

      - Other options are not precluded.

  - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

Issue 2-4-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

[Wayforward]:

FFS: whether solution 2 is needed if solution 1 is agreed in issue 2-4-1:

  - UE may change the searcher occupancy ratio of SCC measurement to speed up measurement.

    - FFS whether network indication is needed

      - Details is FFS

    - FFS on whether and how PCC and PSCC resource sharing is supported

Issue 2-4-3: Tentative solution 3: 3 searchers for CSSF

Agreement:

RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is:

    - FFS: FR1 scenario (CA and NR-DC) only.

    - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only.

    - FFS: All scenarios.

Issue 2-5: feature capability of L3 measurement delay reduction by optimizing CSSF for tentative solution 1 and 2

Agreement:

    - RAN4 to introduce new individual capability(s) for CSSF reduction in R19.

      - delay the capability detail discussion to the end of the core part.

Issue 2-7: Other WID scope discussion

Agreement:

-	after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable

Topic #3: Fast SCell activation for UE supporting Rel-18 EMR

Agreement: RAN4 work on this objective is closed.

Issue 1-1-1: Applicability of fast SCell activation delay requirements

Agreement:

- For R19 fast SCell activation for UE supporting R18 EMR, when neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured:

  - Fast SCell activation delay requirements (same with conclusion of the case where either measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured) apply under the following conditions:

    - For EMR carriers:

      - Consider the case when the T331 timer hasn’t expired at the moment of initiation of RRC state transition to Connected mode. i.e., measIdleDuration-r16 is configured, and T331 is running.

  - Otherwise, follow the legacy unknown SCell activation delay requirements.

Issue 1-1-2: Target scenarios of fast SCell activation

Agreement:

- Fast SCell activation requirements defined in R19 do NOT apply to the following scenarios:

  - SCell being contiguous to PCell in FR1 (i.e. intra-band contiguous CA)

  - SCell being in the same band as PCell in FR2 (i.e. intra-band CA)

Issue 1-2-1a: The fast SCell activation delay components for direct SCell activation at SCell addition

Agreement:

- For single direct SCell activation at SCell addition, for FR1

  - provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured:

    - use existing SSB based known SCell activation delay requirement with 2 SSB samples, i.e. Tactivation_time is TFirstSSB_MAX + Trs + 5ms.

- For single direct SCell activation at SCell addition, for FR2

  - provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured:

    - use existing SSB based known SCell activation delay requirement with 2 SSB samples.

      - delay components: 2 SSB samples * beam sweeping +1 SSB sample

      - Further discuss the detailed requirements when drafting the CR

Issue 1-2-1b: The fast SCell activation delay components for MAC CE based single SCell activation

Agreement:

- For MAC CE based SCell activation delay requirements with eEMR report, reuse the same conclusions as direct SCell activation at SCell addition.

Issue 1-2-1c: The time gap condition for MAC CE based single SCell activation

Agreement:

- For MAC CE based SCell activation delay requirements with eEMR report, reuse the same conclusions as direct SCell activation at SCell addition, i.e., [Y] equals to 5s for FR1, and 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 for FR2.

Issue 1-2-1d: The SSB detectability condition for both direct SCell activation and MAC CE based single SCell activation

Agreement:

- The earliest possible RRC Idle/Inactive measurement time instance is defined as below:

  - From the time UE receives the configuration of measurement in idle and inactive modes.

- Discuss the details when drafting the CR.

Agreement:

- Agree on the following as baseline for CR:

  - If the configured validity duration is only based on Rel-16 EMR, it refers to the time instance when the UE obtained the configuration parameter (i.e., measIdleDuration-r16).

  - If the configured validity duration includes Rel-18 eEMR, the configured, it refers to the time instance measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 before Msg1 transmission.

Issue 1-2-2: Consideration on measurement period condition of FR1 known SCell activation

Agreement:

- The issue can be covered by issue 1-2-1a, and no further agreements are needed.

Issue 1-2-3: Whether the indication to network is needed

Agreement:

- It is RAN4 common understanding that fast SCell activation requirements apply for any of reported cells, as long as side conditions (agreed in previous issues) are met. No need to introduce additional indication to the network for fast SCell activation.

Issue 1-2-4: UE capability

Agreement:

- Discuss the UE capability by the end of core part.

RAN4#112bis meeting

The following tdocs have been approved/endorsed in RAN4#112bis meeting for core part:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2416853 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2416872 | Ad-hoc minutes for NR_RRM_Ph5 | Apple |
| R4-2416854 | WF on RRM requirements for NR_RRM_Ph5_Part2 | CATT |

Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF:

Agreement in online session:

For scenario 1/2/3/4:

  - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions.

    - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions.

  - The detailed criteria/condition(s) is FFS.

  - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details.

For Scenario 5/7:

  - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch).

  - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

Issue 1-3: feature capability of L3 measurement delay reduction by optimizing Rx BSF

Agreement:

- RAN4 is to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19.

  - The detailed definition can be postponed to the end of the core part discussion.

Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF

Agreement in online session:

On top of the previous agreement on the baseline assumption of 2 searchers, for 3 searchers:

- Discuss the applicable scenario(s) and/or solution(s) based on 3 searchers. Decide whether or not to define the requirement later.

Topic #3: Fast SCell activation for UE supporting Rel-18 EMR

Issue 1-1-2: Scope of fast SCell activation for UE supporting Rel-18 eEMR

Agreement:

- On top of Single Direct SCell Activation at SCell addition, MAC CE based Single SCell Activation is also supported.

  - Note: MAC CE based PUCCH SCell activation is included.

Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting

Agreement:

- RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception.

  - For FR1,

    - [Y] equals to [5s] for direct SCell activation

    - FFS: Y for other SCell activation case.

  - For FR2,

    - Reuse the existing value of Y in direct SCell activation

    - FFS: Y for other SCell activation case.

Agreement:

- the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting).

  - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

  - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

Issue 1-2-2: Consideration on SINR condition during the whole procedure for known SCell activation

Agreement:

- Not to update the SINR condition (i.e., Ês/Iot≥ -2dB) for SCell activation delay requirements.

#### 2.4.2	Remaining Open issues

As captured in WF R4-2414026 and R4-2414041.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#113 meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2417722 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | CATT |
| R4-2417723 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | CATT |
| R4-2417724 | Discussion on Fast SCell activation for UE supporting Rel-18 EMR | CATT |
| R4-2417725 | CR on Fast SCell activation for UE supporting Rel-18 eEMR | CATT, Apple |
| R4-2417960 | Discussion on FR2-1 SSB based L3 measurement delay reduction by RX beam | Xiaomi |
| R4-2417961 | Discussion on FR2-1 SSB based L3 measurement delay reduction by CCSF optimization | Xiaomi |
| R4-2418070 | FR2-1 SSB based L3 measurement delay reduction with CSSF optimization | Nokia, Nokia Shanghai Bell |
| R4-2418103 | Discussion on RRM requirements for Fast SCell activation for UE supporting R18 EMR | LG Electronics Inc. |
| R4-2418251 | Fast SCell activation based on Rel-18 eEMR | Qualcomm Incorporated |
| R4-2418276 | Topic summary for [113][218] NR_RRM_Ph5_Part1 | Moderator (Apple) |
| R4-2418277 | Topic summary for [113][219] NR_RRM_Ph5_Part2 | Moderator (CATT) |
| R4-2418294 | Discussions on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor | NTT DOCOMO, INC. |
| R4-2418411 | Discussion on RRM requirements for FR2-1 L3 measurement delay reduction | LG Electronics Inc. |
| R4-2418449 | Discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor | CMCC |
| R4-2418450 | Discussion on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC | CMCC |
| R4-2418451 | Discussion on fast SCell activation for UE supporting Rel-18 EMR | CMCC |
| R4-2418466 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | China Telecom |
| R4-2418467 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | China Telecom |
| R4-2418468 | Discussion on Fast SCell activation for UE supporting Rel-18 EMR | China Telecom |
| R4-2418508 | Discussion on the fast SCell activation for UE supporting Rel-18 EMR | ZTE Corporation, Sanechips |
| R4-2418536 | On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Apple |
| R4-2418571 | On FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA DC | Apple |
| R4-2418584 | Discussion on fast SCell activation for UE supporting Rel-18 EMR | Apple |
| R4-2418627 | Discussion on delay reduction by optimizing Rx beam sweeping factors | Intel Corporation |
| R4-2418628 | draftCR on reduced BSF for handover and re-establishment | Intel Corporation |
| R4-2418651 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Huawei, HiSilicon |
| R4-2418652 | Discussion on fast SCell activation for UE supporting Rel-18 EMR | Huawei, HiSilicon |
| R4-2418677 | Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC | Huawei, HiSilicon |
| R4-2418757 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | vivo |
| R4-2418758 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | vivo |
| R4-2418768 | On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | OPPO |
| R4-2418769 | On FR2-1 L3 measurement delay reduction by optimizing CSSF | OPPO |
| R4-2418770 | On fast SCell activation with EMR | OPPO |
| R4-2418888 | On L3 beam sweeping factor reduction with multi-Rx | Nokia, Nokia Shanghai Bell |
| R4-2418906 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Ericsson |
| R4-2418907 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | Ericsson |
| R4-2419091 | Discussion on fast SCell activation for UE supporting Rel-18 EMR | vivo |
| R4-2419189 | Discussion on fast SCell activation for UE supporting eEMR | Samsung |
| R4-2419192 | Discussion on L3 measurement enhancement by optimizing Rx CSSF | Samsung |
| R4-2419193 | Discussion on L3 measurement enhancement by optimizing Rx BSF | Samsung |
| R4-2419194 | Draft CR on detected cell requirement during state transition and Idle& Inactive mode for UE supporting eEMR | Samsung,Nokia |
| R4-2419262 | Discussion on CSSF optimization of R19 RRM enhancements | ZTE Corporation, Sanechips |
| R4-2419263 | Discussion on Rx beam factor optimization of R19 RRM enhancements | ZTE Corporation, Sanechips |
| R4-2419445 | Discussion on fast scell activaiton for UE supporting Rel-18 EMR | Ericsson |
| R4-2419446 | Draft CR for fast direct Scell activation at Scell addition for UE supporting Rel-18 EMR | Ericsson |
| R4-2419667 | CR to 38.133 on known condition for fast SCell activation with EMR | Nokia |
| R4-2419730 | Discussion on Fast SCell activation with EMR | Nokia |
| R4-2419731 | LS to RAN2 on fast Scell activation with Rel18 eEMR | Nokia |
| R4-2419742 | Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor | MediaTek inc. |
| R4-2419743 | Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF | MediaTek inc. |
| R4-2419744 | Discussion on the RRM requirements for fast SCell activation for UE supporting Rel-18 EMR | MediaTek inc. |
| R4-2419747 | FR2-1 L3 measurement delay by optimizing RX beam sweeping factor | Qualcomm Incorporated |
| R4-2419748 | Enhancement on FR2 CSSF | Qualcomm Incorporated |
| R4-2420082 | Ad-hoc minutes for NR_RRM_Ph5 WI | Apple |
| R4-2420093 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2420094 | WF on RRM requirements for NR_RRM_Ph5_Part2 | CATT |
| R4-2420132 | draftCR on Fast SCell activation for UE supporting Rel-18 eEMR | CATT, Apple, Nokia, Ericsson, Huawei, HiSilicon, Qualcomm, Samsung, ZTE Corporation, Sanechips, MediaTek, CMCC, China Telecom, LG Electronics, OPPO, vivo |

RAN4 #112bis meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2415062 | Discussion on FR2-1 SSB based L3 measurement delay reduction by RX beam | Xiaomi |
| R4-2415063 | Discussion on FR2-1 SSB based L3 measurement delay reduction by CCSF optimization | Xiaomi |
| R4-2415150 | Discussion on fast SCell activation for UE supporting Rel-18 EMR | vivo |
| R4-2415179 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | CATT |
| R4-2415180 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | CATT |
| R4-2415181 | Discussion on Fast SCell activation for UE supporting Rel-18 EMR | CATT |
| R4-2415182 | draftCR on Fast SCell activation for UE supporting Rel-18 EMR | CATT, Apple |
| R4-2415196 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | vivo |
| R4-2415197 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | vivo |
| R4-2415244 | Discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor | CMCC |
| R4-2415245 | Discussion on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC | CMCC |
| R4-2415246 | Discussion on fast SCell activation for UE supporting Rel-18 EMR | CMCC |
| R4-2415346 | Fast SCell activation based on Rel-18 eEMR | Qualcomm Incorporated |
| R4-2415361 | Discussion on RRM requirements for Fast SCell activation for UE supporting R18 EMR | LG Electronics Inc. |
| R4-2415487 | Discussion on fast SCell activation for UE supporting Rel-18 EMR | Apple |
| R4-2415513 | On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Apple |
| R4-2415545 | On FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA DC | Apple |
| R4-2415574 | FR2-1 SSB based L3 measurement delay reduction with CSSF optimization | Nokia, Nokia Shanghai Bell |
| R4-2415575 | Discussion on fast SCell activation for UE supporting eEMR | Samsung |
| R4-2415578 | Discussion on L3 measurement enhancement by optimizing Rx BSF | Samsung |
| R4-2415656 | Topic summary for [112bis][209] NR_RRM_Ph5_Part1 | Moderator (Apple) |
| R4-2415657 | Topic summary for [112bis][210] NR_RRM_Ph5_Part2 | Moderator (CATT) |
| R4-2415696 | Discussion on CSSF optimization of R19 RRM enhancements | ZTE Corporation, Sanechips |
| R4-2415697 | Discussion on Rx beam factor optimization of R19 RRM enhancements | ZTE Corporation, Sanechips |
| R4-2415732 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | China Telecom |
| R4-2415733 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | China Telecom |
| R4-2415734 | Discussion on Fast SCell activation for UE supporting Rel-18 EMR | China Telecom |
| R4-2415834 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Huawei, HiSilicon |
| R4-2415835 | Discussion on Fast SCell activation for UE supporting Rel-18 EMR | Huawei, HiSilicon |
| R4-2415838 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | Huawei, HiSilicon |
| R4-2415879 | Discussion on RRM requirements for FR2-1 L3 measurement delay reduction | LG Electronics Inc. |
| R4-2415887 | FR2-1 L3 measurement delay by optimizing RX beam sweeping factor | Qualcomm Incorporated |
| R4-2415888 | Enhancement on FR2 CSSF | Qualcomm Incorporated |
| R4-2415929 | Discussions on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor | NTT DOCOMO, INC. |
| R4-2416017 | On RRM Ph5 L3 BSF reduction | Nokia |
| R4-2416078 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Ericsson |
| R4-2416079 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | Ericsson |
| R4-2416087 | Discussion on delay reduction by optimizing Rx beam sweeping factors | Intel Corporation |
| R4-2416088 | Discussion on delay reduction by optimizing CSSF outside gaps | Intel Corporation |
| R4-2416117 | Discussion on L3 measurement delay by optimizing CSSF | Samsung |
| R4-2416118 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | OPPO |
| R4-2416119 | Discussion on FR2-1 L3 measurement delay reduction by optimizing CSSF | OPPO |
| R4-2416120 | On fast SCell activation with EMR | OPPO |
| R4-2416168 | Disucssion on fast Scell activation for UE supporting Rel-18 EMR | Ericsson |
| R4-2416289 | Discussion on Fast SCell activation for UE supporting Rel-18 EMR | Nokia |
| R4-2416387 | Discussion on the fast SCell activation for UE supporting Rel-18 EMR | ZTE Corporation, Sanechips |
| R4-2416511 | Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor | MediaTek inc. |
| R4-2416512 | Discussion on the RRM requirements for fast SCell activation for UE supporting Rel-18 EMR | MediaTek inc. |
| R4-2416853 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2416872 | Ad-hoc minutes for NR_RRM_Ph5 | Apple |

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template