3GPP TSG RAN Meeting #106	RP-242570

Madrid, Spain, December 9-12, 2024

Agenda item:	9.2.8

Source:	Apple

Title:	Motivation on scope change of the SI fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

RAN4 has been discussing this SI for a couple of meetings. In this contribution, we share our views why a scope change is proposed.

# 2	Discussion

As to the number of fragments and whether the fragments are contained the 100MHz range, there are three different cases. This is shown in Fig. 1. In Case 1, two fragments are fully contained in a 100MHz range, which is the scope of the SI; in Case 2, two fragments are not fully contained in a 100MHz range; in Case 3, while the first two fragments, i.e., Fragments 1 and 2, are fully contained in a 100MHz range, there are additional fragments, say Fragment 3, which falls outside of the 100MHz range.

Case 1

Case 2

Case 3

Figure 1. Different cases in terms of number of fragments and whether the fragments are contained in 100MHz range

In terms of implementation, one RF chain including ADC can handle more than 100MHz. On the other hand, usually BB can handle up to 100MHz per CC, since 100MHz is the max. CBW in FR1. More CCs can always be processed separately. In Case 2, the bandwidth of Fragment 1 or Fragment 2 or both have to be reduced to be able to fit into 100MHz if we follow the scope of the SI.

On the other hand, if we only focus on Case 1 and leave Cases 2 and 3 out in the study, we may run the risk that the final requirements (to be specified in a follow-on WI of this SI) may be different from those for Cases 2 and 3. This is undesirable from RF design standpoint since the RF design usually considers the entire band, not part of the band.

In RAN4 meeting#112bis, the following agreement was made [2], recognizing the scope change needs to be discussed and approved in RAN:

#### Sub-topic 1-2: Study scope clarification Agreement: - Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz as illustrated below: - Any update of scope needs be decided by RAN

Therefore, we propose to update the SI scope to include both Case 2 and Case 3. Meanwhile, to better organize the RAN4 discussion, it is OK to prioritize Case 1.

Proposal 1: It is proposed to update the SI scope to include both Case 2 and Case 3. To better organize the RAN4 discussion, it is OK to prioritize Case 1.

The revised SID is provided [2].

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: It is proposed to update the SI scope to include both Case 2 and Case 3. To better organize the RAN4 discussion, it is OK to prioritize Case 1.

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

R4-2417201, "WF on fragmented DL carriers study," MediaTek

RP-242571, "Revised SID: Study on NR FR1 DL fragmented carriers," Apple