3GPP TSG RAN Meeting #106							RP-242574

Madrid, Spain, December 9-12, 2024

Source:	Qualcomm Incorporated

Title:	Views on Ambient IoT WI in Rel-20

Agenda Item:	15.2

Document for: 	Discussion and Decision

# Background

In Rel-19, study on solutions for ambient IoT has been conducted, and the following has been captured in the TR38.769 as a conclusion of the study [1]:

RAN1 recommends that in a normative phase, the design should start from the descriptions in this TR. (1) The following techniques are recommended in the first normative phase: - PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R - Modulations of OOK-1 and OOK-4 in R2D - Multiplexing in time domain for R2D - Convolutional FEC in D2R and no FEC in R2D - R2D and D2R signal(s) (2) At least the following aspects may be considered for inclusion or non-inclusion in a given first or subsequent Release: - Device 1 and/or Device 2a (with or without large frequency shift) and/or Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture) - D1T1 and/or D2T2 - Scenario A1 and/or A2 and/or B and/or C - Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R - Multiple access by TDMA and/or FDMA and/or CDMA for D2R; FDMA for device 2b (IF-ED/ZIF receiver) in R2D - OOK and/or BPSK and/or MSK modulation for D2R - Carrier-wave waveform 1 and/or 2, and transmission case(s) - Device (un)availability direction 1 and/or 2 - Proximity determination solution 1 and/or 2

In this contribution, we provide our views on Ambient IoT work item for Rel-20.

# Work item scope for Rel-20 A-IoT

Due to the tight timeline, the Rel-19 A-IoT work item, if approved, would need to focus on specifying only essential components for enabling A-IoT. In contrast, Rel-20 can include aspects identified as beneficial or attractive during the Rel-19 study, to develop a new 3GPP-standardized A-IoT ecosystem. Here are our views on the scope for Rel-20 A-IoT work.

## 2.1	Use-cases

In Release 19, the traffic types under consideration included device-oriented by device-terminated trigger (DO-DTT) and device-terminated (DT), with a focus on indoor inventory and indoor command use cases. These areas have been studied and should also be supported in the Rel-20 work item. In addition, the device-originated autonomous (DO-A) traffic can also be considered in the Rel-20 work item.

## 2.2	Topologies

In Rel-19, both D1T1 (deployment scenario 1 with connectivity topology 1) and D2T2 (deployment scenario 2 with connectivity topology 2) were studied. D2T2 presents a feasible and useful deployment/topology for indoor A-IoT scenarios. For instance, a customer can use a handheld terminal or smartphone to scan and inventory A-IoT devices in environments such as smart homes, factories, and warehouses. Intermediate UE as a reader for D2T2 is also useful for a certain outdoor deployment where A-IoT devices are distributed outdoors and a handheld/smartphone reader comes to scan/read the devices nearby. In this scenario, the devices can still rely on RF energy harvesting and can use RF envelope detector receiver, since the distance between the reader and the devices is similar to indoor.

D2T2 requires specifying the Uu interface/signaling and relevant procedures for intermediate UE. However, these could be transparent to A-IoT devices.

## 2.3	Device type(s)

In Rel-19, three device types have been studied and documented in the TR38.769. Given the limited time left in Rel-19 timeline, even if the Rel-19 work item is approved, it may not be possible to specify all device types. For Rel-20, we recommend supporting device type 2b, if not covered in Rel-19, with advanced features to achieve larger coverage, improved energy efficiency, better spectral efficiency, etc.

## 2.4	Essential features for Rel-20

Based on the RAN1 conclusion captured in TR38.769, we share our views on the essential features to support device 2b in D1T1 and D2T2.

- The following techniques, if defined in the first normative phase, can be reused for Rel-20 work item

- PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R

- Modulations of OOK-1 and OOK-4 in R2D

- Line code(s) for R2D

- Multiplexing in time domain for R2D

- Convolutional FEC in D2R

- R2D and D2R signal(s)

- At least the following aspects should be considered for inclusion in Rel-20 work item:

- Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture)

  - Synchronization of device 2b clock/LO via R2D transmission from a reader

- D1T1 and D2T2

  - Scenario C

- FEC/repetition with scrambling in R2D and D2R

- D2R waveform/modulation/coding and spectrum shaping

  - No line code

  - OOK and/or BPSK and/or MSK

  - Double sideband and/or Single sideband

- Multiple access by TDMA and/or FDMA and/or CDMA for D2R; potential FDMA for device 2b (IF-ED/ZIF receiver) in R2D

- Solution(s) for Device (un)availability due to energy harvesting direction 2

  - Support at least a solution controlling device states (ON, OFF, SLEEP) (e.g., duty-cycle monitoring, any necessary periodic signaling, etc.)

- Proximity determination solution 2

  - D2R power measurement at reader side

RAN2 objectives

- Specify the A-IoT radio resource management to support intermediate UE as Reader

  - Uu signaling for A-IoT resource request and resource configuration

- Specify the signaling and procedure of UE Reader performing A-IoT operation in all RRC states

- Specify the signaling and procedure of UE Reader performing A-IoT operation upon temporary out of connection

- Study and if found feasible, specify the UE Reader performing A-IoT operation in out of coverage scenario.

RAN3 objectives

- Specify RAN architecture and protocols to support A-IoT operations with intermediate UE as Reader using user-plane transport between the UE Reader and the AIoTF (AIoT Network Function)

- Reader-CN signaling to support Inventory-only, Command-only, Inventory and Command

- Xn signaling to support UE Reader mobility

- Reader-CN signaling to support other A-IoT use cases (e.g., sensors, tracking, outdoor use cases)

- Finer granular techniques for locating the A-IoT device if needed.

RAN4 objectives

- Specify a frequency band and the system parameters for Ambient IoT system

  - Example band(s) is needed

- Specify RF requirements for Ambient IoT devices, reader (gNB and intermediate UE) and external CW node

  - RX and TX requirements

    - Conduct the coexistence analysis based on simulation assumptions and methodology in TR 38.769 (e.g., ACLR, ACS for readers and unwanted emission for devices)

    - Study and specify the guard-band between NR and Ambient IoT system if necessary

  - The feasibility of CW interference cancelation and its impact on the performance and energy harvesting shall be taken into account.

- Study and define the test methodology for devices RF requirements verification

- List new specification documents if so decided

NOTE: Coordination with other RAN WGs and SA WGs on the above objectives as needed.

# Timeline

Supporting device 2b may need substantial enhancements compared to the Rel-19 A-IoT (if specified). It is reasonable to allocate the entire Rel-20 timeline (i.e., 18 months) for the Rel-20 A-IoT work item.

# Conclusion

In this contribution, we provided our views on Ambient IoT work item for Rel-20. The proposal is summarized as follows.

Proposal:

- RAN to approve a Rel-20 WI for Ambient IoT

  - Support device 2b

  - Support both D1T1 and D2T2 with scenario C

    - Specify necessary Uu signalling/procedure for intermediate UE as a reader

  - Support multiplexing schemes to improve spectrum efficiency in R2D and D2R

  - Support Solution(s) for device (un)availability due to energy harvesting Direction 2

  - Specify RAN architecture and protocols to support A-IoT operations with intermediate UE as Reader using user-plane transport between the UE Reader and the A-IoTF

  - Support UE Reader performing A-IoT operation in all RRC states, temporarily out-of-connection and in out-of-coverage

  - Support Proximity determination solution 2

# Reference

- 3GPP TR38.769