3GPP TSG RAN Meeting #106							RP-242577

Madrid, Spain, December 9-12, 2024

Source:	Qualcomm Incorporated

Title:	Views on the check point for Rel-19 Ambient IoT SI

Agenda Item:	9.2.2

Document for: 	Discussion and Decision

# Background

In Rel-19, study on solutions for ambient IoT has been conducted, and the following has been captured in the TR38.769 as a conclusion of the study [1]:

RAN1 recommends that in a normative phase, the design should start from the descriptions in this TR. (1) The following techniques are recommended in the first normative phase: - PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R - Modulations of OOK-1 and OOK-4 in R2D - Multiplexing in time domain for R2D - Convolutional FEC in D2R and no FEC in R2D - R2D and D2R signal(s) (2) At least the following aspects may be considered for inclusion or non-inclusion in a given first or subsequent Release: - Device 1 and/or Device 2a (with or without large frequency shift) and/or Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture) - D1T1 and/or D2T2 - Scenario A1 and/or A2 and/or B and/or C - Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R - Multiple access by TDMA and/or FDMA and/or CDMA for D2R; FDMA for device 2b (IF-ED/ZIF receiver) in R2D - OOK and/or BPSK and/or MSK modulation for D2R - Carrier-wave waveform 1 and/or 2, and transmission case(s) - Device (un)availability direction 1 and/or 2 - Proximity determination solution 1 and/or 2

In this contribution, we provide our views on potential Ambient IoT work item for Rel-19.

# Work item scope for Rel-19 A-IoT

Due to the tight timeline, the Rel-19 A-IoT work item, if approved, would need to focus on specifying only essential/urgent components for enabling A-IoT. In Release 19, indoor inventory and indoor command use cases should remain as focus use cases with the support of device-originated by device-terminated trigger (DO-DTT) and device-terminated (DT) traffic.

In Rel-19, both D1T1 (deployment scenario 1 with connectivity topology 1) and D2T2 (deployment scenario 2 with connectivity topology 2) were studied. D1T1 and D2T2 are both feasible and useful deployments/topologies for indoor A-IoT scenarios. However, RAN needs to consider the required time for the potential specification impact and decide which topology to be supported in Rel-19, if any.

In Rel-19, three device types have been studied and documented in the TR38.769. Given the limited time left in Rel19 timeline, even if the Rel-19 work item is approved, it may be possible to specify all device types.

# Essential features for Rel-19

Based on the RAN1 conclusion captured in TR38.769, we share our views on the essential features enabling Rel-19 A-IoT.

RAN1 Objectives:

- The following techniques in (1) of TR38.769 conclusion can be the baseline in the first normative phase

- PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R

- Modulations of OOK-1 and OOK-4 in R2D

- Multiplexing in time domain for R2D

- Convolutional FEC in D2R and no FEC in R2D

- R2D and D2R signal(s)

- For the aspects in (2) of TR38.769 conclusion, we suggest at least following for Rel-19:

- Manchester coding for R2D

- A simple CP handling that does not increase device complexity for R2D OOK-1/4

- Either no line code or Manchester coding option 1 for D2R

- Device clock sync/calibration targeting SFO <= 10^4 ppm enabling multiple access for D2R

- Nested polynomials for D2R FEC and R2D/D2R CRC

RAN2 Objectives:

- The following aspects should be specified in the normative phase

  - A-IoT paging including procedure and message content

  - A-IoT random access including contention-based and contention-free solution(s)

  - A-IoT data transmission procedure

  - A-IoT AS protocol stack and interaction with AIoT upper layers

    - A-IoT MAC functionalities such as segmentation, Energy Status report, indication of remaining D2R message size.

RAN3 Objectives:

- Specify RAN architecture and protocols to support A-IoT

- Specify signaling to support Inventory-only, Command-only, Inventory and Command

- Specify signaling to support reader awareness at A-IoTF (A-IoT Network Function)

- Specify signaling to support A-IoT device context management (if needed)

- Specify signaling to locate the A-IoT device at reader granularity

RAN4 objectives:

- Specify a frequency band and the system parameters for Ambient IoT system

  - Example band(s) is needed

- Specify RF requirements for Ambient IoT devices, reader and external CW node

  - RX and TX requirements

    - Conduct the coexistence analysis based on simulation assumptions and methodology in TR 38.769 (e.g., ACLR, ACS for readers and unwanted emission for devices)

    - Study and specify the guard-band between NR and Ambient IoT system if necessary

  - The feasibility of CW interference cancelation and its impact on the performance and energy harvesting shall be taken into account.

- Study and define the test methodology for devices RF requirements verification (assuming OTA is the baseline test methodology for devices)

- List new specification documents if so decided

NOTE: Coordination with other RAN WGs and SA WGs on the above objectives as needed.

# Timeline

Rel-19 A-IoT (if converted to WI) recommended to be completed within the Rel-19 timeframe without any extension.

# Conclusion

In this contribution, we provided our views on Ambient IoT work scope and timeline during remaining Rel-19 time. The proposal is summarized as follows.

Proposal:

- RAN to decide whether to have a Rel-19 WI for Ambient IoT. If RAN decides to go forward, it should have a reasonably narrow scope to enable completion within the Rel-19 timeframe.

# Reference

- 3GPP TR38.769