3GPP TSG RAN meeting #106                                                                                  	RP-242590

Madrid, Spain, December 9-12, 2024

Agenda Item:	10.4.5

Source:	Thales

Title:	Proposed way forward on the introduction of IoT-NTN TDD mode

Document for:	Discussion and decision

# Introduction

- A new work item on introduction of IoT-NTN TDD mode was approved in RAN plenary #105. The detailed scope of this WI can be found in RP-242415 [1]. According this WID [1], there is a checkpoint at RAN #106 as follows:

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. The study and work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band. This work item includes the following objectives: - Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4] - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

In this contribution, we propose a way forward on the introduction of IoT-NTN TDD mode.

# Discussion

The study of the new NB-IoT NTN TDD mode was carried out in RAN1/RAN4.  Based on the study outcome, it was determined that using at least N=9 is feasible based on:

- Downlink Synchronization: Evaluations at RAN1#119 concluded that this configuration meets the necessary link budget requirements for downlink synchronization channel/signals (NPSS/NSSS/NPBCH). Three specific cases confirmed feasible include:

  - Case 1: N=9, D=8

  - Case 2: N=9, D=20

  - Case 3: N=9, D=30

- Coexistence with Legacy TDD Frame Structure: The proposed configuration also adheres to the design constraints pertaining to the existing TDD frame structure within the 1616-1626.5 MHz MSS band. Specifically, RAN1 concludes that D=8 and U=8 configurations are feasible with N=9.

Overall, based on RAN1/RAN4 study, the feasibility of the configuration of N=9, D=8 and U=8 ensures that the proposed system design can operate in a compatible way with existing legacy systems and meet synchronization requirements.

However, according to the WID [1] the value of N and the configuration of the periodic pattern are fixed per band. In our view fixing the periodic pattern parameters per band is an unnecessary constraint that introduces less flexibility in system design and might not suit all future deployment scenarios and all orbits. We present in the following the rationale why the periodic pattern parameters should not be fixed per band and thereby a revision of the WID is needed.

The TDD-DL-UL-Pattern can be defined with the following parameters:

- DL-UL-TransmissionPeriodicity (referred to as N in the WID) which indicates the periodicity of the DL-UL pattern. It can be provided in number of radio frames. At least N equal to 9 Radio frames should be supported.

- nrOfDownlinkSubframes (D) corresponding to the number of consecutive/ contiguous DL subframes at the beginning of each DL-UL pattern. To suit the existing system deployed at targeted band the minimum of D should be equal to 8.

- nrOfUplinkSubframes (U) which provides the number of consecutive/ contiguous UL subframes. To suit the existing system deployed at targeted band the minimum of U should be equal to 8. And these consecutive/ contiguous UL subframes should not be necessary placed at the end of the TDD pattern window.

- DownlinkToUplinkGuardPeriod which indicates the Downlink to uplink Guard Period for TDD operation. This parameter in provided in number of subframes.

These parameters are illustrated in Figure 1.

Figure 1 Definition of the periodic TDD-DL-UL-Pattern

When defining the DownlinkToUplinkGuardPeriod in Non-Terrestrial networks, as outlined in clause 7.3.6 of [2, 38.811] this guard time directly depends on the propagation delay between UE and Base Station. Thereby, it should commensurate at least the round trip delay (RTD). The RTD could take different values depending on satellite orbit (LEO600km, LEO1200km or GEO) and the position of the reference point for uplink synchronization. For example:

- Minimum Downlink to Uplink Guard time for TDD operation for LEO at 600 km would range from 12.88 ms to 25.77 ms

- Minimum Downlink to Uplink Guard time for TDD operation for LEO at 1200 km would range from 20.88 ms to 41.77 ms.

Observation 1

Downlink to uplink Guard Period for TDD operation should commensurate at least the round trip delay (RTD). Thereby, to suit different deployments/orbits it should not be fixed per band.

Further, in our view, for the UE to perform cell search procedure and initial cell selection, the UE does not need to be aware of all the TDD pattern configuration parameters listed above. To acquire time and frequency synchronization within a cell and detects the Cell ID of that cell, it is enough for the UE to be aware of the DL-UL-TransmissionPeriodicity (e.g. value of N equal to 9) that can be linked to the band. Moreover, the UE may assume a default value of D that can be used for cell search procedure e.g. to determine the number of NPSS per each TDD pattern if more than one NPSS is to be considered for a successful NPSS detection.

We propose to set the default value of D that is assumed by the UE during initial access to a standardized value that is provided per band. A value of D equal to 8 could be adopted as baseline.

After NB SIB acquisition, the UE may be configured with different values of DownlinkToUplinkGuardPeriod, D and U. As outlined above, the value of DownlinkToUplinkGuardPeriod could be determined based on the orbit with a maximum value of 64 subframes (signaled in 6 bits). Regarding the values to be considered for D and U we think that it should be possible to limit the number of subframes transmitted per each DL and UL transmission opportunity to no longer than 8 subframes. This is beneficial to comply with the design constraints of constellation deployed in target band being considered in the WID. These design constraints are captured in RAN1#119 agreement as follows:

Agreement Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

It is worth noting also, by considering the existing system constraints, the value of  DownlinkToUplinkGuardPeriod is different for each DL/UL pair. (i.e. the DL/UL gaps for DL1/UL1, DL2/UL2, DL3/UL3, DL4/UL4 are all different).

For NB-IoT multi carrier mode operation, Anchor carrier and Non-Anchor carrier may have different TDD pattern.

Observation 2

The DL slot/UL slot pairs (e.g. DL1/UL1 and DL4/UL4) in existing system have different transmission gap and thereby different Downlink to uplink Guard Period should be supported.

Based on the above discussion, we have the following proposals:

Proposal 1

In NB-IoT NTN TDD mode, for cell search procedure a UE may assume a predefined value for DL-UL-TransmissionPeriodicity (N) and nrOfDownlinkSubframes (D).

Proposal 2

The value of DL-UL-TransmissionPeriodicity used for NB-IoT NTN TDD operation may be fixed per band.

Proposal 3

The value of DL-UL-TransmissionPeriodicity is equal to 9 radio frames for the 1616-1626.5 MHz MSS allocated band.

Proposal 4

For NB-IoT NTN TDD mode of operation, the TDD pattern configuration parameters may be provided in SIB for Anchor carrier and may be configured by RRC signaling for Non Anchor carriers.

Proposal 5:

Revised the WID RP-242415 are follow:

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N is fixed per band and the configuration of the other periodic pattern parameters (D, U and Downlink to uplink Guard Period) are fixed per band may be provided in SIB for Anchor carrier and may be configured by RRC signaling for Non Anchor carriers.

- For the 1616-1626.5 MHz MSS allocated band, N=9 and minimum of D=8 and U=8 to be supported.

# Conclusion

In this contribution. we made the following observations and proposals:

Observation 1

Downlink to uplink Guard Period for TDD operation should commensurate at least the round trip delay (RTD). Thereby, to suit different deployments/orbits it should not be fixed per band.

Observation 2

The DL slot/UL slot pairs (e.g. DL1/UL1 and DL4/UL4) in existing system have different transmission gap and thereby different Downlink to uplink Guard Period should be supported.

Proposal 1

In NB-IoT NTN TDD mode, for cell search procedure a UE may assume a predefined value for DL-UL-TransmissionPeriodicity (N) and nrOfDownlinkSubframes (D).

Proposal 2

The value of DL-UL-TransmissionPeriodicity used for NB-IoT NTN TDD operation may be fixed per band.

Proposal 3

The value of DL-UL-TransmissionPeriodicity is equal to 9 radio frames for the 1616-1626.5 MHz MSS allocated band.

Proposal 4

For NB-IoT NTN TDD mode of operation, the TDD pattern configuration parameters are provided in SIB for Anchor carrier and configured by RRC signaling for Non Anchor carriers.

Proposal 5:

Revised the WID RP-242415 are follow:

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N is fixed per band and the configuration of the other periodic pattern configuration parameters (D, U and Downlink to uplink Guard Period) are fixed per band are provided in SIB for Anchor carrier and configured by RRC signaling for Non Anchor carriers.

- For the 1616-1626.5 MHz MSS allocated band, N=9 and minimum of D=8 and U=8 to be supported.

# References

- RP-242415	 New WID on introduction of IoT-NTN TDD mode, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024

- TR 38.811 Study on New Radio (NR) to support non-terrestrial networks (Release 15)