3GPP TSG-RAN#106 Meeting			RP-242614

Madrid, Spain, December 9th – 12th, 2024

Agenda Item:	9.3.2.4

Source:	IIT Kharagpur

Title:	RRC Connection Establishment for Multihop-Parallel Relay

Document for:	Discussion

# Introduction

The present contribution defines the methodology to establish RRC connection in Multi-hop parallel relay configuration to facilitate vehicle-to-everything (V2X) services network as specified in 3GPP TS 23.287 [3] for:

a) V2X communication among the UEs over the PC5 interface; and

b) V2X communication between the UE and the gNB over the Uu interface.

The advent of Fifth Generation New Radio, abbreviated as 5G-NR, marks a significant stride in cellular technology. With a vision to interconnect all entities, be it machines, devices, or gadgets, 5G introduces a novel core and access network. Its overarching objective is to furnish a broader user base with accelerated multi-Gbps peak data rates, minimal latency, heightened reliability, expansive network capacity, and a uniform user experience. This advancement not only enhances performance and efficiency but also fosters the integration of diverse industries, epitomizing the ethos of ubiquitous connectivity.

Nevertheless, the potential for user equipment (UE) outage persists, often attributed to signal fading or being beyond the coverage area of a gNB. To counteract such instances and uphold seamless connectivity, relays emerge as indispensable components. Presently, the 5G-NR standard exclusively addresses single-hop or multi-hop relays, without incorporating multi-hop parallel relay configurations.

# Discussion

Key Proposal:

i.  The multihop-parallel relay network may be established between a remote UE and a destination UE or a gNB. The relays will work in decode forward fashion. The proposal is to establish a radio resource control (RRC) connection between a UE and a gNB/UE using a multihop-parallel relay topology.

ii.	We propose multiple parameters like signal to noise ratio (SNR) and reference signal received power (RSRP) based relay selection/connection protocol at the RRC level using PC-5 and Uu links. At every hop, a RRC level procedure is proposed to connect multiple relay devices simultaneously.

Multihop Parallel Relay Topology:  A schematic diagram of the proposed scheme is depicted in Fig. 1 and Fig. 2. In Fig 1 a remote UE, 101 is connected to the gNB, 110, through L hops. The connection between Lth hop and gNB is through Uu link, 109. 101 is connected to 1st hop, 103, through PC-5 link, 102. A hop, 103 may contain many relay devices like 104. 1st hop, 103 is connected to 2nd hop, 106 through PC-5 links 105. Similarly, Lth hop, 108 also receives PC-5 links, 107 from previous hop.

In Fig 2 a multihop-parallel network is suggested between a remote UE, 101 and a destination UE, 202. The destination UE, 202 is connected to Lth hop, 108 through PC-5 links, 201.

Fig 1: Schematic diagram for multihop-parallel relay between a remote UE and gNB

Fig 2: Schematic diagram for multihop-parallel relay between a remote UE and a destination UE

Brief Description of Work flow:  A remote UE may find a gNB or the destination UE through L hops (typically L ≥ 1). At lth hop, we propose to limit the connection to maximum ND number of relay devices. The ith device at the lth hop may be connected to the multiple devices at (l+1)th hop through different radio links. Similarly, multiple devices of (l-1)th hop may be connected to the ith device at lth hop through separate radio links. At every layer, it checks whether the destination UE or gNB has been reached or not. If reached, the network terminates there. If a remote UE is in outage due to fading or outside of the coverage area of a gNB, it looks for possible candidate relay devices. The remote UE discovers possible relay devices in the vicinity. The discovered relay devices are ranked according to the signal to noise ratio (SNR) and reference signal received power (RSRP). At the first hop (l =1), the remote UE selects a maximum of ND devices according to the descending order and establishes PC-5 connections with them. A relay chain ID has also been provided to all selected relay devices. The relay devices at the lth hop look for destination UE or gNB; if found, the network terminates there. Otherwise, the relay devices at lth hop look for the next set of relay candidates. Each relay device at lth hop connects with at least one relay device at the (l+1)th hop if available. There will be a check for relay devices' ID at the (l+1)th hop. If all the devices are unique, proceed to the next hop. However, if devices are not unique, discover leftover relay candidates at (l+1)th hop and connect them according to the descending order of SNR and RSRP. Repeat this process until ND unique relay devices are available at (l+1)th hop. The relay network selection procedure is continued till l=L hop. If the destination UE or gNB is not found till then, the network terminates.

A detailed diagram of the proposed scheme is depicted in Fig. 3 and Fig. 4. In Fig2 a remote UE, 101 is connected to the gNB, 110, through L hops. The connection between L th hop UEs such as 319, 320, and 321and gNB, 110 is through Uu link, 322, 323, and 324. 101 is connected to 1 st hop, UEs such as 305, 306, and 307 through PC-5 links, 302, 303, and 304. A relay device R 11 of 1 st hop, is connected to a relay device R 21 through PC-5 link 309. Similarly, at L th hop, relay devices 319, 320, 1nd 321 also receive PC-5 links, through 316, 317, and 318 from previous hop.

In Fig 4 a multihop-parallel network is suggested between a remote UE, 101 and a destination UE, 202. The destination UE, 202 is connected to L th hop relay devices 319, 320, and 321 through PC-5 links 401, 402, and 403.

Fig 3: Detailed diagram for establishing a relay network between a remote UE and a gNB

Fig 4: Detailed diagram for establishing a relay network between a remote UE and a destination UE

Fig. 5 (5A, 5B, 5C, 5D) shows the steps for establishing a multihop-parallel relay network between a remote UE and gNB. The network consists of a maximum of L hops; for each hop, there may be a maximum of ND user equipment (UE) devices. The details of the steps are as follows:

Fig 5A: Steps for multihop-parallel relay network establishment between a remote UE and gNB-part A

Fig 5B: Steps for multihop-parallel relay network establishment between a remote UE and gNB-part B

Fig 5C: Steps for multihop-parallel relay network establishment between a remote UE and gNB-part C

Fig 5D: Steps for multihop-parallel relay network establishment between a remote UE and gNB-part D

Fig. 6 (6A, 6B, 6C, 6D) shows the steps for establishing a multihop-parallel relay network between a remote UE and a destination UE. The network consists of a maximum of L hops; for each hop, there may be a maximum of ND user equipment (UE) devices. The details of the steps are as follows:

Fig 6A: Steps for multihop-parallel relay network establishment between a remote UE and a destination UE- part A

Fig 6B: Steps for multihop-parallel relay network establishment between a remote UE and a destination UE- part B

Fig 6C: Steps for multihop-parallel relay network establishment between a remote U and a destination UE- part C

Fig 6D: Steps for multihop-parallel relay network establishment between a remote UE and a destination UE- part D

Fig 7: Proposed relay ranking algorithm

A relay ranking algorithm has also been suggested in Fig. 7. At step 1, a remote or relay UE discovers possible relay devices. The discovered relay devices are counted in step 2. The device counter is set to one at step 3. Whether the device count is less than the number of discovered devices is checked in step 4. If the device count is less than the total discovered devices, the RSRP of the particular device is better than the minimum specified level and is checked at step 6. If RSRP is not greater than the minimum specified value, the device is not a relay candidate, and this decision is taken at step 7 and goes to step 12 through 8. If the RSRP of the device is greater than the minimum specified value, the SNR of the device is checked at step 9. If SNR is less than the specified value, go to step 12 through steps 7 and 8. If SNR exceeds the specified value, the device is a potential candidate for relay device at step 10. At step 11, the device is marked. The device counter is incremented by one at step 12. At step 13, the procedure is returned to step 4. If step 4 is not successful, go to step 14 through step 905. At step 14, the marked devices are arranged in descending order.

# Results

Fig 8: SER vs transmit power of UE-s, P (dBm) Plot

Fig. 8 shows the relationship between SER of the parallel-multihop relay combination system and transmit power of UEs, i.e. P. It can be observed that if P is increased the average SER performance of the overall system improves due to improvement of received SNR at each UE. Moreover, it can be found that if the number of hops, i.e. L is increased the SER performance also improves. This occurs due to the fact that if L is increased, the distances between intermediate hops reduces, therefore the pathloss reduces, hence the received SNR at each UE gets improved. This leads to the improvement of average SER performance of the given system.

Fig 9: SER vs N0 Plot

Fig. 9 flashes on the relationship between SER of the parallel-multihop relay combination system and noise power, i.e., N0. It can be observed that if N0 is reduced, SER performance improves due to the SNR improvement at each UE.

Fig 10: Overall throughput (Td) vs transmit power of a UE (P) plot for different values of relay UEs per hop (Nph).

Fig. 10 depicts the relationship between overall throughput (Td) of the parallel-multihop relay communication system and transmit power of an UE, i.e., P for different number of relays. It can be observed that if P is increased, the overall throughput performance improves due to the SNR improvement at each UE. Moreover, if the number of parallel relays per hop (Nph) is increased from one to two, the throughput of the overall system improves. The reason behind this is, if Nph is increased, the more data symbols can be transmitted to the remote UE by gNB through the relay paths simultaneously.

Simulation parameters are used to obtain the results are given below:

| Simulation parameter | Value |
| --- | --- |
| ND (number of parallel relays per hop) | 2 |
| Nc (cyclic prefix length) | 7 |
| α (Pathloss exponent) | 4 |
| Ns (number of sub-carriers) | 256 |
| dgu (distance between gNB and remote UE) | 24 m |

Table 1: Key simulation parameters

# Conclusion

Based on the discussions and the observations, we have the following proposals:

Proposal 1:	The multihop-parallel relay network may be established between a remote UE and a destination UE or a gNB. The relays will work in decode forward fashion. The proposal is to establish a radio resource control (RRC) connection between a UE and a gNB/UE using a multihop-parallel relay topology.

Proposal 2:	We propose multiple parameters like signal to noise ratio (SNR) and reference signal received power (RSRP) based relay selection/connection protocol at the RRC level using PC-5 and Uu links. At every hop, a RRC level procedure is proposed to connect multiple relay devices simultaneously.

# References

[1] 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 3GPP RP-193253 "New SID: Study on NR sidelink relay".

[3] 3GPP TS 23.303 "Proximity-based services (ProSe);Stage 2 ".

[4] RP-202208, Revised SID: Study on NR Sidelink Relay.