3GPP TSG RAN Meeting #106						                                         RP-242615

Madrid, Spain, December 9-12, 2024

## Status Report to TSG

Agenda item:			9.4.1.4

| WI / SI Name | # Introduction of another NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_Sband |  |  |  |  |
| Unique ID | 1040129 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241885 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 3/2025 | Performance part: 6/2025 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 50% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Laehteensuo, Toni |
|  | Company | Qualcomm Incorporated |
|  | Email | tlaehtee@qti.qualcomm.com |

# 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

# 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

### 2.1.1	Agreements

### 2.1.2	Remaining Open issues

## 2.2	RAN2

### 2.2.1	Agreements

### 2.2.2	Remaining Open issues

## 2.3	RAN3

### 2.3.1	Agreements

### 2.3.2	Remaining Open issues

## 2.4	RAN4

### 2.4.1	Agreements

#### 2.4.1.1	RAN4 #112bis

All agreements were captured in R4-2417206 “WF on Rel-19 FR1 NTN bands” and are reproduced below

#### Issue 1-1-1: Regional Applicability

- Agreement

  - Consider adding a note in clause 5.2 operating band to clarify regional applicability of band n252

#### Issue 1-1-2: General Methodology for UE to UE Coexistence between n252 UL and n2/b2/n25/b25 DL

- Agreement:

  - Define a new more relaxed spurious emission requirement for TN-NTN UE to UE coexistence compared to -50dBm/MHz, that allows the NTN service to work effectively, across the full n252 band, whilst still protecting the TN UE at least in the majority of cases

    - Further discuss the actual value and methodology for the requirement (e.g. corresponding separation assumption)

    - Companies are requested to propose what emission level in dBm/MHz could be achieved for frequencies below 1995 MHz, considering the UE RF implementation, different uplink channel bandwidths and RB allocations

    - Further discuss how to capture the solutions in the specification

#### Issue 1-1-3: TN-NTN Isolation Distance

- Agreement

  - For band n252 UE-to-UE co-existence study, consider a new TN-NTN separation distance lower than 1.5 km and further study the appropriate separation based on coexistence analysis

    - FFS on partial geographic overlap.

#### Issue 1-2-1: SS Raster

- Agreement

  - Specify both Case A and [Case B] as follows

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n252 | 15 kHz | Case A | 5456 – <1> – 5494 |
|  | [30 kHz] | [Case B] | [5460 – <1> – 5488] |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

#### Issue 1-2-3: UE Maximum Output Power

- Agreement

  - Specify PC3 maximum output power for band n252 as follows:

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n252 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

#### Issue 1-2-3: Two antenna port PREFSENS

- Agreement

  - FFS

#### Issue 1-2-4: UL Configuration for REFSENS

- Agreement

  - Agree Option 1

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n252 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
| NOTE: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. |  |  |  |  |  |  |

#### Issue 1-2-5: In-band blocking

- Agreement

Table 7.6.2-2: In-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n252, n254, n255, n256 | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 |  |  |  |  |

#### Issue 1-2-6: Out-of-band Blocking

- Agreement

  - FFS

#### Issue 1-2-7: Narrow Band Blocking

- Agreement

Table 7.6.4-1: Narrow Band Blocking

| Operating Band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 |
| n252, n254, n255, n256 | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 3 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  |
|  | Fuw (offset SCS= 30 kHz)3 | MHz | NA |  |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: The PREFSENS power level is specified in clause 7.3.2. NOTE 3: Fuw shall be rounded to half of SCS. |  |  |  |  |  |  |

#### Issue 1-3-1: SAN Operating Band

- Agreement

  - Specify the following SAN operating band for band n252:

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n252 | 2000 MHz - 2020 MHz | 2180 MHz - 2200MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

#### Issue 1-3-2: SAN Channel Bandwidths and SCS

- Agreement

  - Specify the following SAN channel bandwidths and SCS for band n252

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| n252 | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |

#### Issue 1-3-3: SAN NR-ARFCN with 100 kHz channel raster

- Agreement

  - Specify the following SAN NR-ARFCN for band n252

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n252 | 100 | 400000 – <20> – 404000 | 436000 – <20> – 440000 |

#### Issue 1-3-4: SAN NR-ARFCN with enhanced channel raster

- Agreement

  - Specify the following SAN NR-ARFCN for enhanced channel raster for band n252:

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n252 | 10 | 400000 – <2> – 404000 | 436000 – <2> – 440000 |

#### Issue 1-3-5: SAN SS Raster

- Agreement:

  - Agree Option 2 with both Case A and [Case B]

    - Need check if the 30KHz SCS SS raster is needed and whether case B is correct

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n252 | 15 kHz | Case A | 5456 – <1> – 5494 |
|  | 30 kHz | Case B | 5460 – <1> – 5488 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

#### Issue 1-3-5: Other SAN RF Requirements

- Agreement

  - Reuse existing SAN 1-H and 1-O RF requirements for band n252

#### Issue 1-4-1: Band Grouping

- Agreement

  - Agree Option 1 as follows

Table 3.5.2A-1: NR frequency band groups for satellite access in FR1

| Group | NR FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | NR_FDD_SAB_FR1_A | n254, n255, n256, n252 |
| B | NR_FDD_SAB_FR1_B |  |
| C | NR_FDD_SAB_FR1_C |  |
| D | NR_FDD_SAB_FR1_D |  |
| E | NR_FDD_SAB_FR1_E |  |
| F | NR_FDD_SAB_FR1_F |  |
| G | NR_FDD_SAB_FR1_G |  |
| H | NR_FDD_SAB_FR1_H |  |
| I | NR_FDD_SAB_FR1_I |  |
| J | NR_FDD_SAB_FR1_J |  |

#### 2.4.1.2	RAN4 #113

All agreements were captured in R4-2420465 “WF on Rel-19 FR1 NTN bands” and are reproduced below

#### Issue 1-1-1: Regional Applicability

Agreement:

  - No note is needed in clause 5.2 operating band of TS 38.108 and TS 38.101-5 to clarify regional applicability of band n252.

#### Issue 1-1-2: TN-NTN Isolation Distance

- Offline Agreement:

  - The TN-NTN coexistence assumption of 1.5 km separation between NTN UE and edge of the TN coverage from TR 38.863 has been used to guarantee the minimal performance on both TN and NTN networks. Nevertheless, this might not represent a typical deployment, and an NTN UE might still be able to operate within this isolation distance

    - Revision of the general TN-NTN coexistence assumptions is not in scope of this WI

#### Issue 1-1-3: UE to UE Coexistence between n252 UL and n2/b2/n25/b25 DL

- Tentative Agreement:

    - Agree to two different additional spurious emission requirements, each associated to an NS value and corresponding A-MPR, for protection of band b25/n25/b2/n2

      - FFS: -30 dBm/MHz for relaxed value.

        - FFS: This emission level applies and the associated A-MPR is allowed whether or not the NS value is signalled. Also, the protection level should a note equivalent to Note 15 in 38.101-1 which says “These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.”

      - FFS: [-40] dBm/MHz for the stricter value based on current b23 specification

    - The requirement is applicable in every country where b25/n25/b2/n2 and n252 are applicable

    - Specify asymmetric channel bandwidths with 5, 10 and [15] MHz UL CHBW for band n252.

#### Issue 1-1-4: Protection of n70 and n66

Agreement:

  - For new S-band n252, the following spurious emission requirements for UE-to-UE coexistence can be specified for addressing the UE-to-UE coexistence with band n70, n66.

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n252 | NR Band n70 | FDL_low | - | FDL_high | NA | NA | X |
|  | NR Band n66 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE X: The co-existence between n252 and band 70 is subject to regional/national regulation. |  |  |  |  |  |  |  |

#### Issue 1-2-1: SS Raster

Tentative agreement:

- For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster entries with SSB pattern Case B and Case C could be introduced.

#### Issue 1-2-2: UE TX-RX Separation

Agreement:

- Specify the flexible Tx-Rx separation for n252 with the wording aligned with n255 and n256 in 38.101-5.

#### Issue 1-3-1: SAN SS Raster

Tentative agreement:

- For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster entries with SSB pattern Case B and Case C could be introduced.

### 2.4.2	Remaining Open issues

- UE RF requirements including A-MPR, UE-coexistence requirements, reference sensitivity

- Detailed specification updates

## 2.5	RAN5

### 2.5.1	Agreements

### 2.5.2	Remaining Open issues

### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

### 2.6.1	Agreements

### 2.6.2	Remaining Open issues

# 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 3.1	SAx/CTs

### 3.1.1	Agreements with cross-TSG impacts

### 3.1.2	Remaining Open issues with cross-TSG impacts

# 4.	References

## RAN4#112bis

[1] R4-2415088	Discussion on SAN RF requirements for the NTN new S-band

[2] R4-2415811	draftCR to TS38.108 Introduction of NR-NTN S band

[3] R4-2416373	Introduction of NR-NTN S-band

[4] R4-2416475	Discussion on UE-to-UE coexistence

[5] R4-2415089	Discussion on UE RF requirements for the NTN new S-band

[6] R4-2415300	Methodology for n252 UE to UE coexistence evaluation

[7] R4-2415314	Initial view on the UE power back-off while operating in 2000-2020 MHz UL range

[8] R4-2415810	draftCR to TS38.101-5 Introduction of NR-NTN S band

[9] R4-2415960	Discussion on NTN-TN coexistence study for new NTN SBand

[10] R4-2416051	Discussion on the NTN S band definition

[11] R4-2416144	New NTN S-band - UE RF requirements

[12] R4-2416459	Co-existence considerations for the new S-band

[13] R4-2416507	NTN S-Band UE-UE coexistence with TN

## RAN4#113

[14] R4-2417753	On UE power back-off while operating in 2000-2020 MHz UL range of the NTN band n252

[15] R4-2417808	Discussion on general aspects for the NTN new S-band

[16] R4-2417809	Discussion on UE RF requirements for the NTN new S-band

[17] R4-2417810	Discussion on SAN RF requirements for the NTN new S-band

[18] R4-2417822	DraftCR for TS 38.108, introduction on operating band for NTN new S band

[19] R4-2418353	draftCR to TS38.101-5 Introduction of NR-NTN S band

[20] R4-2418354	draftCR to TS38.108 Introduction of NR-NTN S band

[21] R4-2418485	Introduction of NR-NTN S-band

[22] R4-2418913	New NTN S-band - UE RF requirements

[23] R4-2419068	Discussion on the NTN S band definition

[24] R4-2419375	NTN S-Band UE-UE coexistence

[25] R4-2419434	On n252 UE Coexistence Relaxation

[26] R4-2419519	Discussion on NR-NTN n252 UE RF requirements

[27] R4-2419522	Discussion on UE-to-UE coexistence of n252 UL interference with n2 DL and n25 DL

[28] R4-2419528	Co-existence considerations for the new S-band

[29] R4-2419716	Discussion on RF requirements for NR-NTN S-band