3GPP TSG RAN Meeting #106									RP-242623

Madrid, Spain, December 9-12, 2024

Status Report to TSG

Agenda item:		9.3.4.8

| WI / SI Name | WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | TRP_TRS_MIMO_OTA_Ph3 |  |  |  |  |
| Unique ID | 1030085 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241623 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Sep/2025 | Performance part: Mar/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 45% | Performance Part: 25% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Wang, Ruixin |
|  | Company | vivo |
|  | Email | ruixin.wang@vivo.com |
| Rapporteur | Name | Zhu, Siting |
|  | Company | CAICT |
|  | Email | zhusiting@caict.ac.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.4	RAN4

2.4.1	Agreements

RAN4 #112bis

General

- Topic summary for WI in [34][35]

- WF for MIMO OTA and TRP TRS were approved in [40] and [41], respectively.

- TP on Dynamic MIMO Aspects was agreed in [39]

- TR 38.762 skeleton on MIMO OTA dynamic test methodology was agreed in [12]

- Ad-hoc meeting minutes for TRP TRS topic in [38]

- Reply LS to CTIA on XR device type was approved in [36]

- TR 38.762 v0.1.0 was approved in [30]

Agreements for TRP TRS topic:

    - Sub-topic 1-1 General for WI

Issue 1-1-1: Discussion on phantom definition and positioning guideline for LTE and NR

Agreements:

  - RAN4 confirm that the LTE phantom definition and positioning guideline is same as NR, i.e., the phantom definition and positioning guideline is RAT agnostic.

Issue 1-1-2: Solution 1 in ETSI LS: Which RAN4 LTE OTA spec should be updated based on ETSI requests

Agreements:

  - RAN4 should update both TR 37.902 and TS 37.144 to accommodate the request from ETSI.

Issue 1-1-3: draft CR on adding RAN4 LTE test method for UE >72mm

Agreements:

  - Draft CR R4-2415770 is endorsed.

Issue 1-1-5: Reply LS to ETSI in RAN4 or RAN Plenary

Agreements:

  - Further confirm next meeting. Based on MCC feedback, RAN4 reply directly is workable.

    - Sub-topic 2-1 XR test methods and configurations

Issue 2-1-1: Reply LS to CTIA on two high-prioritized XR types

Agreements:

  - The reply LS to CTIA on XR phantom in R4-2416575 is approved.

Issue 2-1-2: 2RX XR configuration during testing

Agreements:

  - Set cellBarred2RxXR-r18 (ENUMERATED) to {false} during OTA tests for XR devices with 2Rx. This can be captured in TR as part of XR UE configuration.

Issue 2-1-3: General TRP TRS test method for XR

Agreements:

  - The glasses and head mounted displays should be tested starting from the existing TRP and TRS measurement methodology. Further optimization of test method is not precluded.

Issue 2-1-4: System QZ size for XR device

Agreements:

  - Encourage companies to check whether all XR devices can be fully accommodate within 30cm QZ when mounted on a head phantom.

  - RAN4 further discuss whether minimum measurement distance based on 50cm QZ should be added into TR.

    - Sub-topic 2-2 Performance metric for XR devices

Issue 2-2-1: Performance metric for XR devices

Agreements:

  - The glasses and head mounted displays should have the same performance metrics.

  - Further discuss whether performance requirements for the above two types of XR devices could be the same or not.

Sub-topic 2-3 Testing time reduction solutions for XR OTA

Issue 2-3-1: Coarse measurement grids for XR device testing

Agreements:

  - RAN4 further discuss coarse measurement grid based on XR device antenna pattern.

  - FFS whether current coarse grid in TR 38.870 can be used for XR device testing.

Issue 2-3-2: Low UL power solution

Agreements:

  - To reduce TRS test time, Single Point Offset Test (SPOT) with maximum transmit power can be performed and the end of the measurement to scale the TRS result measured under low UL power. The low transmit power is set as 10 dBm.

  - The final TRS is achieved by offsetting the TRS from the delta at Peak EIS, using both maximum output power (single point EIS) and low transmit power (full EIS).

  - Detailed test procedure and equation for this method can be provided next meeting.

Sub-topic 3-1 UE type and usage scenarios for NTN (NR-NTN and IoT-NTN)

Issue 3-1-1: Usage scenarios for NR-NTN mobile handheld UE

Agreements:

  - NTN UEs not supporting voice call or only supporting voice call with loud speaker etc. are only required and tested with hand only mode.

Issue 3-1-2: Check whether the UE orientation is arbitrary or directional during NR-NTN communication

Agreements:

  - For NR-NTN, the target usage of NR-NTN UE is close to normal NR UE.

Sub-topic 3-2 UE performance metric

Issue 3-2-1: Performance metric for NTN device

Agreements:

  - For NR-NTN UEs, take a whole sphere measurement with TRP and TRS as the performance metrics for all usage modes as baseline. Other performance metric can be considered if issue is identified.

  - For IoT-NTN UEs, FFS performance metric.

Issue 3-2-2: Whether performance metric should be different for GEO and NGEO for handheld UEs

Agreements:

  - FFS whether different metrics for GEO and NGEO would be needed.

Sub-topic 3-3 NTN OTA test methodologies

Issue 3-3-1: detailed test parameters for NR-NTN band n254

Agreements:

  - RAN4 further check whether the test parameters in Table 1 and Table 2 can be used as test parameters for NR-NTN band n254 OTA testing or not.

- Table 1: NR FR1 NR-NTN TRP measurement parameters

| NR Band | CBW [MHz] | SCS (kHz) | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | UL RB Allocation (LCRB @ RBstart) | DL configuration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n254 | 5 | 15 | DFT-s-OFDM QPSK | Low | 322500 | 1612.5 | 497200 | 2486 | 12@6 | N/A |
|  |  |  |  | Mid | 323700 | 1618.5 | 498400 | 2492 |  |  |
|  |  |  |  | High | 324800 | 1624 | 499500 | 2497.5 |  |  |

- Table 2: NR FR1 NR-NTN TRS measurement parameters

| NR Band | CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | UL RB Allocation (LCRB @ RBstart) | DL Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n254 | 5 | 15 | CP-OFDM QPSK | DFT-s-OFDM QPSK | Low | 322500 | 1612.5 | 497200 | 2486 | 25@0 | 25@0 |
|  |  |  |  |  | Mid | 323700 | 1618.5 | 498400 | 2492 |  |  |
|  |  |  |  |  | High | 324800 | 1624 | 499500 | 2497.5 |  |  |

Issue 3-3-2: Detailed test parameters for IoT-NTN

Agreements:

  - RAN4 further check whether the following parameters can be used for IoT-NTN:

  - For NB IoT-NTN, select QPSK with 1@0 Ntones as TRP-related test parameter, QPSK with 12 Subcarriers as TRS-related test parameters.

  - For Cat M1 NTN, select QPSK with 1RB allocation as TRP-related test parameters, QPSK with 4 RB allocation as TRS-related test parameters.

Issue 3-3-4: How to resolve UE antenna circular polarization issue

Agreements:

  - RAN4 to further study the need for circular polarization for NR NTN conformance testing

  - Feedback from OEMs and Satellite Vendors on the need for CP testing and test system impacts to support CP OTA conformance testing from TE/System vendors is requested.

Sub-topic 4-1 2Tx FR1 TRP TRS requirements

Issue 4-1-1: working procedure for 2Tx lab alignment activity

Agreements:

  - RAN4 adopt the following working procedure to guide Rel-19 2Tx lab alignment activity. Further improvement of the working procedure is not precluded.

Working procedure for Rel-19 AC 2Tx Lab Alignment Campaign

- The purpose of 2Tx (non-coherent UL-MIMO) Lab Alignment Campaign is to ensure there is no unexpected lab deviation and establish full trust and confidence on the results of UE with 2Tx configuration.

- Test labs are invited to participate to the lab alignment and test campaign, the following conditions should be fulfilled:

  - Participating lab should be accredited under ISO 17025 (ISO 17025 accredited labs) and have any of 3GPP TS 37.544, CCSA YD/T 1484.6, and CTIA OTA Test Plan listed on its accreditation scope.

  - Participating lab should have anechoic chamber(s) ready to support 1Tx and 2Tx TRP TRS testing based on 3GPP TR 38.870.

  - Participating lab should have sufficient test resource to provide the on-time measurement results without delay.

- Test methodology:

  - Test plan: 3GPP TR 38.870 v18.2.0;

- Test cases for Lab Alignment Campaign:

  - Test bands: For volunteer labs that have already been declared aligned in Rel-17 and Rel-18 lab alignment campaigns: focus is only on [n41, n78], two typical bands non-coherent UL-MIMO; (For volunteer labs that have not been declared aligned in Rel-17 and Rel-18: In addition to 2Tx, also complete 1Tx lab alignment on at least n41 (high band) [and n28 (low band)])

  - Number of test cases per band: TRP and TRS; [BHHL and BHHR at low/mid/high channel, HL and HR at low/mid/high channel];

  - Use scenario: [Head and Hand phantom (Talk mode) and Hand only (Browsing mode)]

  - Hand Phantom: Wide Grip hand

  - Operation mode: NR Standalone (SA)

  - Number of Tx chain: UE with 2Tx (non-coherent UL MIMO)

- Lab Alignment Device (LAD): in total 2 devices

  - 2 New devices (LAD 5 and LAD 6) for band n41 and n78 non-coherent UL-MIMO;

  - UE information will be further added:

    - Smartphone DUT size: width >72mm and ≤92mm

    - DUT capability: support for all the prioritized bands listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the lab alignment campaign (also pending further prioritization of the bands in the WID)

  - Power Class: PC2/PC3 (n41 and n78)

  - Current priority is UEs supporting non-coherent UL-MIMO. Coherent UL-MIMO supporting smartphones and XR devices are FFS. TxD is not part of scope.

- Test results submitting:

  - Using the same worksheet template in [Rel-18 R4-2313892 as starting point, update is needed] to submit the measurement results

  - The measurement results of LADs should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - Results shall not be shared between labs before submitting to RAN4 meetings or sharing in the RAN4 reflector. Comparison and lab alignment analysis should only be done in RAN4 meetings/discussions

  - Sharing of the measurements results of a specific device from test labs is permitted only towards the 3GPP member that provided such device (i.e., device’s owner) before submitting to RAN4

- Lab alignment criteria:

  - The pass/fail criteria are defined as the maximum deviation between the measurement result and the reference value

  - Confirm the reference value derived based on the per-band per-PC averaging approach (linear average with dBm) of lab alignment data pool from ≥3 labs submitted before end of [RAN4#TBD] as baseline.

  - Apparent outliers will not be considered in averaging process. The value deviates over 1.5*MU from all the other lab’s results should be identified as apparent outlier.

  - Pass/fail limit for lab alignment should be defined as 0.75*MU as baseline. MU value is the expanded MU for 2Tx condition (need further check whether same as 1Tx).

  - The summation form for TRP and TRS lab alignment should keep consistent during the calculation process of TRP TRS lab alignment from each company i.e. sin weights approach. Only traditional approach (15-degrees TRP and 30-degrees TRS) should be used during lab alignment activity to reduce unnecessary uncertainty.

  - The timeline of 2Tx AC alignment activity is in [TBD]. How to treat late submission results and confirm the alignment: TBD

- Test lab procedures:

  - LAD delivery scheme

    - Decide LAD delivery scheme after all the test lab and LAD information being confirmed (after the confirmation of volunteered labs and LADs).

  - LAD measurement time in each test lab: finalize LAD measurement within [5] workdays, and deliver to next lab ASAP with LAD delivery In/Out information shared in reflector.

  - Encourage test labs to share resulting combined MU based on their own systems.

Table 1: Rel-19 2Tx LAD information

| Rel-19 2Tx Lab Alignment Devices (LADs) for non-coherent UL MIMO |  |  |  |
| --- | --- | --- | --- |
| DUT | Provider/Owner | Contact | Note |
| LAD5 (new device for Rel-19 2Tx) | vivo | Ruixin Wang ruixin. wang@vivo.com | For n41 and n78 testing. |
| LAD6 (new device for Rel-19 2Tx) | OPPO | Qifei Liu liuqifei@OPPO.com | For n41 and n78 testing. |
| Note: To keep the test burden, two devices per band. |  |  |  |

Issue 4-1-2: Collecting volunteer test labs for 2Tx lab alignment activity

Agreements:

  - WI rapporteur trigger offline discussion to collect volunteer test labs for 2Tx lab alignment activity.

Issue 4-1-3: Whether there is MU difference for 2Tx test system

Agreements:

  - RAN4 can further discuss this issue to check whether there is MU impact.

Issue 4-1-4: detailed timeline schedule for 2Tx lab alignment

Agreements:

  - RAN4 adopt the following timeline schedule for 2Tx lab alignment activity. Further optimization is not precluded.

  - Generally, the timeline should follow the WI workplan in R4-2412056.

  - RAN4 confirm 2Tx LADs information and volunteer test labs after RAN4#112bis meeting. The lab alignment activity can be started after LADs and >3 volunteer test labs confirmed.

  - It is planned to finalize the lab alignment testing before RAN4#114 meeting in China test labs. And finalize all the test labs testing before [RAN4#114bis].

Issue 4-1-5: 2Tx requirements related performance measurement campaign

Agreements:

  - RAN4 need to check the feasibility of obtaining a sufficient number of commercial devices that support non-coherent UL MIMO.

  - Encourage vendors to provide input and support, facilitating the gathering of relevant data on available devices.

Sub-topic 4-2 1Tx FR1 TRP TRS requirements

Issue 4-2-1: How to specify PDA hand related TRP/TRS requirements (assume no large scale measurement campaign, current requirement is a basis)

Agreements:

  - RAN4 can initiate a preliminary (not large scale) measurement campaign for size 2 UEs, FFS how to down select between measurement-based approach and the offset-based approach.

  - Encourage volunteer test labs to perform the above measurement campaign, Rel-17 and/or Rel-18 aligned test lab is preferred.

  - Encourage companies to provide seize 2 UEs (small phone with PDA hand) for preliminary measurements.

  - Note: Size 1 (wide, width >72mm and ≤92mm), Size 2 (narrow, width ≥56mm and ≤72mm);

Issue 4-2-2: PDA hand related TRP/TRS requirements

Agreements:

  - RAN4 further discuss how to specify PDA hand related TRP/TRS requirements (Size 2 UEs).

Issue 4-2-3: Scope for 1Tx requirements in Rel-19

Agreements:

  - Further discuss and conclude the scope for 1TX requirements in Nov meeting. This will be reflected in WID in Dec RAN-P.

Issue 4-2-4: Working procedure for 1Tx requirements related performance measurement campaign

Agreements:

  - RAN4 adopt the following working procedure for 1Tx UE performance measurement campaign. Further optimization is not precluded.

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: [1Tx at new bands n3, n5, n7, n8, n20, n38, n77, n79; 2Tx at bands n41 and n78;]

  - Use scenarios: Both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR

  - Hand Phantom: Wide Grip Hand as first priority, PDA hand is not precluded if sufficient devices are available

  - Operation mode: SA with 1Tx  as phase 1. SA with 2Tx (non-coherent UL-MIMO) as phase 2 after 2Tx lab alignment activity.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: Size 1(width >72mm and ≤92mm)

  - DUT capability: support for Bands those listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: [2022-2025]

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. For bands support both PC2 and PC3, focus on PC2 measurements, but both PC2 and PC3 requirements are needed. For PC3 bands, only PC3 requirements.

- Devices provisioning:

  - Any 3GPP member can work with test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

    - FFS for the same UE model with supporting different set of bands

  - Logistical aspects for devices provisioning to the labs are TBD

- Test results submitting:

  - UE information disclosure: the following rules should be considered

    - UE model information based on template [Rel-18 R4-2408104 as starting point, update is needed] can be shared to MCC. RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure.

    - laboratories use the same template to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

      - Information of the devices that are going to be measured shall be shared with the RAN4 Secretary as soon as available (i.e., before the measurement activity on such devices starts)

      - The RAN4 Secretary updates the summary of statistical information  and publishes it to 3GPP RAN4 (i.e., living document) in order to monitor the achievement of the thresholds in a timely manner and take the proper actions if these are not met

    - Statistics of UE information can be shared to RAN4 reflector after TBD meeting by RAN4 secretary after post-processing

    - Consider following aspects as thresholds related to the devices pool for Rel-19

      - Minimum number of devices for each band, each device size, each power class: TBD

      - Minimum number of device models: TBD

      - Minimum number of devices' vendors: TBD

      - Percentage of devices from second-half 2022 to 2025: TBD

      - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: TBD

  - Laboratories use the same worksheet template in [Rel-18 R4-2315844 as starting point, update is needed] to submit the measurement results for Rel-19 3GPP TRP TRS performance data pool.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - The allowed maximum number of submitted devices from each lab is [15] (depends on how many test labs will join the activity) with a minimum of 3 devices.

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to share on the RAN4 reflector (for example - how many devices have been measured and on which bands)

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): [30]

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-19.

  - Method of limits derivation: per-band Data driven approach

  - The value at [80%] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - For a band supporting both PC2 and PC3, specify PC3 requirement based on finalized PC2 requirements, for TDD bands 3dB offset, for FDD bands TBD.

- Test lab procedures for 1Tx

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - For UE support PC2 at one band, PC3 testing is not needed.

  - Newly defined Coarser measurement grid in TR 38.870 can be used for TRP TRS measurement

- Test lab procedures for 2Tx

  - [For each device, record the per-band UE capability listed under the rf-Parameters -> pusch-TransCoherence where the possible entries are

pusch-TransCoherence: ENUMERATED {nonCoherent, partialCoherent, fullCoherent}

  - This is to confirm and align the non-coherent UL-MIMO capability of the devices picked for lab alignment and later on, the performance campaigns.]

Agreements for dynamic MIMO OTA topic:

Sub-topic 1-1 Dynamic channel model generation and validation

Issue 1-1-1: Dynamic MIMO OTA channel model generation

Agreements:

- Encourage CE vendors to provide more details on how to handle the fixed channel parameters in dynamic MIMO OTA channel model, e.g., the coupling of ray angles, in dynamic MIMO OTA channel model for continuous channel modelling.

Issue 1-1-2: Dynamic channel model validation target values

Agreements:

- Approve the dynamic channel model target values in R4-2416578 with the understanding that additional changes and modifications can be made.

Sub-topic 1-2 Test system and Test parameters for Dynamic MIMO OTA

Issue 1-2-1: The maximum number of HARQ transmissions

Agreements:

- Take the following test parameters as the starting point and further check the feasibility and potential impact on dynamic MIMO OTA testing.

  - Maximum number of HARQ transmission: [4]

  - Number of HARQ processes for TDD bands: [8]

  - Number of HARQ processes for FDD bands: [4]

- Measurements can be performed to compare the measurement results with Maximum Number of HARQ Transmission = 1 vs. 4.

Issue 1-2-2: CSI configuration for dynamic link adaptation

Agreements:

- Remove option “00000010 for fixed Rank 2” from the CSI configuration Table and only keep option “00001111 for follow RI”.

Issue 1-2-3: TP on Dynamic link adaptation

Agreements:

- The changes in Clause 9.3 in R4-2416578 are approved.

Issue 1-2-4: TP on Minimum range length for MPAC system

Agreements:

- The changes in Clause 6.3 in R4-2416578 are approved.

Sub-topic 1-3 Dynamic MIMO OTA test and UE performance metric

Issue 1-3: Dynamic MIMO OTA test and UE performance metric

Agreements:

- Consider dynamic channel MIMO OTA tests to include both UMa and UMi channels with tests on UMa and UMi channels carried out separately and their measurement results combined into a single data pool for processing.

Sub-topic 1-4 Dynamic MIMO OTA Measurement Campaign

Issue 1-4: Preliminary results from Dynamic MIMO OTA Measurement Campaign

Agreements:

- Given the lack of similarities in ranking between TRS, dynamic MIMO OTA with UE noise-limited and SIR-based environments, continue with the UE noise-limited environment in 3GPP for dynamic MIMO OTA.

Sub-topic 2-1 General issues on Rel-19 MIMO OTA requirements

Issue 2-1-1: Target bands for Rel-19 MIMO OTA requirements

Agreements:

- Take the bands listed in the WID (n3, n8, n77, n79) as baseline, check if there is a need to further prioritize the bands and/or introduce new bands.

- Keep a reasonable workload with 4 bands in Rel-19 MIMO OTA performance requirements work.

Issue 2-1-2: 4x4 requirement or 2x2 requirement for band n3

Agreements:

- RAN4 should make the decision on defining which MIMO OTA requirement for band n3 at RAN4#113 (Nov. 2024).

  - Option 1: 4x4 MIMO OTA requirement

  - Option 2: 2x2 MIMO OTA requirement

- Encourage OEMs to provide the information on the proportion of 4Rx UEs in band n3.

Sub-topic 2-2 Framework for Rel-19 MIMO OTA requirements

Issue 2-2-1: Overall work flow

Agreements:

- Approve the following overall work flow:

The overall work flow of Rel-19 MIMO OTA performance requirements development for bands <1GHz (n8) and bands >1GHz (n3/n77/n79) are illustrated in Figs. 1 (a) and (b), respectively.

(a) for bands <1GHz (n8)                           (b) for bands >1GHz (n3/n77/n79)

Fig. 1.  Work flow of Rel-19 MIMO OTA performance requirements development

In general, Rel-19 MIMO OTA performance requirements will be derived based on a measurement data pool of commercial devices per band. To establish valid and trustable measurement data pools for defining FR1 MIMO OTA performance requirements, the following activities are required.

  - For bands <1GHz (n8)

    - Channel Model Validation: Companies shall complete channel model validation before submitting measurement results, validation results should be submitted to RAN4 for review. Working procedures of the channel model validation are provided in Section 3.2.1.

      - Note: Labs that submitted channel model validation results for band n8 in Rel-18 (as captured in TR 38.761) do not need to submit the results again.

    - Measurement Campaign: Collect measurement data from Rel-18 aligned labs (Apple, CAICT, CMCC&BUPT, MediaTek, OPPP, Xiaomi) for defining FR1 MIMO OTA requirements for bands < 1GHz. Working procedures of the measurement campaign are provided in Section 3.2.2.

  - For bands >1GHz (n3/n77/n79)

    - Channel Model Validation: Companies shall complete channel model validation before submitting measurement results, validation results should be submitted to RAN4 for review. Working procedures of the channel model validation are provided in Section 3.2.1.

      - Note: Rel-17 aligned labs do not need to submit channel model validation results for band n77, because the channel model validation results for band n78 are applicable to band n77 due to the same channel model and validation frequency.

    - Measurement Campaign: Collect measurement data from Rel-17 aligned labs (Apple, CAICT, CMCC&BUPT, Huawei, MediaTek, Xiaomi) for defining FR1 MIMO OTA requirements for bands > 1GHz. Working procedures of the measurement campaign are provided in Section 3.2.2.

Detailed working procedures for Rel-19 MIMO OTA performance requirements development are described in Section 3.2.

Issue 2-2-2: Working procedures for channel model validation

Agreements:

- Approve the following working procedures as the starting point:

3.2.1 Channel Model Validation

- The purpose of Channel Model Validation is to ensure that the channel models are correctly implemented and hence capable of generating the propagation environment, as described by the model, within the test zone of the MPAC system.

- The channel model validation measurements shall be performed as described in Annex C.3 of TS 38.151, including:

  - Power Delay Profile (PDP)

  - Doppler/Temporal correlation

  - Spatial correlation

  - Cross-polarization

  - Power validation

- Test band: n3, n8, n79; Test frequency: as specified in Tables C.3.1-1 and C.3.1-2 of TS 38.151

- Channel model: as specified in Annex C.1 of TS 38.151

  - For band n8: FR1 UMi CDL-C (2x2 MIMO)

  - For band n79: FR1 UMa CDL-C (4x4 MIMO)

  - For band n3: TBD (FR1 UMa CDL-C or FR1 UMi CDL-C)

- Pass/fail limits: as defined in Annex C.4 of TS 38.151

Issue 2-2-3: Working procedures for measurement campaign

Agreements:

- Approve the following working procedures as the starting point:

3.2.2 Measurement Campaign

- The purpose of Measurement Campaign is to collect measurement results of commercial devices from permitted labs for specifying FR1 MIMO OTA performance requirements.

- Test cases for Rel-19 FR1 MIMO OTA Measurement Campaign:

  - Test bands: the bands listed in the WID (n3, n8, n77, n79) is the baseline; FFS whether to further prioritize the bands in the WID, and whether to introduce more bands (keep a reasonable workload with 4 bands)

  - MIMO layer:

    - For band n8: 2x2 MIMO (with FR1 UMi CDL-C channel model)

    - For bands n77, n79: 4x4 MIMO (with FR1 UMa CDL-C channel model)

    - For band n3: TBD (2x2 or 4x4)

  - Use scenario: Free space

  - Operation mode: SA (first stage)

  - Power class: PC3 (first stage)

- Commercial device (Smartphone) selection criteria for FR1 MIMO OTA Measurement Campaign:

  - DUT capability: supporting all the bands n3, n8, n77, n79 listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - DUT variety: the selection of commercial devices should cover various of devices in the market. The following selection criteria can also be considered:

    - Year of production: 2021-2025

    - Brand variety

    - Price range (to cover different price ranges, including High/Mid/Low-end products)

    - Popularity

    - Number of bands supported

  - For band n3, UEs equipped with [TBD, 4 or 2] Rx antenna ports shall be selected for the Measurement Campaign. Labs can try to identify 4Rx or 2Rx UEs in any methods listed below:

    - Method 1: If a UE can be connected to BS simulator with 4x4 MIMO, the UE can be confirmed as a 4Rx UE

    - Method 2: Obtain the MIMO layer information from BS simulator, e.g., check the IE maxNumberMIMO-LayersPDSCH

    - Method 3: Directly collect the information from OEMs.

    - Other methods are not precluded

- Commercial devices provision:

  - Test labs can prepare and collect commercial devices by themselves based on the above selection criteria.

  - Any 3GPP member can work with the selected test labs to provide devices

    - A test lab shall measure only one UE model in case different samples are provided

    - Same UE model supporting different sets of bands can be measured. For this case, the UE model should be marked as different model, e.g., model A-1, model A-2. (guidance on how to manage this case are provided in the spreadsheet in R4-2321191)

    - For band n3, make sure the devices are [TBD, 4Rx or 2Rx] UEs

    - The 3GPP member providing the DUTs should contact one of the selected labs to check their availability to receive the DUTs and define together the related provisioning aspects. Any issue should be reported to the rapporteur in a timely manner to discuss for an alternative solution

- Measurement results submission:

  - RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure

  - UE information disclosure: labs use the spreadsheet in R4-2321191 to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

  - Labs use the worksheet template in [TBD, use Rel-18 template R4-2403019 as starting point] to submit the measurement results for Rel-19 3GPP FR1 MIMO OTA performance data pool. Do NOT apply any systematic offset to the measurement results.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed):

    - The minimum number of submitted devices from each lab is 3, the maximum number is 15. Meanwhile, labs are encouraged to provide as much data as possible within 15

    - Volunteer labs provide the device information sheet ONLY to the RAN4 Secretary and the sheet used to submit measurement results to 3GPP RAN4

  - RAN4 Secretary ONLY publishes to 3GPP RAN4 the following summary of statistical information after anonymizing the sensitive UE information data, i.e., UE model name and vendor name:

    - Total number of devices

    - Total number of models

    - Total number of devices vendors

    - Percentage of devices per vendor

    - Percentage of devices per Power Class

    - Percentage of devices per each supported band

    - Percentage of devices per year of production

    - Percentage of the devices that are certified by at least one of certification bodies as following: PTCRB, GCF, NAL/CTA (Chinese Network Access Licensed Test), FCC, CE

      - Once the device gets the above certification, for RAN4 discussion that means the device is not a prototype

- Percentage of the devices that are certified for each certification body (for information only)

Issue 2-2-4: Working procedures for specifying performance requirements

Agreements:

- Approve the following working procedures as the starting point:

3.2.3 Specifying Performance Requirements

- Only measurement data from the aligned labs with channel model validation completed will be considered for specifying requirements, as described in Section 3.1.

- Thresholds for specifying FR1 MIMO OTA performance requirement:

  - Minimum amount of data per band: 15

  - Minimum amount of UE models per band: 12

- Method of requirements derivation: per-band Data driven approach

- The value at [85%] percentile of the CDF curve can be selected as the starting point for requirement discussion

- Performance part of the work will proceed in a contribution-driven manner.

Issue 2-2-5: Time plan for Rel-19 MIMO OTA requirements work

Agreements:

- Approve the initial time plan as below:

1. Finalize the working procedures and time plan for Rel-19 MIMO OTA requirements development at RAN4 #113 (Nov 2024).

2. Measurement Campaign for bands n77/n8 can start at an early stage with the Rel-17/Rel-18 aligned labs that completed channel model validations for these bands.

3. Labs should submit channel model validation results for bands n3/n8/n79 by formal T-docs no later than RAN4 #114-bis (Apr 2025) to participate in the Measurement Campaign at the corresponding bands. Earlier submission is encouraged.

4. To properly plan the Measurement Campaign, the following actions are requested no later than RAN4#115 (May 2025):

  - The rapporteur checks with the volunteer labs the amount of DUTs (minimum 3, maximum 15) they expect to be able to measure AND how many DUTs they can accommodate from 3GPP members

  - The 3GPP members providing devices for the measurement campaign (if any) check how many samples they intend to provide (in terms of maximum number)

  - Planning of the measurement campaign could be reviewed based on the above points

5. The 3GPP members (if any) should provide the devices to volunteer labs no later than RAN4#116 (Aug 2025).

6. Labs should submit measurement results of commercial devices by formal T-docs no later than RAN4#117 (Nov 2025), also submit the devices information to RAN4 Secretary.

7. Conclude FR1 MIMO OTA performance requirements no later than RAN4#118 (Feb 2026).

RAN4 #113

General

- Topic summary for WI in [32][33]

- WF for MIMO OTA topic and TRP TRS topic were approved in [43] and [38], respectively.

- Framework for Rel-19 MIMO OTA requirements was approved in [41]

- Ad-hoc meeting minutes for TRP TRS topic in [37]

- Draft Rel-19 CRs on XR and NTN OTA test methods were endorsed in [35][36]

- Reply LS to ETSI on 3GPP LTE TRP TRS test method for phone >72mm was approved in [10]

- (Template)Measurement results for Rel-19 TRP TRS 2Tx AC lab alignment activity was agreed in [15]

Agreements for TRP TRS topic:

    - Sub-topic 1-1 General for WI

Issue 1-1-1: Reply LS to ETSI on 3GPP LTE TRP TRS test method for phone >72mm

Agreements:

  - The reply LS in R4-2418206 is approved.

Issue 1-1-2: draft Rel-19 CR for general part of TRP TRS test method

Agreements:

  - The draft CR in R4-2419803 is endorsed.

Sub-topic 2-1 XR test methods and configurations

Issue 2-1-1: System QZ size for XR device

Agreements:

  - RAN4 further check whether minimum measurement distance for AC system with 50cm QZ should be added into spec.

Sub-topic 2-2 Performance metric for XR devices

Issue 2-2-1: General views on Performance requirements for XR devices

Agreements:

  - FFS on whether Glasses and head-mounted displays should have separate performance requirements.

Sub-topic 2-3 Testing time reduction solutions for XR OTA

Issue 2-3-1: Coarse measurement grids (those already specified in TR 38.870) for XR device testing

Agreements:

  - FFS whether coarse grid defined in TR 38.870 can be used for XR glasses.

Issue 2-3-2: Other Coarse measurement grids not specified in TR 38.870

Agreements:

  - Study on other coarse grids (e.g., Icosahedron grid) can be contribution driven manner.

  - The interpolated coarse measurement grids method for TRP measurements in R4- 2413195 can be captured in the TR.

Issue 2-3-3: alternative scan procedure for TRP to reduce testing time

Agreements:

  - The spiral scan procedure for TRP measurements can be captured in the TR. MU of this procedure should be studied.

  - The applicability of this procedure can be further discussed.

Issue 2-3-4: Low UL power solution

Agreements:

  - The draft CR in R4-2419806 for Low UL power solution is approved.

Sub-topic 3-1 UE type, usage scenarios and performance metric for NTN (NR-NTN and IoT-NTN)

Issue 3-1-2: General view on Usage scenarios for handheld IoT-NTN and NR-NTN

Agreements:

  - For IoT-NTN and NR-NTN,  typical usage scenarios are:

    - Scenario 1: both talk mode (head+hand, and hand only) and browsing mode.

    - Scenario 2: browsing mode (hand only)

  - For IoT-NTN UEs not supporting voice call or only supporting voice call with loud speaker etc. are only required and tested with hand only mode.

  - UEs supporting Scenario 1 are tested with head+hand and hand only modes

  - UEs supporting Scenario 2 only are only tested with hand only mode

Issue 3-1-3: Handheld IoT-NTN and NR-NTN performance metric

Agreements:

  - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 1 the performance metrics are full sphere TRP and TRS

  - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 2 only, further study performance metric

Issue 3-1-4: non-Handheld IoT-NTN performance metric

Agreements:

  - FFS on partial sphere metrics, such as the radiated power and receive sensitivity within a certain angular range, should be performance metric for non-handheld IoT NTN devices.

  - This can be considered as a package with issue 3-1-3.

Issue 3-1-5: Whether performance metric should be same for GEO and NGEO for handheld UEs (both NR/IoT NTN)

Agreements:

  - For UEs support both GSO and NGSO, single set of performance metric is required for GSO and NGSO handheld UEs.

  - For UE support only GSO or NGSO, FFS performance metric should be the same.

Issue 3-1-6: How to determine hand only mode for NR/IoT NTN mobile handheld UE (if we call it as Type 1 NTN UEs, usage-mode restriction)

Agreements:

  - RAN4 to discuss how to distinguish UEs which support Scenario 1 (talk and browsing modes) vs. Scenario 2 only (browsing mode only)

Sub-topic 3-2 NTN OTA test methodologies

Issue 3-2-1: detailed test parameters for NR-NTN band n254

Agreements:

  - The test parameters in Issue 3-3-1 in WF R4-2416590 for NR-NTN n254 OTA testing is confirmed. CR will be provided next meeting.

Issue 3-2-2: Detailed test parameters for IoT-NTN

Agreements:

  - The test parameters in Issue 3-3-2 in WF R4-2416590 for IoT-NTN OTA testing is confirmed. CR will be provided next meeting.

Issue 3-2-3: Doppler conditions for NTN

Agreements:

  - The following setting can be a starting point for further checking:

    - NR/IoT NTN radiated requirements shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

Issue 3-2-4: How to resolve UE antenna circular polarization issue

Proposals

  - Proposal 1: Use circular polarisation test antennas to assess the performance of the UE. This enables us to characterise the exact loss seen by the UE when operating in a system that utilises circular polarisation antennas, as all satcom systems do.

    - If we ignore circular polarisation testing at this stage, it may preclude the ability of an operator to leverage XPD to increase the capacity of their system.

  - Proposal 2: The NR FR1 NTN conformance testing should focus on LP rather than CP assessments.

- WF

  - FFS

Issue 3-2-6: Testing time reducing method for NTN

Agreements:

  - FFS on the NTN RTT (Round Trip Time) to reduce TRS measurement time. Feedback from NTN operators and TE vendors requested.

Sub-topic 4-1 2Tx FR1 TRP TRS requirements

Issue 4-1-1: working procedure update for 2Tx lab alignment activity

Agreements:

  - The updated working procedure below to guide Rel-19 2Tx lab alignment activity is approved.

Working procedure for Rel-19 AC 2Tx Lab Alignment Campaign

- The purpose of 2Tx (non-coherent UL-MIMO) Lab Alignment Campaign is to ensure there is no unexpected lab deviation and establish full trust and confidence on the results of UE with 2Tx configuration.

- Test labs are invited to participate to the lab alignment and test campaign, the following conditions should be fulfilled:

  - Participating lab should be accredited under ISO 17025 (ISO 17025 accredited labs) and have any of 3GPP TS 37.544, CCSA YD/T 1484.6, and CTIA OTA Test Plan listed on its accreditation scope.

  - Participating lab should have anechoic chamber(s) ready to support 1Tx and 2Tx TRP TRS testing based on 3GPP TR 38.870.

  - Participating lab should have sufficient test resource to provide the on-time measurement results without delay.

- Test methodology:

  - Test plan: 3GPP TR 38.870 v18.2.0;

- Test cases for Lab Alignment Campaign:

  - Test bands: For new labs only join 2Tx lab alignment, the additional lab alignment work of 1Tx is not needed

  - Number of test cases per band: TRP and TRS; BHHL and BHHR at low/mid/high channel;

  - Use scenario: Head and Hand phantom (Talk mode)

  - Hand Phantom: Wide Grip hand

  - Operation mode: NR Standalone (SA)

  - Number of Tx chain: UE with 2Tx (non-coherent UL MIMO)

- Lab Alignment Device (LAD): in total 2 devices

  - 2 New devices (LAD 5 and LAD 6) for band n41 and n78 non-coherent UL-MIMO;

  - UE information will be further added:

    - Smartphone DUT size: width >72mm and ≤92mm

    - DUT capability: support for all the prioritized bands listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the lab alignment campaign (also pending further prioritization of the bands in the WID)

  - Power Class: PC2 (n41 and n78)

  - Current priority is UEs supporting non-coherent UL-MIMO. Coherent UL-MIMO supporting smartphones and XR devices are FFS. TxD is not part of scope.

- Test results submitting:

  - Using the same worksheet template in R4-2418214 to submit the measurement results

  - The measurement results of LADs should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - Results shall not be shared between labs before submitting to RAN4 meetings or sharing in the RAN4 reflector. Comparison and lab alignment analysis should only be done in RAN4 meetings/discussions

  - Sharing of the measurements results of a specific device from test labs is permitted only towards the 3GPP member that provided such device (i.e., device’s owner) before submitting to RAN4

- Lab alignment criteria:

  - The pass/fail criteria are defined as the maximum deviation between the measurement result and the reference value

  - Confirm the reference value derived based on the per-band per-PC averaging approach (linear average with dBm) of lab alignment data pool from ≥3 labs submitted before end of [RAN4#TBD] as baseline.

  - Apparent outliers will not be considered in averaging process. The value deviates over 1.5*MU from all the other lab’s results should be identified as apparent outlier.

  - Pass/fail limit for lab alignment should be defined as 0.75*MU as baseline. MU value is the expanded MU for 2Tx condition (need further check whether same as 1Tx).

  - The summation form for TRP and TRS lab alignment should keep consistent during the calculation process of TRP TRS lab alignment from each company i.e. sin weights approach. Only traditional approach (15-degrees TRP and 30-degrees TRS) should be used during lab alignment activity to reduce unnecessary uncertainty.

  - The timeline of 2Tx AC alignment activity is in [R4-2416590]. How to treat late submission results and confirm the alignment: TBD

- Test lab procedures:

  - LAD delivery scheme

    - Decide LAD delivery scheme after all the test lab and LAD information being confirmed (after the confirmation of volunteered labs and LADs).

  - LAD measurement time in each test lab: finalize LAD measurement within [5] workdays, and deliver to next lab ASAP with LAD delivery In/Out information shared in reflector.

  - Encourage test labs to share resulting combined MU based on their own systems.

Issue 4-1-2: latest status of volunteer test labs for 2Tx lab alignment activity

Agreements:

  - Confirm the volunteer test labs information for 2Tx lab alignment below.

| Rel-19 Volunteer lab for 2Tx lab alignment activity |  |  |  |
| --- | --- | --- | --- |
| Volunteer lab | Location | Contact | Note |
| vivo | Dongguan | Ruixin. Wang, ruixin.wang@vivo.com |  |
| CAICT | Beijing | Siting Zhu, zhusiting@caict.ac.cn Xuan Yi, yixuan@caict.ac.cn |  |
| OPPO | Dongguan | Qifei Liu, liuqifei@oppo.com Rui Zhou, zhourui2@OPPO.COM |  |
| Sporton USA | Milpitas, CA | Will Ni (WillNi@sporton-usa.com) Alex Ho (Alexander@sporton.com.tw) |  |
| Element Materials Technologies | Columbia, MD | Edward Ortiz (edward.ortiz@element.com) |  |
| SGS Wireless | Shenzhen | Panta Sun (Panta.Sun@sgs.com) Kenny Lai (Kenny.Lai@sgs.com) |  |
| Huawei | Dongguan | Li Jinxing, lijinxing3@huawei.com, Hai Zhou, hai.zhou1@huawei.com |  |
| SRTC | Beijing | Gong Jian, gongjian1@srtc.org.cn |  |
| Samsung | Suwon, KR | Bozhi Li, bozhi.li@samsung.com Jeongheum Lee, jh0413.lee @samsung.com |  |

Issue 4-1-3: latest status of Lab Alignment Devices (LADs) for 2Tx lab alignment activity

Agreements:

  - Confirm the Rel-19 2Tx Lab Alignment Devices (LADs) information in table below.

| Rel-19 2Tx Lab Alignment Devices (LADs) for non-coherent UL MIMO |  |  |  |
| --- | --- | --- | --- |
| DUT | Provider/Owner | Contact | Note |
| LAD5 (new device for Rel-19 2Tx) | vivo | Ruixin Wang ruixin. wang@vivo.com | For n41 and n78 testing. Available. |
| LAD6 (new device for Rel-19 2Tx) | OPPO | Qifei Liu liuqifei@OPPO.com | For n41 and n78 testing. Not yet, will be available in Dec. |
| Note: To keep the test burden, two devices per band. |  |  |  |

Issue 4-1-4: 2Tx related MU impacts

WF

  - FFS

Issue 4-1-5: 2Tx requirements related performance measurement campaign

Agreements:

  - Confirm browsing mode only for 2Tx requirements in Rel-19.

Sub-topic 4-2 1Tx FR1 TRP TRS requirements

Issue 4-2-1: How to specify PDA hand related TRP/TRS requirements (assume no large scale measurement campaign, current requirement is a basis)

Agreements:

  - RAN4 further discuss how to specify requirements for Size 2 (narrow) UE.

  - Further align the understanding on target of preliminary measurement campaign.

    - Option 1: for defining the offset value, on top of size 1 requirements

    - Option 2: for collecting data to generate CDF and define requirements based on CDF ( this is traditional measurement campaign, the difference is only less number of devices required)

Issue 4-2-4: Scope for 1Tx requirements in Rel-19

Agreements:

  - The number of band and which band will be decided in next RAN plenary meeting.

  - Feedback from interested operators is expected before RAN plenary meeting.

Issue 4-2-5: Working procedure for 1Tx requirements related performance measurement campaign

Agreements:

  - On top of agreed working procedure in WF R4-2416590, the following aspect is added:

    - For new labs of 1Tx requirements work, those not declared aligned in Rel-17 and Rel-18 also need to complete 1Tx lab alignment on at least n78 (high band) and n28 (low band).

  - Further discuss on the preferrable measurement grid methods to be used for the performance measurement campaign.

Issue 4-2-6: For RC low band harmonization issue in Rel-19

Agreements:

  - Update WID in next RAN-P to add RC harmonization scope. RAN4 further discuss how to resolve the low band RC harmonization issue.

  - Consider the following solutions:

    - Increase the number of sample points per TRS measurement.

    - Provide a separate uplink path in the chamber for TRS.

    - Define a minimum system dwell time at each position for device adaptation.

Issue 4-2-7: How to specify 1Tx requirements in Rel-19

Agreements:

  - Further discuss whether a proper methodology (e.g., threshold) is needed to identify and exclude outlier results of TRP TRS data pool.

Agreements for dynamic MIMO OTA topic:

Sub-topic 1-1 Dynamic channel model emulation and validation

Issue 1-1-1: BS beamforming configuration

Agreement:

- In this release RAN4 focus on current BS configurations in TR38.762 v0.1.0.

Sub-topic 1-3 Dynamic MIMO OTA test and UE performance metric

Issue 1-3: Dynamic MIMO OTA test and UE performance metric

Agreement:

- For discussing performance metric perspective, further discuss whether the measurement results of UMi and UMa should be combined.

- Down-selection of a single channel model for performance requirements is not precluded.

Sub-topic 2-1 General issues on Rel-19 MIMO OTA requirements

Issue 2-1-1: Target bands for Rel-19 MIMO OTA requirements

Agreement:

- Single band is needed to replace n79. Which band is FFS.

- Companies are encouraged to share views on how to down-select within n7, n38, and n20. Consider the consistency with Rel-19 TRP TRS requirements work.

Issue 2-1-2: 4x4 requirement or 2x2 requirement for band n3

Agreement:

- specify 4x4 for band n3 in Rel-19.

Issue 2-1-3: Device type for MIMO OTA measurement campaign

Agreement:

- Rel-19 MIMO OTA TRMS measurement campaign is targeted on smart phone without power class restriction.

Issue 2-1-4: Minimum number of submitted devices from each volunteer lab

Agreement:

- Keep the minimum number as 3 to avoid any data disclosure, and encourage volunteer labs to provide as much data as possible within 15.

Sub-topic 2-2 Framework for Rel-19 MIMO OTA requirements

Issue 2-2: Framework for Rel-19 MIMO OTA requirements

Agreement:

- The Framework in R4-2419813 is approved with the understanding that further refinement can be made.

2.4.2	Remaining Open issues

- Core part

    - Further discuss XR OTA test method and UE performance metric

    - Further discuss NTN OTA test method and UE performance metric

    - Further discuss FR1 dynamic MIMO OTA test method and UE performance metric

    - TR for FR1 dynamic MIMO OTA

- Performance part

    - 2Tx lab alignment activity for TRP TRS

      - Framework and requirements for 2Tx TRP TRS

    - Measurement campaign for 1Tx TRP TRS and static MIMO OTA

    - Final Requirements and recommended TT for 1Tx TRP TRS and static MIMO OTA

2.5	RAN5

2.5.1	Agreements

RAN5 #105

- No discussion this meeting

2.4.2	Remaining Open issues

- Core part

  - Detailed RAN5 Workplan for Rel-19 OTA MU assessment

  - XR OTA test methods related MU assessment

  - FR1 NTN OTA (NR-NTN and IoT-NTN) test methods related MU assessment

  - FR1 dynamic MIMO OTA related MU assessment

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

4.	References

RAN4 #112bis

- R4-2415056	Discussion on OTA test for FR1 NTN devices	Xiaomi

- R4-2415267	on MIMO dynamic channel tests	Huawei, HiSilicon

- R4-2415268	on NTN OTA test	Huawei, HiSilicon

- R4-2415269	on XR OTA test	Huawei, HiSilicon

- R4-2415277	Discussion on OTA Test for XR Devices	Meta

- R4-2415309	On FR1 NTN devices OTA	Qualcomm Incorporated

- R4-2415310	On FR1 dynamic MIMO OTA test method	Qualcomm Incorporated

- R4-2415320	[draft] LS on XR positioning guidelines and MU for head phantom OTA testing	Apple

- R4-2415321	Discussion on XR test method and Configurations	Apple

- R4-2415340	Inputs to Rel-19 TRP TRS Lab Alignment and Performance Campaign	Apple

- R4-2415560	On XR UE OTA test	Samsung

- R4-2415561	On NTN UE OTA test	Samsung

- R4-2415562	On HARQ parameter for FR1 dynamic MIMO OTA	Samsung

- R4-2415563	On performance of Rel-19 OTA WI	Samsung

- R4-2415769	Discussions and draft reply LS on 3GPP LTE TRP TRS test method for phone >72mm	vivo

- R4-2415770	Draft CR on adding test method for UE over 72mm wide	vivo

- R4-2415775	Reply LS to CTIA on XR types for phantom development	vivo

- R4-2415776	Further discussions on IoT-NTN and NR-NTN OTA test method	vivo

- R4-2415777	Working procedure for Rel-19 2Tx lab alignment and UE performance measurement campaign	vivo

- R4-2416063	Discusion on XR OTA test method	CAICT

- R4-2416064	Discussion on TRP TRS requirements	CAICT

- R4-2416165	Reference values for FR1 dynamic channel models validation	CMCC

- R4-2416210	Test methodology for FR1 non-RedCap headworn XR devices	Nokia

- R4-2416211	Rel-19 FR1 OTA requirements	Nokia

- R4-2416212	Reply to LS R4-2414918	Nokia

- R4-2416376	Consideration of test time reduction for XR device	OPPO

- R4-2416377	Further discussion of NTN OTA performance metrics	OPPO

- R4-2416392	Operator views on Usage Modes for NTN Smartphones	Inmarsat, Viasat

- R4-2416432	Framework for Rel-19 MIMO OTA requirements (Oct 2024)	CAICT

- R4-2416433	TR 38.762 V0.1.0 on MIMO OTA dynamic test methodology for FR1 UEs	CAICT

- R4-2416478	TP on Dynamic MIMO Aspects	Keysight Technologies UK Ltd, Spirent Communications

- R4-2416479	Preliminary Results from Dynamic MIMO OTA Measurement Campaign	Keysight Technologies UK Ltd, CAICT

- R4-2416480	On NTN OTA Testing	Keysight Technologies UK Ltd

- R4-2416544	Topic summary for [112bis][323] TRP_TRS_Ph3	Moderator (vivo)

- R4-2416545	Topic summary for [112bis][324] MIMO_OTA_Ph3	Moderator (CAICT)

- R4-2416575	Reply LS to CTIA on XR device type for phantom development	vivo

- R4-2416576	Way Forward for [112bis][323] TRP_TRS_Ph3	Vivo

- R4-2416577	Ad-hoc meeting minutes for [112bis][323] TRP_TRS_Ph3	Vivo

- R4-2416578	TP on Dynamic MIMO Aspects	Keysight Technologies UK Ltd, Spirent Communications

- R4-2416579	Way Forward for [112bis][324] MIMO_OTA_Ph3	CAICT

- R4-2416590	Way Forward for [112bis][323] TRP_TRS_Ph3	Vivo

RAN4 #113

- R4-2417589	on TRP TRS OTA tests	Huawei, HiSilicon

- R4-2417590	on XR OTA tests	Huawei, HiSilicon

- R4-2417591	on NTN OTA tests	Huawei, HiSilicon

- R4-2417592	on MIMO OTA tests	Huawei, HiSilicon

- R4-2417756	Discussion on XR devices TRP and TRS test time reduction	Apple

- R4-2417757	On NTN TRP TRS test methodology	Apple

- R4-2417758	On Rel-19 TRP TRS performance aspects	Apple

- R4-2417940	Discussion on OTA test for FR1 NTN devices	Xiaomi

- R4-2418027	Discussion on XR test method	Ericsson-LG Co., LTD

- R4-2418206	Reply LS to ETSI on 3GPP LTE TRP TRS test method for phone >72mm	vivo

- R4-2418210	Discussions on NTN OTA test method	vivo

- R4-2418211	Draft Rel-19 CR for 38.870 on general part for XR and NTN OTA	vivo

- R4-2418212	Draft Rel-19 CR for 38.870 on XR OTA alternative test method	vivo

- R4-2418213	Discussions on Rel-19 TRP TRS requirements work	vivo

- R4-2418214	(Template)Measurement results for Rel-19 TRP TRS 2Tx AC lab alignment activity	vivo

- R4-2418340	On OTA performance metric for handheld NTN UE	Samsung

- R4-2418341	On performance work of Rel-19 TRP TRS	Samsung

- R4-2418342	Discussion on static MIMO OTA in Rel-19	Samsung

- R4-2418745	Reference target values for FR1 UMa and UMi dynamic channel models	CMCC

- R4-2418797	Discussion on OTA Test for XR Devices	Meta

- R4-2418826	OTA Conformance Testing with Circular Polarisation	Inmarsat

- R4-2418850	Views on FR1 NTN devices OTA	Qualcomm Incorporated

- R4-2418851	Discussion on FR1 dynamic MIMO OTA test methodology	Qualcomm Incorporated

- R4-2418880	Framework for Rel-19 MIMO OTA requirements (Nov 2024)	CAICT

- R4-2418881	TR 38.762 V0.2.0 on MIMO OTA dynamic test methodology for FR1 UEs	CAICT

- R4-2418976	Discussion on circular polarization for NTN OTA test	OPPO

- R4-2418977	Discussion on target value of dynamic path gain for UMi route	OPPO

- R4-2419247	On Performance metric fot IoT NTN devices	CAICT

- R4-2419300	Discussion on TRP TRS requirements 	CAICT, SAICT

- R4-2419396	On Circular Polarization for FR1 NTN OTA Conformance Testing	Keysight Technologies UK Ltd, Samsung, Nokia, Verizon Wireless, AT&T, Mediatek, ZTE, Ericsson, ETS-Lindgren, Rohde&Schwarz, Apple, OPPO, Xiaomi, CAICT

- R4-2419397	On Dynamic MIMO OTA Testing	Keysight Technologies UK Ltd, Spirent Communications

- R4-2419612	Topic summary for [113][325] TRP_TRS_Ph3	Moderator (vivo)

- R4-2419613	Topic summary for [113][326] MIMO_OTA_Ph3	Moderator (CAICT)

- R4-2419760	Proposals for Rel-19 TRP TRS requirements topic	Telecom Italia S.p.A., Vodafone, Deutsche Telekom, Orange

- R4-2419803	Draft Rel-19 CR for 38.870 on general part for XR and NTN OTA	vivo

- R4-2419806	Draft Rel-19 CR for 38.870 on XR OTA alternative test method	vivo

- R4-2419807	Ad-hoc meeting minutes for [113][325] TRP_TRS_Ph3	Vivo

- R4-2419808	Way Forward for [113][325] TRP_TRS_Ph3	Vivo

- R4-2419811	On Dynamic MIMO OTA Testing	Keysight Technologies UK Ltd, Spirent Communications

- R4-2419812	Reference target values for FR1 UMa and UMi dynamic channel models	CMCC

- R4-2419813	Framework for Rel-19 MIMO OTA requirements (Nov 2024)	CAICT

- R4-2419814	Way Forward for [113][326] MIMO_OTA_Ph3	CAICT

- R4-2419910	Way Forward for [113][326] MIMO_OTA_Ph3	CAICT

RAN5 #105

N/A

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template