3GPP TSG RAN Meeting #106                                                       RP-242634

Madrid, Spain, December 9-12, 2024

Source:	vivo

Title:	Discussion on IMT-2030 technical performance requirements (TPR)

Agenda Item:	16

Document for:	Discussion and Decision

Introduction

In RAN#103 [1], the following was agreed regarding RAN plenary work toward 6G:

RAN plenary work is split into an ITU focused SI (study item) and General RAN SI

- IMT-2030 discussion is expected in RAN from 09/24 to 12/24

- RP SI Rel-20 focusing on ITU– IMT-2030:  approval 12/24 until 06/25

- RP SI Rel-20 focusing on 6G General: approval 03/25 (after WS), until 06/26

In this contribution, we summarize the status and progress of technical performance requirements (TPR) discussion in ITU-R WP5D and express our views on the TPR.

Current status of IMT-2030 TPRs in ITU-R WP5D

The technical performance requirements (TPR) for IMT-2030 are currently under discussion in ITU-R WP5D SWG radio aspect. According to the ITU-R WP5D timeline, as shown in Figure 1, TPRs have been discussed in the past three meetings, 41 contributions from different countries, alliances and companies are submitted for discussion and a working document towards a preliminary draft new report ITU-R M.[IMT-2030. TECH PERF REQ] [2] is being developed.

For each TPR, the definition, the requirement values and the applicable usage scenarios are to be studied and determined. Based on the input contributions, the working document and the online/offline discussion in the ITU-R WP5D meetings, the following observations on the progress of each TPR are given:

1) For some traditional TPRs, such as peak data rate, peak spectrum efficiency, the definition and applicable usage scenarios are clear and close to agreement, but the requirement values need more study.

2) For some other traditional TPRs, such as user experienced data rate, connection density, average spectrum efficiency etc., their definitions are clear and close to agreement, but their applicable usage scenarios are still under discussion.

3) New TPRs associated with new capabilities, such as sensing-related capability, AI-related capability and sustainability, their definition are still under discussion, including whether these TPRs will be defined as qualitative metric or quantitative metric.

4) New TPRs associated with joint capabilities, such as joint requirement on data rate, latency, and reliability, sustainable data rate, are proposed in the input contributions and their definition are still under discussion.

Figure 1 ITU-R WP5D timeline

In the following Table 1, we summarize the discussion status for each TPR.

Table 1  TPR discussion status in ITU-R WP5D

|  | TPR | Definition | Requirement values | Requirements recommendation example in M.2160 | Usage scenarios |
| --- | --- | --- | --- | --- | --- |
| 1 | Peak data rate | Clear | TBD | 50, 100, 200 Gbit/s | Immersive Communication |
| 2 | User experienced data rate | Clear | TBD | 300 Mbit/s, 500 Mbit/s | TBD [Immersive Communication and/or Ubiquitous Connectivity] |
| 3.1 | Peak Spectrum Efficiency | Clear | TBD | TBD | Immersive communication |
| 3.2 | Average spectrum efficiency | Clear | TBD | 1.5 and 3 times greater than that of IMT-2020 | TBD [Immersive Communication and/or Ubiquitous Connectivity] |
| 3.3 | 5th percentile user spectrum efficiency | Clear | TBD | TBD | TBD [Immersive Communication and/or Ubiquitous Connectivity] |
| 4 | Area traffic capacity | Clear | TBD | 30 Mbit/s/m2, 50 Mbit/s/m2 | Immersive Communication |
| 5 | Connection Density | Clear | TBD | 106 – 108 devices/km2 | TBD [Massive Communication and/or Immersive Communication] |
| 6 | Mobility | TBD | TBD | 500 – 1 000 km/h | TBD [Immersive Communication and/or Massive Communication and/or Ubiquitous Connectivity] |
| 6.1 | Mobility interruption time | Clear | TBD | TBD | TBD [Immersive Communication and/or Hyper-Reliable and Low Latency Communication and/or Massive Communication] |
| 7.1 | User plane latency | TBD | TBD | 0.1 – 1 ms | Immersive Communication and Hyper-Reliable and Low Latency Communication |
| 7.2 | Control plane latency | TBD | TBD | TBD | Immersive Communication and Hyper-Reliable and Low Latency Communication |
| 8 | Reliability | TBD [Joint requirement on data rate, latency and reliability] | TBD | 1-10−5 to 1-10−7 | TBD [Hyper-Reliable and Low Latency Communication and/or Immersive Communication] |
| 9 | Coverage | TBD | TBD | TBD | TBD [Ubiquitous Connectivity and/or General] |
| 10 | Positioning | TBD | TBD | 1 – 10 cm | TBD [Immersive Communication and/or Hyper-Reliable and Low Latency Communication and/or Integrated Sensing and Communication] |
| 11 | Bandwidth | Clear | TBD | TBD | TBD [General] |
| 12 | Sensing-related capabilities | TBD [Detectability (Detection / False Alarm Probability) Localization Accuracy Velocity Accuracy Sensing Resolution] | TBD | TBD | Integrated Sensing and Communication |
| 13 | AI-related capabilities | TBD | TBD | TBD | Artificial Intelligence and Communication |
| 14 | Sustainability/ Energy efficiency | TBD | TBD | TBD | TBD [Immersive Communication and/or Massive Communication and/or Ubiquitous Connectivity and/or Artificial Intelligence and Communication and/or Integrated Sensing and Communication] |
| 15 | Security and resilience | TBD | TBD | TBD | TBD |
| 16 | Interoperability | TBD | TBD | TBD | TBD |
|  | Sustainable data rate | TBD | TBD | TBD | Immersive Communication |
|  | XR area capacity | TBD | TBD | TBD | Immersive Communication |
|  | XR area efficiency | TBD | TBD | TBD | Immersive Communication |
|  | Joint requirement on data rate, latency, and reliability | TBD | TBD | TBD | Immersive Communication and Hyper-Reliable and Low Latency Communication |
|  | Joint requirement of data rate, latency, reliability and capacity | TBD | TBD | TBD | Immersive Communication |

Observation 1: For some traditional TPRs, the definitions and applicable usage scenarios are clear, and requirements value(s) needs further study.

Observation 2: For some other traditional TPRs or new TPRs, the definition and applicable usage scenarios as well as requirements value(s) need further study.

According to the workplan for development of the Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ] [3] of ITU-R WP5D, there are still 4 meetings (#48 to #51) to complete the report, the main actions in each meeting are summarized in Table 2.

Table 2  ITU-R WP5D work plan for TPRs

| Meeting number | Time plan | Main workplan |
| --- | --- | --- |
| #48 | 2025-02-04 to 2025-02-13 | - Preliminarily agree on items for technical performance requirements. |
| #49 | 2025-06-24 to 2025-07-03 | - Continue to discuss the detailed definition of each technical performance requirement. - Start to discuss target value(s) for technical performance requirements. |
| #50 | October 2025 | - Agree on the items for technical performance requirements. - Discuss and preliminarily agree on the detailed definition and the target value(s) for technical performance requirements. |
| #51 | February 2026 | - Finalize a document as Draft New Report and send it to ITU-R SG5 for consideration. |

In additional, in ITU-R WP5D #47 meeting, a “Draft liaison statement to External Organizations – Minimum requirements related to technical performance for IMT-2030 radio interface(s)” [4] was discussed and agreed on the Technology Sub-Working Group (SWG) Radio Aspects.

In this draft LS,

Working Party 5D kindly invites External Organizations to provide their inputs on the minimum technical performance requirements for IMT-2030 radio interface technologies, including but not limited to,

–	Proposed candidate items for minimum technical performance requirements based on M.2160 Usage Scenarios and Capabilities, including necessary background information and justification, which are requested to be provided preferably by WP 5D #48 (February 2025 TBC).

–	Proposed associated target values for the above candidate items for the minimum technical performance requirements which are requested to be provided preferably by WP 5D #49 (June/July 2025 TBC).

RAN should study ITU IMT-2030 technical performance requirements (TPR), including the TPR definitions, applicable usage scenarios and requirements value(s).

Another aspect related to TPR is evaluation. For some TPRs, the requirements values should be defined in some expected deployment scenarios, and for some other TPRs, the requirements values are different for different deployment scenarios. According to the ITU-R WP5D timeline, as shown in Figure 1, the discussion on evaluation of radio interface technologies for IMT-2030 have been started and a working document towards a preliminary draft new report ITU-R M.[ IMT-2030.EVAL] [5] is being developed.

In ITU-R WP5D #47 meeting, the test environments for the six usage scenarios were discussed, which are related to the deployment scenarios in 3GPP. Among them three usage scenarios, including Immersive Communication, Hyper Reliable & Low-latency Communication and Massive Communication, according to the meeting discussion, the test environments in IMT-2020 can be taken as a starting point. For those usage scenarios, new test environments are also proposed in some input contributions. Other three new usage scenarios, including Ubiquitous Connectivity, Artificial Intelligence & Communication and Integrated Sensing & Communication, new test environments are also proposed in contributions. In the following Table 3, we summarize the status of discussion for test environments of each usage scenarios. For the test environments, the name, the definition, the detailed configurations, as well as whether some of test environment(s) is/are needed or not are to be studied and determined.

Table 3  Test  environments discussion status in ITU-R WP5D

| Usage scenarios | Proposed test environments |
| --- | --- |
| Immersive Communication | Indoor Hotspot-IC Dense Urban-IC Rural-IC |
| Hyper-Reliable and Low Latency Communication | Urban Macro–HRLLC Indoor [Factory]–HRLLC |
| Massive Communication | Urban Macro–MC Indoor Factory–MC |
| Ubiquitous Connectivity | Rural-UC |
| Artificial Intelligence and Communication | Indoor − Indoor-ISAC − Indoor Factory–ISAC − Indoor Hotspot–ISAC Outdoor − Outdoor-ISAC − Dense Urban–ISAC − Rural–ISAC − Urban macro – ISAC |
| Integrated Sensing and Communication | Dense Urban–AIAC |

Observation 3: For the test environment(s) of each usage scenario, the definition and detailed configurations need further study.

RAN should study IMT-2030 deployment scenarios, including the definitions, detailed configurations (e.g. layout, carrier frequency, bandwidth, and etc.).

Initial views on IMT-2030 TPR

In the following sections, our initial views on each IMT-2030 TPR are given.

Peak data rate

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Regarding the requirement value(s), in IMT-2020, the peak data rate is evaluated with the assumption that the maximum available bandwidth is aggregated by 16 component carrier (CC) without considering practical frequency bandwidth situations. According to the evaluation results of 3GPP [7], for FR1, 500~900 MHz (i.e. 5~9 CC) with different frame structure can fulfill the requirements, for FR2, 800~1000 MHz (i.e. 2~6 CC) with different frame structure can fulfill the requirements. Actually, in practical deployments, even though less than 16 CC, availability of these bandwidths are also quite challenging. But considering peak data rate is the highest theoretical data rate and the comparative relationship between IMT-2020 and IMT-2030, it’s better to maintain the same definition and same evaluation methodology for peak data rate. According to the discussion in section 3.3.1, the peak spectral efficiency of IMT-2030 is assumed to be 2.0 times of IMT-2020, and assuming the bandwidth for one CC is 1.0~2.0 times of IMT-2020 (i.e. 100/200 MHz per CC), we propose the minimum requirements for peak data rate is 2.0~4.0 times of IMT-2020, i.e., 40~80 Gbps for downlink and 20~40 Gbps for uplink.

User experienced data rate

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Regarding the requirement value(s), the user experience data rate is given by Ruser = W × SEuser. In IMT-2020, the user experience data rate is evaluated with the assumption that the maximum available bandwidth is aggregated by 16 component carrier (CC) without considering practical available frequency bandwidth situations. Referring the evaluation results of 3GPP [7], for FR1, 200~800 MHz (i.e. 2~8 CC) can fulfill the requirements. According to the discussion in section 3.3.3, the downlink 5th percentile user spectral efficiency of IMT-2030 is assumed to be 2.0 times of IMT-2020 and 1.5~2.0 times for uplink 5th percentile user spectral efficiency, and assuming the bandwidth for one CC is 1.0~2.0 times of IMT-2020 (i.e. 100/200 MHz per CC), we propose the minimum requirements for downlink user experience data rate is 2.0~4.0 times of IMT-2020 and for uplink user experience data rate is 1.5~4.0 times of IMT-2020, i.e., 200~400 Mbit/s for downlink and 75~200 Mbit/s for uplink.

Spectrum efficiency

Peak spectrum efficiency

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Regarding the requirement value(s), in IMT-2020, refer to the ITU M.2410 [6], the requirement values were defined assuming an antenna configuration to enable eight spatial layers (streams) in the downlink and four spatial layers (streams) in the uplink. For IMT-2030, considering the possible and practical antenna configuration for a mobile station, assume an antenna configuration to enable eight spatial layers (streams) in both downlink and uplink. Furthermore, higher modulation (e.g. 4096 QAM) for downlink is possible, which have been considered in WiFi technology. Higher coding rate is also possible depending on the development of channel coding scheme. In addition, lower overhead (i.e., radio resources that are used for physical layer synchronization, reference signals or pilots, control signals, guard bands and guard times) is expected due to technical advancement in 6G timeframe, such as application of AI/ML in PHY. Hence, we propose the minimum requirements for peak spectrum efficiency is 2.0 times of IMT-2020, i.e., 60 bit/s/Hz for downlink and 30 bit/s/Hz for uplink.

Average spectrum efficiency

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Regarding the requirement value(s), some initial simulation evaluation results with extreme antenna array assumption in BS are provided in the figure 2 below. In this preliminary simulations, ideal PMI measurement are assumed for the transmission port larger than 32. Other potential and possible performance enhancement technologies, such as higher modulation, enhanced channel coding, enhanced CSI-RS/DMRS, enhanced MIMO schemes, AI for CSI enhancement, overhead reduced and so on, are not modeled or assumed.

Figure 2 Initial simulation results of spectral efficiency of Dense Urban

Form the preliminary simulation results, it can be observed that for downlink, the maximum performance gain is 1.4 times for average spectrum efficiency and 2.0 times for 5th percentile user spectrum efficiency. For uplink with 2 transmission antennas in device, the maximum performance gain is 1.1 times for average spectrum efficiency and 1.5 times for 5th percentile user spectrum efficiency, for uplink with 4 transmission antennas in device, the maximum performance gain is 1.5 times for average spectrum efficiency and 1.6 times for 5th percentile user spectrum efficiency. With the potential enhancement of technologies in IMT-2030, the spectral efficiency can be further enhanced.

Hence, we propose the minimum requirements for average spectrum efficiency is 2.0 times of IMT-2020 for downlink and 1.5~2.0 times of IMT-2020 for uplink.

5th percentile user spectrum efficiency

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Regarding the requirement value(s), according to the preliminary simulation results and preliminary analysis in section 3.3.2, we propose the minimum requirements for 5th percentile user spectrum efficiency is 2.0 times of IMT-2020 for downlink and 1.5~2.0 times of IMT-2020 for uplink.

Area traffic capacity

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Regarding the requirement value(s), the area traffic capacity is given by Carea = ρ × W × SEavg. In IMT-2020, the area traffic capacity is evaluated with the assumption that the maximum available bandwidth is aggregated by 16 component carrier (CC) without considering practical available frequency bandwidth situations as the evaluation of user experience data rate and peak data rate. Referring the evaluation results of 3GPP [7], for FR1, 400~800 MHz (i.e. 4~8 CC) can fulfill the requirements, and for FR2, 800~1200 MHz (i.e. 2~4 CC) can fulfill the requirements. According to the discussion in section 3.3.2, the downlink average spectral efficiency of IMT-2030 is assumed to be 2.0 times of IMT-2020, and assuming the bandwidth for one CC is 1.0~2.0 times of IMT-2020 (i.e. 100/200 MHz per CC), we propose the minimum requirements for area traffic capacity is 2.0~4.0 times of IMT-2020, i.e., 20~40 Mbit/s/m2.

Connection Density

TPR definition: Same as in IMT-2020.

Applicable usage scenario:

In IMT-2020, connection density is mainly considered for mMTC usage scenario and the target requirement is developed for the IoT traffic with low data rate and longer latency. For IMT-2030 immersive communication usage scenario, the typical use cases include immersive XR. For XR use cases, system capacity is an important metric for successful commercial deployments, which have been widely discussed and extensively evaluated in the 3GPP XR study item [8]. One way to quantify system capacity for XR use case is to define connection density requirement, e.g., number of supported XR users per km2. As an example, the minimum requirement for connection density is 106~107 devices per km2 for Massive Communication usage scenario and 103 ~104 devices for Immersive Communication usage scenario.

The connection density should be applicable to both Massive Communication and Immersive Communication usage scenarios.

Mobility

In IMT-2020, mobility TPR is defined as the supported maximum mobile station speed (in km/h) at which a defined QoS can be achieved, which are related to extreme high-speed scenarios such as high speed trains. The defined QoS is normalize traffic channel link data rate (bit/s/Hz). However, although the user experience is good in most scenarios, there are still poor user experience in some scenarios although the device moving speed is not high, which are referred to as pain point scenarios.

The wireless signal quality changes rapidly in certain pain point areas due to factors such as shadow fading within a short period of time, for example entering or exiting of street corner, passing under a driveway bridge, entering or exiting of building, being in an underground parking lot, or using an elevator etc., which are frequent occurrent and very common scenarios in daily life. Take the shadow fading scenarios such as passing under a driveway bridge or using an elevator for example, the rapid change of wireless signal quality will have a certain impact on the quality of user experience. The wireless signal is easily blocked or reflected by obstacles in these environments, resulting in a rapidly weakening of the signal strength and a deterioration of the signal-to-noise ratio, which may cause call dropping, low data transmission rate, and unstable connections.

Another pain point scenario is the high-speed railway station scenario, where the UE’s mobile states change rapidly. The network's failure to promptly recognize the UE state changes could lead to a decline in the UE's communication service experience and interruptions. For example, after the UE enters the station and gets on the train, it is in a low-speed state and camps on the public network instead of the High-Speed Dedicated Network (HSDN), communication failure may occur when the train begins to accelerate rapidly.

Observation 4: Pain point scenarios are frequent occurrent and common scenarios in daily life.

Observation 5: The rapid change of wireless signal quality in pain point scenarios have a certain impact on the quality of user experience.

Currently, there are handover procedures to improve the performance of UE’s mobility. Measurement-based handover procedure is popularly applied to support UE’s mobility. The network sends a handover command to the UE according the measurement report sent by the UE. However, when the UE is suddenly in the pain point areas, the UE may fail to send the measurement report to the network or receive the handover command from the network due to the rapidly-deteriorating radio link, which will cause handover failure. Conditional handover is introduced to enhance the mobility robustness. The network configures some candidate cells to the UE in advance. The UE performs the measurements and handover to a candidate cell once its execution condition is fulfilled. In this use case, when configuring suitable candidate cells to the UE, the network is required to identify that the UE is approaching the pain point area otherwise too much candidate resources are reserved. Furthermore, the conditional handover is not supported in inter-RAT handover. If the underground parking is covered with LTE cells, the conditional handover cannot cover this scenario.

It is observed from the above analysis, even though the current handover procedure may work well and can resolve most of the problems, but cannot fully resolve the issues in pain point scenarios. Considering the frequency of pain point scenarios occurrence and the poor user experience in pain point scenarios, it’s necessary to identify these scenarios and to evaluate the performance of these scenarios and finally to pursue solutions to fundamentally resolve the problem in pain point scenarios to enhance user experience.

Observation 6: The current handover procedure(s) cannot fully resolve the issues in pain point scenarios.

In IMT-2020, the performance metric for mobility is data rate, where the techniques are developed to overcome the impact of high mobility, such as enhanced DMRS pattern design. Considering the impact of handover on data transmission, data rate can also be the performance metric for pain point scenarios. Additionally, the traditional mobility metrics, such as handover failure, radio link failure, mobility interruption time can also be the performance metric for pain point scenarios.

Regarding the deployment scenarios, the Urban Grid deployment scenario with buildings modelled around the lanes are used to model the blocking or reflection by obstacles, which is a typical pain point scenario of entering or exiting of street corner, can be used as a starting point for pain point deployment scenarios.

For mobility, identify the pain point deployment scenarios and define the definition and requirement of performance metrics to quantify the user experience in the pain point scenarios.

For pain point scenarios in mobility, the performance metric can be normalized data rate (bit/s/Hz) and mobility interruption time (ms) same as IMT-2020 mobility evaluation, traditional mobility performance matric, e.g. HOF, RLF can also be the performance metrics.

- FFS detailed performance metric requirements.

Urban Grid deployment scenario with building modelled among the lanes can be used as a starting point for the pain point scenarios.

Latency

User plane latency

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication, Hyper Reliable and Low-Latency Communication.

Preliminary requirement value(s): the minimum requirement for user plane latency is 1ms for Immersive Communication and 0.1ms for Hyper Reliable and Low-Latency Communication.

Control plane latency

In IMT-2020, the definition of control plane latency refers to the time for a device to switch from a battery efficient state (e.g., IDLE/INACTIVE) to start of continuous data transfer (e.g., CONNECTED), including the state transition time from battery efficient state to continuous data transfer state. In 5G NR, the small data packets can already be transmitted in battery efficient state (INACTIVE), which means a device may not always have to switch from INACTIVE to CONNECTED for transmission or reception of small packets. The SDT-like feature with high energy efficiency and low signaling demand for state transition can be a promising technology for IMT-2030 as well. However, the control plane latency in such scenarios would need to be clarified/reconsidered.

For example, the control plane latency can be defined as the latency from a “battery efficient” state to the start of data transfer, including the case with the device state transition (e.g., INACTIVE to CONNECTED) and the case without state transition (e.g., small data transmission during INACTIVE).

Study control plane latency definition to include the cases with and without device state transition.

Applicable usage scenario: Immersive Communication, Hyper Reliable and Low-Latency Communication.

Preliminary requirement value(s): the minimum requirement for control plane latency is 10ms with device state transition and 4ms without device state transition.

Reliability

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Hyper Reliable and Low-Latency Communication.

Preliminary requirement value(s): the minimum requirement for reliability is 1-10-5.

Bandwidth

Preliminary requirement value(s): the maximum bandwidth for single radio frequency (RF) carrier can be 400Mhz, and the maximum aggregated system bandwidth is TBD, which is relevant to the requirement of peak data rate.

Sensing-related TPRs

In the outcome of SA1 study item in TR 22.837, sensing service KPIs for different use cases are captured, such as accuracy of positioning, accuracy of velocity, and so on. These KPIs can be used as starting point for discussion on the IMT-2030 sensing-related TPRs. It should be noted that there are significant differences in the performance requirements among different sensing use cases.

In our opinion, considering a limited number of quantitative sensing TPRs, the basic sensing measurement metrics, such as range, doppler/velocity and angle estimation accuracy which can characterize sensing performance can be considered as the sensing-related TPRs. For the positioning accuracy in TR22.837, since the target position can be calculated through one or both of the range and angle, it is possible to consider evaluating range accuracy and angle accuracy instead of evaluating positioning accuracy.

Hence, we consider the following sensing-related TPRs:

- Range accuracy: the difference between the measured range of and the true range of the target object.

For monostatic sensing, the range can refer to the distance between the target and the sensing receivers. For bistatic sensing, the range can refer to the distance between the target and the sensing receivers, or the sum of the distance between the target and the sensing transmitters and receivers, or the sum of the distance between the target and the sensing transmitters and receivers minus the distance between sensing transmitters and receivers.

- Angle accuracy: the difference between the measured angle and the true angle of the target object.

Angle includes azimuth angle and/or zenith angle. Due to the different antenna configurations in vertical and horizontal, the angle accuracy of azimuth angle and zenith angle could be different. For monostatic sensing, the angle can refer to the arrival angle between the target and the sensing receivers. For bistatic sensing, the angle can refer to the arrival angle between the target and the sensing receivers, or the departure angle between the target and the sensing transmitters.

- Localization accuracy: the difference between the measured position and the true position of the target object.

Regarding the localization accuracy, since position can be calculated by one or both of the range and angle, which are the basic sensing measurement metric, so the localization accuracy can be replaced by range accuracy and angle accuracy, that is to say, localization accuracy can be a joint metric of range accuracy and angle accuracy. For range accuracy and angle accuracy, it’s more suitable for the sensing use cases where only one of the range and angle is needed to be measured. One point to be noted, for independent range accuracy and angle accuracy, two independent performance requirements are needed and whether these two requirements should be satisfied simultaneously for some technical evaluation need further study and clarify.

- Velocity accuracy: the difference between the measured velocity and the true velocity of the target object.

Velocity can refer to the radial velocity. For monostatic sensing, the velocity can refer to the projection of mobility velocity on the line between the target and the sensing receivers. For bistatic sensing, the velocity can refer to the projection of mobility velocity on the bistatic angular bisector, or the sum of the projection of mobility velocity on the line between the target and the sensing receivers and the projection of mobility velocity on the line between the target and the sensing transmitters.

In addition, velocity not only refers to the velocity in linear motion, but also refers to the velocity of periodic reciprocating motion, such as breathing, heartbeat, propeller rotation, repetitive movements (e.g. bird wing flapping, human arm/leg vibration) and so on.

- Detection/Recognition accuracy:

- Detection accuracy: the probability of correctly detecting a target.

Detection probability is used in binary detection scenarios to indicate the probability of correctly judging the presence of a target when it is actually present.

- Recognition accuracy: the probability of correctly identifying the state or category of the target.

Recognition probability is used in multivariate detection scenarios to indicate the probability that a target is in a specific state or belongs to a specific category, and that the target state or category is correctly detected.

Study sensing-related TPRs including range accuracy, angle accuracy, velocity accuracy and detection/recognition accuracy.

AI-related TPR

This TPR is related to Artificial Intelligence and Communication usage scenario. AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 network to support AI services.

The following qualitative requirements from ITU-R M.2160 “Framework and overall objectives of the future development of IMT for 2030 and beyond” can be considered as a starting point for AI-related TPR:

“Besides communication aspects, this usage scenario is expected to include a set of new capabilities related to the integration of AI and compute functionalities into IMT-2030, including data acquisition, preparation and processing from different sources, distributed AI model training, model sharing and distributed inference across IMT systems, and computing resource orchestration and chaining.”

Sustainability

Energy efficiency is a quantifiable metric of sustainability and is important for the success of IMT-2030 technology. The energy efficiency TPR should be defined to ensure that IMT-2030 is designed in an energy efficient way.

In IMT-2020, energy efficiency was evaluated through sleep ratio and sleep duration under unloaded case and demonstrated through high spectrum efficiency under loaded case. However, through the study of multiple energy saving projects in 3GPP release 16/17/18, it can be seen that the power consumptions of device in different power states are different, such as in micro sleep state, the relative power consumption is 45 unit, in PDCCH-only state, the relative power consumption is 100 unit, and in PDCCH+PDSCH state, the relative power consumption is 300 unit.

Under the unloaded case, even with the same configuration of discontinuous reception (DRX), due to the different power consumption in different sleep states and transition power from different sleep states to non-sleep states, the average energy consumption of the device is also different. Therefore, using the sleep ratio and sleep duration cannot fully characterize the energy consumption of the device. In addition, in unloaded case, the latency for paging/control signaling delivery may also be important for some services and device types. So, in unloaded case, the energy efficiency could be defined taking both the impact on paging/control signaling latency and energy consumption into account.

Under the loaded case, due to the traffic characteristics, it can be divided into sleep duration and data transmission duration. Due to the power consumptions of device in different power states are different, technics for decreasing the power consumption can be studied meanwhile minimizing the impact on the latency of data transmission. Therefore, in loaded case, the energy efficiency could be defined by taking both the data rate and power consumption into account, such as EE=VP  or EE=PV ,  where P is the power consumption, v is the user perceived throughput (UPT), which is widely used in 3GPP evaluations in all SI/WIs and reflects the user's experience data rate. P=EC/T, where EC is the energy consumption during an evaluation time T, EC= Pon ×Ton + Psleep ×Tsleep+ Ptransition ×Ttransition .

Although in the above discussion, device side energy efficiency is assumed, the network side energy efficiency is also very important. The definition of network side energy efficiency shall also be carefully studied, common definition between device and network side energy efficiency is preferable if possible.

Observation 7: For energy efficiency, the sleep ratio and sleep duration used in IMT-2020 evaluation cannot accurately characterize the energy efficiency of device or network.

The definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as user experienced data rate, and latency for control signaling/paging or data.

For loaded case, the energy efficiency can be the ratio of data rate and power consumption.

- -  EE=VP  or EE=PV , where P is the power consumption, v is the user perceived throughput (UPT).

Initial views on IMT-2030 deployment scenarios

For the three enhanced usage scenarios, Immersive Communication, Hyper Reliable & Low-latency Communication and Massive Communication, which are envisaged to expand from IMT-2020 (i.e., eMBB, URLLC, and mMTC), the deployment scenarios of IMT-2020 in TR 38.913 [9] can be as staring point with some necessary modification (such as carrier frequency, system bandwidth, antenna configuration and so on) to match the IMT-2030 frequency and system design.

For the urban grid deployment scenarios, additional models can be added based on the discussion in RAN1 study item of Study on channel modelling for ISAC for NR. That is, the environment objects, such as buildings of size 413m x 230m x 20m can be modeled in the scenarios to model the actual blocking or reflecting by obstacles.

Deployment scenarios of IMT-2020 in TR 38.913 can be taken as starting point for IMT-2030 immersive communication, hyper reliable and low-latency communication and massive communication usage scenario.

- Some necessary modification (such as carrier frequency, system bandwidth, antenna configuration and so on) can be considered to match the IMT-2030 frequency and system design.

For urban grid scenarios, environment objects, such as buildings of size 413m x 230m x 20m can be modeled in the scenarios to model the actual blocking or reflecting by obstacles.

For the Integrated Sensing and Communication usage scenarios, the agreed output scenarios of the RAN1 study item of Study on channel modelling for ISAC for NR can be taken as staring point.

Deployment scenarios agreed in RAN1 study item of Study on channel modelling for ISAC for NR can be taken as starting point for IMT-2030 integrated sensing and communication usage scenario.

For Intelligence and communication usage scenario, the deployment scenarios for Immersive Communication can be reused for the intelligence and communication usage scenario.

Conclusions

In summary, we summarize the status and progress of technical performance requirements (TPR) related discussion in ITU-R WP5D and express our views on the TPR and deployment scenarios with following observations and proposals:

Observation 1: For some traditional TPRs, the definitions and applicable usage scenarios are clear, and requirements value(s) needs further study.

Observation 2: For some other traditional TPRs or new TPRs, the definition and applicable usage scenarios as well as requirements value(s) need further study.

Observation 3: For the test environment(s) of each usage scenario, the definition and detailed configurations need further study.

Observation 4: Pain point scenarios are frequent occurrent and common scenarios in daily life.

Observation 5: The rapid change of wireless signal quality in pain point scenarios have a certain impact on the quality of user experience.

Observation 6: The current handover procedure(s) cannot fully resolve the issues in pain point scenarios.

Observation 7: For energy efficiency, the sleep ratio and sleep duration used in IMT-2020 evaluation cannot accurately characterize the energy efficiency of device or network.

- RAN should study ITU IMT-2030 technical performance requirements (TPR), including the TPR definitions, applicable usage scenarios and requirements value(s).

- RAN should study IMT-2030 deployment scenarios, including the definitions, detailed configurations (e.g. layout, carrier frequency, bandwidth, and etc.).

- The connection density should be applicable to both Massive Communication and Immersive Communication usage scenarios.

For mobility, identify the pain point deployment scenarios and define the definition and requirement of performance metrics to quantify the user experience in the pain point scenarios.

For pain point scenarios in mobility, the performance metric can be normalized data rate (bit/s/Hz) and mobility interruption time (ms) same as IMT-2020 mobility evaluation, traditional mobility performance matric, e.g. HOF, RLF can also be the performance metrics.

- FFS detailed performance metric requirements.

Urban Grid deployment scenario with building modelled among the lanes can be used as a starting point for the pain point scenarios.

Study control plane latency definition to include the cases with and without device state transition.

Study sensing-related TPRs including range accuracy, angle accuracy, velocity accuracy and detection/recognition accuracy.

The definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as user experienced data rate, and latency for control signaling/paging or data.

For loaded case, the energy efficiency can be the ratio of data rate and power consumption.

- -  EE=VP  or EE=PV , where P is the power consumption, v is the user perceived throughput (UPT).

Deployment scenarios of IMT-2020 in TR 38.913 can be taken as starting point for IMT-2030 immersive communication, hyper reliable and low-latency communication and massive communication usage scenario.

- Some necessary modification (such as carrier frequency, system bandwidth, antenna configuration and so on) can be considered to match the IMT-2030 frequency and system design.

For urban grid scenarios, environment objects, such as buildings of size 413m x 230m x 20m can be modeled in the scenarios to model the actual blocking or reflecting by obstacles.

Deployment scenarios agreed in RAN1 study item of Study on channel modelling for ISAC for NR can be taken as starting point for IMT-2030 integrated sensing and communication usage scenario.

References

- RP-240823 Additional Considerations for 6G Timeline.

- R23-WP5D-C-0413!H5-N5.07!MSW-E.

- R23-WP5D-241003-TD-0168!!MSW-E.

- R23-WP5D-241003-TD-0167!!MSW-E.

- R23-WP5D-C-0413!H5-N5.02!MSW-E.

- R-REP-M.2410-2017-MSW-E.

- TR 37.910, Study on self evaluation towards IMT-2020 submission.

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR.

- TR 38.912, Study on Scenarios and Requirements for Next Generation Access Technologies.