3GPP TSG RAN Meeting #106									RP-242662

Madrid, Spain, December 9 - 13, 2024

### Agenda Item:		9.1.4

Source:	UIC

Title:				HPUE for CA/DC in bands n100 and n101

Document for:		Discussion and agreement

## Introduction

This contribution discusses the needs of European railways (i.e. 27 national railways represented by the UIC) to enable the use of 5G NR carrier aggregation and dual connectivity (NR CA/DC) with high power UE (HPUE) in their FRMCS networks, using frequency bands n100 and n101.

## Motivation

For the deployment of FRMCS using frequency bands n100 and n101, European railways need to reuse their existing 2G GSM-R basestation sites as that will significantly reduce the RAN deployment costs. A first step to enable this was the specification of PC1 31dBm HPUE in bands n100 and n101 as reflected in TS 38.101-1 table 6.2.1-1.

To increase the available network capacity and, even more importantly, the network service reliability, the current NR_FR1_5MHz_BW_Ph2 WID adds NR CA/DC capability to the bands n100 and n101, however only based on the use of Power Class 3 UE.

In order to benefit from the CA/DC capabilities in uplink, whilst reusing the existing 2G GSM-R basestation sites, railways need to use the CA/DC functionality with HPUE. For this, the following background is provided:

- TS 38.101-1 table 6.2.1-1: UE Power Class, defines PC1 UE in bands n100 and n101

- EU spectrum regulation (ECC Decision (20)02 and EU Implementing Decision 2021/1730) allow simultaneous transmission in bands n100 and n101 with 33 dBm e.i.r.p.

- The railway use case is focussed on FRMCS cab-radio’s, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground. The smartphone form factor is currently not considered for FRMCS HPUE

- The architecture considered for cab radios is to use 1 external power amplifier per frequency band, with a single, combined antenna connector for both frequency bands, or with separate antenna connector for each frequency band

- In addition to NR CA/DC, in future 2x2MIMO or uplink Tx diversity may need to be considered, although shortage of antenna positions on the train roof may significantly limit this from a practical deployment perspective

- In band n101 the dominant TDD frame structure that is expected to be used by FRMCS is as per ECC Report 353. This results in an uplink share of the total frame duration of 43%, thus reducing the average uplink transmit power. For the CA/DC combination of a PC2 HPUE in FDD band n100 plus PC1 HPUE in TDD band n101 this results in a total combo power of 29.7dBm

- In order to avoid potential interference on adjacent frequency bands, the transmit power of a single n101 component carrier within the CA/DC combo should be limited to the PC1 power class limit of 31dBm. It is FFS how this may be accomplished by basestation signalling.

## R19 work request

Based on the motivation given above, European railways (i.e. 27 national railways represented by the UIC) would like to request specifying in 3GPP R19 for NR CA/DC the use of PC2 HPUE in FDD band n100 plus PC1 HPUE in TDD band n101. To accomplish this, we can see two ways forward:

- Extend the scope of the current NR_FR1_5MHz_BW_Ph2 WID

- Define a new R19 WI.

Option a) may be preferred as the anticipated amount of work for this new request is limited. Also, in the original RP-240821 request for support of CA/DC for less than 5MHz CBW that resulted in the current WID, for railways the use of HPUE had already been requested.

Option b) may be preferred if the request is viewed by RAN#106 as being a power class related spectrum topic.

“DDDS1UUDS2UU DDDS1UUDS2UU” with a 30 kHz SCS and S(DL/GP/UL):=(S1 = 6:4:4, S2 = 10:4:0)).