3GPP TSG RAN meeting #106									       	      RP-242766

Madrid, Spain, December 9-12, 2024

## Status Report to TSG

Agenda item:			9.5.1

| WI / SI Name | Work Item on expanded and improved NR positioning |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes | Testing part: No |
| Acronym | NR_pos_enh2, NR_pos_enh2-Perf |  |  |  |
| Unique ID | 981038, 981238 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241186 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2024 | Performance part: 12/2024 | Testing part: N/A |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 100% | Testing part: N/A |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Debdeep Chatterjee |
|  | Company | Intel Corporation |
|  | Email | debdeep.chatterjee@intel.com |
|  | Name | Jianxiang Li |
|  | Company | CATT |
|  | Email | lijianxiang@catt.cn |
|  | Name | Florent Munier |
|  | Company | Ericsson |
|  | Email | florent.munier@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	Decisions during RAN4#112bis

##### 2.4.1.1.1	General aspects

##### 2.4.1.1.2	RF

##### 2.4.1.1.3	RRM performance

The following agreements were made for RedCap positioning (R4-2416869):

Core requirement:

- Update to Nhops,effect,i,j

  - RAN4 to clarify that NrepPRS denotes the number of inter-slot PRS repetitions within a MG occasion, excluding the gap retuning times, which are unmuted and fully or partially overlapped with the sampling duration within each hop.

Performance requirement:

- Group delay margin for accuracy requirements

- A margin to be specified, based on per-hop BW, but the margin is one number for the entire measurement including all hops.

- Frequency drift margin for accuracy requirements

- Reuse frequency drift margin values, Y values in existing specification, for single PFL and multiple PFLs defined for non-RedCap UE to specify impact of frequency drift margin value on RSTD measurement performed by RedCap UE with FH on single PFL and multiple PFLs.

- PRS RMC for RedCap positioning TCs

- A new PRS RMC with large BW and repetition for 30kHz SCS is introduced to be used in FH TCs for accuracy requirement.

- Test configuration for RedCap positioning TCs

  - Test case for RedCap positioning with FH not to be based on BWP switching framework.

  - Cell BW and UE BW are to be separately defined in the test configuration for RedCap positioning with FH.

    - UE BW can be smaller than the cell BW.

  - OCNG to be aligned with cell BW.

    - The generic OCNG pattern OP.1 specified in section A.3.2.1.1 of TS38.133 is used in the test case.

- Accuracy requirements for RedCap positioning measurements

  - Add results from Nokia into R4-2416325.

  - Companies to check updated simulation summary for RedCap positioning revised R4-2416325 and confirm if new average value proposed in the summary document can be used to define accuracy requirement for RedCap positioning.

  - If the numbers are updated in CRs, add [] until RAN4#113.

The following agreements were made for PRS/SRS bandwidth aggregation for positioning (R4-2416869):

Core requirement:

- Discussion on Tnon-aggregate in core requirement for PRS aggregation

  - When UE is indicated by LMF in nr-DL-PRS-JointMeasurementRequest-r18 to perform measurements by aggregating resources from multiple PFLs, UE measures resources that are indicated by LMF as linked resources and resources that are not indicated by LMF as linked resources, i.e. the current requirements apply.

Performance requirement:

- Accuracy requirement for Rx-Tx measurement in AWGN with reduced number of samples.

  - Accuracy requirements for UE Rx-Tx time difference measurements for 2PFLs and 3PFLs in FR1 and FR2 in AWGN with reduced number of samples are defined using the values in Tables in option 2 that are derived by averaging values submitted by companies (R4-2416328).

  - Change the order of columns with 2 and 3 PFLs.

  - Add square brackets.

- Accuracy requirement for Rx-Tx measurement in AWGN with 4 samples.

  - Accuracy requirements for UE Rx-Tx time difference measurements for 2PFLs and 3PFLs in FR1 and FR2 in AWGN with 4 samples are defined using the values in Tables in option 2 that are derived by averaging values submitted by companies (R4-2416328).

  - Change the order of columns with 2 and 3 PFLs.

  - Add square brackets.

- Accuracy requirement for Rx-Tx measurement in fading conditions with 4 samples.

  - Accuracy requirements for UE Rx-Tx time difference measurements for 2PFLs and 3PFLs in FR1 and FR2 in fading channel with 4 samples are defined using the values in Tables in option 2 that are derived by averaging values submitted by companies (R4-2416328).

  - Change the order of columns with 2 and 3 PFLs.

  - Add [].

- Accuracy requirement for RSTD measurement in AWGN conditions with reduced number of samples.

  - Accuracy requirements for RSTD measurements for 2PFLs and 3PFLs in FR1 and FR2 in AWGN channel with reduced number of samples are defined using the values in Tables in option 2 that are derived by averaging values submitted by companies (R4-2416328).

  - Change the order of columns with 2 and 3 PFLs.

  - Add [].

- Accuracy requirement for RSTD measurement in AWGN conditions with 4 samples.

  - Accuracy requirements for RSTD measurements for 2PFLs and 3PFLs in FR1 and FR2 in AWGN channel with 4 samples are defined using the values in Tables in option 2 that are derived by averaging values submitted by companies (R4-2416328).

  - Change the order of columns with 2 and 3 PFLs.

  - Add [].

- Accuracy requirement for RSTD measurement in fading conditions with 4 samples.

  - Accuracy requirements for RSTD measurements for 2PFLs and 3PFLs in FR1 and FR2 in fading channel with 4 samples are defined using the values in Tables in option 2 that are derived by averaging values submitted by companies (R4-2416328).

  - Change the order of columns with 2 and 3 PFLs.

  - Add [].

The following agreements were made for sidelink positioning (R4-2416870):

Core requirement

- RAN1 reply LS (R4-2414905/R1-2407401)

  - For the RAN1 LS response, neither reply nor updates to RAN4 specification are needed.

- End point of SL-PRS based RSTD measurement period requirements

  - Reflect in TS 38.133 the SL PRS measurement of the reference UE link in the SL RSTD core requirement.

Performance requirement

- Update values of accuracy requirement

  - Update the average numbers in R4-2410196 (CATT to update). The latest version can be found in R4-2416905.

SL-PRS signal level configurations

- Agree to have different Es/Iot and Noc numbers in TCs for SL-PRS and PSCCH.

  - Es/Iot levels:

    - SL PRS Es/Iot: -3 dB

    - PSCCH Es/Iot: [3] dB

- Transmitted SL-PRS from different anchor UEs do not overlap in TCs.

- The above agreements can be revisited if the above approach is not supported by RAN1 agreements.

- Comb size configuration

  - (symbol num, comb size): (4, 4) and (2, 4)

- Type of resource pool

  - Add applicability statement in TS 38.133:

    - UE supporting two pools has to pass accuracy TC with one pool configured and delay TC with another pool configured for the same measurement (SL RSTD, SL Rx-Tx, SL RSRP/RSRPP).

    - For other cases, the UE is tested with the supported pool configuration.

- Multiplexing of TX UEs in TCs

  - Discuss further:

    - For 2 transmitting UEs, the time offset between the two SL-PRS configurations can be, e.g.:

    - MUX of multiple TX UEs: TX UE1: slot n, TX UE2: slot n + 1 and slot n + 160ms.

    - Time offset for 3 transmitting anchor UEs needs to be also decided.

    - Come back next meeting.

The following agreements were made for CPP (R4-2416870):

Core requirement

- CPP measurement period requirements with one-shot time window

  - When one-shot time window is configured,

  - the measurement period for RSTD+RSCPD and Rx-Tx+RSCP is ⁡(Lwindow+ Ti), and

  - the measurement period for RSTD/Rx-Tx is maxTeffective,i+Tlegacy,

  - the start of the measurement is the start of the one-shot time window,

  - Ti is the processing time for PFL i,

  - Tlegacy is the legacy requirements for RSTD and Rx-Tx.

  - The above requirements assume scaling factors equal 1 for the PFLs, for which the one-shot time window is configured.

Performance requirement

- Applicable scenarios for CPP measurements

  - RAN4 to only define accuracy for the scenarios where CPP measurements are performed on PRS resources within the same sets of slots and sets of symbols (i.e. the agreed scenario), considering PRS repetitions.

  - Set of symbols is based on PRS configuration.

- Margins for accuracy requirements due to carrier phase offset/RSTD uncertainty

  - Define accuracy requirements based on the available simulation results for:

    - FR1:

      - BW>24 PRBs: [] can be removed

      - 24 PRBs: keep [] until RAN4#113

    - FR2:

      - 60 kHz

      - BW>24 PRBs: [] can be removed

      - 24 PRBs: keep [] until RAN4#113

    - 120 kHz

      - BW>32 PRBs: [] can be removed

      - 32 PRBs: keep [] until RAN4#113

- Accuracy requirements for additional PRS configurations

  - For RSCP/RSCPD accuracy requirements:

    - CATT to update the simulation results summary, including Nokia’s results. The latest version can be found in R4-2416906.

    - Discuss the average numbers and include the agreeable numbers and configurations into CR (revision of R4-2416329)

    - The requirements may be updated in the next meeting (up to the discussion in the next meeting)

  - Interested companies can provide simulation results for deriving the TBDs, e.g. in Table 10.1.43.2 (Measurement Accuracy Requirements for RSCPD).

General for RRM performance requirements and test cases:

  - All draftCR-s are endorsed.

##### 2.4.1.2	Decisions during RAN4#113

##### 2.4.1.2.1	General aspects

##### 2.4.1.2.2	RF

##### 2.4.1.2.3	RRM performance

General for RRM performance requirements and test cases:

  - Big CR to 38133 for RRM performance part for expanded and improved NR positioning is agreed in R4-2420151.

  - Brackets and TBD-s are removed from the agreed big CR.

The following agreements were made for RedCap positioning performance requirement (R4-2420086):

- Group delay margin values for RedCap positioning measurements

  - The issue on group delay margin was resolved in RAN4#112-bis. No further discussion is needed.

The following agreements were made for performance requirement for PRS/SRS bandwidth aggregation for positioning (R4-2420086):

- Accuracy requirement for PRS aggregation for positioning measurements

  - Simulation summary for PRS BW aggregation is updated to include results from R4-2419771. Consider updating the accuracy requirements, based on the agreed configurations and earlier agreed simulations assumptions.

The following agreements were made for sidelink positioning performance requirement (R4-2420087):

- Configurations of TX UEs in TCs

  - For TCs with 2 TX UEs

    - If UE supports at least 2 active slots, the 2 TX UEs are in consecutive SL slots.

    - If UE supports only 1 active slot, the 2nd TX UE is in the next SL slot immediately after UE finishes processing the 1st TX UE.

  - For TCs with 3 TX UEs

    - If UE supports more than 2 active slots, the 3 TX UEs can be in consecutive SL slots.

    - If UE supports 2 active slots, the first 2 TX UEs are in consecutive SL slots, and the 3rd TX UE is in the next SL slot immediately after UE finishes processing the 1st TX UE

    - If UE supports only 1 active slot, the 2nd TX UE is in the next SL slot immediately after UE finishes processing the 1st TX UE, and the 3rd Tx UE is in the next SL slot immediately after UE finishes processing the 2nd TX UE.

- Reporting granularity for SL RTOA

  - To align with SL RSTD measurements, remove timing resolutions k = {0, 1} from the reporting mapping for SL RTOA.

The following agreements were made for CPP performance requirement:

- CPP accuracy measurements

  - Simulation results in R4-2419359 (Ericsson): update simulation summary (CATT), consider the results in deriving accuracy requirements.

  - RAN4 to clarify that CPP measurement accuracy requirements do not apply for comb size 12.

  - Larger BWs: Define accuracy requirements.

- Further discuss off-line to converge on numbers for Y values in RAN4#113, e.g.:

  - 14.2 deg for FR1

  - 28 deg for FR2

- Further agreements:

  - Keep the accuracy requirements for the configurations with 4 repetitions.

  - The following Y values are confirmed and to be used in the CR:

    - 14 deg for FR1

    - 28 deg for FR2

#### 2.4.2	Remaining Open issues

- None.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

###### RAN4 #112bis

- R4-2415160	Draft CR on core requirements for RedCap positioning	CATT

- R4-2415161	Draft CR on core requirements for SRS  BW aggregation	CATT

- R4-2415162	Draft CR on core requirements for SL positioning	CATT

- R4-2415163	Draft CR on core requirements for CPP	CATT

- R4-2415164	Draft CR on SL-PRS configuration for TCs	CATT

- R4-2415165	Discussion on Core requirements of RedCap and BW aggregation Pos	CATT

- R4-2415166	Discussion on Core requirements of SL Pos and CPP	CATT

- R4-2415167	Discussion on Performance requirements of RedCap UE positioning	CATT

- R4-2415168	Discussion on Performance requirements of SL positioning	CATT

- R4-2415605	Corrections on test cases for CPP	CATT

- R4-2415847	Discussion on core requirements for SL positioning and CPP	Huawei, HiSilicon

- R4-2415848	draftCR on RRM requirements for SL positioning	Huawei, HiSilicon

- R4-2415849	draftCR on RRM requirements for CPP	Huawei, HiSilicon

- R4-2415850	Discussion on core requirements for RedCap positioning and PRS/SRS BWA	Huawei, HiSilicon

- R4-2415851	draftCR on requirements for RedCap positioning	Huawei, HiSilicon

- R4-2415852	draftCR on requirements for PRS BWA	Huawei, HiSilicon

- R4-2415853	draftCR on PRS RMC	Huawei, HiSilicon

- R4-2415854	Discussion on performance requirements for SL positioning	Huawei, HiSilicon

- R4-2415855	draftCR on accuracy requirements for SL positioning	Huawei, HiSilicon

- R4-2415856	draftCR on performance requirements for LPHAP	Huawei, HiSilicon

- R4-2415857	Discussion on performance requirements for RedCap positioning	Huawei, HiSilicon

- R4-2415858	draftCR on performance requirements for RedCap positioning	Huawei, HiSilicon

- R4-2415859	draftCR on performance requirements for PRS BWA	Huawei, HiSilicon

- R4-2415860	Discussion on performance requirements for CPP	Huawei, HiSilicon

- R4-2415861	draftCR on performance requirements for CPP	Huawei, HiSilicon

- R4-2415925	On RRM core maintenance for RedCap positioning and PRS/SRS bandwidth aggregation	Qualcomm Incorporated

- R4-2415926	On performance requirements for SL positioning	Qualcomm Incorporated

- R4-2415927	On performance requirements for RedCap positioning	Qualcomm Incorporated

- R4-2415928	On performance requirements for PRS bandwidth aggregation	Qualcomm Incorporated

- R4-2416198	Draft CR 38133 on remaining core issues for SL positioning	Ericsson

- R4-2416199	On remaining performance issues for SL positioning	Ericsson

- R4-2416200	Draft CR 38133 on remaining performance issues for SL positioning	Ericsson

- R4-2416201	Updated draft Big CR to 38133 for RRM performance part for expanded and improved NR positioning	Ericsson

- R4-2416318	draftCR to 38.133 on corrections to core requirements for CPP	Ericsson

- R4-2416319	On core requirements for RedCap positioning and BW aggregation for positioning measurements	Ericsson

- R4-2416320	draftCR to 38.133 on interruption requirement for SRS aggregation for positioning	Ericsson

- R4-2416321	draftCR to 38.133 on corrections to core requirements for RedCap positioning	Ericsson

- R4-2416322	draftCR to 38.133 on test cases for LPHAP	Ericsson

- R4-2416323	On performace requirement for RedCap positioning	Ericsson

- R4-2416324	draftCR to 38.133 on performance requirements for RedCap positioning	Ericsson

- R4-2416325	Updated summary of simulation results for RedCap positioning	Ericsson

- R4-2416326	draftCR to 38.133  on accuracy requirement and test cases for BW aggregation for positioning measurements	Ericsson

- R4-2416327	On performance requirement for bandwidth aggregation for positioning measurements	Ericsson

- R4-2416328	Updated summary of simulation results for PRS aggregation	Ericsson

- R4-2416329	draftCR to 38.133 on performance requirement for CPP	Ericsson

- R4-2416434	Simulation results for PRS Bandwidth Aggregation	Nokia

- R4-2416435	RRM performance requirements for NR Carrier Phase Positioning	Nokia

- R4-2416436	CR 38.133 Introduction of TCs on CPP measurement accuracy	Nokia

- R4-2416437	Simulation results for DL RSCPD and DL RSCP	Nokia

- R4-2416444	RRM Core Requirements for RedCap Positioning	Nokia

- R4-2416445	RRM Performance Requirements for RedCap Positioning	Nokia

- R4-2416446	Simulation Results for RedCap Positioning without Frequency Hopping	Nokia

- R4-2416505	On the Need for Additional Scenarios for CPP	Lenovo

- R4-2416881	Ad-hoc minutes #2 for NR_pos_enh2 WI	Ericsson

- R4-2416882	Ad-hoc minutes #3 for NR_pos_enh2 WI	Ericsson

- R4-2416884	Draft Big CR to 38133 for RRM core part for expanded and improved NR positioning	Ericsson

- R4-2416885	draftCR on requirements for RedCap positioning	Huawei, HiSilicon

- R4-2416886	Draft CR on core requirements for SRS  BW aggregation	CATT

- R4-2416887	draftCR on requirements for PRS BWA	Huawei, HiSilicon

- R4-2416888	Draft CR on core requirements for SL positioning	CATT

- R4-2416889	Draft CR 38133 on remaining core issues for SL positioning	Ericsson

- R4-2416890	draftCR on RRM requirements for CPP	Huawei, HiSilicon

- R4-2416891	draftCR to 38.133 on corrections to core requirements for CPP	Ericsson

- R4-2416892	draft Big CR to 38133 for RRM performance part for expanded and improved NR positioning	Ericsson

- R4-2416893	draftCR on performance requirements for RedCap positioning	Huawei, HiSilicon

- R4-2416894	draftCR to 38.133 on performance requirements for RedCap positioning	Ericsson

- R4-2416895	draftCR on performance requirements for PRS BWA	Huawei, HiSilicon

- R4-2416896	draftCR to 38.133  on accuracy requirement and test cases for BW aggregation for positioning measurements	Ericsson

- R4-2416897	Draft CR on SL-PRS configuration for TCs	CATT

- R4-2416898	Draft CR 38133 on remaining performance issues for SL positioning	Ericsson

- R4-2416899	Corrections on test cases for CPP	CATT

- R4-2416900	draftCR to 38.133 on performance requirement for CPP	Ericsson

- R4-2416901	draftCR on performance requirements for LPHAP	Huawei, HiSilicon

- R4-2416902	draftCR to 38.133 on test cases for LPHAP	Ericsson

- R4-2416903	Updated summary of simulation results for RedCap positioning	Ericsson

- R4-2416904	Updated summary of simulation results for PRS aggregation	Ericsson

- R4-2416905	Updated summary of simulation results for SL positioning accuracy requirements	CATT

- R4-2416906	Updated summary of simulation results for CPP accuracy requirements	CATT

- R4-2416869	WF on RRM requirements for NR_pos_enh2_Part1	Ericsson

- R4-2416870	WF on RRM requirements for NR_pos_enh2_Part2	CATT

###### RAN4 #113

- R4-2417691	Corrections to test cases for SL positioning	CATT

- R4-2417692	Corrections to test cases for RedCap positioning	CATT

- R4-2417693	Corrections to performance parts for CPP	CATT

- R4-2417694	Discussion on Performance requirements of SL positioning	CATT

- R4-2417695	Discussion on Performance requirements of CPP	CATT

- R4-2418785	Draft CR on RRM core requirements for CPP measurements	OPPO

- R4-2418786	Draft CR on RRM core requirements for PRS BW aggregation	OPPO

- R4-2418787	Draft CR on RRM perf requirements for SL positioning	OPPO

- R4-2418788	Discussion on RRM performance requirements for SL positioning	OPPO

- R4-2419133	CR on RRM requirements for SL positioning	Huawei, HiSilicon

- R4-2419134	CR on requirements for RedCap positioning	Huawei, HiSilicon

- R4-2419135	CR on requirements for PRS BWA	Huawei, HiSilicon

- R4-2419136	draftCR on PRS RMC	Huawei, HiSilicon

- R4-2419137	Discussion on performance requirements for SL positioning	Huawei, HiSilicon

- R4-2419138	draftCR on TCs for SL positioning	Huawei, HiSilicon

- R4-2419139	draftCR on performance requirements for RedCap positioning	Huawei, HiSilicon

- R4-2419140	Discussion on performance requirements for CPP	Huawei, HiSilicon

- R4-2419141	draftCR on performance requirements for CPP	Huawei, HiSilicon

- R4-2419351	CR to 38.133 on corrections to core requirement for bandwidth aggregation for positioning measurements	Ericsson

- R4-2419352	CR to 38.133 on corrections to core requirements for RedCap positioning	Ericsson

- R4-2419353	CR to 38.133 on corrections to core requirements for CPP	Ericsson

- R4-2419354	CR to 38.133 on corrections to LPHAP core requirements	Ericsson

- R4-2419355	draftCR to 38.133 on corrections to LPHAP test cases	Ericsson

- R4-2419356	draftCR to 38.133 on accuracy requirement and test cases for RedCap positioning	Ericsson

- R4-2419357	draftCR to 38.133 on accuracy requirement and test cases for BW aggregation for positioning measurements	Ericsson

- R4-2419358	draftCR to 38.133 on corrections to CPP test cases	Ericsson

- R4-2419359	Accuracy requirements for additional PRS configurations for CPP	Ericsson

- R4-2419650	On remaining core issues for NR positioning	Ericsson

- R4-2419651	CR 38133 on remaining core issues for SL positioning	Ericsson

- R4-2419652	On remaining performance issues for SL positioning	Ericsson

- R4-2419653	Draft CR 38133 on remaining performance issues for SL positioning	Ericsson

- R4-2419654	Updated draft big CR to 38133 for RRM performance part for expanded and improved NR positioning	Ericsson

- R4-2419665	RRM Performance Requirements for RedCap Positioning	Nokia

- R4-2419700	On remaining core issues for NR positioning	Ericsson

- R4-2419701	CR 38.133 on remaining core issues for SL positioning	Ericsson

- R4-2419702	On remaining performance issues for SL positioning	Ericsson

- R4-2419703	Draft CR 38133 on remaining performance issues for SL positioning	Ericsson

- R4-2419704	Updated draft big CR to 38133 for RRM performance part for expanded and improved NR positioning	Ericsson

- R4-2419762	Big CR to 38133 for RRM core part for expanded and improved NR positioning	Ericsson

- R4-2419771	Simulation results for PRS Bandwidth Aggregation	Nokia

- R4-2419772	RRM performance requirements for NR Carrier Phase Positioning	Nokia

- R4-2419773	Draft CR 38.133 Introduction of margins for DL RSCPD and DL RSCP measurement accuracy	Nokia

- R4-2419777	On Ambiguity in the Definition of RSCP with Frequency Offsets	Lenovo

- R4-2419778	On Ambiguity in the Definition of RSCP with Carrier Frequency Offsets	Lenovo

- R4-2420133	CR on requirements for RedCap positioning	Huawei, HiSilicon

- R4-2420134	CR to 38.133 on corrections to core requirement for bandwidth aggregation for positioning measurements	Ericsson

- R4-2420135	CR on requirements for PRS BWA	Huawei, HiSilicon

- R4-2420136	Draft CR on RRM core requirements for PRS BW aggregation	OPPO

- R4-2420137	CR to 38.133 on corrections to core requirements for CPP	Ericsson

- R4-2420138	Draft CR on RRM core requirements for CPP measurements	OPPO

- R4-2420139	Corrections to test cases for RedCap positioning	CATT

- R4-2420140	draftCR on performance requirements for RedCap positioning	Huawei, HiSilicon

- R4-2420141	draftCR to 38.133 on accuracy requirement and test cases for RedCap positioning	Ericsson

- R4-2420142	draftCR to 38.133 on accuracy requirement and test cases for BW aggregation for positioning measurements	Ericsson

- R4-2420143	Corrections to test cases for SL positioning	CATT

- R4-2420144	Draft CR on RRM perf requirements for SL positioning	OPPO

- R4-2420145	draftCR on TCs for SL positioning	Huawei, HiSilicon

- R4-2420146	Draft CR 38133 on remaining performance issues for SL positioning	Ericsson

- R4-2420147	Corrections to performance parts for CPP	CATT

- R4-2420150	Big CR to 38133 for RRM core part for expanded and improved NR positioning	Ericsson

- R4-2420151	Big CR to 38133 for RRM performance part for expanded and improved NR positioning	Ericsson

- R4-2420086	WF on RRM requirements for NR_pos_enh2_Part1	Ericsson

- R4-2420087	WF on SL positioning and carrier phase positioning performance	CATT

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template