3GPP TSG-RAN Meeting #106	RP-242788
Madrid, Spain, Dec 09-12, 2024

Title:	RAN task on HPUE with 1UL and DL CA

Source:	OPPO, Huawei

Agenda item:	14

Document for:	Approval

- Introduction

In RAN#104, RAN4 was tasked to investigate how to enable HPUE in DL CA with 1UL CC configured scenario [1] as below table.

Way Forward - RAN tasks RAN4 to investigate how to enable HPUE maximum transmit power in downlink CA with a single configured UL. - RAN task shall conclude by RAN#106 - RAN4 online meeting time should be limited to <0.1 TUs

And in the following RAN4 meetings, some offline have been organized for this topic, and finally got conclusion as below table in last RAN4 meeting [2][3][4].

Issue 1-1: Output power Capture in TS 38.101-1: As an exception, HPUE applicability notes in 5.5A do not restrict power class indication for single configured UL CC with DL CA. UE can indicate power class for the single configured UL CC with DL CA as specified in Table 6.2.1-1 and if UE supports UL MIMO in this carrier, UE can indicate power class for the CA configuration as specified in Table 6.2D.1-1. Issue 1-2: Receiver requirements Capture in TS 38.101-1: When UE supports higher power class than default power class for a CA configuration with a single UL CC with DL CAand applicability note for the supported power class is not present for this configuration in clause 5.5A. - if the corresponding higher power class MSD is not specified, reference sensitivity and exceptions for reference sensitivity of default power class shall be verified with output power limited to default power class - otherwise, the higher power class reference sensitivity and exceptions for reference sensitivity shall be verified with the power class of CA configuration supported by the UE. Issue 1-3: Other agreements - UEs already on the market shall not be retested after the HPUE MSD is defined. - UEs already under design shall not be required to meet the HPUE MSD, i.e. a 6 month transition period shall be applied to the newly introduced requirements in issue 1-1 and 1-2. - Further consider how to reflect above two bullets in the specification and inform RAN5 with LS. - The changes can be reverted/removed in Rel-19 if new MSD rules are agreed. - In the task report to RAN, inform RAN and RAN5 about the agreements and progress on the new MSD rules discussed under Rel-19 NR_ENDC_RF_Ph4 WI. - Further discussion on UE signaling for DL CA with 1 UL - FFS whether and how to update the HPUE applicability notes in clause 5.5A, e.g. change “applicable” to “specified”. - FFS whether and how to make updates on configured transmission power.

This paper shares some information of the RAN4 conclusions, clarifies the open issues lefts and also propose LS to RAN5 on the testing related issues.

- Discussion

  - HPUE MSD application transition period

In last RAN4 meeting, it was agreed to enable the HPUE in DL CA with 1UL CC configured before the HPUE MSD (Maximum Sensitivity Degradation) requirements are defined.

And UE is tested with PC3 MSD requirements first, later on after the PC2 MSD requirements are defined new UEs will be tested with PC2 MSD requirements.

However, to handle the situation that some UEs have already on the market or already under development according to the PC3 MSD requirements, which cannot change its hardware, RAN4 agreed that for these two kinds of UEs they will only be verified with PC3 MSD requirements. And a 6 months transition period shall be applied to the newly introduced requirements as shown in figure 1.

Figure 1 Transition period for HPUE MSD requirement application

Observation 1:   RAN4 agreed to enable HPUE before the HPUE MSD requirements are defined. And the HPUE can be tested with PC3 MSD requirements before the corresponding MSD requirements are defined.

Observation 2:   For UEs already on the market or already under development, they will not be tested with the newly introduced HPUE MSD.

Observation 3:   A 6 months transition period is applied to the newly introduced HPUE MSDs before it can be used to test UEs.

And it was also agreed in RAN4 that RAN5 shall be informed about the above HPUE MSD application rules. Therefore, we propose to send LS to RAN5 in this RAN plenary to inform above agreements when closing this RAN task.

Proposal 1:         Inform RAN5 about the HPUE MSD application principles and transition period. Draft LS is in RP-243248.

  - New MSD rules in UE RF enhancement WI

RAN4 now is discussing the “New MSD rules” in UE RF enhancement WI which is targeting to simplify the basket WI procedures especially the MSD definition approaches by for example look up table [5].

Agreement:

- Work on the feasible unified Equations/LUT/Simplified guidelines for MSD for HPUE band combinations as normative requirements

  - Targeting at introduction of unified Equations/LUT/Simplified guidelines to replace the existing method using HPUE MSD tables at the end of Rel-19 or earlier if agreed.

  - The existing MSD requirements for band combinations for HUPE won’t be changed.

  - Discuss whether the request procedure for HPUE band combinations should be kept

- For Rel-19, follow tradition procedure, including the request of HPUE band combinations, until the work on the feasible unified Equations/LUT/Simplified guidelines is finished.

More specifically, when introducing a new band combination to the specification, currently companies need to calculate the MSD requirements which is the most time-consuming part. If the new look up table approach can be agreed, the MSD requirements can be derived by some look up tables directly and no complex calculation is needed anymore. The time to introduce a band combination will be very short. In this case, the issue that RAN task is trying to solve will be no longer a problem.

Therefore, it was agreed that after the new MSD rules are defined, the specification changes for the RAN task can be reverted/removed in Rel-19. From this sense, the RAN task changes reached now may become a temporary solution.

Observation 4:   RAN4 is working on new MSD rules to simplify the MSD definition by look up table approach.

Observation 5:   If look up table approach is agreed in Rel-19, the time to introduce a band combination will be short, and the issue that RAN task is trying to solve may no longer be a problem.

Observation 6:   RAN4 has agreed that after the new MSD rules are defined, the specification changes for the RAN task can be reverted/removed in Rel-19. From this sense, it should be aware that the agreed RAN task solution now may become a temporary solution.

Proposal 2:         RAN4 continue working on the new MSD rules and FFS on the potential impact on the RAN task solution.

- Conclusion

This paper discussed the RAN task solution for enabling HPUE in DL CA with 1UL CC in RAN4 and also shared some information of the remaining issues that need to be further discussed in RAN4 or RAN5.

HPUE MSD application transition period

Observation 1:   RAN4 agreed to enable HPUE before the HPUE MSD requirements are defined. And the HPUE can be tested with PC3 MSD requirements before the corresponding MSD requirements are defined.

Observation 2:   For UEs already on the market or already under development, they will not be tested with the newly introduced HPUE MSD.

Observation 3:   A 6 months transition period is applied to the newly introduced HPUE MSDs before it can be used to test UEs.

Proposal 1:         Inform RAN5 about the HPUE MSD application principles and transition period. Draft LS is in RP-243248.

New MSD rules in UE RF enhancement WI

Observation 4:   RAN4 is working on new MSD rules to simplify the MSD definition by look up table approach.

Observation 5:   If look up table approach is agreed in Rel-19, the time to introduce a band combination will be short, and the issue that RAN task is trying to solve may no longer be a problem.

Observation 6:   RAN4 has agreed that after the new MSD rules are defined, the specification changes for the RAN task can be reverted/removed in Rel-19. From this sense, it should be aware that the agreed RAN task solution now may become a temporary solution.

Proposal 2:         RAN4 continue working on the new MSD rules and FFS on the potential impact on the RAN task solution.

- Reference

[1] RP-241687 Moderator's summary for offline discussion on CA Power Class Support, RAN vice-chair (AT&T), RAN#104 2024-06

[2] R4-2420520 CR to TS 38.101-1: DL CA HPUE (R17), Qualcomm, RAN4#113 2024-11

[3] R4-2420524 CR to TS 38.101-1: DL CA HPUE (R18), Qualcomm, RAN4#113 2024-11

[4] R4-2420525 WF on solution to enable HPUE max transmit power in DL CA, RAN4#113 2024-11

[5] R4-2420380 WF on MSD rule, Skyworks, Murata, Nokia, Qualcomm, RAN4#113