3GPP TSG-RAN Meeting #106	RP-242801

Madrid, Spain, December 17th – 20th, 2024

Agenda item:	9.2.2

Title:	Discussion on Rel-19 Ambient IoT potential work item scope

Source:	OPPO

Document for:	Discussion and Decision

# 1. Introduction

For the approved study item on solutions for Ambient IoT in NR [1], its corresponding progress and necessary conclusions have been reached in all RAN WGs (RAN1, RAN2, RAN3 and RAN4), respectively, and thus the study can be considered as complete for closure in this RAN plenary. A follow-up work item for A-IoT would then seem reasonable to be approved at least in RAN, but this could be dependent on the timeframe to complete the normative work.

Proposal 1: Since the Release 19 RAN A-IoT study item has been completed in WGs (RAN1/2/4), it is proposed a follow-up normative phase should be setup and approved in this RAN#106, also targeting Release 19.

# RAN1 WG recommendation on the scoping for a normative phase

According to the latest WG endorsed TR 38.769 [2] for the A-IoT study in Release 19, in the conclusion section the following RAN1 recommendations are made on a potential scope of a follow-up new RAN work item for the normative phase. In particular, the detailed aspects and technical features in part (2) require some attentions and decisions at the plenary level.

RAN1 recommends that in a normative phase, the design should start from the descriptions in this TR. (1) The following techniques are recommended in the first normative phase: - PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R - Modulations of OOK-1 and OOK-4 in R2D - Multiplexing in time domain for R2D - Convolutional FEC in D2R and no FEC in R2D - R2D and D2R signal(s) (2) At least the following aspects may be considered for inclusion or non-inclusion in a given first or subsequent Release: - Device 1 and/or Device 2a (with or without large frequency shift) and/or Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture) - D1T1 and/or D2T2 - Scenario A1 and/or A2 and/or B and/or C - Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R - Multiple access by TDMA and/or FDMA and/or CDMA for D2R; FDMA for device 2b (IF-ED/ZIF receiver) in R2D - OOK and/or BPSK and/or MSK modulation for D2R - Carrier-wave waveform 1 and/or 2, and transmission case(s) - Device (un)availability direction 1 and/or 2 - Proximity determination solution 1 and/or 2

Observing from the inclusion and non-inclusion list in part (2) of the recommendations, it is firstly recognized besides deployment related features such as device types, connectivity topologies and proximity determination, other design choices would seem to require some technical analysis and discussions in order to make a decision, which is non-ideal at the plenary. In particular, for those features that require further technical justifications would cause some consequences / improvements to the performance or efficiency of A-IoT operations, such as coding scheme, modulation and multiple access and carrier wave designs. It is, therefore, suggested to postpone the decisions on the inclusion or non-inclusion of the corresponding design choice(s) for these topics during the normative phase in the WG(s).

Observation 1: Inclusion or non-inclusion of some A-IoT design choices or features that require technical justification and/or would have consequence / impact to the performance or efficiency of A-IoT operation will be difficult to make a decision at the RAN plenary level, such as

- Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R

- Multiple access by TDMA and/or FDMA and/or CDMA for D2R

- OOK and/or BPSK and/or MSK modulation for D2R

- Repetition for D2R and R2D

- Carrier-wave waveform 1 and/or 2

Proposal 2: It is proposed the down-selection of the following technical design choices is to be done in the WG(s) during the normative phase

- Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R

- Multiple access by TDMA and/or FDMA and/or CDMA for D2R

- OOK and/or BPSK and/or MSK modulation for D2R

- Repetition for D2R and R2D

- Carrier-wave waveform 1 and/or 2

# Representative use cases, connectivity topologies and device types

For the presentative use cases of A-IoT technology captured in [1], the main traffic types are DO-DTT (Device-originated – device-terminated triggered), DT (Device-terminated) with focus on rUC1 (indoor inventory) and rUC4 (indoor command). For the corresponding UCs in [3] and [4], which are quite wide ranging, they can be generalized into warehouses including logistics and supply chains, factories, supermarkets, shops, homes, hospitals, health care centers and airports/shipping ports. For some of these UCs, it is possible and likely to deploy indoor base stations for the improved performance, coverage and large system capacity. While for other use cases such as 5.2 Medical instruments inventory management and positioning, 5.15 Smart laundry, 5.11 Online modification of medical instruments status, and 5.26 Elderly Health Care [3], typically the number of A-IoT devices are relatively smaller, it is not affordable and sometimes infeasible (e.g., due to regulations) to deploy a BS indoor especially for them in the premise. The situation would be worse when considering that these devices may be distributed with physical blockages in-between or into different rooms separated by walls. Therefore, they will rely on a UE reader which could be a smartphone connects to a serving base station in order to make an inventory or sending commands to the A-IoT devices that are located indoor. Furthermore, with the use of UE readers, it is possible for the already deployed BS can be reused with only software updates to support of A-IoT services. As such, it is functional necessary and was agreed to conduct a RAN WG study on A-IoT for indoor use cases with connectivity topologies 1 and 2 in [1].

According to the study outcomes from all RAN WGs (RAN1, RAN2, RAN3 and RAN4), both connectivity topologies 1 and 2 are studied and can be considered as completion (as declared in either chair notes, liaison statements or the TR [1] from the November WG meetings). In contrast to Topology 1, additional works needed for support of Topology 2 foreseen are as below:

- RAN1: Resource allocation and/or controlling of intermediate UE for R2D and D2R transmissions via the Uu interface between the BS and intermediate UE. RAN1 has studied 2 options, Option 1: Higher-layer signaling is used, and Option 2: L1 and higher-layer signaling are used. Both of them are relating to signaling design and only a small workload is expected as these are common topics specified for many WIs in the past.

- RAN2: As analyzed below, RAN2 has already discussed thoroughly on the following issues that are related to connectivity topology 2, and at least had listed options to deal with issues. In the RAN2 WI stage, there is no need to re-discuss the feasibility of the related enhancements and therefore the standardization effort is manageable.

  - Protocol stack solution choices:

Currently there are two solutions on the table: RRC-based and UP/NAS solutions. RAN2 has already evaluated the impacts by these two solutions and are waiting for the SA2 to down-select the final choice.

  - Cell selection/reselection impact:

It may be beneficial for the UE reader to select / reselect to a cell supporting the A-IoT related operation. However, whether or not it is necessary to capture this enhancement in the specification remains questionable, if the time is limited. In addition, the specification impact is not considerable, in our opinion, if we look at the similar related enhancements, e.g., on cell selection and reselection for sidelink. We can take such related enhancement as baseline or a starting point.

  - UE reader specific indication in RRC Connection Setup/Resume:

This may be not needed if such specific indication could be provided with capability transfer message. We do not think that in the WI stage we need to trigger discussion on this.

  - CN/RAN paging towards potential UE reader(s) in idle/inactive

For UE reader in RRC_IDLE state, the CN could just send legacy paging message towards the UE reader to trigger it to come back to the RRC_CONNECTED state and then receives the related A-IoT command from the network, and therefore nothing enhancement is needed. For the UE reader in RRC_INACTIVE state, current MT-SDT could be re-used or only small enhancement is needed on that to enable the network to send A-IoT command/resource information towards it.

  - UE reader capability reporting:

This is needed as any other new optional feature introduced for the UE.

  - How BS knows when/how many resources to be allocated to a UE reader:

In RAN2 SI stage, we have already discussed the mechanism for the UE reader and/or the CN to provide necessary information to the BS for resource allocation. In RAN WI stage, the main remaining problem is how to define the format of the resource to be indicated to the UE, which may be dependent on RAN1’s discussion outcome. As a result, the standardization effort is also limited.

  - Reader obtains resource to perform A-IoT operation:

In RAN2 SI stage, we have identified two potential solutions for handling the validity of the resource configured towards the UE reader. The first one is to use explicit configuration/release message to control the validity of the resource configured towards the UE reader, and another one is to configure a timer running time together with the resource to the UE reader to define the duration in which the resource is valid. Similar solution could be found in configuration an area-specific SRS resource in R18 positioning. We could take that as a baseline or starting point to minimize the standardization effort.

  - Supporting the A-IoT service continuity in temporary out-of-coverage scenario, e.g., Uu RLF or HO:

The focus of the discussion is still how to deal with the A-IoT resources that have been previously configured to the UE reader, if to suspend the UE reader for subsequent operation or let the UE reader to go on the service. This issue could be discussed together with the previous bullet.

- RAN3: UE reader selection if CN involved.

- RAN 4:  During the study RAN4 has assumed non-concurrent operation of A-IoT and NR for the intermediate UE. Depending on whether a CW will be used in Topology 2, the amount of work will be different. It is observed that if CW is not used in Topology 2 (i.e., for device 2b), most of the RF requirements follow legacy NR UE. According to [2], the following study outcomes are captured.

  - Transmission requirements:

    - To reuse of the legacy NR UE hardware, power class 3 NR UE can serve as a baseline for intermediate UE using FDD UL spectrum as R2D spectrum.

    - Power control is not designed for R2D link currently, and intermediate UE transmit at maximum output power for better coverage, making a minimum output power and power control accuracy requirements unnecessary.

    - For the transmit ON/OFF time mask, it can be determined whether or not a settling time requirement similar to the RFID spec is necessary in WI phase.

    - Frequency error requirement of NR UE can be considered reusing for A-IoT intermediate UE.

    - EVM requirement is needed.

    - Whether and how to define carrier leakage requirement can be further discussed in WI.

    - In-band emission requirement is unnecessary if all RB within the channel bandwidth always allocated for A-IoT transmission.

    - Occupied bandwidth is dependent on the PHY layer design of R2D.

    - Existing spurious emission requirement for legacy NR UE can be applicable.

    - Transmit intermodulation is to be discussed in WI.

    - When CW is used:

      - Minimum output power and power control accuracy can be discussed further during WI

      - Transmit OFF power, the HD-FDD mode could be one option for outside topology and the requirement can be further discussed in WI phase.

      - For the intermediate node that has capability to transmit CW signal, the transient time between R2D to CW transmission also needs to be further discussed.

      - ACLR better than IBE may be required, and the details can be determined in WI phase.

  - Reception requirements:

    - When CW is used,

      - REFSENS requirements corresponding to inside topology and outside topology can be defined.

      - Maximum input power level can be further discussed in WI phase.

      - ACS requirement and the interference level depend on co-existence study.

      - In-band blocking requirement and the interference level depend on co-existence study.

      - Receiver intermodulation: if multiple-tone carrier wave is supported, intermodulation requirements potentially needed.

    - When CW is not used, many of the RX requirements follow legacy NR UE.

It is also observed the two candidate architecture solutions (AIOT RAN and the AIOTF communicate directly and indirectly via an AMF) for Topology 1 and two candidate architecture solutions (UP-based and RRC-based) for Topology 2 captured in the SA2 TR [5] are also completed (with remaining details to be worked out during the normative phase). As seen, both connectivity topologies 1 and 2 are deem to be feasible and either solution is ready to be specified in the normative phase.

Observation 2: It is observed the study of connectivity topologies 1 and 2 are carried out in all WGs (including RAN WGs and SA2) and can be considered completion.

- From the point of view of supporting all agreed representative use cases, it is necessary to support both connectivity topologies 1 and 2 in the first release of normative work for A-IoT.

- From the perspective of workload for Topology 2,

  - RAN1 has a minimal amount of work,

  - Workload for higher layer WGs (i.e.., RAN2/3, SA2) is manageable and expected to complete the normative work in time, and

  - RAN4’s workload in defining requirements heavily depends on whether CW is supported or not (e.g., relating to co-existence and CW cancellation aspects).

In RAN1, candidate device types are categorized into device 1, 2a and 2b, each with different peak power consumption and/or functionalities and/or characteristics targeting different use cases. For instance,

- Device 1 has the lowest peak power consumption at ~1µW, technically the simplest and it relies on a carrier wave to perform D2R backscattering transmissions. This device 1 mimics the functionalities and capabilities of RFID tags. There exists full duplex issue if a BS reader transmits a carrier wave and simultaneously receiving a backscattering wave from the device, to work around to the issue, in the deployment either a dedicated external carrier wave node will be used or the base station transmitter and receiver nodes are spatially separated (e.g., not co-located).

- For device 2a, it has a higher power consumption than device 1 at around a few hundreds of µW for better a processing capability and larger coverage. But the main specialty of device 2a lies in its capability of performing a large frequency shift on a backscattered carrier wave to mitigate the full duplexing issue just described and avoid spectrum usage restriction. During the course of RAN1 study of device 2a, the feasibility of large frequency shifting was discussed but it is not justified in the end due to the uncertainty of image suppression and harmonics suppression. Therefore, the necessity of device 2a diminishes.

- As for device 2b, its peak power consumption is similar to that of device 2a (so a similar coverage), it can support SSB transmission and amplification in R2D/D2R. Thus, Device 2b can achieve higher spectrum efficiency in D2R (thanks to SSB transmission) and larger coverage. In addition, it is capable of generating its own carrier frequency for D2R transmissions without the need of a carrier wave provided externally. For this type of device 2b, it is ideal to work in connectivity topology 2 where the UE reader (an intermediate node) may be a smartphone or a low-cost scanner which is not expected to implement a CW cancellation capability. According to the study in RAN1, device 2b can support IF/ZIF receiver architectures in addition to RF-RD, and hence, it can achieve a better sensitivity and frequency selectivity in R2D.

Based on the above analysis, it is observed that representative use cases targeted by A-IoT in Release 19 can be sufficiently fulfilled/supported by connectivity topologies 1 and 2 with device 1 and 2b. Therefore, it is proposed in the first Release of normative phase for A-IoT, both Topology 1 and 2, and device 1 and 2b should be included / supported to cover the representative use cases set out in [1], [3] and [5]. To harmonize the air interface design between device 1 and 2b, a dedicated signal for device 2b (i.e., dedicated signal for CFO calibration) can be omitted in the first release. Although this may reduce the accuracy of carrier frequency generated internally by device 2b, which may disable the coherent detection in D2R and ZIF receiver in R2D, but active transmission is still feasible with a slightly larger guard band. So does IF receiver, which does not rely on accurate frequency as ZIF receiver and is beneficial for FDMA in R2D and higher receiver sensitivity. Without a dedicated signal for CFO calibration specific work for support of device 2b can be minimized. To further reduce the scope for the normative phase, the corresponding evaluation scenarios from [2] should be: D1T1-A1, D1T1-A2, D1T1-B, D1T1-C and D2T2-C.

Observation 3: Based on the analysis in Section 3 of this contribution, it is observed that representative use cases targeted by A-IoT in Release 19 can be sufficiently fulfilled/supported by connectivity topologies 1 and 2 with device 1 and 2b.

Observation 4: To harmonize the air interface design between device 1 and 2b, no dedicated signal for CFO calibration is considered for device 2b.

Proposal 3: It is proposed evaluation scenarios D1T1-A1/A2/B/C and D2T2-C, and device 1 and 2b with RF-ED and IF receiver architectures for inclusion in the first Release of A-IoT normative phase.

# Device (un)availability direction 1 and/or 2

According to the description of Direction 1 for device (un)availability, reader does not provide information to a device regarding when the device may become available/unavailable, and based on company inputs in [2], it is expected that a solution in Direction 1 for device charging and ensuring devices are able to sustain a round of inventory/command process would require no specification effect (except for device reporting its measured energy status) and be implemented based on A-IoT deployment strategy. Therefore, the discussion in the RAN plenary should focus on the necessity of Direction 2 for A-IoT operations.

Based on company inputs in [2] and also explained in our contributions [6] [7], a solution in Direction 1 may not be sufficient and efficient for the purpose of device charging, and also ensuring a device will always sustain a round of inventory or command process for the following reasons and/or assumptions.

- Devices are able to perform energy harvesting and R2D/D2R operation at the same time

- RF signal for charging with sufficient power is constantly provided

- Devices always have equal charging time and discharging time

- Significant signaling overhead for frequent paging transmissions to ensure a minimum waiting time in the available/ON state to receive a paging (minimize energy spent on paging monitoring)

- Prone to TX collision (and thus long delay) when all devices are aligned / arranged to access at the same time for the same inventory / command process

- Potentially complex energy status reporting behaviour and its impact to the scheduling of Msg. 3 and future transmissions

- Potentially complicated energy-based segmentation operation in RAN2 (on top of the existing TBS-based segmentation)

Furthermore, based on the sustainable operating time (SOT) analysis provided by various companies in RAN1, due to the high power consumption of device 2 (and thus short SOT), a solution in Direction 2 will be necessary to prolong the SOT of device 2 even after an inventory or command process has started.

Observation 5: Since there are already solutions in Direction 1 that can be employed in a A-IoT deployment for the purpose of energy harvesting in a A-IoT device without any specification impact, it is not necessary to discuss the choice between Direction 1 and/or Direction 2. The discussion should be on the benefits and necessity of specification support for Direction 2.

Observation 6: Based on the analysis and reasonings provided in this Section 4 of this contribution, it is observed a solution in Direction 1 may not be sufficient and efficient for the purpose of device charging, and also ensuring a device will always sustain a round of inventory or command process.

Observation 7: Due to the high power consumption of device 2, a solution in Direction 2 will be necessary to prolong the operating time of device 2 even after the start of an inventory or command process.

Proposal 4: It is proposed that a solution for “device (un)availability Direction 2” should be supported in the first release of A-IoT normative phase.

# Positioning and Proximity determination solution 1 and/or 2

RAN3 has agreed to use reader ID to determine the location of a device, which provides similar functionality as proximity solution 1 considered in RAN1, thus it could also serve the purpose of proximity. Based on the positioning / proximity solution being considered and discussed in SA2 [5] and RAN2, only “solution 1” is necessary to be supported in the first release of A-IoT normative phase.

Proposal 5: Reader ID based positioning is included in the first normative phase in RAN. Note this can provide similar functionality as proximity determination solution 1. Based on the positioning / proximity solution being considered and discussed in SA2 [5] and RAN2, only “solution 1” agreed in RAN1 is necessary to be supported in the first release of A-IoT normative phase in RAN.

# Timeframe for A-IoT normative phase in Rel-19

For the above proposed scope of Topology 1 and 2, device 1 and 2b, device (un)availability Direction 2 and proximity solution 1, it is envisioned that a 12-month work is expected for the first Release of normative phase in RAN (e.g., by RAN#110 in December 2025) with RAN1 functional freeze by RAN#109 (i.e., 4 WG meetings). Although it is possible that no RRC layer is anticipated for the R2D/D2R interface, it is worth noting that ASN.1 work is still required for A-IoT due to higher layer architecture solutions considered in RAN3 and SA2 for both connectivity topologies 1 and 2.

In order to complete the above the general scope in terms of connectivity topologies, device types, device (un)availability and proximity determination, and the basic scope of signals/channels, coding, waveform, multiplexing, scheduling, random access and higher layer procedures designs, the TU allocation to achieve the above target completion date should be at least 3.5 TUs per RAN1 meeting, 2.5 TUs per meeting in RAN2, 1 TU per meeting in RAN3, and 1.5 TU per meeting in RAN4.

Proposal 6: For the general scope proposed in Section 3, 4 and 5 of this contribution, and the basic scope / harmonized design for A-IoT, it is proposed the first normative phase should be a 12-month duration (including ASN.1 work), where RAN1 functional freeze should be 1 quarter earlier (as per usual).

- TU allocation should be at least 3.5 TUs per RAN1 meeting, 2.5 TUs per meeting in RAN2, 1 TU per meeting in RAN3, and 1.5 TU per meeting in RAN4.

# Overall – Rel-19 package proposal for A-IoT normative phase

In order to cover the representative use cases that were studied in RAN within Release 19 and to make the first Release of A-IoT to be wisely useable (including consumer products), the following package is proposed for the first Release of A-IoT normative phase.

Package proposal for A-IoT normative phase:

- Evaluation/deployment scenarios: D1T1-A1, D1T1-A2, D1T1-B, D1T1-C, D2T2-C

- Device type:

  - Device 1 with RF-ED receiver architecture;

  - Device 2b with RF-ED and IF receiver architectures, SSB

    - No dedicated signal for device 2b (i.e., signal for CFO calibration) for a harmonized air interface design

    - FDMA for Device 2b with IF receiver in R2D

- Device (un)availability: Direction 2 (Direction 1 can be implemented, if needed, without the specification support)

- Positioning (with same similar functionality as Proximity determination Solution 1)

- Following technical design choices are to be determined during the normative phase:

  - Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R

  - Multiple access by TDMA and/or FDMA and/or CDMA for D2R

  - OOK and/or BPSK and/or MSK modulation for D2R

  - Repetition for D2R and R2D

  - Carrier-wave waveform 1 and/or 2

- Timeframe for core part: 12-month duration for RAN2/3/4 (e.g., by RAN#110) with functional freeze in RAN1 by RAN#109 (4 WG meetings)

- Timeframe for performance part (usual): 6 months after core part completion

- TU allocation: 3.5 TUs per RAN1 meeting, 2.5 TUs per meeting in RAN2, 1 TU per meeting in RAN3, and 1.5 TU per meeting in RAN4

# Conclusion

General aspect of A-IoT

Proposal 1: Since the Release 19 RAN A-IoT study item has been completed in WGs (RAN1/2/4), it is proposed a follow-up normative phase should be setup and approved in this RAN#106, also targeting Release 19.

RAN1 recommendations

Observation 1: Inclusion or non-inclusion of some A-IoT design choices or features that require technical justification and/or would have consequence / impact to the performance or efficiency of A-IoT operation will be difficult to make a decision at the RAN plenary level, such as

- Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R

- Multiple access by TDMA and/or FDMA and/or CDMA for D2R

- OOK and/or BPSK and/or MSK modulation for D2R

- Repetition for D2R and R2D

- Carrier-wave waveform 1 and/or 2

Proposal 2: It is proposed the down-selection of the following technical design choices is to be done in the WG(s) during the normative phase

- Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R

- Multiple access by TDMA and/or FDMA and/or CDMA for D2R

- OOK and/or BPSK and/or MSK modulation for D2R

- Repetition for D2R and R2D

- Carrier-wave waveform 1 and/or 2

Use cases, topologies, device types

Observation 2: It is observed the study of connectivity topologies 1 and 2 are carried out in all WGs (including RAN WGs and SA2) and can be considered completion.

- From the point of view of supporting all agreed representative use cases, it is necessary to support both connectivity topologies 1 and 2 in the first release of normative work for A-IoT.

- From the perspective of workload for Topology 2,

  - RAN1 has a minimal amount of work,

  - Workload for higher layer WGs (i.e.., RAN2/3, SA2) is manageable and expected to complete the normative work in time, and

  - RAN4’s workload in defining requirements heavily depends on whether CW is supported or not (e.g., relating to co-existence and CW cancellation aspects).

Observation 3: Based on the analysis in Section 3 of this contribution, it is observed that representative use cases targeted by A-IoT in Release 19 can be sufficiently fulfilled/supported by connectivity topologies 1 and 2 with device 1 and 2b.

Observation 4: To harmonize the air interface design between device 1 and 2b, no dedicated signal for CFO calibration is considered for device 2b.

Proposal 3: It is proposed evaluation scenarios D1T1-A1/A2/B/C and D2T2-C, and device 1 and 2b with RF-ED and IF receiver architectures for inclusion in the first Release of A-IoT normative phase.

Device (un)availability direction 1 and/or 2

Observation 5: Since there are already solutions in Direction 1 that can be employed in a A-IoT deployment for the purpose of energy harvesting in a A-IoT device without any specification impact, it is not necessary to discuss the choice between Direction 1 and/or Direction 2. The discussion should be on the benefits and necessity of specification support for Direction 2.

Observation 6 Based on the analysis and reasonings provided in this Section 4 of this contribution, it is observed a solution in Direction 1 may not be sufficient and efficient for the purpose of device charging, and also ensuring a device will always sustain a round of inventory or command process.

Observation 7: Due to the high power consumption of device 2, a solution in Direction 2 will be necessary to prolong the operating time of device 2 even after the start of an inventory or command process.

Proposal 4: It is proposed that a solution for “device (un)availability Direction 2” should be supported in the first release of A-IoT normative phase.

Positioning and Proximity determination solution 1 and/or 2

Proposal 5: Reader ID based positioning is included in the first normative phase in RAN. Note this can provide similar functionality as proximity determination solution 1. Based on the positioning / proximity solution being considered and discussed in SA2 [5] and RAN2, only “solution 1” agreed in RAN1 is necessary to be supported in the first release of A-IoT normative phase in RAN.

Timeframe for A-IoT normative phase in Rel-19

Proposal 6: For the general scope proposed in Section 3, 4 and 5 of this contribution, and the basic scope / harmonized design for A-IoT, it is proposed the first normative phase should be a 12-month duration (including ASN.1 work), where RAN1 functional freeze should be 1 quarter earlier (as per usual).

- TU allocation should be at least 3.5 TUs per RAN1 meeting, 2.5 TUs per meeting in RAN2, 1 TU per meeting in RAN3, and 1.5 TU per meeting in RAN4.

Package proposal for A-IoT normative phase:

- Evaluation/deployment scenarios: D1T1-A1, D1T1-A2, D1T1-B, D1T1-C, D2T2-C

- Device type:

  - Device 1 with RF-ED receiver architecture;

  - Device 2b with RF-ED and IF receiver architectures, SSB

    - No dedicated signal for device 2b (i.e., signal for CFO calibration) for a harmonized air interface design

    - FDMA for Device 2b with IF receiver in R2D

- Device (un)availability: Direction 2 (Direction 1 can be implemented, if needed, without the specification support)

- Positioning (with same similar functionality as Proximity determination Solution 1)

- Following technical design choices are to be determined during the normative phase:

  - Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R

  - Multiple access by TDMA and/or FDMA and/or CDMA for D2R

  - OOK and/or BPSK and/or MSK modulation for D2R

  - Repetition for D2R and R2D

  - Carrier-wave waveform 1 and/or 2

- Timeframe for core part: 12-month duration for RAN2/3/4 (e.g., by RAN#110) with functional freeze in RAN1 by RAN#109 (4 WG meetings)

- Timeframe for performance part (usual): 6 months after core part completion

- TU allocation: 3.5 TUs per RAN1 meeting, 2.5 TUs per meeting in RAN2, 1 TU per meeting in RAN3, and 1.5 TU per meeting in RAN4

# References

[1] RP-240826 “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”.

[2] RP-242466 TR 38.769 “Study on solutions for ambient IoT (Internet of Things)”, WG endorsed version for RAN#106.

[3] TR 38.848 v18.0.0 “Study on Ambient IoT (Internet of Things) in RAN”.

[4] TR 22.840 v19.0.0 “Study on Ambient power-enabled Internet of Things”.

[5] TR 23.700 v1.2.0 “Study on Architecture support of Ambient power-enabled Internet of Things”.

[6] RP-241082 “Discussion on Rel-19 Ambient IoT study scope”, OPPO, RAN#104

[7] R1-2408148 “Discussion on frame structure and timing aspects of A-IoT communication”, OPPO, RAN1#118-bis