3GPP TSG RAN Meeting #106 	          	      		            	 RP-242808

Madrid, Spain, December 9-12, 2024

Agenda Item:   	10.4.5

Source:   	LG Electronics

Title: 	Discussion on IoT-NTN TDD mode in Rel-19

Document for:   	Discussion and decision

- Introduction

There is a check point marked in yellow below for the results of the study phase on the impact of TDD pattern on the WID of IoT-NTN TDD mode [1].

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. The study and work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band. This work item includes the following objectives: - Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4] - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24 - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

In this contribution, we will provide our view on the completion of the study phase described above and the start of its normative work.

- Discussion

RAN1 has made the following agreements/observations/conclusions during the study phase to analyse the impact of TDD pattern on various aspects including DL synchronization.

Observation RAN1 makes the following observations on downlink synchronization: Case 1: For N=9, D=8: - For NPSS: - For one-shot detection (90ms delay), 9 sources provided simulation results, with a median required SNR of 1.30dB. The link budget margin is (median margin): - 4.21 dB for LEO-1200 (0dBi antenna gain) - -1.89 dB for LEO-600 (-5.5dBi antenna gain). - For two-shot detection (180ms delay), 3 sources provided simulation results, with a median required SNR of -1.20dB. The link budget margin is (median margin): - 6.71dB for LEO 1200 (0dBi antenna gain) - 0.61dB for LEO-600 (-5.5dBi antenna gain). - For four-shot detection (360ms delay), 3 sources provided simulation results, with a median required SNR of -3.80dB. The link budget margin is (median margin): - 9.31dB for LEO-1200 (0dBi antenna gain) - 3.21dB for LEO-600 (-5.5dBi antenna gain). - NOTE: The assumptions for combining for 2-shot and 3-shot detection are different across companies. - For NSSS: - For one-shot detection (180ms delay), 5 sources provided simulation results, with a median required SNR of -2.50dB. The link budget margin is (median margin): - 8.01dB for LEO-1200 (0dBi antenna gain) - 1.91dB for LEO-600 (-5.5dBi antenna gain) - For NPBCH: - For 90ms combining, 4 sources provided simulation results, with a median required SNR of 3.48dB. The link budget margin is (median margin): - 2.03dB for LEO-1200 (0dBi antenna gain) - -4.07dB for LEO-600 (-5.5dBi antenna gain) - For 640ms combining, 6 sources provided results, with a median required SNR of -4.25dB. The link budget margin is (median margin): - 9.76dB for LEO-1200 (0dBi antenna gain) - 3.66dB for LEO-600 (-5.5dBi antenna gain) Case 2: For N=9, D=20: - For NPSS: - For a combining of 2 consecutive NPSS (80ms delay), 1 source provided simulation results, with a median required SNR of -0.8dB. The link budget margin is (median margin): - 6.31dB for LEO-1200 (0dBi antenna gain) - 0.21dB for LEO-600 (-5.5dBi antenna gain) - For NPBCH: - For 640ms combining, 1 source provided simulation results, with a median required SNR of -5.10dB. The link budget margin is (median margin): - 10.61dB for LEO-1200 (0dBi antenna gain) - 4.51dB for LEO-600 (-5.5dBi antenna gain) Case 3: For N=9, D=30: - For NPSS: - For combining of 3 consecutive NPSS (90ms delay), 1 source provided simulation results, with a median required SNR of -2.4 dB. The link budget margin is (median margin): - 7.91dB for LEO-1200 (0dBi antenna gain) - 1.81dB for LEO-600 (-5.5dBi antenna gain) - For NPBCH: - For 640ms combining, 1 source provided simulation results, with a median required SNR of -7.10dB the link budget margin is (median margin): - 12.61dB for LEO-1200 (0dBi antenna gain) - 6.51dB for LEO-600 (-5.5dBi antenna gain) NOTE: Other evaluated scenarios (LEO-600 with 0dBi antenna gain, LEO-1200 with -5.5dBi antenna gain, LEO-800 with 0dBi antenna gain) have margins in between LEO-1200 (0dBi antenna gain) and LEO-600 (-5.5dBi antenna gain) NOTE: Different companies provided input for different cases. For the companies that evaluated multiple cases, the performance (in terms of acquisition delay) of Case 3 and Case 2 is better than the performance of Case 1. Observation Based on simulations submitted to this meeting, in terms of downlink synchronization (NPSS/NSSS/NPBCH), all the following cases meet the link budget requirements for IOT-NTN TDD and are feasible (from the downlink synchronization point of view) from RAN1 perspective: - Case 1: N=9, D=8 - Case 2: N=9, D=20 - Case 3: N=9, D=30 Observation In Rel-19, RAN1 concludes that at least N=9 is feasible from the following points of view: - DL synchronization, based on the link level evaluations submitted at RAN#119 - Coexistence with the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band Agreement Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints. Observation RAN1 concludes that at least D=8 and U=8 is feasible with N=9 from point of view of the design constraints (from earlier agreement) imposed by the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band. Conclusion For N=9, D=8, RAN1 concludes that SIB1-NB reception following current specifications is feasible for some SIB1-NB TBS values, at least when the configured number of repetitions is 16 and when SIB1-NB TBS value <=X bits. - When considering only NPSS/NSSS/NPBCH/SIB1-NB, and if the number of SIB1-NB repetitions is 16: - If additionalTransmissionSIB1 is set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 43.75% in the anchor carrier. - If additionalTransmissionSIB1 is not set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 37.5% in the anchor carrier. - NOTE: In the above calculations, overhead due to transmission of other SIBs has not been included. - The value of X may be further studied during the normative phase. Note: X is a TBS value supported by the specifications for SIB1-NB. Agreement The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options: - Option 1: Always the same across H-SFNs. - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. - Option 1-2: H-SFN duration is unchanged - Option 2: Different across some H-SFNs. - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) - Option 2-2: Total number of H-SFN is changed to be a multiple of 9. Agreement RAN1 to further discuss how to handle the overlap of NPUSCH with non-U NB-IoT subframes and the overlap of NPRACH (including NPRACH occasions) with non-U NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-U NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of U NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-U NB-IoT subframes. - Impact on segmented pre-compensation. Agreement RAN1 to further discuss how to handle the overlap of NPDCCH/NPDSCH (other than the one carrying SIB1-NB) (including e.g. starting point, windows for SI or RAR and other window sizes for DL channels/signals, PO, etc.) with non-D NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-D NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of D NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes. - The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications Agreement The downlink subframes within D=8 are to be down-selected among the following: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) - Option 2: [4 5 6 7 8 9 0 1] (across two consecutive radio frames) - Option 3: [8 9 0 1 2 3 4 5] (across two consecutive radio frames) - Option 4: [9 0 1 2 3 4 5 6] (across two consecutive radio frames)

To be specific, the details of RAN1’s analysis on the impact of TDD pattern are as follows:

- Whether the link budget requirement of DL synchronization (NPSS/NSSS/NPBCH) can be satisfied for a certain TDD pattern?

- Which TDD patterns can coexist with the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band?

- What are the conditions under which SIB1-NB reception is feasible for a specific TDD pattern (including the overhead ratio occupied by NPSS/NSSS/NPBCH/SIB1-NB in that situation)?

- How to determine/define the offset between the 90ms TDD structure and the 10240ms H-SFN?

- How to handle the overlap of NPUSCH/NPRACH with non-U NB-IoT subframes and the overlap of NPDCCH/NPDSCH with non-D NB-IoT subframes?

- Etc.

In summary, we believe that the investigation into the impact of TDD pattern during the study phase was adequately conducted, and there is no problem in proceeding with the normative work of specifying the IoT-NTN TDD mode based on the outcome of the study.

Proposal: RAN1 starts the normative work of specifying the IoT-NTN TDD mode based on the outcome of the study.

- Conclusion

In this contribution, we discussed the completion of study phase of TDD pattern impact and the start of the normative work of specifying the IoT-NTN TDD mode. The following proposal is given:

Proposal: RAN1 starts the normative work of specifying the IoT-NTN TDD mode based on the outcome of the study.

- Reference

[1] RP-242415, “New WID on introduction of IoT-NTN TDD mode,” Moderator (RAN1 Vice-Chair).