3GPP TSG-RAN Meeting #106			RP-242825

Madrid, Spain, December 9-12, 2024

Agenda item:	16

Title:	Discussions on minimum technical performance requirements for IMT-2030 radio interface(s)

Source:	NTT DOCOMO, INC.

Document for:	Discussion

# 1	Introduction

ITU-R Working Party (WP) 5D has sent the liaison statement on the minimum requirements related to technical performance for IMT-2030 radio interface(s) to external organizations including 3GPP. In this contribution, some discussions on how to collaborate with ITU-R WP 5D (e.g., 3GPP work plan) and candidate items for minimum technical performance requirements (TPRs) are provided.

# 2	Discussions

## 2.1	Outline of 3GPP RAN plenary study related to 6G

According to the endorsed RP-240823 in TSG#103, RAN plenary 1st study item on ITU-R IMT-2030 is planned to be approved in Dec. 2024 and completed in June 2025, followed by 2nd study item on 6G general to be approved in Mar. 2025 and completed in June 2026:

- RAN plenary work is split into an ITU focused SI (study item) and General RAN SI - IMT-2030 discussion is expected in RAN from 09/24 to 12/24 - RP SI Rel-20 focusing on ITU– IMT-2030: approval 12/24 until 06/25 - RP SI Rel-20 focusing on 6G General: approval 03/25 (after WS), until 06/26

Accordingly, in RAN#105, we proposed following work split of two RAN plenary SIs [1]:

- RAN plenary work is split into:

  - Study on items to be defined for Technical Performance Requirements of IMT-2030 radio interface

    - Aim to interact with ITU WP5D and send LS to WP5D if necessary

    - Discuss the LS received from WP5D if any, focusing on the discussion of items to be defined for TPR

  - Study on Scenarios and Requirements for Next Generation Access Technologies

    - Aim to develop deployment scenarios and requirements of 6G access technologies, and to provide guidance to the technical work to be performed in RAN WGs.

    - Develop the typical deployment scenarios and specific requirements for 6G access technologies

For the above two SIs, it is unclear whether the study outcome is summarized in a single TR or separate ones, and whether same or different rapporteurs/editors are assigned. In our understanding, single TR would be enough to summarize the outcome of both SIs, and the same rapporteurs/editors can cover the whole studies. In this regard, there would be another possibility that two SIs are merged into SI with two phases as follows:

- 1st Phase (12/24 - 06/25): Focus on the study on items to be defined for Technical Performance Requirements of IMT-2030 radio interface

- 2nd Phase (03/25 - 06/26): Focus on study on 6G general, whose objectives are to be updated based on the progress and companies input on March 2025.

Between the two approaches mentioned above, we don’t have strong preference since those are fundamentally the same, while it would be beneficial to clarify how to proceed for better meeting management.

## 2.2	Timeline for study on minimum technical performance requirements in 3GPP

According to the liaison statement received from ITU-R WP 5D [2], providing the following information is requested.

- Proposed candidate items for minimum technical performance requirements based on M.2160 Usage Scenarios and Capabilities, including necessary background information and justification, which are requested to be provided preferably by WP 5D #48 (4-13 February 2025).

- Proposed associated target values for the above candidate items for the minimum technical performance requirements which are requested to be provided preferably by WP 5D #49 (24 June – 3 July 2025, TBC)

The deadline of each WP 5D meeting for reception of contributions is 12 calendar days (1600 hours UTC) prior to the start of each meeting.

Therefore, in 3GPP RAN perspective, candidate items for minimum TPRs should be studied and agreed within RAN meeting #106 (Dec. 2024) to send reply LS before ITU-R WP 5D #48. Study on the associated target values for the above candidate items for the minimum TPRs should be done in RAN meeting #107 (March 2025), and the reply LS  to WP 5D should be agreed by (12 June 2025), which is in the middle term of RAN meeting #108 (9-13 June 2025). Since 3GPP RAN plenary study on ITU-R IMT-2030 is planned to be approved in Dec. 2024 and completed in June 2025, this is aligned with the above timeline for the coordination with ITU-R WP 5D.

Fig. 1 Timeline for study on minimum TPRs

Proposal 1: Candidate items for minimum TPRs should be studied and agreed within RAN#106 (Dec. 2024).

Proposal 2: Study on the associated target values for the candidate items should be done and liaison statement to WP 5D should be agreed until the middle term of RAN#108 (June 2025).

## 2.3	Candidate items for minimum TPRs

ITU-R WP 5D has discussed the candidate items for minimum TPRs for IMT-2030 radio interface(s) since WP 5D #45 (January 2024)  based on Recommendation ITU-R M.2160 Usage Scenarios and Capabilities [3], to be completed by WP 5D #51 (February 2026) . We think duplicated work between ITU-R WP 5D and 3GPP should be avoided, and following should be considered as fundamental assumption for the 3GPP study:

- The definition of the candidate items which has been used for IMT-2020 should be reused as starting point unless critical issue is identified

- 3GPP spends sufficient time for the discussion on NEW candidate items which has not been used for IMT-2020, taking ITU-R WP 5D work as starting point.

Accordingly, we propose Candidate items for minimum TPRs in the following sub-sections.

Proposal 3: Candidate items for minimum TPRs should be studied based on Recommendation ITU-R M.2160 Usage Scenarios and Capabilities as follows:

### 2.3.1	Peak data rate

Peak data rate is the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the user peak data rate Rp is given by:

Rp = W × SEp	(1)

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands then the total peak data rate is

R=i=1Q  Wi × SEpi 	(2)

where Wi and SEpi (i = 1,…Q) are the component bandwidths and spectral efficiencies respectively.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak data rate are as follows:

–	Downlink peak data rate is TBD Gbit/s.

–	Uplink peak data rate is TBD Gbit/s.

### 2.3.2	User experienced data rate

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

In case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency through equation (3). Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser = W × SEuser 	(3)

In case bandwidth is aggregated across multiple bands (one or more TRxP layers), the user experienced data rate will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The target values for the user experienced data rate are as follows in the TBD test environment(s):

–	Downlink user experienced data rate is TBD Mbit/s.

–	Uplink user experienced data rate is TBD Mbit/s.

### 2.3.3	Spectral efficiency

#### 2.3.3.1	Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is TBD bit/s/Hz.

–	Uplink peak spectral efficiency is TBD bit/s/Hz.

#### 2.3.3.2	Average spectral efficiency

Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Let Ri (T) denote the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to equation (4).

(4)

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for average spectral efficiency are as follows:

–	Downlink average spectral efficiency is TBD bit/s/Hz.

–	Uplink average spectral efficiency is TBD bit/s/Hz.

#### 2.3.3.3	5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

With Ri (Ti) denoting the number of correctly received bits of user i, Ti the active session time for user i and W the channel bandwidth, the (normalized) user throughput of user i, ri, is defined according to equation (5).

(5)

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for 5th percentile user spectral efficiency are as follows:

–	Downlink 5th percentile user spectral efficiency is TBD bit/s/Hz.

–	Uplink 5th percentile user spectral efficiency is TBD bit/s/Hz.

### 2.3.4	Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through equation (6).

Carea = ρ × W × SEavg	(6)

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The target value for Area traffic capacity in downlink is TBD Mbit/s/m2 in the TBD test environment.

### 2.3.5	Connection density

Connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2).

Connection density should be achieved for a limited bandwidth and number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability.

This requirement is defined for the purpose of evaluation in the Massive Communication usage scenario.

The minimum requirement for connection density is TBD devices per km2.

### 2.3.6	Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

The following classes of mobility are defined:

[TBD]

Table 1 defines the mobility classes that shall be supported in the respective test environments.

TABLE 1

Mobility classes

|  | Test environments for TBD |  |  |
| --- | --- | --- | --- |
| Mobility classes supported |  |  |  |

A mobility class is supported if the traffic channel link data rate on the uplink, normalized by bandwidth, is as shown in Table 2. This assumes the user is moving at the maximum speed in that mobility class in each of the test environments.

This requirement is defined for the purpose of evaluation in the Immersive communication and Massive Communication usage scenarios.

TABLE 2

Traffic channel link data rates normalized by bandwidth

| Test environment | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |

#### 2.3.6.1	Mobility interruption time

Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signaling protocol, or other message exchanges between the mobile station and the radio access network, as applicable to the candidate RIT/SRIT.

This requirement is defined for the purpose of evaluation in the TBD usage scenarios.

The minimum requirement for mobility interruption time is TBD ms.

### 2.3.7	Latency

#### 2.3.7.1	User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

This requirement is defined for the purpose of evaluation in the Immersive communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirements for user plane latency are:

–	TBD ms for TBD

assuming unloaded conditions (i.e. a single user) for TBD IP packets), for both downlink and uplink.

#### 2.3.7.2	Control plane latency

Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state).

This requirement is defined for the purpose of evaluation in the Immersive communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for control plane latency is TBD ms. Proponents are encouraged to consider lower control plane latency, e.g. TBD ms.

### 2.3.8	Reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a TBD data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

This requirement is defined for the purpose of evaluation in the Hyper Reliable and Low Latency Communication usage scenario.

The minimum requirement for the reliability is TBD success probability of transmitting a layer 2 PDU (protocol data unit) of TBD bytes within TBD ms in channel quality of coverage edge for the Hyper Reliable and Low Latency Communication test environment, assuming TBD application data .

### 2.3.9	Joint requirement on data rate, latency, reliability and capacity

The joint performance requirement relates to the capability of transmitting a given amount of traffic within a predetermined latency with high success probability. In addition, the capacity is defined as the number of users or connections per cell that can be satisfied with the fulfilled joint requirement of data rate, latency and reliability.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The joint minimum requirement for various test environments is summarized in Table 3.

TABLE 3

Joint requirement on data rate, latency, reliability and capacity

| Test environment | Data rate |  | Latency |  | Reliability |  | Number of satisfied users |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |

### 2.3.10	Coverage

Coverage refers to the ability to provide access to communication services for users in a desired service area. Coverage is defined as the cell edge distance of a single cell.

The RIT/SRIT shall have a function to provide a larger cell coverage in specific area than that supported by a ground IMT base station using the IMT-2020 technologies. This function may include, but not limited to, inherent functionalities within the RIT/SRIT and utilization of higher altitude applications (e.g. UAS and HIBS).

### 2.3.11	Positioning

Positioning is the ability to calculate the approximate position of connected devices. Positioning accuracy is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of a device.

### 2.3.12	Bandwidth

Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the RIT/SRIT is defined for the purpose of IMT-2030 evaluation.

The requirement for bandwidth is TBD.

### 2.3.13	Sensing-related capabilities

Sensing-related capabilities refer to the ability to provide functionalities in the radio interface to support sensing tasks, including passive object presence/absence detection, object classification, movement characterization, object localization, mapping, and composite sensing tasks such as tracking, SLAM, etc.

A sensing task is a processing function ISAC-capable system, which is performed to achieve a certain sensing goal with respect to one or more sensing target(s), based on radio frequency measurements. Performance of a sensing task is characterized by at least one minimum requirement.

#### 2.3.13.1	Detectability (Detection / False alarm probability)

Detection probability, PD, is the probability of correctly detecting the object status (e.g., presence, category, and state) within sensing range, and False alarm probability, PFA, is the associated probability that an object status is detected given that the object is not present within sensing range.

#### 2.3.13.2	Localization accuracy

Localization accuracy refers to the closeness of the measured sensing result of position of the target object to its true position value within sensing range and is the difference between the calculated location and the actual location of an object.

#### 2.3.13.3	Velocity accuracy

Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing target within sensing range.

#### 2.3.13.4	Sensing resolution

Sensing resolution is defined as the minimum difference of the measured value (e.g, range, location) of two target objects, in order to distinguish these two target objects.

### 2.3.14	AI-related capabilities

Applicable AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications. These functionalities include distributed data processing, distributed learning, AI computing, AI model execution and AI model inference, etc.

### 2.3.15	Sustainability / Energy efficiency

Sustainability, or more specifically environmental sustainability, refers to the ability of both the network and devices to minimize greenhouse gas emissions and other environmental impacts throughout their life cycle.

Energy efficiency is a quantifiable metric of sustainability. It refers to the quantity of information bits transmitted or received, per unit of energy consumption (in bit/Joule). Energy efficiency is expected to be improved appropriately with the capacity increase in order to minimize overall power consumption.

Network energy efficiency is the capability of a RIT/SRIT to minimize the radio access network energy consumption in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to minimize the power consumed by the device modem in relation to the traffic characteristics.

Energy efficiency of the network and the device can relate to the support for the following two aspects:

- Efficient data transmission in a loaded case;

- Low energy consumption when there is no data, namely, in an un-loaded case.

Efficient data transmission in a loaded case is demonstrated by the average spectral efficiency.

This requirement is defined for the purpose of evaluation in the TBD usage scenario.

A quantitative requirement on Energy efficiency can be defined in the following

- A quantitative evaluation to demonstrate the level of sleep ratio and long sleep duration supported by the RIT (Analysis; With TPR limits on sleep ratio and sleep duration)

- A quantitative evaluation to demonstrate the power savings of sleep ratio and sleep duration for the RIT using power models (Analysis without user data traffic; With TPR limits on relative energy savings)

- A quantitative evaluation to demonstrate the power savings of sleep ratio and sleep duration for the RIT using power models (Simulation with user data traffic; With TPR limits on relative energy savings)

### 2.3.16	Security and resilience

Security refers to preservation of confidentiality, integrity, and availability of information, such as user data and signaling, and protection of networks, devices and systems against cyberattacks such as hacking, distributed denial of service, man in the middle attacks, etc.

Resilience refers to capabilities of the networks and systems to continue operating correctly during and after a natural or man-made disturbance, such as the loss of primary source of power, etc.  The RIT/SRIT shall have functionalities to continue operations in emergency situations, either within itself or with the assistance of specific reliable applications with minimal ground network infrastructure (e.g. UAS, HIBS and shipboard base stations), etc.

### 2.3.17	Interoperability

Interoperability refers to the radio interface being based on member-inclusivity and transparency, so as to enable functionality(ies) between different entities of the system.

# 3	Conclusion

In this contribution, the following proposals are provided.

Proposal 1: Candidate items for minimum TPRs should be studied and agreed within RAN#106 (Dec. 2024).

Proposal 2: Study on the associated target values for the candidate items should be done and liaison statement to WP 5D should be agreed until the middle term of RAN#108 (June 2025).

Proposal 3: Candidate items for minimum TPRs should be studied based on Recommendation ITU-R M.2160 Usage Scenarios and Capabilities.

# 4	References

[1] RP-241946, Views on 3GPP RAN 6G requirements study, CMCC, NTT DOCOMO

[2] RP-243202, Minimum requirements related to technical performance for IMT-2030 radio interface(s), ITU-R Working Party 5D

[3] Recommendation ITU-R M.2160, Framework and overall objectives of the future development of IMT for 2030 and beyond, ITU-R