3GPP TSG-RAN Meeting #106	RP-242826

# Madrid, Spain, 9 – 12 December 2024

Agenda item:		9.3.1.3

Source:	Xiaomi

Title:	View on Rel-19 AI/ML UE side data collection

Document for:		Discussion and Decision

Introduction

In RAN#105 meeting, regarding UE side data collection, the proposed way forward (as copied below) in RP-242351 [1] was endorsed:

- SA input is required for further decision on which solutions to proceed with.  RAN will consider SA input in making a final decision in RAN#106.

- RAN extends the study item on data collection by one quarter.  Checkpoint in RAN#106 on how to proceed

- Send an LS to SA

- Provide the table and reference to TR 38.843

- Provide requirements agreed in RAN for a standardized solution (if standardized).

- Provide facts in terms of what RAN2 couldn’t reach consensus on.

- Reference to LS for message size R1-2310681 (RAN1 is still discussing)

- The solutions developed need to meet the requirements.  Ask SA to provide input on the solutions 1b, 2, and 3. at the beginning of LS that all solutions in the table are under consideration in RAN.

- Input from SA will be used by RAN in RAN#106 to use in order to make a decision on how to proceed

In this contribution, we discuss how to proceed on UE side data collection based on progress in SA and RAN WGs.

Discussion

After RAN LS RP-242389 [2] was sent to SA, there was following LS exchanges between SA WGs and RAN WGs:

- SA2 LS S2-2411191 asked RAN2 / RAN3 about clarification on some aspects of the requirement terminology, and RAN2 and RAN3 replied in R2-2411152 and R3-247801 (RAN3 does not have enough information to respond on SA2 question), respectively.

- SA5 LS S5-246299 asked RAN2 about clarification on operator control and standardized data, and RAN2 replied in R2-2411114.

- SA3 LS S3-245138 asked RAN about clarification on the RAN security requirement as well as more inputs on type of data to be collected.

Eventually, SA2 sent LS S2-2412726, with the following input:

…SA2 did not reach consensus on the feasibility, for any of the options outlined above, to meet RAN requirements specified in RP-242389 (S2-2411455).

A full evaluation of potential solutions for all UE side data collection options would require further study in SA2, and coordination with SA5, SA3, and RAN2.

It can be seen that SA2, SA3, and SA5 could not provide input on the solutions 1b, 2, and 3 before RAN#106 meeting. Without input from SA WGs, it is difficult for RAN / RAN2 to make decisions on how to proceed on the UE side data collection options.

Observation 1: Inputs from SA2, SA3, and SA5 on the options 1b, 2, and 3 will not be available before RAN#106 meeting.

RAN2 Rel-19 work will be finished at Q3 of 2025. Following are relevant meeting schedules for RAN plenary and RAN2. If SA WGs spend 1 more quarter for further study, then RAN can make decision in RAN#107 meeting, and RAN2 can have 3 meetings for normative work. If SA WGs spend 2 more quarters for further study, then RAN can make decision in RAN#108 meeting, and RAN2 can have only 1 meeting for normative work, which means that UE side data collection work cannot be finished in Rel-19.

| Month | Meeting |
| --- | --- |
| Mar. 2025 | RAN#107 |
| Apr. 2025 | RAN2#129bis |
| May 2025 | RAN2#130 |
| Jun. 2025 | RAN#108 |
| Aug. 2025 | RAN2#131 |
| Sep. 2025 | RAN#109 |

Observation 2: If SA WGs can provide inputs in RAN#107 meeting, Rel-19 normative work on standardized solution for UE side data collection is possible in RAN WGs. If SA WGs can only provide inputs in RAN#108 meeting, Rel-19 normative work on standardized solution for UE side data collection is not possible.

The 4 options studied by RAN2 are in two categories:

- Option 1a, which has no specification impact

- Option 1b, 2, 3, which have specification impact and need input from SA WGs

From above discussion, for Rel-19 AI/ML Air work, if SA WGs can provide inputs in RAN#107 meeting and all relevant SA/RAN/CT WGs can finish normative work in Rel-19, standardized solution is possible. It is quite challenging for SA WGs to finish their evaluation in one quarter. Consequently, it is challenging to have a standardized solution for UE side data collection in Rel-19. UE side data collection is an essential component in LCM, and a solution is needed to make AI/ML Air features work. It is therefore proposed to take option 1a as baseline in Rel-19 for UE side data collection.

Proposal 1: Option 1a (UE collects and transfers training data to OTT server in 3GPP transparent manner) is taken as baseline in Rel-19 for UE side data collection.

Conclusion

In this contribution, we discuss how to proceed on UE side data collection based on progress in SA and RAN TSG/WGs. We have the following observations:

Observation 1: Inputs from SA2, SA3, and SA5 on the options 1b, 2, and 3 will not be available before RAN#106 meeting.

Observation 2: If SA WGs can provide inputs in RAN#107 meeting, Rel-19 normative work on standardized solution for UE side data collection is possible in RAN WGs. If SA WGs can only provide inputs in RAN#108 meeting, Rel-19 normative work on standardized solution for UE side data collection is not possible.

We proposed the following:

Proposal 1: Option 1a (UE collects and transfers training data to OTT server in 3GPP transparent manner) is taken as baseline in Rel-19 for UE side data collection.

- References

[1] RP-242351, RAN2 chair (Interdigital), “Moderator's summary on offline discussion about RAN2 aspects on REL-19 WI NR_AIML_air”

[2] RP-242389, RAN, “LS on AIML for data collection (to: SA, SA2, SA3, SA5; cc: RAN1, RAN2, RAN3)”