3GPP TSG-RAN Meeting #106	RP-242827

# Madrid, Spain, 9 – 12 December 2024

Agenda item:		9.3.2.3

Source:	Xiaomi, Qualcomm Incorporated, Sony, ZTE Corporation

Title:	Extension of XR WID scope for rate control

Document for:		Discussion and Decision

Introduction

For Rel-19 rate control, WID [2] includes the following objective: “Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion”. The WID emphasizes “control signalling over downlink” and “uplink congestion”.

In RAN2#128 meeting, extension of rate control to DL data was discussed, as from meeting notes [1]:

R2-2409637	XR rate control	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6: Rate control can be applicable to UL data as well as to DL data.

Proposal 7: Query indication (similar to Recommended bit rate query) from UE to gNB is supported.

DISUCSSION on P6:

- CATT asks for the motivation. Xiaomi thinks that the other end will be affected, so DL indication is useful.

- LGE thinks the WID is clear that this is out of scope.

- QCM indicates that DL rate control is supported by legacy MAC CE already and it is useful instead of using CN signalling.

- OPPO understand the motivation but it is not in WID.

- ZTE supports downlink. ZTE clarifies that in RAN plenary they thought there is no impact to RAN2.

RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP.

DISCUSSION on P7:

- Ericsson, LGE, OPPO, Nokia thinks the UL MAC CE is not needed. The gNB can know from CN.

- QCM supports P7, it is useful for the UE indicate.

- Vivo thinks that UE can provide assistance on which flow needs rate control.

- Samsung sees some value, UE can share its preference.

- ZTE would like the enhanced feature to be on par with the legacy feature, so prefers to have both DL indication and rate query

- CATT thinks we need to understand in which scenario this rate query is to be applied, e.g. before or after congestion

RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE

FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

In this contribution, we discuss and propose the extension of XR WID scope for rate control.

Discussion

Legacy Recommended bit rate MAC CE is applicable for both DL and UL data, as from TS 38.321:

Figure 6.1.3.20-1: Recommended bit rate MAC CE

-	Uplink/Downlink (UL/DL): This field indicates whether the recommended bit rate or the recommended bit rate query applies to uplink or downlink. The length of the field is 1 bit. The UL/DL field set to 0 indicates downlink. The UL/DL field set to 1 indicates uplink;

For Rel-19 rate control, WID [2] includes the following objective: “Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion”. The WID emphasizes “uplink congestion”. However rate adaptation is a procedure in end-to-end manner. Considering the use case that gNB indicates to the UE to increase data rate when congestion is relieved. In legacy RAN-assisted codec adaptation, the corresponding signalling flow in LTE is documented in SA4 TS 26.114 Figure 10.7-2, as copied below:

Figure 10.7-2 Uplink bitrate increase based on ANBR

In step 4, AN-1 (Access Network) sends to UE-1 ANBR (Access Network Bitrate Recommendation, i.e. Recommended bit rate MAC CE) to increase data rate. With the increase of UL data rate, the communication peer (UE-2) needs to increase the DL data rate accordingly, therefore UE-2 sends ANBRQ (Access Network Bitrate Recommendation Query, i.e. Recommended bit rate query MAC CE) to AN-2, and AN-2 response with ANBR to configure DL data rate.

Similarly, for Rel-19 rate control, from end-to-end perspective, it is also helpful to support rate control signalling for DL data. The stage-3 impact to support DL data can follow legacy Recommended bit rate MAC CE, and is minimal.

Observation 1: From end-to-end perspective, it is helpful to support rate control signalling for DL data.

Observation 2: Additional specification impact to support rate control for DL data on top of UL data is minimal.

Proposal 1: Update the scope of WID for NR Phase 3 XR enhancement to extend rate control to DL data.

In RAN2 discussion, rate control is related to QoS enforcement and interworking with L4S, as from the agenda item description [1]: “Including aspects such as: per DRB or per flow indication (including analysis of the impact on QoS enforcement, interworking with L4S etc.), bit rate values indication enhancements, indication/assistance from UE to gNB etc.” RAN2 has sent a LS to SA4 / RAN3 (cc to SA2) regarding rate control in R2-2411218. It is suggested to update the WID to include a note about potential coordination with SA WGs for rate control.

Proposal 2: Update the WID for NR Phase 3 XR enhancement with a note about coordination with SA WGs for rate control.

An example change to WID is as follows:

-	Specify rate control signalling [RAN2]:

-	Specify in MAC layer XR rate control signalling per QoS flow/per DRB to enable faster source rate adaption to congestion. Rate control signalling is applicable for both UL data and DL data.

NOTE: coordination with SA WGs of the ongoing work as it may relate to their work.

Conclusion

In this contribution, we discuss and propose the extension of XR WID scope for rate control. We have the following observations:

Observation 1: From end-to-end perspective, it is helpful to support rate control signalling for DL data.

Observation 2: Additional specification impact to support rate control for DL data on top of UL data is minimal.

We proposed the following:

Proposal 1: Update the scope of WID for NR Phase 3 XR enhancement to extend rate control to DL data.

Proposal 2: Update the WID for NR Phase 3 XR enhancement with a note about coordination with SA WGs for rate control.

An example change to WID is as follows:

-	Specify rate control signalling [RAN2]:

-	Specify in MAC layer XR rate control signalling per QoS flow/per DRB to enable faster source rate adaption to congestion. Rate control signalling is applicable for both UL data and DL data.

NOTE: coordination with SA WGs of the ongoing work as it may relate to their work.

- References

[1] R2-2410915, Session chair (Huawei), “Report from session on R18 MBS, R18 QoE and R19 XR”

[2] RP-241771, Nokia (Rapporteur), “Revised WID on XR (eXtended Reality) for NR Phase 3”