3GPP TSG RAN Meeting #106						RP-242893

Madrid, Spain, December 9-12th, 2024

Agenda item:	15.2

Source:	Ericsson

Title:	IMS voice support over GEO IoT NTN

Document for:	Discussion

# Background

The ongoing (Rel-20) SA WG1 study on satellite access includes the following objective [1]:

The objectives of this study are the following: […] - to identify the gaps and potential service requirements to support IMS voice calls using GEO satellite access - regulatory requirements if applicable should be considered. […]

In the study phase, the following potential requirements have been identified [2] (see also [3]):

### 5.1.6 Potential New Requirements needed to support the use case #### 5.1.6.1 Potential Service Requirements [PR 5.1.6-001] The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [3]. [PR 5.1.6-002] The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) considering the transmission data rate, latency and packet size. [PR 5.1.6-003] The 5G system with GEO satellite access shall be able to support a codec suitable for the transfer of voice considering the transmission data rate and latency provided by the 5G system with GEO satellite access. #### 5.1.6.2 Potential KPI Requirements Scenario; UE type; Transmission data rate; Call setup time NOTE 1. UL; DL. IMS voice call using GEO; Handheld; [1-3] kbit/s; [1-3] kbit/s; [4-30] s NOTE 2. NOTE 1: call set up time refers to [4]; NOTE 2: the lower bound of 4s originated from the experience in terrestrial VoNR/VoLTE, while the upper bound of the 20s is derived based on the user’s patience suggestions (30s) in [13];.

Note: these requirements have not been consolidated.

In parallel, RAN WGs are already working on enhancing the capabilities of IoT-NTN for emergencies, for example by supporting broadcasting of public warning messaging over IoT-NTN (with NB-IoT):

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

This contribution discusses the required specification work to support IMS voice calls using GEO satellite access, with a focus on RAN considerations.

# Discussion

IoT NTN connectivity was introduced by 3GPP in Rel-17 and enhanced in Rel-18 and the ongoing Rel-19. However, the specification currently excludes real-time services among the features supported by NB-IoT (cf. TS 36.331 Clause 4.1 [7], see Section 2.1 below). From a radio point of view, connectivity with satellites in the GEO orbits presents challenges due to the low supported bitrates and the long propagation delays.

To address this, the following changes to the 3GPP specifications have been discussed [3]:

- Supporting a new voice codec with low rate, adapted to the typical bitrates supported by a GEO IoT NTN connection. This objective has no impact on RAN working groups.

- Potentially, CN signalling optimizations to minimize the overhead. The need for this objective should be determined by SA2. It has no impact on RAN working groups.

We discuss RAN impact in the following section.

## RAN impact

The RAN specifications currently exclude support for real-time services such as voice and emergency services for terrestrial NB-IoT and, consequently, for IoT-NTN. For example, TS 36.331 states [7]:

## 4.1 Introduction […] In particular, the following features are not supported in NB-IoT and corresponding procedures and messages do not apply to the UE in NB-IoT: […] - Real time services (including emergency call); […]

Supporting voice in the RAN requires some changes to the specifications in the access and connection procedures and for scheduling.

For RRC connection establishment, the existing establishment causes for NB-IoT do not include voice or emergency:

EstablishmentCause-NB information element

-- ASN1START

EstablishmentCause-NB-r13 ::=			ENUMERATED {

mt-Access, mo-Signalling, mo-Data, mo-ExceptionData,

delayTolerantAccess-v1330, mt-EDT-v1610, spare2, spare1}

-- ASN1STOP

In contrast, for LTE access the following are supported:

EstablishmentCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1}

Similarly, access control for LTE differentiates between establishment of RRC connection for emergency calls and other cases, whereas NB-IoT does not [7]:

Except for NB-IoT, upon initiation of the procedure, if the UE is connected to EPC, the UE shall: […] 1> else if the UE is establishing the RRC connection for emergency calls: 2> if SystemInformationBlockType2 includes the ac-BarringInfo: 3> if the ac-BarringForEmergency is set to TRUE: […]

Proposal 1: Specify RAN support for voice over IoT-NTN by NB-IoT for GEO satellites, including initial access and RRC connection procedures. If supported by SA2, specify support for emergency too.

For scheduling, we believe that it is important to minimize the DL overhead and maximize the opportunities for UL transmission. We believe that SPS, which is currently not supported for NB-IoT, is an appropriate tool for this purpose. However, we think the focus in Release 20 should be on GEO satellites, avoiding other type of optimizations to address satellite movement, etc.

Proposal 2: Specify RAN support for semi-persistent scheduling for IoT-NTN by NB-IoT. Specification aspects that are not relevant for GEO satellites are not in scope.

## Modes for transfer of user plane data

For NB-IoT, the specifications support transfer of user plane data in two different modes:

- Over CP, using NAS signaling between the UE and the MME (Control Plane CIoT EPS/5GS optimisation or DoNAS)

- Over UP (User Plane CIoT EPS/5GS optimisations)

In our view, the impact of supporting IMS voice on GEO IoT NTN products should be minimized. This would simplify the introduction of the feature. For existing deployments, it is undesirable to have additional requirements on supporting a different mode for conveying UP data. For this reason, IMS voice support should be possible with both alternatives, maintaining feature parity.

Proposal 3: Support IMS voice over GEO IoT NTN  by NB-IoT for transmission of user data over CP (DoNAS) as well as over UP.

## Other impact

NB-IoT NTN is built on terrestrial NB-IoT, which has a reduced set of features compared to LTE or even LTE-M. For example, it lacks mobility support in RRC Connected. Rather than attempting to address these limitations, the work should be restricted to supporting stationary UEs communicating with GEO satellites.

Proposal 4: Specification work in RAN WGs is limited to supporting stationary UEs communicating over GEO satellites.

# Summary

In this contribution, we have discussed the RAN aspects of supporting IMS voice over GEO IoT NTN.

Proposal 1: Specify RAN support for voice over IoT-NTN by NB-IoT for GEO satellites, including initial access and RRC connection procedures. If supported by SA2, specify support for emergency too.

Proposal 2: Specify RAN support for semi-persistent scheduling for IoT-NTN by NB-IoT. Specification aspects that are not relevant for GEO satellites are not in scope.

Proposal 3: Support IMS voice over GEO IoT NTN  by NB-IoT for transmission of user data over CP (DoNAS) as well as over UP.

Proposal 4: Specification work in RAN WGs is limited to supporting stationary UEs communicating over GEO satellites.

# References

- 3GPP SA WG1, SP-240495 – “New SID: Study on satellite access - Phase 4”, Maastricht, Netherlands, 19-22 March 2024.

- TS 22.887, “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on satellite access - Phase 4;” V0.2.0

- Vivo et al, S1-244889, “Consolidation proposal on single use case IMS Voice Call using GEO satellite access”, 3GPP TSG-SA WG1 Meeting #108, Nov. 18 – 22, 2024, Orlando, USA

- Vivo et al., S2-2412197, “Study on IMS voice call using GEO satellite access”

- Vivo et al., S1-241247, “Discussion paper on IMS voice call using GEO satellite access”

- MediaTeK, RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024

- TS 36.331, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification”