3GPP TSG-RAN Meeting #106                                   	RP-242986

# Madrid, Spain, 9th  – 12th Dec 2024

Agenda item:			9.4.1.6

Source:	Eutelsat Group

Title:	Technical aspects for specifying Mobile VSAT served by NGSO

Document for:	Information

# Introduction

The Rel-18 WI NR_NTN_enh added support for Fixed and Mobile VSAT above 10 GHz served by GSO (Geosynchronous Orbit) satellites. For NGSO, only Fixed VSAT was specified due to the expectation that Mobile VSAT to fast-moving NGSO (Non-Geosynchronous Orbit) satellites would be more challenging. Now that the Rel-18 WI has completed, it has become clearer what additional work is required to specify Mobile VSAT served by NGSO satellites. This paper outlines these issues with a view to including Mobile VSAT to NGSO support within Rel-19.

# Rel-18 Background on NTN Uplink Timing

The Rel-18 NR_NTN_enh work item augmented the RRM core requirements for UL timing to accommodate satellite geometry and velocity, which are vastly different from the terrestrial baseline.

For TN, the UL timing is defined in TS 38.133 sub-clause 7.1 as:

Where:

Timing offset between uplink and downlink radio frames at the UE, as defined in clause 4.2 in TS 38.213.

Fixed timing advance offset, as defined in clause 7.1.2 in TS 38.133.

This is a closed loop requirement that assumes the BS is static and the UE velocity is low. Figure 16.14.2.1.1 from [1] illustrates satellite geometry with the two additional open loop uplink timing parameters added.

For NTN, the UL timing includes two additional open loop parameters and is defined in TS 38.133 as:

The uplink frame transmission takes place NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell.

Where:

NTA,adjcommon	Network-controlled timing correction; see clause 4.3.1

NTA,adjUE	UE-derived timing correction; see clause 4.3.1

NTA,adjcommon is a semi-static timing offset to account for the delay on the feeder link between the gateway on the ground and the satellite, which is common to all UE served by that satellite

NTA,adjUE is a UE-specific delay, computed by the UE, which represents the delay on the service link between the satellite and the UE. This is calculated by the UE using the ephemeris information provided by the network in SIB19, the satellite propagator in the UE and the UE’s knowledge of its own position based on GNSS.

Rel-18 defined core requirements for Fixed VSAT and Mobile VSAT served by GSO satellites. The geometry for GSO mean that the required rate of UL timing adjustment due to satellite and VSAT movement is very small.

Rel-18 also defined core requirements for Fixed VSAT to NGSO. In this geometry, the velocity of the satellite relative to the VSAT is large, with orbital speeds in the range of 7 km/s. This is a much more challenging scenario than GSO. The solution to this continual change in relative path length is mitigated by the UE, which uses the satellite ephemeris in SIB19 along with a propagator in the UE, to predict the satellite position relative to the UE position at the point of transmission. Using this information the UE calculates NTA such that the UL transmission meets the core requirements for Te_NTN given in TS 38.133 table 7.1C.2-3.

# Specifying Mobile VSAT Uplink Timing to NGSO

Rel-18 did not define core requirements for Mobile VSAT served by NGSO. This scenario adds one further variable to the currently specified Fixed VSAT served by NGSO, which is the velocity of the VSAT. Although this is a more difficult scenario, the geometry indicates it is a far closer to the Fixed VSAT served by NGSO case than the GSO case. Most of the relative velocity between the NGSO and the VSAT is due to the orbital speed of the NGSO satellite, which is around 25,000 km/h. By contrast, the maximum speed of a Mobile VSAT is expected to be around 1,500 km/h for the airborne platform.

The current specifications for Fixed VSAT served by NGSO allow for the UE to take account of the movement of the satellite. From TS 38.133 7.1C.2.1

When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE shall adjust the timing such that timing error is to within Te_NTN. The reference timing shall be NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc before the downlink timing of the reference cell

It is assumed that the UE component of the reference timing in NTA,adjUE is as per the Fixed VSAT case, i.e. based on static GNSS positioning, and excludes potential movement of the VSAT.

To specify UL timing requirements for Mobile VSAT served by NGSO two modifications to the current specifications need to be considered:

- The definition of the reference timing could include the UE’s own estimate of its position based on its known velocity relative to the satellite

- The definition of the maximum NTA step size may need to be increased as was the case for ATG to allow for the expected timing changes due to higher UE velocity

For the first point, it is already the case that a Mobile VSAT will use knowledge of its own future position to adjust its timing. Whether this is precluded by the current definition of the reference timing is unclear, but it is an opportunity to clarify or explicitly allow the UE to use this knowledge to continuously update its timing. This knowledge, based on the UE’s propagator, is distinct from GNSS positional updates which may contain unwanted jumps that would exceed Te_NTN.

For the second point, it may or may not be necessary to increase the maximum NTA step size (Tq) in Table 7.1C.2.1-1. If the reference timing in TS 38.133 7.1C.2.1 does not allow the UE to update its position due to velocity, then it will be the case that a cumulative timing error will develop in between GNSS or reference timing updates. Were this occurs, the UE would trigger the need to change its uplink timing when the error exceeds Te_NTN. which has a limit of 7.5 * Tc. However, the currently defined maximum step size Tq for changing NTA is limited in table 7.1C.2.1-1 is 5.5 for FR1-NTN and 2.5 for FR2-NTN.

These step sizes translate into UE velocities of around 1114 km/h and 507 km/h at 30 degrees elevation. This step size limitation is problematic since it is well below the expected maximum velocity of airborne platforms.

This issue of a mobile UE has already been addressed in the core requirements for ATG. It can be seen in TS 38.133 Table 7.1D.2.1-1:

In the ATG case, it is not expected the moving UE is updating its position and relative timing based on GNSS and a UE propagator, therefore the changing path length will accumulate until Te_NTN is exceeded and the UE is allowed to adjust NTA to bring it back into the limit.

There may be enough margin using FR1 numerology to support Mobile VSAT served by NGSO using the existing specifications, but for FR2 numerology, two modifications to the current specifications need to be considered:

- Allow the reference timing to include the UE’s own prediction of its position based on its known velocity

- Increase the allowed step size

The first approach would seem the more elegant since it uses the information available to the UE to mitigate the changing position without tripping the Te_NTN threshold. This approach would appear to align with TS 38.213 Section 4.2:

The alternative would be to use the ATG approach, ignore the UE’s knowledge of its own position and increase the allowed step size. This could work but the step size would be approaching the value of Te_NTN causing the UE to jump in and out of compliance with Te_NTN.

# Summary

In summary, the current Rel-18 specifications cover the case of Fixed VSAT to NGSO. The additional path length uncertainty introduced by a Mobile VSAT is very minor compared with the movement of the satellite which is already accounted for in the UE propagator. The current specifications are not explicit in whether the UE can use knowledge of its own position to mitigate the known position change between GNSS or reference timing updates. Two alternative solutions are proposed:

Proposal 1: Allow the Mobile VSAT to use knowledge of its own position to calculate the reference timing

Proposal 2: Increase the size of Tq to allow for up to 1500 km/h Mobile VSAT velocity (airborne case).

The work to update the current specifications to accommodate basic (single Rx) Mobile VSAT served by NGSO appear minor and could be conducted as follows:

Proposal 3: Add support for Mobile VSAT to NGSO to the current Ku Bands WID as proposed in [2]

Proposal 4: Add support for Mobile VSAT served by NGSO to the proposed new WID in [3] which includes the potential for dual Rx to enable lossless handover.

Proposal 5: Do the required changes to support single Rx Mobile VSAT served by NGSO using TEI-19 Work Item.

# References

[1] TS 38.300 v18.3.0 NR and NG-RAN Overall Description

[2] RP-242981 “Revised WID: Introduction of Ku bands for NR NTN”, Intelsat, RAN #106, Madrid, Dec 2024

[3] RP-242663 New WID on Mobile VSAT with NGSO”, Thales, RAN #106, Madrid, Dec 2024