3GPP TSG RAN Meeting #106						                                         RP-242999

Madrid, Spain, December 9-12, 2024

Status Report to TSG

Agenda item:			10.4.2

| WI / SI Name | # Introduction of another IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | IoT_NTN_FDD_S_band |  |  |  |  |
| Unique ID | 1040130 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242062 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 3/2025 | Performance part: 6/2025 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 50% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Hsieh, Daniel |
|  | Company | MediaTek Inc. |
|  | Email | daniel.hsieh@mediatek.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.1.1	RAN4 #112bis, Oct’18:

All the following agreements are based on R4-2417206 “WF on Rel-19 FR1 NTN bands” [5].

Regional Applicability

- Agreement

  - Consider adding a note in clause 5.2 operating band to clarify regional applicability of band n252

General Methodology for UE to UE Coexistence between n252 UL and n2/b2/n25/b25 DL

- Agreement:

  - Define a new more relaxed spurious emission requirement for TN-NTN UE to UE coexistence compared to -50dBm/MHz, that allows the NTN service to work effectively, across the full n252 band, whilst still protecting the TN UE at least in the majority of cases

    - Further discuss the actual value and methodology for the requirement (e.g. corresponding separation assumption)

    - Companies are requested to propose what emission level in dBm/MHz could be achieved for frequencies below 1995 MHz, considering the UE RF implementation, different uplink channel bandwidths and RB allocations

    - Further discuss how to capture the solutions in the specification

TN-NTN Isolation Distance

- Agreement

  - For band n252 UE-to-UE co-existence study, consider a new TN-NTN separation distance lower than 1.5 km and further study the appropriate separation based on coexistence analysis

    - FFS on partial geographic overlap.

UE operating bands

- Agreement

  - Specify UE operating band as follows:

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. |  |  |  |  |  |  |  |

E-UTRA Channel Numbers

- Agreement

  - Specify E-UTRA channel numbers as follows:

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 252 | 100 | 2180 | 228301 | 228301 –<1>- 228500 | 2000 | 261069 | 261069 –<1>- 261268 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

UE TX-RX Separation

- Agreement

  - Consider 180MHz default Tx-Rx separation as starting point.

    - Support of flexible Tx-Rx separation is not precluded based

    - Applicability for other bands should also be considered

UE TX Regulatory

- Agreement

  - Further discuss whether and how NTN antenna gain could be considered when evaluating other IoT-NTN UE TX requirements related to regulatory requirement.

UE Maximum Output Power for Cat M1 and NB1/NB2

- Agreement

  - FFS on the solutions

    - FFS on whether the general solution is needed.

UE Cat-M1 Reference Sensitivity

- Agreement:

  - Agree to either Option 1/Option 2 (they are equal)

- Table: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -102.2 | FDD |

- Table: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -103 | HD-FDD |

UE Cat-NB1/NB2 REFSENS

- Agreement:

  - Agree to Option 1

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

UL Configuration for REFSENS for Cat-M1

- Agreement:

  - Agree to Option 1 as follows

Table 7.3A-3: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 252 | 61 | FDD and HD-FDD |
| 253 | 61 | FDD and HD-FDD |
| 254 | 61 | FDD and HD-FDD |
| 255 | 61 | FDD and HD-FDD |
| 256 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3A-1). |  |  |

In-band blocking for Cat-M1

- Agreement:

  - Agree to Option 1

- Table 7.6A.2-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256, 255, 254, 253, 252 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

Out-of-band blocking for Cat-M1

- Agreement

  - FFS

Out-of-band blocking for Cat-NB1/NB2

- Agreement

  - Further study whether same issue as B256 applies

SAN Operating Bands

- Agreement:

  - Specify SAN operating band for band 252 as follows:

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. |  |  |  |  |  |  |  |

E-UTRA Channel Numbers

- Agreement:

  - Specify SAN E-UTRA Channel Numbers for band 252 as follows:

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 252 | 100 | 2180 | 228301 | 228301 –<1>- 228500 | 2000 | 261069 | 261069 –<1>- 261268 |
| NOTE: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Band grouping for Cat NB1/NB2

- Agreement:

  - Specify the following band grouping for Cat NB1/NB2

Table 3.5.1A-1: Band groups for NB-IoT for satellite access

| Group | E-UTRA FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | NFDD_SAB_A |  |
| B | NFDD_SAB_B |  |
| C | NFDD_SAB_C |  |
| D | NFDD_SAB_D |  |
| E | NFDD_SAB_E |  |
| F | NFDD_SAB_F |  |
| G | NFDD_SAB_G | 253, 254, 255, 256, 252 |
| H | NFDD_SAB_H |  |
| I | NFDD_SAB_I |  |
| J | NFDD_SAB_J |  |
| K | NFDD_SAB_K |  |
| L | NFDD_SAB_L |  |
| M | NFDD_SAB_M |  |
| N | NFDD_SAB_N |  |

Band grouping for Cat M1

- Agreement:

  - Agree to Option 1 as follows:

- Table 3.5.1A-2: Band groups for category M1 for satellite access

| Group | E-UTRA FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | FDD-M1_SAB_A | 253, 255 |
| B | FDD-M1_SAB_B | 254, 256, 252 |
| C | FDD-M1_SAB_C |  |
| D | FDD-M1_SAB_D |  |
| E | FDD-M1_SAB_E |  |
| F | FDD-M1_SAB_F |  |
| G | FDD-M1_SAB_G |  |
| H | FDD-M1_SAB_H |  |
| I | FDD-M1_SAB_I |  |
| J | FDD-M1_SAB_J |  |
| K | FDD-M1_SAB_K |  |
| L | FDD-M1_SAB_L |  |
| M | FDD-M1_SAB_M |  |
| N | FDD-M1_SAB_N |  |

2.4.1.2	RAN4 #113, Nov’22:

All the following agreements are based on R4-2420465 “WF on Rel-19 FR1 NTN bands” [10].

Regional Applicability

Agreement:

  - No note is needed in clause 5.2 operating band of TS 38.108 and TS 38.101-5 to clarify regional applicability of band n252.

TN-NTN Isolation Distance

- Offline Agreement:

  - The TN-NTN coexistence assumption of 1.5 km separation between NTN UE and edge of the TN coverage from TR 38.863 has been used to guarantee the minimal performance on both TN and NTN networks. Nevertheless, this might not represent a typical deployment, and an NTN UE might still be able to operate within this isolation distance

    - Revision of the general TN-NTN coexistence assumptions is not in scope of this WI

UE to UE Coexistence between n252 UL and n2/b2/n25/b25 DL

- Tentative Agreement:

    - Agree to two different additional spurious emission requirements, each associated to an NS value and corresponding A-MPR, for protection of band b25/n25/b2/n2

      - FFS: -30 dBm/MHz for relaxed value.

        - FFS: This emission level applies and the associated A-MPR is allowed whether or not the NS value is signalled. Also, the protection level should a note equivalent to Note 15 in 38.101-1 which says “These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.”

      - FFS: [-40] dBm/MHz for the stricter value based on current b23 specification

    - The requirement is applicable in every country where b25/n25/b2/n2 and n252 are applicable

    - Specify asymmetric channel bandwidths with 5, 10 and [15] MHz UL CHBW for band n252.

Protection of n70 and n66

Agreement:

  - For new S-band n252, the following spurious emission requirements for UE-to-UE coexistence can be specified for addressing the UE-to-UE coexistence with band n70, n66.

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n252 | NR Band n70 | FDL_low | - | FDL_high | NA | NA | X |
|  | NR Band n66 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE X: The co-existence between n252 and band 70 is subject to regional/national regulation. |  |  |  |  |  |  |  |

UE TX-RX Separation

Agreement:

- Specify the variable Tx-Rx frequency separation for Band B252

- Discuss the applicability of variable Tx-Rx frequency separation for other IoT NTN bands under other agendas.

UE Maximum Output Power for Cat M1 and NB1/NB2

Agreement:

  - The MOP of this band will be aligned with the agreements in other Rel-19 NTN related WIs.

2.4.2	Remaining Open issues

- UE RF requirements including UE-coexistence requirements, MOP, A-MPR

- Detailed specification updates

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

RAN4#112bis, Oct’18:

[1] R4-2415812	draftCR to TS36.102 Introduction of IoT-NTN S band	ZTE Corporation, Sanechips

[2] R4-2415813	draftCR to TS36.108 Introduction of IoT-NTN S band	ZTE Corporation, Sanechips

[3] R4-2416008	Discussion on IoT-NTN B252 UE RF requirements	Mediatek India Technology Pvt.

[4] R4-2416375	Introduction of IoT-NTN S-band	ZTECorporation,Sanechips

[5] R4-2417206	WF on NTN and IoT-NTN bands	Inmarsat

RAN4#113, Nov’22:

[6] R4-2418355	draftCR to TS36.102 Introduction of IoT-NTN S band	ZTE Corporation, Sanechips

[7] R4-2418356	draftCR to TS36.108 Introduction of IoT-NTN S band	ZTE Corporation, Sanechips

[8] R4-2418487	Introduction of IoT-NTN S-band	ZTECorporation,Sanechips

[9] R4-2419520	Discussion on IoT-NTN B252 UE RF requirements	Mediatek India Technology Pvt.

[10] R4-2420465	WF on requirements for NR IoT NTN bands	Inmarsat