3GPP TSG RAN Meeting #106                                                        RP-243082

Madrid, Spain, December 9 – 12, 2024

Source:	Indian Institute of Technology (Madras), CEWiT, Tejas Networks, Indian Institute of Technology (Hyderabad), WiSig Networks

Title:	Views on the study of IMT-2030 requirements

Agenda Item:	16 – 6G Aspects

Document for:	Discussion and Decision

Introduction

In RAN #103 [1, RP-240823], the following was agreed regarding the work in RAN plenary towards 6G:

RAN plenary work is split into an ITU focused SI (Study Item) and General RAN SI

- IMT-2030 discussion is expected in RAN from 09/24 to 12/24

- RP SI Rel-20 focusing on ITU– IMT-2030:  approval 12/24 until 06/25

- RP SI Rel-20 focusing on 6G General: approval 03/25 (after WS), until 06/26

In this contribution, we provide our views on the possible objectives of the RAN plenary level study on IMT 2030, along with our initial views on the minimum Technical Performance Requirements (TPRs) and evaluation scenarios to be considered.

IMT 2030 Study Objectives

Current status of work on IMT-2030 in ITU-R WP5D

The IMT-2030 (6G) framework document envisages six usage scenarios compared to the three in IMT-2020 (5G). The usage scenarios can be divided into two subcategories; Part 1 which includes the expansion of IMT-2020 usage scenarios from Enhanced Mobile Broadband to Immersive Communication (IC), Ultra Reliable Low Latency Communication to Hyper Reliable Low Latency Communication (HRLLC) and Massive Machine Type Communication to Massive Communication (MC). Part 2 consists of three new usage scenarios including Ubiquitous Connectivity (UC), Integrated Sensing & Communication (ISAC) and Artificial Intelligence & Communication (AI&C) which are expected to support new services beyond IMT-2020. Figure 1(a) indicates the set of the above usage scenarios as depicted in ITU-R M.2160 [2].

Furthermore, fifteen capabilities were proposed, as shown in Figure 1(b), including nine capabilities enhanced from IMT-2020 and six new capabilities for IMT-2030 to support new usage scenarios and services. These capabilities are being discussed in the form of minimum Technical Performance Requirements (TPR) for IMT-2030 in ITU-R WP5D Sub-Working Group (SWG) Radio Aspects.

- Usage scenarios                                                                               (b) Capabilities

Figure 1 Usage scenarios and Capabilities of IMT-2030

Figure 2 shows the ITU-R WP5D timeline. Meeting #47 concluded in October 2024 where the focus was on the minimum TPR in SWG-Radio Aspects. There was also an initial discussion on evaluation criteria and methodologies in SWG Evaluation. For both TPRs and Evaluation Methodologies, there have been contributions from different countries, alliances, research institutes and companies. These contributions, as well as those from prior meetings are captured in Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ] [3] and Working Document Towards Preliminary Draft New Report ITU-R M.[IMT-2030.EVAL] [4].

Figure 2: ITU-R WP5D timeline

Furthermore, as per the work plan of SWG Radio Aspects [5] and SWG Evaluation [6], the TPR definitions will be stabilized in February 2025 (Meeting #48) whereas the discussions on the TPR values will progress from June 2025 (Meeting #49) till February 2026 (#51). The SWG Evaluation will stabilize its document in February 2026 (Meeting #51) and finalize it in June 2026 (Meeting #52). This indicates the time window to be followed by 3GPP for its deliberations on IMT 2030 study.

Observation 1: The work in WP5D SWG Radio Aspects towards minimum TPR is expected to be completed in February 2026 with two intermediate check points: 1) The February 2025 meeting will stabilize the definition of the TPRs 2) The June 2025 meeting will start the discussions on TPR values.

Observation 2:   The work on WP5D SWG Evaluation, towards guidelines for the procedure, methodology and criteria (technical, spectrum and service) to be used in evaluating the candidate IMT-2030 Radio Interface Technologies (RITs) or Set of RITs (SRITs) is to be stabilized in February 2026 and finalized in June 2026.

RAN Plenary SI Objectives

Based on the progress of the TPR definitions and evaluation criteria in ITU-R WP5D, it is evident that, since part 1 of the IMT-2030 usage scenarios (IC, HRLLC and MC) also have corresponding usage scenarios in IMT-2020 (eMBB, URLLC, mMTC), the same TPR-usage scenario mapping of IMT-2020 can be reused with some modifications as necessary. For example, new test environments such as Indoor Factory-HRLLC and Indoor Factory-MC, should be considered for applicable TPRs.

Part 2 usage scenarios, i.e. UC, ISAC and AI&C may require new TPRs and test environments which need to be discussed further. For example, for Ubiquitous Connectivity, a new test environment is necessary to evaluate new TPRs such as Coverage, and possibly some legacy TPRs such as 5th percentile spectral efficiency and average spectral efficiency. The same is true for ISAC where positioning and sensing related TPRs may need to be newly introduced. AI&C is essential for new services and capabilities over and above existing 5G technologies. This is already being considered in stage 1 discussions in the 3GPP SA1 group. Considering the future commercial impact of AI&C, it is important to study the necessary RAT impact in terms of TPRs and necessary evaluation criteria, to be able to bring a clear distinction of IMT-2030 technology compared to IMT-2020 technology, from a commercial standpoint.

The IMT 2030 SI in RAN should consider the above facts while defining the objectives.

- Proposal 1: RAN study on IMT-2030 should consider the TPR definitions, applicable usage scenarios, values and test environments.

- The study should consider all six usage scenarios - Immersive Communication, Hyper Reliable and Low Latency Communication, Massive Communication, Ubiquitous Connectivity, Integrated Sensing and Communication and AI & Communication.

- The Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ] and the Working Document Towards Preliminary Draft New Report ITU-R M.[IMT-2030.EVAL] from WP5D meeting #47 should be considered as the baseline.

- KPI(s) for all six use cases should be considered across RAN and SA.

Proposal 2: Part 1 of the use cases including IC, HRLLC & MC, can consider IMT-2020 TPRs and evaluation criteria as baseline. The necessary modifications which need to be brought in over the baseline can be studied in the SI.

Proposal 3: Part 2 of the use cases including UC, ISAC and AI&C, should be evaluated for new TPRs and for possible impact on legacy TPRs with new test environments and evaluation criteria.

Views on the IMT 2030 technical performance requirements and evaluation methodology

Our initial views on the TPRs that could possibly be carried forward from IMT-2020 as well as those that are new to IMT-2030 are given in the table below.

| TPRs possibly carried forward from IMT-2020 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TPR | Definition | Our comments on the definition | Applicable Usage Scenarios | Test Environments | Preliminary TPR Values |
| Peak Data Rate | Mostly the same as IMT-2020. | Further study should consider (1) guard times (2) uplink/downlink ratio for TDD and (3) the bands over which the bandwidth is aggregated | Immersive Communication | _ | 1.5-5x |
| User Experienced Data Rate | Same as IMT-2020 | _ | Immersive Communication, Ubiquitous Connectivity | Dense Urban-IC, Rural-UC | 2-3x Note: For Ubiquitous Connectivity, lower values compared to IMT-2020 can be considered. |
| Peak Spectrum Efficiency | Mostly the same as IMT-2020. | Further study should consider (1) guard times (2) uplink/downlink ratio for TDD | Immersive Communication | _ | 1.5-3x |
| Average Spectrum Efficiency | Same as IMT-2020 | _ | Immersive Communication and Ubiquitous Connectivity | Dense Urban-IC, Indoor Hotspot-IC, Rural-IC, Rural-UC | _ |
| 5th Percentile User Spectrum Efficiency | Same as IMT-2020 | _ | Immersive Communication and Ubiquitous Connectivity | Dense Urban-IC, Indoor Hotspot-IC, Rural-IC, Rural-UC | _ |
| Area Traffic Capacity | Same as IMT-2020 | _ | Immersive Communication | Indoor Hotspot-IC | 2-3x |
| Connection Density | Same as IMT-2020 | _ | Massive Communications | Dense Urban-MC, Indoor Factory-MC | 1-10x |
| Mobility | Same as IMT-2020 | Possibly, split vehicular into two ranges | Immersive Communication and Ubiquitous Connectivity | Dense Urban-IC, Indoor Hotspot-IC, Rural-IC, Rural-UC | _ |
| Mobility Interruption Time | Same as IMT-2020 | _ | Immersive Communication and Hyper Reliable Low Latency Communication | _ | 0ms |
| User Plane Latency | Same as IMT-2020 | _ | Immersive Communication and Hyper Reliable Low Latency Communication | _ | 0.5-1 ms |
| Control Plane Latency | Same as IMT-2020 | _ | Immersive Communication and Hyper Reliable Low Latency Communication | _ | 5-20 ms |
| Reliability | Same as IMT-2020 | _ | Hyper Reliable Low Latency Communication | Dense Urban-HRLLC, Indoor Hotspot-HRLLC | 1-10−5 to 1-10−7 |
| Bandwidth | Same as IMT-2020 | _ | _ |  | _ |
| New TPRs for IMT-2030 |  |  |  |  |  |
| TPR | Definition | Possible New Definition | Applicable Usage Scenarios | Test Environments | Preliminary TPR Values |
| Coverage | New to IMT 2030. | It is the ability to provide access to communication services for users in a desired service area. In the context of this capability, coverage is defined as the cell edge distance of a single cell through link budget analysis | Ubiquitous Connectivity | Rural-UC |  |
| Positioning | New to IMT 2030. | It is the ability to calculate the approximate position of connected devices. Positioning accuracy is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of a device. It is characterized by the 90th percentile on the CDF curve of positioning accuracy for both horizontal and vertical directions. | Immersive Communication and Hyper Reliable Low Latency Communication | Dense Urban-IC, Indoor Hotspot-IC, Rural-IC, Indoor Hotspot-HRLLC | 0.5-3 m |
| Sensing-related Capabilities | New to IMT 2030. | It is the ability to provide functionalities in the radio interface including range/velocity/angle estimation, object detection, localization, imaging, mapping, etc. These capabilities could be measured in terms of accuracy, resolution, detection probability, false alarm probability, etc. It is characterized by Sensing Range, Horizontal/Vertical Sensing Accuracy, Velocity Accuracy, Detection Probability and False Alarm Probability. | Integrated Sensing and Communication | Indoor Hotspot-ISAC, Dense Urban-ISAC, Rural-ISAC | _ |
| AI-related Capabilities | New to IMT 2030. | It is the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications. These functionalities include distributed data processing, distributed learning, AI computing, AI model execution and AI model inference, etc. AI enabled applications include at least the following (1) AI-assisted Network where communication aspects of the RIT/SRIT are enhanced by AI frameworks, via AI models on both the mobile station side and the TRxP side, either by replacing or working in conjunction with non-AI frameworks. (2) Network-assisted AI where use cases employing AI frameworks are supported by the RIT/SRIT. | AI and Communication | Dense Urban-IC, Rural-IC, Indoor Hotspot-HRLLC | _ |

Proposal 4: For the definition of Peak Data Rate and Peak Spectral Efficiency, further study should consider (1) guard times (2) uplink/downlink ratio for TDD and (3) the bands over which the bandwidth is aggregated.

Proposal 5: For the Ubiquitous Connectivity usage scenario, the study should consider 5th percentile and average spectral efficiency along with coverage a TPR.

- A new test environment should be defined for Ubiquitous Connectivity which includes at least the Rural geographical area.

Proposal 6: The positioning TPR can be defined as the ability to calculate the approximate position of connected devices. Positioning accuracy is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of a device.

- It is characterized by the 90th percentile on the CDF curve of positioning accuracy for both horizontal and vertical directions.

Proposal 7: Sensing related capabilities can be defined as the ability to provide functionalities in the radio interface including range/velocity/angle estimation, object detection, localization, imaging, mapping, etc.

- These capabilities could be measured in terms of accuracy, resolution, detection probability, false alarm probability, etc. The capabilities could be characterized by Sensing Range, Horizontal/Vertical Sensing Accuracy, Velocity Accuracy, Detection Probability and False Alarm Probability.

Proposal 8: AI-related capabilities can be defined as the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications. These functionalities include distributed data processing, distributed learning, AI computing, AI model execution and AI model inference, etc.

- AI enabled applications include at least the following (1) AI-assisted Network where communication aspects of the RIT/SRIT are enhanced by AI frameworks, via AI models on both the mobile station side and the TRxP side, either by replacing or working in conjunction with non-AI frameworks. (2) Network-assisted AI where use cases employing AI frameworks are supported by the RIT/SRIT.

Conclusions

In this contribution we presented our views on the progress of IMT-2030 TPRs and Evaluation Criteria & Methodologies in ITU WP5D. We also proposed possible objectives for the RAN level SI. Lastly, we presented our views on possible definitions, test environments and preliminary TPR values. We made the following observations and proposals.

Observation 1: The work in WP5D SWG Radio Aspects towards minimum TPR is expected to be completed in February 2026 with two intermediate check points: 1) The February 2025 meeting will stabilize the definition of the TPRs 2) The June 2025 meeting will start the discussions on TPR values.

Observation 2:   The work on WP5D SWG Evaluation, towards guidelines for the procedure, methodology and criteria (technical, spectrum and service) to be used in evaluating the candidate IMT-2030 Radio Interface Technologies (RITs) or Set of RITs (SRITs) is to be stabilized in February 2026 and finalized in June 2026.

- Proposal 1: RAN study on IMT-2030 should consider the TPR definitions, applicable usage scenarios, values and test environments.

- The study should consider all six usage scenarios - Immersive Communication, Hyper Reliable and Low Latency Communication, Massive Communication, Ubiquitous Connectivity, Integrated Sensing and Communication and AI & Communication.

- The Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ] and the Working Document Towards Preliminary Draft New Report ITU-R M.[IMT-2030.EVAL] from WP5D meeting #47 should be considered as the baseline.

- KPI(s) for all six use cases should be considered across RAN and SA.

Proposal 2: Part 1 of the use cases including IC, HRLLC & MC, can consider IMT-2020 TPRs and evaluation criteria as baseline. The necessary modifications which need to be brought in over the baseline can be studied in the SI.

Proposal 3: Part 2 of the use cases including UC, ISAC and AI&C, should be evaluated for new TPRs and for possible impact on legacy TPRs with new test environments and evaluation criteria.

Proposal 4: For the definition of Peak Data Rate and Peak Spectral Efficiency, further study should consider (1) guard times (2) uplink/downlink ratio for TDD and (3) the bands over which the bandwidth is aggregated.

Proposal 5: For the Ubiquitous Connectivity usage scenario, the study should consider 5th percentile and average spectral efficiency along with coverage a TPR.

- A new test environment should be defined for Ubiquitous Connectivity which includes at least the Rural geographical area.

Proposal 6: The positioning TPR can be defined as the ability to calculate the approximate position of connected devices. Positioning accuracy is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of a device.

- It is characterized by the 90th percentile on the CDF curve of positioning accuracy for both horizontal and vertical directions.

Proposal 7: Sensing related capabilities can be defined as the ability to provide functionalities in the radio interface including range/velocity/angle estimation, object detection, localization, imaging, mapping, etc.

- These capabilities could be measured in terms of accuracy, resolution, detection probability, false alarm probability, etc. The capabilities could be characterized by Sensing Range, Horizontal/Vertical Sensing Accuracy, Velocity Accuracy, Detection Probability and False Alarm Probability.

Proposal 8: AI-related capabilities can be defined as the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications. These functionalities include distributed data processing, distributed learning, AI computing, AI model execution and AI model inference, etc.

- AI enabled applications include at least the following (1) AI-assisted Network where communication aspects of the RIT/SRIT are enhanced by AI frameworks, via AI models on both the mobile station side and the TRxP side, either by replacing or working in conjunction with non-AI frameworks. (2) Network-assisted AI where use cases employing AI frameworks are supported by the RIT/SRIT.

References

- RP-240823 Additional Considerations for 6G Timeline.

- ITU-R M.2160 Framework and overall objectives of the future development of IMT for 2030 and beyond

- R23-WP5D-C-0413!H5-N5.07!MSW-E: Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ] - Minimum requirements related to technical performance for IMT-2030 radio interface(s).

- R23-WP5D-C-0413!H5-N5.02!MSW-E: Working Document Towards Preliminary Draft New Report ITU-R M.[IMT-2030.EVAL] - Guidelines for evaluation of radio interface technologies for IMT-2030

- R23-WP5D-C-0413!H2-N2.09.13!MSW-E: Draft detailed work plan for development of the draft new Report ITU-R M.[IMT-2030.TECH PERF REQ]

- R23-WP5D-C-0413!H2-N2.09.12!MSW-E: Detailed work plan for development of the draft new Report ITU-R M.[IMT-2030.EVAL]