3GPP TSG RAN Meeting #106									RP-243095

Madrid, Spain, Dec. 9-12, 2024

Agenda item:	9.3.2.3

Source: 	Huawei, HiSilicon

Title: 	Checkpoint on XR support for multimodality across TSGs

Document for:	Discussion/Decision

- Introduction

The latest WID for Rel-19 XR enhancements is excerpted as follows [1]:

The Rel-19 XR Phase 3 objectives are as follows: - Specify support for multimodality in RAN for UL and DL [RAN3]: NOTE: This is subject to alignment with SA2, e.g., if MMSID is not available from CN, then UE assistance information-based approach as an alternative. RAN#106 to check handling of uplink discard based on SA2/SA4 outputs on whether the corresponding information is available at the UE. NOTE: Review in RAN#106 the conclusions in SA2 side (including MMSID delivery to RAN) to allocation for further TUs in RAN3 from Q1/25 onwards. - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG. - Specify the following user plane enhancements [RAN2]: - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3]: - PDU set based handling; - ECN marking; - Burst Arrival Time reporting, if needed; - PSI Discard coordination, if needed; NOTE: No RAN2 impact from above items. - Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion. - Specify Core requirements related to the above objectives as necessary [RAN4].

Two notes need to be checked in RAN#106, including detailed solutions for the multimodality objective as well as RAN3 TU allocation.

In this contribution, we focus on these checkpoints marked in the Notes within the WID, and provide our views based on latest progresses of both RAN and SA. In addition, we provide an updated WID accordingly.

- Discussion

Aspects of multi-modality

For multi-modality, RAN2 has considered two multi-modality awareness related indications and potential usages:

- MMSID, which may be used by the gNB for joint admission control, or the gNB may consider this information during QoS flow to DRB mapping.

- Synchronization information among multi-modality flow, which may be used for PDU set discard across QoS flows of the same multi-modal service.

As mentioned in the first note in the WID, multi-modality awareness at RAN is subject to alignment with SA2.

In the SA2’s LS reply on multi-modality awareness at RAN [2], SA2 mainly focused on MMSID, and wanted to first understand the usage of MMSID intended by RAN2 as well as RAN3’s and SA4’s assessments of the applicability, before making the final decision of whether to provide such information to RAN. The content of the LS is excerpted as follows:

1. Overall Description: SA2 thanks RAN2 LS on multi-modality awareness at RAN. SA2 would like to inform RAN2 the following: Question: In this meeting, as part of this study, RAN2 has agreed that multi-modality awareness at RAN is supported for both UL and DL. To continue the discussion during the study phase, RAN2 needs to understand whether/what information can be provided from SA2 (e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN). Answer: SA2 specified policy control enhancements to support multi-modal services in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF, please see TS 23.501 clause 5.37.2 for details. Forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts. Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work. Such feedback from RAN2 needs to also be assessed by RAN3 and SA4 to confirm the applicability and usage of this information. 2. Actions: To RAN WG2, SA WG4, RAN WG3. ACTION: SA2 asks RAN2, RAN3 to provide feedback if any, and SA4 and RAN3 to review RAN2 feedback from XR service perspective and NG-RAN protocols impacts perspective, respectively.

In RAN2#127bis, RAN2 replied a LS [3] to further clarify the usage of MMSID and synchronization information.

In addition, in the RAN3 reply LS R3-245682 [4], it is stated that “RAN3 agreed that it is technically feasible to enhance NGAP to provide the MMSID to NG-RAN node.”  Furthermore, in the RAN3 reply LS R3-247874 [5], RAN3 once again confirmed the usage of MMSID as concluded by RAN2, while for synchronization information, RAN3 would like to rely on progress in other WGs.

In the SA4 reply LS [6], for MMSID, SA4’s reply is excerpted as follows:

Answer: From an XR service perspective, it is generally desirable that the gNB should try to grant all associated QoS Flows for the XR service whenever possible. However, not all applications require all QoS flows to be granted. If the gNB cannot grant all associated QoS Flows, only the application layer can determine how to handle the case when the QoS flow for one modality cannot be established e.g. due to RAN resource limitations. For instance, it may be acceptable to an application if the QoS flow for video cannot be established with the requested QoS requirements but it may not be acceptable if the QoS flow for audio cannot be established. The conditions under which particular QoS flows are considered essential for an application to continue are application specific and cannot be determined by 5GS.

SA4 thinks it is generally desirable that the gNB should try to grant all associated QoS Flows for XR service. SA4 just notes that in case the gNB cannot accept all QoS flows, it is the application layer which should determine how to handle such case.

We think SA4’s understanding is in line with the usage of MMSID intended by RAN2. When MMSID for multi-modality flows is provided to RAN, RAN shall try to guarantee the joint admission of those flows. However, if coverage limitations or congestion prevent the gNB from accepting all flows, the application will need to determine the next course of action, e.g. the application layer may decide to terminate the application if only a single flow gets accepted which would be out of control of RAN..

Despite the positive assessment of MSSID’s applicability and technical feasibility provided by RAN3 and SA4, there was still no consensus made for whether to provide MMSID to RAN via CN according to the discussion in SA2#166 in November.

As captured in the WID, another approach for making RAN aware of multi-modal information is the provision of such information by the UE to the gNB. It was agreed in RAN#105 that this UE assistance information based approach should be an alternative if CN cannot provide the information to RAN.  Considering that discussions on this topic got stalled in SA2 for unclear reasons, in order to facilitate RAN discussion on multi-modality awareness, we suggest RAN to adopt UE assistance information-based approach and add RAN2 to the objective as a leading WG.

Proposal 1: RAN to support UE assistance information-based approach for providing the multi-modal information to gNB for both UL and DL. RAN2 is added to the objective as a leading WG.

At the same time, RAN can further check the progress in SA2, and RAN3 can be involved if SA2 agrees to provide MMSID from CN to RAN.

Proposal 2: RAN3 can be involved if/when SA2 agrees to provide MMSID from CN to RAN.

As for PDU set discard, the relevant reply in the SA4 LS [6] is excerpted as follows:

Answer: For DL, from the application perspective, there are well-defined mechanisms to support synchronized playout among different media flows, e.g. RTP timestamps, jitter buffering mechanism in the RTP client. These mechanisms guarantee frame level synchronization but do not provide packet level synchronization among different media flows. There is no evidence to suggest 5GS per packet inter-modal synchronization is needed for XR services, as the very low round trip latency requirements will prevent the system from introducing unacceptable inter-modal skew. Regarding PDU Set discard, it is recommended that the 5GS not jointly discard packets across multi-modal flows. Receiving packets for some modalities is preferred than receiving none at all as this enables the application to use the received modalities to better manage the user experience (e.g., pause the video while rendering received audio packets, or render received video packets while concealing an audio packet drop).

SA4 clarified that application layer has well-defined mechanisms to support synchronized playout among different media flows, and there is no evidence to suggest 5GS per packet synchronization between different modalities is needed for XR services. Hence, SA4 recommended that the 5GS not jointly discard packets across multi-modal flows.

Observation 1: In SA4’s reply LS, it is not recommended that 5GS performs jointly discard packets across multi-modal flows.

Considering SA4’s assessment, we suggest that RAN does not pursue the work of PDU-set based discard by synchronization thresholds between multi-modals in Rel-19 XR. The last sentence in the first note within the WID can be removed.

Proposal 3: Rel-19 XR will not support PDU set discard based on multi-modal synchronization information.

Aspects of RAN3 TU allocation

In this section, we will first review the progress of SA2’s XRM features, and analyze RAN impacts. As captured in the latest TR 23.700-70 [7], SA2 has concluded to support the following features in Rel-19 XRM:

- PDU Set Information marking without PDU Set QoS requirements.

In the case of no DL PDU Set QoS parameters provided by the AF, according to the Protocol Description and PSA UPF supports of PDU Set Information marking, the SMF sends NG-RAN a new indicator indicating the DL PDU Set Information Marking is supported. The SMF activates the PDU Set Information marking based on information from NG-RAN.

The feature will involve RAN3 to work on interaction for indicating the support of DL PDU Set Information Marking between the SMF and NG-RAN.

- AQP enhancements to include PDU set QoS information.

SA2 concluded to enhance AQP to include PDU set QoS information in the QoS profile. This will involve RAN3 work in the WI phase.

- AL-FEC ratio-based enhancements.

The SMF may provide DL FEC ratio to NG-RAN when establishing/modifying a QoS flow. RAN node may use the static FEC ratio information to perform PDU discard during congestion. It should be noted that in the latest RAN2 meeting, it was concluded that in case the information would be provided to RAN, it is up to gNB how/whether to consider it, which means no impacts on RAN2 specifications.

The feature will involve RAN3 to support semi-static FEC ratio delivery from SMF to RAN.

- Dynamic change of DL traffic characteristics information.

Based on instruction from SMF, the UPF identifies and marks the Burst size, based on the input (i.e. Burst size) carried in N6 DL packet header. The UPF sends the Burst size to NG-RAN via DL GTP-U header. Besides, based on the PCC rule and/or local configuration, the SMF instructs the UPF to identify and mark the Time to Next Burst and to further notify NG-RAN on the Time to Next Burst.

The feature will involve RAN3 to add Burst size information and Time to Next Burst in DL GTP-U header.

- Exposure of available data rate.

For GBR QoS flow, based on the instruction of SMF, the NG-RAN reports the available data rate to the UPF via the GTP-U header of UL packets and UPF further exposes the data rate to the AF.

This feature needs RAN3’s further work on the signaling design from SMF to NG-RAN for control plane and adding available data rate information in UL GTP-U header for user plane.

In order to support those SA2’s XR features, we propose to add a general objective to specify RAN impacts resulted by SA2 enhancements in Rel-19 XRM. The stage-3 impacts should be for RAN3 only.

Proposal 4: Add a general objective to support SA2 enhancements in Rel-19 XRM (i.e. PDU Set Information marking without PDU Set QoS requirements, PDU Set QoS parameters in AQP, AL-FEC ratio-based enhancements, Dynamic change of DL traffic characteristics information and Exposure of available data rate).

Accordingly, RAN needs to allocate RAN3 TU for Rel-19 XR from Q1/25 onwards, e.g. 0.5TU per RAN3 meeting.

Proposal 5: RAN to allocate RAN3 TU for Rel-19 XR from Q1/25 onwards, e.g. 0.5TU per RAN3 meeting.

- Conclusion

In this paper, we provide our views on the checkpoints in the XR WID objectives, and make the following observations and proposals:

Observation 1: In SA4’s reply LS, it is not recommended that 5GS performs jointly discard packets across multi-modal flows.

Proposal 1: RAN to support UE assistance information-based approach for providing the multi-modal information in both UL and DL. RAN2 is added to the objective as a leading WG.

Proposal 2: RAN3 can be involved if/when SA2 agrees to provide MMSID from CN to RAN.

Proposal 3: Rel-19 XR will not support PDU set discard based on multi-modal synchronization information.

Proposal 4: Add a general objective to support SA2 enhancements in Rel-19 XRM (i.e. PDU Set Information marking without PDU Set QoS requirements, PDU Set QoS parameters in AQP, AL-FEC ratio-based enhancements, Dynamic change of DL traffic characteristics information and Exposure of available data rate).

Proposal 5: RAN to allocate RAN3 TU for Rel-19 XR from Q1/25 onwards, e.g. 0.5TU per RAN3 meeting.

- Updated WID objectives

### 4.1 Objective of SI or Core part WI or Testing part WI The Rel-19 XR Phase 3 objectives are as follows: - Specify support for multimodality in RAN for UL and DL [RAN2, RAN3]: - Specify UE assistance information-based approach; NOTE: RAN3 can be involved if SA2 agrees that MMSID can be provided from CN. - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG. - Specify the following user plane enhancements [RAN2]: - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3]: - PDU set based handling; - ECN marking; - Burst Arrival Time reporting, if needed; - PSI Discard coordination, if needed; NOTE: No RAN2 impact from above items. - Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion. - Specify RAN impacts for SA2 related enhancements in Rel-19 XRM, including PDU Set Information marking without PDU Set QoS requirements, PDU Set QoS parameters in AQP, AL-FEC ratio-based enhancements, Dynamic change of DL traffic characteristics information and Exposure of available data rate [RAN3]. - Specify Core requirements related to the above objectives as necessary [RAN4].

- References

RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, 3GPP TSG RAN Meeting #105.

S2-2409444, LS reply on multi-modality awareness at RAN, 3GPP TSG-WG SA2 Meeting #164.

R2-2409272, Reply to SA2 LS on multi-modality awareness, 3GPP TSG-RAN WG2 Meeting #127bis.

R3-245682, Reply LS on multi-modality awareness at RAN, 3GPP TSG RAN WG2#128.

R3-247874, Reply LS on multi-modality awareness, 3GPP TSG-RAN WG3 Meeting #126.

S4-242223, LS Reply on multi-modality awareness, 3GPP TSG-WG SA4 Meeting #130.

3GPP TR 23.700-70 V19.0.0, Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2 (Release 19).

R3-247875, Reply LS to SA2 for PDU Set Information Marking Support without QoS parameters, 3GPP TSG-RAN WG3 Meeting #126