3GPP TSG-RAN WG3 Meeting #126	R3-247056

Orlando, US, 18 - 22 Nov, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0163 | rev | 3 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support positioning of L2 UE-to-network remote UE [PosL2RemoteUE] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT, Xiaomi, ZTE, Huawei, Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R2-2313814 CR#0444 to TS 37.355 LPP spec, RAN2 have added indication to LMF that the target UE for positioning is a Layer 2 UE-to-Network Remote UE (i.e., L2 U2N Remote UE). The indication of L2 U2N Remote UE was also added in the Error cause values in LPP. The motivation of adding such indications to LMF is to avoid sending inappopriate assistance data or request inappropriate measurements from a UE that is connected to the network via a relay UE. Since the LMF may also send assistance data and request for positioning measurements via NRPPa, a similar indication that UE is L2 U2N Remote UE is also needed in NRPPa; and a cause value is needed in case the LMF requests for measurement or trigger an operation that is not possible to handle for the remote UE. Also, it has been commented in RAN3#125 that some positioning methods do not require LPP, like UL-TDOA, where LPP “may” be used according to TS 38.305 section 8.13.3.4 step 2. Also, if for some reason the UE changes configuration and becomes a remote UE, at any given time, then an indication in the POSITIONING INFORMATION UPDATE message is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A request indicator remote UE-Indication Request IE is added in the POSITIONING INFORMATION REQUEST message and the corresponding response indicator remote UE-Indication IE in the POSITIONING INFORMATION RESPONSE message to notify the LMF that the UE being configured is a L2 U2N Remote UE. - An indication that UE became remote UE is added in the POSITIONING INFORMATION UPDATE message. Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR does not have impact from protocol point of view. This CR may have an impact from functional point of view because it adds possibility of indicating remote UE type to LMF based on request from LMF. The impact can be considered isolated because it impacts only the Positioning Information Exchange procedure . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When the target device operates as a L2 U2N Remote UE, the LMF may trigger positioning operations that are not possible, resulting in a failed operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.2, 8.2.7.2, 9.1.1.10, 9.1.1.11, 9.1.1.13, 9.3.4, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Resubmission to RAN3#125bis Rev2: Addition of co-signers Rev3: Resubmission to RAN3#126 |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 8.2.6.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN node positioning information for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the NG-RAN node shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account for allocating proper CG-SDT resources when positioning a UE.

If the UE TEG Information Request IE is included in the POSITIONING INFORMATION REQUEST message and set to "onDemand", the NG-RAN node shall, if supported, provide the UE Tx TEG association in the POSITIONING INFORMATION RESPONSE message.

If the UE TEG Information Request IE is set to "periodic", the NG-RAN node shall, if supported, reply with the POSITIONING INFORMATION RESPONSE message without including any UE Tx TEG association in this message. The NG-RAN node shall then take the UE TEG Reporting Periodicity IE into account when configuring the UE’s periodic UE Tx TEG association reporting and initiate the Positioning Information Update procedure for reporting the UE Tx TEG association received from the UE, if any.

If the New NR CGI IE is included in the POSITIONING INFORMATION RESPONSE message, the LMF shall, if supported, consider it as the new cell identity where the UE has currently resumed and take it into account for subsequent positioning procedures.

If the Time Window Information for SRS IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node shall, if supported, configure the UE to start transmitting its UL SRS transmission at the indicated time instance.

If the Positioning Validity Area Cell List IE and the Validity Area Specific SRS Information IE within the Requested SRS Transmission Characteristics IE  are included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account for configuring SRS transmissions for the UE in the indicated validty area, and it shall include the SRS Configuration IE, the SFN Initialisation Time IE and the Positioning Validity Area Cell List IE in the POSITIONING INFORMATION RESPONSE message.

If the Requested SRS Preconfiguration Characteristics List IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node shall, if supported, take this information into account when preconfiguring area specific SRS configuration for the UE, and include the SRS Preconfiguration List IE in the POSITIONING INFORMATION RESPONSE message.

If the Remote UE-Indication Request IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node shall, if supported, include the Remote UE-Indication IE in the POSITIONING INFORMATION RESPONSE message. If the Remote UE-Indication IE is set to "L2 U2N Remote", the LMF shall consider that the configured UE operates as a Layer 2 UE-to-Network Remote UE.

Next change

#### 8.2.7.1	General

The Positioning Information Update procedure is initiated by the NG-RAN node to indicate to the LMF that a change has occurred in the SRS configuration or in the UE Tx TEG association. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: Positioning Information Update procedure, successful operation

The NG-RAN node initiates the procedure by sending a POSITIONING INFORMATION UPDATE message to the LMF. If the SRS Configuration IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the updated SRS Configuration for the UE. If the SFN Initialisation Time IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the SFN Initialisation Time associated to the SRS Configuration.

If the UE Tx TEG Association List IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider it as the UE Tx TEG association for the SRS resources that have changed their TEG association during the latest reporting interval.

If the SRS Transmission Status IE is included in the POSITIONING INFORMATION UPDATE message and set to "stopped", the LMF shall consider that the SRS transmission has stopped. If the SRS Transmission Status IE is set to "preconfigured SRS activated", the LMF shall consider the preconfigured area specific SRS is activated in the current serving cell.

If the Remote UE Status IE is included in the POSITIONING INFORMATION UPDATE message and set to "L2 U2N Remote", the LMF shall consider that the UE is operating as a Layer 2 UE-to-Network Remote UE. If the Remote UE Status IE is set to "no", the LMF shall consider that the UE is not operating as a Layer 2 UE-to-Network Remote UE.

If the New Cell Identity IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider that as the new cell information of the UE.

Next change

#### 9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by the LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| UE Reporting Information | O |  | 9.2.70 |  | YES | ignore |
| UE TEG Information Request | O |  | ENUMERATED(onDemand, periodic, stop, …) |  | YES | ignore |
| UE TEG Reporting Periodicity | C-ifUeTegInfoReqPeriodic |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | YES | reject |
| Time Window Information SRS List | O |  | 9.2.90 |  | YES | reject |
| Requested SRS Preconfiguration Characteristics List | O |  | 9.2.97 |  | YES | ignore |
| Remote UE-Indication Request | O |  | ENUMERATED(true, …) |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifUeTegInfoReqPeriodic | This IE shall be present if the UE TEG Information Request IE is set to the value "periodic". |

#### 9.1.1.11	POSITIONING INFORMATION RESPONSE

This message is sent by the NG-RAN node to provide positioning information.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| UE Tx TEG Association List | O |  | 9.2.78 |  | YES | ignore |
| New NR CGI | O |  | NR CGI 9.2.9 |  | YES | ignore |
| Positioning Validity Area Cell List | O |  | 9.2.93 |  | YES | ignore |
| SRS Preconfiguration List | O |  | 9.2.98 |  | YES | ignore |
| Remote UE-Indication | O |  | ENUMERATED(L2 U2N Remote, …) |  | YES | ignore |

Next change

#### 9.1.1.13	POSITIONING INFORMATION UPDATE

This message is sent by the NG-RAN node to indicate that a change in the SRS configuration or UE Tx TEG association has occurred.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| UE Tx TEG Association List | O |  | 9.2.78 |  | YES | ignore |
| SRS Transmission Status | O |  | ENUMERATED (stopped, ..., preconfigured SRS activated) |  | YES | ignore |
| New Cell Identity | O |  | NR CGI 9.2.9 |  | YES | ignore |
| Remote UE Status | O |  | ENUMERATED(L2 U2N Remote, no,…) |  | YES | ignore |

Next change

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NRPPa

--

-- **************************************************************

NRPPA-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Skipped text unchanged

MeasurementCharacteristicsRequestIndicator,

MeasurementTimeOccasion,

PRSConfigRequestType,

MeasurementAmount,

PreconfigurationResult,

RequestType,

UE-TEG-ReportingPeriodicity,

MeasurementPeriodicityNR-AoA,

SRSTransmissionStatus,

CGI-NR,

TimeWindowInformation-SRS-List,

TimeWindowInformation-Measurement-List,

PosValidityAreaCellList,

SRSReservationType,

RequestedSRSPreconfigurationCharacteristics-List,

SRSPreconfiguration-List,

PosSRSResourceSet-Aggregation-List,

Remote-UE-Indication-Request,

Remote-UE-Indication,

Remote-UE-Status

Skipped text unchanged

id-UE-TEG-Info-Request,

id-MeasurementCharacteristicsRequestIndicator,

id-MeasurementTimeOccasion,

id-PRSConfigRequestType,

id-MeasurementAmount,

id-PreconfigurationResult,

id-RequestType,

id-UE-TEG-ReportingPeriodicity,

id-MeasurementPeriodicityNR-AoA,

id-SRSTransmissionStatus,

id-NewNRCGI,

id-TimeWindowInformation-SRS-List,

id-TimeWindowInformation-Measurement-List,

id-PosValidityAreaCellList,

id-SRSReservationType,

id-NewCellIdentity,

id-RequestedSRSPreconfigurationCharacteristics-List,

id-SRSPreconfiguration-List,

id-SRSInformation,

id-PosSRSResourceSet-Aggregation-List,

id-Remote-UE-Indication-Request,

id-Remote-UE-Indication,

id-Remote-UE-Status

Skipped text unchanged

-- **************************************************************

--

-- POSITIONING INFORMATION REQUEST

--

-- **************************************************************

PositioningInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationRequest-IEs}},

...

}

PositioningInformationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-RequestedSRSTransmissionCharacteristics	CRITICALITY ignore	TYPE RequestedSRSTransmissionCharacteristics	PRESENCE optional	}|

{ ID id-UEReportingInformation					CRITICALITY ignore	TYPE UEReportingInformation							PRESENCE optional	}|

{ ID id-UE-TEG-Info-Request						CRITICALITY ignore	TYPE UE-TEG-Info-Request								PRESENCE optional	}|

{ ID id-UE-TEG-ReportingPeriodicity				CRITICALITY reject	TYPE UE-TEG-ReportingPeriodicity					PRESENCE conditional	}|

-- This IE shall be present if the UE TEG Information Request IE is set to the value "periodic".

{ ID id-TimeWindowInformation-SRS-List			CRITICALITY reject	TYPE TimeWindowInformation-SRS-List				PRESENCE optional	}|

{ ID id-RequestedSRSPreconfigurationCharacteristics-List	CRITICALITY ignore	TYPE RequestedSRSPreconfigurationCharacteristics-List				PRESENCE optional	}|

{ ID id-Remote-UE-Indication-Request			CRITICALITY ignore	TYPE Remote-UE-Indication-Request				PRESENCE optional	},

...

}

-- **************************************************************

--

-- POSITIONING INFORMATION RESPONSE

--

-- **************************************************************

PositioningInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationResponse-IEs}},

...

}

PositioningInformationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration				PRESENCE optional}|

{ ID id-SFNInitialisationTime		CRITICALITY ignore	TYPE RelativeTime1900			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-UETxTEGAssociationList		CRITICALITY ignore	TYPE UETxTEGAssociationList		PRESENCE optional}|

{ ID id-NewNRCGI					CRITICALITY ignore	TYPE CGI-NR							PRESENCE optional}|

{ ID id-PosValidityAreaCellList		CRITICALITY ignore	TYPE PosValidityAreaCellList	PRESENCE optional}|

{ ID id-SRSPreconfiguration-List	CRITICALITY ignore	TYPE SRSPreconfiguration-List	PRESENCE optional}|

{ ID id-Remote-UE-Indication		CRITICALITY ignore	TYPE Remote-UE-Indication			PRESENCE optional},

...

}

Next change

-- **************************************************************

--

#### -- POSITIONING INFORMATION UPDATE

--

-- **************************************************************

PositioningInformationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationUpdate-IEs}},

...

}

PositioningInformationUpdate-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-SFNInitialisationTime		CRITICALITY ignore	TYPE RelativeTime1900			PRESENCE optional}|

{ ID id-UETxTEGAssociationList		CRITICALITY ignore	TYPE UETxTEGAssociationList		PRESENCE optional}|

{ ID id-SRSTransmissionStatus		CRITICALITY ignore	TYPE SRSTransmissionStatus	PRESENCE optional}|

{ ID id-NewCellIdentity				CRITICALITY ignore	TYPE CGI-NR						PRESENCE optional}|

{ ID id-Remote-UE-Status			CRITICALITY ignore	TYPE Remote-UE-Status			PRESENCE optional},

...

}

Next change

-- R

ReferenceSignal ::= CHOICE {

nZP-CSI-RS								NZP-CSI-RS-ResourceID,

sSB										SSB,

sRS										SRSResourceID,

positioningSRS							SRSPosResourceID,

dL-PRS									DL-PRS,

choice-Extension				ProtocolIE-Single-Container {{ReferenceSignal-ExtensionIE }}

}

ReferenceSignal-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

ReferencePoint ::= CHOICE {

relativeCoordinateID			CoordinateID,

referencePointCoordinate		NG-RANAccessPointPosition,

referencePointCoordinateHA		NGRANHighAccuracyAccessPointPosition,

choice-Extension				ProtocolIE-Single-Container { { ReferencePoint-ExtIEs} }

}

ReferencePoint-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

CoordinateID ::= INTEGER (0..511, ...)

RelativeGeodeticLocation ::= SEQUENCE {

milli-Arc-SecondUnits		ENUMERATED {zerodot03, zerodot3, three, ...},	heightUnits					ENUMERATED {mm, cm, m, ...},

deltaLatitude				INTEGER (-1024.. 1023),

deltaLongitude				INTEGER (-1024.. 1023),

deltaHeight					INTEGER (-1024.. 1023),

locationUncertainty			LocationUncertainty,

iE-extensions						ProtocolExtensionContainer {{RelativeGeodeticLocation-ExtIEs }}	OPTIONAL,

...

}

RelativeGeodeticLocation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RelativeCartesianLocation ::= SEQUENCE {

xYZunit						ENUMERATED {mm, cm, dm, ...},

xvalue						INTEGER (-65536..65535),

yvalue						INTEGER (-65536..65535),

zvalue						INTEGER (-32768..32767),

locationUncertainty			LocationUncertainty,

iE-Extensions				ProtocolExtensionContainer { { RelativeCartesianLocation-ExtIEs} } OPTIONAL,

...

}

RelativeCartesianLocation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RelativePathDelay ::= CHOICE {

k0			INTEGER(0..16351),

k1			INTEGER(0..8176),

k2			INTEGER(0..4088),

k3			INTEGER(0..2044),

k4			INTEGER(0..1022),

k5			INTEGER(0..511),

choice-Extension		ProtocolIE-Single-Container { { RelativePathDelay-ExtIEs} }

}

RelativePathDelay-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ID id-ReportingGranularitykminus1AdditionalPath 	CRITICALITY ignore TYPE ReportingGranularitykminus1AdditionalPath PRESENCE mandatory}|

{ID id-ReportingGranularitykminus2AdditionalPath 	CRITICALITY ignore TYPE ReportingGranularitykminus2AdditionalPath PRESENCE mandatory}|

{ID id-ReportingGranularitykminus3AdditionalPath 	CRITICALITY ignore TYPE ReportingGranularitykminus3AdditionalPath PRESENCE mandatory}|
	{ID id-ReportingGranularitykminus4AdditionalPath 	CRITICALITY ignore TYPE ReportingGranularitykminus4AdditionalPath PRESENCE mandatory}|
	{ID id-ReportingGranularitykminus5AdditionalPath 	CRITICALITY ignore TYPE ReportingGranularitykminus5AdditionalPath PRESENCE mandatory}|
	{ID id-ReportingGranularitykminus6AdditionalPath 	CRITICALITY ignore TYPE ReportingGranularitykminus6AdditionalPath PRESENCE mandatory},

...

}

Remote-UE-Indication-Request ::= ENUMERATED {true, ...}

Remote-UE-Indication ::= ENUMERATED {l2-u2n-remote-ue, ...}

Remote-UE-Status ::= ENUMERATED {l2-u2n-remote-ue, no, ...}

Next change

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

Skipped text unchanged

id-ReportingGranularitykminus3									ProtocolIE-ID ::= 143

id-ReportingGranularitykminus4									ProtocolIE-ID ::= 144

id-ReportingGranularitykminus5									ProtocolIE-ID ::= 145

id-ReportingGranularitykminus6									ProtocolIE-ID ::= 146

id-ReportingGranularitykminus1AdditionalPath					ProtocolIE-ID ::= 147

id-ReportingGranularitykminus2AdditionalPath					ProtocolIE-ID ::= 148

id-ReportingGranularitykminus3AdditionalPath					ProtocolIE-ID ::= 149

id-ReportingGranularitykminus4AdditionalPath					ProtocolIE-ID ::= 150

id-ReportingGranularitykminus5AdditionalPath					ProtocolIE-ID ::= 151

id-ReportingGranularitykminus6AdditionalPath					ProtocolIE-ID ::= 152

id-MeasuredResultsAssociatedInfoList							ProtocolIE-ID ::= 153

id-PointA														ProtocolIE-ID ::= 154

id-NR-PCI														ProtocolIE-ID ::= 155

id-SCS-SpecificCarrier											ProtocolIE-ID ::= 156

id-MeasBasedOnAggregatedResources								ProtocolIE-ID ::= 157

id-UEReportingInterval-milliseconds								ProtocolIE-ID ::= 158

id-Remote-UE-Indication-Request									ProtocolIE-ID ::= xxx

id-Remote-UE-Indication											ProtocolIE-ID ::= xxy

id-Remote-UE-Status												ProtocolIE-ID ::= xxz

END

-- ASN1STOP