3GPP TSG RAN Meeting #106	 RP-243111

Madrid, Spain, December 9-12, 2024

Agenda item:		9.1.1

Source:	CATT

Title:	Way forward after the completion of Rel-19 study on Ambient IoT

Document for:	Discussion and Decision

# Introduction

The Study on solutions for Ambient IoT (A-IoT) in NR was completed in RAN based on the SID [1].  The study target to address the system design aspect in support of Ambient IoT technology, suitable which the devices have low complexity and  ultra-low power consumption from a few µw to hundreds of µw for the target applications.   The coverage of A-IoT devices target to within 10-50 m in distance for indoor application as in the study outcome of TR 38.848 [2].  The deployment scenarios focus on deployment scenario 1 and Topology 1 with micro cell direct communication to A-IoT devices and deployment scenario 2 and topology 2 with UE as an indoor intermediate node to inter-connect between macro cell outdoor and A-IoT device.  Traffic types in the study are DO-DTT and DT with focus on indoor inventory and indoor command use cases.   The evaluations in the A-IoT study were based on the design targets of maximum distance in coverage, application latency, link budget and link performance in TR38.848 [2]

The A-IoT study targets to have the common air interface design for different deployment scenarios and topologies for all types of devices.   The study of A-IoT includes the design of the frame structure and timing, the A-IoT device architecture, the device availability/unavailability associated with energy harvesting, the physical layer aspects, such as modulation, waveform, and channel coding for detection and/or error correction, physical channel/signal and physical layer procedures, and the carrier wave characteristics and operations.  The higher layer study of A-IoT includes the simplified RAN architecture in support of A-IoT devices, the signaling procedure of random access and interaction with upper layers for inventory and comments use cases.   The A-IoT study also includes the co-existence study of Ambient IoT and NR/LTE in license spectrum and RF requirements for the transmission/reception from/to the Ambient IoT, which includes the gNB in Topology 1 and intermediate node UE in Topology 2.  The carrier wave interference suppression has been part of A-IoT study for the carrier wave transmission/reception.

The ambient IoT study in RAN is considered complete with a set of conclusions captured in draft TR38.769[3] for endorsement in RAN#106.  The technical techniques had been endorsed with list of techniques recommended for down selection by the RAN WGs for the normative works.   This paper discusses the way forward on identifying the techniques critical for the first phase of the A-IoT normative works and the timeframe of the normative works.

# 2	Discussion

## 2.1	The Conclusion of Ambient IoT study in RAN

The A-IoT study concludes that it is feasible to have a harmonized air interface design for different A-IoT devices to meet the target maximum distance and latency for inventory in different deployment/topology scenario,

The study of A-IoT concludes with three types of A-IoT devices 1, 2a, and 2b with functional blocks of A-IoT device architectures based on RF-ED receiver for devices 1/2a, and also for receiver architectures based on IF-ED or ZIF for device 2b. The reflection amplifier(s) and the support of large frequency shifter have been studied in the A-IoT device architecture for devices 2a/2b.   The A-IoT operation procedure would take into the time interval of device availability/unavailability caused by the energy harvesting.  The study outcome includes two directions in device availability subject to the reader’s ability in awareness of the time interval of device availability/unavailability.

Direction 1:	Reader does not provide information to a device regarding when the device may become available/unavailable.

Direction 2:	Reader can provide information to a device based on which the device may become available/unavailable.

Two different directions of reader awareness of A-IoT device availability/unavailability would have tremendous impact on the normative works and the network deployment of supporting A-IoT feature.   Since the A-IoT operation is at operator’s license spectrum, the A-IoT device could not be charged by the carrier wave to be available indefinitely for reader not to aware the device availability/unavailability as it was one direction the outcome in the study.  The carrier wave for the device harvest could not be transmitted all the time on operator’s license spectrum and not to be used by other NR channels signals.  Thus, the device availability/unavailability should be aware by the reader as part of A-IoT standard operation procedure

Proposal 1: The carrier wave for the device harvest could not be transmitted all the time on operator’s license spectrum and not to be used by other NR channels signals.  Thus, Direction 2 of the device availability/unavailability should be aware by the reader as part of A-IoT standard operation procedure

The PRDCH as the reader to device (R2D) DL physical channel and the PDRCH as the device to reader (D2R) UL physical channel are identified as the physical channels carrying the information for the A-IoT communication during the A-IoT study.  The preamble was prepended to the DL PRDCH and UL PDRCH for the purposes of timing acquisition/tracking, frequency synchronization, channel estimation and/or interference estimation.  The DL preamble includes the start indicator and clock acquisition part (CAP).  The start indicator is designed to allow the A-IoT devices to detect the start of the DL PRDCH transmission.

Proposal 2:  The preamble designs prepended to PRDCH and PDRCH, including the length and the pattern need to further investigate with a concluding solution to meet the target performance of R2D and D2R links respective.

The OOK modulation with OFDM waveform (OOK-1 and OOK-4) was assumed in the study of the R2D transmission.  For D2R modulation, OOK, BPSK, and MSK are considered as the candidates for D2R transmission in the study for down selection.    The channel coding, including the line code, TBCC forward error correction code, CRC-6 and CRC-16 error detection, bit/chip/block level repetition, had been studied for increasing the reliability and extending the coverage of A-IoT communication.  The repetition could also be used for the small frequency shift for frequency domain multiplexing.

In the conclusion of RAN1 study of A-IoT, the fundamental techniques are recommended for the A-IoT nominative work from the beginning:

-	PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R

-	Modulations of OOK-1 and OOK-4 in R2D

-	Multiplexing in time domain for R2D

-	Convolutional FEC in D2R and no FEC in R2D

-	R2D and D2R signal(s)

Proposal 3:  The physical channels of PRDCH for DL and PDRCH for UL with preamble prepended before the physical should be included in the A-IoT normative work.

Proposal 4:  The waveform and modulation of R2D transmission are the OFDM waveform with OOK-1 and OOK-4 modulation.

Proposal 5:  The CRC error detection should be included in the PDRCH.

Proposal 6:  The PDRCH should include the TBCC for error correction and CRC for error detection of UL D2R information

The waveform of R2D signals is OFDM-based waveform with CP.  There are two methods in CP handling studied in the A-IoT study on the CP impact on R2D timing acquisition, and decoding & performance of PRDCH, reader and device implementation complexities, interference between R2D and NR DL/UL if in the same NR band, spectrum efficiency.

Method Type 1:	Removal of CP at device without specified transmit-side.

Method Type 2:	Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n.

The A-IoT device implementation complexity is the most critical aspect in the evaluation of the CP handling solution since A-IoT device is considered as extreme-low complexity and low power consumption.   The CP handling with CP removal at the receiver required precise timing synchronization with low tolerance in timing drift and samples counting of each OFDM symbol and the associated CP length.  Thus, it might not be feasible for the low complexity passive A-IoT devices 1/2a and even active device 2b.  The chip duration of PDRCH would depend on the solution of CP handling.  The solution of CP handling needs to be determined before any R2D physical channel/signal design.

Proposal 7:  For CP handling, the A-IoT device implementation complexity is the most critical aspect in the evaluation of the CP handling solution since A-IoT device is considered as extreme-low complexity and low power consumption.  The solution of CP handling needs to be determined before any R2D physical channel/signal design

The frame structure of PRDCH was studied with outcome  the beginning immediately after the R2D preamble and two alternatives on deriving the end of PRDCH transmission as follows,

-	Option 1: TBS information via implicit/explicit L1 R2D control information.

-	Option 2: Postamble at the end of PRDCH.

The NR system is designed to have signal processing and control information at fixed transmission time interval (TTI) as the multiple or sub-multiple of base unit (e.g., slot).   The gNB and UE implementation of NR would have the controller configuring the transmitter and receiver based on the transmission time interval.  The transmission time interval along with the TBS of transport channel are the basic functional blocks in 3GPP 3G/4G/5G air interface for the resource allocation for the operation in different air technologies.  The determination of the end of the PRDCH should take into consideration of the gNB and UE implementation for both NR and A-IoT.

Proposal 8:  The determination of the end of the PRDCH should take into consideration of the gNB and UE implementation for both NR and A-IoT.

The maximum data size based on the outcome of RAN Ambient IoT study is around 1000 bits [1].  The TBS size had been discussed in RAN1 and RAN2 in support of 1000 bits with  the functionality of segmentation. It is understood that segmentation and reassembly would add complexity. Therefore, the simplified segmentation in the A-IoT MAC layer is studied, considering following studies in physical layer:

-	In both R2D and D2R directions, there is no lower bound on the minimum TB size, and a maximum TB size of around 1000 bits can be supported.

-	How large TB that can be transported at a given time depends on target coverage/data rate, energy consumption/device availability, etc.

The TBS size of around 1000 bits is possible with the potential degradation of detection and demodulation performance due to large timing drift and frequency wonder caused by large frequency error from the A-IoT device oscillator.  In order to achieve the same detection performance with higher SINR, the coverage area of the A-IoT devices would be reduced.   The SDU segmentation solution would mitigate the detection performance degradation issues with maximum TBS size less than 1000 bits.

Proposal 9:  The determination of the TBS size should consider the detection performance with the potential solution of SDU segmentation of large packet size around 1000 bits.

The physical layer procedure associated with inventory and comments use cases were addressed by the study of random access procedures for DO-DTT type of traffic.  TDMA/FDMA/CDMA multiple access for the A-IoT device random access procedure have been studied for the latency of inventory use cases.  The contention-based random access procedure in RAN2 A-IoT study for Inventory use case also includes two recommended solutions of solution 1 A-IoT Msg1 without data and solution 3 of unified solution supporting both Solution 1 and 2.

-	Solution 1: A-IoT Msg1 without data

-	Solution 2: A-IoT Msg1 with data

-	Solution 3: A-IoT Msg1 optionally includes data (unified solution supporting both Solution 1 and 2)

The details of the solution 1 and 3 would need further investigation with the consideration of A-IoT complexity and multiple access procedure

Proposal 10: The details of the solution 1 and 3 of random access procedure for Inventory use case would need further investigation with the consideration of A-IoT complexity and multiple access procedure.

The time interval between a R2D transmission with the control information and resource allocation and the subsequent D2R transmission was studied with two options as follows,

Option 1:	Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max].

Option 2:	The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ≥ TR2D_min.

There are several variants for both option 1 and option 2 of transmission timing.  The determination of the solution would have the A-IoT operation procedure and the device implementation.  Thus, it is essential to have one solution as the general framework for the A-IoT normative work.

Proposal 11:  It is essential to have one solution of the time interval between the R2D transmission and the subsequence D2R transmission included in the general framework of A-IoT operation.

Carrier-wave waveforms and their characteristics have been identified along with the cases for the deployment of  CW transmission in the spectrum. Two CW waveforms were studied: they are a single-tone or a dual-tone  unmodulated sinusoid. The characteristics of carrier wave were studied in the perspective of D2R backscattering reception performance and CW interference suppression at the reader, spectrum utilization of the backscattered signal, and relative complexity of CW generation.  The feasibility study of CW interference cancellation has been studied in RAN4 with the scheme divided into 3 stages sequentially:

- Spatial isolation for separate TX/RX antenna configuration;

- Analog cancellation from antenna to ADC

- Digital cancellation at baseband.

The transmission spectrum of the unmodulated CW and the backscattered sinusoidal wave needs to take into the consideration of the A-IoT device large frequency shift capability, the regulatory requirement and constraints of spectrum for the DL/UL operation, and the interference management in the deployment.

Proposal 12:  The transmission spectrum of the unmodulated CW and the backscattered sinusoidal wave needs to take into the consideration of the A-IoT device large frequency shift capability, the regulatory requirement and constraints of spectrum for the DL/UL operation, and the interference management in the deployment.

In RAN1 A-IoT study, several aspects may be considered for inclusion or non-inclusion in a given first or subsequent Release of normative works as follows,

-	Device 1 and/or Device 2a (with or without large frequency shift) and/or Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture)

-	D1T1 and/or D2T2

-	Scenario A1 and/or A2 and/or B and/or C

-	Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R

-	Multiple access by TDMA and/or FDMA and/or CDMA for D2R; FDMA for device 2b (IF-ED/ZIF receiver) in R2D

-	OOK and/or BPSK and/or MSK modulation for D2R

-	Carrier-wave waveform 1 and/or 2, and transmission case(s)

-	Device (un)availability direction 1 and/or 2

-	Proximity determination solution 1 and/or 2

The inclusion of the selected technique in a given release needs to be within the A-IoT framework for the operation with targets of common solutions for all devices 1/2a/2b.   The general solution of inclusion should consider the co-existence with NR network operation in a NR operation spectrum and forward compatibility to have common solution for devices even only selected device types would be included in a given release.

Proposal 13:  The inclusion of the selected technique in a given release needs to be within the A-IoT framework for the operation with targets of common solutions for all devices 1/2a/2b.   The general solution of inclusion should consider the co-existence with NR network operation in a NR operation spectrum and forward compatibility to have common solution for devices even only selected device types would be included in a given release

The study of Topology 1 and Topology 2 has concluded to be feasible in the area of general AS procedures, A-IoT paging, A-IoT random access, A-IoT data transmission and Topology 2 resource allocation.  RAN2 has recommended the following features for the normative phase in the A-IoT MAC layer:

-	A-IoT paging

-	A-IoT random access:

-	For contention-based random access, further down-selection between Solution 1 (3step-only) and Solution 3 (unified solution) in sub-clause 6.3.4.

-	A-IoT data transmission

For Topology 2, at least the following aspects are considered:

-	Forwarding of A-IoT upper layer data and A-IoT radio resource management.

Proposal 14:  Topology 1 and Topology 2 are considered feasible for the deployment scenario and topologies included in the normative works.

## Way forward after the Conclusion of Ambient IoT study

The A-IoT study is considered as complete within the 12 months schedule from the RAN working groups.  There are aspects with clear conclusion to be included in the normative work recommended from working groups as the frameworks of the NR system design in support of the A-IoT works.

- Physical layer –

  - PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R

  - Modulations of OOK-1 and OOK-4 in R2D

  - Multiplexing in time domain for R2D

  - Convolutional FEC in D2R and no FEC in R2D

  - R2D and D2R signal(s)

- MAC layer

  - A-IoT paging

  - A-IoT random access

  - A-IoT data transmission

Several aspects and techniques would be selected included the normative works in different phases.   However, the A-IoT framework should be set as the foundation to include the generic functions and techniques in order to extend the feature with normative works in one or more NR releases.   There are several aspects with different alternatives/options/methods to be selected from would have deterministic impact on the NR framework in support of A-IoT feature as follows,

- The frame structure and the end of PRDCH/PDRCH

- The CP handling of PRDCH and the associated chip duration for PRDCH

- The timing determination between DL R2D transmission and UL D2R transmission

- The random access procedures

Since the selection of this functions would have definite effects to the framework of the A-IoT system design for the first release and the subsequent releases.   It is important to define a solid framework of the A-IoT system design in order to allow the extension to the support of most or all A-IoT use cases within the framework.   It would very challenge to be selected during the draft of normative works without technical discussions.   The last 2 quarters of Rel-19 in the 1st and 2nd quarters of 2025 could have alternatives as follows,

- Continue A-IoT study on those critical issues in the framework of the A-IoT system design

- Define a simple Rel-19 work item with a solid framework for the A-IoT system design with forward compatibility to those functions for the future release.

Proposal 15:  It is import to have a solid framework of the A-IoT system design for the first release and subsequent releases.  Two alternatives for the remaining 2 quarters in Rel-19 are as follows,

- Continue A-IoT study on those critical issues in the framework of the A-IoT system design

- Define a simple Rel-19 work item with a solid framework for the A-IoT system design with forward compatibility to those functions for the future release.

If the normative works would be set for Rel-19 A-IoT, the objectives should be supported as follows,

- Specify the NR functional enhancements for A-IoT deployments indoor with Topology 1 with gNB reader and Topology 2 with UE reader

- Specify the required NR frameworks in the support of A-IoT operations of devices 1/2a/2b

  - Specify the physical channels of DL PRDCH and UL PDRCH with preamble signals prepended to the physical channels to carry the DL and UL information respectively.

    - For PRDCH

      - Specify the OOK-4 modulation with OFDM waveform with M less than 16 for PRDCH

        - CP handling of OFDM waveform does not require A-IoT device for CP removal  and to ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n.

      - Specify the Manchester coding for line code

      - Specify CRC-6 and CRC-16

      - Specify the Control information included in the PRDCH

    - For PDRCH

      - Specify the MSK waveform for the PDRCH

      - Specify the chip duration(s) for PDRCH

      - Specify the rate 1/3 and constraint length 7 TBCC code with polynomial from TS36.212

      - No line code for PDRCH

      - Specify CRC-6 and CRC-16

  - Specify the TBS size table for PRDCH and PDRCH

  - Specify the MAC procedures for the A-IoT device operation

    - Paging

    - 3-step random access procedure for both TDMA and FDMA multiple access.

    - Segmentation of large packet size

  - Specify the A-IoT resource allocation by higher layer signaling and dynamic signaling for UE reader in Topology 2

# 3	Conclusions

We have discussed the completion of the A-IoT study in RAN working groups and the potential way forward for the remaining time of Rel-19 as follows,

- Proposal 1: The carrier wave for the device harvest could not be transmitted all the time on operator’s license spectrum and not to be used by other NR channels signals.  Thus, Direction 2 of the device availability/unavailability should be aware by the reader as part of A-IoT standard operation procedure

- Proposal 2:  The preamble designs prepended to PRDCH and PDRCH, including the length and the pattern need to further investigate with a concluding solution to meet the target performance of R2D and D2R links respective.

- Proposal 3:  The physical channels of PRDCH for DL and PDRCH for UL with preamble prepended before the physical should be included in the A-IoT normative work.

- Proposal 4:  The waveform and modulation of R2D transmission are the OFDM waveform with OOK-1 and OOK-4 modulation.

- Proposal 5:  The CRC error detection should be included in the PDRCH.

- Proposal 6:  The PDRCH should include the TBCC for error correction and CRC for error detection of UL D2R information

- Proposal 7:  For CP handling, the A-IoT device implementation complexity is the most critical aspect in the evaluation of the CP handling solution since A-IoT device is considered as extreme-low complexity and low power consumption.  The solution of CP handling needs to be determined before any R2D physical channel/signal design

- Proposal 8:  The determination of the end of the PRDCH should take into consideration of the gNB and UE implementation for both NR and A-IoT.

- Proposal 9:  The determination of the TBS size should consider the detection performance with the potential solution of SDU segmentation of large packet size around 1000 bits.

- Proposal 10: The details of the solution 1 and 3 of random access procedure for Inventory use case would need further investigation with the consideration of A-IoT complexity and multiple access procedure.

- Proposal 11:  It is essential to have one solution of the time interval between the R2D transmission and the subsequence D2R transmission included in the general framework of A-IoT operation.

- Proposal 12:  The transmission spectrum of the unmodulated CW and the backscattered sinusoidal wave needs to take into the consideration of the A-IoT device large frequency shift capability, the regulatory requirement and constraints of spectrum for the DL/UL operation, and the interference management in the deployment.

- Proposal 13:  The inclusion of the selected technique in a given release needs to be within the A-IoT framework for the operation with targets of common solutions for all devices 1/2a/2b.   The general solution of inclusion should consider the co-existence with NR network operation in a NR operation spectrum and forward compatibility to have common solution for devices even only selected device types would be included in a given release

- Proposal 14:  Topology 1 and Topology 2 are considered feasible for the deployment scenario and topologies included in the normative works.

- Proposal 15:  It is import to have a solid framework of the A-IoT system design for the first release and subsequent releases.  Two alternatives for the remaining 2 quarters in Rel-19 are as follows,

  - Continue A-IoT study on those critical issues in the framework of the A-IoT system design

  - Define a simple Rel-19 work item with a solid framework for the A-IoT system design with forward compatibility to those functions for the future release.

If the normative works would be set for Rel-19 A-IoT, the objectives should be supported as follows,

- Specify the NR functional enhancements for A-IoT deployments indoor with Topology 1 with gNB reader and Topology 2 with UE reader

- Specify the required NR frameworks in the support of A-IoT operations of devices 1/2a/2b

  - Specify the physical channels of DL PRDCH and UL PDRCH with preamble signals prepended to the physical channels to carry the DL and UL information respectively.

    - For PRDCH

      - Specify the OOK-4 modulation with OFDM waveform with M less than 16 for PRDCH

        - CP handling of OFDM waveform does not require A-IoT device for CP removal  and to ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n.

      - Specify the Manchester coding for line code

      - Specify CRC-6 and CRC-16

      - Specify the Control information included in the PRDCH

    - For PDRCH

      - Specify the MSK waveform for the PDRCH

      - Specify the chip duration(s) for PDRCH

      - Specify the rate 1/3 and constraint length 7 TBCC code with polynomial from TS36.212

      - No line code for PDRCH

      - Specify CRC-6 and CRC-16

  - Specify the TBS size table for PRDCH and PDRCH

  - Specify the MAC procedures for the A-IoT device operation

    - Paging

    - 3-step random access procedure for both TDMA and FDMA multiple access.

    - Segmentation of large packet size

  - Specify the A-IoT resource allocation by higher layer signaling and dynamic signaling for UE reader in Topology 2

# References

RP-240826: New SID (revised): Study on solutions for Ambient IoT (Internet of Things) in NR

TR38.848v18.0.0: Study on Ambient IoT (Internet of Things) in RAN

TR38.7.6.9v2.0.0: Study on solutions for ambient IoT (Internet of Things )