3GPP TSG RAN meeting #106                                                                                     RP-243113

Madrid, Spain, December 9-12, 2024

Source:	CATT, CSCN, CAICT, Baicells, HONOR, Lenovo, OPPO, Spreadtrum, TCL, xiaomi

Title:	SSB periodicity extension on R19 NR NTN

Agenda Item:	9.3.2.2

Document for:	Discussion and decision

Introduction

- According to WID [1], RAN1 needs to report a list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any).

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

- In this contribution, we provide our views on SSB periodicity extension, which is part of the NTN DL coverage enhancements at both system level and link level.

Discussion

Motivation of SSB periodicity extension

In realistic deployment, considering satellite weight and cost, active beams are often limited. For one satellite, in total there are more than 1000 beam footprints. Due to the limitation of beam number, the coverage ratio is small and there are large coverage holes. In order to improve coverage ratio, one effective method is to enable beam hopping to reduce the coverage holes. Based on RAN1 study, beam hopping requires SSB periodicity extension as illustrated in Figure 1.

Figure 1: SSB periodicity extension to support beam hopping

As shown in the table 1, the coverage ratio can approach 100% if using more than 320ms SSB period configuration and 16 active beams based on RAN1 assumption [2].

Table 1: coverage ratio in different SSB periodicities

| SSB Period | 20ms | 40ms | 80ms | 160ms | 320ms | 640ms |
| --- | --- | --- | --- | --- | --- | --- |
| N1 (beam footprints are in state “off”) | 993 | 929 | 801 | 545 | 33 | 0 |
| N2 (beam footprints are in state “common messages only”) | 0 | 0 | 56 | 312 | 824 | 761 |
| N3 (beam footprints are in state “active traffic”) | 64 | 128 | 200 | 200 | 200 | 296 |
| Coverage ratio (N2+N3/ total beam footprints) | 6.05% | 12.11% | 24.22% | 48.44% | 96.88% | 100% |

Observation 1: Adjusting SSB periodicity is able to meet different levels of coverage ratio requirement, with 96.88% coverage ratio at 320ms period and 100% coverage ratio at 640ms.

Observation 2: In order to meet the coverage requirements of satellite operators to achieve full coverage, extending SSB periodicity with 320ms or above is critical.

RAN WGs progress on SSB periodicity extension

In RAN1, SSB periodicity extension has been investigated in past several meetings and the following observations and agreements were reached.

Observation Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km: - For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied. - Note: coverage ratio is N2+N3/ total beam footprints - Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs. Observation Based on the results of DL coverage evaluation at system level collected from different sources, it is observed that extending the default value of SSB periodicity (different from 20ms) in NTN with LEO600km satellite parameter sets where the beam footprint diameter is 50 km, is beneficial in terms of reduction of common control channel overhead, when targeting a full coverage of 1058 beam footprints: - With Set 1-1 FR1 and Set 1-3 FR1, the common messages (SSB, SIB1) overhead is around 40% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead ratio could be reduced to less than 14% when 160ms SSB/SIB1 periodicity is used. - With Set 1-2 FR1, the common message (SSB, SIB1) overhead is greater than 100% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead could be reduced to around 25.8% when 640ms SSB/SIB1 periodicity is used. - Note: the overhead of SIB19 was included in some of the results - Note: an observation when SSB/SIB1 periodicity is 320 ms will be discussed and added to the observation Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms Observation Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan. Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

Based on above observations, 320ms SSB periodicity is beneficial to improve coverage ratio and is able to reduce common channel overhead significantly. Regarding backward compatibility, if one cell is configured with 320ms period, legacy UE may not be able to access this cell, but the Rel-19 UE is able to connect this cell via configuring different SSB periodicities to support different UEs accessing different beams, shown in the figure 2. It means legacy UE and Rel-19 UE can co-exist within one satellite coverage, but just with different SSB periodicity for different beam coverage. In this context, one legacy UE can be served by network with effective beam configuration. Therefore, backward compatibility is kept with maximum possibility.

Figure 2: Different beams with different SSB periodicities

In [3], the simulation results show that the performance degradation of synchronization performance and demodulation performance is less than 1dB when SSB periodicity is 320ms. It means extending SSB periodicity to 320ms will not cause obvious performance loss in link level, which can work well in the SINR range of case 1-1 and case 1-2 as agreed satellite scenarios in RAN1.

Observation 3: Based on RAN1 evaluation, 320ms SSB periodicity is beneficial and feasible for Rel-19 NTN when taking into account overhead reduction of common channel, coverage ration improvement and link-level performance.

In RAN2, there were also some discussions on SSB periodicity extension. If 320ms periodicity is supported, relevant configurations and parameters will be changed accordingly, but will not impact the feasibility of 320ms SSB periodicity itself. Besides, the potential enhancements to DL coverage enhancements in RAN2 (w.r.t. e.g. cell/beam level DRX/DTX, cell reselection, access control, etc.) can be adaptive to the SSB periodicity extension, regardless of whether the extended periodicity is 160ms or 320ms. So from RAN2 perspective, there is no showstopper that prevents the 320ms SSB periodicity extension being introduced.

Observation 4: Current RAN2 discussion on potential DL coverage enhancements can adapt to SSB periodicity extension, regardless of the specific extended value (160ms or 320ms).

For RAN4, the SSB periodicity of 160ms and 320ms was discussed. Regarding more than 160ms, there is one agreement on RRM requirement discussion in [4]:

### Issue 1-1: SSB periodicity enhancement Agreement (online): For SSB periodicity larger than 160ms if supported by other WGs: - Whether RAN4 will define the RRM requirements for SSB periodicity larger than 160ms is depending on whether it is supported by WGs.

Therefore, SSB periodicity larger than 160ms is not excluded at least in RAN4, and RAN4 agreed that whether RAN4 will define the RRM requirements for SSB periodicity larger than 160ms is depending on whether it is supported by WGs, which means that if it is supported by other WGs, RAN4 will define the RRM requirements for SSB periodicity larger than 160ms.

Observation 5: Based on RAN4 discussion, 320ms SSB periodicity can be considered depending on other WGs’ decision.

Based on overall technical analysis and WGs progress, it is proposed from RAN perspective to provide guidance to RAN WGs that 320ms SSB periodicity is supported in Rel-19 NR NTN.

Proposal 1: Based on current RAN WGs progress on SSB periodicity discussion, it is proposed to support 320ms SSB periodicity in addition to 160ms SSB periodicity.

Summary

This contribution presented our views on SSB periodicity of NTN Rel-19. Based on technical analysis and evaluation, supporting 320ms SSB periodicity is feasible. Within this target, the observations and proposals are listed as follows:

Observation 1: Adjusting SSB periodicity is able to meet different level requirement of coverage ratio, with 96.88% coverage ratio at 320ms period and 100% coverage ratio at 640ms.

Observation 2: In order to approach the target of 100% coverage ratio, extending SSB periodicity with 320ms or above is critical to meet the requirements of satellite operators.

Observation 3: Based on RAN1 evaluation, 320ms SSB periodicity is beneficial and feasible for Rel-19 NTN when taking into account overhead reduction of common channel, coverage ration improvement and link-level performance.

Observation 4: Current RAN2 discussion on potential DL coverage enhancements can adapt to SSB periodicity extension, regardless of the specific extended value (160ms or 320ms).

Observation 5: Based on RAN4 discussion, 320ms SSB periodicity can be considered depending on other WGs’ decision.

Proposal 1: Based on current RAN WGs progress on SSB periodicity discussion, it is proposed to support 320ms SSB periodicity in addition to 160ms SSB periodicity.

Reference

- RP-240775, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT, RAN#103

- Chair's notes RAN1#116 9.11 R19 NTN (eom)

- R1-2408033, Further consideration on downlink coverage enhancement for NR NTN, CATT

- R4-2420105, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)