3GPP TSG-RAN Meeting #106		RP-243127

Madrid, Spain, 9th – 13th, Dec, 2024

Source:	CATT

Title:	Consideration on NTN in 5G-A Rel-20

Agenda Item:	15.2

Document for:	Discussion and Decision

Introduction

NR NTN and IoT-NTN have been specified in 3GPP RAN Rel-17, since then been optimized in Rel-18 and Rel-19. Commercial deployment is ongoing at this moment. Based on real deployment or deployment plans, further evolution of NR NTN and IoT-NTN are needed.

In this contribution, we provide our views on NTN evolution (both NR NTN and IoT NTN) in 5GA Rel-20 with some potential enhancements.

Discussion

NR NTN evolution

2.1.1 Regenerative payload with CU-DU split architecture

In Rel-17 and Rel-18, only Transparent Payload architecture is supported for NR NTN. In Rel-19, Regenerative Payload with full gNB on-board satellite is supported.

To further consider the complexity, the weight, the power consumption of the satellites, it’s beneficial to consider the RAN split architecture (i.e. CU-DU split) in Rel-20, which allows more deployment options for the satellite operators besides Transparent Payload and Regenerative Payload with full gNB.

With the CU-DU split architecture, gNB-CU and gNB-DU could be connected with each other via the feeder link, or inter satellite link (ISL). Regenerative Payload with CU-DU split architecture is shown in the figure below:

Figure 1. Regenerative Payload with CU-DU split architecture

As above, some potential use cases include:

Case 1: gNB-DU on satellite, connected to the gNB-CU on ground (DU4 + CUx);

Case 2: One satellite holds CU + DU and it serves the gNB-DUs on the neighbor satellites, on ground, or on vehicles.

For Case 1, gNB-DU on-board connects to the CU on ground via the feeder link, as the DU4 and CUx in the figure above. This is the most basic architecture option for CU-DU split.

For Case 2, we can assume some of the low cost “small” satellites only hold the DU, and some of the “big” satellites hold the CU+DU. They could connect to each other and work together, as shown in the figure above, where DU2 is connected to CU1, DU6 is connected to CU5. With such kind of deployment, the “small” satellites only need to connect with the neighbor “big” satellite, without the need to establish and maintain the feeder link towards the NTN GW, which could further reduce the complexity and cost of the “small” satellites.

Furthermore, the on-board CU could also serve the DUs deployed on ground, on the air vehicles, the ship, etc.  As shown in the figure above, CU5 could also serve the DU7 and DU8 deployed in the ship or air vehicle. In this case, the NR UEs without NTN capability (e.g. legacy mobile phones) could obtain the NR services in the air vehicle, ship.

As above, it’s beneficial to further consider regenerative payload with CU-DU split architecture in NR NTN, which provides additional and flexible deployment options. To make support of regenerative payload with CU-DU split architecture, the major impact is the management of F1 due to the movement of the NGSO satellites.

Proposal 1: Consider regenerative payload with CU-DU split architecture in 5G-A R20.

2.1.2 Multi-orbits coordination

In the SA1 new SID (SP-240495), there are several new use cases on support of multi-orbits satellite access (e.g. LEO/MEO/GEO) for different services, which aims to provide the better experience for the UE services.

Figure 2. Example of multi-orbits coordination

Idle UE could select/reselect a proper satellite orbit to initiate the service according to UE capability and the QoS requirement of the service, e.g. select the TN/LEO, if feasible, for the service(s) which require high throughput and/or low latency.

For Connected mode UE, the QoS requirement of the services could also be taken into account when deciding the target cell (e.g. select proper satellite from different orbits) for HO.

Thus, it’s proposed to consider multi-orbits coordination, including network selection/reselection/HO. The detailed specification impact needs to be further investigated, with coordination with SA1 and SA2.

Proposal 2: Consider multi-orbits coordination in Rel-20 with coordination with SA1 and SA2.

2.1.3 Robust page notification

For satellite coverage, there are some possibilities with very low SINR. In these scenarios, users may experience poor reception conditions and thus miss calls and messages, which can be especially detrimental for public safety or emergency purpose paging messages. These poor reception condition scenarios can happen when UE is placed in pockets, backpacks or in vehicles, boats, etc., or in conditions where there are clutter losses. In order to improve the coverage and user experience, one effective method is to inform the UE with short message and UE can consciously choose a good spot to initiate communication.

In this context, a notification alert feature is proposed to address the problem of missed paging messages to UE in low SNR conditions (e.g. Non Line Of Sight). The feature should be able to mitigate this additional 20dB loss compared to the link margin required in Line of Sight conditions.

It is noted that SA1 is working on relevant requirement definition of this feature. From RAN level, it is necessary to have a dedicated page channel in worse SINR condition and moreover, potential page or wakeup procedure should be defined to enable one specific UE to enter one good area to access the network after this special page.

Proposal 3: Consider to support robust notification for very low SINR condition.

2.1.4 Positioning assisted UE access

In 5G NR NTN, UE is assumed with good GNSS position information and is able to conduct timing and frequency compensation to access the network. However, evidence has been shown that GNSS jamming (leading to denial of service) and spoofing (leading to incorrect location reporting and potentially denial of service) may occur on a temporary basis. Also, a terminal or group of terminals attempting to connect over access channels or once connected to the network could also cause denial or degradation of service for all other users in the cell if using an altered GNSS position.

Therefore, there is a need to increase the robustness of uplink time and frequency synchronization in NR NTN based access against of GNSS temporary unavailability. One simple method is to consider network assisted UE positioning to help UE to get its position. For example, network can indicate emperies information to help UE to get satellite location, and UE can measure the reference signal from satellite to determine its location. With network assistance, using legacy DL-TDOA technique can help UE to estimate its location.

Proposal 4: Consider network assisted positioning to help UE to enable initial access in case of GNSS position information unavailable.

2.1.5 Interference coordination

When the mega-LEO satellite constellation is deployed, if the frequency band is limited, frequency reusing factor will be 1 or 2. The interference among adjacent satellites is expected to be very strong in overlapped areas in case of same frequency resource assigned. When UE is located in overlapped area, the signal quality is often not guaranteed due to un-coordinated beam scheduling for adjacent satellites. In some cases it may cause severe inter-satellite interference and poor user experience, e.g. two overlapped beams from different satellites with relatively long service time. In this context, it is necessary to coordinate resource allocation and beam direction for the overlapped area. As shown in the figure 3, UE2 and UE3 will suffer the interference because these two UEs are associated with different satellites and resource assignment is possible to collide. If with the coordination for inter-satellites in time, frequency and beam direction, the interference will be significantly reduced. Potentially it may require information exchange via Xn interface.

Figure 3. Inter-satellite interference in overlapped area

Proposal 5: Study interference coordination solution for inter-satellites sharing same frequency band and specify it to improve user experience.

IoT NTN evolution

2.2.1 5GC connection

In the beginning of Rel-19, some satellite operators showed strong interest to support 5GC connecting for the eNB in IoT NTN. But due to corresponding work is not included in SA2 Rel-19 work scope, it was agreed that 5GC supporting IoT-NTN is not part of the objectives for the Rel-19 WI.

Considering 5GC connection for terrestrial NB-IoT/eMTC eNB is already supported since Rel-16. Enabling 5GC connection for IoT NTN makes the deployment more flexible, e.g. not necessary to deploy the EPC, 5GC could be used instead, with that the same 5GC could be used for IoT NTN, TN, and NR NTN.

It’s proposed to support 5GC connection for IoT NTN, and corresponding mechanisms specified for terrestrial NB-IoT/eMTC in Rel-16 could be taken as the baseline, if any gap should be further confirmed and specified.

Proposal 6: Support 5GC connection for IoT NTN with coordination with SA2. Terrestrial NB-IoT/eMTC 5GC connection could be taken as the baseline.

2.2.2 Voice over GEO

In SA1, the feasibility study is on-going for Voice over GEO IoT NTN. Due to wider coverage for satellite service, the voice service is promising for GEO and LEO. Since the link budget is limited in GEO, the date rate will be a bit lower. In order to support Voice over GEO NTN, the following techniques can be considered:

- Low rate encoding/decoding

- Voice dedicated bear for QoS control

- IMS signaling optimization to support IoT Voice

For smart phone accessing the satellite network, the expected voice service may suffer from the long delay and low voice quality. But anyway, it brings new service capability and possibly increases the revenues for satellite operator, even TN operator. From this perspective, it is necessary to develop this technique in Rel-20.

Proposal 7: Support Voice over GEO IoT NTN to bring new business opportunity.

Conclusion

According to the discussions above, it’s proposed to consider the following potential enhancements in 5G-A Rel-20:

NR NTN potential enhancements:

- Regenerative payload with CU-DU split

- Multi-orbits coordination

- Robust notification for very low SINR condition.

- Network assisted positioning to help UE to enable initial access in case of GNSS unavailable

- Interference coordination for inter-satellites sharing same frequency band.

IoT NTN potential enhancements:

- 5GC connection for IoT NTN

- Support Voice over GEO