3GPP TSG RAN Meeting #106						                                                             RP-243169

- Madrid, Spain, December 9-12, 2024

Source:	ZTE

Title:	Discussion on the scope of NR-NTN

Agenda Item:	9.3.2.2

Document for: Discussion

- Introduction

In the WID of Rel-19 [1], the study for downlink coverage enhancements in NTN is targeted to be completed by RAN#106.

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In this contribution, the views on above objective are elaborated.

- Discussion on DL coverage enhancement

  - 2.1 Background

In RAN1#116 and RAN1#116bis, as reflected in the agreements listed in appendix, three satellite parameter sets (i.e., with different assumption on the simultaneously active beams, link budget at given elevation angle) have been agreed to reflect the realistic limitation and assumptions of corresponding satellite implementation, which are essential parts of the “NTN deployment constraints”.  As the target to address the potential issues, in this item, when the potential solutions of DL coverage enhancement are considered, all relevant issues should be either interpedently and equally considered or jointly considered as a workable solution. And the DL coverage enhancement should ensure that satisfactory performance can be achieved.

Observation 1: The parameters sets agreed in RAN1 reflects the existing NTN deployment constraints.

Proposal 1: The DL coverage enhancement should be able to provide the complete solution to satisfy the needs of all deployment since the issues have been identified in Rel-19.

  - 2.2 Discussion on the system level enhancement

For the system level enhancement, the following observation and agreement have been achieved in RAN1:

Observation Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km: - For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied. - Note: coverage ratio is N2+N3/ total beam footprints - Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs. Based on the results of DL coverage ratio evaluation at system level collected from 3 sources for a deployment scenario implementing wide beam footprint: - 1 source reports that with a deployment of wide beam covering 4 narrow (of 50km size) beams, which means Set 1-2 FR1 with additional EIRP reduction of 6dB, using SSB periodicity of 80 ms can provide coverage ratio of 96.8%, and Set 1-1/1-3 FR1 with additional EIRP reduction of 6dB, SSB periodicity of 80 ms can provide coverage of 100%. - 1 source observed that for Set 1-1, 1-2 and 1-3, the coverage ratio can be improved from 1.5% to 100% using the legacy default SSB periodicity of 20ms during initial access, by choosing a wide beam footprint with beam footprint sizes of 84 km and 56 km respectively. - Note: the PDCCH and the PDSCH for SIB19 is assumed to be transmitted within 2 OFDM symbols and 5 MHz bandwidth. the PDSCH for SIB1 is assumed to be transmitted within 3 OFDM symbols and 5 MHz bandwidth. This assumes no SIB1 and SIB19 transmission in N2 beam footprints. This assumes non-aligned SFN timing across different beams. - 1 source observed, for Set 1-1 with increased beam size, that the legacy SSB periodicity of 20ms during initial access is usable with NTN beam hopping, by choosing a deployment scenario implementing wide beam footprint with beam footprint sizes of 70.7 km and 86.6 km, leading to a total of 529 and 353 beam footprints within the satellite coverage area, respectively, and the coverage ratio is 80% and 90%, respectively, and a ratio of simultaneously active beam footprints to the total number of beam foot prints equal to 20% and 30%. - Note: Beam footprint size is increased by increasing only the adjacent beam spacing without increasing the 3dB beamwidth. Note: RAN1 will further investigate the impact of SSB periodicity extension Note: Any needed clarification “SSB channel enhancement is not considered” in the WID is up to RAN plenary Note: RAN1 will further investigate the impact of wider beam of SSB and/or other channels on performance (e.g. link budget, capacity...) Observation Based on the results of DL coverage evaluation at system level collected from different sources, it is observed that extending the default value of SSB periodicity (different from 20ms) in NTN with LEO600km satellite parameter sets where the beam footprint diameter is 50 km, is beneficial in terms of reduction of common control channel overhead, when targeting a full coverage of 1058 beam footprints: - With Set 1-1 FR1 and Set 1-3 FR1, the common messages (SSB, SIB1) overhead is around 40% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead ratio could be reduced to less than 14% when 160ms SSB/SIB1 periodicity is used. - With Set 1-2 FR1, the common message (SSB, SIB1) overhead is greater than 100% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead could be reduced to around 25.8% when 640ms SSB/SIB1 periodicity is used. - Note: the overhead of SIB19 was included in some of the results - Note: an observation when SSB/SIB1 periodicity is 320 ms will be discussed and added to the observation Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

RAN1 has agreed to extend the SSB periodicity at least to 160ms. Whether to support 320ms periodicity is FFS. According to RAN1 observation, it can be found that extending SSB periodicity from 20ms to 320ms is necessary when utilizing beam hopping to improve the coverage ratio to near 100% for satellite parameter set 1-2. For other solutions, it may either require the changes of the assumption on the agreed parameter sets or should be jointly used along with the extension of the SSB periodicity and other additional enhancement (e.g., mitigation of additional degradation of the link budget).

Observation 2: Extending SSB periodicity to 320ms is important to achieve satisfactory coverage ratio at least for satellite parameter set 1-2 agreed in RAN1.

Moreover, for set 1-3, RAN1 has agreed on link level enhancements for some common channels, e.g., repetition of PDCCH CSS and PDSCH with SIB1, which further increase the dwell time of beam hopping and common channel overhead.  For example, according to RAN1 observations, at least two repetitions of SIB1 and two transmissions of SIB19 are necessary to achieve the target CNR of -8dB for set 1-3. Thus, the transmission time for minimum required system information (i.e., SSB+SIB1+SIB19) will increase from 3ms to 5ms as shown in Figure 1. When jointly consider the link level enhancement and system level enhancement, the required SSB periodicity to achieve target coverage ratio will be even longer than that in RAN1 observation since repetition of common channel was not considered when making observations above. Moreover, to allow initial access within a single hop, the dwell time should be long enough to cover the whole initial access procedure shown in Figure 2. Assuming 2 repetitions of msg4 (already agreed by RAN1 for link level enhancement) and no repetition of UL transmission, the time required for complete initial access procedure can be as large as 34.5ms as summarized in Table 1, which means only 1 beam footprint can be served by a beam within 40ms. If only 160ms SSB periodicity is supported, the coverage ratio will be only 106*(160/40)/1058=40%. Besides, according to RAN1 observation, longer SSB periodicity is beneficial for reduction of common channel overhead, which is common for all satellite parameter sets. Therefore, supporting 320ms SSB periodicity can also be beneficial for set 1-1 and set 1-3 at least when jointly considering the link level enhancement for common channel.

Figure 1 Time domain resource for minimum required system information for set 1-3

Figure 2 Procedure of initial access

Table 1 Time required for an initial access procedure

| Name | Time |
| --- | --- |
| SSB+SIB1+SIB19 (with repetition) | 5ms |
| msg1 (format 1) | 3ms |
| msg2 | 1ms |
| msg3 | 1ms |
| msg4(with repetition) | 2ms |
| msg5 | 1ms |
| Single trip propagation delay (LEO600, 30-degree elevation angle) | 3.6ms |
| Total time if failed to receive msg1 from UE | 15.2ms (= 5+3+3.6*2) |
| Total time | 34.5ms (= 5+3+1*3+2+3.6*6) |

Observation 3: When the enhancements are implemented to satisfy the needs of link budget, the extension of the SSB periodicity to 320ms is simultaneously required for all satellite parameter sets agreed in RAN1 to enable the beam hopping with increased dwell time per beam and reduce the common channel overhead caused by repetition based enhancement.

Proposal 2: RAN endorse the normative phase of DL coverage enhancement to specify system level enhancement with support of 320ms SSB periodicity for initial access.

Regarding using different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, we think it is an implementation issue and can be achieved using existing beam management. The main issue of using wide beam to improve DL coverage is that the link budget will decrease accordingly, including link budget for UL. Generally, the link budget for UL is more challenging than DL, which can be the bottleneck of beam size. That is, further enhancement for wide and narrow beam is mainly targeting for UL instead of DL. Moreover, considering that PRACH repetition has already been supported in Rel-18, the UL coverage loss caused by wide beam can already be mitigated to some degree. Hence, it is not urgent to introduce UL enhancement for application of different satellite beam size across the satellite footprint. If the UL enhancement for wide and narrow beam in NTN is thought beneficial, we can postpone the discussion and include it as one objective of Rel-20 UL coverage enhancement.

Observation 4: Applying different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint can be supported by existing beam management and PRACH repetition mechanisms and the enhancement for DL beam hopping with larger periodicity is still required.

Proposal 3:  Any further enhancement for applying different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint (if justified) can be considered in Rel-20 UL coverage enhancement.

  - 2.2 Discussion on the link level enhancement

For the link level enhancement, the following agreements have been achieved in RAN1:

Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement Agreement For PDSCH with Msg4 Link level enhancement: - Support PDSCH repetition - FFS: signalling design including number of repetitions - FFS: impact on UE capability - Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. Agreement For link level enhancement of PDSCH with SIB1: - Support PDSCH repetitions within 20 ms duration - The number of repetitions is fixed to 2 repetitions - Further discuss the specification impact for at least the following: - Procedure and signaling (enabling repetitions, associated time resource determination, etc.) - Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size. - Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern. Agreement For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN. - FFS: intra-slot and/or inter-slot

It can be found that RAN1 has agreed to support PDSCH repetition for link level enhancement of PDSCH with Msg4 or SIB1 and support PDCCH repetition for link level enhancement of PDCCH CSS except Type-3. Therefore, RAN can conclude to specify repetition based link level enhancement in normative phase at least for PDSCH with Msg4, PDSCH with SIB1, and PDCCH CSS except Type-3. Regarding PDSCH with SIB19, which was agreed to be considered in RAN1 but not concluded to be enhanced, RAN1 can further discuss whether enhancement is needed in normative phase.

Proposal 4: RAN endorse the normative phase of DL coverage enhancement to specify link level enhancement at least for PDSCH with Msg4, PDSCH with SIB1, and PDCCH CSS except Type-3.

- Conclusion

- In this contribution, the detailed views on the objectives for NR-NTN are provided with following proposals:

Observation 1: The parameters sets agreed in RAN1 reflects the existing NTN deployment constraints.

Proposal 1: The DL coverage enhancement should be able to provide the complete solution to satisfy the needs of all deployment since the issues have been identified in Rel-19.

Observation 2: Extending SSB periodicity to 320ms is important to achieve satisfactory coverage ratio at least for satellite parameter set 1-2 agreed in RAN1.

Observation 3: When the enhancements are implemented to satisfy the needs of link budget, the extension of the SSB periodicity to 320ms is simultaneously required for all satellite parameter sets agreed in RAN1 to enable the beam hopping with increased dwell time per beam and reduce the common channel overhead caused by repetition based enhancement.

Proposal 2: RAN endorse the normative phase of DL coverage enhancement to specify system level enhancement with support of 320ms SSB periodicity for initial access.

Observation 4: Applying different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint can be supported by existing beam management and PRACH repetition mechanisms and the enhancement for DL beam hopping with larger periodicity is still required.

Proposal 3:  Any further enhancement for applying different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint (if justified) can be considered in Rel-20 UL coverage enhancement.

Proposal 4: RAN endorse the normative phase of DL coverage enhancement to specify link level enhancement at least for PDSCH with Msg4, PDSCH with SIB1, and PDCCH CSS except Type-3.

- References

- RP-241789, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#105.

Appendix

Agreement For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1: LEO600km Set1-1 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size(Note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 34. Payload Total DL power level (dBW); 31.24. Aggregated EIRP (Total) (dBW); 61.24*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 41. Total number of beam footprints***; 1058. Total number of simultaneously active beams **; 106. % simultaneously active beams**; 10.02 %. *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. LEO600km Set1-2 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size (note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 34. Payload Total DL power level (dBW); 23. Aggregated EIRP (Total) (dBW); 53*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 41. Total number of beam footprints; 1058. Total number of simultaneously active beams**; 16. % simultaneously active beams**; 1.5 %. *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. LEO600km Set 1-3 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size (note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 26. Payload Total DL power level (dBW); 23.24. Aggregated EIRP (Total) (dBW); 53.24*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 33. Total number of beam footprints; 1058. Total number of simultaneously active beams**; 106. % simultaneously active beams**; 10.02 %. *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework. Observation The CNRs for the satellite payload parameters Set 1-1, Set 1-2 and Set 1-3 are equal to -1.9 dB, -1.9 dB and -9.9 dB respectively.