3GPP TSG RAN Meeting #106													RP-243183

Madrid, Spain, December 9-12, 2024

Agenda Item:	9.1.1

Source:	CEWiT

Title:	New WI proposals for Ambient IoT (Internet of Things) in NR

Document for:	Discussion and decision

# Introduction

Ambient IoT is a new 3GPP IoT technology, which relies on ultra-low complexity devices with ultra-low power consumption for very-low end IoT applications. A RAN WG level study on solutions for Ambient IoT was agreed in RAN#102 [1]. The details about the scope of study are captured in latest SID [2]. Different RAN WGs conducted the study, and the outcome of the study is captured in TR 38.769 [3]. In this document, we provide the motivation for proceeding with a work item on Ambient IoT and list our views on the potential scope of the work item.

# Discussion

The study on solutions for Ambient IoT was completed in the recently concluded RAN WG meeting and the details of the study and the recommendations are captured in TR 38.769 [3]. The study has focused on all device types: 1, 2a, and 2b. However, the key features of the Ambient IoT devices, i.e., ultra-low complexity and power consumption, backscattering, etc., are more relevant to Device types 1 and 2a. Also, device types 1 and 2a can have unified design. Device type 2b is supposed to have advanced capabilities and features compared to Device 1 and 2a. Therefore, a unified design for device type 2b along with other device types is not optimal for Device 2b. Therefore, the WI should start with device types 1 and 2a.

Observation 1: Regarding Ambient IoT device types, the following observations are made

- Device type 1 and 2a have similar features and requirements

- Unified design is applicable for Device 1 and Device 2a as baseline

- Unified design for all device types is not optimal for advanced devices type like Device 2b.

Proposal 1: Device type 1 and Device type 2a should be prioritized in Rel 19 WI.

The connectivity topologies for Ambient IoT networks, D1T1 and D2T2, have been analyzed, with their coverage performance evaluated across various scenarios. The results indicate that each topology offers distinct coverage benefits depending on the use case. For e.g., D1T1 is ideal when both the reader and devices are indoors, whereas D2T2 is more suitable when the reader is outdoors and the devices are indoors Also, the design can be common for D1T1 and D2T2, in most of the cases, as the device is unaware of the topology. Therefore, to ensure comprehensive support and maximize performance across different applications, both D1T1 and D2T2 should be included in the WI phase. Furthermore, as noted in our contribution [4], the target value for applicable maximum distance for Device 1 in D2T2 topology is significantly limited and does not meet the required range of 10 to 50 meters. However, for D1T1 topology, both Device 1 and Device 2a satisfy the target value for applicable maximum distance outlined in [4].

Observation 2: Regarding topologies for Ambient-IoT system, following observations are made

- Topologies D1T1 and D2T2 offer distinct coverage benefits depending on the use case

- Baseline design can be common for D1T1 and D2T2, as the topology is transparent to the device

- The target value for applicable maximum distance for Device 1 in D2T2 topology does not meet the requirement of 10 to 50 meters.

Proposal 2: Following Topologies should be supported in Rel 19 WI

- D1T1 for Device 1 and Device 2a.

- D2T2 for Device 2a.

Different deployment scenarios were studied for Ambient IoT system, of which the CW outside scenario (i.e., D1T1-B and D2T2-B) is simple and provides maximum coverage and minimum interference. The CW inside scenario D1T1-A2 is also simple and provides good coverage and decent interference suppression, if the device has large frequency shift capability. However, D1T1-A1 and D2T2-A1 topologies are complex and require coordination between readers, transmitting in R2D and receiving in D2R. Further, UE-to-UE coordination in D2T2-A1 needs further study. Also, scenario A1 introduces complex signaling exchanges, tight synchronization requirements and unnecessary deployment challenges. Therefore, scenarios D1T1-B, D2T2-B, D1T1-A2 and D2T2-A2 should be prioritized in Rel. 19.

Observation 3: The following observations are made regarding various scenarios studied

- CW outside scenario (i.e., D1T1-B and D2T2-B) is simple, provide maximum coverage and good interference suppression

- Scenario D1T1-A2 and D2T2-A2 are simple, provide good coverage and decent interference suppression, if the device has large frequency shift capability

- Scenario, D1T1-A1 and D2T2-A1 are complex, require tight synchronization/coordination between readers and bring unnecessary deployment challenges

Proposal 3: Scenarios D1T1-B, D2T2-B, D1T1-A2 and D2T2-A2 should be prioritized in Rel 19 WI.

As already mentioned under deployment scenario, support of large frequency shifter at the device is recommended as it offers significant advantages in managing interference at the reader and simplifying the operations at the reader. By shifting the backscattered signal to a different frequency band than the carrier wave (CW), self-interference and cross-link interference (CLI) between the CW and the backscattered signal can be minimized. Also, it eliminates the need for full duplex operation at the reader in D1T1-A2 and D2T2-A2 scenario and simplifies filtering in bistatic setups.

Observation 4: The following observations are made regarding support of large frequency shifter for device 2a

- Offers significant advantages in managing interference and simplifying the operations at the reader

- Eliminates the need for full duplex capability at the reader

- Minimizes self-interference and CLI

- Meeting spectrum regulation in Tx/Rx

Proposal 4: Large frequency shifter is supported for device 2a.

Regarding spectrum for transmission of CW, the main issues are related to the reader in full duplex (FD) mode of operation and the regulation in transmission/reception. In case of D1T1-A2, the reader is transmitting CW and device 1 does not have LFS capability. Therefore, the reader must operate in FD mode irrespective of the spectrum of CW (i.e., DL or UL). Also, the regulation issue persists irrespective of the spectrum of CW. e.g., the BS must transmit in UL spectrum in case of CW in UL and receive in DL spectrum from device type 1 in case of CW in DL. Neither are supported by regulation. However, for other scenarios (i.e., D1T1-B, D2T2-A2 and D2T2-B), when the CWN transmits the CW in UL spectrum, the reader does not have to operate in FD mode (since device 1 is not suitable for D2T2 as stated earlier) and is not creating any regulation issue. Therefore, CW transmission in UL spectrum should be prioritized in Rel. 19 for CW outside topology (D1T1 and D2T2) and inside topology in case of D2T2-A2 scenario.

Observation 5: The following observations are made regarding spectrum for CW transmission

- For D1T1-A2, the reader must operate in FD mode and transmit/receive against regulation irrespective of the spectrum of CW

- For D1T1-B, D2T2-A2 and D2T2-B, CW in UL spectrum creates neither necessity of FD mode operation at the reader nor regulation issue

Proposal 5: Carrier wave transmission in UL spectrum should be prioritized in Rel. 19

In topology D2T2, the intermediate UE acts as reader and communicates with the Ambient IoT devices. However, the intermediate UE must be under the serving gNB and needs control information from the gNB for serving the Ambient IoT devices and for multiplexing NR and Ambient IoT operations. The following options are studied for control of intermediate UE in D2T2:

- Option 1: Higher-layer signaling is used

- Option 2: L1 and higher-layer signaling are used

In our view, option 2 is recommended as Option 2 provides higher flexibility and better resource utilization. Also, option 2 is in line with the configurations in NR.

Proposal 6: For controlling intermediate node in D2T2, Option 2 (i.e., ‘L1 and higher-layer signaling are used’) should be prioritized.

Regarding modulation schemes for R2D and D2R, various schemes were studied. For R2D, both OOK1 and OOK 4 should be considered. For D2R OOK, BPSK and MSK were studied and captured in TR. From the given options, MSK has lower spectral efficiency compared to OOK and BPSK because of phase inaccuracies at the device. Also, implementing MSK increases device complexity. Therefore, OOK and BPSK should be prioritized for D2R.

Proposal 7: For modulation schemes for Ambient IoT communication

- OOK1 and OOK4 should be prioritized for R2D

- OOK and BPSK should be prioritized for D2R.

Regarding multiple access techniques for Ambient-IoT devices different options were considered during study for R2D and D2R. In R2D, TDMA is the simplest option with minimum device complexity. Implementing FDMA requires devices to operate in certain frequency ranges, which depends on tuning and filtering capabilities of the device. Therefore, FDMA is not ideal for low complexity devices. For D2R, TDMA, FDMA and CDMA were studied. The advantage for TDMA is the low implementation complexity for both device and reader, which makes it the most suitable option. And the potential benefit of FDMA is to increase the transmission efficiency and reduce collisions. Also, the FDMA can be implemented in D2R using the small frequency shifter feature at the Ambient-IoT device. In contrast, with CDMA, due to poor synchronization performance in time and frequency domain of device would degrade the code orthogonality and thus results in a bad cross-correlation performance and additionally CDMA might be too complex to implement in Ambient IoT devices, which might involve complexities with generating orthogonal sequences. Therefore, CDMA should be deprioritized.

Observation 6: The following observations are made regarding different multiple schemes

- TDMA is the simplest option with minimum implementation complexity for both R2D and D2R

- FDMA increases transmission efficiency and resource utilization.

- FDMA for R2D requires devices to operate in certain frequency ranges, which depends in its tuning and filtering capabilities, which increases device complexity and cost

- FDMA implementation in D2R using the small frequency shifter at the Ambient-IoT device is less complex

- CDMA implementation is complex and requires tight synchronization between nodes

Proposal 8: For multiple access schemes for R2D and D2R

- TDMA should be prioritized in R2D

- TDMA and FDMA should be considered in D2R.

Regarding device unavailability, two directions are studied. In direction 1, the reader neither provides any input to the device nor is notified about when the device becomes available or unavailable. Essentially, the device becomes operational (i.e., ON state) once it is fully charged and will continue to carry out transmission and reception (Tx/Rx) functions until it runs out of energy. In this case a device can have two states i.e. ON and OFF state. The device can perform the Tx/Rx in ON state, but it cannot perform any operation during OFF state. Direction 1 is simpler and compatible for both device 1 and device 2a. However, the reader is unaware of the energy remaining at the device or about the device availability. Therefore, the paging/triggering from the reader can be for a device which is unavailable. Also, the device can become unavailable in the middle of a process due to lack of energy. Therefore, direction 1 is inefficient and can lead to resource wastage.

In direction 2, the reader can control the availability/unavailability of the device. In this case the device can have three states i.e. ON, OFF, and sleep state. The device can perform the Tx/Rx operation during ON state and device can do the energy harvesting during its sleep state. In this direction the reader is aware of the device availability status and based on this reader can transmits the paging/trigger messages to the device. Therefore, direction 2 offers the advantages of low communication delay and efficient resource allocation, as the reader provides control information to help the device determine whether/when to become available or unavailable. Further, at least for Device 2a, direction 2 enhances its capabilities by extending its operational time, allowing for energy harvesting and enabling the device to resume transmission and reception operations after recharging its energy.

Therefore, both direction 1 and direction 2 should be supported and utilized based on the device types and capabilities. This approach allows for more efficient use of each device's capabilities, ensuring optimal performance tailored to the specific requirements and complexities of the devices.

Observation 7: Regarding directions studied for device unavailability

- Direction 1 is simpler and compatible for both device 1 and device 2a but is inefficient and can lead to resource wastage.

- Direction 2 offers low communication delay, efficient resource allocation and optimum use of device capabilities

Proposal 9: Both Direction 1 and Direction 2 for device unavailability should be considered in Rel. 19.

# Conclusion

Observation 1: Regarding Ambient IoT device types, the following observations are made

- Device type 1 and 2a have similar features and requirements

- Unified design is applicable for Device 1 and Device 2a as baseline

- Unified design for all device types is not optimal for advanced devices type like Device 2b.

Proposal 1: Device type 1 and Device type 2a should be prioritized in Rel 19 WI.

Observation 2: Regarding topologies for Ambient-IoT system, following observations are made

- Topologies D1T1 and D2T2 offer distinct coverage benefits depending on the use case

- Baseline design can be common for D1T1 and D2T2, as the topology is transparent to the device

- The target value for applicable maximum distance for Device 1 in D2T2 topology does not meet the requirement of 10 to 50 meters.

Proposal 2: Following Topologies should be supported in Rel 19 WI

- D1T1 for Device 1 and Device 2a.

- D2T2 for Device 2a.

Observation 3: The following observations are made regarding various scenarios studied

- CW outside scenario (i.e., D1T1-B and D2T2-B) is simple, provide maximum coverage and good interference suppression

- Scenario D1T1-A2 and D2T2-A2 are simple, provide good coverage and decent interference suppression, if the device has large frequency shift capability

- Scenario, D1T1-A1 and D2T2-A1 are complex, require tight synchronization/coordination between readers and bring unnecessary deployment challenges

Proposal 3: Scenarios D1T1-B, D2T2-B, D1T1-A2 and D2T2-A2 should be prioritized in Rel 19 WI.

Observation 4: The following observations are made regarding support of large frequency shifter for device 2a

- Offers significant advantages in managing interference and simplifying the operations at the reader

- Eliminates the need for full duplex capability at the reader

- Minimizes self-interference and CLI

- Meeting spectrum regulation in Tx/Rx

Proposal 4: Large frequency shifter is supported for device 2a.

Observation 5: The following observations are made regarding spectrum for CW transmission

- For D1T1-A2, the reader must operate in FD mode and transmit/receive against regulation irrespective of the spectrum of CW

- For D1T1-B, D2T2-A2 and D2T2-B, CW in UL spectrum creates neither necessity of FD mode operation at the reader nor regulation issue

Proposal 5: Carrier wave transmission in UL spectrum should be prioritized in Rel. 19

Proposal 6: For controlling intermediate node in D2T2, Option 2 (i.e., ‘L1 and higher-layer signaling are used’) should be prioritized.

Proposal 7: For modulation schemes for Ambient IoT communication

- OOK1 and OOK4 should be prioritized for R2D

- OOK and BPSK should be prioritized for D2R.

Observation 6: The following observations are made regarding different multiple schemes

- TDMA is the simplest option with minimum implementation complexity for both R2D and D2R

- FDMA increases transmission efficiency and resource utilization.

- FDMA for R2D requires devices to operate in certain frequency ranges, which depends in its tuning and filtering capabilities, which increases device complexity and cost

- FDMA implementation in D2R using the small frequency shifter at the Ambient-IoT device is less complex

- CDMA implementation is complex and requires tight synchronization between nodes

Proposal 8: For multiple access schemes for R2D and D2R

- TDMA should be prioritized in R2D

- TDMA and FDMA should be considered in D2R.

Observation 7: Regarding directions studied for device unavailability

- Direction 1 is simpler and compatible for both device 1 and device 2a but is inefficient and can lead to resource wastage.

- Direction 2 offers low communication delay, efficient resource allocation and optimum use of device capabilities

Proposal 9: Both Direction 1 and Direction 2 for device unavailability should be considered in Rel. 19.

# References

- [1] RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, Huawei (moderator, RAN1 Vice-Chair), RAN#102, December 2023.

- [2] RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR’’, CMCC, Huawei, T-Mobile USA, RAN#103, March 2024.

- [3] TR 38.769 , “Study on solutions for Ambient IoT (Internet of Things) in NR’’

- [4] R1-2410658, “Discussion on Evaluation assumptions and results’’, CEWiT, RAN1#119, Orlando, November 2024