Annex 1

Specification of the 3GPP 5G-NTN radio interface technology – RIT

Background

[to be filled by ITU-R WP4B]

# Overview of the satellite radio interface technology

The IMT-2020 satellite radio interface specifications, known as 5G NTN, have been developed by 3GPP and consist of IoT-NTN based on long-term evolution (LTE) and NR-NTN based on new radio (NR) as part of its Releases 17 and beyond.

5G-NTN is a set of radio interface technologies (RITs) consisting of IoT-NTN as one component RIT and NR-NTN as the other component RIT.

5G-NTN fulfils all technical performance requirements in all three selected test environments: Rural – enhanced Mobile Broadband (eMBB-s), Rural – massive Machine Type Communication (mMTC-s) and Rural – High Reliability Communications (HRC-s).

Also, 5G-NTN fulfils the service and the spectrum requirements of the IMT2020 SRI. Both component RITs, NR-NTN and IoT-NTN, utilize the frequency bands below 7.125 GHz. In addition, the NR-NTN component RIT can also utilize the frequency bands above 7.125 GHz, i.e. above 10 GHz, including frequency bands allocated to mobile satellite services in the ITU Radio Regulations.

The complete set of standards for the satellite radio interface of IMT-2020 identified as NR-NTN and IoT-NTN is based on the key characteristics of the 3GPP “5G” IMT-2020 terrestrial radio interface [hyperlink to be added by ITU] and the additional NTN capabilities both of which are continuing to be enhanced.

The 3GPP 5G System (5GS) also includes specifications for its non-radio aspects, such as the core network elements (the Evolved Packet Core and 5G Core Networks), security, codecs, network management, etc. These non-radio specifications are not included in the so-called “Global Core Specifications (GCS)” of IMT-2020.

## Overview of the component RIT: NR-NTN

This overview provides the NTN specific enhancements to the 3GPP “5G” IMT-2020 terrestrial radio interface in Recommendation M.2150 [hyperlink to be added by ITU].

A number of features have been introduced into the NR protocol and NG-RAN architecture enabling the support of non-terrestrial networks (NTN). NTN refers to networks, or segments of networks, using a spaceborne vehicle or an airborne vehicle for transmission, defined as follows:

- Spaceborne vehicles: Geosynchronous (GSO) and Non-Geosynchronous (NGSO) orbiting satellites. NGSO includes Low Earth Orbit at altitude approximately between 300 km and 1500 km and Medium Earth Orbit at altitude approximately between 7000 km and 25000 km.

- Airborne vehicles: High Altitude Platforms (HAPs) encompassing Unmanned Aircraft Systems (UAS) including Lighter than Air UAS (LTA), Heavier than Air UAS (HTA), all operating in altitudes typically between 8 and 50 km, quasi-stationary.

The addressed radio specifics of NTN compared to 5G terrestrial networks include

•	Delay variation, Doppler variation as well as possible Earth moving radio cells, due to the motion of space/airborne vehicles

•	Long latency due to the altitude of the space/airborne vehicles

•	Differential delay and possible multi country cell coverage due to larger radio cell size

•	Different propagation channel model

•	Different radio unit performance due to specific payload performance

### Supported bands

The considered operating bands are defined in Table 1 for frequency range FR1 and Table2 for frequency range FR2:

Table 1: Satellite operating bands in frequency range FR1

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |

Table 2: Satellite operating bands in frequency range FR2

| n512 | 27.5 - 30.0 GHz | 17.3 - 20.2 GHz | FDD |
| --- | --- | --- | --- |
| n511 | 28.35 - 30.0 GHz | 17.3 - 20.2 GHz | FDD |
| n510 | 27.5 - 28.35 GHz | 17.3 - 20.2 GHz | FDD |

### Overall architecture and general aspects

As illustrated in Figure 1, non-terrestrial access is provided by means of an NTN payload, i.e. a network node on-board a satellite or HAPS, and an NTN Gateway interconnected by a feeder link, the UE accessing NTN network services through the NTN payload via a service link.

Figure 1: Overall illustration of an NR-NTN

The NTN payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. A gNB may serve multiple NTN payloads while an NTN payload may be served by multiple gNBs.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the NTN gNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while gNB operating with GSO satellite can provide Earth fixed cell coverage.

### Timing, Synchronization and HARQ enhancements

The network broadcast ephemeris information and common Timing Advance (common TA) parameters in each NTN cell. Since NTN capable UE are expected to be all GNSS-capable, they shall acquire a valid GNSS position as well as the satellite ephemeris and common TA before connecting to an NTN cell.

To achieve uplink synchronization, before performing random access, the UE shall autonomously pre-compensate the Timing Advance, as well as the frequency Doppler shift by considering the common TA (information from the gNB), the UE position, the satellite position and satellite velocity through the satellite ephemeris. In connected mode, the UE shall continuously update the Timing Advance and frequency pre-compensation. If the UE does not have a valid GNSS position and/or valid satellite ephemeris, it does not communicate with the network until both are regained. The UEs may be configured to report Timing Advance at initial access or in connected mode. In connected mode triggered reporting of the Timing Advance is supported.

While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE for the uplink, the management of Doppler shift experienced over the feeder link is left to the network implementation.

To accommodate the propagation delay in NTNs, several timing relationships are enhanced by a Common Timing Advance (Common TA) and two scheduling offsets Koffset and kmac . Common TA is a configured offset that corresponds to the Round Trip Time (RTT) between the Reference Point (RP) and the NTN payload. Koffset is a configured scheduling offset that approximately corresponds to the sum of the service link RTT and the common TA. kmac is a configured offset that approximately corresponds to the RTT between the RP and the gNB.

To mitigate the impact of HARQ stalling in NTN the HARQ feedback can be disabled in the presence of ARQ re-transmissions at the RLC layer (e.g., in GSO satellite systems) and/or the number of HARQ processes for re-transmissions at the MAC layer can be increased to 32 (e.g., in NGSO satellite systems).

### Mobility Management

To enable mobility in NTN, the network provides serving cell's and neighbouring cell's satellite ephemeris needed to access the target serving NTN cell in the handover command.

UE supports mobility between NTN and Terrestrial Network (i.e. from NTN to Terrestrial Network (hand-in) and from Terrestrial Network to NTN (hand-out)), but is not required to connect to both NTN and Terrestrial Network at the same time. It may also support mobility between radio access technologies based on different orbit (GSO, NGSO at different altitude).

Triggering conditions upon which UE may execute Conditional Hand-Over (CHO) to a candidate cell, have been introduced: event A4, time-based trigger condition, location-based trigger condition. The two last conditions are configured together with one of the measurement-based trigger conditions. Location is defined by the distance between UE and a reference location. Time is defined by the time between T1 and T2, where T1 is an absolute time value and T2 is a duration started at T1.

For the measurements the network can configure multiple SS/PBCH Block Measurement Timing Configuration (SMTCs) in parallel per carrier and for a given set of cells depending on UE capabilities using propagation delay difference and ephemeris information. It can also configure measurement gaps based on multiple SMTC.

The adjustment of SMTCs is possible under network control based on UE assistance information if available for connected mode and under UE control based on UE location and satellite assistance information (e.g., ephemeris, common TA parameters) for idle/inactive modes.

In the quasi-earth fixed cell scenario, UE can perform time-based and location-based measurement in RRC_IDLE/RRC_INACTIVE. The timing and location information associated to a cell are provided via system information. They refer respectively to the time when the serving cell is going to stop serving a geographical area and to the reference location of serving cell.

A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN. The network may broadcast multiple Tracking Area Codes (TAC) per PLMN in a NR NTN cell in order to reduce the signaling load at cell edge, in particular for Earth-moving cell coverage. A TAC change in the System Information is under network control and may not be exactly synchronized with real-time illumination of beams on ground.

Regarding the UE location aspects, upon network request, after AS security is established in connected mode, a UE should report its coarse UE location information (most significant bits of the GNSS coordinates, ensuring an accuracy in the order of 2 km) to the NG-RAN if available.

### Switch-over

A service link switch refers to a change of serving satellite.

A feeder link switch over is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. The feeder link switch over is a Transport Network Layer procedure. Both hard and soft feeder link switch over are applicable to NTN.

Service and feeder link switch overs apply mostly for the case of NGSO.

### NG-RAN signaling

The Cell Identity, indicated by the gNB to the Core Network as part of the User Location Information corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported. It is used for Paging Optimization in NG interface, Area of Interest and Public Warning Services.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target radio cell as well as for RAN paging.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network. The gNB is responsible for constructing the Mapped Cell ID based on the UE location info received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

The gNB reports the broadcasted TAC(s) of the selected Public Land Mobile Network (PLMN) to the Access and Mobility Management Function (AMF) as part of UE Location Information (ULI). In case the gNB knows the UE's location information, the gNB may determine the Tracking Area Indicator (TAI) the UE is currently located in and provide that TAI to the AMF as part of ULI.

### RF performances and RRM requirements

Figure 2 illustrate the satellite access node which encompass on ground non-NTN infrastructure gNB functions, gateway and feeder link and the RF functions of the NTN payload.

Figure 2: Satellite Access Node (SAN)

RF requirements of an NTN capable UE require the same RF performance as UE operating with terrestrial network.

Note that RF requirements of the SAN are lower compared to the BS RF requirements of a terrestrial network.

Specific requirements for radio resource management in NTN mostly relate to delay as well as timing and frequency errors in the different procedures.

### Other enhancements

An additional set of features have been introduced into the NR and NG-RAN architecture for an optimized support of NTN with the capabilities to

- Offer increased uplink coverage performance especially when addressing commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port)

- Support of Geostationary Orbit (GSO) as well as non-Geostationary Orbit (NGSO) deployment scenarios in the (DownLink: 17.7 -20.2 GHz; UpLink: 27.5 – 30.0 GHz) frequency bands

- Support the verification by the network of the location (i.e. GNSS coordinates) reported by the UE assuming a single satellite in view in order to fulfil the regulatory requirements (e.g., Lawful Intercept, emergency call, Public Warning System)

- Provide NTN-TN and NTN-NTN measurement, mobility and service continuity enhancements considering the NTN characteristics such as large propagation delay and satellite movement

- Define 30 MHz channel bandwidth for n256 and n255 for 5G NR-NTN and introduce the corresponding SAN and UE RF core requirements, limited to Operating Bands and Channel Arrangement requirements for these specifications

This set of enhancements are described here after.

#### Uplink coverage enhancements

To improve NR uplink coverage in NTN, the following enhancements have been introduced:

- PUCCH repetition for Msg4 HARQ-ACK:

  - Supported number of transmissions are 1, 2, 4, 8.

    - If a single value from {2, 4, 8} is configured via SIB, the configured repetition factor is applied.

    - If multiple values from {1, 2, 4, 8} are configured via SIB, one of the multiple values is indicated in DAI field of DCI format 1_0 with CRC scrambled by TC-RNTI.

  - The existing mechanism on repetition slot counting is applied.

  - Frequency hopping mechanism defined for PUCCH transmission for Msg4 HARQ-ACK, in every slot is applied.

  - A RSRP threshold can be configured via SIB when the number of repetitions is configured by SIB. If the RSRP threshold is configured, UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK via Msg3 PUSCH only if measured RSRP is lower than the configured RSRP threshold. If the RSRP threshold is not configured, UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK via Msg3 PUSCH.

  - The repetition factor applied to Msg4 HARQ-ACK is used also for any PUCCH transmission before dedicated PUCCH resource is provided.

- NTN-specific PUSCH DMRS bundling enhancement that enables DMRS bundling in presence of timing drift, whereby UE maintains phase continuity considering effects of transmission delay variation between UE and uplink time synchronization reference point to enable improved channel estimation.

#### NR-NTN deployment in above 10 GHz bands a.k.a FR2-NTN frequency range (17300 MHz – 30000 MHz)

To support NR NTN deployment in FR2-NTN using FDD duplexing mode, the following aspects were considered:

- For operation in FR2-NTN, the value range in ms for K_offset and K-MAC shall be the same as for FR1.

- For operation in FR2-NTN, use a reference SCS of 15 kHz for the indication of K_offset and K_MAC.

- For operation in FR2-NTN, no additional MAC CE TCI application delay is introduced to facilitate mechanical beam steering with VSAT.

- For operation in FR2-NTN, for cell search procedure, Case D and Case E are used to allow FDD operation in bands defined by FR2-NTN without any update to SSB pattern.

- From RAN1 perspective, for operation in FR2-NTN, the granularity used for TA reporting is the same as corresponding to the reference subcarrier spacing applied for K_offset.

Moreover, the following VSAT classes and types have been introduced:

| UE class | UE type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT supporting GSO and LEO with mechanical steering antenna. |
|  | 2 | Fixed VSAT supporting GSO and LEO with electronical steering antenna. |
|  | 3 | Fixed VSAT supporting LEO only with electronical steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT supporting GSO with mechanical steering antenna. |
|  | 5 | Mobile VSAT supporting GSO with electronical steering antenna. |
| Note: Assuming that UE has single beam towards one single satellite at a given time. |  |  |

#### Network verified UE location

Necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view have also been introduced.

For UE location verification based on multi-RTT with single satellite in NTN, at least the following UE and gNB measurements are reported: gNB receive-transmit time difference at the uplink time synchronization reference point, UE receive-transmit time difference, UE receive-transmit time difference subframe offset and DL timing drift.

The assistance information reported to the CN may include ephemeris information including accurate satellite position and velocity at the time of multi-RTT measurement, and common TA parameters (ta-Common, ta-CommonDrift, ta-CommonDriftVariant, Epoch time).

Note that network verified UE location is an optional UE feature. The verification is triggered by the CN and is up to the CN to decide when to trigger the procedure.

#### Mobility Management enhancements

Mobility management enhancements have been introduced to ensure service continuity covers inter-satellite, inter-gNB, inter-RAT.

For the NTN to TN mobility, the Non-Terrestrial network may broadcast NR and EUTRA TN cell coverage areas and cell information in the new SIB25. The coverage information consists in a list of geographical TN areas, with associated frequency information. UE can reduce its power consumption by skipping TN measurement based on the TN coverage information.

For the TN to NTN mobility, the TN cell can broadcast NTN neighbour satellites ephemeris via SIB 19. The UE can use ephemeris to connect to the NTN.

RACH-less handover is supported in NR NTN. It is a L3 mobility procedure that avoids a RACH procedure during the handover (between gNBs or during feeder/satellite switch), reducing random access congestion in the target cell.

Conditional handover associated to a time-based or a location-based trigger condition already introduced for quasi-Earth fixed cells is also supported for Earth-moving cells. For the location condition, the candidate cell’s reference location re-use the cell reselection procedure as the cell moves.  CondEventD2 event is introduced that includes a reference location and distance threshold for source and target cell. Moreover, for the time-based, the CHO procedure can be combined with the RACH-less HO procedure.

Figure 3 Satellite switch with re-sync

Satellite switch with-resync allows the UE to synchronize with the target satellite without handover, i.e. no L3 mobility, no PCI change, upon both hard and soft satellite switch over in the quasi-Earth fixed scenario with the same SSB frequency and the same gNB. CHO can be configured simultaneously. In the soft switch case, the capable UE can start synchronizing with the target satellite before the source satellite ends to serve the cell without required to be connected to both satellites, as long as the SSBs from the source satellite and target satellite are not overlapped in time.

#### Channel bandwidth up to 30 MHz in FR1

In the previous the channel bandwidth was limited to 20 MHz in FR1 bands. With this release, channel bandwidth can be set up to 30 MHz for n256 and n255 frequency bands. The corresponding SAN and UE RF core requirements have been introduced however, limited to Operating Bands and Channel Arrangement requirements.

END

Developed by 3GPP Proponent as “5G-NTN, Release 17 and beyond – 5G-NR RIT”.