3GPP TSG RAN WG1 #118          						R1-2405967

Maastricht, NL, Aug 19th – Aug 23rd, 2024

Source:	Tejas Networks Ltd.

Title:	Discussion on CW waveform characteristics of A-IoT

Agenda item:	9.4.2.4

Document for:     Discussion and Decision

# Introduction

The Ambient IoT (A-IoT) Device type 1 and 2a do not have any dedicated active RF component. The device communicates with the reader using backscatter communication, where the device receives carrier wave (CW) signal from an external RF energy source and backscatter information on the CW signal by switching the load impedance between Z1 and Z2. The base station (BS) or user equipment (UE) can act as a reader and transmit CW signal (CW inside topology) to the device to receive information in PDRCH signal. In CW outside topology, a separate CW transmitter (CWT) transmits a CW signal to the device for backscattering information to the reader through PDRCH signal. The carrier wave (CW) signal transmitted from the base station for backscatter communication of Device 1/2a also can provide RF energy on-demand basis to the device.

# Discussion on CW transmission with large frequency shift

The agreements made on RAN1 #116-bis [1] on multiple unmodulated single-tone CW waveform transmission is produced below:

Agreement

For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI.

- Two unmodulated single-tones as a starting point

  - FFS: Other number of tones

  - FFS: how large gap is needed between tones

Agreement

For multiple unmodulated single-tone transmitted by one CW node, other number of tones (i.e. >2) is deprioritized.

- Note: other number of tones (i.e. >2) is studied only when obvious gains are provided.

The agreements made on RAN1 #117 [2] on multiple unmodulated single-tone CW waveform transmission is produced below:

The BS or UE or CWT should be capable of transmitting at least two unmodulated single-tones to the device for backscatter communication. If CWT transmits at frequency f1, the large frequency shifter module of the device can shift the frequency to f2 and transmit the PDRCH signal at f2 to the reader. The gap between the two tones should be such that the two frequencies are orthogonal to each other, so that the reader can transmit both the CW waveforms at the same time to serve multiple devices. The below options can be studied with two tones for communication between reader and device:

Option 1: The CWT transmits at frequency f1 in DL band (with higher power) to the device and the device can backscatter at frequency f2 in UL band using large frequency shifter.

Option 2: The CWT transmits at both frequencies f1 and f2 simultaneously to two different devices and the devices can communicate with the reader two devices simultaneously at two different frequencies f1 and f2. The large frequency shifter can be omitted; however, a wide band antenna at the device side is required for communicating at both frequencies f1 and f2.

Observation 1: The gap between the two tones should be such that the two frequencies are orthogonal to each other, so that the reader can transmit both the CW waveforms at the same time to serve multiple devices.

Proposal 1: The below options can be studied with two tones for communication between reader and device:

- The CWT transmits at frequency f1 in DL band (with higher power) to the device and the device can backscatter at frequency f2 in UL band using large frequency shifter.

- The CWT transmits at both frequencies f1 and f2 simultaneously to two different devices and the devices can communicate with the reader two devices simultaneously at two different frequencies f1 and f2. The large frequency shifter can be omitted; however, a wide band antenna at the device side is required for communicating at both frequencies f1 and f2.

  - CW transmission for topology 1

Fig. 1: Base station transmitting CW signal at two different frequencies.

Considering option 2, where the CWT (reader or external CW transmitter) can transmit two CW waveforms simultaneously, multiple devices can be served through beamforming. As shown in Fig. 1, two steered beams at frequencies f1 and f2 can be transmitted from the BS simultaneously to communicate with two A-IoT tags places side-by-side. The tags backscatter to incident CW signals at the respective frequencies to transmit the UL data through PDRCH signal. Thus, multiple devices can be served simultaneously without interference while reducing the overall latency.

  - CW transmission for topology 2

Fig. 2: UE transmitting CW signal at two different frequencies.

In topology 2, the UE acts as an intermediate node that communicates with the BS and the device. As shown in Fig. 2, the UE can transmit two CW waveforms simultaneously to communicate with two tags placed side-by-side by transmitting steered beams at two different frequencies. By increasing the number of tones, the number of devices to be served at a time can be increases. However, hardware complexity of the A-IoT device will also increases. Therefore, the number of tones > 2 is deprioritized.

Observation 2: Increases the number of tones > 2, increases the hardware complexity of the device. Increasing the bandwidth of the power amplifier and the antenna, increases the power consumption of the device.

Proposal 2: The CWT can transmit two steered beams of CW waveforms at frequencies f1 and f2 simultaneously to communicate with two A-IoT devices placed side-by-side. The tags backscatter to incident CW signals at the respective frequencies to transmit the UL data through PDRCH signal. Thus, multiple devices can be served simultaneously without interference while reducing the overall latency.

- Conclusion

This study includes the discussion on CW transmission for A-IoT backscatter communication with large frequency shift for both topology 1 and topology 2. We propose to transmit two unmodulated single-tones simultaneously to serve two devices at a time. Thus, multiple devices can be served simultaneously without interference while reducing the overall latency. The observations and proposals related to the CW transmission of A-IoT are as follows:

Observation 1: The gap between the two tones should be such that the two frequencies are orthogonal to each other, so that the reader can transmit both the CW waveforms at the same time to serve multiple devices.

Proposal 1: The below options can be studied with two tones for communication between reader and device:

- The CWT transmits at frequency f1 in DL band (with higher power) to the device and the device can backscatter at frequency f2 in UL band using large frequency shifter.

- The CWT transmits at both frequencies f1 and f2 simultaneously to two different devices and the devices can communicate with the reader two devices simultaneously at two different frequencies f1 and f2. The large frequency shifter can be omitted; however, a wide band antenna at the device side is required for communicating at both frequencies f1 and f2.

Observation 2: Increases the number of tones > 2, increases the hardware complexity of the device. Increasing the bandwidth of the power amplifier and the antenna, increases the power consumption of the device.

Proposal 2: The CWT can transmit two steered beams of CW waveforms at frequencies f1 and f2 simultaneously to communicate with two A-IoT devices placed side-by-side. The tags backscatter to incident CW signals at the respective frequencies to transmit the UL data through PDRCH signal. Thus, multiple devices can be served simultaneously without interference while reducing the overall latency.

# 4. References

[1] 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #116-bis, Changsha, Hunan Province, China, April 15th – 19th, 2024.

[2] 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #117, Fukuoka, Japan, May 20th – May 24th, 2024.