3GPP TSG RAN WG1 #118		R1- 2406175

Maastricht, NL, August 19th – 23rd, 2024

Source:	vivo

Title:	Discussion on CSI compression

Agenda Item:	9.1.3.2

Document for:	Discussion and Decision

Introduction

In RAN #102 meeting, further study objectives on AI CSI compression have been identified in the WID [1]:

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

In this contribution, we discuss the key issues involved in the objectives and present our views as well as simulation results.

Issues related to inter-vendor training collaboration

In terms of issues related to inter-vendor training collaboration, following key agreements and conclusions have been achieved in RAN1 #117:

Conclusion Standardized signalling, if feasible and specified, can be used for parameter / model exchange in option 3a/5a and 3b to alleviate/resolve the inter-vendor training collaboration complexity. - Standardized signalling may be reused for exchanging CSI generation part, CSI reconstruction part, or both, etc, when necessary and feasible. - Standardized signalling may be over-the-air, or other approaches. Standardized signalling, if feasible and specified, can be used for dataset exchange in option 4 to alleviate/resolve the inter-vendor training collaboration complexity. - Standardized signalling may be reused for dataset exchanging, when necessary and feasible. - Standardized signalling may be over-the-air, or other approaches. Note: feasibility will be discussed separately. Agreement - For option 3a/3b/4/5a and their sub-options, at least the following potential specification impacts have been identified. Further study the necessity, feasibility, their specification impact. - Exchange - Parameter / model exchange methods, format/contents, and related spec impacts (3a/3b/5a) - Dataset exchange methods, format/type/contents of data/dataset, and related spec impacts (4) - Additional information, if necessary, that may be shared from the NW-side to help UE-side offline engineering and provide performance guidance (3a/5a/4) - Performance target (3a/5a/4) - Dataset or information related to collecting dataset (3a/5a) - Any other additional information - Model pairing (3a/3b/4/5a) - UE capability (3a/3b/4/5a) - Model related aspects, such as scalability (e.g., payload sizes, antenna ports, bandwidth), rank and layer handling (3a/3b/4/5a) - Quantization of feedback (3a/3b/4/5a) - Model structure details (3a/3b) Note: Option 3a/4/5a and option 3b serve two different deployment time scales, UE capabilities, device-side optimizations, and training methods, and therefore may be complementary to each other, with potential specification of both. - Specification of option 1, if needed from RAN1, can reuse specification of option 3a/3b, with the additional specification of parameters. Agreement For option 1 / 3 / 4 / 5 and their sub-options, study mechanisms (e.g., post-deployment performance monitoring) for identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation to guarantee good performance in the field.

Generally, we believe that some progress has been made towards consensus for inter-vendor training collaborations. An important conclusion acknowledges that standardized signalling (if feasible and specified) can be used for parameter/model exchange in options 3a/5a and 3b to alleviate/resolve the inter-vendor training collaboration complexity. In the following, we continue to discuss the remaining issues and present our prospects towards future work.

Identifying the cause of performance degradation

One of the major issues remaining yet undiscussed is how to identify the cause of performance degradation in two-sided CSI compression cases, where possible causes include NW side (cause), UE side (cause), data drift, or any combinations of them. Generally, when some performance degradation has been observed during monitoring phase, it would be beneficial for the feature to deploy if NW and/or UE could something to help to find the cause of performance degradation.

Since both NW- and UE-side monitoring are under discussion now, we would like to discuss cause identification for both methods. Note that we assume the NW side dominates the training of two-sided model (i.e., model/parameter/dataset exchange originates from NW to UE side) for the following analysis, as suggested in the agreements in RAN1 #116bis.

- Cause identification with NW-side monitoring

- Illustration of cause identification with NW-side monitoring

The procedure of NW monitoring is the same for option 1/3/4/5 (at least for inter-mediate KPI based monitoring), i.e., UE reporting ground-truth CSI to help NW compute the inter-mediate KPI. Assuming an obvious performance degradation for ongoing model has been observed by NW, NW side can:

a) check whether NW side CSI reconstruction model is correctly deployed by comparing the expected output and real output given the same input. Actually, checking CSI reconstruction model does not rely on real-time measurement, which can be done at any time.

b) compare the performance between NW-side original model (i.e., green part in the above Fig. 1) with reported ground-truth CSI from UE (NW-side original model refers to the model firstly trained by NW, and the possible model transfer/dataset delivery in option 3/4/5 are all based on the original model) and ongoing model (i.e., UE-side generation and NW-side reconstruction). If the performance of NW-side original model is satisfying while that of ongoing model is not good, we can identify that at least UE side causes the degradation. If the performance of NW-side original model also degrades, we can identify that at least data drifts happen for the ongoing model.

c) compare reconstructed target CSI based on the expected CSI feedback (i.e., output of NW-side original model) or reconstructed target CSI’ based on reported CSI feedback. If the reconstructed target CSI and reconstructed target CSI’ is not aligned (e.g., the difference is larger than the threshold), we can identify it’s UE side cause for the degradation. Otherwise, the UE side cause can be excluded. Or, we can compare the expected CSI feedback (i.e., output of NW-side original model) or reported CSI feedback. If the expected CSI feedback and reported CSI feedback is not aligned (e.g., the difference is larger than the threshold), we can identify UE side cause for the degradation. Otherwise, the UE side cause can be excluded. Note that to guarantee a fair comparison between expected and reported CSI feedback we may need to further study the necessity of taking quantized ground-truth as UE side model input for the reported CSI feedback.

Based on the above analysis, we can observe that identifying the cause of performance degradation is not a challenge for NW-side monitoring, where the reported ground-truth CSI can help to identify the cause.

- For Option 3b (and potentially Option 3a/5a/4, if needed), the following methods can be considered to identify the cause of the performance degradation assuming NW-side monitoring

- Identifying data drift: with ground-truth CSI reported from UE, NW could check the SGCS of NW-side original model

- Identifying UE side cause:

  - With ground-truth CSI reported from UE, NW compares expected CSI feedback and reported CSI feedback (FFS the necessity of taking quantized ground-truth as UE side input), and/or

  - NW compares Intermediate KPI of ongoing model and NW-side original model

- Identifying NW side cause: NW could check whether NW side decoder is correctly deployed by comparing the expected output and real output given the same decoder input

- Cause identification with UE-side monitoring

- Illustration of cause identification for UE-side monitoring

Identifying the cause of performance degradation for UE-side monitoring will be more complicated. Basically, it is challenging for NW side to identify whether it is data drift and/or UE side cause, because ground-truth CSI is not available at NW side for UE side monitoring. The only thing that NW can do is to check whether NW side CSI reconstruction model is correctly deployed by comparing the expected output and real output given the same input. Meanwhile, it is also not easy for UE to identify data drifts, since UE can hardly acquire the original CSI generation and reconstruction model simultaneously. For UE-side cause identification, we may further discuss according to the training collaboration considered (also illustrated in Fig. 2):

a) If the original CSI generation model is available at UE side (e.g., for option 3a-1/5a-1 and option 3b), UE could compare the expected output of transferred CSI generation model and real output of deployed CSI generation model to identify whether it is the UE side cause for the performance degradation. We believe that the comparison of expected output and real output is effective enough for cause identification, because model optimization at UE side usually targets mimicking the original CSI generation model with another model structure in most cases where it is available at UE side.

b) If the original CSI generation model is not available at UE side (e.g., for option 3a/5a-2 and option 4), it is challenging for UE to identify whether the deployed UE side model is correctly developed. In such cases, assistance information such as testing datasets from NW can help to identify UE side cause.

If there is performance degradation but no gNB-side or UE-side causes are identified, the cause of the performance degradation would then should be determined as data drift.

Based on the above analysis, we can find that identifying data drifts could be challenging at both NW and UE when considering UE-side monitoring. However, there are still some approaches to identify UE side cause (at UE side) especially for option 3a/5a-1 and 3b.

- For UE-side monitoring, towards identifying the cause of performance degradation, we have the following considerations (assuming NW first training for option 3/4/5)

- Identifying UE side cause:

  - For option 3a/5a-1 and 3b, UE could check the expected PMI of transferred CSI generation model and deployed CSI generation model

  - If original CSI generation model is not available at UE side (e.g., option 3a/5a-2 and option4), assistance information such as testing dataset from NW can help to identify UE side cause

- Identifying NW side cause: NW could check whether NW side decoder is correctly deployed by comparing the expected output and real output given the same decoder input

- If no UE side cause or NW side cause are identified, it can be determined that the performance degradation is due to data drift.

On the specification of model structure in Option 3a/3b

In RAN1 #117, it has been concluded that standardized signalling, if feasible and specified, can be used for parameter / model exchange in option 3a/5a and 3b to alleviate/resolve the inter-vendor training collaboration complexity. Meanwhile, among all related spec impacts as suggested in the agreement, we believe that the specification of model structure for option 1/3/5 will be one of the most important issue. Therefore, our stance is that we should focus on defining model structure from RAN1 perspective during the remaining time of R19, aiming to draw some conclusions at least on the feasibility and/or roadmap.

We notice that RAN4 group is achieving good progress on specifying model structure to facilitate the testing of two-sided models. Specifically, several initial reference model structures have been aligned (e.g., A CNN model with corresponding necessary hyper-parameters) and companies are encouraged to submit their evaluation results for the aligned reference model structure. While companies may have the comment that reference model structure in RAN1 could be different from that in RAN4, we believe that ongoing work in RAN4 could undoubtedly be a good reference for RAN1 to study how to specify a reference model in 3GPP. Inspired by this, we present our views on how to study the specification of reference model structure in RAN1:

- Step 0: Aligning evaluation assumptions. Previous aligned simulation assumptions could be used as a starting point. Take a step further, it would be useful to align the dataset containing only channel information. Companies could bring their own generated dataset and multiple datasets from different companies can be merged into one dataset, which is the aligned dataset. This dataset can be generated through 3GPP synthetic channel models. For CSI compression, this dataset only needs the channel information, i.e. the input for the encoder or the output for the decoder.

- Step 1: Determine the model backbone based on consensus and evaluation results on complexity and performance. Model complexity may include Flops or model storage size, which can be restricted to avoid exceeding hardware capabilities and also serve as KPI of model quality. When evaluating model quality, it is important to consider not only its performance but also its implementation complexity. During this process, some restrictions may be aligned directly related to model complexity, e.g., model backbone and some important hyperparameters.

- Step 2: Determine the model hyperparameters that need to be aligned. Below are some important hyperparameters that need to be aligned for the model:

  - Number of layers in the neural network.

  - Number of neurons in each layer.

  - Activation function(s) for each layer.

  - Configuration of normalization layers.

  - Special connection relationships between layers:

    - Residual connections

    - Merges or branches

  - Special hyperparameters for CNN:

    - Parameters for the convolutional layers such as kernel size, stride, padding, activation function, bias, and channel number.

  - Special hyperparameters for Transformer:

    - Implementation method for multi-head attention, parameters for multi-head attention such as number of heads and dimensions of heads.

- Step 3: Align the hyperparameters of the model. Based on the aligned model backbone, detailed hyperparameters would be further aligned based on consensus and evaluation results on complexity and performance. If the dataset with only channel information is aligned in Step 1, each company could provide their own model trained by the aligned dataset, and then the best model structure may be selected from these models.

Then we have the following proposal

- RAN1 considers specifying the reference model structure as a starting point for the specification of a reference model.

- Towards the specification of reference model structure, the following procedures can be considered in RAN1:

      - Step 0: Aligning evaluation assumptions

      - Step 1: Determine the model backbone based on consensus and evaluation results on complexity and performance.

      - Step 2: Determine the model hyperparameters that need to be aligned.

      - Step 3: Align the hyperparameters of the model.

Study conclusion on inter-vendor training collaboration

As RAN1 #118 is the last group meeting before the agreed check point for the study phase in R19, we believe that it is the time to conclude inter-vendor collaboration study in this meeting and recommend the followed-up work/study. Our views towards each option are summarized in the following table:

- Characteristics of options to alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression can be summarized as:

|  | Inter-vendor collaboration complexity | Performance | Interoperability and RAN4 / testing related aspects | Feasibility | Deployment timescale |
| --- | --- | --- | --- | --- | --- |
| Option 1 | Minimum complexity | Restricted | Solved | feasible | \ |
| Option 3b | Minimum complexity with over the air signalling; | Optimum | Solved | Feasible | Short |
| Option 3a/5a | High complexity in server to server manner; Medium complexity with over the air signalling; | Optimum | Not solved | Infeasible with only short time scale model update | Long |
| Option 4 | High complexity in server to server manner; Medium complexity with over the air signalling; | Better than Option 1, but worse than Option3 and Option5 | Not solved | Infeasible with short time scale model update | Long |
| Option 5b | Need clarification |  |  |  |  |

Based on the summary, we believe that option 3b enjoys some advantages at least in inter-vendor collaboration complexity, performance, inter-operability, and deployment timescale, while its feasibility can be guaranteed. Therefore, we propose to support option 3b to address inter-vendor training collaborations. In addition, as analysed in subsection 2.2, we also propose to study how to specify reference model structure in RAN1 in the following agenda of R19 with high priority. One possible workplan towards this objective has been given in our proposal.

- RAN1 concludes that it is recommended to support option 3b to address inter-vendor training collaboration for CSI compression

Improving trade-off between performance and complexity/overhead

In this section, we continue to discuss the trade-off between performance and complexity/overhead.

Localized models

Agreement For the evaluation of AI/ML-based CSI compression using localized models in Release 19, regarding training, - The k-th local model is trained on region #B_k (the k-th local region), 1<=k<=N. - The generalized model is trained on Region #A that may be constructed via any of the following methods that is appropriate for the given generalized/local region modeling approach. - Region #A is the same as the union of regions #B_1, …, #B_N. - Region #A is a proper superset of the union of regions #B_1, …, #B_N. - Region #A is generated separately from regions #B_1, …, #B_N. - Note: companies to report which method was used. For the evaluation of AI/ML-based CSI compression using localized models in Release 19, regarding testing, - The trained generalized model, local model, and the non-AI/ML benchmark are tested on the regions #B_1, …, #B_N. - In case N>1, when reporting the results, companies may report the performance of the generalized model, the local models, and the non-AI/ML benchmark, by averaging the performance over the regions #B_1,…,B_N. Companies to report the value of N. Agreement For collecting evaluation results for temporal domain aspects of AI/ML-based CSI compression using localized models, use the same results template used to collect evaluation results for AI/ML-based CSI compression using localized models - Adding the same temporal setting that is used for results template used to collect evaluation results for temporal domain compression Case 1/2/5. Temporal setting; Temporal domain aspect Case 1-5. CSI-RS configuration: periodic or aperiodic For periodic: periodicity For aperiodic: # of resources K in the CSI-RS burst / time internal m in msec. CSI reporting periodicity. Usage of historical CSI at UE side: number / time distance. Usage of historical CSI at NW side: number / time distance. Prediction window: number / time distance between prediction instances / distance from the last observation instance to the 1st prediction instance (Only applicable to Case 3,4).

RAN1 #117 continued to clarify the evaluation methodology of localized model. On the one hand, a clarified definition of generalized model is provided to facilitate a fair comparison for localized model. On the other hand, companies can submit their localized model results in the case of TSF compression. In this section, we will update our results according to the agreements.

Evaluation assumptions

According to the agreements in RAN1 #117, the dataset for generalized model can be constructed in the following ways: 1) Region #A is the same as the union of regions #B_1, …, #B_N; 2) Region #A is a proper superset of the union of regions #B_1, …, #B_N; 3) Region #A is generated separately from regions #B_1, …, #B_N. In our simulation, Region A is a dataset with CSI from 21 cells, and Region B_x is a dataset with CSI from each 3-cell (i.e., a site). That is to say, we consider Region A the same as the union of Region B_x. More detailed evaluation parameters can be referred to the following table, which hold for both SF and TSF compression results.

Simulation parameters for SF compression with localized models

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban (Macro only) |
| Frequency Range | 4GHz |
| Inter-BS distance | 200m |
| Channel model | According to TR 38.901 with spatial consistency |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8)λ for Dense Urban |
| Antenna setup and port layouts at UE | 4 ports: (1,1,2,1,1,2,1), (dH, dV) = (0.5, 0.5)λ |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| SCS | 30kHz for 4GHz |
| Simulation bandwidth | 20 MHz for 30kHz |
| CSI feedback | CSI feedback periodicity (full CSI feedback): 5 ms |
| CSI-RS | CSI-RS periodicity: 5ms |
| UE distribution | Dense Urban: Configuration 1: 100% outdoor (30km/h) Configuration 2: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| Feedback assumption | ideal |
| Channel estimation | Ideal DL channel estimation |
| Baseline for performance evaluation | Rel-16 Type II Codebook |
| Rank number | 2 |
| Spatial consistency | On |
| Dataset size for training and inference | 200, 000 randomly dropped UEs per region |
| General model setting | Region #A is the same as the union of regions #B_1, …, #B_N. |

And an illustration for the considered data collecting region can be found in the following Fig. 3.

- Schematic of localized data collection for UMa scenarios

In addition to the simulation parameters, we would also like to introduce our model design for TSF compression for information. As demonstrated in Fig. 4, the basic idea is to add an internal information flow between the compression of successive PMIs on top of SF model. The internal information flow originates from some deep feature (i.e., closer to output) in the processing of previous PMI and ends at some shallow feature (i.e., closer to input) in the processing of current PMI. It is similar to the LSTM design but with smaller complexity and fewer parameters (actually, the complexity/parameter-scale of such TSF model is almost the same as those for the corresponding SF model). Such internal information applies both for CSI generation and reconstruction part. For training strategy, we let the model compress and reconstruct 6 PMIs in an auto-regressive manner, and the loss function is the averaged SGCS of all 6 PMIs. We consider fewer PMIs during training stage to reduce the complexity, and we also observe that 6 PMIs are enough for the model to converge to a relative stable performance.

- Illustration of TSF model design considered in our evaluation of localized model (only CSI generation part depicted, but the same design also applied for CSI reconstruction part)

Results on SF compression with localized models

We first consider SF compression with localized model. Firstly, the impact of temporal variations is investigated. Specifically, we consider two cases: 1) cell/site specific model trained on data starting from slot0, where influence of doppler shift is not included; 2) cell/site specific model trained on data starting from slot1000 or later, where influence of doppler shift is included. The results are given below:

Results for localized model trained based on data from slot0 (Dense urban with 100% outdoor UEs, and layer1 considered)

| Scenario: Dense urban with 100% outdoor UEs | R16 eT2 CB (PC1) | SF compression (64payload) | R16 eT2 CB (PC3) | SF compression (104payload) |
| --- | --- | --- | --- | --- |
| Localized model#1 | 0.627 | 0.926 (+47.6%) | 0.699 | 0.928 (+34.1%) |
| Localized model#2 | 0.779 | 0.972 (+24.7%) | 0.829 | 0.978 (+17.9%) |

Results for localized model trained based on data after slot1000 (Dense urban with 100% outdoor UE, layer1 PC1)

| Scenario: Dense urban with 100% outdoor UEs | R16 eT2 CB | SF compression |
| --- | --- | --- |
| General model | 0.718 | 0.737(+2.6%) |
| Localized model#3 | 0.679 | 0.750(+10.4%) |

Generalization of localized model over time (Dense urban with 100% outdoor UEs considered, PC1)

| Scenario: Dense urban with 100% outdoor UEs | R16 eT2 CB | Model trained on ~1k slots |
| --- | --- | --- |
| Test on PMI from ~slot1k | 0.679 | 0.737(+10.4%) |
| Test on PMI from ~slot10k | 0.679 | 0.736(+10.3%) |

Firstly, it can be observed in Table 2 and Table 3 that cell model could offer significant performance gain over generalized model on slot0 data (i.e., when influence of doppler shift is not included), while the gain reduces to a normal level on ~slot1000 data where influence of doppler shift is included. We conjecture one of the potential reasons for such phenomenon is that doppler shifts introduce diversity of data distribution, especially for those samples located in similar positions. Nevertheless, we emphasize that cell specific model is still able to obtain additional gains compared to the general model studied in R18 under the impact of doppler shifts. We then check the generalization capability over different slots for models trained on data from ~1k slots, and the results are provided in Table 4. It can be observed that there is almost no performance degradation when testing the model on data from slot10k, indicating a good generalization of the model over slots with large gaps. Therefore, we will consider to use data from ~slot1k to train localized models in the remaining part of the section.

Next, we consider averaging the performance over the regions #B_1,…,B_N to stabilize the statistic results. Specifically, detailed results for all site-specific SF compression models are presented in the Table 5. It can be observed that the performance of localized models does vary obviously in different sectors, ranging as high as 0.9+ to as low as 0.7. However, after averaging over all sectors, the performance will converge to a stable value, which is more insightful for companies to reproduce the results.

More results on scenario with 100% outdoor UEs

| Cell ID | eT2 CB PC1 SGCS | SF SGCS |
| --- | --- | --- |
| 1 | 0.758 | 0.800 |
| 2 | 0.881 | 0.913 |
| 3 | 0.899 | 0.935 |
| 4 | 0.704 | 0.749 |
| 5 | 0.820 | 0.849 |
| 6 | 0.738 | 0.802 |
| 7 | 0.702 | 0.763 |
| 8 | 0.789 | 0.866 |
| 9 | 0.630 | 0.682 |
| 10 | 0.637 | 0.686 |
| 11 | 0.666 | 0.744 |
| 12 | 0.787 | 0.839 |
| 13 | 0.665 | 0.725 |
| 14 | 0.791 | 0.711 |
| 15 | 0.797 | 0.822 |
| 16 | 0.797 | 0.834 |
| 17 | 0.770 | 0.819 |
| 18 | 0.560 | 0.614 |
| 19 | 0.694 | 0.740 |
| 20 | 0.805 | 0.854 |
| 21 | 0.676 | 0.726 |
| All cell | Mean:0.741 | Mean:0.790(+6.6%) |

- SF compression with cell/site specific model provides up to 6.6% gain compared to legacy codebook in Uma scenario with 0% indoor UE distribution while general model only provides 2.6% SGCS gains.

In addition, we also consider the case of Dense urban Uma with 80% indoor UEs and InH with 100% indoor UEs. While the illustration of Uma scenario has been provided, the illustration of localized model for InH scenario can be found in the following Fig. 5.

- Schematic of cell/site data collection area for InH

Towards the case of InH with 100% indoor UEs, we draw observations based on results from three sectors.

Results for cell/site specific model trained based on data after slot1000 (InH with 100% indoor UEs considered)

| Scenario: InH with 100% indoor UEs | R16 eType II CB | SF compression |
| --- | --- | --- |
| General model | 0.781 | 0.832 (+6.5%) |
| Cell/site model on region#1 | 0.739 | 0.811(+9.7%) |
| Cell/site model on region#2 | 0.746 | 0.811(+8.7%) |
| Cell/site model on region#3 | 0.757 | 0.824(+8.9%) |

- SF compression with cell/site specific model provides up to 9.7% gain in certain case (cell/sector) compared to legacy codebook in InH scenario with 100% indoor UE distribution while general model provides 6.5% SGCS gains.

Results on TSF compression with localized models

In this subsection, TSF compression with localized models will be discussed, and we mainly focus on TSF compression Case2. Similar to the discussion of SF compression with localized model, we also investigate the impact of temporal variations first. As mentioned in the previous part, we consider to collect data after slot 1k to include the influence of doppler shift, and the results of generalization over different slots are provided below.

Results for TSF compression compared between General model and localized model (Dense urban considered, 8:2 indicates that the ratio between indoor and outdoor UEs is 8:2, PC1 per slot)

| Dense urban(8 | R16 eT2 CB | TSF compression with CNN encoder | TSF compression with transformer encoder |
| --- | --- | --- | --- |
| General model (8:2) | 0.720 | 0.683(-5.1%) | 0.745(+3.5%) |
| Localized model (8:2) | 0.596 | 0.720(+20.6%) | 0.758(+26.9%) |

Generalization of localized model over time (Dense urban considered, 8:2 indicates that the ratio between indoor and outdoor UEs is 8:2, PC1 per slot)

| Scenario: Dense urban with 100% outdoor UEs | R16 eT2 CB | Localized model trained on ~1k slots |
| --- | --- | --- |
| Test on PMI from ~slot1k | 0.596 | 0.758(+26.9%) |
| Test on PMI from ~slot10k | 0.596 | 0.755(+26.3%) |

From results presented in Table 7 and Table 8, it can be first observed that TSF compression with cell/site model has additional gain compared to the general model, and the additional gain in some regions (random seeds) is very promising compared to the general model. For example, in the UMa dense urban scenario with 8:2 indoor&outdoor UE distribution, the gain can be enlarged from 3.5% to 26.9%. We acknowledge that variations in performance gain will occur, and we will discuss the issue in the next part. Meanwhile, we find that the generalization of TSF localized models over different slots is also satisfying, as little performance loss can be observed when model trained on slot ~1k is tested on slot ~10k. In addition to the results under a powerful transformer encoder, we also present the results under a relatively simple CNN encoder. The results in Table 8 suggests that even with an encoder with simpler structure, TSF compression with cell/site specific models can still achieve higher performance than general model with powerful encoder structure (Note that when considering different cells/sectors, the observed performance gain might vary. We will provide the averaged stable results in the following meeting.). On the contrary, if CNN encoder is considered, TSF compression with a general model provide almost no performance gain. That is to say, it is possible to simplify model structure design with cell/site specific models, which is obviously beneficial to inter-vendor collaborations.

- When TR 38.901 channel model is considered, localised TSF compression (case2) models trained on data including the impact of Doppler shift generalize well over different slots even with large gaps (e.g., slot1k training vs. slot10k testing).

Then we show detailed performance of localized model on each site, which is given in Table 9. All results will be averaged to derive a stable value for observation.

TSF compression (case2) with localized model (indoor UE:ourdoor UE=8:2, layer1/layer2 results are given)

| Site ID | TSF localized model pc1 | TSF localized model pc3 | TSF localized model pc6 |
| --- | --- | --- | --- |
| 1 | 0.76/0.62 | 0.84/0.73 | 0.92/0.86 |
| 2 | 0.82/0.69 | 0.85/0.74 | 0.92/0.87 |
| 3 | 0.82/0.72 | 0.88/0.75 | 0.94/0.90 |
| 4 | 0.81/0.69 | 0.84/0.74 | 0.92/0.86 |
| 5 | 0.75/0.61 | 0.85/0.74 | 0.92/0.86 |
| 6 | 0.80/0.71 | 0.85/0.77 | 0.93/0.89 |
| 7 | 0.83/0.70 | 0.86/0.75 | 0.93/0.88 |
| Averaged results | 0.80/0.67 | 0.86/0.76 | 0.93/0.88 |

Table 9 reveals that performance of localized models varies a lot in different cells/sectors, possibly due to the varying ratios of indoor UEs (with 3kmph velocity) and outdoor UEs (with 30 kmph velocity). Intuitively, TSF compression can offer higher additional gain on UEs with low velocity, as the temporal domain correlations in CSIs is larger given the same measurement periodicity. Our understanding is that when collecting data within a region the dropped UEs can also follow very different indoor/outdoor ratios even with the same expected one (Specifically, this is because in the case of spatial consistency modelling, in order to achieve the effect that the channel is slowly changing within a similar space or location, the random number that determines the type of distribution of the UE is affected by the random seed in the simulation as well as by the location of the UE, so that the channels within a similar space and location have a certain degree of similarity.). Nevertheless, we emphasize that if we average the results over all sites, we can obtain a much stable result which is consistent in different seeds. Therefore, we can make observations based on the averaged results, and it can be foreseen that the results from companies will also follow the similar trend.

Finally, we present our throughput results for localized SF compression model, which is given in the following Table 10. From our results, it can be verified that localized model could also have an obvious gain in system level throughputs, which is almost consistent with those observed in inter-mediate KPI.

Throughput Results for localized TSF compression model (indoor UE:ourdoor UE=8:2, rank2 results)

| Scenario: Dense urban with 100% outdoor UEs | R16 eT2 CB | General TSF compression model | Localized TSF compression model (averaged over B_N) |
| --- | --- | --- | --- |
| SGCS results (layer1/layer2, PC1) | 0.69/0.51 | 0.73/0.59 (+5.8%/15.7%) | 0.80/0.67 (15.9%/31.4%) |
| Full buffer mean UPT SE in bps/Hz (PC1) | 100% | 110.9% | 115.2% |
| Full buffer 5% UPT SE in bps/Hz (PC1) | 100% | 114.7% | 117.6% |
| SGCS results (layer1/layer2, PC3) | 0.76/0.63 | 0.77/0.64 (+1.3%/1.5%) | 0.86/0.76 (+13.2%/20.6%) |
| Full buffer mean UPT SE in bps/Hz (PC3) | 100% | 104.1% | 110.8% |
| Full buffer 5% UPT SE in bps/Hz (PC3) | 100% | 102.7% | 112.8% |
| SGCS results (layer1/layer2, PC6) | 0.84/0.75 | 0.87/0.78 (+3.6%/4.0%) | 0.93/0.88 (+10.7%/17.3%) |
| Full buffer mean UPT SE in bps/Hz (PC6) | 100% | 105.2% | 111.9% |
| Full buffer 5% UPT SE in bps/Hz (PC6) | 100% | 110.3% | 120.5% |

We have the following observation related to TSF localized model:

- TSF compression case2 with cell/site specific model provides 10.7%~15.9% layer1 SGCS gain and 17.3%~31.4% layer2 SGCS gain compared to legacy codebook in Uma scenario with 80% indoor UE distribution while general model only provides 1.3%~5.8% SGCS gain for layer1 and 1.5%~15.7% SGCS gain for layer2.

- TSF compression case2 with cell/site specific model provides 10.8%~15.2% SE gain in full buffer mean UPT compared to legacy codebook in Uma scenario with 80% indoor UE distribution while general model only provides 4.1%~10.9% SE gain.

- TSF compression case2 with cell/site specific model provides 12.8%~20.5% SE gain in full buffer 5% UPT compared to legacy codebook in Uma scenario with 80% indoor UE distribution while general model only provides 2.7%~14.7% SE gain.

Complexity study on TSF compression

Agreement For temporal domain aspects Case 3/4, change the small / medium / large payload region definition as follows: Note: X, Y, Z, A, B, and C are feedback overhead rates in bits per time unit of 5ms. Note: For X, Y, and Z, α=[2] for rank=1/2 and α=[4] for rank=4 Note: For A, B, and C, β=[0.5] for rank=1 and β=[0.75] for rank=2/4. Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression (Cases 1-5), in addition to FLOPs, also consider FLOPs per normalized time unit. Use 5msec as the normalized time unit. Agreement In the results template for capturing the evaluation of temporal domain aspects Case 3/4 of AI/ML based CSI compression, regarding the “upper bound”, capture both of the following: - upper bound based on ideal CSI prediction and without CSI compression - upper bound based on benchmark CSI prediction and without CSI compression

Complexity issue has been identified as one of the critical topics in the WID for R19 AI CSI study. In RAN1 # 117, some agreements towards the evaluation of model complexity have been made to facilitate companies to contribute on this issue. In this subsection, we would like to present our observations on how TSF compression helps to achieve better trade-off between performance improvement and complexity. The considered simulation settings are listed in the follow table:

Simulation parameters for complexity reduction study

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban (Macro only) |
| Frequency Range | 4GHz |
| Inter-BS distance | 200m |
| Channel model | According to TR 38.901 with spatial consistency |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8)λ for Dense Urban |
| Antenna setup and port layouts at UE | 4 ports: (1,1,2,1,1,2,1), (dH, dV) = (0.5, 0.5)λ |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| SCS | 30kHz for 4GHz |
| Simulation bandwidth | 20 MHz for 30kHz |
| CSI feedback | CSI feedback periodicity (full CSI feedback): 5 ms |
| CSI-RS | CSI-RS periodicity: 5ms |
| UE distribution | Dense Urban: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| Feedback assumption | ideal |
| Channel estimation | Ideal DL channel estimation |
| Baseline for performance evaluation | Rel-16 and Rel-18 Type II Codebook |
| CSI prediction algorithm (if applicable) | Ideal or AR CSI prediction |
| Rank number | 1 or 2 |

We first investigate how TSF compression case2 help to reduce model complexity, and our results provided below:

Results for TSF compression case2 (PC1)

|  | R16 eT2 CB | SF compression model0 | SF compression model1 | TSF compression |
| --- | --- | --- | --- | --- |
| Complexity (Flops/parameter scale if applicable) | 1.043M/not applicable | 10M/0.4M | 311M/12.1M | 16M*/0.4M |
| SGCS result (layer1/layer2) | 0.686/0.511 | 0.67/0.52 (-2.4%/+1.7%) | 0.70/0.56 (+2.04%/9.58%) | 0.755/0.613 (+10.1%/20.0%) |
| full buffer mean UPT SE in bps/Hz | 100% | / | / | 115.2% |

Results for TSF compression case2 (PC3)

|  | R16 eT2 CB | SF compression model0 | SF compression model1 | TSF compression |
| --- | --- | --- | --- | --- |
| Complexity (Flops/parameter scale if applicable) | 1.064M/not applicable | 10M/0.43M | 311M/12.2M | 17M*/0.43M |
| SGCS result (layer1/layer2) | 0.759/0.631 | 0.72/0.58 (-5.2%/-8.1%) | 0.77/0.62 (+1.5%/-1.8%) | 0.792/0.667 (+4.34%/5.70%) |
| full buffer mean UPT SE in bps/Hz | 100% | / | / | 109.3% |

Results for TSF compression case2 (PC6)

|  | R16 eT2 CB | SF compression model0 | SF compression model1 | TSF compression |
| --- | --- | --- | --- | --- |
| Complexity (Flops/parameter scale if applicable) | 1.064M/not applicable | 10M/0.47M | 311M/12.5M | 17M*/0.47M |
| SGCS result (layer1/layer2) | 0.846/0.754 | 0.82/0.71 (-3.1%/-5.8%) | 0.87/0.77 (+2.83%/2.12%) | 0.875/0.793 (+3.4%/5.17%) |
| full buffer mean UPT SE in bps/Hz | 100% | / | / | 108.7% |

In Table 12 to Table 14, we compare TSF compression case2 with legacy baseline and two SF compression models (one with small scale of parameter, the other one with larger scale of parameter). Note that the parameter scale of TSF compression model is the same as that of small SF compression model. It can be observed that TSF compression case2 helps to significantly reduce the model complexity by about one order of magnitude both in Flops and parameter scale while achieves even higher performance gain. In addition, our throughput results also verified similar observations. While we acknowledge that it is still challenging to make the complexity of AI model comparable to legacy CB, we believe that it is also feasible to further optimize the complexity of AI model to an acceptable level.

- By considering TSF compression studied in R19, two-sided model complexity could be reduced compared with that of models in R18 SF compression. Our simulation results reveal that the parameter scale/FLOPs can be reduced to ~1/10 while achieving even higher performance gain.

We would also like to present our results for TSF compression case 3. To align with typical R18 CB configuration, we consider a window of length 4 in our simulation. For the target CSI in TSF case3, both the cases of ideal CSI prediction and realistic CSI prediction are considered in the evaluation. specifically, an AR based prediction algorithm is adopted, which could effectively generate future 4 CSIs. The detailed results are presented below.

Results for TSF compression case3 (ideal prediction)

|  | R18 eT2 CB, N4=4 | TSF compression case3 |
| --- | --- | --- |
| SGCS results (layer1, PC2) | 0.696 | 0.77 (+10.6%) |
| Full buffer mean UPT SE in bps/Hz (PC2) | 100% | 108.8% |
| Full buffer 5% UPT SE in bps/Hz (PC2) | 100% | 111.9% |
| SGCS results (layer1, PC5) | 0.771 | 0.83 (+7.6%) |
| Full buffer mean UPT SE in bps/Hz (PC5) | 100% | 104.8% |
| Full buffer 5% UPT SE in bps/Hz (PC5) | 100% | 106.25% |

Results for TSF compression case3 (realistic AR prediction)

|  | R18 eT2 CB, N4=4 | TSF compression case3 |
| --- | --- | --- |
| SGCS results (layer1, PC2) | 0.64 | 0.72 (+12.5%) |
| Full buffer mean UPT SE in bps/Hz (PC2) | 100% | 110.6% |
| Full buffer 5% UPT SE in bps/Hz (PC2) | 100% | 111.4% |
| SGCS results (layer1, PC5) | 0.72 | 0.78 (+8.3%) |
| Full buffer mean UPT SE in bps/Hz (PC5) | 100% | 106.7% |
| Full buffer 5% UPT SE in bps/Hz (PC5) | 100% | 104.8% |

From Table 15 and Table 16, it can be observed that the compression of 4 slots using the joint TSF performs better than the R18 DD codebook compression under both ideal and realistic AR CSI prediction in terms of both inter-mediate KPI and throughput. The reason why joint TSF compression is advantageous is that the process of compressing CSI is able to make better use of the correlation between CSI information in multiple slots.

- For dense urban scenarios, TSF compression case3 can improve SGCS by 7.6%~10.6% under ideal CSI prediction and 8.3%~12.5% under realistic CSI prediction compared to R18 DD codebook.

- For dense urban scenarios, TSF compression case3 can improve mean UPT SE by 4.8%~8.8% under ideal CSI prediction and 6.7%~10.6% under realistic CSI prediction compared to R18 DD codebook.

- For dense urban scenarios, TSF compression case3 can improve 5% UPT SE by 6.25%~11.9% under ideal CSI prediction and 4.8%~11.4% under realistic CSI prediction compared to R18 DD codebook.

Other specification aspects regarding inference, data collection, and monitoring

During R18 study, quite a number of specification aspects related to CSI compression with two-sided model have been identified but not yet concluded. Therefore, we continue to present our views in the discussion of R19.

Monitoring

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches.

In the agreement from RAN1 #117, all options related to monitoring for CSI compression with two-sided model are on board. From our view, both NW side and UE side monitoring can be supported, as different trade-off between monitoring accuracy and reporting overhead can be achieved.

For NW-side monitoring, the real-time CSI measurement is always missing and the complete model can be available if joint training at NW or separate training with NW-side first training are considered. Given the superior computation and storage capability at NW, we can assume model training happens on NW side, enabling complete models available at NW. To acquire real-time CSI measurement at NW side, a data collection procedure can be considered. According to the evaluation in the Rel-18, it is efficient to use enhanced legacy codebook to report ground-truth CSI measurement for monitoring. The disadvantage of above method is that the overhead of CSI measurement can be large, e.g., ~1000 bits per sample to achieve enough reporting accuracy.

For UE-side monitoring, we believe that one of the most important motivations is to reduce the over-the-air overhead. Therefore, we prefer proxy model-based solutions, which does not incur additional information exchange such as precoded RS. Towards the design of the proxy model for monitoring, our view is that both directly estimating SGCS (i.e., Case 2-1) and generating reconstruction CSI at UE (i.e., Case 2-2) can be considered for further study. The acquisition of proxy model is the key issue to be studied in the future. For provision of the reference model for UE-side monitoring purpose, it could directly follow the corresponding inter-vendor collaboration methods for Option 3, i.e., proxy model transfer or directly include monitoring related output as the model output. Note that acquiring proxy model from NW can help to relieve the concerns on the LCM issue for proxy model, as NW can fully control its training procedure (and result). On the contrary, if the proxy model is trained by UE, additional procedures may be required to align the management of proxy model.

In addition, we would like to point out that above monitoring methods (both NW and UE side) can help to facilitate inter-vendor training collaborations, e.g., thoroughly identifying the cause of performance degradation to alleviate the concerns on mismatch between transferred models and deployed models in option 3b.

- Both NW side and UE side monitoring can be supported for CSI compression with two-sided models

      - For NW-side monitoring, consider target CSI reporting by UE via legacy eT2 codebook or eT2-like high-resolution codebook ;

      - For UE-side monitoring, consider monitoring based on either direct estimation of SGCS (Case 2-1) or the output of reconstruction model at the UE (Case 2-2);

        - For provision of the reference model for UE-side monitoring purpose, it could directly follow the corresponding inter-vendor collaboration methods for Option 3, e.g., proxy model transfer or directly include monitoring related output as the model output.

- Monitoring methods (both NW and UE side) can help to facilitate inter-vendor training collaborations, e.g., thorough identifying the cause of performance degradation to alleviate the concerns on the mismatch between transferred models and deployed models in option 3b.

Conclusion

In this contribution, we have the following observations:

- For Option 3b (and potentially Option 3a/5a/4, if needed), the following methods can be considered to identify the cause of the performance degradation assuming NW-side monitoring

- Identifying data drift: with ground-truth CSI reported from UE, NW could check the SGCS of NW-side original model

- Identifying UE side cause:

  - With ground-truth CSI reported from UE, NW compares expected CSI feedback and reported CSI feedback (FFS the necessity of taking quantized ground-truth as UE side input), and/or

  - NW compares Intermediate KPI of ongoing model and NW-side original model

- Identifying NW side cause: NW could check whether NW side decoder is correctly deployed by comparing the expected output and real output given the same decoder input

- For UE-side monitoring, towards identifying the cause of performance degradation, we have the following considerations (assuming NW first training for option 3/4/5)

- Identifying UE side cause:

  - For option 3a/5a-1 and 3b, UE could check the expected PMI of transferred CSI generation model and deployed CSI generation model

  - If original CSI generation model is not available at UE side (e.g., option 3a/5a-2 and option4), assistance information such as testing dataset from NW can help to identify UE side cause

- Identifying NW side cause: NW could check whether NW side decoder is correctly deployed by comparing the expected output and real output given the same decoder input

- If no UE side cause or NW side cause are identified, it can be determined that the performance degradation is due to data drift.

- Characteristics of options to alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression can be summarized as:

|  | Inter-vendor collaboration complexity | Performance | Interoperability and RAN4 / testing related aspects | Feasibility | Deployment timescale |
| --- | --- | --- | --- | --- | --- |
| Option 1 | Minimum complexity | Restricted | Solved | feasible | \ |
| Option 3b | Minimum complexity with over the air signalling; | Optimum | Solved | Feasible | Short |
| Option 3a/5a | High complexity in server to server manner; Medium complexity with over the air signalling; | Optimum | Not solved | Infeasible with only short time scale model update | Long |
| Option 4 | High complexity in server to server manner; Medium complexity with over the air signalling; | Better than Option 1, but worse than Option3 and Option5 | Not solved | Infeasible with short time scale model update | Long |
| Option 5b | Need clarification |  |  |  |  |

- SF compression with cell/site specific model provides up to 6.6% gain compared to legacy codebook in Uma scenario with 0% indoor UE distribution while general model only provides 2.6% SGCS gains.

- SF compression with cell/site specific model provides up to 9.7% gain in certain case (cell/sector) compared to legacy codebook in InH scenario with 100% indoor UE distribution while general model provides 6.5% SGCS gains.

- When TR 38.901 channel model is considered, localised TSF compression (case2) models trained on data including the impact of Doppler shift generalize well over different slots even with large gaps (e.g., slot1k training vs. slot10k testing).

- TSF compression case2 with cell/site specific model provides 10.7%~15.9% layer1 SGCS gain and 17.3%~31.4% layer2 SGCS gain compared to legacy codebook in Uma scenario with 80% indoor UE distribution while general model only provides 1.3%~5.8% SGCS gain for layer1 and 1.5%~15.7% SGCS gain for layer2.

- TSF compression case2 with cell/site specific model provides 10.8%~15.2% SE gain in full buffer mean UPT compared to legacy codebook in Uma scenario with 80% indoor UE distribution while general model only provides 4.1%~10.9% SE gain.

- TSF compression case2 with cell/site specific model provides 12.8%~20.5% SE gain in full buffer 5% UPT compared to legacy codebook in Uma scenario with 80% indoor UE distribution while general model only provides 2.7%~14.7% SE gain.

- By considering TSF compression studied in R19, two-sided model complexity could be reduced compared with that of models in R18 SF compression. Our simulation results reveal that the parameter scale/FLOPs can be reduced to ~1/10 while achieving even higher performance gain.

- For dense urban scenarios, TSF compression case3 can improve SGCS by 7.6%~10.6% under ideal CSI prediction and 8.3%~12.5% under realistic CSI prediction compared to R18 DD codebook.

- For dense urban scenarios, TSF compression case3 can improve mean UPT SE by 4.8%~8.8% under ideal CSI prediction and 6.7%~10.6% under realistic CSI prediction compared to R18 DD codebook.

- For dense urban scenarios, TSF compression case3 can improve 5% UPT SE by 6.25%~11.9% under ideal CSI prediction and 4.8%~11.4% under realistic CSI prediction compared to R18 DD codebook.

- Monitoring methods (both NW and UE side) can help to facilitate inter-vendor training collaborations, e.g., thorough identifying the cause of performance degradation to alleviate the concerns on the mismatch between transferred models and deployed models in option 3b.

and the following proposals:

- RAN1 considers specifying the reference model structure as a starting point for the specification of a reference model.

- Towards the specification of reference model structure, the following procedures can be considered in RAN1:

      - Step 0: Aligning evaluation assumptions

      - Step 1: Determine the model backbone based on consensus and evaluation results on complexity and performance.

      - Step 2: Determine the model hyperparameters that need to be aligned.

      - Step 3: Align the hyperparameters of the model.

- RAN1 concludes that it is recommended to support option 3b to address inter-vendor training collaboration for CSI compression

- Both NW side and UE side monitoring can be supported for CSI compression with two-sided models

      - For NW-side monitoring, consider target CSI reporting by UE via legacy eT2 codebook or eT2-like high-resolution codebook;

      - For UE-side monitoring, consider monitoring based on either direct estimation of SGCS (Case 2-1) or the output of reconstruction model at the UE (Case 2-2);

        - For provision of the reference model for UE-side monitoring purpose, it could directly follow the corresponding inter-vendor collaboration methods for Option 3, e.g., proxy model transfer or directly include monitoring related output as the model output.

- References

- 3GPP RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, #102, Dec.11-15, 2023.

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V2.0.0, 2023-12

Appendix A: Initial lab test for model transfer with known model structure

An initial lab test for model transfer with known model structure is done to find the main aspects in this procedure and obtain the actual latency. For even 10M parameters, the total latency of UE updating the received model parameters is just 12.472ms, which would result in the low latency of model transfer with known model structure.

From the lab test, it is found that the model parameters updating with known model structure can be listed in 4 steps.

- Step1: UE generates one certain format of model executable file (e.g., engine file for TensorRT), where the model parameters and model structure related part are stored separately. Each model parameter group could be updated separately with very low latency. The results of a lab test would be provided later to show the detailed latency. The input data processing operations would be stored in model structure related part, e. g., matrix multiplications and additions.

- Step2: UE extracts the target model parameters from the received new model file (e.g., ONNX or other open formats) from NW. The extracted parameters would be stored in buffer area temporarily. Simple quantization may be used and details will be discussed in the following.

- Step3: New model parameters would replace the previous model parameters directly.

- Step4: After the replacement of target model parameters, the model is available to run.

Figure A-1. The procedure of model parameters updating with known model structure.

The following is the detailed latency test of model parameter updating for different number of parameters. A full connected model with 5 full connected layers is adopted in this lab test. The number of parameters of 5 layers are 1K, 10K, 100K, 1M and 10M respectively. The new model parameters are stored in ONNX format. The latency of parameter extraction from ONNX in Step 2 and the latency of parameter replacement in Step 3 are both provided. As the number of parameters increases, the latency of Step 2 and Step 3 both increases for more than 10K parameters. For the case with less than 10K parameters, the basic latency of hardware interaction may be the majority, which would have low correlation with the number of parameters. For even 10M parameters, the total latency of Step 2 and Step 3 is just 12.472ms, which would result in the low latency of model transfer with known model structure.

Table A-1. Initial lab test for model parameter updating with known model structure

| Number of parameters in one group | Latency of parameter extraction from ONNX in Step 2 (ms) | Latency of parameter replacement in Step 3 (ms) |
| --- | --- | --- |
| 1K | 0.212 | 0.793 |
| 10K | 0.223 | 0.704 |
| 100K | 0.344 | 2.520 |
| 1M | 1.342 | 4.893 |
| 10M | 6.740 | 5.732 |

The feasibility of model transfer with known model structure were discussed by companies in Rel-18, where the major issues lie in necessity and feasibility of on device compilation. From our observation, the main issue based on previous discussion for on device compilation is how to quantize the trained models. There are two ways to handle this: post training quantization or quantization aware training. On device quantization is one kind of posting training quantization.

The alignment of quantization before model transfer could avoid the on device quantization, e.g., the transferred model parameters have been quantized by NW, using post training quantization or quantization-aware training.

On the other hand, if the quantization is not aligned beforehand, on device quantization would be needed. There are several aspects that need to be considered for on device quantization: the performance of on device quantization and involved complexities.

Since on device quantization is a kind of post training quantization, we did an experiment to compare the performance loss of post training quantization and quantization aware training. The loss would be small. The following tables shows the impact of different quantization levels for different use cases. The INT8 quantization is directly mapping Float32/16 values to INT8/16, which would be worst performance of post training quantization. For positioning, since the model inference may be not in UE modem, there may be no strict requirement on quantization.

Table A-2. An example of the impact of different quantization levels for CSI compression, where the INT8 quantization is directly mapping Float32/16 values to INT8/16.

|  | MLP model | CNN model | Transformer model |
| --- | --- | --- | --- |
| SGCS of FP32 quantized model | 0.9421 | 0.9493 | 0.9581 |
| SGCS of FP16 quantized model | 0.9421 | 0.9493 | 0.9581 |
| SGCS of INT8 quantized model | 0.9413 | 0.9486 | 0.9573 |

Table A-3. An example of the impact of different quantization levels for AI/ML assisted positioning.

|  | 90% positioning accuracy of AI/ML assisted positioning (m) |
| --- | --- |
| FP32 quantized model | 0.970 |
| FP16 quantized model | 0.973 |

Table A-4. An example of the impact of different quantization levels for beam spatial prediction, where the INT8 quantization is directly mapping Float32/16 values to INT8/16.

|  | Top-1 (%) | Top-1 (%) with 1dB margin |
| --- | --- | --- |
| FP32 quantized model | 77.4 | 96.6 |
| FP16 quantized model | 75.5 | 96.3 |
| INT8 quantized model | 71.3 | 94.9 |

On device quantization is also feasible at least for some operations with low complexity. For example, regular quantization from Float32/16 to INT8/16 by directly mapping Float32/16 values to INT8/16, or with minor adjustment based on parameter distribution. More advance quantization, e.g., non-uniform quantization, finetuning after quantization, does not seem to provide additional gains at least for current use cases.

Performance monitoring/assessment could monitor the performance of quantization and may provide some data samples for quantization calibration. UE could get some data samples through the measurement or from the data delivery from NW.