3GPP TSG RAN WG1 #118                                       R1-2406378

Maastricht, NL, August 19th – 23rd, 2024

Source: 	CATT

Title:	Discussion on adaptation of common signal/channel transmissions

Agenda Item:	9.5.3

Document for:	Discussion and Decision

- Introduction

One objective of the Rel-19 NES work item is to specify the adaptation of common signal/common transmission, and the objectives for the adaptation of common signal/channel transmission including the following aspects.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we discuss the potential specification impacts for the adaptation of common signal/channel transmissions.

- Discussion

  - Adaptation of SSB in time domain

- Scenarios of adaptation of SSB in time domain

In RAN1#116bis[1], it was agreed to study scenario(s) for adaptation of SSB in time-domain as following:

Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized

For CD-SSB on the sync raster in Option A1 and Option B1, it can be used to perform cell search. Regarding the cell search, the UE may assume the periodicity of CD-SSB is 20 ms, but it is up to the network implementation to determine the actual periodicity of CD-SSB. Network indicates the actual periodicity of CD-SSB by the higher layer parameter i.e. ServingCellConfigCommonSIB in SIB1. The candidate values of CD-SSB periodicity can be {5, 10, 20, 40, 80, 160} ms, which is shown in the following highlighted part [2].

However, some UE venders consider that the periodicity of CD-SSB for cell search should always be less than or equals to 20 ms. Therefore, the UE vender has concerns about the impact on cell search if the CD-SSB periodicity is adapted to a value which is greater than 20 ms. In this situation, we are open to discuss whether CD-SSB adaptation can be supported in both Option A1 and Option B1.

For NCD-SSB (Non-cell defined SSB) in Option A2 and Option B2, it can be configured to the UE in RRC_CONNECTED by NonCellDefiningSSB-r17 for sync, measurement, RLM, BFD, and beam measurement. For network power saving, one potential deployment is as shown in Figure 1, in which cell 1 is configured with 20ms CD-SSB, and other cells (e.g., cell 2 and cell 3) are configured with NCD-SSB. The cell configured with CD-SSB can be used to provide cell coverage for cell search, and can also be the PCell of the RRC_CONNECTED UEs. Meanwhile, the cell configured with NCD-SSB can be the SCell of the RRC_CONNECTED UEs. In this case, the network can obtain power saving on cell 2 and cell 3 by adapting NCD-SSB periodicity based on traffic requirement. Therefore, Option B2 can be supported for network power savings.

Figure 1: Network deployment for network energy saving

Since NCD-SSB can be configured on the PCell as well and a time offset can be configured between CD-SSB and NCD-SSB, it is beneficial to apply NCD-SSB adaptation mechanism to Option A2 to save power consumption from the transmission of NCD-SSB.

For SSB not on the sync raster in Option A3 and Option B3, it can be CD-SSB or NCD-SSB. Regarding the NCD-SSB not on the sync raster, it has been covered by Option A2 and Option B2, the same analysis can be found above. However, for the CD-SSB not on sync raster, the motivation and benefit of Option A3 and Option B3 are not clear. The applicable scenarios need to be further clarified by the proponents.

Proposal 1: For network energy saving, at least NCD-SSB adaptation in Option A2 and Option B2 should be supported.

- Mechanisms of SSB adaptation in time-domain

In RAN1#117 [2], it has been agreed to support Option 1 for adaptation of SSB in time-domain.

Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

For Option 1, the UE can be configured with two SSB burst periodicities, one with a large SSB burst periodicity, and another with a short SSB burst periodicity. For example, the large SSB burst periodicity could be 160 ms, and the short SSB burst periodicity could be 40 ms, as shown in Figure 2. When the traffic is busy, the network can switch to the SSB burst periodicity of 40ms. Conversely, when the traffic is light, the network can switch to the SSB burst periodicity of 160 ms.

Figure 2: Adaptation of SSB in Option 1 with two SSB burst periodicities

There is a FFS on applicable scenarios. This issue can refer to the last section of the discussion on SSB adaptation scenarios. Another FFS is whether to support cell DTX for connected mode for SSB. In Rel-18, a periodic cell DTX pattern can be configured for RRC_CONNECTED UEs. When the cell DTX pattern is activated for the connected cell, the UE may not monitor PDCCH during DTX non-active duration, but it doesn’t impact SSB transmission. However, if cell DTX pattern were extended to SSB transmission, it would be possible for the network to obtain power saving gain by reducing SSB transmission occasions. For example, if a cell DTX pattern cycle is 320 ms and the active duration is 120 ms, as shown in Figure 4, then when a cell DTX pattern is activated for a cell, the UE receives SSBs during the DTX active duration on that cell, and cannot receive SSBs during the cell DTX non-active duration on that cell. In this way, the network can stop transmitting SSB during the cell DTX non-active duration to obtain power saving gain.

Figure 3: Cell DTX for SSB

Compared to SSB adaptation mechanism in Option 1, cell DTX for SSB is more consistent with DL traffic transmission. This ensures that the network transmits SSB and other DL transmissions in cell DTX active duration and enters network energy saving mode in cell DTX non-active duration. We recommend allowing the network to determine whether to apply cell DTX for SSB or adaptation of SSB in option 1. For example, if a cell DTX pattern has been configured and activated for a cell, the network can apply cell DTX for SSB to the cell as well. If not, the network can apply adaptation of SSB in option 1.

Proposal 2：For the adaptation of SSB in time-domain, in option 1, the UE can be configured with two different SSB burst periodicities and can be instructed to switch between two SSB burst periodicities.

Proposal 3：For the adaptation of SSB in time-domain, cell DTX for SSB can be supported for the cell configured with cell DTX pattern.

- Signaling design

In the current specification, the NCD-SSB periodicity is configured and updated by RRC signaling. Since RRC signaling is typically used for semi-static configuration, it is not efficient to reconfigure NCD-SSB periodicity frequently using RRC signaling. For SSB adaptation based on option 1, the network should send switching indication between two SSB periodicities to the UE. Using the CD-SSB adaptation of option 1 as an example, there are several potential signaling can be further considered:

- Alt. 1: MAC CE

- Alt. 2: UE specific DCI, e.g. DCI format 1_1/1_2

- Alt. 3: Group-common DCI, e.g. DCI format 2_9

All of the above signaling methods can be used to indicate switching between two SSB periodicities for a connected UE. From the perspective of signaling overhead, MAC CE and UE-specific DCI are both UE-specific signaling and should be transmitted to different UEs separately. On the other hand, group-common DCI is more suitable to indicate cell-level information. The network can use one group-common DCI to indicate the change to all the UEs in the cell. Therefore, it’s beneficial for the network to apply group-common DCI (Alt.3) to indicate CD-SSB adaptation.

Another issue is how to indicate the status of SSB adaptation before the UE can receive a dynamic indication, such as when configuring a new cell for the UE. It can be solved by introducing a RRC signaling to indicate the status of SSB adaptation. In this way, the UE can obtain the status of SSB adaptation when the cell is configured for the UE.

Proposal 4: The SSB adaptation operation can be indicated by group common DCI.

Proposal 5: The SSB adaptation status can be indicated to the UE by RRC signaling.

  - Adaptation of PRACH in time domain

In RAN1#116bis [1], it has been agreed to further study the adaptation of PRACH in time domain as follows:

Agreement For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least: - a PRACH configuration index - FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources Study further the following - When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource, - Additional parameter(s) for determining the additional PRACH resources e.g. - Scaled/adjusted PRACH configuration period - Additional timing offset - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Muting/masking ROs - When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource - Additional mechanisms (if any) for determining the additional PRACH resources e.g. - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

- Adaptation mechanism

It was agreed in RAN1#117 to support PRACH adaptation in time domain based on an additional PRACH index. Whether the additional PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources is still under discussion. RAN1 can consider supporting both options that the additional PRACH index is same or different with legacy PRACH index at the same time. The additional parameters for determining the additional PRACH resources are different for the two options.

- Option 1: Time domain offset when PRACH configuration index is same as the legacy PRACH configuration index and

- Option 2: Muting/masking ROs when PRACH configuration index which is different from the legacy PRACH configuration index

In our view, when the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource, the additional PRACH resource can be configured by adding or adjusting additional timing offset of the legacy RACH configuration. In Option 1, gNB provides a time domain offset (e.g. sub-frame-level) based on the legacy PRACH configuration for Rel-19 UE. As shown in Figure 4, gNB provides legacy ROs in blue, and additional PRACH resources can be configured for the Rel-19 UE by a time domain offset in orange. This part of ROs can be configured at relatively flexible positions within the PRACH period. The gNB has more opportunities to go into sleep when ROs are located together and within the PRACH period. In our view, the additional time offset is sufficient and that no additional parameters are needed in Option 1.

Figure 4: Adaptation mechanism with additional PRACH configuration index which is same as legacy index

The gNB can also directly configure an additional index which is different from the legacy PRACH configuration index for the R19 UE, but the configuration flexibility is limited. Adding the muting ROs parameter can improve the configuration flexibility. As shown in Figure 5, gNB can provide a muting/masking ROs pattern and an additional PRACH configuration for Rel-19 UE. Legacy ROs are marked in blue, and additional ROs can be independently configured by an additional index in orange locations. The gNB can mute different levels of ROs to achieve concentrated/clustered RACH resource, such as RO level, SSB-to-RO mapping cycle level and PRACH association period level etc. Since for the PRACH resource configurations, valid ROs are quite flexible, muting/masking ROs scheme in RO level is difficult and will introduce extra signaling overhead. Moreover, the RO level muting/masking will cause non-uniform PRACH resource allocation across SSBs. PRACH association period level muting/masking enables longer time for gNB sleeping but leads to larger random-access latency which means part of the delay-sensitive UEs cannot use this level of adaptation. The SSB-to-RO mapping cycle is the smallest unit for adaptation without impacting UE SSB-to-RO mapping, therefore it is a preferred granularity choice.

Figure 5: Adaptation mechanism with additional PRACH configuration index which is different from legacy index

In the pattern generated by the configuration of Option 1, the additional ROs are always distributed next to the legacy ROs, which restrict the flexibility of the pattern. The benefit of Option 1 is that the signaling overhead is small. In Option 2, the PRACH configuration index for the additional PRACH resources is different from the legacy index and the muting/masking ROs mechanism can be used to increase flexibility of configuration. But correspondingly, it will increase the signaling overhead. In our opinion, RAN1 can support both options.

Proposal 6: For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on the following two options:

- Option 1: Time domain offset when PRACH configuration index is same as the legacy PRACH configuration index

- Option 2: Muting/masking ROs when PRACH configuration index is different from the legacy PRACH configuration index.

Proposal 7: Support the SSB-to-RO mapping cycle level for muting/masking ROs scheme of additional PRACH resources.

- Signaling design

In RAN1#117[2], it was agreed to further study the signaling for the adaptation mechanism.

Agreement For the adaptation mechanism for additional PRACH resources, study further the following: - Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation - Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Strive to re-use existing DCI format(s) - Option 3: Adaptation of PRACH transmission according to predefined condition(s) - FFS: details - Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation - Strive to re-use existing DCI format(s) - Option 5: Enhanced cell DRX

We recommend considering the signaling design applicable to RRC_IDLE/inactive UE first, which is more important for network power saving. In current specification, the cell specific random access related parameters are configured in SIB1, and the SIB update procedure can be used to update the parameters in SIB1. The UE first receives SI change indication that is the paging DCI with short message indication, and then monitors the DCI 1_0 with SI-RNTI scheduling updated SIB1 in the next modification period as shown in Figure 6.

Figure 6: SI update procedure

For signaling design, the following options can be further studied:

- Option 1: Higher layer signalling based PRACH resource adaptation

Option 1 is similar to the current SIB update procedure. But, there are still one potential issues required to be discussed. For SI change indication, both legacy UE and Rel-19 NES UE can receive SI change indication for the adaptation of additional PRACH, but there are actually no SI update for legacy UE.

One potential solution is re-using the reserved bit (e.g. 5-8 bit) in short message field of paging DCI to indicate the update of additional PRACH resource for Rel-19 NES capable UE. For example, as shown in Figure 7, when network transmits a paging DCI using 5th bit in short message field to indicate the update of additional PRACH resource, it can be assumed that both Rel-19 UE and legacy UE can receive the paging DCI. For Rel-19 UE, it can interpret update indication of additional PRACH resource in paging DCI, and then prepare to receive the updated additional PRACH resource configuration (e.g. the PDSCH scheduling by DCI 1_0 with SI-RNTI) in next modification period. For legacy UE, since it can’t interpret new update indication in paging DCI, it shall ignore the adaptation of additional PRACH operation.

Figure 7: Additional PRACH resource update procedure based on higher layer signaling

- Option 2: L1-based PRACH resource adaptation

Another option is to use paging DCI to indicate activation/deactivation of additional PRACH resource for Rel-19 UE. In this solution, the reserved bit (e.g. 5-8 bit) in short message field of paging DCI can be re-used to activate/deactivate additional PRACH resource. For example, as shown in Figure 8, network transmits a paging DCI using 5th bit in short message field to activate/deactivate the additional PRACH resource. And we can assume that both Rel-19 UE and legacy UE receive the paging DCI. For Rel-19 UE, it can interpret the activation/deactivation indication of additional PRACH resource in paging DCI. For legacy UE, since it can’t interpret new activation/deactivation indication in paging DCI, it shall ignore the adaptation of additional PRACH operation.

For the applicable time of additional PRACH adaptation, in option 2, the UE can obtain the activation/deactivation of additional PRACH resource once paging dci is decoded by UE. But, in option 1, the UE only can obtain activation/deactivation information of additional PRACH resource during next modification period.

Figure 8: The activation/deactivation of additional PRACH resource based on paging DCI

Proposal 8: For the adaptation mechanism for additional PRACH resources, study further the following:

- Option 1: Higher layer signalling based PRACH resource adaptation

  - Using the reserved bit (e.g. 5-8 bit) in short message field of paging DCI to indicate the update of additional PRACH resource for Rel-19 UE

- Option 2: L1-based based PRACH resource adaptation to activate/deactivate the additional PRACH resources provided by semi-static signalling

  - Using the reserved bit (e.g. 5-8 bit) in short message field of paging DCI to activate/deactivate of additional PRACH resource for Rel-19 UE

  - Adaptation of paging in time domain

In RAN1#116 [5], for adaptation of paging, it was agreed that details of the specifications should be handled by RAN2:

Agreement For adaptation of paging, - Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

It has been agreed that the specification details for PO/PF determination and paging-related configurations or procedures will be handled by RAN2. RAN1 can discuss those issues that are caused by paging concentration.

One issue that can occur with concentrated paging is that random-access collision may follow the POs if gNB configures concentrated paging occasions. After UEs receive the paging message, plenty of UEs trigger the random-access procedure within a short period which would increase collision severely. Therefore, we suggest to study the random-access collision issue caused by paging concentration.

Observation 1: One issue is that random-access procedure collision may follow the POs if gNB configures concentrated paging occasions.

RAN1 can consider dividing the POs within a PF into multiple sets, and also dividing the RO between two PFs into multiple sets. A mapping/relationship between the PO sets and the RO sets can be designed. As illustrated by Figure 9, when the UE monitors a PO in PO set1, it will trigger the random-access procedure at the RO set1 corresponding to PO set1. This scheme can reduce the PRACH resource collision issue caused by paging concentration.

Figure 9: Correspondence for the PO sets and the RO sets

Proposal 9: Consider partitioning PO and RO into multiple sets and establish mapping between PO set and ROs set to solve the paging concentration problem.

  - Unified signaling design

As mentioned in section 2.3, the adaptation of PO would impact the PRACH configuration. This implies adaptation of PO and adaptation of PRACH may happen simultaneously. On the other hand, the use case of independent adaptation of paging is not clear. Therefore, instead of separate adaptation indication for different signals/channels, a common signaling is preferred to indicate adaptation of PRACH/paging for more efficiency.

Proposal 10: A common signaling is preferred to indicate adaptation of PRACH/paging simultaneously.

- Conclusion

In this contribution, the potential spec impacts for adaptation of common signal/channel transmission are discussed. The observations and proposals are summarized as follows:

Observation 1: One issue is that random-access procedure collision may follow the POs if gNB configures concentrated paging occasions.

Proposal 1: For network energy saving, at least NCD-SSB adaptation in Option A2 and Option B2 should be supported.

Proposal 2：For the adaptation of SSB in time-domain, in option 1, the UE can be configured with two different SSB burst periodicities and can be instructed to switch between two SSB burst periodicities.

Proposal 3：For the adaptation of SSB in time-domain, cell DTX for SSB can be supported for the cell configured with cell DTX pattern.

Proposal 4: The SSB adaptation operation can be indicated by group common DCI.

Proposal 5: The SSB adaptation status can be indicated to the UE by RRC signaling.

Proposal 6: For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on the following two options:

- Option 1: Time domain offset when PRACH configuration index is same as the legacy PRACH configuration index

- Option 2: Muting/masking ROs when PRACH configuration index is different from the legacy PRACH configuration index.

Proposal 7: Support the SSB-to-RO mapping cycle level for muting/masking ROs scheme of additional PRACH resources.

Proposal 8: For the adaptation mechanism for additional PRACH resources, study further the following:

- Option 1: Higher layer signalling based PRACH resource adaptation

  - Using the reserved bit (e.g. 5-8 bit) in short message field of paging DCI to indicate the update of additional PRACH resource for Rel-19 UE

- Option 2: L1-based based PRACH resource adaptation to activate/deactivate the additional PRACH resources provided by semi-static signalling

  - Using the reserved bit (e.g. 5-8 bit) in short message field of paging DCI to activate/deactivate of additional PRACH resource for Rel-19 UE

Proposal 9: Consider partitioning PO and RO into multiple sets and establish mapping between PO set and ROs set to solve the paging concentration problem.

Proposal 10: A common signaling is preferred to indicate adaptation of PRACH/paging simultaneously.

- References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #116 bis.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #117.

- TS 38.331 Radio Resource Control (RRC) protocol specification

- TS 38.300 V18.1.0 NR and NG-RAN Overall Description

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #116.