3GPP TSG RAN WG1 #118		                                                      R1-2406384

Maastricht, NL, August 19th – 23rd, 2024

Source:	CATT

Title:	Views on channel model validation of TR38.901 for 7-24GHz

Agenda Item:	9.8.1

Document for:	Discussion and Decision

- Introduction

As approved in RAN #102 meeting, channel modelling enhancements for 7-24GHz include the following objectives [1]:

- Validate using measurements the channel model of TR38.901 at least for 7-24 GHz - Note: Only stochastic channel model is considered for the validation. - Note: The validation may consider all existing scenarios: UMi-street canyon, UMa, Indoor-Office, RMa and Indoor-Factory. - Adapt/extend as necessary the channel model of TR38.901 at least for 7-24 GHz, including at least the following aspects for applicable scenarios: - Near-field propagation (with consideration being given to consistency between near-field and far-field) - Spatial non-stationarity Note 1: Continuity of the channel model in the frequency domain below 7 GHz and above 24 GHz shall be ensured. Note 2: Mathematical and/or theoretical aspects (if any) may be studied before results of measurement campaigns are available. While measurement results may be available and submitted at any time, the study of measurement results may start later (e.g., Q3 2024).

In this contribution, our views on channel model validation of TR38.901 for 7-24GHz are provided.

- Discussion

  - Aspects of channel model involved in the validation

In RAN1#117 meeting, the following agreement on parameters for existing deployment scenarios for the validation was made [2]. In this section, the necessity of the validation for the listed parameters is analyzed based on the collected results.

Agreement - Further study whether the following parameters for existing deployment scenarios is necessary to be updated: - Delay spread, pathloss, penetration loss, AoD/AoA/ZoA/Zod spreads, ZoD offset, Angle distribution characteristics (e.g. exponential, Gaussian, Laplacian distributions), XPR, number of clusters, number of rays per cluster, LSP correlations type (e.g. site-specific or all correlated), UE antenna modeling parameters, K factor (mean, variance) - Study of updates to other parameters are not precluded and subject to further study.

Pathloss:

The path loss model of 7-24GHz was measured and compared with the model in TR38.901 [3] in [4]-[6]. All these measurement results show that pathloss measurements are in line with existing pathloss models in TR 38.901. Therefore, there is no need to update the path loss model in TR38.901.

Penetration loss:

In TR38.901 [3], penetration losses of the following four materials are modelled: standard multi-pane glass, IRR glass, concrete and wood. In [7], penetration losses of concrete, wood and glass with different thicknesses at 6-24GHz are measured. For each material, the measurement results of at least one thickness can be aligned with the model in TR38.901.

According to [4], penetration losses of standard glass and drywall/wood at 13GHz are in line with the model in TR38.901, but the measurements of IRR glass show smaller losses (the gap is about 4.6-7.6dB) at 13GHz than the model in TR38.901. Since penetration loss is a simplified model where the thickness of the material is not modelled, the gap between the current measurement results and the model in TR38.901 is not evident.

Observation 1: The gap between the penetration loss related measurement results for 7-24GHz and the model in TR38.901 is not evident.

Delay spread:

According to [7], the fitting curves based on the measurement results of delay spread are very close to the curves given by TR 38.901 [3] in UMi LOS/NLOS scenario. Measurement results in [8] show that the measured DS is approximately 50% and 70% smaller than that in TR38.901 for LOS and NLOS case, respectively under the UMa scenario at 6.5 GHz. Similarly, measured RMS delay spreads in NLOS factory are also seen smaller than RMS delay spreads in TR38.901 [6]. Therefore, delay spread in some scenarios (e.g., UMa, indoor factory) can be further validated for 7-24GHz.

Observation 2: The gap between the delay spread measurement results in some scenarios for 7-24GHz and the model in TR38.901 cannot be ignored.

Angular spread:

Similar to delay spread, [8] shows that measured ASA angular spread is smaller than the existing model in TR 38.901 [3] (about 20 degree). However, [7] shows that the fitting curves based on the measurement results of angular spread are very close to the curves given by TR 38.901 in UMi LOS/NLOS scenario. Therefore, angular spread in some scenarios (e.g., UMa) can be further validated for 7-24GHz.

Observation 3: The gap between the angular spread measurement results in some scenarios for 7-24GHz and the model in TR38.901 cannot be ignored.

Number of clusters:

According to the measurement results in [8], the number of clusters for LOS and NLOS indoor scenario at 10GHz is 10 and 11, respectively, which is smaller than the model in TR38.901 (15 for LOS, and 19 for NLOS) [3].  In [5], the number of clusters is decreased as frequency band is increased, which is not aligned with the model in TR 38.901. Therefore, the validation of the number of clusters is needed.

Observation 4: The gap between the number of clusters measurement results for 7-24GHz and the model in TR38.901 cannot be ignored.

In TR38.901 [3], K factor is not related to the frequencies. Considering that the model of K factor has already been validated by sufficient measurement results in other frequencies, the validation of K factor for 7-24GHz can be deprioritized. The necessity of the validation for XPR and UE antenna modeling parameters are assessed in section 2.3 and 2.4, respectively.

Proposal 1: The assessment of the necessity for validation for channel model parameters in Table 1 can be considered.

Table 1 Potential list of parameters in the validation

| Parameters | Whether validation is needed |
| --- | --- |
| Pathloss | Not needed |
| Penetration loss | Not needed |
| Delay spread | Needed |
| AoD/AoA/ZoA/ZoD spreads | Needed |
| ZoD offset | FFS |
| Angle distribution characteristics (e.g. exponential, Gaussian, Laplacian distributions) | FFS |
| XPR | Needed |
| Number of clusters | Needed |
| Number of rays per cluster | FFS |
| LSP correlations type (e.g. site-specific or all correlated) | FFS |
| UE antenna modeling parameters | Needed |
| K factor (mean, variance) | Not needed |

  - Correlation type of the delays among TRPs

In RAN1#117 meeting, the following agreement on correlation type of the delay was made [2].

Agreement - Further study whether/how to reflect absolute delay between links, or whether/how correlation type of the delay needs to be changed from site-specific to all-correlated type in the model - Note: site-specific and all-correlated definitions are provided in TR38.901 Section 7.6.3.4. - FFS: impact of ISD on correlation type for the deployment scenario

When multi-TRP technology is performed, at least propagation delays of multiple TRPs are correlated. Therefore, the correlation type of the delay can be changed from site-specific to all-correlated in spatial consistency model. Regarding to how to model the correlation characteristic among TRPs, legacy spatial consistency procedure in TR38.901 [3] with absolute delay modeling can be considered as a starting point.

Proposal 2: Correlation type of the delay needs to be changed from site-specific to all-correlated type in the spatial consistency model.

  - The variability of the co- and cross polar powers

In RAN1#117 meeting, the following agreement on variability of the co- and cross polar powers was made [2].

Agreement - Further study of whether/how to model the variability of the co- and cross polar powers, in both diagonal and anti-diagonals of the polarization matrix, in the TR 38.901 model. - FFS: variability is applied for per ray or per cluster or per link - FFS: impact of antenna configurations - For example, variability may be random with an i.i.d. zero-mean Gaussian with some standard deviation, via changes to step 9 and eq (7.5-22) and (7.5-28) in clause 7.5 in TR 38.901.

In TR38.901 [3], the cross polarization power ratios (XPR) for each ray m of each cluster n is log-normal distributed, and is constant with time. According to [8], average path gain (relative received power) per combination of transmit and receive polarization is varied with time, which reveals the variability of XPR. Since the measurement result of XPR is not aligned with the model in TR38.901, random power variability in each polarization and the granularity of the power variability (per ray or per cluster) can be further validated and then modelled for 7-24GHz frequencies.

Proposal 3: Variability of the co- and cross polar powers can be further validated and then modelled for 7-24GHz.

  - UE antenna modelling

In RAN1#117 meeting, the following agreement on UE antenna modelling was made [2].

Agreement - Further study whether/how following UE antenna modelling aspects should be considered in the modelling: - UE antenna placement, e.g. placement along edges of a rectangle reflecting UE form factor, - UE antenna orientation of individual antenna elements, e.g. randomize UE antenna element orientation, - Antenna radiation pattern, e.g. consider more realistic antenna patterns, including a phase component, potential reuse the parabolic pattern, - Antenna imbalance - Note: this is only used for calibration.

In TR 38.901 [3], directional antenna radiation pattern is modelled for the BS, and isotropic antenna gain pattern is assumed for UE in the calibration. With the increase of the frequency band, the number of antennas in the UE is usually increased. At the same time, the directional antenna radiation pattern can be observed. Therefore, UE antenna modeling (including UE antenna placement, orientation, and radiation pattern, etc.) can be considered for 7-24GHz.

According to [10][11], UE antennas can be placed on the front and back faces of the UE, or placed on the three edges of the UE. Due to the placement, different antennas may have different orientations. Random orientation can be modelled for each antenna (panel). In [12], directional antenna panel modelling and orientation modelling are provided for 30 GHz and 70 GHz evaluation. The orientation of the first antenna panel is defined as the UE orientation. Similar orientation modelling method can be adopted for 7-24GHz.

UE directional antenna radiation pattern in TR 38.802 [13] is shown in Table 2. Similar parabolic patterns are also provided in [11][12]. All these patterns are similar to directional pattern of the BS in both horizontal and vertical direction, except for the values of SLAV, Am, θ3dB, φ3dB. Therefore, UE directional antenna radiation pattern model in TR 38.802 can be studied as a starting point. Besides directional antenna radiation, isotropic pattern can also be selected or modelled in the simulation.

Table 2 UE directional antenna radiation pattern model in TR 38.802 [13]

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |

Proposal 4: The following UE antenna modelling aspects can be considered in the modelling:

- UE antenna placement.

- UE antenna orientation of individual antenna elements.

- Antenna radiation pattern.

- Antenna imbalance.

Proposal 5: In UE antenna orientation modeling, different antenna panels may have different orientations.

- Random orientation can be modelled for each antenna panel.

- The orientation of the first panel is defined as the UE orientation.

Proposal 6: In UE antenna radiation pattern modeling, the radiation pattern in TR 38.802 can be studied as a starting point.

  - Scaling of angles in CDL models

In RAN1#117 meeting, the following agreement on scaling for angles in CDL models was made [2].  The necessity of correcting the scaling of the angles shall be studied.

Agreement - Further study on correcting the scaling of the angles and other alternative to address angle scaling in TR38.901 Section 7.7.5 to enable accurate desired angle spread.

Figure 1 shows the difference between the generated desired angle spread based on the model in TR 38.901 and the accurate desired AOD spread in CDL-A, CDL-B, CDL-C, CDL-D and CDL-E models. According to TR 38.901, example scaling values are {5, 10, 15, 25} degrees for AOD spread (ASD) for each CDL model. It can be seen that the difference can be ignored when the accurate desired angle spread is smaller than 25 degrees. On the other hand, ASDs calculated by mean value and variance provided in TR 38.901 for UMa, UMi, and indoor-office scenarios are all smaller than 25 degrees, and the cluster ASD for these scenarios are also smaller than 25 degrees. It means that scaling angles are smaller than 25 degrees, considering the practical scenarios. Therefore, the current scaling model in TR 38.901 can still work and there is no need to update the scaling of angles in TR 38.901.

Figure 1 Difference between the generated desired AOD spread based on the model in TR 38.901 and the accurate desired angle spread in CDL-A, CDL-B, CDL-C, CDL-D and CDL-E models

Observation 5: The difference between the accurate desired angle spread and the model in TR38.901 is not evident.

Proposal 7: There is no need to update the scaling of the angles in TR38.901 section 7.7.5 to enable accurate desired angle spread.

  - Sub-urban use cases

In RAN1#117 meeting, the following observation and conclusion on sub-urban use cases were made [2]:

Observation - Some companies provided information that sub-urban deployments cannot be represented by existing deployments in TR38.901 (such as UMi, UMa, RMa). Conclusion The following parameters are used as a starting point for aligning companies understanding of channel model parameters related to suburban use cases. - BS height: [22.5] m - Layout: Hexagonal grid, 19 Macro sites, 3 sectors per site, ISD = [1732] m - Typical building heights: [Up to two floors for residential buildings, up to five floors for commercial buildings] - UT height: [1.5 or 4.5 m for residential buildings], [1.5/4.5/7.5/10.5/13.5 m for commercial buildings] - UT distribution: [Uniform horizontally, 70% indoor residential users are on ground floor, 30% are on upper floor] - FFS: ratio between residential and commercial buildings - Indoor/Outdoor: [80% indoor and 20% outdoor, FFS on in-car users] - LOS/NLOS: LOS and NLOS - Min BS - UT distance(2D): [25] m

According to the conclusion, some parameters in suburban use case are close to the parameters in UMa scenario. For example, BS height in UMa scenario is 25m, and it is 22.5m in suburban use case. The layout in suburban use case is also similar to that in UMa scenario, except for ISD. Indoor ratios of UMa scenario and suburban use case are the same. Besides, the differences of building heights, UT height, and min BS - UT distance (2D) in these two cases are not large. If pathloss model, LOS probability or other models have significant difference in these two cases, suburban use case can be seen as a sub-case of UMa scenario, otherwise, UMa scenario is enhough to characterize suburban use case.

Proposal 8: Sub-urban use case can be modelled as a sub-case of UMa scenario if necessary.

- The value of ISD can be used to differentiate whether the use case is suburban or not.

- Conclusions

In this contribution, aspects of channel model involved in the validation are discussed for 7-24GHz. Based on the discussion, the following observations and proposals are made:

Observation 1: The gap between the penetration loss related measurement results for 7-24GHz and the model in TR38.901 is not evident.

Observation 2: The gap between the delay spread measurement results in some scenarios for 7-24GHz and the model in TR38.901 cannot be ignored.

Observation 3: The gap between the angular spread measurement results in some scenarios for 7-24GHz and the model in TR38.901 cannot be ignored.

Observation 4: The gap between the number of clusters measurement results for 7-24GHz and the model in TR38.901 cannot be ignored.

Observation 5: The difference between the accurate desired angle spread and the model in TR38.901 is not evident.

Proposal 1: The assessment of the necessity for validation for channel model parameters in Table 1 can be considered.

Proposal 2: Correlation type of the delay needs to be changed from site-specific to all-correlated type in the spatial consistency model.

Proposal 3: Variability of the co- and cross polar powers can be further validated and then modelled for 7-24GHz.

Proposal 4: The following UE antenna modelling aspects can be considered in the modelling:

- UE antenna placement.

- UE antenna orientation of individual antenna elements.

- Antenna radiation pattern.

- Antenna imbalance.

Proposal 5: In UE antenna orientation modeling, different antenna panels may have different orientations.

- Random orientation can be modelled for each antenna panel.

- The orientation of the first panel is defined as the UE orientation.

Proposal 6: In UE antenna radiation pattern modeling, the radiation pattern in TR 38.802 can be studied as a starting point.

Proposal 7: There is no need to update the scaling of the angles in TR38.901 section 7.7.5 to enable accurate desired angle spread.

Proposal 8: Sub-urban use case can be modelled as a sub-case of UMa scenario if necessary.

- The value of ISD can be used to differentiate whether the use case is suburban or not.

- References

- RP-234018, New SID: Study on channel modelling enhancements for 7-24GHz for NR, RAN#102, December 11th – 15th, 2023.

- 3GPP TSG RAN WG1 Meeting RAN1 #117 Chairman's Notes, May 20th – 24th, 2024.

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz V17.1.0, 2023-12.

- R1-2405169, Channel model validation of TR 38.901 for 7-24GHz, Qualcomm, RAN1 #117, May 20th – 24th, 2024.

- R1-2404129, Discussion on channel model validation of TR38.901 for 7-24 GHz, Samsung, RAN1 #117, May 20th – 24th, 2024.

- R1-2403996, Discussion on Channel model validation of TR38.901 for 7-24GHz, Nokia, Anritsu, RAN1 #117, May 20th – 24th, 2024.

- R1-2404212, Discussion on channel model validation for 7-24 GHz, ZTE, RAN1 #117, May 20th – 24th, 2024.

- R1-2403925, Considerations on the 7-24GHz channel model validation, Huawei, HiSilicon, RAN1 #117, May 20th – 24th, 2024.

- H. Asplund et al., Propagation characteristics of polarized radio waves in cellular communications, 2007 IEEE 66th Vehicular Technology Conference, Baltimore, MD, USA, 2007, pp. 839-843.

- R. Vasanthan et al., Antenna placement and performance tradeoffs with hand blockage in millimeter wave systems, IEEE Transactions on Communications, Volume: 67, Issue: 4, pp. 3082-3096, April 2019.

- R1-2403997, Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz, Nokia, RAN1 #117, May 20th – 24th, 2024.

- ITU-R Rec. M.2412-0: Guidelines for evaluation of radio interface technologies for IMT-2020, International Telecommunication Union Radiocommunication Sector ITU-R, 10/2017.

- 3GPP TR 38.802, Study on new radio access technology physical layer aspects V14.0.0, 2017-03.