3GPP TSG RAN WG1 #118			   		             R1-2406484

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	9.7.2

Source:	Sony

Title:	Discussion on Channel Modelling for ISAC

Document for:	Discussion and Decision

# Introduction

A revised study item on integrated sensing and communication (ISAC) channel model was approved as part of 3GPP Release 19 in RANP#103 in Maastricht [1]. The objective of the study is to identify ISAC deployment scenario and further define channel modeling details to support the deployments and use cases. In RAN1#117 [2], The following agreements were made mainly on modeling of the target channel and others:

Agreement - Multiple sensing targets can be modelled in the ISAC channel of a pair of sensing Tx and sensing Rx - FFS whether to model a propagation path from Tx to Rx interacting with more than one sensing target - The same sensing target can be modelled in the ISAC channels of multiple pairs of sensing Tx and Rx Agreement - For discussion purpose, the propagation paths in the target channel are classified - The direct path, i.e., LOS ray from Tx to target + LOS ray from target to Rx - The indirect paths, i.e., any propagation path other than the direct path, including - LOS ray from Tx to target + NLOS ray from target to Rx - NLOS ray from Tx to target + LOS ray from target to Rx - NLOS ray from Tx to target + NLOS ray from target to Rx - For radio propagation Case 1, - For a direct path, the following parameters are [deterministically] generated at least based on the geometry location of Tx, target and Rx - AoA/ZoA at Rx - AoD/ZoD at Tx - AoA/ZoA/AoD/ZoD at target - delay - FFS initial phase - Doppler - FFS power/polarization including the impact of RCS - FFS the number of direct path(s) for a target - FFS on detailed modelling of indirect path(s) - FFS on details of modelling of indirect paths in radio propagation Case 2/3/4 - To generate the channel coefficients of direct/indirect path(s) in the target channel, the channel coefficient generation function in step 11 in section 7.5 of TR 38.901 (e.g., formula 7.5-22) is used as the start point - Note: modification to step 11 is deemed necessary - FFS adding impact of small scale RCS - FFS Doppler Agreement - Spatial consistency should be supported for ISAC channel - Spatial consistency should be supported based on movement of sensing Tx, sensing target and/or sensing Rx - FFS EO handling Agreement When the stochastic cluster is used to generate the indirect paths in the target channel of a target - The stochastic cluster generation in section 7, TR 38.901 is used as starting point. - FFS a stochastic cluster is generated between Tx and Rx satisfying Tx-target-Rx geometry, or between Tx/Rx and target - FFS modification to stochastic cluster generation in section 7, TR 38.901 - FFS use of sub-cluster to model the indirect paths Note: RAN1 continues studying using EO to generate the indirect paths in the target channel of a target Agreement When the stochastic cluster is used to model indirect path in the target channel - For bistatic, the LOS condition from Tx to target and from target to Rx is determined separately for a target - FFS: The correlation of LOS condition of Tx-target and Rx-target links of a target - For monostatic, a same LOS condition is determined for Tx to target and target to Rx - The LOS condition from Tx to target and/or from target to Rx is determined with the LOS probability - The probability schemes in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, etc. are considered as start point - FFS: How to consider the impacts of target height on LOS probability. Agreement When stochastic cluster is used to model indirect path in the target channel, down-select between the following options - Option 1: modelled by concatenation of path(s) from Tx to target and from target to Rx - For each of the Tx-target link and target-Rx link, - The parameters delay, power, angle, [initial phase], [Doppler] of NLOS ray(s) in the link Tx-to-Target or Target to RX are generated - FFS following cluster generation in section 7, 38.901 - The target channel is generated by concatenating the parameters of the Tx-target link and target-Rx link. - FFS on Convolutional or 1-by-1 coupling or 1-to-many coupling - FFS how to combine the clusters in target channel and the clusters in background channel - Option 2: modelled by Tx-to-Rx path(s) satisfying Tx-target-Rx geometry of the direct path - The parameters delay, power, angle, initial phase of a stochastic (sub-)cluster between Tx and Rx are generated following cluster generation in section 7, 38.901 - The parameters [delay], [power], [angle], [Doppler] of the (sub-)cluster are updated by the target property - FFS how to combine the clusters in target channel and the clusters in background channel

In this contribution, we will discuss our views and preferences on channel modeling, with main focus on the required new features to support sensing targets modeling.

# Discussion

## ISAC channel model

### Topology of ISAC channel model

In our view, the common topology for ISAC channel model can be illustrated in Figure 1. It was agreed in the previous meeting that the ISAC channel is composed of target channel and background channel. The target channel includes all the multipath components impacted by the sensing target and the background channel including other multipath components not belonging to the target channel. In our understanding, this proposed topology suggests defining five different sensing entities: Tx node, Rx node, Sensing Target (T), Environment Object (EO) and Background (B). The Sensing Target (T) here refers to the passive objects such as vehicles, UAVs and humans that are to be detected by the sensing operation. In contrast, Environment Object (EO) and Background (B) represent objects or clutters that are not of interest to the sensing operation (i.e., other objects than the sensing target objects). The key distinction between EO and B is their location information, EO has known location, but the B does not.

Figure 1: Illustration of the ISAC channel model topology

In addition to the sensing target, the inclusion of EO(s) was proposed in the sensing channel components. In our view, the EO(s) were introduced in ISAC channel modelling primarily to account for the two-bounce propagation link Tx-T-EO1-Rx or TX-EO2-T-RX, which might be utilized to detect or localize the sensing target. Therefore, modelling the link between T-EO becomes necessary. In terms of the channel modelling from environment object to different other entities, such as Tx-EO, EO-Rx and T-EO, we prefer to model them with only LOS ray. We believe that adding NLOS rays to these links, such as a link Tx-T-B-EO-Rx or a link Tx-EO-B-Rx, would not significantly contribute to the sensing performance, as the power level of such links with NLOS rays is relatively small and considered negligible.

Proposal 1:Environment object related links (i.e., Tx-EO, EO-Rx and T-EO) should contains LOS ray only in target channel modelling. This can reduce the modelling complexity.

As for the EO modelling, two types of environment objects, EO type-1 and EO type-2, were considered in the target channel modelling as discussed in RAN1#116bis meeting. EO type-1 is defined similar to the sensing target such as a vehicle or a human in the automotive scenario, or a bird in the UAV scenario. This type of EO can be modeled by following the same way as modeling of the sensing target. In contrast, EO type-2 is different from the sensing target. It represents the objects which has extremely large size like a ground or a wall of building. This type of EO can be modeled by reusing the ground reflection feature as described in section 7.6.8 in TR 38.901.  In our view, both of these EO types may have influence on the signal scattered through the target, e.g., Tx-EO-T-Rx, and thus should be considered in the target channel.

Proposal 2: Both of the EO type-1 and EO type-2 should be considered in the target channel modelling.

Proposal 3: Reflection from the ground (EO type-2) can be modelled based on section 7.6.8 in TR 38.901.

The background channel of the ISAC channel includes other multipath components not belonging to target channel as illustrated in Figure 1. It comprises the LOS ray between the Tx and Rx, the rays scattered from the background, and the rays scattered by the environment object such as the EO1 as shown in the figure. We consider that the environment objects EO1, which is common to both target and background channel, can be utilized to improve the sensing performance. For instance, in Figure 1 EO1 is involved into two types of links in the channel topology: the blue (Tx-T-EO1-Rx) and the black (Tx-EO1-Rx) links. As a way to obtain target information, the sensing receiver may consider taking the properties difference between the blue link in the target channel and the black link in background channel. Thus, the common environment object, i.e., the EOs that is common to both the target channel (i.e., TX-Target-EO-RX) and the background channel (TX-EO-RX) should be considered in the channel modelling.

Proposal 4: The environment object (EO) that are common to both target channel and background channel, such as EO1, should be considered in the channel modelling.

### Target channel model

RAN1#117 meeting made an agreement on these two options to generate the target channel.

- Option1: modelled by concatenation of path(s) from Tx to target and from target to Rx.

- Option2: modelled by Tx-to-Rx path(s) satisfying Tx-target-Rx geometry of the direct path.

We consider both option 1 and option 2 can be used to model the target channel. It can also achieve a similar performance if the channel parameters are well calibrated. In our view, the only distinction between option 1 and option 2 is the selection of the ray which act as the reference. In TR 38.901 geometry-based channel model, the reference ray is the LOS ray from Tx and Rx, which need to be generated based on the geometry location of Tx and Rx. When further generating the MPCs (e.g., calculating the power, delay and angle of the MPCs), the channel model will take the LOS ray from Tx to Rx as the reference. For instance, the powers of clusters are scaled by k-factor in relation with the power of the reference ray. The delays of clusters are also scaled by the delay spread in relation with the delay of the reference ray. In the light of the communication channel model design, we observe that the reference ray in Option 1 and Option 2 are not the same: Option 1 needs separate modelling of Tx-Target and Target-Rx and thus the reference rays are the Tx-Target LOS ray and the Target-Rx LOS ray, while in Option 2, the reference ray is the scattered Tx-Target-Rx ray.

Observation 1: Concatenation method (Tx-target + Target-Rx) and joint method (Tx-Target-Rx), have different reference rays when generating MPCs. Concatenation method takes the LOS Tx-Target ray and the LOS Target-Rx ray as references in generating MPCs, while joint method takes the scattered ray Tx-Target-Rx as the reference.

Therefore, we can foresee that the selection of the modelling method may not have big impact to the performance. On the other hand, the essential part that may impact the performance is the small-scale parameters which are measured relative to the reference ray. From the aspect of channel measurement, we think that the channel parameters defined for communication channel between Tx and Rx in TR 38.901 can be mostly reused in option 1 for modelling of Tx-Target and Target-Rx. The target (such as human) may have the same/similar location distribution as UE and subsequently experience a similar channel. By considering that, we would slightly prefer option 1 considering the easiness of channel measurement in the calibration phase.

Proposal 5: Support using concatenation method (Option 1), e.g., concatenating of path(s) from Tx to target and from target to Rx, to model the target channel, for the ease of channel parameters measurement in the calibration phase.

As for the target channel generation of the concatenation method, two methods have been identified. Method 1 is by coupling of the Tx-target clusters and the target-Rx clusters. Method 2 is by convolving the channel impulse response of the Tx-target channel and the target-Rx channel. Between these two methods, we prefers the convolution-based method as it can mostly reuse the existing channel modelling method with minor specification change. In concrete, we propose the following steps:

- Step 1: Generate the LOS/NLOS condition of Tx-target link and target-Rx link reuse TR38.901/36.777/37.885.

- Step 2: Generate the overall pathloss by calculating the pathloss of Tx-target link and target-Rx link independently reusing TR 38.901 and the RCS of the target also needs to be reflected in the pathloss equation.

PLtarget,dBd1,d2,σRCS,f=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS

- Step 3: Generate the small-scale parameters of direct/indirect path(s) from Tx to target, and from target to Rx independently. The small scale parameters include cluster angle (i.e., AOA/AOD/ZOA/ZOD), cluster delay, cluster power, initial phases, doppler, XPR, etc., and the small scale parameters generation in section 7.5 of TR 38.901 is considered as the baseline.

- Step 4: Generate TX-target and target-RX channel impulse response in a similar way as step 11/12 in section 7.5 of TR38.901.

- Step 5: Convolve two channel impulse responses into one.

Proposal 6: The target channel is modelled by convolution of Tx-to-Target channel and Target-to-Rx channel.

As we proposed in proposal 1, environment object related links (i.e., Tx-EO, EO-Rx and T-EO) should contains LOS ray only in the target channel modelling. If the EO type-1 is considered in the channel, it primarily contributes to the indirect path in the target channel such as the Tx-EO-Target-Rx link. Also as EO type-1 shares the same physical characteristics with sensing target, it can basically reuse the same model as the target, i.e., the single or multiple scattering point model. Therefore, we suggest postponing the EO type-1 discussion until RAN1 makes any progresses on target modelling.

Regarding the EO type-2 modelling, the ground reflection model described in TR 38.901 chapter 7.6.8 can be reused as a starting point, to explicitly model the reflection from the ground in sensing channel. While for the other EO type-2 with a large and flat surface such as building, wall and ceiling, this model is still applicable, but a modification is needed. Furthermore, we shall discuss the EO type-2 assumptions (EO type-2 type, material, size and layout) under different scenarios, and further study how to combine channel components from different EO type-2s in the target channel.

Observation 2: EO type-1 modelling characteristics can be the same as the sensing target modelling, e.g., single or multiple scattering points assumption.

Proposal 7: EO type-1 modelling discussion shall be postponed after RAN1 makes consensus on the target modelling.

Proposal 8: EO type-2 modelling need to be defined in different sensing scenario and further study the method to combine channel components from different EO type-2s in the target channel.

### 2.1.3 Background channel modelling

We discussed in section 2.1.1 that the background channel contains sensing TX, sensing RX, stochastic clusters and common EO.  As illustrated in Figure 1, the background channel includes the direct path from TX to RX and multiple indirect paths generated by the stochastic clutters and common EO, those indirect paths generated by the stochastic cluster can be modeled reusing the 7.5 of [4]. As for the indirect path passing through the EO in background channel, for the EO type-2, we can model the path impact similar with EO type-2 in target channel. For the EO type-1 is intended target, it should be modelled in the same way as the sensing target to make channel modelling simpler.

Proposal 9: In background channel, indirect path generated by stochastic clutters is modeled in the same way as in communication channel described in TR 38.901.

Proposal 10: In background channel, indirect path generated by the common EO is modeled in the same way as in the target channel.

## Sensing Target Modelling

In RAN1#116bis meeting [5], one of the main discussions was about whether to model the sensing target with single or multiple scattering points. Based on our understanding, the multiple scattering points model will be used mainly for object identification. This feature will allow the sensing receiver to resolve the sensing signals in delay or angular domain, which is not feasible for single scattering point model. For example, a target may have multiple scattering points, and each of scattering points may have their unique interactions with the sensing signal that leads to different scattered ray toward the outcoming direction. This further leads to a result that, the scattered rays have different propagation time and different arrival angle at the receiver side. Correspondingly, a “spread” may appear in the measured channel response.

Figure 2 depicts a ray-tracing simulation of bi-static sensing for vehicle detection. In this simulation, a 4.7m x 1.8m x 1.68m size cuboid vehicle with metal surface works as a target object. A Tx and a Rx are placed 20 meters apart from the geometric center of the vehicle with a 90-degree bi-static angle. Bi-static angle in this case is defined as the angle at the target between the direction of the incoming signal from the transmitter and the direction of the outgoing signal to the receiver. Since the sensing channel in the simulation doesn’t have any environmental object/background object, the measurement would be solely depending on the Tx-Rx LOS ray and the reflection and scattering rays interacted with the target object which may indicate the RCS value (or the RCS components at different delay or angle.).

The right figure shows the CIR measurement using 2GHz bandwidth (i.e., for high resolution sensing purposes). As one can observe, the rays from different scattering points can be separated/resolved in the delay spectrum. Each tap or sample in the CIR represents a summing of rays sharing the same propagation delay. In such Rx/Tx configuration, the second arrival tap in the CIR indicates the specular reflection, as it has the shortest propagation time in the group of taps. Following by the specular reflection tap, there are multiple extensive taps representing the scattered rays from multiple scattering points at the vehicle. The scattered rays with the same propagation time may have different incident angles (phase) and thereby could be combined constructively or destructively, which leads to the fluctuation in the extension.

Figure 2: Ray-tracing simulation of bi-static vehicle sensing with multiple scattering points.

As one can also observe, the duration of the extension (or it can be measured by rms delay spread) is around 4ns and which can be further translated to 1.2 meters propagation range difference between the specular reflection ray and the longest scattered ray. This information could be useful in object identification. Therefore, we propose RAN1 to study the possibility of deterministic multiple-scattering point model. In concrete, as one way forward, we can start from studying the power distribution in delay or angular spectrum due to the multiple scattering points.

Observation 3: Object with multiple scattering points may generate multiple rays at different propagation time and arrival angle, which can be utilized for object identification.

Proposal 11: Study the power distribution in delay and angular spectrum due to the multiple scattering points, assuming the rays interacting with different scattering points have different propagation time and outcoming angles.

The SID defines the sensing targets to be studied during the study item phase, such as UAV, vehicle and human [1]. The reference [3] describes a common dimension for each of them: UAV [0.3 m x 0.3 m x 0.1 m], Vehicle [4.7 m x 1.7 m x 1.5 m] and Human [0.5 m x 0.5 m x 1.75 m]. It can be observed that the maximum dimension of Human (1.75m) and Vehicle (4.7m) are larger than the delay resolution 0.75m in FR2 system. Therefore, these two types of targets should consider to be modeled using multiple scattering points model. In contrast, UAV has smaller size than the resolution and it should consider single scattering point only.

Proposal 12: For modeling of vehicle and human, consider applying multiple scattering points, while for UAV, use single scattering point model only.

In RAN1#116b meeting, we also discussed the RCS modelling when single scattering point model is used. Three options were considered:

- RCS value randomly generated by a distribution,

- RCS is a deterministic value, and

- Combination of both 1) and 2).

In our view, the above-mentioned agreement with 3 options basically covers all possible ways to generate the RCS values, e.g., deterministic, stochastic, and the combined method. However, it does not elaborate in which specific RCS groups these methods are applied. We propose RAN1 to discuss / study the follow up discussion on these three options and the modelling of multiple RCS values, such as:

- The modelling of multiple RCS values among different target drops, if each target drop has only one RCS value.

- The modelling of multiple RCS values of a specific type of target, if one target type has multiple RCS values, e.g., at different angle or scattering points.

Proposal 13: Further study on the RCS modelling with a group of RCS values:

- Option 1: The modelling of multiple RCS values among different target drops, if each target drop has only one RCS value.

- Option 2: The modelling of multiple RCS values of a specific type of target, if one target type has multiple RCS values, e.g., at different angle or scattering points.

## Spatial Consistency Procedure

Spatial consistency procedure is essential in ISAC, especially for drop-based model where clusters are generated stochastically. This feature ensures some levels of correlation among links connecting different entities, such as UEs and gNBs, throughout the space. It is especially important for the closely located links, as this method provides similar channel coefficients, instead of entirely random values.

Reference [4] defines spatial consistency procedure for generating channel coefficients of links from gNB to UEs. In legacy communication, one gNB may communicate with multiple UEs. As depicted in Figure 3, multiple UEs (UE1, UE2 and UE3) are dropped randomly at different locations within the same cell. gNB1 establishes communication with these three UEs, resulting three links, e.g., link gNB1-UE1, link gNB1-UE2 and link gNB1-UE3.

Figure 3: A typical topology of channels for communication.

The channel coefficients, including LOS/NLOS states, cluster power/delay/angle, XPR, initial phase, are generated via a spatial consistency procedure. For the simulation purpose, this procedure can be carried out by three steps:

Calculate the correlation coefficients (or a correlation matrix) between links, such as the correlation coefficient between link gNB1-UE1 and link gNB1-UE2.

Generate a set of Gaussian (Normal) distributed N(0, 1) (or uniform distributed U[0, 1]) random variables that follows a prescribed correlations generated from the previous step.

Compute the channel coefficients based on the correlated random variables using the model provided in TR 38.901 [4].

In step 1, the correlation between one gNB-to-UE link and another gNB-to-UE link is considered. For simplicity, a mathematical form Coxx UEa-gNBiUEb-gNBj is used here to describe the correlation coefficient of the link UEa-gNBi and the link UEb-gNBj. The UE and gNB in these two links can refer to either the same or different entities, resulting in four 4 different combinations of links: {a≠b, i=j, a=b, i≠j, a=b, i=j, a≠b, i≠j}.

Reference [4] defines a so-called site-specific correlation, indicating that the co-sited links, e.g., the UE-gNB links that are established within the same gNB site (a≠b, i=j), are correlated. The corresponding correlation coefficients is an exponential decay function dependent of the distance between two UEs.

Coxx UEa-gNBiUEb-gNBj=exp-dUEa-UEbdcorr,        a≠b, i=j                    0 ,                    a=b, i≠j                    1 ,                    a=b, i=j                   0 ,                    a≠b, i≠j

Where dUEa-UEb denotes the distance from UEa and UEb. dcorr denotes the correlation distance.

For other combinations, the links between different gNBs to the same UE (a=b, i≠j), and the disjoint links (a≠b, i≠j), are defined as uncorrelated, resulting in correlation coefficients of 0. Conversely, the links from same UE drops and same gNB drops (a=b, i=j) are fully correlated, thus the correlation coefficient is 1. According to this equation, one can observe that the correlated random variables are independent to gNB’s location. Only the UE-to-UE distance is relevant.

Observation 4: Channel coefficients, including LOS/NLOS states, cluster power/delay/angle, XPR, initial phase is defined as site-specific in TR 38.901. The correlation of the co-sited links is dependent of UE-to-UE distance.

In Step 2, after calculating the correlation coefficients or a correlation matrix, one can generate a set of correlated Gaussian (Normal) distributed N(0, 1) (or uniform distributed U[0, 1]) random variables. This can be done by various approaches. One typical approach is by Cholesky decomposition. To visualize the spatial distribution of these variables, we simulate a Gaussian random variable map, as shown in Figure 4. The upper plot depicts the map with spatial consistency, while the lower one depicts the map without consistency. In this example, random variables are generated at each 1m-by-1m grid in the layout, resulting in a total of 120-by-60 samples of random variables captured in each plot.

Figure 4: a set of Gaussian distributed N(0, 1) random variables Σ generated with / without spatial correlation. Each of the colored grid has size of 1 m2.

Based on this simulation, we can observe a different random variables distribution across the space. In the upper plot, where spatial consistency is applied, the random variables evolve smoothly. This guarantees two similar channel coefficients can be observed at two spatially close UEs. his spatial consistency feature benefit not only the scenario where two separate UEs are close to each other but also scenarios involving a single moving UE. This is because a moving UE can be modeled as multiple correlated channel realizations at different location along its trajectory.  In contrast, lower plot shows rapid changes in random variables across space, which may not accurately represent the real-world channel characteristics.

Observation 5: Spatial consistency allows the channel coefficients evolve smoothly throughout the space, which accurately represent the real-world channel characteristics.

Step 3 is simply a procedure of transforming a set of normal distributed random variables Σ ~ N(0, 1)  into channel coefficients. For example, reference [4] defines a mapping between Normal-distributed random variable and the LSPs. This can be achieved by simply scaling up the correlated random variables to a log-normal distribution, i.e.,  lg⁡(X) ~ N(μ, σ2), with specified μ and σ value for different scenarios.

For ISAC channel modeling, we propose reusing the methodology outlined above as much has possible to support channel modeling between objects (i.e., sensing targets, environment objects) and gNBs/UEs. Specifically, we can apply the aforementioned three steps, to generate multiple correlated channel coefficients. However, to adopt a similar procedure, we need to first address the following two aspects regarding the spatial correlation in ISAC:

- Identifying which two links in ISAC topology should be considered correlated and which are not.

- Determining how to model the correlation coefficient among different links – whether it is distance/angle-dependent or constant.

Proposal 14: RAN1 to study these two aspects for defining new spatial correlation for ISAC:

- Identifying which two links in ISAC topology should be considered correlated and which are not.

- Determining how to model the correlation coefficient among different links – whether it is distance/angle-dependent or constant value.

Regarding the first aspect, ISAC has more complicated topology than the legacy positioning systems. It involves five different sensing entities: Tx, Rx, Sensing Target (S), Environmental object (EO), Background (B). Each of the entities may have connections to others, resulting in various types of links, such as link Tx-Rx, Tx-S, S-Rx, S-EO, etc. But these links are modeled via different approaches. Some of the links may be modeled as LOS ray only, while other links may be modeled as LOS ray + NLOS rays.

In our view, the links containing only LOS ray inherently support spatial consistency as LOS properties, like delay and angle, are dependent of the geometry location of the sensing entities. In contrast, the links modeled as LOS ray + NLOS rays requires specific spatial consistency procedure, especially when the NLOS rays are modeled stochastically using model as described in [4].

Among the various identified ISAC links, the links S-EO and E-Rx may contain only LOS ray, thus natively supporting spatial consistency. In contrast, the links Tx-Rx, Tx-S and S-Rx may comprise both LOS ray and stochastic NLOS rays. Therefore, additional spatial consistency procedure should be considered in these cases.

Observation 6: In ISAC channel topology, the links with only LOS ray inherently support spatial consistency and thus no additional procedure should be carried out in simulation. Specifically, these links could be the link S-EO and link EO-Rx.

Observation 7: For the links modeled with multipath components, additional spatial consistency procedure should be carried out, especially if the clutters are generated stochastically (e.g., using the method as described in TR 38.901 [4]). Specifically, these links could be the link Tx-Rx, Tx-S and S-Rx.

Regarding the second aspect, given the fact that the Tx-Rx links, Tx-S links and S-Rx links may contains stochastic multipath components, defining a new correlation among those links becomes necessary. To simplify to problem, we treat Tx and Rx (or gNB and UE) as the same entities, denoted as AN (Access Node). Thus, we have the bi-static sensing topology depicted in Figure 5. In this case, with two ANs and two sensing targets, we have five different links, e.g., AN1-AN2, AN1-S1, S1-AN2, AN1-S2 and S2-AN2. In this specific example, the correlations among five links, such as the correlation between AN1-S1 and AN1-S2, the one between AN1-AN2 and AN1-S1, the one between AN2-S1 and AN2-S2, etc., should be considered.

Figure 5: Bi-static sensing topologies with only stochastic links.

To extend the case to a larger scale with more ANs and more sensing targets, we use the mathematical form to describe the problems. The correlation can be categorized into two different types:

1) Correlation between AN-S link and another AN-S link denoted as Coxx ANa-SiANb-Sj.

2) Correlation between AN-AN link and AN-S link denoted as Coxx ANa-ANbANc-Si.

The subscripts a, b, c, i, j in these notations may vary in different scenarios, resulting in different correlation. In particular, we select the following scenarios that should be taken care of in ISAC:

- Correlation between AN-S link and another AN-S link Coxx ANa-SiANb-Sj , in cases where {a≠b, i=j, a=b, i≠j, a=b, i=j, a≠b, i≠j}.

- Correlation between AN-AN link and AN-S link Coxx ANa-ANbANc-Si, in cases where {( a≠b, c=a), (a≠b, c=b), (a≠b≠ c)}.

Proposal 15: For ISAC spatial consistency, two types of correlations should be considered: 1) Correlation between AN-S link to another AN-S link, 2) Correlation between AN-AN link to AN-S link. Note: AN is access node (UE/gNB), S is Sensing target.

We discover the 3 options to model the correlation between AN-S link to another AN-S link Coxx ANa-SiANb-Sj:

Option 1: Sensing target-specific (object-specific) correlation:

Coxx ANa-SiANb-Sj=                  0 ,                    a≠b, i=j       exp-dSi-Sjdcorr1,        a=b,  i≠j                    1 ,                    a=b, i=j                   0 ,                    a≠b, i≠j

Where  dSi-Sjis the distance between the i-th sensing target and the j-th sensing target,  dcorr1 is the correlation distance.

Option 2: UE/gNB-specific consistency:

Coxx ANa-SiANb-Sj=  exp-dANa-ANbdcorr2,        a≠b, i=j                      0 ,                    a=b, i≠j                     1 ,                    a=b, i=j                    0 ,                    a≠b, i≠j

Where  dANa-ANbis the distance between the a-th AN and the b-th sensing target,  dcorr2 is the correlation distance.

Option 3: New consistency:

Coxx ANa-SiANb-Sj=  exp-dANi-ANjdcorr1,        a≠b, i=j        exp-dSi-Sjdcorr2,        a=b,  i≠j                    1 ,                    a=b, i=j                    0 ,                    a≠b, i≠j

The first 2 options are defined as a similarly to the spatial consistency procedure as described in [4], focusing on defining correlations between two links that share a common end. In option 1, links sharing a specific sensing target are considered as correlated, whereas in option 2, links sharing a specific AN are correlated.

However, option 1 and 2 only achieve correlation among a subset of links. In option 1, only the links from multiple ANs to the same target are correlated, while in option 2, only the links from one AN to multiple targets are correlated. One may want to achieve correlation across all the ISAC links. In this case, a new correlation such as option 3 is necessary, which defined correlations across all links. It is worth noting that different exponential decay functions may have different correlation distances, denoted as dcorr1 and dcorr2 respectively. These parameters shall be further defined in the calibration phase in the later stage.

Proposal 16: For modeling spatial correlation between Link UE/gNB – sensing target and another Link UE/gNB-target Coxx ANa-SiANb-Sj, the following three options should be considered:

- UE/gNB-specific spatial correlation (Correlated among a subset of links)

- Sensing target-specific spatial correlation (Correlated among a subset of links)

- New spatial correlation supporting both the above two correlations (Correlated across all the links)

In our perspective, option 1 and 2 have both pros and cons. Both options can be applied depending on the sensing use cases. For instance, in mono-static/bi-static object tracking scenarios, where a gNB/UE may need to continuously monitor one specific sensing target and measure different sensing channel realizations along its trajectory. Option 1 - sensing target-specific consistency may be preferred as this method provides an incremental or temporally correlated changes of the channel coefficients across different realizations.  In the other cases such as multi-static sensing, option 2 has advantage since it provides correlation between links from multiple UEs/gNBs to one single target.

Option 3, in comparison to option 1 and 2, can achieve correlation across all the links. However, it comes with downside of increased computational complexity. Option 3 introduces more pairs of correlated links, which significantly enlarges the correlation matrix. Consequently, the processing time for decomposing the correlation matrix will also increase significantly.

Observation 8: Spatial correlation by different options is summarized in the table below:

|  | Applicability | Pros | Cons |
| --- | --- | --- | --- |
| Option 1: Sensing target-specific | Links from multiple UEs/gNBs to a sensing target | 1. low complexity 2. works well in multi-static sensing | Inconsistent channels for links from one UE/gNB to multiple targets |
| Option 2: UE/gNB-specific | Links from one UE/gNB to multiple sensing target or one moving target with multiple realizations | 1. low complexity 2. works well in mono-/bi-static sensing | Inconsistent channels for links from one target to multiple UEs/gNBs |
| Option 3: New correlation | All the links | Can be applied in all scenarios | High computational complexity |

Furthermore, the correlation between link AN-AN and link AN-S can also be modeled in different ways. For simplicity, one way is not defining any correlation, e.g.,  Coxx ANa-ANbANc-Si=0, as the consistency of the clutters of link AN-AN may not significantly impacting sensing outcome.

Proposal 17: For modeling simplicity, the link UE/gNB-UE/gNB and Link UE/gNB-target can be modeled as uncorrelated, e.g, Coxx ANa-ANbANc-Si= 0.

## Doppler shift in ISAC channel

In sensing channel modeling, the movement of Tx, target and Rx during the sensing operation impacts the Doppler frequency component in fast fading modeling. As described in TR 38.901 chapter 7.6.10 [4], doppler frequency in communication channel is determined by the velocity vectors of TX and RX. However, The sensing channel model is not only involving Tx and Rx, but also the sensing target, the doppler frequency component should be updated by the following equation:

vn,m=rrx,n,mT∙vrx-vtarget+rtx,n,mT∙(vtx-vtarget)λ0

Where vn,m denotes the doppler frequency compenent at the n-th cluster and the m-th ray,  rrx,n,mT, rtx,n,mT refer to the direction spherical unit vector from Rx to target, and the one from Tx to target respectively, vrx, vtx and vtarget denote velocity vectors of Rx and Tx and sensing target, λ0 is the wavelength of the sensing signal.

Regarding micro doppler modeling, we consider this is beneficial for sensing. It can be studied in the later stage of the SI. In some sensing use cases such as micro movement recognition and object detection between two similar shapes, relying solely on RCS for target recognition may not always be reliable. An alternative solution could be the Micro-Doppler assisted method, which has proven effective in some use cases.

As the ISAC channel model study item aims to establish a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects. We consider first study the dual-doppler formula in fast fading modeling with considering both the doppler effect between Tx and target and the doppler effect between target and Rx. Subsequently, we can further study the micro-doppler model for object identification.

Proposal 18: In ISAC channel modeling, prioritize to study the dual-doppler formula (i.e., the doppler effect between Tx and target and the doppler effect between target and Rx) in fast fading modeling over the study on micro-Doppler.

# Conclusion

In this contribution, we share our views on the development of ISAC channel model. We made the following observations:

Observation 1: Concatenation method (Tx-target + Target-Rx) and joint method (Tx-Target-Rx), have different reference rays when generating MPCs. Concatenation method takes the LOS Tx-Target ray and the LOS Target-Rx ray as references in generating MPCs, while joint method takes the scattered ray Tx-Target-Rx as the reference

Observation 2: EO type-1 modelling characteristics can be the same as the sensing target modelling, e.g., single or multiple scattering points assumption.

Observation 3: Object with multiple scattering points may generate multiple rays at different propagation time and arrival angle, which can be utilized for object identification.

Observation 4: Channel coefficients, including LOS/NLOS states, cluster power/delay/angle, XPR, initial phase is defined as site-specific in TR 38.901. The correlation of the co-sited links is dependent of UE-to-UE distance.

Observation 5: Spatial consistency allows the channel coefficients evolve smoothly throughout the space, which accurately represent the real-world channel characteristics.

Observation 6: In ISAC channel topology, the links with only LOS ray inherently support spatial consistency and thus no additional procedure should be carried out in simulation. Specifically, these links could be the link S-EO and link EO-Rx.

Observation 7: For the links modeled with multipath components, additional spatial consistency procedure should be carried out, especially if the clutters are generated stochastically (e.g., using the method as described in TR 38.901 [4]). Specifically, these links could be the link Tx-Rx, Tx-S and S-Rx.

Observation 8: Spatial correlation by different options is summarized in the table below:

|  | Applicability | Pros | Cons |
| --- | --- | --- | --- |
| Option 1: Sensing target-specific | Links from multiple UEs/gNBs to a sensing target | 1. low complexity 2. works well in multi-static sensing | Inconsistent channels for links from one UE/gNB to multiple targets |
| Option 2: UE/gNB-specific | Links from one UE/gNB to multiple sensing target or one moving target with multiple realizations | 1. low complexity 2. works well in mono-/bi-static sensing | Inconsistent channels for links from one target to multiple UEs/gNBs |
| Option 3: New correlation | All the links | Can be applied in all scenarios | High computational complexity |

We have also made the following proposals:

Proposal 1:Environment object related links (i.e., Tx-EO, EO-Rx and T-EO) should contains LOS ray only in target channel modelling. This can reduce the modelling complexity.

Proposal 2: Both of the EO type-1 and EO type-2 should be considered in the target channel modelling.

Proposal 3: Reflection from the ground (EO type-2) can be modelled based on section 7.6.8 in TR 38.901.

Proposal 4: The environment object (EO) that are common to both target channel and background channel, such as EO1, should be considered in the channel modelling.

Proposal 5: Support using concatenation method (Option 1), e.g., concatenating of path(s) from Tx to target and from target to Rx, to model the target channel, for the ease of channel parameters measurement in the calibration phase.

Proposal 6: The target channel is modelled by convolution of Tx-to-Target channel and Target-to-Rx channel.

Proposal 7: EO type-1 modelling discussion shall be postponed after RAN1 makes consensus on the target modelling.

Proposal 8: EO type-2 modelling need to be defined in different sensing scenario and further study the method to combine channel components from different EO type-2s in the target channel.

Proposal 9: In background channel, indirect path generated by stochastic clutters is modeled in the same way as in communication channel described in TR 38.901.

Proposal 10: In background channel, indirect path generated by the common EO is modeled in the same way as in the target channel.

Proposal 11: Study the power distribution in delay and angular spectrum due to the multiple scattering points, assuming the rays interacting with different scattering points have different propagation time and outcoming angles.

Proposal 12: For modeling of vehicle and human, consider applying multiple scattering points, while for UAV, use single scattering point model only.

Proposal 13: Further study on the RCS modelling with a group of RCS values:

- Option 1: The modelling of multiple RCS values among different target drops, if each target drop has only one RCS value.

- Option 2: The modelling of multiple RCS values of a specific type of target, if one target type has multiple RCS values, e.g., at different angle or scattering points.

Proposal 14: RAN1 to study these two aspects for defining new spatial correlation for ISAC:

- Identifying which two links in ISAC topology should be considered correlated and which are not.

- Determining how to model the correlation coefficient among different links – whether it is distance/angle-dependent or constant value.

Proposal 15: For ISAC spatial consistency, two types of correlations should be considered: 1) Correlation between AN-S link to another AN-S link, 2) Correlation between AN-AN link to AN-S link. Note: AN is access node (UE/gNB), S is Sensing target.

Proposal 16: For modeling spatial correlation between Link UE/gNB – sensing target and another Link UE/gNB-target Coxx ANa-SiANb-Sj, the following three options should be considered:

- UE/gNB-specific spatial correlation (Correlated among a subset of links)

- Sensing target-specific spatial correlation (Correlated among a subset of links)

- New spatial correlation supporting both the above two correlations (Correlated across all the links)

Proposal 17: For modeling simplicity, the link UE/gNB-UE/gNB and Link UE/gNB-target can be modeled as uncorrelated, e.g, e.g, Coxx ANa-ANbANc-Si= 0.

Proposal 18: In ISAC channel modeling, prioritize to study the dual-doppler formula (i.e., the doppler effect between Tx and target and the doppler effect between target and Rx) in fast fading modeling over the study on micro-Doppler.

# References

- RP-240799, Revised SID: Study on channel modelling for Integrated Sensing and Communication (ISAC) for NR, Xiaomi, AT&T, RAN Meeting 103, Maastricht, Netherlands, March 18-21, 2024.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

- TR 22.837, Feasibility Study on Integrated Sensing and Communication, v19.2.0.

- 3GPP TR 38.901 V18.0.0, Study on channel model for frequencies from 0.5 to 100 GHz (Release 16), 2024-03

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #116-bis, Changsha, Hunan Province, China, April 15th-19th, 2024,