# 3GPP TSG-RAN WG1 Meeting #118                                       R1-2406628

# Maastricht, The Netherlands, August 19th – 23rd, 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on BFD RS set determination for cell-specific BFR under the Rel-18 unified TCI framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | How a UE would implicitly determine a BFD RS set q0 for cell-specific BFR was only specified under the legacy Rel-15/16 TCI framework – the BFD RS set q0 can be implicitly determined from TCI-State indicated for respective CORESETs. Furthermore, under the Rel-18 unified TCI framework, supporting the TRP-specific BFR is an optional UE feature; hence, for a UE that does not report/support this feature, the UE needs to implicitly determine a BFD RS set q0 according to the two indicated TCI states, which is undefined in the current specification(s). In a nutshell, how a UE would implicitly determine a BFD RS set q0 from indicated TCI state(s) under the Rel-17/18 unified TCI framework is unspecified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that, a UE would implicitly determine a BFD RS set q0 according to the RS(s) provided in the indicated TCI state(s) under the Rel-17/18 unified TCI framework. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Under the Rel-17/18 unified TCI framework, UE’s behaviors of implicitly determining a BFD RS set q0 are undefined, making the cell-specific BFR procedure incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6 Link recovery procedures

< Unchanged parts are omitted >

If the UE is not provided q0 by failureDetectionResourcesToAddModList for a BWP of the serving cell, the UE determines the set q0 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by one or two TCI-State(s) for respective CORESETs that the UE uses for monitoring PDCCH. If the UE is not provided q0,0 and  q0,1 for a BWP of the serving cell, the UE determines the set q0,0 and  q0,1 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for first and second CORESETs that the UE uses for monitoring PDCCH, respectively, where the UE is provided two coresetPoolIndex values 0 and 1 for the first and second CORESETs, or is not provided coresetPoolIndex value for the first CORESETs and is provided coresetPoolIndex value of 1 for the second CORESETs, respectively. If there are two RS indexes in a TCI state, the set q0 or q0,0, or q0,1 includes RS indexes configured with qcl-Type set to 'typeD' for the corresponding TCI states. If a CORESET that the UE uses for monitoring PDCCH includes two TCI states and the UE is provided sfnSchemePdcch set to 'sfnSchemeA' or 'sfnSchemeB', the set q0 includes RS indexes in the RS sets associated with the two TCI states.

< Unchanged parts are omitted >