3GPP TSG RAN WG1 Meeting #118		R1-2406747

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:       9.2.3

Source: 	Nokia

Title: 	On the support for 3-antenna-port codebook-based transmissions

Document for:     Discussion and Decision

# 1	Introduction

In RAN#102 one objective was agreed for 3Tx UL MIMO enhancements in Rel-19 WID [1]. The specific objective in the WID RP-234007 is:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported.

After the last RAN1 meeting, the Rel-19 Tx features as indicated in WID [1] are largely specified. 3Tx non-coherent codebook is defined, together with 3Tx SRS design, PT-RS association, and related 3Tx operations.

Following RAN plenary decision, these 3Tx related issues are postponed until RAN #105 meeting:

- Partial-coherent UL codebook for 3Tx multi-panel

- Open loop based UL precoder cycling

The other 3Tx related topic, SRS antenna switching to support 3T6R, is agreed to be “to study and specify” in RP-241656 [2]. The RAN1 discussion shall start from RAN1 #118bis (Oct. 2024) meeting.

This contribution is discussing remaining issues related to Rel-19 3Tx feature, on 3Tx related RRC parameters and non-codebook based transmission. The SRS antenna switching issued related 3T6R is also discussed, following FL’s guidance on “offline discussions” of RAN1 #118 meeting.

# 2	RRC parameters for 4-port SRS

The 3-port SRS resource is agreed to reuse the legacy 4-port SRS resource, where the 4th SRS port is muted, based on the agreement in RAN1 #117 [3][4]:

Agreement Update the agreement made in RAN1 #116bis as the following. Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port.

For a UE with 4-port capability, the UE can support up to max 4-port SRS transmission. Usually, this type of UE can be configured to support 3-port uplink transmission. Therefore, 3-port SRS resource can be configured. Based on the 3-port SRS resource design, the legacy 4-port SRS resource would be configured for the UE. When the legacy 4-port SRS resource is configured, the UE won’t be able to differentiate whether the 3-port or the 4-port SRS resource shall be used.  Therefore, one RRC parameter would be useful to differentiate the 3-port SRS resource configuration.

Proposal 1: Define one RRC parameter to differentiate 3-port SRS from 4-port SRS resource configuration.

# 3	Non-codebook based transmission for 3Tx

Rel-15 non-codebook based transmission supports 2Tx and 4Tx NCB-based Tx and Rel-18 extended the NCB operation to 8Tx. When 3Tx is specified in Rel-19, it would be a straightforward to have 3Tx NCB operation, although the Rel-19 WID only indicated to “Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions”.

To enable 3Tx NCB operation, the first step would be the configure an SRS resource set with usage of “non-codebook” for a UE, which is capable of “3Tx NCB”. This step needs two specification supports:

- Support “3Tx non-codebook” as a UE capability

- Enable SRS resource set with usage “non-codebook” to support 3Tx NCB operation

The second step is related to the number of SRS resources with a given maximum rank for NCB. This step is following the legacy Rel-15 NCB for 4Tx, except that the maximum rank for 3Tx should be limited to 3 instead of 4 of Rel-15 4Tx NCB. There is no need to update related 38.212 tables for SRI, where some editorial clarification for 3Tx would be needed.

With these considerations, we have these proposals for 3Tx NCB:

Proposal 2: Support 3Tx non-codebook transmission in Rel-19.

Proposal 3: Support a new UE capability with “3Tx non-codebook transmission”.

Proposal 4: One SRS resource set with usage “non-codebook” can be configured to support 3Tx NCB with maximum number of 3 resources.

Proposal 5: Reuse Rel-15 4Tx NCB SRI tables with limitation of maximum rank of 3 for 3Tx NCB.

# 4	Offline discussions: SRS antenna switch for 3Tx

The last RAN plenary meeting updated Rel-19 RAN4 WID RP-241656. The related SRS antenna switching is indicated below:

RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN #104, June 2024 RP-241656 - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

Following FL’s proposals on 3T6R (as highlighted below), we are providing our view for offline discussions. The “study and specify, if necessary,” work on 3T6R shall start from RAN1 #118bis meeting, following the RAN4 WID RP-241656.

Offline Proposal 3.1

For performing antenna switching for DL CSI acquisition by a 3TX UE,

- Support 3T6R

We shall follow the WID, as indicated as “study and specify, if necessary”. The 3T6R feature can only be supported after the study, which will be started from RAN1 #118bis.

Offline Proposal 3.2

For performing antenna switching for DL CSI acquisition by a 3TX UE, for the case 3T6R,

- Support use of two 3-port SRS resources transmitted in different symbols.

Offline Proposal 3.3

For performing antenna switching for DL CSI acquisition by a 3TX UE, for the case 3T6R,

- Up to two SRS resource sets can be configured.

Given that 3Tx UE might be considered as an evolution from 2Tx UE, what 2Tx UE supports should be extended to 3Tx as well. 2T4R and 2T8R are supported for 2Tx UE. We may consider whether these 2Tx UE features can be applied to 3Tx UE as well. We shall consider whether 3T4R and/or 3T8R shall be supported, together with 3T6R.

For 3T6R, at least there are two possible options:

- Option 1 (3T6R): 2 SRS resources are configured. Each of SRS resources has 4 ports with 1-port muted.

- Option 2 (3T6R): 3 SRS resources are configured. Each of SRS resources have 2 ports.

Offline Proposal 3.4

For performing antenna switching for DL CSI acquisition by a 3TX UE, for the case 3T6R,

- The resourceType for a configured SRS resource set can be one of ‘semi-persistent’, ‘periodic’ and ‘aperiodic’

- FFS supported number of SRS resource sets for each supported resourceType

The SRS resourceType shall follow the legacy operation, which can be configured as one of “semi-persistent”, “periodic”, and “aperiodic”.

# 5	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

Proposal 1: Define one RRC parameter to differentiate 3-port SRS from 4-port SRS resource configuration.

Proposal 2: Support 3Tx non-codebook transmission in Rel-19.

Proposal 3: Support a new UE capability with “3Tx non-codebook transmission”.

Proposal 4: One SRS resource set with usage “non-codebook” can be configured to support 3Tx NCB with maximum number of 3 resources.

Proposal 5: Reuse Rel-15 4Tx NCB SRI tables with limitation of maximum rank of 3 for 3Tx NCB.

# References

- RP-234007, “New WID: NR MIMO Phase 5,” RAN#102, Edinburgh, Scotland, Dec 11-15, 2023

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4,” RAN #104, June 2024

- RAN1 #116bis meeting Chair’s note, Changsha, China, April 15 – 19, 2024

- RAN1 #117 meeting Chair’s note, Fukuoka, Japan, May 2024

# RAN1 Previous Agreements

- RAN1 #116 agreements

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission.

13100,13010,13001

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission.

13100100, 13100001, 13 001001

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission.

13100010001

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, down-select one of

- Alt1 – Support configuration of X 4-port SRS resources in a resource set where one the ports is muted

- Alt2 – Support configuration of X SRS resources with equal/unequal number of ports (e.g. 2 + 1 or 1 + 1 + 1) in a resource set,

The value for X is FFS, and it will be determined according to the selected alternative.

Agreement

For a 3TX UE, down-select one of the following options for the number of PTRS ports,

- Option-1: A single PTRS port is supported.

- Option- 2: Up to 2 PTRS port may be configured.

Agreement

For a 3-antenna-port codebook-based UL transmission, study PTRS-DMRS association.

Agreement

For a 3-antenna-port codebook-based UL transmission, study power split for each port of SRS and PUSCH.

Agreement

For codebook-based uplink transmission by a 3TX UE, support full-power Mode 0, subject to UE capability.

Conclusion

There is no consensus in RAN1 to support antenna switching for 3TX UE in Rel-19

Agreement

For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for LLS evaluation

- Companies may provide additional evaluation results per their case of interest

- LLS is optionally used for 3Tx UL evaluation, if needed

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Waveform | CP-OFDM |
| SCS | 30 KHz |
| System bandwidth | 20 MHz, 100 MHz |
| Scheduled PRBs | 5, 25, 50, 260 PRBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ |
| UE speed | 3 Km/h |
| Number of Layers | Adaptive, Fixed (reported by company) |
| AMC | Adaptive, Fixed (reported by company) |
| DMRS configuration | Type 1; 1 front loaded + 1 additional symbol |
| Channel estimation | Real |
| Channel Model | CDL-A (30ns), CDL-B (100ns), CDL-C (300ns) |

Agreement

For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for SLS evaluation.

- Companies may provide additional evaluation results per their case of interest.

Note: The considered EVM is to be used as a baseline set of assumption for future potential studies related to 3TX.

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 CP-OFDM symbol slot SCS , 30 KHz |
| Scenario | eMBB: Dense Urban (200m), 3.5GHz Outdoor FWA: UMa (500m), 3.5GHz |
| UE Outdoor/Indoor (%) | eMBB: 80%, 20% Outdoor FWA: 100%, 0% |
| System bandwidth | 20 MHz, 100 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Optional: Classical: two 8x1 xpols, 4λ apart; 4 TXRUs tilt=[104°] |
| gNB antenna radiation pattern parameters | - Outdoor/Indoor Per 38.901, Table 7.3-1 |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 64 QAM - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE antenna config | eMBB: - Xpol+1pol; isotropic ULA - Xpol+1pol; 110°, 4 dBi Outdoor FWA: - Xpol+1pol; isotropic ULA - 3 directional 1pol: 110°, 4 dBi |
| Traffic model | - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional) |
| Suggested benchmarking | Rel-15 2Tx non-coherent |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario |
| UE power rating | eMBB: 23 dBm, UL FPTx mode 0 or Rel-15 power scaling Outdooe FWA: 31 dBm, UL FPTx mode 0 |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

Agreement

For performance evaluation of 3TX UE, consider following reference configurations,

- A linear array (1D) of single-polarized antenna configuration with a spacing of 0.5λ,

  - For example:		 |←0.5λ→|←0.5λ→|

- A configuration of a cross-polarized and a single-polarized antennas,

  - For example:		 ×←0.5λ → ⁄

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, one SRS resource set is configured for single TRP operation.

Agreement

For codebook-based transmission by a 3TX UE,

- Only PUSCH antenna ports 1000, 1001, 1002 are used

- Option- 2: Subject to UE capability, up to 2 PTRS ports may be configured in PTRS-UplinkConfig,

  - FFS whether a single bit or 2 bits are used for PTRS-DMRS association indication.

Above is only for single panel transmission.

- RAN1 #116bis agreements

Agreement

To support codebook-based UL transmission by a 3TX UE, the agreed rank1 precoders in RAN1#116 can also be used when transform precoding is enabled (DFT-s-OFDM ).

Agreement

To indicate precoding information for codebook-based UL transmission by a 3TX UE,

- Reuse legacy TPMI indication framework where TPMI and TRI are jointly indicated

- TPMI field is 2 or 3bits for 3-antenna-port transmission

  - For maxRank equals to 1, TPMI field is 2 bits for DFT-s-OFDM and CP-OFDM

  - For maxRank equals to 2 or 3, TPMI field is 3 bits for CP-OFDM

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, support Alt1,

- Alt1: Support configuration of X 4-port SRS resources in a resource set where one the ports is muted

  - FFS muting mechanism

where X can be up to 2, subject to UE capability.

Agreement

- For codebook-based UL transmission by a 3TX UE, when 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows:

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

- Note: PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT_RS port 0, and PUSCH antenna port 1001 is associated with PT_RS port 1

- Number of bits used for the indication

  - 1 bit

Agreement

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource,

- Option 3: Always a same port is muted, e.g., the 4th port

Agreement

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, UE splits a linear SRS power equally across the 3 unmuted antenna ports of the 4-port SRS resource.

Agreement

For 3-port codebook-based PUSCH transmission for a 3TX UE, scale factor s should be the ratio of the number of antenna ports with a non-zero PUSCH transmission power to 3 (except for full-power Mode 0).

- FFS: Whether specification needs to be updated to reflect the above

Agreement

For codebook-based UL transmission by a 3TX UE, when 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows:

- Alt2: 2-bit indication

PTRS-DMRS association when 1 PT-RS port is configured

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port Reserved |

Agreement

For a 3TX UE, support Rel-17 M-TRP PUSCH repetition where,

- Two SRS resource sets, each with up to 2 of 4-port SRS resources are configured,

Note: The configured 4 port SRS resources are used to enable 3-port SRS transmission

- RAN1 #117 agreements

Agreement

Update the agreement made in RAN1 #116bis as the following.

Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port

Agreement

For codebook-based M-TRP PUSCH repetition by a 3TX UE, scheduled by DCI format 0_1/0_2,

- Reuse Rel-17 M-TRP PUSCH repetition design, where the second precoding information field only indicates TPMI index, and applies same rank as indicated by the first precoding information field.

Agreement

For codebook-based M-TRP PUSCH repetition by a 3TX UE, scheduled by DCI format 0_1/0_2,

- Introduce new tables as Table I, II, III for the second precoding information field, for maxRank=1 or 2 or 3, respectively.

  - Table I: Second precoding information for 3 antenna ports if maxRank=1

| Bit field | codebookSubset=NonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | Reserved |

  - Table II: Second precoding information for 3 antenna ports if maxRank=2

| Bit field | codebookSubset=NonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | 1 layer: Reserved |
| 0 | 2 layer: TPMI=0 |
| 1 | 2 layer: TPMI=1 |
| 2 | 2 layer: TPMI=2 |
| 3 | 2 layer: Reserved |

  - Table III: Second precoding information for 3 antenna ports if maxRank=3

| Bit field | codebookSubset=NonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | 1 layer: Reserved |
| 0 | 2 layer: TPMI=0 |
| 1 | 2 layer: TPMI=1 |
| 2 | 2 layer: TPMI=2 |
| 3 | 2 layer: Reserved |
| 0 | 3 layer: TPMI=0 |
| 1~3 | 3 layer: reserved |

Agreement

For codebook-based M-TRP PUSCH repetition by a 3TX UE, for indication of PTRS-DMRS association,

- When 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, reuse Rel-17 multi-TRP TDM repetition,

- When 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, and maxRank = 2 or 3,

  - A second PTRS-DMRS association field (1 bit) is used to indicate the association between PTRS ports and DMRS ports for the 2nd SRS resource set (i.e.,2nd TRP).

Agreement

For codebook-based UL transmission by a 3TX UE, subject to its capability,

- A 3TX UE may report a maximum number of 3 layers

- A 3TX UE may report a maximum number of SRS ports of up to 3

Note: SRS resource definition is not changed nor the number of SRS ports in the SRS resource.