3GPP TSG RAN WG1 #118			R1-2406751

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	9.3.3

Source:	InterDigital, Inc.

Title:	On CLI handling for SBFD operation

Document for:	Discussion and Decision

# Introduction

In Rel-19, a work item on evolution of NR duplex operation for Sub-band full duplex (SBFD) was approved and updated [1], where the objectives identified for the work item for the CLI handling enhancements as follows:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

In this contribution, we discuss enhancements on CLI handling schemes based on the revised WID [1] and the TR38.858 [2] for both UE-to-UE and gNB-to-gNB co-channel CLI handling.

# Discussions

NR supports dynamic/flexible time division duplex (TDD) based on a slot format indicator (SFI) that can be indicated to a group of UEs by a group-common (GC) DCI (format 2_0). In addition, semi-static configurations via tdd-UL-DL-config-common/dedicated can be configured, where the transmission pattern for each slot/symbol can be configured as either of ‘D’ as downlink, ‘U’ as uplink, and ‘F’ as flexible.

Up to NR Rel-18, most practical assumptions for duplexing are half duplex (HD) for both gNB and UE. In Rel-19, enhancements to support full duplex (FD) at least for gNB were proposed and endorsed as the work item, see Figure 1. Moreover, subband non-overlapping FD (SBFD), as illustrated in Figure 2, has been identified as a promising approach, since it offers greatly reduced FD implementation complexity in terms of cancelling self-interference (SI) and mitigating cross-link interference (CLI), at least, at the gNB side.

Figure 1. Illustration on NR TDD framework based on FD-gNB and HD-UEs in a cell

Figure 2. Illustration on SBFD configurations including DL/UL subbands

## L1-based UE-to-UE CLI measurement and reporting

Up to NR Rel-17, the UE-to-UE CLI measurements and reporting are based on long-term measurements and layer 3 filtering. As identified during Rel-18 NR-duplex study [2], Layer1(L1)/L2 based UE-to-UE CLI measurement can be optimized for short term interference measurement and low latency, which results in enhancing the CLI mitigation performance by providing accurate CLI measurement. Hence, the L1/L2 based UE-to-UE CLI measurement and reporting can facilitate gNB adjusting UE scheduling.

### Frequency resources aspects

#### Types of CLI measurements and associated subbands

In RAN1 #116, following agreement was made on UE CLI measurement within the active DL BWP:

Agreement For SBFD aware UEs, CLI measurements is performed within the active DL BWP and the following can be considered: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Method#3: UE measures RSSI within UL subband - Method#4: UE measures RSSI within guard band, if guard band exists Note: If DL subband, UL subband or guard band is outside the active DL BWP, the above methods does not apply. Note: Method#4 does not imply that guard band is explicitly configured.

This agreement is the follow up to the discussions in the study phase that were captured in TR, on different methods for measuring CLI at the UE. Method #1 is based on UE measuring RSSI within DL subband to detect and mitigate inter-subband CLI. Method #2 and Method #3 are based on UE measuring RSRP and RSSI, respectively, caused by at least one aggressor UE within UL subband. Method #4 is based on UE measuring RSSI within guard bands, if guard bands are configured.

In our opinion, the CLI-RSSI measurement in DL subband can be considered as a baseline approach, where measuring CLI-RSSI in DL subband for inter-subband CLI measurement could be used to indicate the UE-to-UE CLI leakage on the adjacent subbands in SBFD. Also, the CLI measurements on UL subband should be considered which can give a direct estimation on the CLI received power or signal strength to identify a particular source of the CLI, e.g., an SRS transmission from an aggressor UE.

Proposal 1. Support all Methods for CLI measurements agreed to be considered in RAN1 #116 (i.e., Methods #1-4)

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- Method#3: UE measures RSSI within UL subband

- Method#4: UE measures RSSI within guard band, if guard band exists

#### Subband and edge-specific measurement and reporting

The inter-subband CLI in SBFD may impose higher interference on subband-edge RBs within the SBFD DL subband and guard-bands, whereas the resources in the middle of the SBFD DL subband may not experience CLI as much. As such, measuring and averaging the CLI over the whole SBFD subband may result in down estimation of the CLI.

Observation 1. Inter-subband UE-to-UE CLI measurement in SBFD DL subbands based on measuring over configured RB resources and averaging may result in down-estimation, as the subband-edge RBs experience higher CLI compared to RBs in the middle of the subband.

In a first proposal, to achieve an accurate inter-subband CLI measurement, the UE can measure a delta parameter that is based on measuring and calculating the difference between a first CLI-RSSI measured from the resources in the edge of the configured RBs in the SBFD DL subband or within in-between configured guard-bands and a second CLI-RSSI measured from the resources located in the middle of the configured RBs in the SBFD DL subband. As such, the difference between the CLI-RSSI measured in the edge or guard-bands and CLI-RSSI measured in the middle can be used as an indication of inter-subband CLI.

Proposal 2. In UE-to-UE CLI-RSSI measurement techniques within active DL BWP, support measuring and reporting delta-CLI-RSSI based on differences in measured CLI-RSSI in subband-edge or guard-bands with measured CLI-RSSI in the middle of the DL subband.

In a second proposal, the UE can perform frequency-selective measurements and reporting of the CLI, e.g., reusing the CSI reporting subband configuration used for CQI/PMI-based CSI reports. This would enable the UE and network distinguishing the presence and frequency location of the CLI in the DL/UL subbands with a finer granularity, and let the network handle the scheduling accordingly. Reusing the framework of subband reporting, the UE may report with a granularity down to 4PRBs with current CSI subband reporting configuration.

The UE may also be configured with triggers for subband-based L1-based CLI reporting that consider CLI thresholds, potentially with different thresholds for different subbands (e.g. for subband-edges) and/or different thresholds between wideband CLI measurements and subband-based CLI measurement and reports. For instance, when the UE measures a CLI over a threshold for one or more subbands, it may report that CLI and subband to the network, e.g., using aperiodic reporting.

The UE may further be able to perform the measurement on the configured resource and determine itself the PRBs where the UE experiences strong CLI, typically at the edge of the subband. The UE may then report to the network the CLI and the set of PRBs that are beyond a threshold, so that the UE is not constrained with a (pre)configured subband reporting that may not correspond to the frequency-selective CLI.

The UE or the network may also, based on the reported resources (reporting subbands or set of PRBs) with CLI over a threshold, avoid using the resources to reduce the level of CLI in the network, e.g., cancelling a transmission or mute the resources with the CLI.

Proposal 3. Support both wideband and subband-based CLI reporting, e.g., reusing the subband-based CSI reporting configuration and framework as a baseline, and study further on the subband-based CLI measurement triggers and reporting aspects.

#### CLI-RSSI measurement/report across downlink subbands

Regarding CLI-RSSI measurement resource configuration, three methods were agreed in study phase to be studied for configuration of frequency resources for CLI-RSSI measurement and reporting. Below is the agreement from RAN1 #113 meeting:

Agreement For UE-to-UE CLI-RSSI measurement/report across downlink subbands, study the following methods: - Method#1: separate CLI-RSSI measurement resources/reports in each DL subband - Note: supported in existing specifications - Method#2: CLI-RSSI measure/report in one DL subband only - Note: supported in existing specifications - Method#3: CLI-RSSI measurement/report based on non-contiguous CLI-RSSI resource across downlink subbands - FFS: report single or separate CLI-RSSI report(s) - FFS: details on determination of non-contiguous CLI-RSSI resource allocation

The agreement addresses CLI-RSSI measurement in SBFD configurations where the DL subbands are non-contiguous and that the DL BWP is divided (into two) due to UL subband configured for SBFD operation. Method #1 proposes to consider separate frequency resources to measure the CLI-RSSI for the separated DL resources. In other words, Method #1 implies to consider the two separated DL subbands as two separate CLI-RSSI measurement occasions where separate configurations for the measurement resources will be provided and that each of the separate resources will be contiguous in respective configured resources. As a consequence, Method #1 also proposes separate reporting of the measured CLI-RSSI. The separate resource configurations in Method #1 seem to be straightforward method for the configuration of the resources that avoids any ambiguity in determining the non-contiguous resources; however, using Method #1 only would unnecessarily increase the configuration overhead as doubled indications and related operations compared with legacy CLI-RSSI configurations.

Observation 2. Considering the configuration of CLI-RSSI resources, separate resource configurations (Method #1) for non-contiguous resources would unnecessarily increase the configuration overhead for at least two times in supporting SBFD operations.

The proposed Method #2 was regarding measuring and reporting CLI-RSSI only in one DL subband out of the potential two separated DL subbands. In our opinion, the idea behind this proposal is the potential symmetrical CLI imposed on the two DL subbands. Such assumption based on symmetrical transmission power in the UL subband cannot be made in general, as the different UL resources may be allocated to different UEs with different transmission power, and different locations. For example, consider a potential aggressor UE with high transmission power that is scheduled closer to a first the DL subbands and obviously farther from the second DL subband. As such, the CLI-RSSI measurement in the first DL subband will be different from the CLI-RSSI measurement in the second DL subband, and definitely non-symmetrical. Therefore, CLI-RSSI measurement in only one of the DL subbands may result in over-estimation or down-estimation of the overall CLI-RSSI.

Observation 3. Considering the configuration of CLI-RSSI resources, measuring and reporting CLI-RSSI in only one DL subband (Method #2) may result in down-estimation or over-estimation of overall CLI-RSSI, in case of non-symmetrical scheduling of UL resources.

In Method #3, the CLI-RSSI measurement is proposed to be on non-contiguous resources across DL subbands. That is the UE receives a single configuration for CLI-RSSI measurement that is over non-contiguous resources across DL subbands. In our opinion, this method allows an efficient way to configure the resources, since the UE is already configured with potential DL subbands, UL subband, and guard-bands (if available). So, the UE can implicitly determine the resources to measure the CLI-RSSI based on the configured DL subbands. In this method, only a single configuration is sufficient which means at least half overhead compared to Method #1.

Also, since the UE measures throughout the DL subbands, it is beneficial for UE to do a frequency-selective CLI measurement such as subband-edge specific measurement and delta CLI measurement/reporting which enables efficient frequency resource managements at the network side. As for the reporting, there could be a single reporting or more than one reporting corresponding to the two separate DL subbands and based on the frequency-selective CLI measurement if configured. Also, the UE can only report the CLI for the DL subband with higher measured CLI-RSSI or report the CLI for the DL subband with lower measured CLI-RSSI as well in differential values with reference to the DL subband with higher CLI.

Observation 4. Considering the configuration of CLI-RSSI resources, measuring and reporting CLI-RSSI in non-contiguous resources across DL subbands (Method #3) allows different configuration of CLI-RSSI measurement such as frequency-selective and subband-edge specific CLI measurements.

Observation 5. Considering the configuration of CLI-RSSI resources, measuring and reporting CLI-RSSI in non-contiguous resources across DL subbands (Method #3) allows flexible CLI-RSSI reporting configurations such as single report, separate report, reporting only the DL subband with higher CLI-RSSI, or reporting differential value for the DL subband with lower CLI.

Proposal 4. Support measuring and reporting CLI-RSSI in non-contiguous resources across DL subbands (Method #3), in order to enable more accurate and flexible CLI measurement and reporting configurations.

### CLI measurements and reporting enhancements

#### (L1-)SRS-RSRP reporting to identify aggressor UEs

In NR, there are several interference measurement resources (IMR) that can be used for measuring and reporting interference quality. For example, NZP CSI-RS and ZP CSI-RS resources as IMR can be used at UE for determining the interference power/strength. More specifically, CLI-RSSI has been specified to be used for measuring CLI level in TDD operations. However, the mentioned interference measurements are an estimate of the aggregation of received power/strength over all sources of noise and interference. In other words, while the measurements over IMR resources may show considerable interference power/strength, it is not clear if CLI is the main reason of such estimation or if it is caused by other sources or channel impairments.

For example, (L1-)CLI-RSSI is estimated over all sources of noise and interference in the configured time/frequency resources. However, the aggressor UEs configuration may change dynamically, i.e., aggressor UEs’ scheduling, location, beam direction, etc. Relying only on long-term interference measurements may result in over-estimation of the CLI.

Alternatively, the CLI estimation based on the agreed L1-SRS-RSRP can be used to identify potential victim UEs and potential aggressor UEs. The reference signals (e.g., SRS) transmitted from potential aggressor UEs can be used at the victim UE to estimate CLI accurately. As such, the UE can respond to dynamic changes in CLI (e.g., due to the dynamic changes in configuration of aggressor UEs) with efficient CLI mitigation techniques and/or report the CLI to the network. The UE, by reporting the measured (L1-)SRS-RSRP CLI, may explicitly or implicitly report the potential aggressor/victim UE.

Furthermore, NW can use the measurements reported from victim UEs based on distinguished aggressor UEs if the role of the victim UE changes to an aggressor UE, and the role of the aggressor UE changes to a victim UE, e.g., due to changes in the direction of the transmissions for both victim and aggressor UE, based on a beam correspondence property between the UEs.

Observation 6. CLI estimation and reporting at a potential victim UE based on distinguishing aggressor UEs can be used for enhancing CLI mitigation at the UE and further optimal scheduling at the gNB.

Proposal 5. Support enhancements to UE-to-UE co-channel CLI measurement based on supporting CLI measurement and reporting at the potential victim UE that includes implicitly or explicitly distinguishing aggressor UEs.

#### Triggers for CLI measurements and reporting

In general, DL/UL resource assignment for a UE can be based on CSI/beam measurement and reporting procedures supported in NR. Based on the CSI/beam measurements performed on a configured DL RS (e.g., CSI-RS) resource, the UE can report one or more preferred beam/RS indexes along with corresponding quality metrics such as CQI, L1-RSRP, or L1-SINR. The reported contents can be subband-wise, depending on gNB’s configuration, and those can be used for gNB’s scheduling on the DL/UL resource assignment for the UE. The beam information for the DL/UL resource assignment can be based on an indicated transmission configuration indicator (TCI) for a DL/UL channel/signal. When the UE receives a grant (DCI), the UE can apply a spatial filter determined by the indicated beam information to perform the DL reception or the UL transmission. These general procedures are not based on any dynamic information related to CLI (e.g., UE-to-UE CLI) as the UE’s measurement and reporting is based on a configured RS resource which is relevant only with the gNB.

A CLI due to a signal transmitted by other (aggressor) UE can be present and severely degrade a reception performance of a DL signal scheduled by a grant at a victim UE, where the presence of the interference may not be known prior to the grant, e.g., in case when the aggressor UE is associated with a different serving-cell/TRP. Due to the CLI which is not captured in the general CSI/beam reporting, the victim UE may fail to receive the DL signal, which degrades the DL performance. The failure in receiving the DL signal can continue to happen, in case when such unexpected UE-to-UE CLIs exist for a duration of time.

Since such a UE-to-UE CLI can happen unexpectedly, it is beneficial to consider a conditional CLI handling behaviour based on monitoring the beams at the (victim) UE side, where the monitoring can be conducted per subband which can be configured to the victim UE as performing subband-wise CLI measurement and reporting. Then, the victim UE can report the subband-wise CLI monitoring results, which can be used at the gNB to determine whether the assigned DL/UL resource to the victim UE should be changed to a different subband to avoid the CLI on a subband. The conditional behavior on CLI monitoring done at the victim UE could greatly simplify the network and UE implementation for CLI handling, as this procedure is transparent to the aggressor UE side. The condition can at least include a case when the victim UE detects a PDSCH reception failure, e.g., along with NACK transmission in general.

Observation 7. Since a general CSI/beam reporting in NR is not based on dynamic CLI-related information, a victim UE may unpredictably experience DL performance degradation if a UE-to-UE CLI occurs especially when an aggressor UE is served by a different serving gNB/TRP.

Proposal 6. Support a conditional CLI handling behaviour based on monitoring the beams at the victim UE side, where the condition can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI.

#### Beam aspects for CLI-based measurement and reporting

In our understanding, the current behaviour for the UE when measuring CLI is to use the RX beam corresponding to the ongoing transmissions. However, this restricts the measurements in a single direction to evaluate the CLI and do not allow the UE to monitor different options, while the CLI may have a strong directional selectivity due to the nature of the interference.

In beam selection, the UE selects the best beam for Rx/Tx from gNB based on measured RSRP, SINR, etc. In SBFD systems with potential directional CLI, the beam management techniques need further enhancement. That is, a beam that was selected as the best beam direction between a victim UE and the gNB may not be the preferred beam in case a directional CLI is received at the victim UE from an aggressor UE, see Figure 3. As such, a beam that was selected by UE to be the best beam, for example based on wideband RSRP measurements, may not be the best beam in one or more of the subbands due to potential CLI in the corresponding subbands.

In NR, joint beam management has proven to be an effective procedure for finding the best beam pairs between the UE and gNB. For joint beam management in SBFD systems, two main aspects can be considered.  One aspect would be to detect and identify the aggressor UEs and the beam directions from the aggressor UEs that are causing CLI. As such, instead of blind CLI measurement and reporting, the victim UE could report the unfavorable beam-pairs for which the aggressor UEs are causing CLI. In this way, the gNB could prevent the CLI between the victim and aggressor UEs by spatial coordination methods. That is, the gNB could only avoid the beams at the aggressor UEs that are causing the CLI for the victim UE. In other words, if a potential aggressor UE is not actually causing CLI to a victim UE, the gNB could know that based on the report received from the victim UE, and the gNB could allow the non-interfering potential aggressor UE to continue its operation. Similarly, for an actual aggressor UE that is causing CLI to the victim UE, the gNB could still schedule the aggressor UE but just avoiding the beam directions that are causing the CLI to the victim UE. This could result in more enhanced and flexible scheduling and resource management at the gNB-side despite the CLI.

Another aspect is to enable the victim UE to measure, detect, and report the optimal beam direction for the UE-to-gNB communication, based on the direction of the beams at the aggressor UE. In contrast to UE-gNB beam selection that is based on highest RSRP or SINR, the beam selection between the victim UE-aggressor UE can be based on the lowest measured SRS-RSRP or CLI-RSSI. This can be achieved by performing a two-step beam sweeping procedure as part of spatial coordination procedure. As such, the first beam sweeping procedure could be between the victim UE and the aggressor UE, and the second beam sweeping procedure could be between the victim UE and the gNB.

Figure 3. Directional CLI from aggressor UE in UL received at the victim UE in DL

The proposed aspects can be summarized in below two steps:

Step 1. Best beam selection based on wideband measurements via beam sweeping/selection between victim UE and the gNB. As the first step, the victim and the gNB can perform a beam sweeping procedure to find the best beam to Rx/Tx from/to gNB. The beam selection between the UE and gNB can be based on wideband RSRP measurements.

Step 2: Best beam selection based on subband-wise measurements via beam sweeping between victim and aggressor UEs: As the second step, the victim and the aggressor UEs can perform a beam sweeping procedure to find the best and the worst beam pairs, where the pairs include pairing of {UE-gNB beam index, SRI from aggressor UE}. The SRI may be indicated to the UE as part of SRS measurement configurations. This may be called an opportunistic beam pairing, as the victim UE finds its best beam to communicate with the gNB based on the beams causing the least CLI from the aggressor UE. As the result of this step, the gNB can choose the best beam to communicate with the UE based on the reported best beam pair, even in the existence of the directional CLI. On the other hand, by reporting the results of this step to the gNB, the gNB can mitigate the CLI by preventing the victim UE to use the UE-gNB beam index and preventing the aggressor UEs to use the worst or unfavourable beams QCL-ed with the indicated SRIs in the reported worst beam pairs causing the highest CLI. The beam pairs determination and reporting may be based on subband-wise CLI measurements to include the CLI measurements in full-duplex operations.

To enable the CLI beam sweeping measurements and reporting, a UE may be configured by the network with a set of SRS resources (that will be transmitted by another UE) and be configured to report one or more measurement results from the different SRS resources. For example, the UE may be configured to report one or more of the (L1-)SRS-RSRP corresponding to the highest or lowest levels of CLI.

The UE may also be configured with one or more RX beams to receive the different resources within a SRS resource set or to receive different SRS resources sets.

Observation 8. Transmitting and monitoring other UE’s SRS can help UE in the following aspects to be considered:

- Preventive aspects, that is determining the victim and aggressor UEs beam pairs to be avoided.

- Beam pairing aspects, that is determining the gNB and victim UE beam pairs to be used based on directional CLI from the aggressor UEs.

Proposal 7. Support UE configuration where the UE is indicated one or more RX beam(s) for some subband-wise CLI resources (e.g., L1-SRS-RSRP), e.g., by indication of a CSI resource indicator (CRI) from the network.

Proposal 8. Support UE configurations to enable subband-wise CLI-based beam-sweeping, where the UE may determine and report the most and least favourable beam(s) or beam pairing(s) between the victim and aggressor UEs.

Proposal 9. Support the UE determination of subband-wise beam pairing between (victim) UE and gNB based on directional subband-wise CLI measurements and reports.

##### Further enhancements for scheduling based on directional CLI reporting

Based on the measurement and reporting of (L1-)SRS-RSRP helping to identify aggressor and victim UEs, an aggressor UE could determine the UL beam directions that could cause CLI on other (nearby) UEs. In cases where an aggressor UE is configured with UL transmission based on a configured grant, one way to handle the CLI is to restrict the aggressor UE to not transmit the configured UL on the UL beam directions that may cause interference. However, the restricted UL beam direction may be the best beam direction detected by the gNB or UE for UL transmission from the aggressor UE. As such, the beam restriction could result in lower performance in UL transmission as the aggressor UE now has to transmit UL based on a suboptimal UL beam direction.

Moreover, the victim UEs that are impacted by UL from the aggressor UE in the restricted UL beam direction may not always be configured with DL reception in the same time instance as the aggressor UE is configured for UL transmission. In other words, restricting the UL beam directions may not even be required if the victim UEs are not scheduled to receive DL.

Observation 9. Restricting one or more UL beam directions at the aggressor UE due to causing CLI on victim UEs, throughout all occasions of a configured UL transmission, could degrade the UL performance, as the aggressor UE may be restricted to transmit based on suboptimal UL beam directions.

Observation 10. Restricting one or more UL beam directions at the aggressor UE due to causing CLI on victim UEs, throughout all occasions of a configured UL transmission, could be unnecessary as the respective victim UEs may not be scheduled for DL reception in all corresponding configured UL occasions.

Proposal 10. Support methods to restrict UL beam directions for a configured UL transmission at an aggressive UE based on scheduled victim UEs, that is only for the occasions that a respective victim UE is scheduled for DL reception.

The aggressor UE can be configured with a candidate UL beam direction, based on the L1-SRS-RSRP report,  that is different from the active/best UL beam direction, where the aggressor UE could select to use the candidate UL beam in case a victim UE is scheduled for DL reception. That is, the aggressor UE can select to use the active/best UL beam direction for UL transmission in case none of the victim UEs are scheduled.

The aggressor UE can receive configurations on the victim UEs that are scheduled for DL receptions that coincide with the configured UL transmission occasions. The aggressor UE can receive the indications on the scheduled DL transmissions from gNB based on a list of corresponding DL reference signals or TCI-states, where the DL beam directions correspond to or are QCL-ed with the indicated reference signals or TCI-states. Alternatively, the aggressor UE can receive the indications on the scheduled DL transmissions from gNB based on a list of corresponding SRIs, where the indicated SRIs imply the victim UEs that are scheduled for DL reception.

Moreover, a potential aggressor UE (UE #1 in Figure 4) that has measured UE-to-UE CLI with a potential victim UE (e.g., UE#2 in Figure 4) can determine that if the victim UE is configured for DL reception in the same symbol that the aggressor UE is configured for UL transmission, this could result in CLI caused for the victim UE. In such case scenarios, the aggressor UE might be configured to drop the UL transmission. Alternatively, the aggressor UE could be configured with a second candidate UL beam direction (green beam in Figure 4) to be used in such cases where UL transmission in the first UL beam direction (blue beam in Figure 4) could cause CLI on the victim UE.

On the other hand, if UE#2 is not configured for DL, the UL transmission from UE#1 could be accomplished based on the first UL beam direction (blue beam in Figure 4).

Figure 4. UL beam configuration in aggressor UE

Observation 11. An aggressor UE can be configured with a first UL beam direction and a second candidate UL beam direction, where the first UL beam direction can be used in case no CLI is caused, and the second candidate UL beam direction can be used in case CLI is caused.

Proposal 11. Support CLI mitigation techniques based on configuring a second candidate UL beam direction at the aggressor UE to be used in case the UL transmission based on the first UL beam direction could cause CLI to other UEs.

#### Performance analysis on L1-based CLI measurement and reporting

We present SLS results investigating impact of UE-to-UE CLI measurement and reporting. For these simulations we focus on an outdoor UMa scenario that utilizes a clustered dropping of UEs, resulting in high UE-UE CLI. The simulation assumptions are based on those provided in section 7 of [3]. We considered three different SBFD schemes:

- Scheme 1 - baseline SBFD (legacy without L1 based CLI enhancement)

- Scheme 2 (L1-based CLI handling without CSI enhancement, with genie aided scheduling) – under this scheme UE reports L1-SRS-RSRP/L1-CLI-RSSI to the gNB, based on Alt.1 of the agreement,

- Scheme 3 (L1-based CLI handling with CSI enhancement utilizing a SINR-type metric) based on Alt.3 of the agreement. Scheme 3 captures the CLI in existing CSI report configuration/framework, allowing for use of CLI resources for IMR, which can then be captured in CQI, PMI, RI calculation – i.e., measurements on CLI-IMR are included in the interference measurement terms for CSI report. Additional details for this scheme can be found in [4].

The CLI measurement for schemes 2 and 3 is available at the gNB, allowing the gNB to utilize an interference aware scheduling scheme where uplink and downlink scheduling for SBFD subbands is done based on minimizing impact of UE-to-UE CLI.

The results we present here are an updated version of the results we presented previously [5]. The results presented in [5] were based on wideband CSI reports for all schemes, whereas here, we utilize subband reporting for CSI reports, allowing for L1-based CLI handling with CSI enhancement (Scheme 3) to capture CLI in subband CSI reports (CQI,PMI).

From Table 1 we observe that at both low and medium load, both schemes 2 and 3 significantly outperform scheme 1. This is a result of coupling L1-based CLI with subband CSI reports for both of these schemes, allowing for more accurate subband CSI reports which enhance SBFD performance when compared to scheme 1 which has no L1 based CLI enhancement for the subband CSI reports. We observe that accounting for CLI in CSI report (scheme 3) doesn’t provide noticeable improvement in DL performance over scheme 2 at low load but shows 30% improvement over SBFD scheme 1. However, at increased load, use of this enhanced feedback does help further mitigate DL losses (over scheme 2) – utilizing implicit CLI information in the subband CSI report yields 3 % improvement over SBFD scheme 2 and 55 % improvement over SBFD scheme 1. This is a result of the feedback accounting for impact of (UE-to-UE) CLI on DL UE performance leading to improving link adaptation, which provides the gNB with a more accurate estimate of UEs channel conditions, and the fact that as load increases the probability of CLI also increases, allowing the system to reap the benefit of this new information.

Table 1: Performance of CLI measurement and reporting schemes

| Reported Parameters | Low load |  |  |  | Medium load |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TDD | SBFD |  |  | TDD | SBFD |  |  |
|  |  | Scheme 1 | Scheme 2 | Scheme 3 |  | Scheme 1 | Scheme 2 | Scheme 3 |
| DL UPT [Mbps] | 428.665 | 304.35 (-29.00%) | 393.794 (-8.14%) | 397.05 (-7.37%) | 378.392 | 216.592 (-42.76%) | 325.321 (-14.02%) | 335.388 (-11.42%) |
| UL UPT [Mbps] | 78.173 | 78.290 (+0.14%) | 83.554 (+6.88%) | 86.136 (+10.18%) | 74.416 | 76.171 (+2.36%) | 77.165 (+3.69%) | 83.442 (+12.13%) |

Observation 12: Use of CLI measurement and reporting schemes in conjunction with CSI subband reporting can help the gNB schedule downlink and uplink UEs to reduce the effects of UE-to-UE CLI on downlink UE performance.

Proposal 12. Support subband reporting of CLI measurement (L1-CLI-RSSI, L1-SRS-RSRP) in addition to wideband reporting, based on gNB’s configuration on the granularity of reporting subbands.

## UL resource muting for PUSCH

The updated WID focuses the gNB-to-gNB CLI handling on UL resource muting for PUSCH, to improve how gNB may perform measurements with reduced interference from other UL transmissions or DL CLI. In RAN1#116bis and RAN1#117 meeting, the following agreements were also reached:

Agreement If non-transparent UL resource muting is supported for interference covariance matrix measurement for gNB-to-gNB CLI handling, the following are recommended to be specified - Definition and indication of UL resource muting pattern - Collision with DMRS/PTRS - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination - Power allocation in symbols with muted REs considering potential impact to phase continuity - TB size determination Note: The existing reference signal time-frequency resource pattern, e.g., PT-RS, comb-2 SRS, are the candidates for the UL resource muting pattern. Note: Consider pattern without adverse impact on PAPR Note: The potential impact on transmit signal quality/MPR requirement may need to checked with RAN4. Note: The above does not apply for PUSCH transmission during random access procedures. Agreement If non-transparent UL resource muting is supported for gNB-to-gNB CLI measurement for gNB-to-gNB CLI handling, the following are recommended to be specified - Definition and indication of UL resource muting pattern - Collision with DMRS/PTRS - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination - Power allocation in symbols with muted REs considering potential impact to phase continuity - TB size determination - Exchange of information across gNBs on measurement resources Note: The existing reference signal time-frequency resource pattern, e.g., CSI-RS, are used to determine the UL resource muting pattern. Note: Consider pattern without adverse impact on PAPR Note: The potential impact on transmit signal quality/MPR requirement may need to checked with RAN4. Note: The above does not apply for PUSCH transmission during random access procedures. Agreement If non-transparent UL resource muting is supported for gNB-to-gNB CLI handling, Option 1 is recommended to be specified - Option 1: Comb-2 for both DFT-S-OFDM and CP-OFDM - Note: Additional details can be discussed within RAN1#117. - Note: Power boosting is assumed for REs in the symbol with UL resource muting. FFS: Details. - Above is subject to separate UE capability (including separate capabilities for DFT-S-OFDM and CP-OFDM).

The UL resource muting for PUSCH is considered to enhance measurements accuracy for the network. Two use-cases benefit from the UL resource muting, i.e., the gNB measurement of the gNB-to-gNB CLI, and the gNB measurements of interference covariance matrix.

It has been agreed that the UE resource muting would be based on a comb-2 muting, e.g., to avoid strong PAPR changes, and that no new DCI/MAC indication should be introduced for the determination/indication of the muted resources. It has been pointed out that even if the RE muting may be over 2 symbols in the time domain, if one of these symbols is placed at the end of a slot, the channel estimation and proper MMSE received configuration may be delayed to be ready, increasing overall delays and implementation constraints.

Therefore, and to minimize the standardization impact, one solution is to reuse existing RS patterns that indicate the time/frequency positions of the REs and to configure the UE not to transmit on these RS or RS-based patterns. The UE may perform rate-matching over the remaining resources and apply transmit power boosting on the RE that are not muted in a symbol including muted REs. E.g., a +3dB on the remaining REs in the same symbol to compensate for the muted REs, so that the different symbols of a same PUSCH would have the same overall transmit power.

PT-RS and comb-2 SRS are proposed as candidates for the resource muting patterns. Introducing a ZP-SRS configuration is an effective solution, where the UE may be configured with reusing existing SRS patterns and configurations that supports comb-2. The SRS resources and ZP may be indicated using RRC configuration, and is very flexible, enabling the network to control the time and frequency positions of the muted resources (e.g. to allow which symbol(s) in time to perform measurement – adjacent or not – based on PDCCH transmission parameters and/or the need for early channel estimation for processing), while being possibly (de)activated or triggered reusing existing DCI/MAC indications. Other existing RS patterns (e.g., PT-RS) may also be considered if marginal specification impact is expected.

The UE may perform RE muting in a slot or symbol which may be configured to be an SBFD slot/symbol and/or for specifically configured sub-bands. The RE muting based on reference signals may be used in the slots/symbols in which the UE is scheduled to transmit PUSCH and the associated DCI (granting UL resources) indicates to use RE muting or indicates the RS trigger. Then, the RE muting (i.e. RE rate-matching) may be used for PUSCH if a PUSCH RE collides with a configured RS.

Proposal 13: Support RS-based pattern indication of the RE to be muted, e.g., based on a ZP comb-2 SRS configuration or other existing RSs with marginal specification impacts.

# Conclusion

In this contribution, we discussed issues on CLI mitigation aspects in SBFD systems. From the discussions, we made following observations and proposals:

2.1 L1-based UE-to-UE CLI measurement and reporting

2.1.1 Frequency resources aspects

Proposal 1. Support all Methods for CLI measurements agreed to be considered in RAN1 #116 (i.e., Methods #1-4)

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- Method#3: UE measures RSSI within UL subband

- Method#4: UE measures RSSI within guard band, if guard band exists

Observation 1. Inter-subband UE-to-UE CLI measurement in SBFD DL subbands based on measuring over configured RB resources and averaging may result in down-estimation, as the subband-edge RBs experience higher CLI compared to RBs in the middle of the subband.

Proposal 2. In UE-to-UE CLI-RSSI measurement techniques within active DL BWP, support measuring and reporting delta-CLI-RSSI based on differences in measured CLI-RSSI in subband-edge or guard-bands with measured CLI-RSSI in the middle of the DL subband.

Proposal 3. Support both wideband and subband-based CLI reporting, e.g., reusing the subband-based CSI reporting configuration and framework as a baseline, and study further on the subband-based CLI measurement triggers and reporting aspects.

Observation 2. Considering the configuration of CLI-RSSI resources, separate resource configurations (Method #1) for non-contiguous resources would unnecessarily increase the configuration overhead for at least two times in supporting SBFD operations.

Observation 3. Considering the configuration of CLI-RSSI resources, measuring and reporting CLI-RSSI in only one DL subband (Method #2) may result in down-estimation or over-estimation of overall CLI-RSSI, in case of non-symmetrical scheduling of UL resources.

Observation 4. Considering the configuration of CLI-RSSI resources, measuring and reporting CLI-RSSI in non-contiguous resources across DL subbands (Method #3) allows different configuration of CLI-RSSI measurement such as frequency-selective and subband-edge specific CLI measurements.

Observation 5. Considering the configuration of CLI-RSSI resources, measuring and reporting CLI-RSSI in non-contiguous resources across DL subbands (Method #3) allows flexible CLI-RSSI reporting configurations such as single report, separate report, reporting only the DL subband with higher CLI-RSSI, or reporting differential value for the DL subband with lower CLI.

Proposal 4. Support measuring and reporting CLI-RSSI in non-contiguous resources across DL subbands (Method #3), in order to enable more accurate and flexible CLI measurement and reporting configurations.

2.1.2	CLI measurements and reporting enhancements

Observation 6. CLI estimation and reporting at a potential victim UE based on distinguishing aggressor UEs can be used for enhancing CLI mitigation at the UE and further optimal scheduling at the gNB.

Proposal 5. Support enhancements to UE-to-UE co-channel CLI measurement based on supporting CLI measurement and reporting at the potential victim UE that includes implicitly or explicitly distinguishing aggressor UEs.

Observation 7. Since a general CSI/beam reporting in NR is not based on dynamic CLI-related information, a victim UE may unpredictably experience DL performance degradation if a UE-to-UE CLI occurs especially when an aggressor UE is served by a different serving gNB/TRP.

Proposal 6. Support a conditional CLI handling behaviour based on monitoring the beams at the victim UE side, where the condition can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI.

Observation 8. Transmitting and monitoring other UE’s SRS can help UE in the following aspects to be considered:

- Preventive aspects, that is determining the victim and aggressor UEs beam pairs to be avoided.

- Beam pairing aspects, that is determining the gNB and victim UE beam pairs to be used based on directional CLI from the aggressor UEs.

Proposal 7. Support UE configuration where the UE is indicated one or more RX beam(s) for some subband-wise CLI resources (e.g., L1-SRS-RSRP), e.g., by indication of a CSI resource indicator (CRI) from the network.

Proposal 8. Support UE configurations to enable subband-wise CLI-based beam-sweeping, where the UE may determine and report the most and least favourable beam(s) or beam pairing(s) between the victim and aggressor UEs.

Proposal 9. Support the UE determination of subband-wise beam pairing between (victim) UE and gNB based on directional subband-wise CLI measurements and reports.

Observation 9. Restricting one or more UL beam directions at the aggressor UE due to causing CLI on victim UEs, throughout all occasions of a configured UL transmission, could degrade the UL performance, as the aggressor UE may be restricted to transmit based on suboptimal UL beam directions.

Observation 10. Restricting one or more UL beam directions at the aggressor UE due to causing CLI on victim UEs, throughout all occasions of a configured UL transmission, could be unnecessary as the respective victim UEs may not be scheduled for DL reception in all corresponding configured UL occasions.

Proposal 10. Support methods to restrict UL beam directions for a configured UL transmission at an aggressive UE based on scheduled victim UEs, that is only for the occasions that a respective victim UE is scheduled for DL reception.

Observation 11. An aggressor UE can be configured with a first UL beam direction and a second candidate UL beam direction, where the first UL beam direction can be used in case no CLI is caused, and the second candidate UL beam direction can be used in case CLI is caused.

Proposal 11. Support CLI mitigation techniques based on configuring a second candidate UL beam direction at the aggressor UE to be used in case the UL transmission based on the first UL beam direction could cause CLI to other UEs.

Observation 12: Use of CLI measurement and reporting schemes in conjunction with CSI subband reporting can help the gNB schedule downlink and uplink UEs to reduce the effects of UE-to-UE CLI on downlink UE performance.

Proposal 12. Support subband reporting of CLI measurement (L1-CLI-RSSI, L1-SRS-RSRP) in addition to wideband reporting, based on gNB’s configuration on the granularity of reporting subbands.

2.2 UL resource muting for PUSCH

Proposal 13: Support RS-based pattern indication of the RE to be muted, e.g., based on a ZP comb-2 SRS configuration or other existing RSs with marginal specification impacts.

# References

RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei.

TR 38.858, “Study on Evolution of NR Duplex Operation”, V18.0.0.

R1-2212993, “Final Summary on evaluation on NR duplex evolution”, Moderator (CMCC).

R1-2305336, “On potential enhancements on dynamic/flexible TDD”, Qualcomm Incorporated.

R1-2404049, “On CLI handling for SBFD operations”, InterDigital, Inc.

# Appendix – Simulation assumptions

Table 2. SLS simulation assumptions, based on section 7 of [3]

| Parameter | Deployment Scenario |
| --- | --- |
|  | Urban Macro (UMa) (from 38.913) |
| Layout | 21 cells with wraparound ISD: 500m |
| Channel Model | UMa (38.901) |
| UE Distribution | 80% indoor, 20% outdoor. 20 UEs per cell. UE clustering distribution with 2 UE clusters and 8 UEs per cluster. |
| UE Mobility | 3 Km/hr |
| Carrier frequency | 3.5 GHz |
| System bandwidth | 100 MHz |
| Subcarrier spacing | 30 kHz |
| BS height | 25 m |
| UE height | The UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. 1.5m |
| Open-loop power control | Default: P_0 = -92 dBm, alpha=1.0 |
| BS/UE TX power | BS: 53 dBm, UE: 23 dBm |
| BS antenna configuration | Baseline TDD: (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 4, 2, 1, 1, 4, 4, 0.8, 0.5). Shared Tx/Rx antenna array with 32 TxRUs SBFD: Separate antenna array for Tx/Rx Option 2 (# of antenna elements for SBFD is two times that of TDD): (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 4, 2, 1, 2, 4, 4, 0.8, 0.5). Separate Tx/Rx antenna array – 16 TxRUs for DL, 16 TxRUs for UL. |
| Slot structure | Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] SBFD Alt. 2 - Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. SBFD Subband configuration#1 with {DUD} pattern, one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth. All cells use the same SBFD slot configuration |
| Inter-subband CLI reporting | Scheme 1: L3-based CLI measurement and reporting Scheme 2: L1/L2-based CLI reporting (genie aided) |
| gNB self-interference | 1 dB UL desense |
| CO-site inter sector inter SB CLI | Option 2: 93 dB (spatial isolation), 0 dB digital isolation |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| UE antenna configuration | (M,N,P,Mg,Ng,Mp,Np,dV,dH) = (1, 2, 2, 1, 1, 1, 2, 0.5, 0.5). 4 ports 4 Tx, 2 Rx |
| Channel estimation | Realistic |
| Transmission scheme | 16/32 Tx Type 1 Codebook |
| Scheduler | SU-MIMO (with PF) |
| Target BLER | 10% first transmission BLER |
| HARQ/repetition | 3 HARQ retransmission |
| Metric | DL/UL User Perceived Throughput |
| Traffic model | FTP3 (0.5MB for DL, 0.125 MB for UL) Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) DL:UL traffic ratio 1:1 and 2:1 are considered Load scenario Low load Type-2 RU < 10%) Medium load Type-2 RU 20%-40% |