3GPP TSG RAN WG1 #118                          		   R1-2407071

Maastricht, Netherlands, 19th August – 23th August, 2024

Agenda Item:	9.4.2.3

Source: 	China Unicom

Title: 	Discussion on downlink and uplink channel aspects for A-IoT

Document for: 	Discussion and Decision

1. Introduction

3GPP RAN has approved a new SID to study Ambient IoT (Internet of Things) in NR [1]. The overall objective shall be to work out a harmonized air interface design with minimized differences for Ambient IoT to enable the two types of devices with peak power consumption of about 1 mW and below a few hundred mW. While some initial agreements have been reached regarding the downlink and uplink channel/signal aspects in RAN1#116,  RAN1#116bis and RAN1#117, there are still many details that require further discussion [2-4].

In this contribution, we present our views about downlink channel design about the periodic broadcast for Ambient IoT.

- Discussion

As discussed in RAN1#117, the following agreements related to the R2D and D2R control information were made.

RAN1#117 agreement [4] For R2D, the only physical channel is PRDCH. - PRDCH carries any higher-layer payload - PRDCH carries L1 R2D control information (if defined) - FFS details of device behaviour(s) for receiving PRDCH For D2R - PDRCH carries any higher-layer payload - PDRCH carries L1 D2R control information (if defined) - Note: PDRCH carries the response agreed at RAN1#116

Under the asynchronous system architecture, the Ambient IoT devices cannot maintain the synchronization with the readers all the time. Furthermore, some different features like device capabilities and the device deployment can affect the charging cycle directly. Therefore, it is extremely difficult for each signal to be aligned in time. If a dynamic adjustment process is introduced, it will lead to a communication protocol that involves complicated scenarios and many subcases, which could hinder the progress of our WI. Therefore, in the initial version in R19, it is better to simplify the energy harvest process appropriately by keeping just the charging indication in the R2D channel and setting a fixed duration. For the D2R channel, the  only the essential feedback can be kept avoiding any dynamic adjustments. Simple indicators in R2D and D2R channel can be introduced to simplify the energy harvest without independent signaling process.

Proposal 1: It is suggested to introduce straightforward indicators in the R2D and D2R channels to streamline energy harvesting without dynamic process.

As discussed in RAN1#116bis and RAN1#117, some preliminary agreements on preamble had been made for study as the baseline.

RAN1#116bis agreement [3] For the R2D timing acquisition signal immediately preceding the transmission of a physical channel, study a preamble with at least two parts which includes a start-indicator part and a clock-acquisition part, where the start-indicator part immediately precedes the clock-acquisition part: - Start-indicator part provides the start of the R2D transmission - FFS: Details of start-indicator part - Clock-acquisition part provides at least the chip synchronization of the subsequent physical channel transmission - FFS: Details of clock-acquisition part, e.g. structure, encoding, length, etc. - FFS: Methods to determine chip duration of the subsequent physical channel transmission - FFS: Other functionalities - Note: the preamble is considered not to be part of a physical channel - FFS: other part(s) of the preamble, if any - FFS: whether the above clock acquisition is sufficient for all devices - FFS: how to make the preamble compact RAN1#117 agreement [4] For the start-indicator part of the R2D time acquisition signal, study the two options below: - Option 1: ON/OFF pattern i.e. high/low voltage transmission - Option 2: OFF pattern, i.e. low voltage transmission

For now, the preamble is the essential part in R2D transmission. However, with the increasing number of ambient IoT devices, relying solely on the start-indicator part and clock-acquisition parts of a preamble is not sufficient for efficient random access. It is inefficient to activate all devices simultaneously during random access, and it is not ideal to have the reader distinguish target devices instead of leaving them unresponsive, which would be a waste of spectrum resources. Therefore, it is crucial to integrate the activation function into the preamble and include a device matching function in the control information part on the PR2DCH to enhance the system operational efficiency.

Proposal 2: It is suggested that the activation function should be implemented in the preamble, while the devices matching function can be implemented in control information on the PR2DCH.

- Conclusion

According to the discussion, following proposal is provided:

Proposal 1: It is suggested to introduce straightforward indicators in the R2D and D2R channels to streamline energy harvesting without dynamic process.

Proposal 2: It is suggested that the activation function should be implemented in the preamble, while the devices matching function can be implemented in control information on the PR2DCH.

- Reference

RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#102

R1-2401857, Final feature lead summary on downlink and uplink channel/signal aspects, RAN#116

R1-2403749, Feature lead summary#3 on downlink and uplink channel/signal aspects, RAN#116bis

R1-2404292, FL summary#3 on downlink and uplink channel/signal aspects, RAN#117