3GPP TSG RAN WG1 #119	R1-2409367

Orlando, US, November 18th - 22nd, 2024

Source:	Nokia

Title:	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device

Agenda item:	9.4.2.4

Document for:	Discussion and Decision

# Introduction

At RAN#102, a new study item “Study on solutions for Ambient IoT (Internet of Things) in NR” (FS_Ambient_IoT_solutions) was approved; the SID was most recently revised at RAN#103 [1].The following objective(s) is/are relevant for the present agenda item:

2. Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). RAN1-led: For the Ambient IoT DL and UL: o Frame structure, synchronization and timing, random access o Numerologies, bandwidths, and multiple access o Waveforms and modulations o Channel coding o Downlink channel/signal aspects o Uplink channel/signal aspects o Scheduling and timing relationships o Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR base station. For Topology 2, no difference in physical layer design from Topology 1.

# Discussion

## Previous agreement in RAN1#118bis

Agreement For the observation of the CW characteristics which would need control of the CW node(s), remove the sub-bullet of “FFS other characteristics”, and adopt the following TP for TR 38.769 The following CW waveform characteristics which would need control of the CW node(s) are identified: - When CW is transmitted or not transmitted - Transmission Power - Frequency resources, e.g., frequency location(s) for waveform (e.g., waveform 1, waveform 2) Other CW waveform characteristics which would need control of the CW node(s), if any, can be further identified in a later stage..

RAN1#116 (Athens, Greece, February 26th – March 1st, 2024) Agreement For R19 A-IoT study item, at least single-tone unmodulated sinusoid waveform is a candidate waveform for carrier wave for D2R backscattering. Agreement For R19 A-IoT study item, multi-tone waveforms for carrier wave for D2R backscattering can be studied. Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. - Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum - Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum - Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. - Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum - Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum - Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

RAN1#116bis (Changsha, China, April 15th – 19th, 2024) Agreement For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones - FFS: how large gap is needed between tones Agreement For CW waveform for D2R backscattering, contiguous multi-tone OFDM signal is not studied in R19 SI. Agreement Study at least the following characteristics of unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering: - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation

RAN1#117 (Fukuoka, Japan, May 20th – 24th, 2024) Agreement For the study of characteristics of CW waveforms, the following table is adopted as a template for capturing observations. Note 1: Further row(s) can be added, if other CW waveform characteristic is agreed. CW waveform characteristics; Observations and/or comparisons of single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform for backscattering; … (if any). D2R reception performance. Spectrum utilization of backscattered signal corresponding to the CW waveforms. CW interference suppression at D2R receiver. Relative complexity of CW generation. Note: For two unmodulated single-tones waveform, the two tones are transmitted from the same CW node.. Observation For D2R reception performance, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides [X Y] dB frequency diversity gain in a fading channel, at least depending on the gap between the two tones and the channel’s coherence bandwidth. - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. Observation For CW interference suppression at D2R receiver, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Requires additional complexity if RF interference cancellation is used at least with CW waveform reconstruction. - Note: RF interference cancellation is needed when the received CW interference power exceeds the blocking threshold of the receiver - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. Agreement For multiple unmodulated single-tone transmitted by one CW node, other number of tones (i.e. >2) is deprioritized. - Note: other number of tones (i.e. >2) is studied only when obvious gains are provided. Observation For relative complexity of CW generation - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Leads to higher PAPR of the generated CW, which impacts the implementation of the power amplifier in the CW node. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

RAN1#118 (Maastricht, Netherlands, August 19th – 23rd, 2024) R1-2407312 FL summary#1 on CW waveform characteristics for A-IoT Moderator (Spreadtrum Communications) Observation For D2R reception performance, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides [0, 8] dB or [-0.2, 3] dB frequency diversity gain at 1% or 10% BLER target in a fading channel for a 1Rx reader and a 1Tx CW transmitter, at least depending on the gap between the two tones and the channel’s coherence bandwidth. - In a TDL-A fading channel with 30ns delay spread - For the gap between [75KHz, 900KHz], the frequency diversity gains at 1% BLER target observed by 6 sources are within [0, 1.5] dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are almost 0dB. - For the gap between [1.08MHz, 4.2MHz], the frequency diversity gains at 1% BLER target observed by 6 sources are within [3, 5.8] dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are within [0.4, 2.5] dB. - For the gap between [5MHz, 10MHz], the frequency diversity gains at 1% BLER target observed by 5 sources are within [5, 8] dB, and the frequency diversity gains at 10% BLER target observed by 5 sources are within [1.3, 4] dB. - In a TDL-D fading channel with 30ns delay spread - For 10MHz gap, 1 sources (R1-2405855) observed 0.7 dB@1%BLER and -0.2dB@10%BLER frequency diversity gain. (Note: loss due to the power split in TDL-D) - In a TDL-A fading channel with 150ns delay spread - For the gap is 180Khz, the frequency diversity gains at 1% BLER target observed by 2 sources are within [1, 3] dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within [0, 2.5] dB. - For the gap is 2.16MHz, the frequency diversity gains at 1% BLER target observed by 2 sources are within [7, 8] dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within [2.5, 5.5] dB. - For the gap is 5MHz, the frequency diversity gains at 1% BLER target observed by 2 sources are within [7, 8] dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within [2.5, 3] dB. - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform. Observation For the D2R transmission BW corresponding to the CW waveforms, compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform requires 2 times the frequency domain resources for D2R transmission, if the frequency gap between the two tones is no smaller than the transmission bandwidth of the corresponding D2R transmission. Observation For interference suppression complexity for different CW waveforms at D2R receiver, compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones requires individual cancellation for each of the two tones, e.g. two RF or IF narrow-band bandpass filters. Observation For the gap between two tones to be able to leverage frequency diversity gain, bandwidth and spectrum characteristics of the D2R transmission, and coherence bandwidth, should be taken into account. R1-2407313 FL summary#2 on CW waveform characteristics for A-IoT Moderator (Spreadtrum Communications) R1-2407468 FL summary#3 on CW waveform characteristics for A-IoT Moderator (Spreadtrum Communications) Agreement For the cases that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, capture the following table in the TR. - Further observations (if any) on these CW transmission cases are not precluded CW Transmission case; observations. Case 1-1; - NO need for BS to support full-duplex capability in DL spectrum for scenario ‘A1’ - Spatial isolation is possible for scenario ‘A1’, reducing the received CW interference power at BS side. - Cross-link interference handing for CW at BS side for scenario ‘A1’. - BS needs to support full-duplex capability (including self-interference suppression for CW) in DL spectrum for scenario ‘A2’. - Higher CW transmission power can be assumed in the DL spectrum than that of in the UL spectrum.. Case 1-2; - NO need for BS to support full-duplex capability in UL spectrum for scenario ‘A1’ - Spatial isolation is possible for scenario ‘A1’, reducing the received CW interference power at BS side. - Cross-link interference handing for CW at BS side for scenario ‘A1’. - BS needs to support full-duplex capability (including self-interference suppression for CW) in UL spectrum for scenario ‘A2’. - Lower CW transmission power can be assumed in the UL spectrum than that of in the DL spectrum.. Case 1-4; - NO need for BS to support full-duplex capability in UL spectrum - Spatial isolation is possible, reducing the received CW interference power at BS side. - Cross-link interference handing for CW at BS side. - Estimation of CW interference may be needed for successful D2R reception (i.e., at BS). - Lower CW transmission power can be assumed in the UL spectrum than that of in the DL spectrum.. Agreement For the cases that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, capture the following table in the TR. - Further observations (if any) on these CW transmission cases are not precluded CW Transmission case; Observations. Case 2-2; - NO need for intermediate UE to support full-duplex capability in UL spectrum for scenario ‘A1’ - Spatial isolation is possible for scenario ‘A1’, reducing the received CW interference power at intermediate UE side. - Cross-link interference handling for CW at intermediate UE side for scenario ‘A1’. - Intermediate UE needs to support full-duplex capability (including self-interference suppression for CW) in UL spectrum for scenario ‘A2’. - Lower CW transmission power can be assumed in the UL spectrum than that of in the DL spectrum.. Case 2-3; - NO need for intermediate UE to support full-duplex capability in DL spectrum - Spatial isolation is possible, reducing the received CW interference power at intermediate UE side. - Cross-link interference handling for CW at intermediate UE side. - Estimation of CW interference may be needed for successful D2R reception (i.e., at intermediate UE). - Higher CW transmission power can be assumed in the DL spectrum than that of in the UL spectrum.. Case 2-4; - NO need for intermediate UE to support full-duplex capability in UL spectrum - Spatial isolation is possible, reducing the received CW interference power at intermediate UE side. - Cross-link interference handling for CW at intermediate UE side. - Estimation of CW interference may be needed for successful D2R reception (i.e., at intermediate UE). - Lower CW transmission power can be assumed in the UL spectrum than that of in the DL spectrum.. Observation With respect to the following RAN guidance: - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc. - From RAN1 perspective, the following CW characteristics are identified: - When CW is transmitted or not transmitted - Transmission Power - Frequency resources - FFS other characteristics

The following deployment scenarios and topologies are within the scope of the study SIDError! Reference source not found.:

- D1T1

  - gNB as the carrier wave source

  - Carrier wave node is outside the topology

- D2T2

  - Intermediate UE as the carrier wave source

  - Carrier wave node is outside the topology.

Both topologies can be implemented in a monostatic or bistatic setup. The activator (activating device) and the reader (receiving device) are co-located in a monostatic setup, and it is expected to operate in a full duplex mode, i.e., transmitting and receiving at the same time. To be specific, the reader can transmit a carrier wave, which will be used by the AIoT devices, i.e., device type 1/2a, to perform backscattering their responses on the same frequency as the carrier wave.

In the bistatic setup, the CW node, which transmits the carrier wave, and the reader that receives the backscattered signal from the AIoT device are not co-located and are separate devices, thus relaxing the full duplex capability of the reader/activator device. This will facilitate the generic 3GPP nodes, such as gNB and NR UEs, to take the responsibility of being an activator or a reader to perform AIoT transmission. The main drawback of the bistatic setup is the cross-link interference (CLI) between the activator and the reader, i.e., the carrier wave transmitted by the CW node may interfere with the backscattered signal from the AIoT device at the reader, thus seeks for better interference management between the CW node and the reader.

Alternatively, a simple approach to avoid the CLI between the AIoT response and the CW node is to perform frequency shift by the AIoT device, in particular by device type 2a/2b, wherein the backscattered signal is shifted in frequency by a small offset, thereby enabling some filtering operation to be performed at the reader to avoid the CLI or to minimize the impact of CLI while detecting the AIoT backscattered signal. This is only possible by the more advanced AIoT device type 2a/2b, which has its own power storage charged by other means than the AIoT activating signal alone.

The CW node and the reader may or may not be the same device depending on the use case, i.e., monostatic or bistatic.

## UE as CW node

The generation of CW tone should be coordinated by the NW as the CW tone uses the licensed spectrum for transmission. If a UE is configured to act as a CW node, i.e., case 2-2, then the following options are applicable depending on the RRC state of the UE.

- If the UE is in RRC connected and scheduled for DL, then the UE:

  - finishes the current reception and switches to CW transmission. Thus, it is assumed that the CW signal is defined with lower priority than the DL signal. The UE may be configured by the NW via a DL control channel to switch carriers and initiate a CW transmission.

  - Alternatively, in an FDD scenario, the UE may continue the DL reception while transmitting the CW signal in UL spectrum. The UE may be configured by the NW via a DL control channel to continue the DL reception and start a CW transmission.

- If the UE is RRC connected and scheduled for UL, then the UE may be configured to perform either of the following actions:

  - Finish the currently scheduled UL transmission and to switch carriers to the CW transmission.

  - Alternatively, perform the currently scheduled UL transmission and the CW transmission concurrently depending on the capability of the UE.

- If the UE is in RRC inactive, then the UE may be configured with a CW transmission without transitioning to RRC connected. To this end, the UE may perform an UL SDT transmission carrying the CW signal.

Depending on the RRC state and the capability of the UE participating as CW node for AIoT operation, the scheduled NR traffic can either be performed concurrently or completed before CW transmission, respectively.

## Impact of separation distance on received signal power

The transmitted CW signal (for example a UL signal in a FDD configuration) will also be received at the reader in a bi-static set-up. Power level of that signal can be many orders of magnitude higher than the backscattered response from the AIoT device, especially in line-of-sight channel (LOS) conditions. This is illustrated in Fig. 1 below for a scenario where the AIoT device type 1 is 5 m away from the CW node transmitting the CW signal and in the direct path (worst-case) between the activator and reader.

Fig. 1 Received power levels at the reader of the AIoT device activation signal and the back scattered signal from the AIoT for a LOS bistatic scenario.

It is seen from Fig. 1 that the power level of the CW signal can be up to 50 dB higher than the received emitted response from the AIoT device at the Reader. Such high level of interference can be problematic for common RF receiver architectures and even at baseband level and will have to be addressed within RAN1. The interference level of the activation signal at the Reader will mostly be affected by the available antenna gain for an AIoT device type (i) and/or the potential reflection gain of an AIoT device type (ii). As such, every dB of antenna gain at the AIoT device will in theory reduce the interference level by a factor twice as large as the antenna gain value. However, even a 10 dB antenna gain at the AIoT device (very large device) or a reflection gain of 20 dB, will only reduce the potential interference level to around 30 dB, which is still a very high interference level. It is worth observing that the antenna gain of an AIoT device is expected to be low (less than 3 dBi) as AIoT devices are small in size and thereby also electrical small compared to the frequency of the activation signal. Hence, backscattered signals from an AIoT device type (i) can be expected to be received at the Reader with a high interference level in a bi-static setup.

CW signal can induce very high interference levels (up to 50 dB) at the reader in bi-static setup.

Different mitigation techniques already known within 3GPP could be adopted for interference mitigation for AIoT, like adding a QCL relation between reference signals transmitted by the activator and the CW signal itself, whereby the reader can estimate the channel between itself and the activator. This would enable the reader to cancel out the activation signal at its receiver. A second option is to utilize cross link interference (CLI) measurements at the reader and reported back to the gNB, whereby the gNB can optimize the scheduling of activators and readers for the AIoT session. A third option valid for devices that can transmit with more than one layer, is to characterize the channel in between the CW node and the reader, whereby the reader can determine an optimal pre-coding of the CW signal for interference mitigation when configured for receiving the backscattered response from the AIoT device.

RAN1 to study if mitigation procedures/techniques based on existing tools in RAN1 specification, e.g., QCL relationships between reference signals, the cross-link interference (CLI) framework or pre-coding of the CW signal, can be used for mitigating the AIoT activation signal interference at the reader in bi-static setup.

## Carrier Wave Design

Within the frequency response range of the components, the AIoT device can be activated by a carrier-wave with any waveform and backscatter (reflect) a modulated signal, whether the activation signal is single-tone carrier waveform or multiple tones that may either contiguous or discrete. The original activation signal may be already modulated or may be a constant amplitude signal. It is always possible to modulate the D2R data using at least the simplest method of OOK on top of, for example, the OFDM signal on which user data are modulated.

RAN1 to identify the aspects and metrics that should drive the selection of the carrier-wave signal type and study the impact that a certain waveform selection has on the main L1 aspects considered in this SI.

Furthermore, there is a clear difference with respect to how single-tone carrier waveform and multiple-tone carrier waveform should be supported, and we need to treat the design of each separately. For example, for the former, a new narrowband single-tone activation signal would be needed. For the latter, we can reuse the NR/LTE signals as the activation signal. A very intuitive example is to use UE's uplink transmission signal as activation for the AIoT device. This would also motivate the support of coexistence of Ambient IoT and NR/LTE, since time-frequency resources used by NR/LTE physical channels and signals would be reused in this case.

Furthermore, since the AIoT device may send data with variable payload, a variable length CW signal may be considered.

To enable multiple AIoT devices to reply with their different payload size data, the CW may be configured to have a fixed duration and a variable repetition size.

Finally, it should be noted that in case the activation signal serves also as source of energy, i.e., it is used for RF energy harvesting, then the envelope and spectrum would strongly impact the AC-to-DC conversion efficiency, which in turn affect the charging time of the device and its (albeit rudimentary) capabilities. This is particularly relevant for devices which do not have any energy storage, such as device type 1. For instance, it is well-known that multi-carrier/tone waveforms who display high PAPR are better suited for the RF energy harvester, which in turn could charge the device more quickly. This can have a significant impact on aspects such as the delay at which a device can be become operative, the number of active AIoT devices in a given area, the interference measured at the reader, the capability of the AIoT device to satisfy certain timing/synchronization requirements and so on.

Existing NR/LTE OFDM waveforms must be used as baseline for study, where considering new waveforms, if adequately justified by practical evidence, is also not precluded.

Agreement For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones - FFS: how large gap is needed between tones

The spacing between multiple tones is determined the data rate of the underlying AIoT transmission and available BW of AIoT transmission. The larger gap narrower will be the time domain pulse and the number of repetitions will be more. Alternatively, as the gap between two tones gets smaller, pulse will be wider and fewer, following the basic FFT properties of up sampling and folding in the dual domain. As the number of tones increases, PAPR will have an impact on the time domain signal, which may benefit the device harvesting.

## CW Characteristics

In the previous RAN1#117 meeting, following agreement has been made to study the characteristics of carrier wave used for AIoT transmission.

Agreement For the study of characteristics of CW waveforms, the following table is adopted as a template for capturing observations. Note 1: Further row(s) can be added, if another CW waveform characteristic is agreed. CW waveform characteristics; Observations and/or comparisons of single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveforms for backscattering; … (if any). D2R reception performance. Spectrum utilization of backscattered signal corresponding to the CW waveforms. CW interference suppression at D2R receiver. Relative complexity of CW generation. Note: For two unmodulated single-tones waveforms, the two tones are transmitted from the same CW node..

In the existing RFID specification, only a single carrier wave is used for communication wherein a single carrier wave, which is a single sinusoid, is generated by the reader, which is then backscattered by the RFID tag, thus enabling the reader to receive a portion of energy to detect the tag content. Though this is a short-range communication, the presence of multiple reflectors, render the channel frequency selective and thus suffer from fading effects. To over this issue, in AIoT, two-tone carrier wave is proposed by majority of companies to improve both coverage and robustness against fading impairments. However, this comes with a price in terms of transmit power and interference cancellation capability at the reader.

### Single tone CW

| CW waveform characteristics | Single tone | … (if any) |
| --- | --- | --- |
| D2R reception performance | Under the mercy of fading. If more scatterers are present between the reader and the device, communication cannot be feasible. |  |
| Spectrum utilization of backscattered signal corresponding to the CW waveforms | Depending on the BW/rate of backscattered transmission. |  |
| CW interference suppression at D2R receiver | Simple, since the suppression can be done by either using a notch filter or via digital processing to supress a single tone. |  |
| Relative complexity of CW generation | Generation is simple for a reader, which can also be a NR UE using the existing HW. |  |

### Two tone CW (transmitted continuously for entire AIoT session)

In this option, two independent CW tones are generated by the same reader, which can be used for backscattering purposes. To ensure fair comparison, if the total power remains constant, then the power used by two discrete tones will be halved to retain the total transmit power constraint. Even though having two discrete tones provides robustness against the frequency selective fading if the tones are separated by at least the coherent BW, the coverage will be severely limited by the power reduction.

Due to the presence of two discrete tones that are separated by the coherence BW or AIoT transmission BW (whichever is larger), CW interference suppression may pose a challenge at the reader. Though it can be handled efficiently by the digital processing techniques, the complexity is significantly larger than the single tone CW counterpart. Thus, the benefit of having two CW tone used always may not provide all the available benefits.

| CW waveform characteristics | Two tones (always transmitted) | … (if any) |
| --- | --- | --- |
| D2R reception performance | Robustness against fading. However, if the total transmitted power is fixed, coverage may be sacrificed due to power distribution among the CW tones transmitted. |  |
| Spectrum utilization of backscattered signal corresponding to the CW waveforms | Depending on the BW/rate of backscattered transmission together with sufficient guard BW to avoid overlapped backscattered transmission. |  |
| CW interference suppression at D2R receiver | Complex when compared to single tone transmission due to the two parallel processing for two discrete CW tones. |  |
| Relative complexity of CW generation | Generation is simple for a reader, which can also be a NR UE using the existing HW. |  |

### Two tone CW (hopping pattern, i.e., only one at a time)

In this option, only a single CW tone will be used at any given time for communication thus ensuring robustness against fading and extending the coverage like single tone CW approach. However, the challenge is to identify the tone that can overcome the fading. To do so, one option is to utilize hopping pattern at regular intervals to study the channel response from the backscattered signal, which is then followed by a communication phase with a CW tone that enjoys good channel condition. The overall behaviour is like that of single tone CW with an increased time for completing AIoT transmission due to frequent channel detection to determine the tone selection.

| CW waveform characteristics | Two tones (hopping pattern) | … (if any) |
| --- | --- | --- |
| D2R reception performance | Robust against fading as the tone that is stronger can be selected for transmission from the training phase. |  |
| Spectrum utilization of backscattered signal corresponding to the CW waveforms | Depending on the BW/rate of backscattered transmission. |  |
| CW interference suppression at D2R receiver | Simple, since the suppression can be done by either using a notch filter or via digital processing to supress a single tone. |  |
| Relative complexity of CW generation | Generation is simple for a reader, which can also be a NR UE using the existing HW. |  |
| Training phase | To select an optimal tone, a training phase is used. |  |

Using two discrete tones that are separated at least by the coherent BW in a hopping pattern ensures same transmit power as that of single tone CW generation, thus, ensuring similar coverage.

RAN1 to study two tone CW hopping pattern with sufficient learning time to identify the stronger one from the available tones.

### Simulation results

In this section, we discuss the impact of frequency hopping for D2R link using OOK style backscattering from AIoT device to the reader. The AIoT D2R 1.92MHz as the occupied BW and the carrier wave is located at the center for no hop case and the carrier is placed equally around the DC in this simulation. The hopping tones considered in this evaluation includes {240KHz,480KHz,960KHz}. The payload size used for evaluation is 96bits and the OOK chip rate are varied from {1,2,4,8} correspondingly. The SFO introduced at the AIoT tag uses [0,100,1000] ppm. Manchester encoding assumed in the transmission; thus the threshold calculation is easier at the reader side processing.

The D2R backscattering is performed as shown in Fig. 2. Note that in a realistic scenario, around the AIoT occupied BW, adjacent NR transmissions may happen concurrently as illustrated in Fig. 2, thus it is the responsibility of the reader to filter those signals to perform detection. The comparison is made with different M values and as we can see, the SNR is improved by a margin of at least 2dB with respect to non-hopping case for all M values evaluated. The impact of SFO degrades the performance of all schemes, however the gain of having hopping vs non-hopping case is still visible in Fig. 3. The receiver employed non-coherent in this case, i.e., the total energy is calculated from two adjacent symbols and averaged to obtain the ON/OFF duration energy.

Fig. 2 AIoT D2R transmission/reception illustrated.

The channel assumed is TDLA30ns channel, which shows milder frequency selectivity thus the separation of hopping tones must be large enough to extract a meaningful frequency diversity. As shown in Fig. 3, larger the separation between the hops, larger the diversity gain utilizing the frequency diversity from the selective fading channel.

Fig. 3 SNR difference between hopping vs non-hopping case for 10% BLER in D2R link using NCOH DTCT.

As can be seen from Fig. 3, the behavior between 480KHz and 960KHz becomes comparable when there is a sampling offset present in the receiver, thus the hopping frequencies must be separated at least by 480KHz to extract the channel diversity from the selective fading.

If the hopping tones are separated by 960KHz≈1MHz, the frequency diversity provides ≈-4dB, irrespective of the modulation order used for OOK based transmission with respect to no hopping case.

However, things are different when the receiver employs coherent processing, i.e., after knowing the duration of each pulse, reader can determine the samples and perform FFT to determine the tone in frequency domain. Then the energy over two adjacent symbols can be used to estimate the contribution from that OOK pulse. This way, the performance can be improved, however, in the presence of SFO of 100pmm, this may degrade the performance as shown in

Fig. 4 SNR difference between hopping vs non-hopping case for 10% BLER in D2R link using COH DTCT

The SFO should be restricted within 100ppm to obtain any benefit from larger payload size. Though Manchester encoding can be used to recover the clock, it may incur additional complexity at the reader side processing.

The separation between two tones used to perform frequency hopping must at least be separated by 480KHz to obtain 3dB gain with respect to non-hopping case.

## Interference to legacy NR devices

The need for guard band in D2R link using backscattered transmission is imperative as the ON/OFF keying used for backscattering may not be aligned with the NR timing. This may cause interference to the UE receptions in case 1-1 and 2-3 as the backscattering is performed in the DL spectrum. In this case, the legacy UEs receiving the NR transmission from the gNB will collect all the energies within the respective BWP. If the AIoT D2R transmission is within the BWP of certain UEs, the FFT process may introduce leakage to adjacent NR carriers. Thus, to avoid any detrimental effect on the legacy NR reception, a sufficient guard band should be used around the AIoT D2R transmission. Fig. 5 illustrates the impact of OOK based D2R leakage to adjacent NR signal. In this case, we assumed OOK signal with M=1, i.e., with one bit per OFDM symbol. The unaligned D2R link causes sufficient leakage to adjacent NR signal if UE in case 2-3 is receiving DL signal adjacent to NR transmission.

| (a) Effect of single tone carrier waveform with OOK D2R | (b) Effect of 2 PRB worth of carrier waveform with OOK D2R |
| --- | --- |

Fig. 5 Impact of OOK leakage to adjacent NR subchannels if D2R is unaligned with NR transmission

Sufficient guard band should be used for AIoT transmissions to avoid any interference caused by the AIoT signals to legacy NR UEs.

# Conclusion

In this contribution, we have made the following observations and proposals related to Ambient IoT:

Observation 1:	The CW node and the reader may or may not be the same device depending on the use case, i.e., monostatic or bistatic.

Observation 2:	Depending on the RRC state and the capability of the UE participating as CW node for AIoT operation, the scheduled NR traffic can either be performed concurrently or completed before CW transmission, respectively.

Observation 3:	CW signal can induce very high interference levels (up to 50 dB) at the reader in bi-static setup.

Observation 4:	To enable multiple AIoT devices to reply with their different payload size data, the CW may be configured to have a fixed duration and a variable repetition size.

Observation 5:	Using two discrete tones that are separated at least by the coherent BW in a hopping pattern ensures same transmit power as that of single tone CW generation, thus, ensuring similar coverage.

Observation 6:	If the hopping tones are separated by 960KHz≈1MHz, the frequency diversity provides ≈-4dB, irrespective of the modulation order used for OOK based transmission with respect to no hopping case.

Observation 7:	The SFO should be restricted within 100ppm to obtain any benefit from larger payload size. Though Manchester encoding can be used to recover the clock, it may incur additional complexity at the reader side processing.

Proposal 1:	RAN1 to study if mitigation procedures/techniques based on existing tools in RAN1 specification, e.g., QCL relationships between reference signals, the cross-link interference (CLI) framework or pre-coding of the CW signal, can be used for mitigating the AIoT activation signal interference at the reader in bi-static setup.

Proposal 2:	RAN1 to identify the aspects and metrics that should drive the selection of the carrier-wave signal type and study the impact that a certain waveform selection has on the main L1 aspects considered in this SI.

Proposal 3:	Existing NR/LTE OFDM waveforms must be used as baseline for study, where considering new waveforms, if adequately justified by practical evidence, is also not precluded.

Proposal 4:	RAN1 to study two tone CW hopping pattern with sufficient learning time to identify the stronger one from the available tones.

Proposal 5:	The separation between two tones used to perform frequency hopping must at least be separated by 480KHz to obtain 3dB gain with respect to non-hopping case.

Proposal 6:	Sufficient guard band should be used for AIoT transmissions to avoid any interference caused by the AIoT signals to legacy NR UEs.

# References

- RP-240826, ”Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103