3GPP TSG RAN WG1 Meeting #119	R1-2409379

Orlando, US, November 18th – 22nd, 2024

Source: 	ZTE Corporation, Sanechips

Title:	Discussion on 3-antenna-port codebook-based transmissions

Agenda item:	9.2.3

Document for:	Discussion and Decision

Introduction

In RAN#105 meeting, Rel-19 WID of support for 3-antenna-port codebook-based transmission was revised. The detail is given as follows [1].

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

In this contribution, we offer our perspectives on further potential enhancements for antenna switching for a 3-antenna-port transmission UE.

Antenna switching for 3Tx UE

In TDD mode, antenna switching is one fundamental feature to achieve DL CSI acquisition. In RAN1#118bis meeting, 3T6R antenna switching was agreed as follows [2]:

Agreement: To support 3T6R antenna switching for a 3TX UE, - The resourceType for a configured SRS resource set can be one of ‘periodic’, ‘semi-persistent’ and ‘aperiodic’, - Reuse the principles used for 4T8R, i.e., - For 4T8R 3T6R, zero or one or two SRS resource sets configured with a different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources, transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports, or - For 4T8R 3T6R, zero or one or two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports. In the case of two resource sets a total of two SRS resources are transmitted in different symbols of two different slots, and where the SRS ports of each SRS resource in the given two sets are associated with a different UE antenna ports. Note: This is not meant to be a text proposal to be captured for the specifications.

- 2.1 3-port SRS transmission scheme considering the support of 2T4R antenna switching SRS

In practical commercial field setup, ‘codebook’ and ‘antenna switching’ SRSs are typical reused to save the total number of SRS resources. For 3Tx UE, the reuse of SRSs with different usages should also be considered in designing the 3-port SRS transmission scheme. Given that a 3Tx UE can support degraded antenna switching configurations (i.e., 2T4R or 1T4R), one typical case is that a 3Tx UE supports 2T4R antenna switching. In such case, the ‘4-1’ and ‘2+2-1’ 3-port SRS transmission schemes are compared as follows:

- Scheme-1: ‘4-1’ 3-port ‘codebook’ SRS transmission scheme

- In such scheme, one 4-port SRS resource is configured for ‘codebook’, and one of the SRS ports is muted. Meanwhile, two 2-port SRS resources are configured for ‘antenna switching’. To reduce the SRS resource overhead, two SRS ports configured within one SRS resource for ‘antenna switching’ can be reused for ‘codebook’. In other words, two SRS ports configured within one ‘antenna switching’ SRS resource and two SRS ports configured within the 4-port ‘codebook SRS’ resource actually occupy the same TD/FD/CD resources. As a result, a total of 5 SRS ports are needed.

- Scheme-2: ‘2+2-1’ 3-port ‘codebook’ SRS transmission scheme

- In such scheme, two 2-port SRS resources are configured for both ‘codebook’ and ‘antenna switching’. All of four SRS ports are used for ‘antenna switching’, while only three of the four SRS ports are reused for ‘codebook’. As a result, a total of 4 SRS ports are needed.

Consequently, Figure 1 illustrates the reuse between ‘antenna switching’ SRS and ‘codebook’ SRS for scheme-1 and Scheme-2.

(a) Scheme-1: ‘4-1’ structure for 3-port ‘codebook’ SRS transmission

(b) Scheme-2: ‘2+2-1’ structure for 3-port ‘codebook’ SRS transmission

Figure 1 Comparison of SRS resource overhead while supporting 3-Tx CB SRS and 2T4R AS SRS

Besides, Table 1 provides the comparison of the SRS configuration and the total number of required SRS ports for Scheme-1 and Scheme-2.

Table 1 Comparison of SRS resource overhead while supporting 3-Tx CB SRS and 2T4R AS-SRS

| 3-port SRS transmission | Codebook SRS (CB-SRS) configuration | Antenna-switching SRS (AS-SRS) configuration (2T4R) | SRS ports in total (CB SRS + AS SRS) |
| --- | --- | --- | --- |
| Scheme-1: ‘4-1’ | One 4-port SRS resource with one port muted | Two individual 2-port SRS resources | 3 + 2 = 5 |
| Scheme-2: ‘2+2-1’ | Two 2-port SRS resources, where one port ‘muted/unused’ of the second resource | Two individual 2-port SRS resources | 2 + 2 = 4 |

Proposal 1: Support the ‘2+2-1’ ‘codebook’ SRS transmission scheme to enable the reuse of 3-port ‘codebook’ SRS and 2T4R ‘antenna switching’ SRS.

- Two 2-port SRS resources are configured, one SRS port in one SRS resource (e.g., the 2nd SRS port in the 2nd SRS resource) is unused for ‘codebook’ usage.

- 2.2 Determination of guard period for 3T6R

Considering that the SRS resource/resource set configuration of 4T8R was reused for 3T6R, the guard period principles of 4T8R should also be reused for 3T6R, i.e.,

- The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot.

- For the inter-set guard period, the UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission.

Proposal 2: Reuse the guard period principles of 4T8R for 3T6R.

- 2.3 Configuration of power control parameters for aperiodic 3T6R SRS

For 3T6R antenna switching SRS, when two aperiodic SRS resource sets are configured, the same power control parameters configuration principles of 4T8R should be reused, i.e., the parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates should be configured with same values for the two aperiodic SRS resource sets.

Proposal 3: Reuse the power control parameters configuration principles of 4T8R for 3T6R, i.e.,

- When two aperiodic SRS resource sets are configured, the parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates are configured with same values for the two SRS resource sets.

Specification details of power splitting for 3Tx

In RAN1#116bis meeting, SRS configuration and power splitting for 3Tx was agreed as follows [3]:

Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, - Option 3: Always a same port is muted, e.g., the 4th port Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, UE splits a linear SRS power equally across the 3 unmuted antenna ports of the 4-port SRS resource. Agreement For 3-port codebook-based PUSCH transmission for a 3TX UE, scale factor s should be the ratio of the number of antenna ports with a non-zero PUSCH transmission power to 3 (except for full-power Mode 0). - FFS: Whether specification needs to be updated to reflect the above

For a 3Tx UE, the 4th port of a 4-port SRS resource is muted to achieve 3-port SRS transmission. For power splitting, it was agreed to split a linear SRS power across the 3 unmuted antenna ports. To accurately specify the power splitting rule, the following TP can be adapted in section 7.1 of TS 38.213.

- if ul-FullPowerTransmission in PUSCH-Config is provided, the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where: - if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode1, and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE in one SRS resource - if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode2, - s=1 for full power TPMIs reported by the UE [18, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with an SRS resource indicated by an SRI field in a DCI format scheduling the PUSCH transmission if more than one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', - s=1, if an SRS resource with a single port is indicated by an SRI field in a DCI format scheduling the PUSCH transmission when more than one SRS resource is provided in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or if only one SRS resource with a single port is provided in the SRS-ResourceSet with usage set to 'codebook', and - if ul-FullPowerTransmission in PUSCH-Config is set to fullpower, s=1 - else, if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of non-zero-power SRS ports supported by the UE in one SRS resource.

Proposal 4: Adapt the following TP in TS 38.213 to specific the power splitting across the 3 unmuted SRS port within a 4-port SRS resource.

- if ul-FullPowerTransmission in PUSCH-Config is provided, the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where: - if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode1, and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE in one SRS resource - if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode2, - s=1 for full power TPMIs reported by the UE [18, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with an SRS resource indicated by an SRI field in a DCI format scheduling the PUSCH transmission if more than one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', - s=1, if an SRS resource with a single port is indicated by an SRI field in a DCI format scheduling the PUSCH transmission when more than one SRS resource is provided in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or if only one SRS resource with a single port is provided in the SRS-ResourceSet with usage set to 'codebook', and - if ul-FullPowerTransmission in PUSCH-Config is set to fullpower, s=1 - else, if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of non-zero-power SRS ports supported by the UE in one SRS resource.

Conclusion

In this contribution, we provide our views on the 3Tx UL transmission, and we have the following proposals.

Proposal 1: Support the ‘2+2-1’ ‘codebook’ SRS transmission scheme to enable the reuse of 3-port ‘codebook’ SRS and 2T4R ‘antenna switching’ SRS.

- Two 2-port SRS resources are configured, one SRS port in one SRS resource (e.g., the 2nd SRS port in the 2nd SRS resource) is unused for ‘codebook’ usage.

Proposal 2: Reuse the guard period principles of 4T8R for 3T6R.

Proposal 3: Reuse the power control parameters configuration principles of 4T8R for 3T6R, i.e.,

- When two aperiodic SRS resource sets are configured, the parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates are configured with same values for the two SRS resource sets.

Proposal 4: Adapt the following TP in TS 38.213 to specific the power splitting across the 3 unmuted SRS port within a 4-port SRS resource.

- if ul-FullPowerTransmission in PUSCH-Config is provided, the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where: - if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode1, and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE in one SRS resource - if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode2, - s=1 for full power TPMIs reported by the UE [18, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with an SRS resource indicated by an SRI field in a DCI format scheduling the PUSCH transmission if more than one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', - s=1, if an SRS resource with a single port is indicated by an SRI field in a DCI format scheduling the PUSCH transmission when more than one SRS resource is provided in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or if only one SRS resource with a single port is provided in the SRS-ResourceSet with usage set to 'codebook', and - if ul-FullPowerTransmission in PUSCH-Config is set to fullpower, s=1 - else, if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of non-zero-power SRS ports supported by the UE in one SRS resource.

References

- RP-242394, WID Revision: NR MIMO Phase 5, Samsung (Moderator)

- Chair notes RAN1#118bis eom0

- Chair notes RAN1#116bis eom2