3GPP TSG-RAN WG1 Meeting #119                                                                                         	   R1-2409403

Orlando, USA, November 18-22, 2024

Agenda Item:	9.8.2

Source:	Huawei, HiSilicon,

Title:	Considerations on the 7-24GHz channel model extension

Document for:	Discussion and Decision

# Introduction

In RAN1#118-bis meeting, the following agreements were reached on 7-24 GHz channel model extension [1]:

Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE, - Angular domain parameters - FFS: Doppler shift, Delay with Option-2 “Determined by the antenna element locations of both TRP and UE”. Note: Angular domain parameters can be optionally modelled. Agreement For near-field channel, if necessary, the antenna element-wise channel parameters of non-direct path between TRP and UE is modelled by: - For the non-direct paths, the antenna element-wise channel parameters are derived based on at least the distance between the BS/UE and a point associated with cluster, i.e., - d1 is used to determine the 1st point associated with cluster to calculate the parameter at least for BS side; - FFS: d2 is used to determine the 2nd point associated with cluster to calculate the parameter at least for UE side; - FFS: whether one point is sufficient - FFS: the details to generate the d1, d2,e.g., if the d2 is needed, both d1 and d2 will be generated pair wise. - FFS: The ratio of non-direct path which show the phenomenon with spherical wavefront - FFS: The determination of the point associated with cluster is conducted in Cluster-level Agreement For near-field channel, if necessary, the following parameters of the non-direct path between TRP and UE should be modeled as antenna element-wise parameter. - Angular domain parameters, Delay (both can be optionally modelled) - Working Assumption - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay. - FFS: Doppler shift Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameter of direct path between TRP and UE, - Delay (can be optionally modelled) - Working Assumption - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay. with Option-2 “Determined by the antenna element locations of both TRP and UE”. Agreement For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, at least for blockage model B, the following new blocker type/size can be introduced in the Table 7.6.4.2-5 in TR 38.901: Typical set of blockers; Blocker dimensions; Mobility pattern. Outdoor; Billboard; Cartesian: w=2.4m; h=3.6m; Stationary. Outdoor; Street lamp; Cartesian: w=0.4m; h=0.8m; Stationary. Outdoor; Building edge; Cartesian: w=X m; h=Y m; Stationary. Indoor; Pillar; Cartesian: w=0.3m; h=3m; Stationary. - FFS: the value of X and Y for the blocker dimensions of building edge is needed. - FFS: the details related to the user hand/head: Indoor;Outdoor; FFS: User hand; Cartesian: w=[0.2]m; h=[0.1]m; Stationary. Indoor;Outdoor; FFS: User head; Cartesian: w=[0.24]m; h=[0.20] m; Stationary. - FFS: The details of blockage model B to implement the impact of user hand and head. - FFS: The location of the user hand/head Agreement For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, for blockage model B, the procedure to determine the number of blockers and locations are same as existing blockage model B. For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, an attenuation per antenna element is introduced, the following options can be considered with down-selection by RAN1#120: - Option-1: generated by leveraging the existing blockage Model-B with potential updates. - At most 2 hand type blockers and one head type blocker for a specific UE is assumed. - Option-2: generated by leveraging the existing blockage Model-A with potential updates - Option-3: generated by a distribution or a fixed value due to the hand grip and/or head proximity - FFS: the details, e.g., how to determine the value of loss for the case as one hand grip, a dual hand grip and/or head proximity

In this contribution, we provide our views on the near-field and spatial non-stationary channel modelling.

# Near-field channel modeling

## Basic modelling methodology

As analyzed in [2], when the near-field assumption applies, the departure/arrival angle and phase of a ray observed at an antenna element should be computed according to the geometry relationship between the corresponding first-/last-bounce scatterer (i.e., 1st/2nd point associated with cluster) and the element. Specifically, the delay and angle of a ray generated according to current TR 38.901 [3] can be treated as the observed delay and angle at the reference antenna element, which is used to locate the first-/last-bounce scatterer, and the observed delay and angle at other antenna elements can be subsequently calculated after scatterer locating.

Figure 1 Illustration of near-field modelling methodology

Following is the 38.901-based step-wise channel coefficient generation procedure taking aforementioned near-field modelling methodology into consideration:

Step 1-10: Generate the delays, angles, and initial phases for a pair of reference antenna elements.

Step 10.5: For each ray:

- Locate the first-/last-bounce scatterer.

The scatterer locating approach in the section 3.4 of [4] can be referred to, which is described detailedly in Appendix A.

- Calculate the observed angle at each antenna element as well as the distances between the first-/last-bounce scatterer and each antenna element.

Step 11-12: Generate channel coefficients according to formula (1)-(3) and apply pathloss and shadowing.

Hu,s,1LOSt=Frx,u,θθLOS,ZOA,u,φLOS,AOA,uFrx,u,ϕθLOS,ZOA,u,φLOS,AOA,uT100-1Ftx,s,θθLOS,ZOD,s,φLOS,AOD,sFtx,s,ϕθLOS,ZOD,s,φLOS,AOD,s

exp-j2πd0,0λ0 expj2πd0,0-du,sλ0expj2πrrx,LOST.vλ0t

(1)

Hu,s,n,mNLOSt=PnMFrx,u,θθn,m,ZOA,u,φn,m,AOA,uFrx,u,φθn,m,ZOA,u,φn,m,AOA,uTexpjΦn,mθθκn,m-1expjΦn,mθφκn,m-1expjΦn,mφθexpjΦn,mφφ

Ftx,s,θθn,m,ZOD,s,φn,m,AOD,sFtx,s,φθn,m,ZOD,s,φn,m,AOD,sexpj2πdrx,n,m,0-drx,n,m,uλ0expj2πdtx,n,m,0-dtx,n,m,sλ0expj2πrrx,n,mT.vλ0t

(2)

Hu,sLOSτ,t=1KR+1n,mHu,s,n,mNLOStδτ-τn,m,0,0+KRKR+1Hu,s,1LOStδτ-τ1

(3)

where d0,0 represents the distance between Tx reference antenna element and Rx reference antenna element, drx,n,m,0 represents the distance between the last-bounce scatterer and Rx reference antenna element , dtx,n,m,0 represents the distance between the first-bounce scatterer and Tx reference antenna element, and τn,m,0,0 represents the propagation delay from Tx reference antenna element to Rx reference antenna element through the NLOS path (n, m).

Proposal 1: For the near-field propagation model of direct and non-direct path, the channel coefficient should be generated according to formulas (1)-(3).

Some companies questioned that the aforementioned near-field modelling methodology cannot match the reflection characteristics. For example, as shown in the left part of Figure 2, when there exists a reflection surface in the propagation environment close to the BS/UE side, locating the first-/last-bounce scatterer at the dashed circle position will lead to inaccurate spherical-wavefront.

While we don’t think constructing the precise correspondence between stochastic channel model and deterministic propagation environment is that meaningful, even if a company insists to do so, the stochastically generated scatterer locating at the image position shown in Figure 2 should meet their requirement.

Figure 2 Propagation environment with reflection surface

## Pending issues

As attached above, several pending issues are listed in the agreement, one of which is to decide the ratio of non-direct path with spherical-wavefront phenomenon. Given that the planer-wavefront is originally an approximation of spherical-wavefront when the far-field assumption holds, setting the ratio of non-direct path with spherical-wavefront phenomenon as 1 (i.e., applying same implementations for both near- and far-field channel realization), which matches the propagation characteristic better and contributes to the conservation of spec. effort, is more preferred. Regarding the concern on computational complexity, although setting the ratio as 1 will incur non-negligible increase for the time consumption of channel initialization, considering channel initialization only takes up tiny proportion of overall simulation time, the increase is inconspicuous.

Proposal 2: Support to set the ratio of non-direct path with spherical-wavefront phenomenon as 1.

Another pending issue is whether the point associated with cluster should be determined per ray or per cluster. Notice that determining the point associated with cluster per cluster will equivalently eliminate the concept of ray by reusing the same set of channel parameters (e.g., phase, angular domain parameters) for the rays within a cluster, which obviously violates the principle of current TR 38.901, per-ray determination is more preferred. In terms of the computational complexity, similar to aforementioned discussion, we believe it’s acceptable.

Proposal 3: For non-direct path, support to determine the point associated with cluster per ray.

# Spatial non-stationary channel modelling

- Origin of spatial non-stationarity

The origin of spatial non-stationarity can be classified into two different categories [5]. First, the blockage effect caused by obstacles may lead to that multipaths can only be received by part of antenna elements in the antenna array. Second, when the physical size of an obstacle is smaller than that of the antenna array (i.e., incomplete scatterer), incomplete reflection, diffraction or scattering may happen in large antenna array. As shown in Figure 3, the reflected/diffracted/scattered path can only be received by part of antenna elements, which results in the spatial non-stationary effect.

Figure 3 Incomplete scatterer

Observation 1: Incomplete reflection, diffraction and scattering can result in spatial non-stationary effect.

To accommodate various origin of spatial non-stationarity, stochastic rather than deterministic modelling methodology is undoubtedly a better choice.

- Visibility-based approach

During RAN1#117 and #118 meeting, several undetermined details of visible probability (VP)/visibility region (VR)-based approach, both of which belong to stochastic modelling methodology, were listed [6][7]. Given the technical commonality of these two approaches, here we provide a visibility-based approach taking both of them into account. Following is the corresponding 38.901-based step-wise channel coefficient generation procedure considering the spatial non-stationary effect at BS side (similar procedure can be adopted to simulate the impact of spatial non-stationarity at UE side):

Step 1-10: Generate the delays, angles, and initial phases for a pair of reference antenna elements.

Step 10.5: For each cluster n:

- Calculate the VP according to the following formula:

Vn=Α∙e-maxPn-Pnλ+Β+δ                                                          (4)

where A and λ are the parameters of exponential distribution, Pn is the cluster power, B is the lower bound of VP, and δ~N(0, ξ2). The elaborate parameterization procedure can be found in Appendix B.

- Decide whether the spatial non-stationarity happens.

If the calculated VP is greater than or equal to 1, there doesn’t exist spatial non-stationary effect at corresponding side.

If the spatial non-stationarity happens, execute the following steps:

- Locate the first-bounce scatterer.

The scatterer locating approach in the section 3.4 of [4] can be referred to, which is described detailedly in Appendix A.

- Calculate the distance dn between Tx antenna elements and the first-bounce scatterer.

- Calculate the VR.

The VR is assumed to be a sphere centered at first-bounce scatterer with radius Rn, where Rn equals to the Vn∙Nallth largest dn and Nall represents the total number of Tx antenna elements.

Figure 4 Exemplified visibility region

- Calculate the power attenuation factor for Tx antenna element s according to the following formula:

αs,n=              1           ,ds,n≤RneRn-ds,ndnmax-dnmin∙C,ds,n>Rn                                                              (5)

where C represents the roll-off coefficient between the visible and invisible region.

Step 11-12: Generate channel coefficients according to formula (7)-(8) and apply pathloss and shadowing.

Hu,s,1LOSt=αs,1Frx,u,θθLOS,ZOA,u,φLOS,AOA,uFrx,u,ϕθLOS,ZOA,u,φLOS,AOA,uT100-1Ftx,s,θθLOS,ZOD,s,φLOS,AOD,sFtx,s,ϕθLOS,ZOD,s,φLOS,AOD,s

exp-j2πd0,0λ0expj2πd0,0-du,sλ0expj2πrrx,LOST.vλ0t

(6)

Hu,s,n,mNLOSt=αs,nPnMFrx,u,θθn,m,ZOA,u,φn,m,AOA,uFrx,u,φθn,m,ZOA,u,φn,m,AOA,uTexpjΦn,mθθκn,m-1expjΦn,mθφκn,m-1expjΦn,mφθexpjΦn,mφφ

Ftx,s,θθn,m,ZOD,s,φn,m,AOD,sFtx,s,φθn,m,ZOD,s,φn,m,AOD,sexpj2πdrx,n,m,0-drx,n,m,uλ0expj2πdtx,n,m,0-dtx,n,m,sλ0expj2πrrx,n,mT.vλ0t

(7)

Proposal 4: The spatial non-stationary propagation characteristic should be modelled based on visibility according to formulas (4)-(7).

# Conclusions

In this contribution, we provide our views on the near-field and non-stationary channel modelling. Following observations and proposals are given:

Observation 1: Incomplete reflection, diffraction and scattering can result in spatial non-stationary effect.

Proposal 1: For the near field propagation model of direct and non-direct path, the channel coefficient should be generated according to formulas (1)-(3).

Proposal 2: Support to set the ratio of non-direct path with spherical-wavefront phenomenon as 1.

Proposal 3: For non-direct path, support to determine the point associated with cluster per ray.

Proposal 4: The spatial non-stationary propagation characteristic should be modelled based on visible probability according to formulas (4)-(7).

# References

- RAN1 Chair’s Notes, RAN1 #118-bis, Hefei, China, October 14-18, 2024

- R1-2402010, Considerations on the 7-24GHz channel model extension, RAN1#116-bis, Changsha, China, 15-19 April, 2024

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

- https://quadriga-channel-model.de/#Download

- Z. Yuan, J. Zhang, Y. Ji, G. F. Pedersen and W. Fan, "Spatial Non-Stationary Near-Field Channel Modeling and Validation for Massive MIMO Systems," in IEEE Transactions on Antennas and Propagation, vol. 71, no. 1, pp. 921-933, Jan. 2023, doi: 0.1109/TAP.2022.3218759.

- RAN1 Chair’s Notes, RAN1#117, Fukuoka, Japan, May 20-24, 2024

- RAN1 Chair’s Notes, RAN1#118, Maastricht, NL, August 19-23, 2024

# Appendix A

For the LOS ray, the location of Tx and Rx antennas are already known. To computed the total pathlength ln,m of each NLOS ray, the absolute delay should be computed based on the generated relative delay. It can be referred to Section 7.6.9 in TR 38.901, where the distribution of the offset ∆τ should be given for all the scenarios.

Figure 5 Scatterer locating

For NLOS ray, we assume multi-bounce propagation and locate the first-/last-bounce scatterer based on the departure/arrival angles and delays. As shown in Figure , the total pathlength of a typical NLOS ray can be divided into three segments. Then the total propagation distance is given by

ln,m=bn,m+cn,m+an,m.

The direction vector bn,m for ray m of cluster n from the TX to the first bounce scatterer is given by

bn,m=sin⁡(θn,m,ZOD)cos⁡(ϕn,m,AOD)sin⁡(ϕn,m,ZOD)sin⁡(θn,m,AOD)cos⁡(θn,m,ZOD)=bn,mbn,m.

Similarly, the direction vector an,m for ray m of each cluster n from Rx to the last bounce scatterer is given by

an,m=sin⁡(θn,m,ZOA)cos⁡(ϕn,m,AOA)sin⁡(θn,m,ZOA)sin⁡(ϕn,m,AOA)cos⁡(θn,m,ZOA)=an,man,m.

To give unique solutions of an,m and bn,m, we minimize the propagation distance between the first and last bounce scatterer cn,m. Then, an,m and bn,m can be calculated by solving

minimize an,m,bn,m     cn,m=ln,m-bn,m-an,m                                                 (8)

subject to  r=bn,m∙bn,m+cn,m∙cn,m-an,m∙an,m

bn,m≥dmin

an,m≥dmin

The minimum distance dmin might be fixed according to the scenario and center frequency, e.g., 1 m for outdoor propagation and 0.1 m for indoor propagation [4]. Given an,m and bn,m, the coordinates of the first-bounce and last-bounce scatterers can be computed directly.

The multi-bounce assumption falls to single-bounce when there is no solution for the equation (8), then

ln,m=bn,m+an,m,

The cosine theorem can be used to compute bn,m directly by

an,m2=r2+bn,m2-2rbn,mcosβn,m,

(ln,m-bn,m)2=r2+bn,m2-2bn,mrTbn,m,

bn,m=ln,m2-r22(ln,m-rTbn,m),

Then an,m can be computed by an,m=bn,m∙bn,m-r.

# Appendix B

The parameterization procedure for formula (4) based on a ray-tracing simulation is described in this section. The simulation assumption and scenario are shown in Table 1 and Figure 6, respectively.

Table 1 Simulation assumption

| Assumption | Value |
| --- | --- |
| Frequency | 6.7GHz |
| BS antenna array | 1008 elements, antenna array size:0.5m(H) x 1.5m(V) |
| BS height | 25m |
| UE height | 1.5m |
| BS number | 7 |
| UE number | 420 |

Figure 6 A typical UMa scenario

In order to fit the VP model, we need to collect the raw VP from the simulation data, which can be calculated for each ray as below:

Vnraw= NnNall

where Nn represents the number of BS antenna elements that can observe the nth cluster and Nall represents the total number of BS antenna elements. The black curve in Figure 7 depicts the raw VPs calculated from the simulation data, based on which the VP model can be fitted as the blue curve in exponential-distribution form, i.e., formula (4). It can be easily obtained from that the VP of a cluster is negatively correlated with its power, which means a cluster with lower power is more susceptible to the spatial non-stationarity.

Figure 7 The VP under given scenario