3GPP TSG-RAN WG1 Meeting #119	R1-2409406

Orlando, US, November 18 – 22, 2024

Agenda Item:	9.11.2

Source:	Huawei, HiSilicon

Title:	Discussion on HD-FDD RedCap UEs and eRedCap UEs for FR1-NTN

Document for:	Discussion and Decision

Introduction

In RAN#104, the WID of NTN for NR Phase 3 was updated [1]. The updated objectives for the operation of HD-FDD RedCap UEs in FR1-NTN focused on specifying enhancements targeting the HD-FDD collision cases 3 and 4 for mitigating issues caused by the mismatch between actual TA used by the UE and TA assumed for the UE at the gNB. In RAN1#118, the following agreements were achieved [2]:

Agreement The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported. - Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - Note: it is not precluded that the same direction is prioritized for all different use cases - Option B: network indicates the priority configuration to the UE - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - It is not precluded to define default rules, if necessary Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied

In the previous meeting RAN1#118bis [3], Options A and B for the priority rule for Case 3 collisions were further discussed and the following conclusion was made to clarify the different Case 3 ‘use cases’ as follows:

Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS

In this contribution, we provide our views and proposed solutions as per the conclusions and agreements achieved thus far.

Discussion

  - TA mismatch in NTN scenario

In NTN, the TA mismatch between gNB and UE on the determination of DL and UL overlapping and back-to-back non-overlapping symbols is due to the large open loop TA adjustment, where UE compensates TA by itself according to the UE position and serving satellite ephemeris, i.e. NUE, adjTA as defined in clause 4.3.1 of TS38.211.

When UE is configured to report TTA  (based on the UE capability of uplink-TA-Reporting-r17) to gNB, including the component of NUE, adjTA, the misalignment between gNB and UE is caused by the report granularity of the Timing Advance Report MAC CE and the infrequent reporting instances. The mismatch between the TA latest known to gNB and the actual TA used by UE can be as large as 16ms when UE is configured with conditional TA reporting by offsetThresholdTA of 15ms plus the 1ms report granularity of the Timing Advance Report MAC CE as defined in clause 6.1.3.56 of TS38.321. Whereas the TA mismatch is still up to 1.5ms when the UE is configured with the least offsetThresholdTA value of 0.5ms which entails the most frequent triggering of TA reporting at the UE leading to increased signalling overhead and UE power consumption. The latter configuration would be of particular concern for the operation of HD-FDD (e)RedCap UEs in NTN.

From gNB’s perspective, if a collision happens at T0 assuming the latest reported TA from UE, the actual collision that happens to UE can be any time from T0-TAmax,mc to T0+TAmax,mc where TAmax,mc is the maximum TA mismatch between gNB and UE according to the TA report configuration. It should be noted that for a UE incapable of TA reporting, the maximum TA mismatch TAmax,mc can be as large as the difference of maximum and minimum TA in the cell coverage area. For example, the TA difference is round 0.72ms in a LEO600 edge cell covered by 4 SSB beams, assuming 50km beam diameter, at 30-degree elevation angle. It can be as large as 6.83ms in a GEO edge cell.

Observation 1: Based on the current TA reporting mechanism, the TA mismatch between UE and gNB can be as large as +/- 16ms with the least frequent report triggering, and up to +/- 1.5ms with the most frequent report triggering raising concerns on increased signalling overhead and UE power consumption for the operation of HD-FDD (e)RedCap UEs in particular.

Observation 2: For a UE incapable of TA reporting or not configured with TA reporting, the TA mismatch can be as large as the difference between the maximum and minimum TA in the cell coverage area.

It should be noted that even in the ideal case when an exact value of the TA latest available prior to the MAC PDU assembly is conditionally reported, the residual TA mismatch between UE and gNB may still occur for a period of time due to the latency in securing and sending the UL-SCH carrying the MAC CE.

For instance, after the reporting condition has already been met, the UE would need to wait for the earliest available UL-SCH resources for accommodating the MAC CE, otherwise the UE would need to send an SR over the long RTT of the NTN cell to request such resources. Hence, the TA mismatch exists and the collision cases can occur until the updated TA report is received and applied at gNB. Handling rules of such collision cases would thus be needed even if the TA reporting is enhanced.

Observation 3: Even if TA reporting is enhanced to reduce resource waste, residual TA mismatch may still occur, at least due to the latency in securing and sending the earliest available UL-SCH resources that can accommodate the MAC CE, e.g. UE may need to send an SR over the long RTT if there is no available resource. Hence, the TA mismatch would exist and the collision cases can occur until the updated TA report is received and applied at gNB. Collision rules are needed despite TA reporting enhancements.

  - Enhancements for Case 3 and Case 4

Priority rules

It was agreed in the previous meeting RAN1#118 that at least HD collisions between semi-statically configured DL receptions and semi-statically configured UL transmissions in Case 3 can occur in NTN due to the TA mismatch between gNB and the HD-FDD UE, and thus can no longer be considered an error case when realized at the UE side [2]. That is, the UE would have to apply a priority rule anyway to determine which of a DL reception and a colliding UL transmission is dropped. Therefore, two options for priority rule(s) were agreed in RAN1#118 for further down selection as follows:

Agreement The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported. - Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - Note: it is not precluded that the same direction is prioritized for all different use cases - Option B: network indicates the priority configuration to the UE - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - It is not precluded to define default rules, if necessary

Option A aims in principle to specify different fixed priority rules for different use cases, i.e., depending on the type of the configured DL reception and the configured UL transmission in collision, unless it is agreed that the priority rule in a given use case is left to UE implementation. Whereas Option B aims in principle at specifying a network indication providing the UE with a general priority rule, which may override a default one, that is applicable to all Case 3 collision use cases except for the ones agreed to be left to UE implementation.

In the previous meeting RAN1#118bis, Options A and B were further discussed and the following conclusion was made to clarify the different Case 3 ‘use cases’ as follows regardless of whether a network indicated or a predefined rule will be adopted:

Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS

As a result of extensive offline and online discussions, the majority views were in favour of applying a same priority rule of keeping one direction and overriding the other to all collision use cases except for some exceptional use case(s), if identified. While it remains for further study whether the same priority rule is network indicated or predefined (fixed) rule, companies have brought up several potential exceptional use cases as listed below in the proposed working assumption 1-v3 captured in the session notes in [4]:

[Proposal 1-v3 for Working Assumption]: To address the issue of collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission), a same priority rule of keeping one direction and overriding another direction is applied to collision use cases other than exceptional use case(s) if identified - Potential exceptional use case(s), if any, at least among the list below: - DL reception and PUCCH with SR - PDCCH USS (e.g. DL grant) and UL transmission - Cases for which there is no sufficient processing time for PUSCH - PDCCH CSS (including for SIB19) and UL transmission - DL reception and PUCCH with SP-CSI - CSI-RS and UL transmission - DL reception and SRS - PRS and UL transmission - FFS: how to determine the priority rule (e.g., network indicated, predefined) - FFS: collision of more than two channels/signals

In the next subsection we discuss the different collision use cases according to the conclusion, in particular the potential use cases listed in the proposed WA. In the subsection after, we share our views on whether the priority rule should be indicated by network or predefined in the specifications.

Discussion on exceptional collision use cases

The following table summarizes the Case 3 collision use cases along with the potential predefined priority rule if such a use case is an exceptional use case or a default rule is specified for such a use case to be possibly overridden by a network indicated priority rule:

Table 1. Case 3 collision use cases

|  | CG-PUSCH | Configured PUCCH | SRS |
| --- | --- | --- | --- |
| SPS-PDSCH | Potential Default Rule: Cancel SPS-PDSCH as UL is often more important than DL for RedCap applications NW indication: Can be to either Cancel UL or DL based on its QoS provisioning for the RedCap application | Potential Default Rule: Cancel PUCCH since UE can send UCI on PUSCH. Periodicity of PUCCH carrying SR is unlikely to same as that of SPS-PDSCH. NW indication: Should be same as default if provided | Potential Default Rule: Partial cancellation of SRS symbols colliding NW indication: Should be same as default if provided unless NW intends to maintain quality of CSI measurements w/o dynamic overhead of scheduling A-SRS |
| P/SP-CSI-RS | Potential Default Rule: Cancellation of P/SP-CSI-RS reception NW indication: Should be same as default if provided unless NW intends to maintain quality of UE CSI measurements w/o dynamic overhead of scheduling A-CSI-RS | Potential Default Rule: Cancel PUCCH since UE can send UCI on PUSCH, and the periodicities of such UL and DL are unlikely to be exactly the same. NW indication: Should be same as default if provided | Potential Default Rule: cancellation of either P/SP-CSI-RS or symbols of SRS colliding by UE implementation. NW indication: decides which of the DL RS and the UL RS to be (partially) cancelled |
| DL-PRS | Potential Default Rule: Cancellation of DL-PRS if configured, since UE is GNSS capable NW indication: Should be same as default if provided unless NW intends to use DL-PRS to verify UE’s location. | Potential Default Rule: Cancellation of DL-PRS if configured, since UE is GNSS capable NW indication: Should be same as default if provided unless NW intends to use DL-PRS to verify UE’s location. | Potential Default Rule: Cancellation of DL-PRS if configured, since UE is GNSS capable NW indication: Should be same as default if provided unless NW intends to use DL-PRS to verify UE’s location. |
| Type 0 PDCCH Scheduling SIB1 | Potential Default Rule: Cancel UL NW indication: Should be same as default, if provided | Potential Default Rule: Cancel UL NW indication: Should be same as default, if provided | Potential Default Rule: Partially Cancel UL symbols colliding NW indication: Should be same as default, if provided |
| Type 0A PDCCH Scheduling OSI including SIB19 | Potential Default Rule: By UE implementation, e.g., based on UL synch status and validity timer NW indication: if applicable, should be same as default | Potential Default Rule: By UE implementation, e.g., based on UL synch status and validity timer NW indication: if applicable, should be same as default | Potential Default Rule: By UE implementation, e.g., based on UL synch status and validity timer NW indication: if applicable, should be same as default |
| Type 0B PDCCH MBS | Potential Default Rule: if applicable to NTN RedCap, Cancel UL NW indication: if applicable, should be same as default | Potential Default Rule: if applicable to NTN RedCap, Cancel UL NW indication: if applicable, should be same as default | Potential Default Rule: if applicable to NTN RedCap, Cancel UL NW indication: if applicable, should be same as default |
| Type 1 PDCCH RAR PDSCH | Potential Default Rule: UE implementation can prioritize DL reception before UE attempts to perform RA, then cancel DL reception otherwise NW indication: if applicable, should be same as default | Potential Default Rule: UE implementation can prioritize DL reception before UE attempts to perform RA, then cancel DL reception otherwise NW indication: if applicable, should be same as default | Potential Default Rule: UE implementation can prioritize DL reception before UE attempts to perform RA, then cancel DL reception otherwise NW indication: if applicable, should be same as default |
| Type 2 PDCCH Paging | Potential Default Rule: Cancel UL NW indication: if applicable, should be same as default | Potential Default Rule: Cancel UL NW indication: if applicable, should be same as default | Potential Default Rule: Cancel UL NW indication: if applicable, should be same as default |

Network indicated vs. Predefined priority rule

In our view, the following points should be taken into account when down selecting between network indicated rule and predefined (fixed) rule:

- Potential specification impact: Given the fact that RAN1 has not agreed yet on the proposed WA, let alone agreeing to confirm it, the possibility of multiple priority rules is not yet precluded.  Therefore, the approach of specifying a fixed priority rule per use case poses the risk of entailing significant specification impact compared to specifying a single priority rule as indicated by gNB, except for exceptional cases, if any. This is due to the fact that the specifications would have to capture the UE procedures for all possible collision combinations of DL receptions and UL transmissions among configured PDCCH/SPS PDSCH/CSI-RS/DL-PRS, and configured SRS/PUCCH/PUSCH, respectively. Even if RAN1 eventually agrees on adopting a single priority rule, the number of exceptional use cases needs to be minimized as much as possible for the same reasons.

Optimization of (e)RedCap performance/resource utilization: Given that the applications of (e)RedCap could range from sensor networks to surveillance cameras, UL transmission may often be of higher priority than DL reception for some HD-FDD UEs than others. Imposing the same set of fixed priority rules on all HD-FDD UEs would unnecessarily restrict the network’s ability to optimize the performance of different (e)RedCap and/or the network’s resource utilization.  Therefore, it is beneficial to give the network the flexibility to indicate to each FDD-HD UE whether or not UL transmission/DL reception is prioritized when collision occurs at the UE.

- Alignment between gNB and UE understanding of procedures: In our view, it makes sense to refer to UE implementation when it comes to collisions with DL common control/broadcast channels/signals such as reception of PDCCH scheduling SIB-19. This is because each UE can determine the priority of a specific instance of such DL receptions based on its individual UL synchronization status and validity timer. Meanwhile, gNB needs to attempt detection of potential UL transmissions from the HD-FDD UEs during such DL occasions. Whereas when referring to UE implementation for collisions with UE-specific DL receptions, gNB would need to transmit DL and attempt to receive UL from each UE at the same time with more processing capability while the radio resource at one direction will be definitely wasted. Therefore, it is not beneficial to completely avoid referring to UE implementation by attempting to specify a priority rule for each of the potential exceptional cases listed.

Based on the above analysis, we propose to support a single network indicated priority rule of Case 3 collisions with the exception of collisions with DL common control/broadcast channels/signals, such as reception of PDCCH scheduling SIB-19, which can be left to UE implementation.

Proposal 1: For collision case 3 (semi-static DL and semi-static UL), support one priority rule applies to all use cases due to less specification impact, more flexibility of the network to optimize (e)RedCap performance/resource utilization, and better alignment between gNB and UE understandings of the procedures.

Proposal 2:  Network should have the flexibility to indicate to each FDD-HD UE whether or not UL transmission is prioritized to DL reception on symbols where collision is determined by the UE with the exception of collisions with DL common control/broadcast channels/signals, such as reception of PDCCH scheduling SIB-19, which can be left to UE implementation.

Collision determination with TA mismatch

For Case 3 or Case 4, when configured/indicated UL transmission from a UE is assumed to be cancelled due to collision with DL with priority rule from gNB’s persepective, the UE may still transmit on the configured/indicated UL resource and interfere with the UL transmissions scheduled to another UE, because the potential collision at UE may not actually happen due to the TA mismatch, as illustrated in Figure 1. In fact, the uplink throughput might be impacted either due to UL interference from the configured UL or by conservative scheduling at the gNB in attempt to avoid the ambiguity of the TA assumed for the UE.

Figure 1. UL cancellation based on the timing at gNB when there is TA mismatch between UE and gNB

Observation 4: When the TA mismatch exists between gNB and UE, there might be less resources available for UL transmission in a cell, if gNB attempts to avoid potential UL interference from HD-FDD RedCap/eRedCap UE which is expected to cancel its UL transmission from gNB perspective based on priority rules.

Therefore, in addition to checking the DL and UL collision based on the actually used TA, the UE can further check the UL and DL collision based on the latest reported TA, assuming gNB makes the scheduling based on this reported TA. If UE cancels the UL transmission on the virtually colliding symbol/slot from gNB’s perspective, gNB can safely re-schedule the UL resource to another UE.

Proposal 3: When determining whether to cancel a UL transmission, UE can further check the DL/UL collision according to the UL timeline with the latest reported TA in addition to the UL timeline with the actually used TA. The UL cancellation rules should be applied if conditions for a cancellation are satisfied based on either the TA actually used or the TA latest reported.

Transmissions involved in multiple collisions

In reference to the above proposed WA, it is proposed to study Case 3 collisions wherein a DL reception would overlap with two UL transmissions based on the UE’s actual timing such that two conflicting priority rules would need to be applied to the DL reception. Similarly, Case 3 collisions wherein a UL transmission would overlap with two DL receptions based on the UE’s actual timing such that two conflicting priority rules would need to be applied to the UL transmission.

In our view, if RAN1 agrees to adopt a single priority rule prioritizing one direction to the other, such an issue would not be of concern since either the one transmission is cancelled or the overlapping two transmissions according to that single rule, unless one of such collisions is one of the exceptional use cases for which the priority rule happens to conflict with the single priority rule. Even then, a straightforward approach would be to maintain the DL/UL direction in accordance with the exceptional priority rule.

Observation 5: if RAN1 adopts a single priority rule prioritizing one direction to the other there is no concern regarding UL/DL involved in multiple collisions with DL/UL, unless one of such collisions is an exceptional use case for which the priority rule happens to conflict with the single priority rule. Even then, a straightforward approach would be to maintain the DL/UL direction in accordance with the exceptional priority rule.

Enhanced TA reporting for reducing TA mismatch

It was agreed in RAN1#118 [2] that TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied to mitigate the collisions of Case 3 and Case 4. Several candidate enhancements were listed as follows:

Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied

First, it is important to note that TA reporting enhancement is beneficial for HD-FDD RedCap/eRedCap UEs regardless of whether gNB attempts to avoid a collision, e.g. by conservative scheduling, or cancellations would occur consequently for Case 3/4.

Second, as analysed earlier in Section 2.1, with the current TA report mechanism, the TA mismatch, and thus the ambiguity period on UE timing at gNB side, can be large due to 1) the TA report granularity, and more importantly 2) the configured TA report triggering offset threshold. Therefore, at least for UEs capable of TA reporting, enhancing the TA report without significantly increasing the reporting overhead to reduce the TA mismatch should be the design objective when selecting from the candidate enhancements listed above.

Since the dynamic overhead of TA reporting depends on both the report size and the frequency of triggering, it is important for an enhanced TA report to use a MAC CE of same or smaller size as that of the legacy TAR (2 Octets: 14bits + 2 Reserved bits), especially if gNB intends to configure a small triggering threshold offsetThresholdTA to cope with TA variations at the UE.

Observation 6: It is important for an enhanced TA report to use a MAC CE of same or smaller size as that of the legacy TAR (2 Octets: 14bits + 2 Reserved bits), especially if gNB intends to configure a small triggering threshold offsetThresholdTA to cope with TA variations at the UE.

In terms of finer TA report granularity and smaller offset threshold; in our view there is a straightforward reporting enhancement that can be considered with respect to the existing TA reporting mechanism to provide finer report granularity but using a new MAC CE of half the size of the legacy TAR MAC CE, i.e. only 1 Octet when a triggering offset threshold less than or equal to 0.25 ms is configured:

Time Advance Delta Reporting (TADR)

Reporting only the change in current TTA value (δTTA) from the last report during the validity period. In other words, TADi is determined as the difference between the current estimate of TA value TTAi and the TA value estimated at the last report within the validity period. An initial legacy TAR report (TAR0) should be sent to current serving cell before triggering the TADR procedure.

Since much fewer bits would be needed to indicate the TADR values compared to the legacy TAR, the MAC CE carrying the TADR can be even designed to be only 1 Octet in size and can also include a Report Index field to enable the gNB to track and accumulate the received TADRs. As shown in Figure 3.

Figure 2. Example of Timing Advance Delta Report MAC CE with Report Index indication field

In order to investigate the performance benefits, the signaling overhead and the feasibility of each enhancement as per the above agreement, including our proposed TADR scheme, SL simulations have been conducted as discussed in the following subsection.

System Level Simulation Results

We have conducted SL simulations based on the assumptions and parameters agreed for Rel-19 NTN DL coverage enhancement WI objective for Set 1-2. These SL simulations calculate the remaining resources available for UL scheduling for each UE when gNB applies the TA ambiguity period before and after the common and UE-specific DL transmissions to avoid HD collisions based on the latest reported TA. VoIP DL traffic, 16 active Earth-fixed beams with one SSB per beam, and SSB/SIB1/SIB19 periodicity of 20ms were assumed. In addition, the following assumptions and parameters were considered as applicable to the HD-FDD operation of (e)RedCap UEs and the legacy/enhanced TA reporting schemes in use:

- TA ambiguity period in symbols after the required DL-UL gap from the end of a DL transmission, or before the required UL-DL gap from the start of the DL transmission, accounts for the configured triggering offset threshold plus the (e)TAR granularity as shown in Figure 3.

- Common and UE-specific DL reception has been prioritized to UL transmission to assess the impact on resource availability.

- (e)TAR is sent on the earliest available resources after a minimum delay at least the one-way propagation delay between the UE and the Satellite node plus the PUSCH preparation delay in FR1.

- (e)TAR triggering offset threshold values considered are {0.25, 0.1, 0.05} ms in addition to the 0.5ms minimum value available for the legacy TAR

- (e)TAR granularities considered are {1OS, Absolute (without quantization), and 13us being the DL-ULGap required for HD operation} in addition to the 1ms granularity of the legacy TAR

- Triggering rate for each (e)TAR scheme has been computed as the total number of triggered reports over the whole simulation time.

- For the TADR scheme, the maximum value of the TA delta has been recorded over the whole simulation time for each of the triggering offset threshold values to allow for calculating the number of bits needed in the new TADR MAC CE assuming a granularity of the DL-UL-Gap duration.

Figure 3. Example of UL cancellation at gNB based on the latest (e)TAR, report granularity and triggering offset; and UL cancellation at the UE based on the actual TA.

From the results captured in Table 2 and Table 3, the following observations can be made:

- The legacy TAR scheme with the 1ms granularity, even with the smallest value of the triggering offset threshold of 0.5ms, substantially renders the NTN (e)RedCap HD UEs resource deprived (< 8%) for scheduling UL transmissions. This emphasizes the need for RAN1 to consider enhancing the TA reporting for enabling the HD operation of (e)RedCap UEs.

- Across all schemes, using a smaller triggering offset threshold renders more available resources for UL scheduling at the expense of increasing the triggering rate by the same factor. However, in contrast to the legacy TAR scheme, the enhanced TA reporting schemes with finer granularity (1OS or less) render around 22% more resources for UL when the threshold is decreased from 0.5ms to 0.05ms.

- Across all schemes, for any given triggering threshold, the TADR scheme renders the largest percentage of UL available resources that is closest to the ideal scheme assuming an absolute (non-quantized) TA report. Intuitively, the granularity of 13us (required DL-UL/UL-DL Gap in FR1) allows for better HD collision avoidance than the granularity of 1OS.

- Observing the maximum TADR values recorded across all UEs over the simulation time, for  a triggering threshold of 0.25ms or less,  a 1-Octet MAC CE is sufficient, assuming the granularity of DL-UL Gap, to indicate all the +/- delta levels while the remaining bits can be used for indicating to gNB the respective report index.

- Although the TADR scheme outperforms the enhanced TAR scheme with 1OS granularity by 5~3% for the triggering offset threshold 0.25~0.05ms, such gain comes with %50 savings in UL dynamic signalling overhead due to the reduced MAC CE size.

Table 2. SLS results for legacy and enhanced TA reporting schemes employing different values of report granularity and triggering offset threshold

Table 3. Proposed new 1-Octet TADR MAC CE design options based on maximum simulated TADR value for the maximum triggering offset threshold

Observation 7: Based on the SL simulation results, the following observations are made:

-	Even with the smallest value of the triggering offset threshold (0.5ms), the legacy TAR scheme with 1ms granularity renders substantially limited resources (< 8%) for scheduling UL. Thus, TA reporting enhancements are essential for enabling the HD operation of (e)RedCap UEs.

-	Smaller triggering thresholds render more resources available for UL at the expense of increased triggering rate by the same factor. With finer granularity (1OS or less), around 22% more resources are released for UL when the threshold is decreased from 0.5ms to 0.05ms.

-	TADR scheme renders the largest percentage of UL available resources closest to the ideal scheme assuming an absolute (non-quantized) TA report.

- 	The granularity of 13us allows for better HD collision avoidance than the granularity of 1OS.

-	For a triggering threshold of 0.25ms or less, a 1-Octet MAC CE is sufficient, assuming 13us granularity, to indicate all the +/- delta levels while the remaining bits can be used for indicating the respective report index.

-	TADR scheme outperforms the enhanced TAR scheme with 1OS granularity by 5~3% for the triggering offset threshold 0.25~0.05ms, however such gain comes as well with %50 savings in UL dynamic signalling overhead due to the reduced MAC CE size.

Proposal 4: RAN1 Considers enhanced TA reporting with finer report granularity than 1ms and triggering offset threshold of at most 0.25ms using a MAC CE of a smaller size than the existing TAR MAC CE, i.e. 1 Octet to compensate for the increased triggering frequency in UL dynamic overhead.

TA reporting with a new triggering mechanism from gNB: It should be noted that a new triggering mechanism from gNB could be easily integrated with our proposed enhancements above for finer TA granularity with the following potential impact to the legacy TAR procedure depicted in 38.321. That is, the legacy TAR procedure may be extended to include additional triggering events for the UE MAC entity to recover from missed eTAR/TADR at gNB:

A Timing Advance report (TAR) shall be triggered if any of the following events occur:

-	upon indication from upper layers to trigger a Timing Advance report;

-	upon configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell;

-	if the variation between the current estimate of the Timing Advance value and the last reported Timing Advance value is equal to or larger than offsetThresholdTA, if configured;

-	upon determining that a Timing Advance Delta Report has not been received at gNB, or upon receiving a request via a DL MAC CE or via a scheduling DCI (e.g. UL grant) to send the latest TA value in a legacy TAR

Proposal 5: the legacy TAR procedure can be extended to include additional triggering events upon receiving a request via a DL MAC CE or via a scheduling DCI (e.g. UL grant) to send the latest TA value in a legacy TAR or for the UE MAC entity to recover from missed eTAR/TADR at gNB.

Triggering a TA report when the collision is detected at the UE: As can be seen from the triggering events listed in the TAR procedure above, a TAR can be triggered in the UE MAC entity ‘upon indication from upper layers’ which is an event that could encompass any detection or decision by the UE to trigger the TA report. Therefore, we believe this proposal may be transparent to such specifications.

Observation 8: TAR can be triggered in the UE MAC entity ‘upon indication from upper layers’ which is an event that could encompass any detection or decision by the UE to trigger the TA report. Therefore, the proposal to trigger a TAR when collision is detected at the UE may be transparent to such specifications.

Reporting TA drifting rate: Based on the quadratic drifting model adopted in the specifications, it is expected that the overall TA drifting rate is a piecewise linear function in time with reference to the epoch time. Despite the additional complexity of drifting rate checking and calculations at the UE, sending drifting rate samples within the linear segments of such function does not provide any additional information to the gNB except at transition points at which the slope changes. Therefore, we believe that if RAN1 determines such reporting is beneficial, then it makes more sense to rather report the drifting rate delta values which target these transition points.

Observation 9: Given that overall TA drifting rate is a piecewise linear function in time, sending drifting rate samples within the linear segments does not provide any additional information to the gNB except at transition points of slope changes. Therefore, it makes more sense to rather report the drifting rate delta values which target these transition points.

Collision handling for Case 4

It can be observed that even though gNB attempts to avoid collisions between dynamic DL reception and dynamic UL transmission for a given HD-FDD UE by proper scheduling, such scheduling would be based on the latest TA assumed or reported by the UE. As such, Case 4 collisions may still occur due to the TA mismatch with the actual TA used by that UE. This situation is in fact not different from Case 3 collisions.

During RAN1 discussions, some companies argued that gNB can rely on existing scheduling offsets to dynamically avoid such collisions. However, gNB use of scheduling offsets, e.g. Koffset, only ensures that the RTT and processing delays are accounted for within the gap between a dynamically scheduled UL and the associated DL reception, e.g., the scheduling grant. This does not ensure though avoiding collision between that scheduled UL and any other dynamically scheduled DL reception at the HD-FDD UE side.

Therefore, similar to Case 3, RAN1 should agree that collision case 4 (Dynamically scheduled DL reception collides with dynamically scheduled UL transmission) can’t be assumed as error case and the design of the enhancements should be common to both collision cases.

Observation 10: Even though gNB attempts to avoid collisions between dynamic DL reception and dynamic UL transmission for a given HD-FDD UE by proper scheduling, such scheduling is based on the latest TA assumed or reported by the UE. As such, Case 4 collisions may still occur due to the TA mismatch with the actual TA used by that UE.

Observation 11: gNB use of scheduling offsets, e.g. Koffset, only ensures that the RTT and processing delays are accounted for within the gap between a dynamically scheduled UL and the associated DL reception, e.g., the scheduling grant. This does not ensure that collisions between that scheduled UL and any other dynamically scheduled DL reception can be avoided at the HD-FDD UE side.

Proposal 6: Similar to Case 3, RAN1 should agree that collision Case 4 (Dynamically scheduled DL reception collides with dynamically scheduled UL transmission) can’t be assumed as error case and the design of the enhancements should be common to both collision cases.

  - Common designs improve collision cases other than Cases 3/4

As captured in the revised WID [1], this section discusses whether enhancements for HD collision cases 3 and 4 can improve other HD collision cases. In our view, the determination of UL and DL collision, the proposed collision determination and enhanced TA reporting for Case 3 and Case 4 are also applicable for the other cases which will be impacted by the TA mismatch.

In cases where configured/indicated UL transmission from a UE is cancelled due to collision with DL (e.g. case 1/5/7), aligning the collision determination between UE and gNB by the proposed scheme for Case3/4 can also improve the uplink throughput compared to conservative scheduling at gNB.

Observation 12: UL and DL collision determined based on the UL timeline with the latest reported TA in addition to the UL timeline with the actually used TA can also improve case 1/5/7 when configured/indicated UL transmission from a UE is cancelled due to collision with DL.

Since the common reason for inflexibility and less resource availability for gNB scheduling of DL/UL across all the cases above is the TA mismatch between actual TA used by the UE and assumed TA at the gNB. Reducing the TA mismatch by more accurate TA report without significantly increasing the reporting overhead from a UE capable of TA report is also beneficial for other cases.

Observation 13: Enhancing TA report without increasing the reporting overhead can also improve other HD collision cases in addition to Case3/4.

Proposal 7: For HD-FDD RedCap UEs and eRedCap UEs, enhancements for HD collision cases 3 and 4 for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB are as follows, where 2) and 3) can also improve other HD collision cases：

- In addition to collision case 3, case 4 should not be an error case; and UE should apply a priority rule configured by the network except for collisions with DL common control/broadcast channels/signals which can be left to UE implementation

- UL and DL collisions are determined based on the UL timeline with the latest reported TA in addition to the UL timeline with the actually used TA, and possibly accounting for the TA ambiguity period at gNB

- Enhance the TA report granularity and triggering offset without increasing the reporting overhead for a UE capable of TA reporting

Conclusions

In this contribution, enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD collisions cases (cases 3 and 4) to support HD-FDD RedCap UEs and eRedCap UEs operation in FR1-NTN are discussed. The following proposals and observations are presented:

Observation 1: Based on the current TA reporting mechanism, the TA mismatch between UE and gNB can be as large as +/- 16ms with the least frequent report triggering, and up to +/- 1.5ms with the most frequent report triggering raising concerns on increased signalling overhead and UE power consumption for the operation of HD-FDD (e)RedCap UEs in particular.

Observation 2: For a UE incapable of TA reporting or not configured with TA reporting, the TA mismatch can be as large as the difference between the maximum and minimum TA in the cell coverage area.

Observation 3: Even if TA reporting is enhanced to reduce resource waste, residual TA mismatch may still occur, at least due to the latency in securing and sending the earliest available UL-SCH resources that can accommodate the MAC CE, e.g. UE may need to send an SR over the long RTT if there is no available resource. Hence, the TA mismatch would exist and the collision cases can occur until the updated TA report is received and applied at gNB. Collision rules are needed despite TA reporting enhancements.

Observation 4: When the TA mismatch exists between gNB and UE, there might be less resources available for UL transmission in a cell, if gNB attempts to avoid potential UL interference from HD-FDD RedCap/eRedCap UE which is expected to cancel its UL transmission from gNB perspective based on priority rules.

Observation 5: if RAN1 adopts a single priority rule prioritizing one direction to the other there is no concern regarding UL/DL involved in multiple collisions with DL/UL, unless one of such collisions is an exceptional use case for which the priority rule happens to conflict with the single priority rule. Even then, a straightforward approach would be to maintain the DL/UL direction in accordance with the exceptional priority rule.

Observation 6: It is important for an enhanced TA report to use a MAC CE of same or smaller size as that of the legacy TAR (2 Octets: 14bits + 2 Reserved bits), especially if gNB intends to configure a small triggering threshold offsetThresholdTA to cope with TA variations at the UE.

Observation 7: Based on the SL simulation results, the following observations are made:

-	Even with the smallest value of the triggering offset threshold (0.5ms), the legacy TAR scheme with 1ms granularity renders substantially limited resources (< 8%) for scheduling UL. Thus, TA reporting enhancements are essential for enabling the HD operation of (e)RedCap UEs.

-	Smaller triggering thresholds render more resources available for UL at the expense of increased triggering rate by the same factor. With finer granularity (1OS or less), around 22% more resources are released for UL when the threshold is decreased from 0.5ms to 0.05ms.

-	TADR scheme renders the largest percentage of UL available resources closest to the ideal scheme assuming an absolute (non-quantized) TA report.

- 	The granularity of 13us allows for better HD collision avoidance than the granularity of 1OS.

-	For a triggering threshold of 0.25ms or less, a 1-Octet MAC CE is sufficient, assuming 13us granularity, to indicate all the +/- delta levels while the remaining bits can be used for indicating the respective report index.

-	TADR scheme outperforms the enhanced TAR scheme with 1OS granularity by 5~3% for the triggering offset threshold 0.25~0.05ms, however such gain comes as well with %50 savings in UL dynamic signalling overhead due to the reduced MAC CE size.

Observation 8: TAR can be triggered in the UE MAC entity ‘upon indication from upper layers’ which is an event that could encompass any detection or decision by the UE to trigger the TA report. Therefore, the proposal to trigger a TAR when collision is detected at the UE may be transparent to such specifications.

Observation 9: Given that overall TA drifting rate is a piecewise linear function in time, sending drifting rate samples within the linear segments does not provide any additional information to the gNB except at transition points of slope changes. Therefore, it makes more sense to rather report the drifting rate delta values which target these transition points.

Observation 10: Even though gNB attempts to avoid collisions between dynamic DL reception and dynamic UL transmission for a given HD-FDD UE by proper scheduling, such scheduling is based on the latest TA assumed or reported by the UE. As such, Case 4 collisions may still occur due to the TA mismatch with the actual TA used by that UE.

Observation 11: gNB use of scheduling offsets, e.g. Koffset, only ensures that the RTT and processing delays are accounted for within the gap between a dynamically scheduled UL and the associated DL reception, e.g., the scheduling grant. This does not ensure that collisions between that scheduled UL and any other dynamically scheduled DL reception can be avoided at the HD-FDD UE side.

Observation 12: UL and DL collision determined based on the UL timeline with the latest reported TA in addition to the UL timeline with the actually used TA can also improve case 1/5/7 when configured/indicated UL transmission from a UE is cancelled due to collision with DL.

Observation 13: Enhancing TA report without increasing the reporting overhead can also improve other HD collision cases in addition to Case3/4.

Proposal 1: For collision case 3 (semi-static DL and semi-static UL), support one priority rule applies to all use cases due to less specification impact, more flexibility of the network to optimize (e)RedCap performance/resource utilization, and better alignment between gNB and UE understandings of the procedures.

Proposal 2:  Network should have the flexibility to indicate to each FDD-HD UE whether or not UL transmission is prioritized to DL reception on symbols where collision is determined by the UE with the exception of collisions with DL common control/broadcast channels/signals, such as reception of PDCCH scheduling SIB-19, which can be left to UE implementation.

Proposal 3: When determining whether to cancel a UL transmission, UE can further check the DL/UL collision according to the UL timeline with the latest reported TA in addition to the UL timeline with the actually used TA. The UL cancellation rules should be applied if conditions for a cancellation are satisfied based on either the TA actually used or the TA latest reported.

Proposal 4: RAN1 Considers enhanced TA reporting with finer report granularity than 1ms and triggering offset threshold of at most 0.25ms using a MAC CE of a smaller size than the existing TAR MAC CE, i.e. 1 Octet to compensate for the increased triggering frequency in UL dynamic overhead.

Proposal 5: the legacy TAR procedure can be extended to include additional triggering events upon receiving a request via a DL MAC CE or via a scheduling DCI (e.g. UL grant) to send the latest TA value in a legacy TAR or for the UE MAC entity to recover from missed eTAR/TADR at gNB.

Proposal 6: Similar to Case 3, RAN1 should agree that collision Case 4 (Dynamically scheduled DL reception collides with dynamically scheduled UL transmission) can’t be assumed as error case and the design of the enhancements should be common to both collision cases.

Proposal 7: For HD-FDD RedCap UEs and eRedCap UEs, enhancements for HD collision cases 3 and 4 for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB are as follows, where 2) and 3) can also improve other HD collision cases：

- In addition to collision case 3, case 4 should not be an error case; and UE should apply a priority rule configured by the network except for collisions with DL common control/broadcast channels/signals which can be left to UE implementation

- UL and DL collisions are determined based on the UL timeline with the latest reported TA in addition to the UL timeline with the actually used TA, and possibly accounting for the TA ambiguity period at gNB

- Enhance the TA report granularity and triggering offset without increasing the reporting overhead for a UE capable of TA reporting

Appendix:

- Existing handling rules for the Cases 1-7

The existing handling rules for the case 1-7 are captured in clause 17.2 of TS38.213 for a FDD-HD RedCap UE in TN.:

- Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission

  - Examples: Transmission of SRS, PUCCH, or PUSCH configured by higher layers colliding with reception of CSI-RS or PDSCH indicated by a DCI format.

  - Collision handling Rules: (same as R15/16 TDD)

    - No cancellation of PUCCH or PUSCH if the first symbol in the set occurs within Tproc,2  relative to a last symbol of a PDCCH reception carrying the DCI format. Otherwise, cancel PUCCH or PUSCH.

    - No cancellation of SRS that occurs in symbols within Tproc,2 relative to a last symbol of a PDCCH reception carrying the DCI format. Cancel the remaining symbols of SRS.

- Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission

  - Examples: Reception of PDCCH, SPS PDSCH, CSI-RS, or DL PRS configured by higher layers colliding with transmission of PUSCH, PUCCH, PRACH, or SRS indicated by a DCI format.

  - Collision handling rule: Reception of PDCCH, SPS PDSCH, CSI-RS, or DL PRS configured by higher layers is cancelled. (same as R15/16 TDD)

- Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission

  - Examples: DL reception configured by dedicated higher layers parameters or configured Type0/0A/0B/1/2-PDCCH CSS collides with UL transmission configured by dedicated higher layer parameters.

  - Collision handling rule: UE monitors paging during paging occasion (type 2 CSS) and cancels CG-PUSCH transmission in RRC_INACTIVE state when a paging occasion overlaps with a CG-SDT transmission. Otherwise, it is considered as an error case, same as in R15/16 TDD.

- Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

  - Examples: UE detects a DCI format scheduling a reception in a set of symbols and a DCI format scheduling a transmission in any symbol from the set of symbols

  - Collision handling rule: It is considered as an error case, same as in R15/16 TDD.

- Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission

  - Examples: Transmission of PRACH triggered by a PDCCH order, or PUSCH, or PUCCH, or SRS collides with symbols of SS/PBCH blocks within the active DL BWP as indicated by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB

  - Collision handling rule: UE does not transmit PRACH, PUSCH, PUCCH if any symbol overlaps with the SSB symbols. UE does not transmit SRS on the symbols overlapping with SSB symbols (same as R15/16 TDD).

- Case 6: Dynamic or semi-static DL collides with valid RO

  - Examples: Transmission of a PRACH or MsgA PUSCH triggered by higher layers in a set of symbols collides with reception of a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS, or symbols of SS/PBCH blocks within the active DL BWP as indicated by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB

  - Collision handling rule: Up to UE implementation (different from R15/16 TDD)

- Case 7: Collision due to direction switching

  - Example 1: A HD-UE would transmit a PUSCH, or PUCCH, or SRS based on a configuration by higher layers and the HD-UE is indicated presence of SS/PBCH blocks within the active DL BWP as indicated by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB

  - Collision handling rule for example 1:

    - UE cancels PUSCH or PUCCH if a last symbol would not be at least NTx-Rx⋅Tc prior to a first symbol of the next earliest SS/PBCH block.

    - UE cancels PUSCH or PUCCH if a first symbol would not be at least NRx-Tx⋅Tc after a last symbol of the previous SS/PBCH block.

    - UE does not transmit SRS in symbols that would not be at least NTx-Rx⋅Tc prior to a first symbol of the next earliest SS/PBCH block.

    - UE does not transmit SRS in symbols that would not be at least NRx-Tx⋅Tc after a last symbol of the previous latest SS/PBCH block.

  - Example 2: The transmission of PRACH or MsgA PUSCH starting or ending at a symbol that is earlier or later than NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of the reception of a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS configured by higher layers or SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB

  - Collision handling rule for example 2: Up to UE implementation

- References

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

Chairman notes, RAN1#118.

Chairman noted, RAN1#118bis

R1-2409226, Session notes for 9.11 (Non-Terrestrial Networks for NR Phase 3 and Internet of Things Phase 3)	Ad-Hoc Chair, Huawei