3GPP TSG-RAN WG1 Meeting #119	R1-2409413

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.5.1

Source:	Huawei, HiSilicon

Title:	On-demand SSB SCell operation for eNES

Document for:	Discussion and Decision

Introduction

In this paper, we continue the discussion on on-demand SSB SCell operation for eNES. We cover various aspects including the signaling, the transmission timing of indicated SSB, the relationship with other signals and neighbor cell issues based on on-demand SSB.

Signaling of on-demand SSB operation

The deactivation of on-demand SSB transmission

Regarding the deactivation of on-demand SSB, the following agreement was achieved in the last meeting.

Agreement - For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options

The options can be categorized in three sets:

- Explicit deactivation indication signaling: Option 1, 1A and 6.

- Implicit deactivation UE behavior: Option 4, 4A, 5.

- Pre-configured deactivation indication: Option 2, 3.

Some implicit behaviors can be defined in spec for certain events within an SCell’s life-cycle. For example, after the deactivation of an SCell, on-demand SSB transmission can be halted to preserve NW energy saving. In this case, UE can assume that on-demand SSB is deactivated when the SCell is deactivated. Option 4 and 4A can be supported so that no redundant signaling is needed. However, after an SCell is activated, it should be up to gNB’s decision whether to halt the SSB transmission, since it can still serve for sync and beam management in Scenario #3B. Thus, we propose not to support Option 5.

- For the implicit deactivation UE behavior of on-demand SSB, support options 4 and 4A. And do not support Option 5.

Consider that on-demand SSB can be activated and deactivated anytime, explicit signaling based options should be supported. In previous meetings, RRC and MAC CE were agreed to function as the signaling that indicate on-demand SSB transmission. It is straightforward to reuse such mechanism to deactivate the transmission by, e.g., a flag in the RRC / MAC CE that can indicate the state of activation or deactivation, while other designs can remain the same. DCI-based signaling seems redundant for both activation and deactivation. To minimize spec workload, we support Option 1 and 1A, and not support Option 6.

- For the explicit deactivation indication signaling of on-demand SSB, support options 1 and 1A. And do not support Option 6.

Furthermore, timer-based deactivation can also be supported. For L1-/L3-measurement, if the UE did not receive any SSBs while the timer is running then it assumes the SSBs are deactivated. A maximum number N of rounds of bursts, or a maximum transmission time window can be configured to handle such exception. Option 2 or 3 can be supported.

- For Pre-configured deactivation indication of on-demand SSB, support either Option 2 or Option 3 (only the Configuration).

The RRC configuration of on-demand SSB transmission

Regarding the configuration of on-demand SSB by RRC, the following agreement was achieved in the last meeting.

Agreement - For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

In legacy configuration of an SCell, the SCS of SSB is provided via the field ssbSubcarrierSpacing in RRC IE: ServingCellConfigCommon. This field is mandatory present upon SCell addition for UE’s correct monitoring. Similarly, the field of SCS of the on-demand SSB should be mandatorily present at least for Case #1. For Case #2, it is possible for its absence, whilst UE may assume that on-demand SSB have the same SCS as the always-on SSB on the cell.

- For the configuration of on-demand SSB, sub-carrier spacing of the on-demand SSB should be mandatorily present for Case #1, and can be absent for Case #2.

As discussed in Section 2.1, the maximum number of bursts or the maximum transmission duration of on-demand SSB can be configured to avoid endless transmission. Such number N (or equivalently, duration T) can be provided as a parameter in the RRC, supplementary to the explicit deactivation signalling. For this or all the other agreed parameters, whether to use legacy or new RRC is out of RAN1’s scope and should be decided by RAN2.

- The maximum number of bursts or the maximum transmission duration of on-demand SSB can be configured in RRC.

- At least for the already-agreed parameters of SCS, physical cell ID, and downlink transmit power, it is up to RAN2 to decide to use the legacy or new RRC IE(s).

Regarding time domain location of on-demand SSB, the following proposal w.r.t. the SSB positions within a burst was raised for discussion in the last meeting:

Proposal #4-2 (RRC+MAC-CE indication): For a cell supporting on-demand SSB SCell operation, in addition to periodicity of the on-demand SSB, for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst - [The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated or duration of on-demand SSB transmission window]

For on-demand SSB, enabling to dynamically choose one of multiple possible beam patterns on an SCell is one of the most promising use cases that motivate the design of this feature in Rel-19. For example, for an FR2 SCell, on-demand SSB beam patterns can be designed (on the SCell) for each one of the actually transmitted SSB on PCell (being FR1 or FR2 cell; collocated or non-collocated with the SCell). If gNB knows the best serving SSB beam(s) of the UE on the PCell, by e.g., using L1 SSB measurements on PCell, then at least in Scenario #2A, gNB can activate the corresponding on-demand SSB pattern for SCell activation, enabling more accurate UE CSI-reporting at the end of SCell activation (e.g., if the corresponding SSB beam pattern is narrow) and/or faster SCell activation to increase the NES gain (e.g., if the corresponding SSB beam pattern is wide but repeated many times in the SCell SSB burst).

Figure 1: A subset of on-demand SSB beams can be sent on SCell when priori information on UE’s best serving beams is available.

Therefore, we support the FL Proposal #4-2 (in FL summary of RAN1#118bis) of configuring multiple SSBs in SSB burst and to use the MAC CE to dynamically indicate one of them. Sending the bit map of the pattern in the MAC CE is also acceptable, which we can leave to RAN2 to decide and design.

- Support the indication of the SSB positions within an on-demand SSB burst either using, multiple candidate values configured by RRC and the applicable value is indicated by MAC CE or by directly indicating the value in the MAC CE. RAN2 to decide the design of the MAC CE and RRC.

Next, we further examine whether to introduce new RRC parameters and spec change to dynamically and flexibly configure and indicate the SSB positions and/or the gap between them within an on-demand SSB burst to pursue additional NES gains for, e.g., advanced Rel-19 UEs. For legacy SSB, ssb-PositionsInBurst is a bitmap to indicate the actually transmitted SSBs in SSB burst where the positions of candidate SSBs are defined and fixed in the RAN1 spec. This design has not changed since Rel-15, and it is clearly not suitable any more for on-demand SSB operation, especially for SCell activation procedure, since a UE is not needed to do blind search of SSB as the case for SSBs from initial access on PCell. Because of the above considerations, we propose to consider on-demand SSB with a compact pattern, which is enabled by the following configurations:

- A parameter configures the number of SSBs in a compact SSB pattern.

- A parameter configures the gap in symbols between each SSB.

With the above, a UE can formulate the transmitted SSBs in a periodical and compact manner. Note the above configurations can depend on UE capability thus would be a separate configuration from legacy case. If needed, legacy SSB patterns can be restricted as one configuration choice for R19 on-demand SSB, associated with the legacy ssb-PositionsInBurst parameter.

- To enable more efficient on-demand SSB operation for SCell, new UE capability as well as RRC configurations that indicate a compact pattern, comprising (1) the number of SSBs of one burst and (2) the gaps in between the SSBs, can be introduced.

Transmission timing of the indicated on-demand SSB

Regarding transmission timing of on-demand SSB, the following agreement was made in the last meeting:

Agreement The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

It was agreed that the on-demand SSB time domain position is configured by gNB, so UE would know where the first possible burst is to be placed. However, we’d like to point out there exists critical scheduling problems for gNB when more than one UEs are indicated with a same SSB at different time.

For example, on-demand SSB#1 is turned ON for UE#1 at time point X. At some point later, this cell is configured as an SCell for UE#2, with on-demand SSB#1 configured simultaneously. Then, gNB finds out that the configuration of on-demand SSB#1 also fits UE#2’s demand, so it decides to indicate UE#2 with the transmission of on-demand SSB#1. However, as the offset between the indicating signaling and the upcoming burst has been fixed in configuration, gNB has to schedule MAC CE at a very particular time point (e.g., on-demand SSB #1 is set to be sent with an SFN offset of 1 after the MAC CE, so gNB must send MAC CE one frame ahead of the upcoming on-demand SSB#1.) and this will bring up with much pressure to the scheduler.

Figure 2: Illustration of the difficulty on scheduling when on-demand SSB is configured with a fixed offset

This can be possibly solved by:

- Option 1: Using group-common DCI. However, this would bring a quantity of spec workload to design a redundant signaling.

- Option 2: An additional offset is given in the MAC CE to indicate the SSB, i.e., UE#2 assumes that SSB would be transmitted ‘configured offset ± additional offset’ after the MAC CE. For instance, there exists a demand for UE#2 to use SSB at subframe #N, and there is a being-transmitted on-demand SSB with configured offset of 10 subframes and its next burst starts from subframe #(N+6). The additional offset can be set as -4 subframes so that UE#2 would have a precise understanding of the SSB transmission. Clearly, the scheduling possibility is increased and UE#2 can be indicated with the timeliest SSB burst to monitor.

Figure 3: An additional offset is given in the MAC CE to facilitate scheduling possibility

In a word, we propose:

- RAN1 to support a dynamic offset given in the MAC CE that indicate the time position of the first “possible” on-demand SSB burst.

On collision between on-demand SSB and other signals

Though being a signal that can be indicated in a user-specific manner (via dedicated RRC messages or MAC CE), on-demand SSB should be considered as cell-specific and it can be served for more than one UEs to avoid multiple transmissions from gNB’s point of view. That is to say, gNB may not be able to freely adjust the on-demand SSB to avoid collision with other signals. It is important to scrutinize such topic in case our new Rel-19 design would negatively impact the overall NR system performance.

On-demand SSB colliding with always-on SSB

This topic is associated with the inquiry of the relationship between always-on SSB and on-demand SSB from RAN4 [1]. For the analysis of this topic, please refer to the discussion on the reply to RAN4 LS in [2].

On-demand SSB colliding with other common / dedicated signals

In current spec, SSB transmission/reception is prioritized over other signals when collision happens. Consider that SSB is transmitted on-demand on a Rel-19 eNES cell, the legacy collision handling with other common/dedicated signals can be revisited. For the details of this subsection, please refer to [3].

- RAN1 to consider the collision handling between on-demand SSB and other common/dedicated signals on a cell under Obj3 for consistence with the design of SSB adaptation.

On neighbor cell issue of on-demand SSB operation

When on-demand SSB is dynamically turned ON/OFF or adapted on a Rel-19 eNES cell, it would also raise concerns of neighbor cells. Below, we identify two significant issues, covering the interference management of cell-edge scenario and the L3 measurement enhancement of mobility scenario, respectively.

Inter-cell interference

When UE moves to the edge of a Rel-19 eNES cell, and the on-demand SSB is turned ON/OFF or adapted from time to time, the data/control channel or SSB transmission on the same frequency of neighbor cell may cause severe inter-cell interference. For instance, when there exists a 20ms SSB on the neighbor cell, and the 20ms on-demand SSB on this serving SCell is turned OFF, the neighboring SSB would interfere data/control channel on this serving SCell, causing deterioration of data reception. On the other hand, when there exists a 20ms SSB on the neighbor cell, and a 5ms on-demand SSB on this serving SCell is turned ON, the neighboring data/control channel would interfere SSB on this serving SCell, causing deterioration of measurement or sync. To avoid the deterioration of performance of edge users causing by the constant ON/OFF/adaptation of SSB, RAN1 can consider potential enhancement w.r.t. the cell-edge scenarios, e.g., inter-cell rate-matching mechanism to lessen inter-cell interference.

- RAN1 to consider potential enhancement w.r.t. the cell-edge scenarios, e.g., inter-cell rate-matching mechanism to lessen inter-cell interference introduced by the dynamic ON/OFF/adaptation of on-demand SSB.

Neighbor cell measurement

From mobility perspective, in current spec, UE can measure the signal quality of the neighboring cells to determine whether SCell change is needed. As UE may not know what is the current state of an on-demand SSB transmission on a non-serving cell, it may not properly report certain event due to the absence of neighbor cell SSB in SMTC windows. Regarding this issue, there can be three main directions to consider.

- Direction 1: Follow the legacy rule/requirements and leave the potential mis-measurement to gNB / UE implementation.

- Direction 2: Consider an inter-cell / inter-node on-demand SSB state notification mechanism. As SSB turns ON or OFF on a Rel-19 eNES cell, gNB should broadcast the information to UEs who consider this cell as a to-be-measured neighbor cell.

- Direction 3: Consider modification on existing neighbor cell measurement procedure/requirement, particularly on legacy L3 measurement (procedure). For example, for event-triggered reports, since gNB starts to enable flexible SSB ON/OFF, the existing report conditions could be varying and it may be too strict for UE to verify a full set of conditions as legacy.

Figure 6: Three directions for spec to consider for L3 neighbor cell measurement on on-demand SSB

It can be observed that, Direction 1 does not provide a minimum necessary measurement procedure as the potential issue can happen without being identified from gNB or UE. Direction 2 is able to provide a comprehensive solution, yet spec effort might be high as well as at the cost of large amount of interaction among the cells / nodes. Direction 3 can provide a compromising solution to maintain the pipeline of L3 neighbor cell measurement without involving inter-cell / inter-node interaction. RAN2/4 expertise would also be needed for further study of Direction 3.

- RAN1 to consider the needed modification on existing L3 measurement procedure/requirement.

Conclusion

This paper discussed how to design on-demand SSB operation for SCell, with the following observation/proposals:

- For the implicit deactivation UE behavior of on-demand SSB, support options 4 and 4A. And do not support Option 5.

- For the explicit deactivation indication signaling of on-demand SSB, support options 1 and 1A. And do not support Option 6.

- For Pre-configured deactivation indication of on-demand SSB, support either Option 2 or Option 3 (only the Configuration).

- For the configuration of on-demand SSB, sub-carrier spacing of the on-demand SSB should be mandatorily present for Case #1, and can be absent for Case #2.

- The maximum number of bursts or the maximum transmission duration of on-demand SSB can be configured in RRC.

- At least for the already-agreed parameters of SCS, physical cell ID, and downlink transmit power, it is up to RAN2 to decide to use the legacy or new RRC IE(s).

- Support the indication of the SSB positions within an on-demand SSB burst either using, multiple candidate values configured by RRC and the applicable value is indicated by MAC CE or by directly indicating the value in the MAC CE. RAN2 to decide the design of the MAC CE and RRC.

- To enable more efficient on-demand SSB operation for SCell, new UE capability as well as RRC configurations that indicate a compact pattern, comprising (1) the number of SSBs of one burst and (2) the gaps in between the SSBs, can be introduced.

- RAN1 to support a dynamic offset given in the MAC CE that indicate the time position of the first “possible” on-demand SSB burst.

- RAN1 to consider the collision handling between on-demand SSB and other common/dedicated signals on a cell under Obj3 for consistence with the design of SSB adaptation.

- RAN1 to consider potential enhancement w.r.t. the cell-edge scenarios, e.g., inter-cell rate-matching mechanism to lessen inter-cell interference introduced by the dynamic ON/OFF/adaptation of on-demand SSB.

- RAN1 to consider the needed modification on existing L3 measurement procedure/requirement.

# References

- R4-2416913, LS on SSB relation in On-demand SSB and SSB adaptation on SCell, 3GPP TSG-RAN WG4, October, 2024.

R1-2410609, Discussion on RAN4 LS on SSB relation in On-demand SSB and SSB adaptation on SCell, Huawei, 3GPP TSG RAN WG1 #119, November, 2024.

- R1-2409415, On common channel/signal adaptation for eNES, Huawei, 3GPP TSG RAN WG1 #119, November, 2024.