3GPP TSG RAN WG1 Meeting #119													R1-2409468

Orlando, US, Nov 18th – 22nd, 2024

Agenda Item:		9.8.2

Source:				InterDigital, Inc.

Title:						Further Discussion on Channel Model Extension for FR3

Document for:		Discussion and Decision

# Introduction

In RAN1 #118b meeting, RAN1 continued the discussion on the extension of the existing channel model for near-field study in FR3, resulting in the following agreements [1],

Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE, - Angular domain parameters - FFS: Doppler shift, Delay with Option-2 “Determined by the antenna element locations of both TRP and UE”. Note: Angular domain parameters can be optionally modelled. Conclusion For near-field channel, no changes are expected on following parameters of the non-direct path between TRP and UE: - Amplitude Agreement For near-field channel, if necessary, the antenna element-wise channel parameters of non-direct path between TRP and UE is modelled by: - For the non-direct paths, the antenna element-wise channel parameters are derived based on at least the distance between the BS/UE and a point associated with cluster, i.e., - d1 is used to determine the 1st point associated with cluster to calculate the parameter at least for BS side; - FFS: d2 is used to determine the 2nd point associated with cluster to calculate the parameter at least for UE side; - FFS: whether one point is sufficient - FFS: the details to generate the d1, d2,e.g.,if the d2 is needed, both d1 and d2 will be generated pair wise. - FFS: The ratio of non-direct path which show the phenomenon with spherical wavefront - FFS: The determination of the point associated with cluster is conducted in Cluster-level Agreement For near-field channel, if necessary, the following parameters of the non-direct path between TRP and UE should be modeled as antenna element-wise parameter. - Angular domain parameters, Delay (both can be optionally modelled) - Working Assumption - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay. - FFS: Doppler shift Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameter of direct path between TRP and UE, - Delay (can be optionally modelled) - Working Assumption - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay. with Option-2 “Determined by the antenna element locations of both TRP and UE”. Conclusion RAN1 confirms that the modelling of near-field propagation characteristics (i.e., characteristics of spherical wavefront) is taken as an additional modelling component, and the relevant progress (e.g., the agreement(s) with/without “if necessary”) in AI 9.8.2 will be captured in TR 38.901. Agreement For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, at least for blockage model B, the following new blocker type/size can be introduced in the Table 7.6.4.2-5 in TR 38.901: Typical set of blockers; Blocker dimensions; Mobility pattern. Outdoor; Billboard; Cartesian: w=2.4m; h=3.6m; Stationary. Outdoor; Street lamp; Cartesian: w=0.4m; h=0.8m; Stationary. Outdoor; Building edge; Cartesian: w=X m; h=Y m; Stationary. Indoor; Pillar; Cartesian: w=0.3m; h=3m; Stationary. - FFS: the value of X and Y for the blocker dimensions of building edge is needed. - FFS: the details related to the user hand/head: Indoor;Outdoor; FFS: User hand; Cartesian: w=[0.2]m; h=[0.1]m; Stationary. Indoor;Outdoor; FFS: User head; Cartesian: w=[0.24]m; h=[0.20] m; Stationary. - FFS:The details of blockage model B to implement the impact of user hand and head. - FFS: The location of the user hand/head Agreement For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, for blockage model B, the procedure to determine the number of blockers and locations are same as existing blockage model B. For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, an attenuation per antenna element is introduced, the following options can be considered with down-selection by RAN1#120: - Option-1: generated by leveraging the existing blockage Model-B with potential updates. - At most 2 hand type blockers and one head type blocker for a specific UE is assumed. - Option-2: generated by leveraging the existing blockage Model-A with potential updates - Option-3: generated by a distribution or a fixed value due to the hand grip and/or head proximity - FFS: the details, e.g., how to determine the value of loss for the case as one hand grip, a dual hand grip and/or head proximity

Based on the progress made in the last meeting, in this contribution, we provide our views on aspects related to the extension of TR38.901 channel model for FR3 (7-24GHz).

# Extension of channel coefficient generation in TR 38.901

According to agreement from the last meeting, for near-field channel model, RAN1 strives to design a unified model to explicitly reflect the new properties of near-field (NF) and existing properties of far-field under the structure of existing stochastic model TR 38.901. However, whether the same or different implementations, e.g., procedures/equations, are used for near- and far-field channel realization are to be studied.

According to TR 38.901 [2], the procedure for channel coefficient generation comprises 12 steps, each involving modelling of a different aspect of the wireless channel. For FR3 NF stochastic channel modelling in Rel-19, the goal is to verify the relevance, functionality, and associated parameters for each of the 12 steps from the perspective of a NF transmission. The degree of change may vary from one step to another, and changes in one step may be more involved than the other. However, it should be noted that FR3 channel modelling does not necessarily require a fundamental change neither in the general procedure, nor in the basic principle of processing for most of the steps. Table 1 captures the state of progress for FR3 NF channel modelling in RAN1.

Table 1 – Summary of state of FR3 NF channel modeling

| Steps | State of progress |
| --- | --- |
| Step 1: Set Scenario | Uma, UMi, Indoor office and Indoor factory (RAN1 #116b) |
| Step 2: Assign propagation condition | No changes are expected (RAN1#118) |
| Step 3: Calculate (NF) pathloss | No changes are expected (RAN1#118) |
| Step 4: Generate correlated large-scale parameters (DS, AS, SF, K) | No changes are expected (RAN1#118) |
| Step 5: Generate NF cluster delay, and location information or equivalent information) | Under discussion: Enhancing delay generation mechanism to account for each path and associated cluster delay. |
| Step 6: Generate (NF) cluster powers | Under discussion: Considering NF effect and its stochastic property. The exponential decay behaviors of cluster power with respect to their delays, as used for SCME channel model may not be applicable for NF [3]. |
| Step 7: Generate (NF) SWM | Antenna element-wise is the baseline approach (RAN1#118). Impact of SnS is under discussion. |
| Step 8: Perform random coupling of rays | Under discussion: Pending outcome of the discussions on cluster definition and SnS. |
| Step 9: Generate XPRs | No changes are expected (RAN1#117, RAN1#118) |
| Step 10: Draw random initial phases | Under discussion. |
| Step 11: Generate channel coefficient (including SnS) | Under discussion. SnS includes both at TRP and UE side and blockage model A and/or B will be further discussed. |
| Step 12: Apply pathloss and shadowing | No change is expected. |

Observation 1: SnS and cluster consideration have a high impact on details related to other steps.

Proposal 1: Prioritize discussions on SnS and cluster consideration to facilitate progress on other topics.

# Antenna element-wise channel parameters discussion

In the RAN1#118 meeting, the proposed alternatives for modelling of antenna element-wise channel parameters of non-direct path between TRP and UE were agreed to be narrowed down to the following 2 options:

- Option-1: The antenna element-wise channel parameters are derived based on at least the distance between the BS/UE and a point associated with cluster. - FFS: How to obtain the distance. - FFS: Other parameters. - FFS: association between a point and a cluster - Option-2: The antenna element-wise channel parameters are determined based on the existing spatial consistency procedure of TR 38.901 with updates. - FFS: Details to obtain the antenna element-wise parameters.

In the RAN1#118b meeting, the following agreement has been made:

For near-field channel, if necessary, the antenna element-wise channel parameters of non-direct path between TRP and UE is modelled by:

- For the non-direct paths, the antenna element-wise channel parameters are derived based on at least the distance between the BS/UE and a point associated with cluster, i.e., - d1 is used to determine the 1st point associated with cluster to calculate the parameter at least for BS side; - FFS: d2 is used to determine the 2nd point associated with cluster to calculate the parameter at least for UE side; - FFS: whether one point is sufficient - FFS: the details to generate the d1, d2,e.g.,if the d2 is needed, both d1 and d2 will be generated pair wise. - FFS: The ratio of non-direct path which show the phenomenon with spherical wavefront - FFS: The determination of the point associated with cluster is conducted in Cluster-level

In our view, as we discuss in the following, Option-1 represents a more practical propagation environment for the spherical wave effect of a NF transmission.

One reference point model (d1 only) or two reference points model (d1 and d2) for non-direct paths

In our view, the one reference point model is a special case of the two-reference points model when the first bounce cluster and the last bounce cluster are the same one. In option 1, when the first bounce cluster and the last bounce cluster are the same, only d1 needs to be determined and d2 can be simply calculated as d2=cτn- d1, where τn is the total propagation delay for cluster n. In the real world, a propagation typically experiences more than one reflection. Therefore, to support more flexibility and accuracy in modelling the NF channel, it may be better that both d1 and d2 are considered. On the other hand, consideration of d1 and d2 result in a higher computational complexity, hence further study is needed whether a d1-based approach can sufficiently support the accuracy and needs of NF channel modelling.

Observation 2: A propagation typically experiences more than one reflection. Therefore, to support more flexibility and accuracy in modelling of the NF channel, it may be better that both d1 and d2 are considered.

Proposal 2: Support d1-based solution, only if there are sufficient analysis and measurement to validate its accuracy and flexibility.

In the most straightforward approach, the distances d1 and d2, can be independently generated from different distribution functions. For example, d1 and d2 can be independently generated based on a uniform distribution, and then a delay value may be generated for the virtual link, connecting the two points. However, this seems unnecessarily a complicated approach. Assuming a general formulation of d2,n=cτn-d1,n where n=1⋯N, represents the number of clusters N, the process of determination of the two distances can be somehow simplified by considering a stochastic distribution for generation of the total delay τn and d1,n. Clearly, different assumptions may be considered,

- In one approach, a single total delay per cluster may be considered, and the variability between the delay in propagation paths may be left to the delay correspond to the distance d1.

- Alternatively, if supported by measurement, completely collapse d2 , and only consider stochastic-based generation of τn and d1,n.

Observation 3: The process of determination of the two distances can be somehow simplified by considering a stochastic distribution for generation of the total delay τn and d1,n.

Proposal 3: Study when d1 is generated from a distribution function (e.g. a uniform distribution) and d2 is generated from the cluster delay τn and the d1.

Near-field/far-field condition of non-direct paths

As indicated in the SID [3], the resulting channel model should support a seamless transition between NF and FF transmissions to assure consistency in the behaviour of the channel. Such accommodation can be achieved through the revised procedure for Step 11. In practice, UE array aperture is relatively small compared to TRP/gNB. Therefore, we further analyse NF NLOS with the NF/FF conditions. Especially we further partition the NLOS path into two independent set of paths, i.e., Tx-Sc (transmit to cluster/scatter) and Sc-Rx (receive to cluster/scatter). The following simulations demonstrate the NLOS channel with different NF/FF assumption/condition for each path. The simulation assumptions are summarized as follows:

- Cluster/scatter location is known,

- Single bounce cluster is assumed,

- Central frequency is set at 10 GHz,

- Antenna element separation is equal to λ2 spaced.

- Tx aperture is less than 1m.

To study, we consider different combinations of FF and NF and study the phase error regime incurred w.r.t the channel model, as listed below:

- FF-FF: Far field assumption on both paths (Tx-Sc, Sc-Rx),

- FF-NF: Far Field assumption for Tx-Sc path, and Near-Field approximation for Sc-Rx path,

- NF-FF: Near Field assumption for Tx-Sc, and Far-Field assumption for Sc-Rx path,

- NF-NF: Near field assumption for both paths (Tx-Sc, Sc-Rx).

As an example, we consider the following case where a 45×45 Tx array is placed at (0,0,0) and 3×3 Rx array is placed at (0, Rdtx2, 0) where Rdtx=63.44m denotes the Rayleigh distance corresponding to the Tx array. A non-direct propagation distance of 50 m, which corresponds to a propagation delay of 0.167 μs, is assumed. Table 2 shows the phase errors when we employ any of the four assumptions. In the table, Rdrx=0.12m denotes the Rayleigh distance corresponding to the Rx array.

Table 2 Maximum phase error for four NF/FF approximations when the scatterer is not close to the Rx array

| Tx - Sc Path 1 |  | Sc - Rx Path 2 |  | Maximum phase error in radians |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length (m) | >Rdtx | Length (m) | >Rdrx | FF-FF |  |  | FF-NF |  |  | NF-FF |  |  | NF-NF |  |  |
|  |  |  |  | Path 1 | Path 2 | Total | Path 1 | Path 2 | Total | Path 1 | Path 2 | Total | Path 1 | Path 2 | Total |
| 40.86 | No | 9.14 | Yes | 0.610 | 0.005 | 0.615 | 0.610 | 0.000 | 0.610 | 0.000 | 0.005 | 0.005 | 0.000 | 0.000 | 0.000 |
| 31.07 | No | 18.93 | Yes | 0.802 | 0.002 | 0.804 | 0.802 | 0.000 | 0.802 | 0.005 | 0.002 | 0.007 | 0.005 | 0.000 | 0.005 |
| 18.93 | No | 31.07 | Yes | 1.316 | 0.002 | 1.317 | 1.316 | 0.000 | 1.316 | 0.005 | 0.002 | 0.006 | 0.005 | 0.000 | 0.005 |
| 10.35 | No | 39.65 | Yes | 2.406 | 0.001 | 2.407 | 2.406 | 0.000 | 2.406 | 0.041 | 0.001 | 0.042 | 0.041 | 0.000 | 0.041 |

Note that, since the scatterer is at a distance below the Tx Rayleigh distance, an FF approximation for the Tx-Sc path leads to a phase error greater than π8=0.393 as shown in the table. On the other hand, since the Rx array aperture is low, the Sc-Rx path is typically at a distance greater than the Rx Rayleigh distance (0.12 m). Thus, a far-field approximation is sufficient to model the Sc-Rx path. Thus, in effect, as the table demonstrates, employing an NF-FF approximation works well in terms of the phase error performance.

Observation 4: For non-direct path, the Tx-Scatterer path and Scatterer-Rx sub path can be independently modelled using NF or FF condition depending on the Tx, Rx and Scatterer locations and the Tx and Rx aperture sizes.

Proposal 4: The near- or far-field condition for the non-direct paths between BS and UE is assessed by using the distance between antenna array and cluster locations.

# Spatial non-stationary properties discussion

In NF, due to the large array size, and relative proximity of a receiver, different spatial channels can be observed by different antenna elements in the array. There are two types of scenarios that may cause SnS. The first case is a partial blockage of the antenna panel due to a physical blocker that disrupts a direct or indirect path in a Tx-Rx pair {s,u}. In another case, some clusters may be only visible for portions of Tx-Rx pair {s,u} due to unequal channel path power gain. Different parts of the array may have different views of the propagation environment, observing the same channel paths with different power, or different channel paths.

In the last meeting [1], SnS at UE side was discussed, and the following agreement has been agreed,

For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, for blockage model B, the procedure to determine the number of blockers and locations are same as existing blockage model B. For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, an attenuation per antenna element is introduced, the following options can be considered with down-selection by RAN1#120: - Option-1: generated by leveraging the existing blockage Model-B with potential updates. - At most 2 hand type blockers and one head type blocker for a specific UE is assumed. - Option-2: generated by leveraging the existing blockage Model-A with potential updates - Option-3: generated by a distribution or a fixed value due to the hand grip and/or head proximity FFS: the details, e.g., how to determine the value of loss for the case as one hand grip, a dual hand grip and/or head proximity

TR 38.901 blockage model A assumes a stochastic approach to characterize human and vehicular blocking, including non-self-blocking and self-blocking components. Self-blocking is mainly caused by the person’s hand and other body parts when the UE is held in particular orientations with particular gestures and this feature represents the user's blockage by creating an attenuation region (the attenuation is of 30 dB) in the UE's local coordinate system (LCS) for a device oriented in portrait or landscape mode as shown in Figure 1. Model A self-blockage is a stochastic blocker that is only specified in the angular domain, i.e. which angular intervals that are blocked, please refer TR 38.901 Table 7.6.4.1-1.

Figure 1 - Blockage model A self-blocking attenuation region. (a) Portrait mode. (b) Landscape mode

Self-blocking in model A only considers the angular blocking regions on the blockage direction. Without any enhancement, blockage model A may not be applicable to support SnS especially at UE side due to lacking the information related to each individual antenna element. To support SnS using model A self-blocking at UE side, one of proposals is to add the blocker’s location and size information for calculating the blockage conditions based on each individual antenna element (antenna element wise). If the blocker's location and size are specified in the stochastic blockage model A, then this enhancement becomes closer to the geometric blocker which is defined in the model B. The other possible approach is to define a stochastic blocker (e.g., distribution) with the angular range for antenna elements or each antenna elements, but this approach requires measurement efforts to ensure the proposed distribution reflecting the practical scenarios.

In TR 38.901, the blockage of a certain cluster is correlated in both space and time according to the UE mobility and both the blockage and the spatial consistency options applied for the update of the channel parameters, i.e., the cluster blockage is updated before the channel coefficients are recomputed with the spatial consistency procedure. One advantage of using model A as an alternative approach for SnS is due to its lower complexity and it may be easier to incorporate with the spatial consistency procedure.

In the last meeting [1], physical blockage model A or B approach was discussed for addressing the modelling of SnS at UE side. In our view, the blockage model B [2], that adopts a geometric method, seems a more accurate way for modelling of spatial non-stationarity at UE side. It is consistent in time, frequency and space, and is more suitable for simulations with arbitrary blockage at the price of being more computationally intensive.

Self-blocking study for blockage model B

In last meeting [1],  two new blocker parameters (user hand/head) have been discussed:

- FFS: the details related to the user hand/head: Indoor; Outdoor; FFS: User hand; Cartesian: w=[0.2]m; h=[0.1]m; Stationary. Indoor; Outdoor; FFS: User head; Cartesian: w=[0.24]m; h=[0.20] m; Stationary. - FFS: The details of blockage model B to implement the impact of user hand and head. - FFS: The location of the user hand/head

The blockage model B being a geometry-based approach can be used at both the BS and UE side.  In order to differentiate  the blockage effects across various parts of the antenna array, we further consider blockage model B with new blocker types to support self-blocking characterization as in portrait mode and landscape mode in  model A. To demonstrate this self-blockage effects in model B, we consider a UE with dimensions typical for handheld devices, specifically a rectangular structure with length 0.14 m and width 0.06 m. The UE’s antenna array configuration is such that an antenna is positioned along each of the edge centers. We assume a hand width of 0.20 m and a hand height of 0.10 m consistent with the agreement during last meeting for modeling SnS. We demonstrate the portrait mode from a model B perspective in our simulations. In this demonstration, a single hand blocker (marked in orange) is modeled with its center around the middle of the bottom edge.  Figure 2 illustrates a self-blockage effect in portrait mode in model B.

Figure 2: Self blocking in model B for hand (blocker) in portrait mode

For each UE antenna element, blocking regions can be determined by calculating an angular span on elevation angle (θ) and azimuth angle (ϕ) for each hand’s blocking area w.r.t the UE antenna position. In blockage model B, blockers are modelled as rectangular screens characterized by their height hk, width wk,  with each blocker centred at coordinates (xk,yk,zk). The position and dimensions of the physical blocker directly influence the angular span range of the Tx/ Rx antenna elements by determining which paths/rays are obstructed. For a given antenna element, a path from/to the UE antenna element may intersect with the blocker plane. If this intersection occurs within the plane’s boundaries, the path can be considered as blocked, and an attenuation (e.g. -30 dB) will be applied for the blocked path. The point of intersection is dependent on the blocker, UE and array locations as well as their geometries. For a given antenna element, if the path from a cluster to the element intersects with the blocker plane, the indirect path can be considered as blocked.

Given the blocker/hand dimensions (hk,wk), center coordinates (xk,yk,zk) and the locations of the antenna elements, the blocked angular region for each antenna elements can be determined.  For each antenna location, we compute the angular obstruction by translating the region spanned by the hand/blocker into spherical coordinates. Specifically, we calculate the azimuth (ϕ) and elevation (θ) angles from the antenna to each point on the hand, representing the angular area the hand blocks. Since each antenna element may be located on a different edge or position on the UE, the resulting blocked angular region varies accordingly.

In Figure 3, the variation in the blocked angular region for each UE antenna element is evaluated when the hand blocker is above UE by a distance equal to 0.01 m and 0.04 m. For a given distance, we demonstrate 4 cases each corresponding to one of the 4 UE antennas for studying antenna element wise SnS. The angular area marked in dark blue represents regions blocked by the hand, while the lighter region shows unblocked angular directions. The blocked region is set to -30 dB attenuation whereas the unblocked region is set to 0 dB to be adopted from the binary attenuation in model A. Note that in general, the shape of the blocked region can exhibit arc like patterns due to the spherical projection of the rectangular blocker plane in the θ-ϕ space. Antenna 1 positioned at the bottom edge experiences a larger blocked angular span due to its proximity to the hand in portrait mode. Antenna-2 (left edge) and antenna-4 (right edge) show similar blockage patterns due to symmetric geometric antenna element location. Antenna-3, positioned at the top edge has the least obstruction due to it being the farthest from the hand blocker. When the hand is raised from 0.01 m to 0.04 m, the blocked angular region for antenna-1 (bottom) reduces, while it increases for antennas 2,3 and 4. This occurs because the increased distance slightly reduces the blockage for the bottom edge antennas whereas the other edges are now slightly more obstructed. Since different antennas along different edges experience distinct blockage effects, elementwise SnS modeling can more accurately capture the variation of signal blockage across different antennas.

The blocked angular region defined in self-blockage model A can be presented using model B with some updates as shown in Figure 3. When the blocker location and size are known or given, the blocked angular region for each antenna element at UE side can be determined and the antenna element wise blockage effect for SnS can be emulated. In addition, when- considering self-blocking in model B, the blocker location (distance) to the UE may be constrained by a range, e.g. from 0.01 m to 0.08 m and the knife-edge can be ignored in this scenario.

Figure 3: Hand blocker: Blocked angular region for Portrait mode at different blocker to UE distances

Based on the described procedures, modelling based on Options 1 and 2 requires a relatively complex computations considering various factors. However, the outcome of either option is to generate some loss factor per antenna element according to the way a UE is self-blocked. Option 3 proposes a much simpler procedure than the other two options with a similar outcome, where a set of loss factors are generated by using a distribution or a fixed value to emulate the effect of hand grip and/or head proximity. In other words, Option 3 is an abstract approach with a similar end results as other options for modelling of self-blockage. According to the described procedure, a set of loss factor are generated. The value of loss factors may be fixed or determined based on a distribution to emulate the effect of hand grip and/or head proximity.

Observation 5: Option 3 is an abstract approach with a similar end results as other options for modelling of self-blockage.

Proposal 5: Support Option 3, where the value of loss factors per antenna element may be fixed or determined based on a distribution to emulate the effect of hand grip and/or head proximity.

Variation Attenuation of power terms for path/ray status

For model A blockage modelling, for each cluster, an attenuation due to blockers has been considered. Additional per-cluster attenuation due to self-blockage can be considered. However, for model B, the attenuation is introduced per sub-path. Therefore, the variation (e.g., reduction) of power for the impacted ray/cluster within the element-pair can be enhanced and introduced for modelling spatial non- stationarity of the channel. In addition, we can introduce a parameter, αu,s, as an indicator of path status, e.g., visible (reflection or direct path), non-visible/blockage or diffraction. For example,

αu,s=  -30 dB,                                         block 0 dB,                                       nonblock-30 dB<…< 0 dB,    diffraction (if supported)

To simplify the channel model, the existing knife edge attenuation model used in blockage model A/B can be adopted to model the power attenuation per path status. The phase term due to knife edge diffraction can be further studied if this option is adopted. In addition, for each ray/cluster, rotating the blocker to ensure the arrival/departure direction at each Receive/Transmit antenna element is always perpendicular to the screen, respectively, can be supported to simplify the channel model assumption for variation of power term due to SnS. In other words, maintaining the condition of the perpendicular to the screen property can facilitate the determination of path power variation, i.e., blocked, non-blocked or diffraction.

The αu,s (path status) parameter can be applied in the channel generation coefficient in Step 11 for both LOS/direct path and NLOS/non-direct path in TR 38.901 Eq. (7.5-30). For example, for each pair {s,u}, of Tx-Rx antenna elements, we can formulate each channel model for the 1st cluster as,

Hu,s,1LOS,SnS=αu,s,1Hu,s,1LOS

Observation 6: Rotating the blocker to ensure the arrival/departure direction at each Receive/Transmit antenna element can be reused for the determination of path power variation, i.e., blocked, non-blocked or diffraction.

Proposal 6: Reuse the existing mechanism for rotating the blocker to ensure the arrival/departure direction at each Receive/Transmit antenna element.

Observation 7: The power attenuation term can be used for spatial non-stationary status for a path, i.e., blockage, reflection, or knife-edge diffraction in model B.

Proposal 7: Introduce an attenuation parameter as the path status for every element-pair/Tx-Rx for modelling of non-stationarity property in Step 11.

# Conclusions

In this contribution, we shared and discussed our views for Rel-19 FR3 NF channel modeling. Based on presented discussions, the following observations and proposals are made,

Observation 1: SnS and cluster consideration have a high impact on details related to other steps.

Proposal 1: Prioritize discussions on SnS and cluster consideration to facilitate progress on other topics.

Observation 2: A propagation typically experiences more than one reflection. Therefore, to support more flexibility and accuracy in modelling of the NF channel, it may be better that both d1 and d2 are considered.

Proposal 2: Support d1-based solution, only if there are sufficient analysis and measurement to validate its accuracy and flexibility.

Observation 3: The process of determination of the two distances can be somehow simplified by considering a stochastic distribution for generation of the total delay τn and d1,n.

Proposal 3: Study when d1 is generated from a distribution function (e.g. a uniform distribution) and d2 is generated from the cluster delay τn and the d1.

Observation 4: For non-direct path, the Tx-Scatterer path and Scatterer-Rx sub path can be independently modelled using NF or FF condition depending on the Tx, Rx and Scatterer locations and the Tx and Rx aperture sizes.

Proposal 4: The near- or far-field condition for the non-direct paths between BS and UE is assessed by using the distance between antenna array and cluster locations.

Observation 5: Option 3 is an abstract approach with a similar end results as other options for modelling of self-blockage.

Proposal 5: Support Option 3, where the value of loss factors per antenna element may be fixed or determined based on a distribution to emulate the effect of hand grip and/or head proximity.

Observation 6: Rotating the blocker to ensure the arrival/departure direction at each Receive/Transmit antenna element can be reused for the determination of path power variation, i.e., blocked, non-blocked or diffraction.

Proposal 6: Reuse the existing mechanism for rotating the blocker to ensure the arrival/departure direction at each Receive/Transmit antenna element.

Observation 7: The power attenuation term can be used for spatial non-stationary status for a path, i.e., blockage, reflection, or knife-edge diffraction in model B.

Proposal 7: Introduce an attenuation parameter as the path status for every element-pair/Tx-Rx for modelling of non-stationarity property in Step 11.

# References

- Chairman’s notes, 3GPP TSG RAN1 Meeting #118b, HEFEI, CN, Oct 2024

- 3GPP TR 38.901, Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz, (Release 18)

- RP-234018, New SID: Study on channel modelling enhancements for 7-24GHz for NR, RAN#102, December 2023